# Quantification of serum hCG-β

The serum hCG-β levels was measured by automated quantitative enzyme linked fluorescent immunoassay (ELFA) (VIDAS®; bioMérieux; FRANCE) for the determination of human Chorionic Gonadotropin (hCG) concentration in human serum or plasma to aid in the early detection of pregnancy.

## Procedure

All assay steps and assay temperature are controlled by the instrument.  A pipette tip-like disposable device, the Solid Phase Receptacle (SPR®), serves as a solid phase for the assay as well as a pipetting   device. The SPR is coated   at   the   time   of   manufacture with mouse monoclonal anti-hCG antibodies. The assay configuration prevents nonspecific reactions with the SPR.  Reagents for the assay are in the sealed Reagent Strips.  The sample is transferred into the well containing anti-hCG antibody conjugated with alkaline phosphatase. The sample/conjugate mixture is cycled in and out of the SPR and the hCG will bind to antibodies coated on the SPR and to the conjugate forming a “sandwich”.  Wash steps remove unbound conjugate. A fluorescent substrate, 4-methylumbelliferyl phosphate, is cycled through the SPR. Enzyme remaining on the SPR wall will catalyse the conversion of the substrate to the fluorescent product 4-methylumbelliferone.  The intensity of fluorescence is measured by the optical scanner in the instrument; it is proportional to the hCG concentration present in the sample.  When the assay is completed, the results are analysed automatically by the instrument and a report is printed for each sample. The acceptable range of results for the assay is 2 mIU/mL to 1500 mIU/ml.