Supplementary table S1: Summary of the time domain features

There are more than 27-time domain features, supplementary table S1 shows a summary of some of these features, and formulas are calculated after dividing the signal x into windows of length L, xi,k  represents the kth element of the ith window.

Supplementary table S1 Summary of the time domain features

| Feature | Formula | Explanation |
| --- | --- | --- |
| Mean Absolute Value (MAV) (1) | MAV (xi) = 1Lk=1Lxi,k | The moving average of the signal. |
| Waveform length (WL) (2) | WL (xi) = k=1Lxi,k- xi,k-1 | Offers a simple characterization of the amplitude, duration and frequency of the signal |
| Zero Crossing (ZC) (3) | {xi,k>0 and xi, k+1<0} or {xi,k>0 and xk+1<0} and xk- xk+1 ≥ϵ | Counts the frequencies at which the signal passes through zero (a threshold ∈>0 is used to avoid noise). |
| Variance (VAR) (4) | VAR=1Lk=1Lxi,k2 | An index to the power of the signal. |
| Root Mean Square (RMS) | RMS xi= 1L k=1Lxi,k2 | Also known as the quadratic Mean. Related to the Standard deviation when the mean of the signal =0. |
| Average Amplitude Change (AAC) (3) | AAC=1L k=1Lxi,k+1- xi,k | Shows the mean value by which the amplitude of the signal changes |
| Slope sign change (SSC) (3) | ssc= (xi,k- xi, k-1)* (xi,k- xi, k+1) ≥ϵ | Measures the frequency at which the signal changes the slope sign (derivative). |
| Skewness (SKEW) (3) | SKEW = k=1L(xi,k- xi)3L* σ3 Where σ is the standard deviation | Measures the asymmetry of the distribution. |
| Autoregressive coefficient (AR) (3) | xi,k=j=1Pρjxi,k-j+ϵt Where p is the model order and ρj is the jthcoefficient of the model and ϵt is the residual noise | Aims to predict the future values of the signal based on the weighted average of the previous data. It shows each sample point as a linear combination of previous samples and an error. |
| Integrated EMG (IEMG) | IEMG xi= k=1Lxi,k | Returns the absolute sum of the segment |
| Myopulse Percentage Rate (MYOP) | MYOP= lLK=1Lif (xi>threshold) | Shows the mean absolute value of the segment of the windows that is larger than an amplitude threshold value |
| Temporal Moment (TM) | TM= 1LK=1Lxi,korder | The 1st order is the MAV, and the 2nd order is the variance, thus it usually starts from the 3rd order. It is a statistical analysis technique that can be also used as a feature |
| V -order (VO) | VO= 1LK=1Lxiorder1order | According to (Phinyomark, et al., 2012).), it gives an insight into the force of the muscle contraction. |
| Mean Absolute Derivative (MAD) | MAD= 1Lk=1Lxi,k- xi | Shows the distance between each sample of the window and the mean. |

## Supplementary table S2: Summary of the Frequency domain features

Supplementary table S2, shows a summary from (59) of some of these features, formulas are calculated after dividing the signal x with frequency f and power spectrum p into windows of length L, xi,j  represents the jth element of the ith window.

Supplementary table S2: Summary of the Frequency domain features

| Feature | Formula | Explanation |
| --- | --- | --- |
| Mean Frequency (MNF) | MNF= j=1Mfi,j Pi,j k=1L xi,j | The average frequency. |
| Median frequency (MDF) | j=1MDFPi,j = j=MDFMPi,j= 12j=1MPi,j | The frequency divides the spectrum into two regions that are equal in amplitude. |
| Mean power frequency (MNP) | MNP= j=1MPi,j M | The average power of the power spectrum. |
| Peak frequency (PF) | PKF = max (Pi,j ) where j=1,2…….M. | The frequency corresponds to the highest power. |
| Total power (TTP) | TTP = j=1MPi,j | A summation of the sEMG power spectrum. |
| Frequency ratio (FR) | FR= j=LLCULCPi,j j=LHCUHCPi,j where ULC and LLC are the upper- and lower-cutoff frequency of the low-frequency band and UHC and LHC are the upper- and lower-cutoff frequency of the high-frequency band, respectively | The ratio between the highest and lowest frequency components of the sEMG signals. used to distinguish between the contraction and relaxation of the muscles. |
| Power spectrum ratio (PSR) | PSR= P0P= j= fo-nfo+nPi,j j=-∞∞Pi,j Where f0 is a feature value of the FPK and n is the limit for integration | The ratio between the energy (nearly the maximum value of the sEMG power spectrum) and the energy P which is the whole energy of the sEMG power spectrum. |

## Supplementary table S3: Summary of the time-frequency domain features (5):

Some of the time-frequency features are found in Supplementary table S3.

Supplementary table S3: Summary of the time-frequency domain features

| Feature | Formula | Explanation |
| --- | --- | --- |
| Continuous Wavelet transform (CWT) | CWT (a,b)= 1a-∞+∞xtψ*t-badt where ψ tis the mother wavelet, a is a scale parameter and b is a translation parameter | Uses every possible wavelet in a range of locations and scales through the changing parameters and b. |
| Discrete Wavelet Transformation (DWT) | DWT n,m=-∞+∞xtψn,mtdt And, ψn,mt= 1a0m ψ(t-n b0 a0ma0m) Where m is the dilation parameter, n is the translation parameter and a0 is the step parameter | Uses defined wavelets in a range of locations and scales. |

References

1.	Abbaspour S, Lindén M, Gholamhosseini H, Naber A, Ortiz-Catalan M. Evaluation of surface EMG-based recognition algorithms for decoding hand movements. Med Biol Eng Comput [Internet]. 2020 Jan 1 [cited 2023 May 19];58(1):83. Available from: /pmc/articles/PMC6946760/

2.	Qin P, Shi X. Evaluation of Feature Extraction and Classification for Lower Limb Motion Based on sEMG Signal. Entropy 2020, Vol 22, Page 852 [Internet]. 2020 Jul 31 [cited 2023 May 19];22(8):852. Available from: https://www.mdpi.com/1099-4300/22/8/852/htm

3.	Côté-Allard U, Fall CL, Drouin A, Campeau-Lecours A, Gosselin C, Glette K, et al. Deep Learning for Electromyographic Hand Gesture Signal Classification Using Transfer Learning. IEEE Transactions on Neural Systems and Rehabilitation Engineering. 2019 Apr 1;27(4):760–71.

4.	Li W, Shi P, Yu H. Gesture Recognition Using Surface Electromyography and Deep Learning for Prostheses Hand: State-of-the-Art, Challenges, and Future. Front Neurosci. 2021 Apr 26;15:259.

5.	Tanwar S, Nayyar A, Rameshwar R. Machine Learning in Signal Processing: Applications, Challenges, and The Road Ahead [Internet]. CRC Press; Chapman & Hall; 2022 [cited 2023 May 19]. Available from: https://www.routledge.com/Machine-Learning-in-Signal-Processing-Applications-Challenges-and-the/Tanwar-Nayyar-Rameshwar/p/book/9780367618902