Comprehensive study on the occurrence of heavy metals and their environmental risks in PM2.5 of indoor dusts in Xi’an, China

Naiwang Yang a, Yiyu Wang b, Pingqiang Gao b, Jiayin Zhang c, Wenbin Song a, Xuejuan Song a, Shiyun Liu d, Huijun Su b,*

a Xi’an Environmental Monitoring Station, Xi’an, Shaanxi, China

b School of Chemistry and Chemical Engineering, Yulin University, Yulin, Shaanxi, China

c Shaanxi Environmental Monitoring Center, Shaanxi Key Lab for Environmental Monitoring and Forewarning of Trace Pollutants, Xi’an, Shaanxi, China

d Clear Science & Technology Co., Ltd., Beijing, China

* Corresponding author:

Huijun Su

Tel.: 86-1-518-073-5884

Email: hjsu@yulinu.edu.cn.

Table S1. Measurement conditions and limits of quantification for target analytes in PM2.5 of indoor dusts by EDXRF

| Element | Secondary target | Analyzed line | Accelerating voltage (kV) | Current (mA) | Analysis time (s) | Limits of quantification (LOD, mg/kg) |
| --- | --- | --- | --- | --- | --- | --- |
| Cr | Fe | Kα | 100 | 6 | 200 | 0.003 |
| Mn | Ge | Kα | 100 | 6 | 200 | 0.040 |
| Ni | Ge | Kα | 100 | 6 | 200 | 0.007 |
| Cu | Ge | Kα | 100 | 6 | 200 | 0.006 |
| Zn | Ge | Kα | 100 | 6 | 200 | 0.009 |
| Pb | Al2O3 | Lα | 100 | 6 | 200 | 0.174 |
| As | KBr | Kα | 100 | 6 | 200 | 0.007 |
| Cd | Al2O3 | Kα | 100 | 6 | 200 | 0.007 |

Table S2. Geo-accumulation index (Igeo) to determine the heavy metal pollution levels

| Igeo | Classification | Degree of pollution |
| --- | --- | --- |
| Igeo ≤ 0 | Ⅰ | Unpolluted |
| 0 < Igeo ≤ 1 | Ⅱ | Unpolluted to moderately polluted |
| 1 < Igeo ≤ 2 | Ⅲ | Moderately polluted |
| 2 < Igeo ≤ 3 | Ⅳ | Moderately to strongly polluted |
| 3 < Igeo ≤ 4 | Ⅴ | Strongly polluted |
| 4 < Igeo ≤ 5 | Ⅵ | Strongly to very strongly polluted |
| Igeo > 5 | Ⅶ | Very strongly polluted |

Table S3. Risk grades indexes and grades of potential ecological risk of heavy metal contamination

| Class | Eri value | RI value | Risk grade |
| --- | --- | --- | --- |
| 1 | <40 | <150 | Low potential ecological risk |
| 2 | 40–80 | 150–300 | Moderate potential ecological risk |
| 3 | 80–160 | 300–600 | Considerable potential ecological risk |
| 4 | 160–320 | ≥600 | High potential ecological risk |
| 5 | ≥320 | / | Significantly high potential ecological risk |

Table S4. The definitions and reference values of evaluation parameters of average daily exposure to heavy metals

| Parameters | Definitions | Unit | Adult | Children |
| --- | --- | --- | --- | --- |
| C | Concentration of metal | mg/kg | - |  |
| D | Dose | mg/(kg·d) | - |  |
| RfD | Reference dose | mg/(kg·d) | The reference values see Table S4 |  |
| SF | Slope factor | - | The reference values see Table S4 |  |
| HQ | Hazard quotient | - | - |  |
| HI | Hazard index | - | - |  |
| IngR | Ingestion rate | mg/d | 100 | 200 |
| InhR | Inhalation rate | mg/d | 14.5 | 7.5 |
| EF | Ingestion rate | d/a | 350 | 350 |
| ED | Exposure duration | a | 24 | 6 |
| SA | Exposed skin area | cm2 | 5373.99 | 2848.01 |
| SL | Skin adherence factor | mg/ (cm2·d) | 0.07 | 0.2 |
| ABS | Dimensionless dermal absorption factor | - | 0.001 | 0.001 |
| PEF | Particle emission factor | m3/kg | 1.36×109 | 1.36×109 |
| BW | Average body weight | kg | 61.8 | 19.2 |
| AT | Averaging time | d | ED × 365 days for non-carcinogens, and 70 × 365 = 25550 days for carcinogens |  |

Table S5. The reference doses and carcinogenic slope factors of target heavy metals (mg/(kg·d)) in this study

| Parameters | Cr | Ni | Cu | Zn | Cd | Pb | Mn | As |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfDinh | 2.86×10-5 | 2.06×10-2 | 4.02×10-2 | 3.00×10-1 | 1.00×10-3 | 3.52×10-3 | 1.43×10-5 | 3.01×10-4 |
| RfDing | 3.00×10-3 | 2.00×10-2 | 4.00×10-2 | 3.00×10-1 | 1.00×10-3 | 3.50×10-3 | 4.60×10-2 | 3.00×10-4 |
| RfDdermal | 6.00×10-5 | 5.40×10-3 | 1.20×10-2 | 6.00×10-2 | 1.00×10-5 | 5.25×10-4 | 1.84×10-3 | 1.23×10-4 |
| SF | 42 | 0.84 | - | - | 6.3 | - | - | 15.1 |

Table S6. Hazard quotient (HQ) and risk for each metal in PM2.5 of indoor dusts in Xi’an

| Metals | Adult |  |  |  | Children |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HQing | HQinh | HQdermal | HI | HQing | HQinh | HQdermal | HI |
| Cr | 0.1 | 1.12×10-9 | 0.0189 | 0.119 | 0.889 | 2.57×10-9 | 0.0919 | 0.981 |
| Mn | 0.0234 | 8.02×10-9 | 0.00225 | 0.0256 | 0.0234 | 2.07×10-9 | 0.011 | 0.0343 |
| Ni | 0.00558 | 5.78×10-13 | 7.78×10-5 | 0.00566 | 0.0495 | 1.33×10-12 | 0.000379 | 0.0499 |
| Cu | 0.0111 | 1.18×10-12 | 0.00014 | 0.0113 | 0.0987 | 2.71×10-12 | 0.000681 | 0.0994 |
| Zn | 0.00691 | 7.37×10-13 | 0.00013 | 0.00704 | 0.0613 | 1.69×10-12 | 0.000633 | 0.0619 |
| Pb | 0.162 | 1.73×10-11 | 0.00409 | 0.166 | 1.44 | 3.97×10-12 | 0.02 | 1.46 |
| As | 0.242 | 2.57×10-11 | 0.00222 | 0.244 | 2.14 | 5.89×10-11 | 0.0108 | 2.15 |
| Cd | 0.0108 | 1.15×10-12 | 0.00407 | 0.0149 | 0.096 | 2.65×10-12 | 0.0199 | 0.116 |

Figure S1. Mean Igeo values of target analytes in the PM2.5 of indoor dusts collected from five sampling areas (a-d: the sampling areas located at east, west, south, north and central of Xi’an, and named CJZ, KGC, CHA, JIK and BAQ, respectively).

Figure S2. Potential ecological risks of heavy metals in PM2.5 of indoor dust from Xi’an.