Section A1: Accounting Methods of Regulating Services (ERV)

- Water retention: It is the function of the vegetation to enhance soil infiltration, conserve soil moisture and increase available water resources. The biophysical values of water retention were measured by the water balance method, where the annual evapotranspiration is calculated using the average daily precipitation, average daily temperature provided by the National Meteorological Science Data Center from 28 observation stations located within Shanxi Province and Gaoqiao's formula(Song et al., 1999). The Takahashi formula can be expressed in the following formulas. And the replacement cost method was adopted for the monetary quantitys of water retention, whereas the engineering cost of reservoir unit capacity was obtained by using the annual national industrial producer purchase price index and cumulative price index (Table A3).

|  |  | (1) |
| --- | --- | --- |

Whereis monthly evaporation (mm), is monthly rainfall (mm), and is the monthly average temperature (°C).

- Soil retention: It refers to the ability of the vegetation to protect the soil, abate rainfall erosion and reduce soil loss. The biophysical values of soil conservation were obtained using the Revised Universal Soil and Water Loss Equation (RUSLE), in which the rainfall erodibility factors were calculated using daily rainfall information and the semimonthly rainfall erodibility model(Zhang et al., 2002). In contrast, the soil erodibility factors were obtained by using the data on percent organic matter content, percent powder and clay content from the 1:1 million soil database established by the Second Chinese Land Survey and the Nomo model. Slope length factors and slope factors were derived using DEM data, hydrological analysis, slope analysis in ArcGIS and the raster calculator. The monetary quantitys were assessed in terms of both reduction of surface source pollution and reduction of sedimentation (Table A3).

- Sandstorm retention: It is the feature of the vegetation to reduce soil erosion and wind damage caused by heavy winds. The biophysical values of sandstorm retention were obtained by adopting the Modified Wind Erosion Model (RWEQ), in which the climatic erosion factors can be calculated based on the following equation with the average daily wind speed, rainfall, temperature, sunshine hours and latitude provided by the National Meteorological Science Data Center and the Chinese snow depth long time series data set provided by the National Qinghai-Tibet Plateau Science Data Center. The calculation of the monetary quantitys applied the vegetation restoration cost with replacement cost method (Table A3).

|  |  | (2) |
| --- | --- | --- |

Where, is the wind speed at 2m, m/s; is the critical wind speed at 2m; is the number of days of the experiment; is the air density, kg/m3; is the frequency of wind speed observation;is gravitational acceleration, m/s2; and are two dimensionless quantities, respectively expressed as soil moisture factor and snow cover factor.

- Flood mitigation: It is the function of the distinctive ecological structure of an ecosystem to mitigate the threat and damage caused by flood peaks during the flood season through the absorption of rainfall and transit water. The biophysical values of flood mitigation applied the water storage model, consisting of vegetation, reservoir and lake flood storage. The monetary quantitys were obtained by the shadow engineering method (Table A3).

- Carbon sequestration: It is the mechanism by which ecosystems absorb carbon dioxide to synthesize organic matter and fix carbon in vegetation or soil. The biophysical values of carbon sequestration, constituted by the carbon fixation of farmland, forest, scrub, grassland, and wetland, were accounted by the carbon sequestration rate method, with the help of the crop yields, fertilizer usage, carbon sequestration rate values, grass to grain ratio, and the implementation rate of straw return to farmland(Chen et al., 2020) (Table A3). The monetary quantitys adopted the carbon price with the replacement cost method, where the carbon price was obtained using the Swedish carbon tax and the exchange rate of US dollar to CNY.

- Oxygen supply: It refers to the function of vegetation to release oxygen during the process of photosynthesis, which can preserve the stability of oxygen in the atmosphere. The measurement of the biophysical values adopted the oxygen release mechanism model, and the monetary quantitys were obtained by using the industrial oxygen price and the replacement cost method, where the industrial oxygen prices were derived from the oxygen price released in the spring of 2007 on the website of the Ministry of Health of the People's Republic of China, and the Shanxi Province Industrial Producer Ex-Factory Price Indexes (Table A3).

- Air purification: It is the capacity of the ecosystem to absorb, filter, block and break down atmospheric pollutants. The biophysical values were measured with the pollutant purification model, and in this paper, we mainly focused on this function of the ecosystem in absorbing the SO2, NOx and the dust. The unit purification quantity of these can be obtained by referring to relevant literature(Li et al., 2015;Wang et al., 2017) (Table A1). The monetary quantitys were estimated using the emission levy standard and the replacement cost method (Table A3).

Table A1. Statistical table of purification amount for the unit area of different ecosystems.

| # Species of ecosystem | # Air pollutants | # Purification amount/t.km-2‧a-1 |  |
| --- | --- | --- | --- |
| Forest | SO2 |  | 22.64 |
|  | NOx |  | 0.82 |
|  | Dust |  | 0.6 |
| Shrubs | SO2 |  | 8.96 |
|  | NOx |  | 0.6 |
|  | Dust |  | 0.087 |
| Grassland | SO2 |  | 1.13 |
|  | NOx |  | 0.06 |
|  | Dust |  | 0.03 |

- Water purification: Apart from the air purification, the ecosystems of water bodies such as wetlands and rivers also have the abilities to adsorb and degrade the pollutants in water areas. The pollutant purification model and the replacement cost method were employed to assess the biophysical and monetary quantitys of the wetland, respectively (Table A3). In this study, we mainly consider the purification values of ecosystems for Chemical Oxygen Demand(COD), nitrogen, and phosphorus(Jia et al., 2015;Wang et al., 2017).

- Climate regulation: It is the feature of the ecosystem that absorbing solar energy, decreasing the temperature and increasing the humidity of the atmosphere through processes such as transpiration by vegetation and evaporation by the water surface. The value of climate regulation includes the energy consumed by the vegetation transpiration and water surface evaporation. And the monetary quantitys of climate regulation estimated by the electricity prices and replacement cost method (Table A3).

Table A2. Summary of source and function of original data.

| Data | Frequency of data acquisition | Sources | Types and Resolutions (unit) | Functions |
| --- | --- | --- | --- | --- |
| Gross output and output value of farming, forestry, animal husbandry and fisher | Annually | Shanxi Statistical Yearbook | Statistics /ton、CNY | EPV; Carbon sequestration - the amount of carbon sequestration in farmland soil |
| Hydropower generation | Annually | China Statistical Yearbook | Statistics / kW‧h | EPV |
| The usage of straw, firewood, and biogas | Annually | Environmental Statistics Yearbook | Statistics /ton | EPV |
| Rainfall | Everyday | National Meteorological Science Data Center | Statistics /mm | Water retention; Soil retention - rainfall erosivity factor; Sandstorm retention- climatic erosion factor; Flood mitigation - vegetation flood storage capacity |
| Mean temperature | Everyday | National Meteorological Science Data Center | Statistics /°C | Water retention; |
| Maximum temperature | Everyday | National Meteorological Science Data | Statistics /°C | Climate regulation - Number of days air conditioning is open |
| Sunshine duration | Everyday | National Meteorological Science Data | Statistics /h |  |
| Wind speed | Everyday | National Meteorological Science Data | Statistics / m‧s-1 |  |
| Long-term series of daily snow depth dataset | Everyday | National Tibetan Plateau Scientific Data Center (Liyun and Tao 2015) | Raster/8*8 km |  |
| Regional inflow、Regional outflow | Annually | Water Resources Bulletin of Shanxi Province | Statistics /m3 | Water retention |
| Harmonized World Soil Database | - | National Tibetan Plateau Scientific Data Center (Food and International Institute for Applied Systems 2019) | Raster/1*1 km | Soil retention - Soil erodibility factor; Sandstorm retention - soil erosion Factor , soil crusting factor |
| Digital Elevation Model (DEM) | - | Data Center for Resources and Environmental Sciences, Chinese Academy of Sciences | Raster/1*1 km | Soil retention - slope length factor, slope factor, soil and water conservation measures factor; , surface roughness factor |
| Normalized Difference Vegetation Index (NDVI) | 1990/1995 | National Tibetan Plateau Scientific Data Center(The National Center for Atmospheric 2018) | Raster/8*8 km | Soil retention- vegetation cover and management factor; Sandstorm retention- vegetation cover factor |
|  | 2000-2015 | Data Center for Resources and Environmental Sciences, Chinese Academy of Sciences(Xu 2018) | Raster/1*1 km |  |
|  | 2020 | NASA.EarthData | Raster/1*1 km |  |
| Land use/ Land cover (LULC) | Every five years | Data Center for Resources and Environmental Sciences, Chinese Academy of Sciences | Raster/1*1 km | EPV; ; ; ; Reservoir flood storage capacity; Flood storage capacity in the lake area; Carbon sequestration - forest carbon sequestration ,grassland carbon sequestration , wetland carbon sequestration , carbon sequestration in farmland ; Air purification, water purification, climate regulation - ecosystem area |
| Irrigated area and amount | Annually | China Statistical Yearbook | Statistics / hectare |  |
| Agricultural Area | Annually | China Statistical Yearbook | Statistics / hectare |  |
| Total reservoir capacity | Annually | China Statistical Yearbook | Statistics/m3 |  |
| Fertilizer usage | Annually | China Statistical Yearbook | Statistics / ton |  |
| Annual total number of tourists, entrance fee, accommodation, transportation, travel days | Annually | Local tourism bureau | Statistics | Cultural service- leisure tourism |

Table A3. Summary of GEP accounting indicators and methods in Shanxi Province.

| Ecosystem Service Types | Accounting Subjects | Biophysical quantity accounting methods | Monetary quantity accounting methods |
| --- | --- | --- | --- |
| Provisioning services (EPV) |  | is the total yield of ecosystem products (), is the yield of the i-th product () | Market value method is the monetary quantity of provisioning services (), is the price of category i ecosystem products() |
| Regulating services (ERV) | Water retention | Water balance equation represents the water retention capacity (), is annual rainfall (), is the annual evaporation () | Replacement cost method represents the accounting value of water retention (), is the average cost of reservoir construction () |
|  | Soil retention | Revised Universal Soil and Water Loss Equation (RUSLE) represents the soil retention capacity () | Replacement cost method represents the accounting value of soil retention (), is the reduced cost of dredging (), is the reduced cost of non-point source pollution treatment ()， is the soil bulk density (), is the pure content of nitrogen and phosphorus in soil (), is the cost of reservoir dredging per unit () |
|  | Sandstorm retention | Modified Wind Erosion Model (RWEQ) represents the sandstorm retention capacity () | Replacement cost method is the monetary quantity of sandstorm retention(), is the thickness of soil desertification covering sand (), is unit vegetation restoration cost () |
|  | Flood mitigation | Water storage model, including vegetation-regulated storage and flood lag (ponds, lakes) represents the flood mitigation capacity () | Shadow engineering method is the monetary quantity of flood mitigation() |
|  | Carbon sequestration | This mainly includes the amount of carbon sequestration in terrestrial ecosystems is the amount of carbon sequestration in ecosystems ()， is the coefficient for C to CO2 | Replacement cost method is the monetary quantity of carbon sequestration(), is the carbon price () |
|  | Oxygen supply | Oxygen release mechanism model represents the oxygen supply capacity () | Replacement cost method is the monetary quantity of oxygen supply(), is the price of industrial oxygen () |
|  | Air purification | Pollutant purification model is the ecosystem air purification capacity (), is the purification amount per unit area of the j-th air pollutant in the i-type ecosystem () | Replacement cost method is the monetary quantity of air purification(), is the purification amount of the I type of air pollutant (), is the treatment cost of i-type air pollution () |
|  | Water purification | Pollutant purification model represents the ecosystem water purification capacity (), is the purification amount per unit area of the j-th type of water pollutant in the class i ecosystem | Replacement cost method is the monetary quantity of water purification(), is the purification amount of the i-type of water pollutant (), is the treatment cost of the i- type water pollutant () |
|  | Climate regulation | represents the energy consumed by ecosystems to regulate climate (), is the energy consumed by ecosystem transpiration (), is the energy consumed by ecosystem water evaporation () | Replacement cost method is the monetary quantity of climate regulation(), is the residential electricity price () |
| Cultural services (ECV) | Leisure tourism | represents the biography quantity of leisure tourism, is the number of national scenic spots, is the average number of annual tourist arrivals in national scenic spot, is the number of provincial scenic spots, is the average number of annual tourist arrivals in provincial scenic spot | Travel expense method is the monetary quantity of leisure tourism, is the number of national scenic spots, is the annual average travel cost in national scenic spot (), is the number of provincial scenic spots, is the annual average travel cost in provincial scenic spot () |

Table A4. Values for three types of GEP services、GEP and GDP from 1990 to 2020.

| Category of Ecosystem Services |  | 1990 |  | 1995 |  | 2000 |  | 2005 |  | 2010 |  | 2015 |  | 2020 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Monetary Value | % of Total Value | Monetary Value | % of Total Value | Monetary Value | % of Total Value | Monetary Value | % of Total Value | Monetary Value | % of Total Value | Monetary Value | % of Total Value | Monetary Value | % of Total Value |
| EPV/Billion CNY |  | 12.75 | 10.84 | 30.63 | 6.3 | 32.76 | 1.84 | 46.3 | 1.8 | 101.96 | 4.45 | 145 | 3.37 | 154.65 | 3.37 |
| ERV | Water retention /Billion CNY | 45.98 | 39.08 | 118.94 | 24.48 | 235.86 | 13.22 | 90.47 | 3.52 | 120.36 | 5.26 | 131.56 | 3.06 | 235.86 | 5.13 |
|  | Soil retention /Billion CNY | 16.34 | 13.89 | 69.36 | 14.28 | 51.09 | 2.86 | 103.43 | 4.02 | 138.17 | 6.03 | 101.86 | 2.37 | 203.47 | 4.43 |
|  | Sandstorm retention /Million CNY | 1.4 | 0 | 3.17 | 0 | 0.37 | 0 | 0.37 | 0.01 | 0.37 | 0.02 | 0.43 | 0.01 | 0.5 | 0.01 |
|  | Flood mitigation /Billion CNY | 8.17 | 6.94 | 23.1 | 4.75 | 25.9 | 1.45 | 30.61 | 1.19 | 40.85 | 1.78 | 48.62 | 1.13 | 52.28 | 1.14 |
|  | Carbon sequestration /Billion CNY | 4.84 | 4.11 | 179.16 | 36.87 | 1244.47 | 69.75 | 1968.22 | 76.48 | 1423.88 | 62.17 | 3072.46 | 71.5 | 2968.41 | 64.6 |
|  | Oxygen supply /Billion CNY | 26.39 | 22.43 | 59.78 | 12.3 | 181.61 | 10.18 | 299.91 | 11.65 | 347.9 | 15.19 | 443.2 | 10.31 | 682.48 | 14.85 |
|  | Air purification /Million CNY | 34.55 | 0.03 | 34.48 | 0.01 | 166.31 | 0.01 | 453.79 | 17.63 | 543.23 | 23.72 | 844.81 | 19.66 | 1092.26 | 23.77 |
|  | Water purification /Million CNY | 33.41 | 0.03 | 34.09 | 0.01 | 32.4 | 0 | 168.14 | 6.53 | 169.66 | 7.41 | 170.32 | 3.96 | 314.75 | 6.85 |
|  | Climate regulation /Billion CNY | 2.82 | 2.4 | 3.16 | 0.65 | 4.08 | 0.23 | 4.78 | 0.19 | 8.06 | 0.35 | 8.74 | 0.2 | 4.33 | 0.09 |
|  | SUM /Billion CNY | 104.62 | 88.92 | 453.57 | 93.35 | 1743.23 | 97.71 | 2498.05 | 97.07 | 2079.93 | 90.82 | 3807.44 | 88.6 | 4148.23 | 90.28 |
| ECV/Billion CNY |  | 0.28 | 0.24 | 1.67 | 0.34 | 8.13 | 0.46 | 29.2 | 1.13 | 108.34 | 4.73 | 344.75 | 8.02 | 292.01 | 6.36 |
| GEP/Billion CNY |  | 117.65 | - | 485.87 | - | 1784.12 | - | 2573.55 | - | 2290.23 | - | 4297.19 | - | 4594.89 | - |
| GDP/Billion CNY |  | 42.93 | - | 107.60 | - | 184.57 | - | 424.69 | - | 924.08 | - | 1279.34 | - | 1765.19 | - |
| GEP/GDP |  | 2.74 | - | 4.52 | - | 9.67 | - | 6.06 | - | 2.48 | - | 3.36 | - | 2.6 | - |

Table A5. The occupation of different levels of GEP in the study area.

| Year | Level 1/ % | Level 2/ % | Level 3/ % | Level 4/ % | Level 5/ % | Level 6/ % | Level 4 and above/ % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 18.17 | 29.68 | 5.83 | 39.35 | 6.81 | 0.16 | 46.32 |
| 1995 | 10.88 | 26.82 | 20.44 | 3.09 | 38.57 | 0.2 | 41.86 |
| 2000 | 21.23 | 26.84 | 10 | 3.47 | 38.34 | 0.12 | 41.93 |
| 2005 | 28.67 | 21.31 | 9.18 | 2.78 | 37.9 | 0.16 | 40.84 |
| 2010 | 25.12 | 24.95 | 9.72 | 2.45 | 37.6 | 0.16 | 40.21 |
| 2015 | 29.3 | 25.19 | 7.26 | 0.63 | 37.45 | 0.17 | 38.25 |
| 2020 | 21.62 | 27.98 | 12.81 | 2.24 | 35.17 | 0.18 | 38.59 |

Table A6. The occupation of the positive and negative correlation for LISA.

| Year | Percentage of Positive LISA/% | Percentage of Negative LISA /% |
| --- | --- | --- |
| 1990 | 28.72 | 71.28 |
| 1995 | 26.85 | 73.15 |
| 2000 | 29.16 | 70.84 |
| 2005 | 21.66 | 78.34 |
| 2010 | 25.79 | 74.12 |
| 2015 | 27.68 | 72.32 |
| 2020 | 25.02 | 74.98 |

Table A7. Values in the GEP in eleven cities of Shanxi from 1990 to 2020.

| Year |  | Datong /Billion CNY | Shuozhou /Billion CNY | Xinzhou /Billion CNY | Lvliang /Billion CNY | Yangquan /Billion CNY | Taiyuan /Billion CNY | Jinzhong /Billion CNY | Changzhi /Billion CNY | Linfen /Billion CNY | Jinchen /Billion CNY | Yunchen /Billion CNY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | EPV | 0.18 | 1.51 | 1.19 | 1.26 | 0.32 | 0.73 | 1.36 | 1.18 | 1.67 | 0.85 | 2.5 |
|  | ERV | 9.28 | 7.67 | 17.74 | 12.37 | 2.8 | 4.33 | 9.87 | 11.17 | 11.1 | 6.29 | 12 |
|  | ECV | 0.07 | 0 | 0.11 | 0 | 0 | 0.01 | 0 | 0.01 | 0.07 | 0 | 0 |
|  | GEP | 9.53 | 9.18 | 19.04 | 13.63 | 3.12 | 5.07 | 11.23 | 12.36 | 12.84 | 7.14 | 14.5 |
| 1995 | EPV | 1.78 | 2.01 | 2.46 | 3.1 | 0.6 | 1.92 | 3.61 | 3.33 | 3.99 | 1.69 | 6.13 |
|  | ERV | 45.28 | 35.98 | 74.57 | 53.44 | 10.78 | 21.99 | 42.45 | 46.59 | 49.52 | 26.18 | 46.79 |
|  | ECV | 0.23 | 0 | 0.46 | 0.23 | 0 | 0.11 | 0.11 | 0.11 | 0.22 | 0 | 0.2 |
|  | GEP | 47.29 | 37.99 | 77.49 | 56.77 | 11.38 | 24.02 | 46.17 | 50.03 | 53.73 | 27.87 | 53.12 |
| 2000 | EPV | 1.93 | 2.32 | 5.19 | 3.16 | 1.93 | 2.05 | 3.16 | 3.15 | 3.16 | 1.53 | 5.19 |
|  | ERV | 167.08 | 147.6 | 238.15 | 204.83 | 30.34 | 65.77 | 150.21 | 196.62 | 215.28 | 103.97 | 223.38 |
|  | ECV | 1.25 | 0.01 | 1.95 | 1.24 | 0 | 0.34 | 0.35 | 0.69 | 1.19 | 0.01 | 1.1 |
|  | GEP | 170.26 | 149.93 | 245.29 | 209.23 | 32.27 | 68.16 | 153.72 | 200.46 | 219.63 | 105.51 | 229.67 |
| 2005 | EPV | 4.01 | 3.32 | 3.73 | 3.24 | 0.66 | 3.28 | 6.13 | 5.01 | 5.17 | 2.61 | 9.13 |
|  | ERV | 217.4 | 211.65 | 338.94 | 301.64 | 40.49 | 89.59 | 208.92 | 269.2 | 320.55 | 156.17 | 343.5 |
|  | ECV | 5.1 | 0.05 | 6.26 | 5.03 | 0.01 | 0.59 | 0.58 | 1.17 | 5.39 | 0.55 | 4.48 |
|  | GEP | 226.51 | 215.02 | 348.93 | 309.91 | 41.16 | 93.46 | 215.63 | 275.38 | 331.11 | 159.33 | 357.11 |
| 2010 | EPV | 7.01 | 7.57 | 8.65 | 8.15 | 1.36 | 5.89 | 11.62 | 7.22 | 13.02 | 5.66 | 25.81 |
|  | ERV | 173.62 | 167.33 | 279.85 | 248.37 | 35.29 | 73.54 | 179.43 | 229.77 | 274.37 | 134.92 | 283.44 |
|  | ECV | 19.63 | 0.23 | 19.61 | 14.93 | 0.03 | 2.23 | 2.24 | 13.11 | 16.49 | 4.37 | 15.46 |
|  | GEP | 200.26 | 175.13 | 308.11 | 271.45 | 36.68 | 81.66 | 193.29 | 250.1 | 303.88 | 144.95 | 324.71 |
| 2015 | EPV | 10.43 | 11.62 | 11.37 | 9.93 | 1.94 | 7.21 | 17.77 | 10.37 | 16.94 | 9.07 | 38.36 |
|  | ERV | 334.77 | 327.89 | 532.03 | 461.81 | 61.73 | 137.05 | 315.07 | 403.86 | 475.59 | 229.01 | 528.63 |
|  | ECV | 38.99 | 11.43 | 39.44 | 61.82 | 7.99 | 15.86 | 18.36 | 56.76 | 42.88 | 20.61 | 30.6 |
|  | GEP | 384.19 | 350.94 | 582.84 | 533.56 | 71.66 | 160.12 | 351.2 | 470.99 | 535.41 | 258.69 | 597.59 |
| 2020 | EPV | 11.56 | 11.19 | 13.67 | 12.8 | 1.85 | 5.39 | 17.33 | 10.3 | 18.47 | 8.7 | 43.38 |
|  | ERV | 380.24 | 348 | 581.04 | 496.52 | 75.74 | 144.11 | 330.63 | 434.69 | 548.7 | 244.74 | 563.82 |
|  | ECV | 31.25 | 8.44 | 35.49 | 50.53 | 16.06 | 11.67 | 23.71 | 41.66 | 33.97 | 15.13 | 24.1 |
|  | GEP | 423.05 | 367.63 | 630.2 | 559.85 | 93.65 | 161.17 | 371.67 | 486.65 | 601.14 | 268.57 | 631.3 |

Table A8. The occupation of different levels of GEP in eleven cities of Shanxi from 1990 to 2020.

| City Names | 1990 |  |  |  |  |  | 2020 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Level1 | Level2 | Level3 | Level4 | Level5 | Level6 | Level1 | Level2 | Level3 | Level4 | Level5 | Level6 |
| Datong | 4.11 | 25.62 | 28.33 | 13.87 | 27.8 | 0.28 | 14.8 | 32.5 | 12.99 | 13.53 | 25.94 | 0.24 |
| Shuozhou | 2.46 | 27.54 | 16.18 | 12.32 | 41.32 | 0.18 | 26.95 | 18.82 | 4.36 | 12.53 | 36.99 | 0.34 |
| Xinzhou | 1.19 | 15.45 | 44.55 | 15.55 | 23.19 | 0.06 | 26.25 | 27.85 | 11.3 | 12.39 | 22.16 | 0.05 |
| Lvliang | 17.14 | 34.08 | 12.25 | 11.88 | 24.6 | 0.05 | 26.79 | 29.29 | 10.77 | 10.9 | 22.15 | 0.1 |
| Yangquan | 2.73 | 21.12 | 51.68 | 14.55 | 9.92 | 0 | 19.47 | 35.82 | 21.89 | 12.31 | 10.44 | 0.06 |
| Taiyuan | 6.97 | 31.34 | 24.59 | 15.86 | 21.14 | 0.1 | 16.72 | 38.32 | 15.38 | 12.77 | 16.72 | 0.1 |
| Jinzhong | 3.85 | 36.47 | 25.82 | 11.33 | 21.62 | 0.91 | 30.56 | 29.59 | 10.93 | 11.16 | 17.38 | 0.38 |
| Changzhi | 9.11 | 32.45 | 10.32 | 12.46 | 35.17 | 0.5 | 7.22 | 26.39 | 21.87 | 13.23 | 31.05 | 0.24 |
| Linfen | 24.81 | 31.42 | 4.71 | 11.26 | 27.7 | 0.09 | 16.47 | 30.96 | 13.26 | 11.62 | 27.57 | 0.12 |
| Jinchen | 8.5 | 36.49 | 13.65 | 12.82 | 28.51 | 0.02 | 18.01 | 26.23 | 17.19 | 12.75 | 25.74 | 0.07 |
| Yunchen | 27.07 | 10.47 | 2.81 | 10.69 | 48.51 | 0.45 | 22.89 | 12.23 | 7.48 | 11.24 | 45.71 | 0.44 |

Table A9. Changes of each LULC in Shanxi from 1990 to 2020.

|  | Year | Cultivated Land | Forest | Grassland | Water area | Industrial Land | Residential Land | Unused Land |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Area(km2) | 1990 | 61123 | 44145 | 45656 | 1577 | 397 | 3524 | 147 |
|  | 1995 | 61173 | 43993 | 45530 | 1611 | 398 | 3716 | 148 |
|  | 2000 | 61192 | 43867 | 45648 | 1529 | 419 | 3765 | 149 |
|  | 2005 | 60650 | 43863 | 45837 | 1549 | 559 | 3962 | 149 |
|  | 2010 | 60177 | 44104 | 45791 | 1565 | 713 | 4072 | 147 |
|  | 2015 | 59948 | 44067 | 45690 | 1571 | 891 | 4254 | 148 |
|  | 2020 | 57878 | 44489 | 44080 | 1431 | 1945 | 6649 | 101 |
| Ratio (%) | 1990 | 39.04 | 28.2 | 29.16 | 1.01 | 0.25 | 2.25 | 0.09 |
|  | 1995 | 39.07 | 28.1 | 29.08 | 1.03 | 0.25 | 2.37 | 0.09 |
|  | 2000 | 39.08 | 28.02 | 29.16 | 0.98 | 0.27 | 2.4 | 0.1 |
|  | 2005 | 38.74 | 28.02 | 29.28 | 0.99 | 0.36 | 2.53 | 0.1 |
|  | 2010 | 38.43 | 28.17 | 29.25 | 1 | 0.46 | 2.6 | 0.09 |
|  | 2015 | 38.29 | 28.15 | 29.18 | 1 | 0.57 | 2.72 | 0.09 |
|  | 2020 | 36.97 | 28.41 | 28.15 | 0.91 | 1.24 | 4.25 | 0.06 |
| Variations(km2) | 1990-1995 | 50 | -152 | -126 | 34 | 1 | 192 | 1 |
|  | 1995-2000 | 19 | -126 | 118 | -82 | 21 | 49 | 1 |
|  | 2000-2005 | -542 | -4 | 189 | 20 | 140 | 197 | 0 |
|  | 2005-2010 | -473 | 241 | -46 | 16 | 154 | 110 | -2 |
|  | 2010-2015 | -229 | -37 | -101 | 6 | 178 | 182 | 1 |
|  | 2015-2020 | -2070 | 422 | -1610 | -140 | 1054 | 2395 | -47 |
|  | 1990-2020 | -3295 | 496 | -1450 | -180 | 1547 | 2933 | -47 |

Table A10. Transition area matrix of Human Disturbances areas from 1990 to 2020.

|  |  | Cultivated Land | Forest | Grassland | Water Area |
| --- | --- | --- | --- | --- | --- |
| 1990-1995 | Residential Land | 174 | 9 | 20 | 1 |
|  | Industrial Land | 20 | 4 | 18 | 0 |
| 1995-2000 | Residential Land | 45 | 0 | 4 | 2 |
|  | Industrial Land | 19 | 0 | 0 | 0 |
| 2000-2005 | Residential Land | 171 | 6 | 16 | 3 |
|  | Industrial Land | 106 | 17 | 18 | 1 |
| 2005-2010 | Residential Land | 93 | 10 | 4 | 4 |
|  | Industrial Land | 92 | 19 | 42 | 0 |
| 2010-2015 | Residential Land | 147 | 6 | 22 | 4 |
|  | Industrial Land | 94 | 18 | 67 | 2 |
| 2015-2020 | Residential Land | 3530 | 287 | 585 | 101 |
|  | Industrial Land | 821 | 231 | 452 | 58 |

Table A11. Variations of GEP causing by human disturbances in Shanxi Province from 1990 to 2020.

| Category of Ecosystem Services | Human Disturbances | 1990-1995 | 1995-2000 | 2000-2005 | 2005-2010 | 2010-2015 | 2015-2020 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EPV /Million CNY | Residential Land | -44.62 | -36.79 | -123.01 | -85.61 | -277.19 | -8906.02 |
|  | Industrial Land | -6.63 | -10.15 | -85.09 | -92.53 | -182.68 | -2662.78 |
| Water retention /Million CNY | Residential Land | -88.24 | -39.92 | -156.28 | -81.14 | -173.2 | -4810.82 |
|  | Industrial Land | -18.52 | -26.68 | -132.74 | -124.67 | -135.26 | -1872.53 |
| Soil retention /Million CNY | Residential Land | -8.36 | -7.54 | -21.23 | -19.55 | -119.03 | -1428.08 |
|  | Industrial Land | -3.08 | -2.73 | -27.44 | -77.36 | -86.32 | -751.81 |
| Sandstorm retention /103 CNY | Residential Land | 0 | 0 | 0 | 0 | 0 | -0.03 |
|  | Industrial Land | 0 | 0 | 0 | 0 | 0 | -0.01 |
| Flood mitigation /Million CNY | Residential Land | 0 | -136.82 | 0 | 0 | -362.32 | -4209.41 |
|  | Industrial Land | 0 | 0 | 0 | 0 | 0 | -1530.69 |
| Carbon sequestration /Million CNY | Residential Land | -22.77 | -210.47 | -5991.48 | -4750.69 | -4913.21 | 235037.9 |
|  | Industrial Land | -2.62 | -88.87 | -3715.17 | -4650.44 | -3143.87 | -54771.63 |
| Oxygen supply /Million CNY | Residential Land | -124.12 | -70.23 | -874.36 | -723.89 | -1200.44 | -33889.71 |
|  | Industrial Land | -14.29 | -29.65 | -542.17 | -708.61 | -768.14 | -7898.7 |
| Air purification /103 CNY | Residential Land | -0.02 | 0 | -0.03 | -0.19 | -0.13 | -8.16 |
|  | Industrial Land | 0 | 0 | -0.07 | -0.38 | -0.34 | -6.37 |
| Water purification /103 CNY | Residential Land | -0.03 | 0 | -0.1 | -0.69 | -0.47 | -13.17 |
|  | Industrial Land | 0 | -0.06 | -0.03 | 0 | -0.31 | -8.22 |
| Climate regulation /Million CNY | Residential Land | -0.13 | -0.03 | -0.19 | -0.09 | -0.41 | -12.05 |
|  | Industrial Land | -0.07 | 0 | -0.22 | -0.39 | -0.41 | -47.22 |

References

Chen, Y., Xia, X., Yang, L., Liu, B., Zhang, M., and Nie, X. (2020). Straw return is the realistic way of straw resource utilization. Soil and Fertilizer Sciences in China.299-307

Food, A. O. o. t. U. N., and International Institute for Applied Systems, A. (2019). China soil map based harmonized world soil database (HWSD) (v1.1) (2009).C. National Tibetan Plateau Data.Available online at:

Jia, J., Luo, W., Du, T., Li, Z., and Lv, Y. (2015). Valuation of changes of ecosystem services of Tai Lake in recent 10 years. Acta Ecologica Sinica. 35:2255-2264

Li, G., Zhao, W., Wei, Y., Fang, X., Gao, B., and Dai, L. (2015). Evaluation on the influence of natural forest protection program on forest ecosystem service function in changbai mountain. Acta Ecologica Sinica. 35:984-992

Liyun, D., and Tao, C. (2015). Long-term series of daily snow depth dataset in China (1979-2020).C. National Tibetan Plateau Data.Available online at:https://dx.doi.org/10.11888/Geogra.tpdc.270194.

Song, Z., Yang, H., and Zhang, Q. (1999). Spatiotemporal variation characteristics of water resources components in North China. Plateau Meteorolog.552-566

The National Center for Atmospheric, R. (2018). Global GIMMS NDVI3g v1 dataset (1981-2015).C. National Tibetan Plateau Data.Available online at:

Wang, L., Xiao, Y., OuYang, Z., Wei, Q., Bo, W., Zhang, J., and Ren, L. (2017). Gross ecosystem product accounting in the national key ecological function area: an example of Arxan. China Population,Resources and Environment. 27:146-154

Xu, X. (2018). Spatial distribution dataset of annual vegetation index (NDVI) in China.doi:10.12078/2018060601.

Zhang, W., Xie, Y., and Liu, B. (2002). Rainfall Erosivity Estimation Using Daily Rainfall Amounts. Scientia Geographica Sinica.705-711