Supplementary Material

SM1 - The BCR-701 sequential extraction for trace metals.

| Steps | Extractable phase | Extractable components | Reagents | Extraction conditions |  |
| --- | --- | --- | --- | --- | --- |
| Step 1 | Exchangeable and carbonates fraction | Exchangeable and carbonates fraction | 0.11 M CH3COOH, 40 mL | Shake for 16 h at 90 rpm at 22 ± 5ºC |  |
| Step 2 | Fe and Mn oxides fraction | Fe and Mn oxides fraction | 0.5 M NH2OH·HCl, 40 mL (adjusted to pH 2 with 2 M HNO3) | Shake for 16 h at 90 rpm at 22 ± 5ºC |  |
| Step 3 | Organic matter and sulfides fraction | Organic matter and sulfides fraction | 30% H2O2, (2 times) 10 mL, 1.0 M CH3COONH4, 50 mL (adjusted to pH 2 with HNO3) | Digested using H2O2 until almost dry at 85 ± 5ºC, shaken for 16 h at 90 rpm at 22 ± 5ºC using CH3COONH4 |  |
| Step 4a | Residual | The fraction bound to mineral crystals | HCl: HNO3 = 3:1 (inverse aqua regia) | Digested using microwave - the temperature rises to 175 ± 5 C in approximately 5.5 ± 0.25 min and remain at 175 ± 5 C° for 4.5 min, or the remainder of the 10-min digestion period. |  |

a - The procedure adopted for step 4 is the EPA 3051a.

SM2 – BCR reference material recovery.

| Fraction | Cd | Cr | Cu | Ni | Pb | Zn |
| --- | --- | --- | --- | --- | --- | --- |
| F1 | 86.58 | 97.02 | 100 | 89.77 | 83.4 | 93.64 |
| F2 | 84.46 | 69.23 | 85.89 | 95 | 90.76 | 100,46 |
| F3 | 81.48 | 83.7 | 86.28 | 113.67 | 111.14 | 82.6 |
| F4 | 113.43 | 115.95 | 89.3 | 85.84 | 96.27 | 95.09 |

SM3a: Raw data of the BCR-701 Sequential Extraction.

| Samples | Exchangeable Geochemical Fraction - Elements (mg Kg-1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fe | Al | Cd | Cr | Cu | Ni | Pb | Zn |
| S-1 | 433.72 | 195.41 | 1.10 | 0.24 | 0.27 | 0.92 | 0.58 | 75.51 |
| S-2 | 718.18 | 272.77 | 0.73 | 0.39 | 0.75 | 0.40 | 0.24 | 68.76 |
| S-3 | 677.91 | 170.11 | 0.82 | 0.26 | 0.51 | 0.59 | 0.11 | 99.77 |
| S-4 | 931.38 | 224.56 | 0.87 | 0.49 | 0.53 | 0.58 | 0.70 | 108.16 |
| S-5 | 1115.71 | 385.70 | 1.20 | 0.76 | 0.94 | 0.82 | 0.25 | 157.33 |
| S-6 | 768.95 | 260.89 | 1.53 | 0.71 | 1.11 | 0.84 | 0.08 | 156.29 |
| S-7 | 361.68 | 214.93 | 1.88 | 0.47 | 0.97 | 0.82 | 0.64 | 164.90 |
| S-8 | 493.58 | 310.05 | 1.74 | 0.44 | 0.43 | 0.80 | 0.10 | 161.98 |
| S-9 | 301.77 | 95.98 | 0.45 | <LD | <LD | 0.80 | 0.20 | 35.22 |
| S-10 | 927.36 | 270.05 | 1.40 | 0.65 | 1.05 | 1.11 | 1.40 | 135.72 |
| S-11 | 780.16 | 192.57 | 2.94 | 1.03 | 1.20 | 1.29 | 1.02 | 369.17 |
| S-12 | 616.50 | 185.96 | 2.54 | 0.56 | 1.03 | 0.88 | 0.48 | 259.64 |
| S-13 | 959.60 | 170.63 | 2.21 | 0.66 | 1.42 | 1.54 | 0.92 | 269.83 |
| S-14 | 743.70 | 520.62 | 1.00 | 0.49 | 1.06 | 0.74 | <LD | 95.70 |
| S-15 | 883.19 | 234.98 | 0.67 | 0.42 | 0.78 | 0.59 | 0.34 | 89.10 |
| S-16 | 767.45 | 202.89 | 1.06 | 0.68 | 0.72 | 0.56 | 0.26 | 119.22 |
| S-17 | 845.78 | 360.23 | 1.38 | 0.67 | 1.04 | 0.85 | 0.16 | 166.92 |
| S-18 | 617.93 | 250.92 | 1.72 | 0.48 | 0.54 | 1.05 | 0.43 | 156.15 |
| Medium | 719.14 | 251.07 | 1.40 | 0.55 | 0.84 | 0.84 | 0.46 | 149.41 |
| Standard Deviation | 218.60 | 96.87 | 0.67 | 0.15 | 0.26 | 0.28 | 0.29 | 81.38 |
| Maximum | 1115.71 | 520.62 | 2.94 | 1.03 | 1.42 | 1.54 | 1.40 | 369.17 |
| Minimum | 301.77 | 95.98 | 0.45 | 0.24 | 0.27 | 0.40 | 0.08 | 35.22 |

SM3b: Raw data of the BCR-701 Sequential Extraction.

| Samples | Reducible Geochemical Fraction - Elements (mg Kg-1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fe | Al | Cd | Cr | Cu | Ni | Pb | Zn |
| S-1 | 5798.50 | 2030.59 | 0.17 | 3.02 | 2.96 | 0.78 | 8.37 | 42.84 |
| S-2 | 4277.03 | 593.10 | 0.09 | 1.55 | 1.31 | 0.21 | 4.07 | 20.10 |
| S-3 | 12071.94 | 1799.55 | 0.36 | 3.57 | 2.08 | 0.63 | 7.60 | 90.42 |
| S-4 | 11312.97 | 1448.40 | 0.34 | 3.01 | 1.63 | 0.67 | 6.76 | 66.09 |
| S-5 | 11845.99 | 2342.10 | 0.29 | 4.14 | 2.83 | 0.63 | 9.77 | 80.68 |
| S-6 | 11342.16 | 2118.22 | 0.32 | 5.35 | 4.01 | 0.74 | 12.63 | 60.94 |
| S-7 | 6991.97 | 2262.30 | 0.26 | 4.89 | 5.10 | 0.74 | 12.69 | 72.11 |
| S-8 | 5835.37 | 3266.75 | 0.25 | 5.15 | 3.63 | 0.69 | 11.63 | 77.12 |
| S-9 | 4588.94 | 1654.94 | 0.28 | 5.00 | 3.72 | 0.65 | 11.65 | 57.12 |
| S-10 | 10552.70 | 2365.54 | 0.13 | 2.28 | 1.12 | 0.64 | 6.37 | 29.67 |
| S-11 | 11671.90 | 2454.71 | 0.29 | 6.22 | 4.71 | 0.95 | 15.34 | 65.37 |
| S-12 | 7783.03 | 2145.99 | 0.38 | 8.76 | 5.84 | 0.93 | 33.23 | 137.83 |
| S-13 | 8229.34 | 1680.20 | 0.34 | 6.25 | 5.13 | 0.78 | 17.27 | 98.36 |
| S-14 | 8603.24 | 2152.58 | 0.35 | 5.40 | 4.78 | 1.00 | 20.86 | 87.84 |
| S-15 | 7591.51 | 773.10 | 0.18 | 3.25 | 2.42 | 0.46 | 7.47 | 36.29 |
| S-16 | 7674.01 | 1299.12 | 0.17 | 2.21 | 0.93 | 0.35 | 4.73 | 36.24 |
| S-17 | 13229.84 | 2998.80 | 0.28 | 3.55 | 2.45 | 0.52 | 8.94 | 51.96 |
| S-18 | 6815.00 | 2900.53 | 0.41 | 5.04 | 3.02 | 0.72 | 12.15 | 68.92 |
| Medium | 8678.64 | 2015.92 | 0.27 | 4.37 | 3.21 | 0.67 | 11.75 | 65.55 |
| Standard Deviation | 2773.44 | 708.60 | 0.09 | 1.41 | 1.25 | 0.20 | 4.66 | 28.38 |
| Maximum | 13229.84 | 3266.75 | 0.41 | 8.76 | 5.84 | 1.00 | 33.23 | 137.83 |
| Minimum | 4277.03 | 593.10 | 0.09 | 1.55 | 0.93 | 0.21 | 4.07 | 20.10 |

SM3c: Raw data of the BCR-701 Sequential Extraction.

| Samples | Oxidizable Geochemical Fraction - Elements (mg Kg-1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fe | Al | Cd | Cr | Cu | Ni | Pb | Zn |
| S-1 | 3018.56 | 2030.59 | 0.17 | 15.46 | 3.48 | 2.35 | 3.03 | 9.45 |
| S-2 | 2991.91 | 593.10 | 0.09 | 6.80 | 1.63 | 2.29 | 1.45 | 7.75 |
| S-3 | 6429.61 | 1799.55 | 0.36 | 12.56 | 2.66 | 4.91 | 3.05 | 20.00 |
| S-4 | 6158.33 | 1448.40 | 0.34 | 10.97 | 2.50 | 4.68 | 1.68 | 18.89 |
| S-5 | 6164.60 | 2342.10 | 0.29 | 13.20 | 3.42 | 4.06 | 2.25 | 19.22 |
| S-6 | 6826.12 | 2118.22 | 0.32 | 15.10 | 4.06 | 3.92 | 1.46 | 18.97 |
| S-7 | 3326.38 | 2262.30 | 0.26 | 17.55 | 2.82 | 1.70 | 1.70 | 9.00 |
| S-8 | 5682.95 | 3266.75 | 0.25 | 9.92 | 3.12 | 2.82 | 1.93 | 11.48 |
| S-9 | 4915.07 | 1654.94 | 0.28 | 24.51 | 3.36 | 3.07 | 2.27 | 10.70 |
| S-10 | 6681.47 | 2365.54 | 0.13 | 18.07 | 4.44 | 4.06 | 2.15 | 16.66 |
| S-11 | 6329.65 | 2454.71 | 0.29 | 20.78 | 4.80 | 3.44 | 3.56 | 30.51 |
| S-12 | 3556.44 | 2145.99 | 0.38 | 17.21 | 2.45 | 1.85 | 2.13 | 11.74 |
| S-13 | 7378.60 | 1680.20 | 0.34 | 16.11 | 4.25 | 4.34 | 2.54 | 19.86 |
| S-14 | 3831.84 | 2152.58 | 0.35 | 10.81 | 2.61 | 3.24 | 1.59 | 10.75 |
| S-15 | 6517.30 | 773.10 | 0.18 | 8.79 | 1.16 | 2.89 | 0.88 | 9.63 |
| S-16 | 7611.25 | 1299.12 | 0.17 | 12.11 | 2.10 | 3.83 | 1.55 | 14.61 |
| S-17 | 3906.52 | 2998.80 | 0.28 | 18.15 | 3.55 | 3.92 | 2.41 | 18.79 |
| S-18 | 4251.57 | 2900.53 | 0.41 | 10.42 | 1.56 | 1.33 | 0.70 | 5.24 |
| Medium | 5309.90 | 2015.92 | 0.27 | 14.36 | 3.00 | 3.26 | 2.02 | 14.62 |
| Standard Deviation | 1574.79 | 708.60 | 0.09 | 3.74 | 0.83 | 1.05 | 0.58 | 6.21 |
| Maximum | 7611.25 | 3266.75 | 0.41 | 24.51 | 4.80 | 4.91 | 3.56 | 30.51 |
| Minimum | 2991.91 | 593.10 | 0.09 | 6.80 | 1.16 | 1.33 | 0.70 | 5.24 |

SM3d: Raw data of the BCR-701 Sequential Extraction.

| Samples | Residual Geochemical Fraction - Elements (mg Kg-1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fe | Al | Cd | Cr | Cu | Ni | Pb | Zn |
| S-1 | 11707.16 | 54781.23 | 0.18 | 61.82 | 9.00 | 10.81 | 13.17 | 49.41 |
| S-2 | 6210.32 | 12749.86 | 0.11 | 13.60 | 4.66 | 5.98 | 2.89 | 29.31 |
| S-3 | 12644.58 | 19857.00 | 0.25 | 25.13 | 10.15 | 11.37 | 4.56 | 62.55 |
| S-4 | 10996.60 | 44879.26 | 0.21 | 21.93 | 8.64 | 9.78 | 4.91 | 54.77 |
| S-5 | 13240.21 | 20902.76 | 0.28 | 26.40 | 11.42 | 11.45 | 6.23 | 66.48 |
| S-6 | 13908.01 | 29289.00 | 0.27 | 30.21 | 13.25 | 13.09 | 7.38 | 69.40 |
| S-7 | 13000.15 | 75321.56 | 0.25 | 35.09 | 10.05 | 10.95 | 25.92 | 64.09 |
| S-8 | 9044.60 | 20414.41 | 0.18 | 19.85 | 6.01 | 7.07 | 8.68 | 40.47 |
| S-9 | 16945.29 | 65007.37 | 0.36 | 49.02 | 10.14 | 17.19 | 18.09 | 80.24 |
| S-10 | 17148.21 | 92056.78 | 0.37 | 36.15 | 17.09 | 17.69 | 7.03 | 83.39 |
| S-11 | 18776.30 | 61067.96 | 0.44 | 41.56 | 21.55 | 22.09 | 12.24 | 139.91 |
| S-12 | 14015.80 | 53427.51 | 0.27 | 34.43 | 11.76 | 14.89 | 15.20 | 76.39 |
| S-13 | 16134.82 | 77893.21 | 0.39 | 32.22 | 16.60 | 17.00 | 5.82 | 83.86 |
| S-14 | 10924.87 | 26993.73 | 0.20 | 21.61 | 9.82 | 10.56 | 4.46 | 49.84 |
| S-15 | 8182.28 | 18429.06 | 0.17 | 17.58 | 6.59 | 8.40 | 3.15 | 41.02 |
| S-16 | 12333.20 | 30806.53 | 0.23 | 24.22 | 10.20 | 11.48 | 5.68 | 54.09 |
| S-17 | 17137.84 | 46876.56 | 0.39 | 36.29 | 16.07 | 17.51 | 8.78 | 88.03 |
| S-18 | 9634.52 | 33106.28 | 0.18 | 20.83 | 8.34 | 9.79 | 7.94 | 43.41 |
| Medium | 12888.04 | 43547.78 | 0.26 | 30.44 | 11.19 | 12.62 | 9.01 | 65.37 |
| Standard Deviation | 3443.83 | 23619.29 | 0.09 | 9.23 | 3.27 | 4.26 | 4.40 | 25.25 |
| Maximum | 18776.30 | 92056.78 | 0.44 | 61.82 | 21.55 | 22.09 | 25.92 | 139.91 |
| Minimum | 6210.32 | 12749.86 | 0.11 | 13.60 | 4.66 | 5.98 | 2.89 | 29.31 |

SM4 – Igeo calculation values

| Sample Stations | Cd | Zn | Ni | Pb | Cr | Cu |
| --- | --- | --- | --- | --- | --- | --- |
| S-1 | 0,45 | 0,23 | -1,80 | -1,30 | -1,58 | -1,27 |
| S-2 | -0,19 | -0,21 | -2,61 | -2,53 | -2,71 | -2,15 |
| S-3 | 0,77 | 0,91 | -1,64 | -1,66 | -1,82 | -1,47 |
| S-4 | 0,70 | 0,78 | -1,80 | -1,86 | -2,00 | -1,67 |
| S-5 | 0,91 | 1,18 | -1,66 | -1,44 | -1,70 | -1,10 |
| S-6 | 1,07 | 1,08 | -1,52 | -1,23 | -1,49 | -0,79 |
| S-7 | 1,03 | 1,12 | -1,85 | -0,80 | -1,33 | -0,92 |
| S-8 | 0,95 | 1,10 | -2,20 | -1,24 | -2,00 | -1,32 |
| S-9 | 0,63 | 0,08 | -1,28 | -1,01 | -0,92 | -1,17 |
| S-10 | 0,90 | 0,77 | -1,17 | -1,68 | -1,38 | -0,99 |
| S-11 | 1,66 | 2,06 | -0,91 | -0,71 | -1,06 | -0,42 |
| S-12 | 1,42 | 1,82 | -1,47 | 0,08 | -1,22 | -0,82 |
| S-13 | 1,47 | 1,76 | -1,15 | -0,76 | -1,38 | -0,53 |
| S-14 | 1,09 | 0,78 | -1,76 | -0,67 | -1,89 | -0,99 |
| S-15 | 0,30 | 0,27 | -2,12 | -1,98 | -2,26 | -1,85 |
| S-16 | 0,62 | 0,61 | -1,74 | -2,23 | -1,91 | -1,78 |
| S-17 | 1,14 | 1,12 | -1,23 | -1,45 | -1,33 | -0,97 |
| S-18 | 1,06 | 0,99 | -1,97 | -1,30 | -1,95 | -1,59 |
| Medium | 0,89 | 0,91 | -1,66 | -1,32 | -1,66 | -1,21 |
| Standard Deviation | 0,33 | 0,44 | 0,32 | 0,47 | 0,36 | 0,38 |
| Maximum | 1,66 | 2,06 | -0,91 | 0,08 | -0,92 | -0,42 |
| Minimum | -0,19 | -0,21 | -2,61 | -2,53 | -2,71 | -2,15 |

SM5 – EF calculation values

| Sample Stations | Cd | Zn | Pb | Cu | Cr | Ni |
| --- | --- | --- | --- | --- | --- | --- |
| S-1 | 3,34 | 2,87 | 1,00 | 0,97 | 0,29 | 0,66 |
| S-2 | 7,44 | 7,33 | 1,46 | 1,83 | 1,22 | 1,30 |
| S-3 | 8,60 | 9,47 | 1,60 | 1,74 | 1,35 | 1,51 |
| S-4 | 8,52 | 8,98 | 1,45 | 1,58 | 1,23 | 1,41 |
| S-5 | 8,70 | 10,50 | 1,70 | 2,06 | 1,34 | 1,37 |
| S-6 | 7,46 | 7,51 | 1,52 | 1,98 | 1,20 | 1,17 |
| S-7 | 5,17 | 5,51 | 1,45 | 1,28 | 0,95 | 0,66 |
| S-8 | 9,20 | 10,23 | 2,01 | 1,84 | 1,12 | 0,97 |
| S-9 | 2,77 | 1,89 | 0,89 | 0,77 | 0,90 | 0,69 |
| S-10 | 4,03 | 3,69 | 0,67 | 1,05 | 0,78 | 0,90 |
| S-11 | 5,88 | 7,74 | 1,14 | 1,33 | 0,84 | 0,93 |
| S-12 | 5,80 | 7,66 | 2,29 | 1,18 | 0,88 | 0,73 |
| S-13 | 6,31 | 7,71 | 1,35 | 1,51 | 0,83 | 0,96 |
| S-14 | 8,45 | 6,83 | 2,50 | 1,93 | 1,01 | 1,10 |
| S-15 | 6,77 | 6,64 | 1,39 | 1,47 | 1,08 | 1,19 |
| S-16 | 5,31 | 5,30 | 0,74 | 0,97 | 0,87 | 0,97 |
| S-17 | 5,33 | 5,25 | 0,88 | 1,19 | 0,91 | 0,97 |
| S-18 | 6,99 | 6,69 | 1,37 | 1,07 | 0,82 | 0,80 |
| Medium | 6,45 | 6,77 | 1,41 | 1,43 | 0,98 | 1,02 |
| Standard Deviation | 1,57 | 1,81 | 0,37 | 0,34 | 0,19 | 0,22 |
| Maximum | 9,20 | 10,50 | 2,50 | 2,06 | 1,35 | 1,51 |
| Minimum | 2,77 | 1,89 | 0,67 | 0,77 | 0,29 | 0,66 |