Table 3:  Personal Exposure to Pollutants during Commutes in Asian Cities

| Countries | Parameters | Equipment/ Monitoring Strategy | N | Commute Description | Subway (Platforms and in-cabin) | Car/Taxi | Bus (Bus station and in-cabin) | Walking | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Exposure Study |  |  |  |  |  |  |  |  |  |
| Guangzhou, China | PM10 | ﻿DustTrak (TSI Model 8520) portable aerosol monitors | 5 consecutive weekdays 20 trips | Subway Air-Conditioned Bus Non-air-conditioned Bus | PM10 – 67.0 | - | Non-AC bus PM10 - 203 AC bus PM10 - 128 | - | (Chan et al. 2002b) |
| ﻿Taipei, Taiwan | PM10, PM2.5, PM1 | ﻿DUST-check Portable Dust Monitors (Model 1.108, Grimm Labortechnik Ltd., Aining, Germany) | 16 Trips | -- | PM10 – 46.5 PM2.5 – 28.9 PM1 – 22.5 | PM10 - 41.7 PM2.5 – 22.2 PM1 – 16.2 | PM10 – 69.6 PM2.5 – 38.5 PM1 – 31.3 | - | (Tsai et al. 2008) |
| Taipei, Taiwan | PM10 and PM2.5 | ﻿Grimm Series 1.108 OPC (Grimm Tech., Inc., Douglasville, GA, USA) ﻿subject carrying monitors in a backpack | 80 times on each route | Underground and Ground level train | Underground PM10 – 37 PM2.5 - 28 Ground level PM10 – 25 PM2.5 - 19 | - |  | - | (Cheng et al. 2012) |
| Taipei, Taiwan | PM2.5 | ﻿personal dust monitor (DUST-check portable dust monitor, model 1.108 | ﻿360 1-hour measurements | Electric powered subway Gas powered bus Gasoline powered car Walking | Electric powered subway PM10 – 31.5 PM2.5 – 22.3 | Gasoline powered car PM10 – 34.3 PM2.5 – 29.2 | Gas powered bus PM10 – 39.6 PM2.5 – 32.2 | Walking PM10 – 50.2 PM2.5 – 42.1 | (Liu et al. 2015) |
| Taipei, Taiwan | PM10 and PM2.5 | ﻿(Model 8533, DustTrak™ DRX Aerosol for personal exposure monitoring ﻿Air pollution monitoring data (from Taiwan EPA’s air quality monitoring stations for estimating exposure in different transport modes | - | - | - | - | - | 133.4 | (Li et al. 2017a) |
| Foshan, China | PM2.5 | ﻿portable real-time PM monitors (MicroDust pro, Casella CEL, Bedford, UK) | 20 days | Metro Diesel Powered Bus Taxi | PM2.5 – 27.9 | PM2.5 – 56.8 | PM2.5 – 75.9 | PM2.5 – 74.1 | (Wu et al. 2013) |
| Hong Kong, China | PM10 | Dust Trak 8520 (TSI) | - | Subway Railway Car/Taxi Bus/minibus | Subway PM10 – 78.0 Railway PM10 – 95.0 | PM10– 120.0 | PM10 – 137.5 | - | (Chau et al. 2002) |
| Hong Kong, China | PM10 and PM2.5 | ﻿DustTrak (TSI Model 8520) portable aerosol monitor | - | Rail Transport Non-air-conditioned bus Air-conditioned bus Non-air-conditioned public light bus Air-conditioned public light bus | Rail Transport PM10 – 50 PM2.5 - 39 | Air-conditioned taxi PM10 - 58 | Non-air-conditioned bus PM10 – 112 PM2.5 - 93 Air-conditioned bus PM10 – 74 PM2.5 - 51 Non-air-conditioned public light bus PM10 – 137 PM2.5 - 97 Air-conditioned public light bus PM10 – 63 PM2.5 – 45 | - | (Chan et al. 2002a) |
| Hong Kong, China | PM10 | ﻿DustTrak, TSI, USA | 52 trips | - | - | - | Bus Euro II PM10 – 182.0 Bus Euro III PM10 – 240.0 Bus Euro IV PM10 – 70.0 | - | (Wong et al. 2011) |
| Hong Kong, China | PM2.5 | ﻿Optical Particle Sizer (OPS, TSI® model 3330) was used for PM2.5 concentration measurement | 45 weeks, 119 trips | ﻿rail-based MTR 5 Diesel franchised buses and 5 LPG public light buses | PM2.5 – 10.2 | - | Diesel franchised buses. PM2.5 – 8.2 LPG public light buses PM2.5 – 11.7 | - | (Yang et al. 2015) |
| Hong Kong, China | PM2.5 | ﻿Portable light-scattering laser photometer, TSI 155 DustTrak II Aerosol Monitor 8530 | ﻿Four weeks | Train Double decker bus Minibus Train Station Bus stop Walking | Train PM2.5 – 23 Train Station PM2.5 - 28 | - | Double decker bus PM2.5 – 30.0 Minibus PM2.5 – 27.0 Bus stop PM2.5 – 40.0 | 43.0 | (Che et al. 2016) |
| Hong Kong, China(b) | PM2.5 | ﻿DustTrak monitor | - | ﻿Underground metro train ﻿Ground-level metro train ﻿Metro station ﻿Minibus ﻿Double-decker bus; | Underground metro train PM2.5 – 31 ﻿Ground-level metro train PM2.5 - 47 ﻿Metro station PM2.5 - 45 | - | Minibus PM2.5 - 39 Double-decker bus PM2.5 – 42 Bus Stop PM2.5 - 47 | - | (Li et al. 2017b) |
| Beijing, China | PM10, PM2.5 and PM1 | ﻿Dustmate fume and dust detector | - | - | PM10 – 108.0 PM2.5 – 36.9 PM1 – 14.7 | - | - | - | (Li et al. 2006) |
| Beijing, China | PM2.5 | ﻿Portable aerosol spectrometer (model LD-6S, Beijing Green Technology Digital Co., Ltd, China) A seat was chosen at random, and instrument was carried in a bag pack by the experimentalist | 6 days sampling period with 45 trips in total | Diesel powered buses | - | PM2.5 – 31.6 | PM2.5 – 42.4 | - | (Huang et al. 2012) |
| Beijing, China | PM2.5 | ﻿DustTrak Photometer (Model 8520, TSI Inc. St. Paul, MN, 1.7 L min-1) | 5 trips for subway 17 trips for bus 39 trips for walking | ﻿air-conditioned (AC) and non-air-conditioned (non-AC) buses | 56.9 | - | 32.9 | 26.7 | (Yan et al. 2015) |
| Xi’an, China | PM10, PM2.5 and PM1 | ﻿Portable aerosol Monitor (Grimm spec- tonometer, Model 1.109) Portable and included in a shoulder bag | 1 day | Electric powered train CNG bus with two ventilation modes AC and Naturally Ventilated Gasoline powered car with different ventilation type namely WC+VC, WC+AC Rec, WC+AC FA and W1/2O + VC | Station Platform PM10 – 245.0 PM2.5 – 111.0 PM1 – 31.0 In-cabin PM10 -74.7 PM2.5 – 43.21 PM1 – 18.1 | WC+VC PM10 – 44.9 PM2.5 – 30.0 PM1 – 24.5 WC+AC Rec PM10 – 11.8 PM2.5 – 10.1 PM1 – 9.5 WC+AC FA PM10 – 23.1 PM2.5 – 22.1 PM1 – 21.6 W1/2O + VC PM10 – 105.5 PM2.5 – 67.5 PM1 – 51.9 | AC Bus PM10 – 122.7 PM2.5 – 54.4 PM1 – 33.6 Naturally Ventilated Bus PM10 – 123.6 PM2.5 – 58.6 PM1 – 40.7 | PM10 – 127.3 PM2.5 – 71.6 PM1 – 57.01 | (Qiu et al. 2017)(a) |
| Xi’an, China | PM10, PM2.5 and PM1 | Portable optical article size spectrometer (GRIMM 11-A) Determination of PM exposure and exposure dose estimation in two different route of Xi’an, China. | 21 trips 10 trips for the CNG bus and 27 trips for the PE bus 32 Trips for walking | Two subway Lines (Line 2 and Line 3) CNG-powered Bus and PE-Powered Bus Walking along the roadside | PM10 – 68.0 PM2.5 – 46.3 PM1 – 35.6 | - | CNG bus PM10 – 130.0 PM2.5 – 58.0 PM1 – 39.3 PE bus PM10 – 24.3 PM2.5 – 16.7 PM1 – 15.0 | PM10 – 149.4 PM2.5 – 69.8 PM1 – 53.7 | (Qiu and Cao 2020) |
| Shanghai, China | PM1 | ﻿DustTrakÔ dust monitors (Model 8530 and Model 8533) |  | ﻿Diesel buses gasoline taxi | 122.0 | 139.0 | 147.0 | 145 | (Yu et al. 2012) |
| Shanghai, China | PM2.5 | ﻿Dustraks (Model 8532, TSI Inc., Shoreview, MN, USA) | 2882 outdoor samples and 17296 indoor samples with samples collected at 3 min and 1 second’s interval respectively. | Old and new metro line Underground and aboveground lines | PM2.5 - 58 | - | - | - | (Gong et al. 2019) |
| Shanghai, China | PM10, PM2.5 and PM1 | ﻿Optical real-time PM monitor | 24 field campaign | Subway High speed train Bus Intercity Bus | Subway PM10 – 5.8 PM2.5 – 5.4 PM1 – 4.3 High-Speed Train PM10 – 9.0 PM2.5 – 8.1 PM1 – 6.0 | - | Bus PM10 – 21.2 PM2.5 – 17.6 PM1 – 11.8 Intercity Bus PM10 – 9.1 PM2.5 – 8.2 PM1 – 6.0 | - | (Lin et al. 2022) |
| Nanjing, China | PM2.5 and PM1 | ﻿TSI DustTrak II Aerosol Monitors (Model 8532, TSI Inc., USA) | 30 sets of data during Subway trips ﻿ 16 sets sampled in subway station. 28 set of data for walking trips | Subway cabin and station | Subway cabin PM2.5 – 54.4 PM1 – 38.3 Subway Station PM2.5 – 90.5 PM1 – 54.8 | - | PM2.5 – 74.7 PM1 – 56.0 | PM2.5 – 80.3 PM1 – 59.5 | (Shen and Gao 2019) |
| Nanjing, China | PM10, PM2.5, PM1 | ﻿Portable air quality monitor (BoHu model BH1-B3, China) |  | Train 1(Short haul) Train 2(Long haul) Subway Bus (WC+AC) Car (WC+AC) | Train 1 PM2.5 – 43.8 Train 2 PM2.5 – 20.2 Subway PM2.5 – 97.7 | PM2.5 – 166.7 | PM2.5 – 118.9 | PM2.5 – 141.1 | (Zhang et al. 2021) |
| Zhengzhou, China | PM2.5 and PM10 | ﻿Plant Tower PMS1003 sensors | 7 trips over 4 days 34 trips by bus over 17 days | Subway Bus Taxi | PM10 – 68 PM2.5 - 25 | PM10 – 50 PM2.5 - 20 | PM10 – 61 PM2.5 - 2711 |  | (DeSouza et al. 2021) |
| Singapore | PM2.5 | ﻿DustTrak Aerosol Monitor (TSI 8534) | 45 | Train commute including time spent at the mall, platforms, and side walking. Bus trip including time spent at the bus stop, mall, underpass, and sidewalk | PM2.5 – 26.0 | PM2.5 - 4.0 | PM2.5 – 28.0 | PM2.5 – 36.0 | (Tan et al. 2017) |
| Singapore | PM2.5 | ﻿portable nephelometers (SidePak AM520, TSI, USA) Personal monitors carried by two different individuals namely office staff (OS) and university student (US) | 30 days | Walking Bus Taxi/car MRT train | US PM2.5 – 20.0 | US PM2.5 – 10.8 OS PM2.5 – 14.3 | US PM2.5 – 13.8 OS PM2.5 – 13.7 | OS PM2.5 –21.3 | (Tran et al. 2021) |
| Jakarta, Indonesia | PM2.5 | DustTrak 8520, TSI Inc., Shoreview, MN), fitted with a PM2.5 inlet | 25 days | Private cars | - | 91.0 | - | - | (Both et al. 2013) |
| Hanoi, Vietnam | PM10 | Nephelometer, ﻿Thermo Inc., model PDR -1000 | - | - | - | PM10 – 408.0 | PM10 – 262.0 | PM10 – 495.0 | (Saksena et al. 2008) |
| Bangkok, Thailand | PM2.5 | ﻿PTFE filters with PMP support ring (Model 225- 1709, SKC Inc. USA) with a single-stage personal impactor (Personal Environmental Monitor, Model 761-203A, SKC Inc. USA) | 4 routes with 80 trips in total | Air-conditioned bus Non-air-conditioned bus | - | - | Air-conditioned bus PM2.5 – 210.0 Non-air-conditioned bus PM2.5 – 370.0 |  | (Kongtip et al. 2012) |
| New Delhi, India | PM2.5 | ﻿portable scattering nephelometer (pDR-1500, Thermo Fisher, Franklin, MA, USA) equipped with a PM2.5 cyclone |  |  |  |  | PM2.5 – 53.9 |  | (Pant et al. 2017) |
| New Delhi, India | PM2.5 | ﻿portable TSI 2 DustTrak DRX 8433 aerosol monitor | 150 trips | Open Window car Air-conditioned car Open window bus Air-conditioned bus Bus Stop Metro | PM2.5 – 87.0 | Open Window car PM2.5 – 180.0 Air-conditioned car PM2.5 – 56.0 | Open window bus PM2.5 - 277 Air-conditioned bus PM2.5 – 140 Bus stop PM2.5 - 195 | 234 | (Goel et al. 2015) |
| ﻿Delhi, India | ﻿inhalable, thoracic and alveoli particles | ﻿aerosol spectrometer (Grimm technology Model 1.108, Germany) | - | - | - | - PM10 320(M) 324(E) - PM2.5 292(M) 302(E) - PM1 222(M) 229(E) | - PM10 332(M) 254(E) - PM2.5 292(M) 219(E) - PM1 205(M) 146(E) | - | (Kumar and Gupta 2016) |
| ﻿Andhra Pradesh, India | PM10 PM2.5 PM1 | ﻿Grimm 1.108 and EVM-7 | 40 Km Road | Bus AC Bus Car Car FA Car RC | - | Car PM10 – 106 PM2.5 – 29 PM1 – 16 Car FA PM10 – 43 PM2.5 – 24 PM1 – 20 Car RC PM10 – 20 PM2.5 – 10 PM1 - 7 | Bus PM10 – 134 PM2.5 – 37 PM1 – 18 AC Bus PM10 – 80 PM2.5 – 25 PM1 - 15 | - | (Kolluru and Patra 2020) |
| Mumbai, India | PM10 | ﻿DustTrak | - | Bus Train AC Taxi Non-AC car | PM10 - 156 | AC car PM10 – 69 Non-AC Car PM10 - 311 | PM10 - 503 | - | (Namdeo et al. 2016) |
| ﻿Vellore city, India | PM2.5 and PM1 | ﻿portable SidePak aerosol monitoring instrument (AM520, TSI Inc., USA) | 312 one-way trips | Walking Car Bus | - | PM2.5 – 182 PM1 - 172 | PM2.5 – 127 PM1 - 89 | PM2.5 – 128 PM1 - 57 | (Manojkumar et al. 2021) |
| ﻿Nur-Sultan, Kazakhstan | PM1 | ﻿DustTrak DRX (DustTrak DRX Aerosol Monitor 8533, TSI, USA) | - | Non-Air-conditioned Bus Non-Air-conditioned Car | - | - | PM1 – 11-99 | - | (Amouei Torkmahalleh et al. 2020) |
| Al-Hillah, Iraq | PM1 PM2.5 | Portable SidePak aerosol type AM520, TSI Inc., USA. Commuter seated behind the bus driver with instruments on the laps, two different periods of the day (Morning and Night) | - | Car Windows Open Car Windows Closed (Air Cond) 16 seats minibus | - | Open window for 60 street view - PM2.5 - 355.7(M), 299.5(A) - PM1 – 181.7(M), 155.8(A) Open window for 80 street view - PM2.5 - 246.9(M), 233.0(A) - PM1 – 154.3(M), 150.2(A) Closed window for 60 street view - PM2.5 – 105.5(M), 100.4(A) - PM1 – 87.3(M), 66.9(A) Closed window for 80 street view - PM2.5 – 92.6(M), 78.8(A) - PM1 – 87.0(M), 67.0(A) | 60 street - PM2.5 – 298.9(M), 305.5(A) - PM1 – 187.7(M), 155.8(A) 80 street view - PM2.5 – 288.2(M), 242.4(A) - PM1 – 161.2(M), 156.2(A) | - | (Al-Sareji et al. 2022) |
| ﻿Istanbul, Turkey | PM2.5 | ﻿portable real-time aerosol monitor pDR 1200 | 8-10 sample size depending on the type of transporting mode | ﻿Bus Metro–bus Air-conditioned car Non-air-conditioned car (recirculation on) Walking near the street | - | Air-conditioned car PM2.5 –67.9 Non-air-conditioned car (recirculation on) PM2.5 – 30.6 | Bus PM2.5 – 120.4 Metro-bus PM2.5 – 45.4 | PM2.5 – 89.2 | (Onat and Stakeeva 2013) |
| CO2 Exposure Studies |  |  |  |  |  |  |  |  |  |
| Hong Kong, China | CO2 | Q-Trak 8551 (TSI) | - | Subway Railway Car/Taxi Bus/minibus | Subway CO2 – 2245.0(c) Railway CO2 – 1326.0(c) | CO2 – 2356.2(c) | CO2 – 1574.3(c) | - | (Chau et al. 2002) |
| Hong Kong, China | CO2 | ﻿(IAQ Meter, TSI, USA) | 52 trips | Bus Euro II Bus Euro III Bus Euro IV | - | - | Bus Euro II CO2 – 1226.0 Bus Euro III CO2 – 1143.0 Bus Euro IV CO2 - 1510 | - | (Wong et al. 2011) |
| Nanjing, China | CO2 | ﻿CO2 concentrations were measured by a real-time CO2 monitor (Model WEZY-1, TJHY Inc., China) | ﻿16 sets sampled in subway station | Subway | - | - | - | - | (Shen and Gao 2019) |
| Taipei, Taiwan | CO2 | ﻿﻿TSI Model 7565 Q-Trak (TSI Inc., Shoreview, MN, USA) ﻿subject carrying monitors in a backpack | 80 times on each route | 111 | Underground CO2 - 1267 Ground level CO2 - 1131 | - | - | - | (Cheng et al. 2012) |
| Beijing, China | CO2 | ﻿portable carbon dioxide sampler |  |  | CO2 - 1177 | - | - | - | (Li et al. 2006) |
| Bangkok, Thailand | CO2 | ﻿(TSI Model 8552/8554 Q-Trak Plus, USA) | 4 routes with 80 trips in total | Air-conditioned bus Non-air-conditioned bus | - | - | Air-conditioned bus CO2 – 1280.0 Non-air-conditioned bus CO2 – 414.0 | - | (Kongtip et al. 2012) |
| Singapore | CO2 | ﻿﻿TSI Model 7545 Q-Trak Personal monitors carried by two different individuals namely office staff (OS) and university student (US) | 30 days | Walking Bus Taxi/car MRT train | - | US CO2 - 1515 | US CO2 - 571 | US CO2 - 347 | (Tran et al. 2021) |
| ﻿Nur-Sultan, Kazakhstan | CO2 | ﻿DustTrak DRX (DustTrak DRX Aerosol Monitor 8533, TSI, USA) |  |  | - | - | - | - | (Amouei Torkmahalleh et al. 2020) |
| TVOC Exposure Studies |  |  |  |  |  |  |  |  |  |
| Beijing, China | TVOC | ﻿Phocheck 5000 portable TVOC monitor |  |  | TVOC – 0.3 | - | - | - | (Li et al. 2006) |
| Taipei, Taiwan | TVOC | ﻿﻿TVOCs monitor (SeeAiR, model M5-201, Chen-Wei VR International Co., Ltd., Kaohsiung, Taiwan) | ﻿360 1-hour measurements | Electric powered subway Gas powered bus. Gasoline powered car. Walking | Electric powered subway TVOC – 55.2 | Gasoline powered car. TVOC – 69.1 | Gas powered bus. TVOC – 77.5 | Walking TVOC – 100.5 | (Liu et al. 2015) |

Note: M is morning; A is afternoon; WC+VC is the window closed and vent closed; WC+AC Rec is window closed and air condition and recirculation; WC+AC FA is Window closed and AC system on fresh air intake; W1/2O + VC is all windows half opened and vent closed; (a) means only morning value was selected; (b) signifies only winter value was selected