Supplementary Material

- The mice fecal samples preparation

The ratio between sample and extraction solvent is sample/solvent value = 1/100, and the ratio between different extraction solvents is: methyl tert-butyl ether (MTBE)/80 % methanol/water solvent system (4.5:1:2.5). This is the extraction step: First, 80% ice-cold methanol is added to the sample and vortexed for 30s. This is followed by the addition of 4.5 times the volume of methanol MTBE. Samples are mixed by vortexing and extracted by sonication for 20 min. Next, add 2.5 times the volume of methanol water, incubate at 4 °C for 10 min, and centrifuge at 5,000 rpm/min for 15 min. Transfer the water phase to a 2 mL eppendorf tube or glass tube and dried using a Speedvac vacuum concentrator (Eppendorf, Germany) and a nitrogen purge evaporator.

Sample redissolve: 150 μL of 80% methanol redissolves sample, transfer the redissolved sample to a 1.5 mL centrifuge tube, centrifuge at 12,000 rpm at 4°C for 10 min, and take the supernatant for UHPLC Q-TOF MS injection analysis. To ensure the stability and repeatability of the experiments, 10 μL each sample were pooled together as a quality control (QC) sample.

- UHPLC Q-TOF MS analysis of the mice fecal samples

For ultrahigh-performance liquid chromatography (UHPLC)–MS analysis, The sample of feces extracts were reconstituted with 150 μL of methanol/water (80/20) solution. LC–MS analysis was performed on a liquid chromatography system (ExionLC AD) coupled with a Triple TOF 5600+ mass spectrometer (AB Sciex, CA, USA). Chromatographic separations of polar and non-polar extracts were both performed by using a Waters Acquity UHPLC HSS T3 column (2.1 mm × 100 mm, 1.8 μm) with a temperature of 40 °C. The injected volume was 5 μL. The sample of feces extracts were 0.1% formic acid aqueous solution (A) and acetonitrile (B). The LC pump was programmed at a flow rate of 0.3 mL/min, and the gradient elution was optimized as follows:

| Time (min) | 0.1% Formic acid (A, %) | Acetonitrile (B, %) | Flow rate (mL/min) |
| --- | --- | --- | --- |
| 0–5 | 98 | 2 | 0.3 |
| 5–5.5 | 98-85 | 2–15 | 0.3 |
| 5.5–10 | 85 | 15 | 0.3 |
| 10–25 | 85-40 | 15–60 | 0.3 |
| 25–29 | 40 | 60 | 0.3 |
| 29–30 | 40-5 | 60–95 | 0.3 |
| 30–34 | 5 | 95 | 0.3 |
| 34–34.5 | 5-98 | 95–2 | 0.3 |
| 34.5-38 | 98 | 2 | 0.3 |

The parameter settings of the electron spray ionization (ESI) source, which was operated in the negative and positive ion modes, were as follows: ion spray voltages of −4500 V (negative) and 5500 V (positive), a turbo spray temperature of 550 °C, nebulizer gas (gas 1) of 55 psi, heater gas (gas 2) of 55 psi, curtain gas (CUR) of 30 psi, and declustering potential (DP) of 60 V. The mass spectrometer was operated in TOF MS coupled with information-dependent acquisition (IDA) trigger product ion scan modes. One MS cycle was composed of one TOF MS survey scan (accumulation time, 0.25 s) followed by up to 10 IDA-product ion scans (accumulation time, 0.09 s). IDA criteria were set as follows: the intensity threshold was 100 cps, and the dynamic background subtraction was switched on. The scan range of TOF MS was m/z 100–1500. Product ion scans were acquired from m/z 50–1250 with high sensitivity. The collision energy (CE) was set to 35 eV with a CE spread (CEs) of 15 eV. To ensure the stability and repeatability of the experiments, 10 μL each sample were pooled together as a quality control (QC) sample. For each batch (the sample of feces extracts  acquired in positive and negative ion modes, respectively), the QC sample was injected once every five injections to monitor the stability of the instrument.

- The mice lung samples preparation

Take 50 mg of lung tissue into a 1.5 mL centrifuge tube, add 3-4 steel balls, add pre-cooled 80% methanol (sample: methanol = 1:10), and place the centrifuge tube into the tissue grinder. The parameters of the grinder are set to 60 Hz, 2 min. After the end of grinding, incubate on ice for 1 h, then centrifuge with a cryogenic high-speed centrifuge (13,000 rpm/min, 15 min), after which 400 μL of the supernatant is placed into a new sterile centrifuge tube, and finally dried with a vacuum centrifugal concentrator.

- UHPLC Q-TOF MS analysis of the mice lung samples

For ultrahigh-performance liquid chromatography (UHPLC)–MS analysis, The sample of lung extracts were reconstituted with 100 μL of methanol/water (80/20) solution. LC–MS analysis was performed on a liquid chromatography system (ExionLC AD) coupled with a Triple TOF 5600+ mass spectrometer (AB Sciex, CA, USA). Chromatographic separations of lung extracts were performed by using a Waters Acquity UHPLC HSS T3 column (2.1 mm × 100 mm, 1.8 μm) with a temperature of 40 °C. The injected volume was 5 μL. The sample of lung extracts were 0.1% formic acid aqueous solution (A) and acetonitrile (B). The LC pump was programmed at a flow rate of 0.2 mL/min, and the gradient elution was optimized as follows:

| Time (min) | 0.1% Formic acid (A, %) | Acetonitrile (B, %) | Flow rate (mL/min) |
| --- | --- | --- | --- |
| 0-2 | 98 | 2 | 0.2 |
| 2-3 | 98-65 | 2-35 | 0.2 |
| 3-17 | 65-30 | 35-70 | 0.2 |
| 17-18 | 30 | 70 | 0.2 |
| 18-29 | 30-2 | 70-98 | 0.2 |
| 29-31 | 2 | 98 | 0.2 |
| 31-33 | 2-98 | 98-2 | 0.2 |
| 33-35 | 98 | 2 | 0.2 |

The parameters of Q-TOF MS are described in 2.