Supplemental Materials

Supplemental Tables

Table e-1. Sequence parameters for MR protocols (Ingenia; Phillips Healthcare)

| Parameter | T1WI | T2WI | T2 FLAIR | T1WI | GRE | DWI | CE 3D T1WI | 3D intracranial TOF-MRA | CE MRA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequences | 2D, SE | 2D, TSE | 2D, TSE | 2D, SE | 2D, FFE | EPI | 3D, FFE | 3D, FFE | 3D, FFE |
| TE (ms) | 10 | 80 | 125 | 10 | 15 | 60 | NA | 3.5 | 1.6 |
| TR (ms) | 450 | 3000 | 11000 (TI* 2800) | 450 | 650 | 3000 | (TI* 1000) | 25 | 4.3 |
| Flip angle (°) | 70 | 90 | 90 | 70 | 18 | 90 | 8 | 20 | 27 |
| FOV (mm) (RL × AP × FH)† | 160 × 220 | 180 × 230 | 180 × 230 | 180 × 230 | 180 × 230 | 250 × 250 | 240 × 240 | 200 × 250 × 100 | 320 × 110 × 380 |
| Number of slices | 23 | 22 | 22 | 22 | 22 | 22 | 55 | 180 | 180 |
| Slice orientation | Sagittal | Axial | Axial | Axial | Axial | Axial | Sagittal | Axial | Coronal |
| Slice thickness (mm) | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 1.2 | 1.2 |

* Inversion time. † Right/left; anterior/posterior; and foot/head. T1WI = T1-weighted image. T2WI = T2-weighted image. FLAIR = fluid-attenuated inversion recovery image. GRE = gradient echo image. DWI = diffusion weighted image. CE = Contrast-enhanced. TOF-MRA = time-of-flight magnetic resonance angiography. SE = spin echo. TSE = turbo spin echo. FFE = fast field echo. EPI = echo-planar imaging. TE = echo time. TR = repetition time. FOV = field of view

Table e-2. Sequence parameters for MR protocols (Architect; GE Healthcare)

| Parameter | T1WI | T2WI | T2 FLAIR | T1WI | GRE | DWI | CE 3D T1WI | 3D intracranial TOF-MRA | CE MRA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequences | 2D, TSE | 2D, TSE | 2D, TSE | 2D, TSE | 2D, SPGR | 2D, MUSE | 3D, MPRAGE | 3D, SPGR | 3D, SPGR |
| TE (ms) | 24 | 102 | 105 | 24 | 15 | Minimum | 3 | 3.4 | 1.6 |
| TR (ms) | 2300 | 4000 | 9000 (TI* 2462) | 2400 | 500 | 5318 | 7 | 23 | 4.3 |
| Flip angle (°) | 111 | 142 | 160 | 111 | 20 | NA | 8 | 18 | 22 |
| FOV (mm) (RL × AP × FH)† | 230 × 230 | 230 × 230 | 230 × 230 | 230 × 230 | 230 × 230 | 230 × 230 | 180 × 256 × 256 | 230 × 190 × 108 | 380 × 100 × 266 |
| Number of slices | 23 | 22 | 22 | 22 | 22 | 22 | 55 | 168 | 172 |
| Slice orientation | Sagittal | Axial | Axial | Axial | Axial | Axial | Sagittal | Axial | Coronal |
| Slice thickness (mm) | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 1.2 | 1.2 |

* Inversion time. † Right/left; anterior/posterior; and foot/head. T1WI = T1-weighted image. T2WI = T2-weighted image. FLAIR = fluid-attenuated inversion recovery image. GRE = gradient echo image. DWI = diffusion weighted image. CE = Contrast-enhanced. TOF-MRA = time-of-flight magnetic resonance angiography. TSE = turbo spin echo-accelerated. MPRAGE = magnetization prepared rapid gradient echo. SPGR = spoiled gradient recalled echo. TE = echo time. TR = repetition time. FOV = field of view

Table e-3. Sequence parameters for MR protocols (Skyra; Simens Medical Solutions)

| Parameter | T1WI | T2WI | T2 FLAIR | T1WI | GRE | DWI | CE 3D T1WI | 3D intracranial TOF-MRA | CE MRA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequences | 2D, SE | 2D, TSE | 2D, TSE | 2D, SE | 2D, FLASH | RESOLVE | 3D, MPRAGE | 3D, FLASH | 3D, FLASH |
| TE (ms) | 7.4 | 86 | 105 | 7.4 | 16 | 70 | 3 | 21 | 3.5 |
| TR (ms) | 400 | 4000 | 9000 (TI* 2500) | 450 | 450 | 4200 | 1800 | 3.7 | 1.4 |
| Flip angle (°) | 75 | 150 | 150 | 70 | 20 | 180 | 9 | 22 | 22 |
| FOV (mm) (RL × AP × FH)† | 230 × 230 | 210 × 210 | 210 × 210 | 210 × 210 | 210 × 210 | 210 × 210 | 370 × 108 × 256 | 220 ×198 × 108 | 370 × 108 × 277 |
| Number of slices | 20 | 20 | 20 | 20 | 20 | 20 | 50 | 180 | 180 |
| Slice orientation | Sagittal | Axial | Axial | Axial | Axial | Axial | Sagittal | Axial | Coronal |
| Slice thickness (mm) | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 0.5 | 0.6 |

* Inversion time. † Right/left; anterior/posterior; and foot/head. T1WI = T1-weighted image. T2WI = T2-weighted image. FLAIR = fluid-attenuated inversion recovery image. GRE = gradient echo image. DWI = diffusion weighted image. CE = Contrast-enhanced. TOF-MRA = time-of-flight magnetic resonance angiography. SE = spin echo. TSE = turbo spin echo. FLASH = fast low angle shot. MPRAGE = magnetization prepared rapid gradient echo. TE = echo time. TR = repetition time. FOV = field of view

Supplemental Figures

Figure e-1. Images of a 27-year-old man with fever and drowsy mentality. Two small rim-enhancing lesions in the right frontal lobe and right basal ganglia were visualized on (a) contrast-enhanced T1-weighted imaging. The larger lesion shows hyperintense peripheral edema on (b) fluid-attenuated inversion recovery imaging and central diffusion restriction on (c, d) diffusion-weighted imaging that suggested an abscess. Left-sided infective endocarditis was confirmed after echocardiography and blood culture.

Figure e-2. Images of a 20-year-old woman with left-sided infective endocarditis. (a, b) Brain MRI and MR angiography revealed a 3-mm mycotic aneurysm (arrows in a and b) at the superior branching of the right MCA (right M3 frontal branch) on (a) intracranial 3D time-of-flight (TOF) MR angiography and (b) contrast-enhanced T1-weighted imaging. After 3 days, the patient developed left hemiparesis and drowsy mentality. (c) Axial non-contrast-enhanced CT imaging shows a new acute intracranial hemorrhage in the right frontotemporal lobe, along with diffuse intraventricular hemorrhage. (d, e) Images from contrast-enhanced CT and CT angiography show an increase in the size of the mycotic aneurysm (arrows in d and e) at the right M3 frontal branch (3 mm  5 mm), suggesting that a mycotic aneurysmal rupture at the right M3 frontal branch caused the acute intracranial and intraventricular hemorrhages. After emergency clipping operation and hematoma evacuation, (f) the mycotic aneurysm was no longer visible on follow-up transfemoral cerebral angiography.