Supplementary Materials for

Nano-Clustering in Water-Lean Solvents Establishes Novel CO2 Chemistry

Julien Leclaire,*1 David J. Heldebrant,*2,3Katarzyna Grubel,2 Jean Septavaux,1 Marc Hennenbelle,1 Eric Walter,2 Ying Chen,2 Jose Leobardo Bañuelos4 Difan Zhang,2 Manh-Thuong Nguyen,2 Debmalya Ray,2 Sarah Allec,2  Deepika Malhotra,2 Jaelynne King.2

1Univ Lyon, Univ Lyon 1, CNRS, INSA, CPE, ICBMS, F-69622 Lyon, France.

2Pacific Northwest National Laboratory; Richland, WA 99352, United States of America

3Washington State University, Pullman, WA 99164, United States of America

4University of Texas El Paso; El Paso, TX 79968, United States of America

*Corresponding authors. Email: julien.leclaire@univ-lyon1.fr; david.heldebrant@pnnl.gov

Table of content

Materials and Methods	3

Materials	3

Instruments	3

Synthetic Procedures and characterization	5

Analytical procedures	8

PVT Measurements	8

Polarity measurement	9

Loading assessment and EEMPA-CO2 covalent adduct detection/quantification	10

Matlab data Analysis	12

Computational procedures	14

DFT Calculations of the stability of the different clusters	14

Static DFT calculations of the rotamer energy landscape on E(2)R species	14

AIMD modelling of Tetrameric Structures as a function of time at 25% CO2 loading	14

Supplementary figures	16

pKa determination	16

Gaz-liquid partition	17

Polarity measurements	20

Binding isotherms	21

NMR speciation	23

13C-NMR  spectra of the carbamic acid E(1)	29

Fitted NMR data	33

Speciation models	37

FT-IR data	39

DFT-computed clustering and absorption energies	43

Structural data from DFT calculations	46

AIMD data for Tetrameric Structures as a function of time at 25% CO2 loading	47

13C NMR monitoring of E(2) decay	51

DFT analysis of CO2 absorption mechanisms from 50% loading state	54

NMR and MS data of in situ and ex situ E(2)R formation	56

DFT-computed conformational energies on E(2)R derivatives	66

Supplementary tables	67

Abbreviations and schematic representations	67

Binding isotherms and speciation data	69

WAXS parameters	71

References	73

Materials and Methods

Materials

Trimethylsilyldiazomethane in hexanes was purchased from Thermo Fisher and used without further purification. Liquid state NMR were recorded on either 500 MHz Varian, or 500 MHz Bruker NOE spectrometers. 13CO2 (99%) was purchased from Cambridge isotopes. Boc2O and DMAP were purchased from Sigma-Aldrich and used without purification.

Instruments

Mass Spectrometry analysis was performed using an Q Exactive Plus mass spectrometer (Thermo Scientific, San Jose, CA) outfitted with a Heated ESI source. The source temperature, ion transfer tube temperature and spray voltage were 110ºC, 300ºC and 3.9 kV, respectively. Data weas collected for 2.5 min, with data acquisition being initiated immediately a stable signal is observed. FT-MS spectra was acquired from 50 to 650 m/z at a resolution of 140k (AGC target 3e6).

Negative mode Cryo ToF-SIMS analysis was performed on ToF-SIMS 5 (IONTOF, Műnster, Germany) using 10 kHz 100 µs cycle time and flood gun of 2.10 Å. Temperature of the experiment was set to -128 °C and raised to -115 °C and then to -80 °C. Time of the experiment was 30 min for the first raise and 45 min. for the second temperature raise. During first step, a 2 mm spacer was used in the cell, the during second step no spacer was inserted. Prior to analysis, sample was frozen in liquid nitrogen and mounted on a cooling stage.

High and low pressure IR spectra were collected on Nicolet iS10 Thermo Scientific instrument using high pressure demountable transmission liquid Harrick cell, and OMNIC 9 software.

Regular liquid state NMR spectra were recorded on 500 MHz Varian iNOVA spectrometer, or 500 Bruker Avance NEO spectrometer using standard pulse sequences. Where deuterated solvent was used, obtained spectra were referenced to residual solvent signal. Where no solvent was used and reaction was recorded in “neat” conditions, 13C NMR spectra were referenced to methyl signal of 2-EEMPA set to 16.1 ppm.

High-pressure liquid state NMR. All spectra were recorded on 500 MHz Varian iNOVA spectrometer, using in-house manufactured PEEK NMR tubes connected to commercial Parr reactor and Teledyne ISCO pump. All experiments were recorded using standard pulse sequences with delay time d1 = 5 or 10 seconds. Spectra were recorded without use of additional solvent and referenced to acetonitrile as an external reference

Solid-state NMR 13C MAS-NMR spectra were collected on a 600 MHz Bruker Avance III spectrometer using 5 mm zirconia rotors spinning at 3–5 kHz. We utilized a home-built custom HX probe as described previously where the design is compatible with our in-house-developed WHiMS rotor system, additionally, (1)magnetic susceptibility matched wire for the RF coil further enhances the spectral resolution of the mixed-phase samples. The 90° pulse width calibrated for the probe was 5 µs for both 1H and 13C, and the relaxation delay was optimized for full relaxation before each scan. 1H and 13C spin-lattice relaxation time (T1) measurements utilized the inversion-recovery (180° - tdelay - 90° - acquisition) sequence. 1H-1H and 13C-13C exchange spectroscopy (EXSY) experiments using a standard pulse sequence were carried out with several mixing times (5, 50, and 100 ms).

WAXS Measurements. Wide-angle x-ray scattering experiments were carried out on a Xenocs Xeuss 2.0 SAXS/WAXS system employing a monochromated Cu Kα (λavg= 1.54189 Å) source and an effective Q range of 0.1- 2.3 Å-1. The sample pressure was controlled using a manual piston pump and in-house high pressure tubing manifold connected to a 1 mm inner diameter polyimide tubing.

Potentiometric Titration was performed using a Mettler Toledo T5 instrument with a DGi102-Mini glass electrode calibrated with standard calibration solution at pH = 4.01; 7.01 and 9.00  purchased from Mettler Toledo.

Solvatochromatic Analysis. Solvatochromic measurements were performed using Nile red (Sigma-Aldrich). Nile red dye was dissolved in the solvent to produce a concentration of 10-4 M. Absorbance was measured with an Agilent Cary 60 UV-Vis spectrophotometer at room temperature.

EPR analyses. All EPR measurements were performed on a Bruker ELEXSYS E580 spectrometer equipped with an SHQE resonator. A quartz capillary, ID 0.8 mm and OD 1 mm, was used to hold the solution in the EPR cavity with both ends sealed by Critoseal. Instrument settings used to record the spectra were microwave frequency = 9.32 GHz, sweep width = 50 G, sweep time = 42s, microwave power = 2 mW, field modulation amplitude = 0.1 G, with four scans averaged. Spin Hamiltonian parameters for the spectra were obtained by fitting with EasySpin.1

Synthetic Procedures and characterization

## EEMPA E0

Synthesis. N-(2-ethoxyethyl)-3-morpholinopropan-1-amine (2-EEMPA, E) was synthesized using the synthetic methodology developed in our previous work2 and was successfully characterized using proton 1H and 13C carbon NMR spectroscopy. The reaction was carried out under nitrogen atmosphere. All the reagents were purchased from Sigma Aldrich, TCI and Fisher scientific and were used without purification.

Assessment of Water content in EEMPA. Coulometric Karl Fischer titrator C30S was used to measure the water content in synthetic batches of 2-EEMPA. Each batch had water content analysis performed in triplicate, with contents of all batches ranging between 309 ppm to 580 ppm H2O.

pKa Measurement by potentiometric titration. A 0.37 M solution of EEMPA was prepared by diluting 1,47 g of EEMPA in 20 mL of triply distilled water. A 0.5 M HCl solution was freshly prepared by dilution of 37% HCl solution with triply distilled water. 10 mL of the 0.37 M EEMPA sample was added into a 50 mL glass-jacketed titration cell containing a Teflon-coated stirrer. Aliquots (0.2 mL) of 0.5 M HCl solution were then added to the sample and the pH was monitored. The [H+] of the solutions was determined by the measurement of the electromotive force of the cell, E=E0 +Q log[H+]. The term pH is defined as -log[H+]. E0 and Q were determined by calibration of the electrode using a solution of known hydrogen-ion concentration at pH= 4.00; 7.01 and 9.00. Two equivalent points were detected during the titration (Fig S1a) of E(0) by HCl, corresponding to the sequential formation of E(0)+and E(0)2+ (Scheme S1).

Scheme S1. sequential protonation reactions on EEMPA.

The Bjerrum function ( n̄ ) detailed in (eq 1) was used for accurate pKa determination. This function quantifies the average protonation state per unit of analyte concentration 3, as :

n̄ = Total bound protonTotal analyte= cH+ -[ H+]ct       							  (1)

Where cH+ and ct are the total concentrations of the proton and analyte, respectively. The difference between the total and free hydrogen ion concentration corresponds to the concentration in bound hydrogen ions, and it can be obtained by measuring the pH of the solution and computing the total hydrogen ion concentration. pKa values were extracted from the inflexion points on the corresponding curve (Fig. S1b).

pKa(E(0)/E(0)+) = 9.25 		and 	pKa(E(0)+/ E(0)2+) = 6.01

In situ Synthesis of E(2)Me.

10 mL of ~0.6 M trimethylsilyldiazomethane in hexanes (corresponding to 0.685 g, 6∙10-3 mol) was transferred into an Ace pressure tube with a plunger and a front seal and put under vacuum until more than 90% of hexanes has been removed (at this point, TMS-diazomethane started to evaporate as well, as evidenced by yellow color of the condensate in the trap). Next, 1.3 g (6∙10-3 mol) of 2-EEMPA and 0.24 mL (0.192 g, 6∙10-3 mol) of MeOH were added. This mixture freeze-pump-thawed twice to remove any residual air. Next, 13CO2 was introduced to the mixture, which was briefly cooled down to condense as much 13CO2 as possible. Reaction was left stirring overnight. After this time, reaction was concentrated and purified on column eluting with Hex:AcOEt 1:1 and then 4:1. Obtained 456 mg (23.7 % yield) liquid of 99.5% purity (vide NMR). All of the following spectra were recorded on 500 MHz Bruker equipped with PI HR-500-S2-BBF/H/D-5.0-Z SP probe and at 298 K. Spectrum in Fig. S41 and S42 were collected neat, while spectra in Fig. S43-46 were collected using CDCl3 and referenced to residual solvent signal (CHCl3 at 7.26 for 1H and 77.16 ppm for 13 C NMR).

1H NMR (CDCl3, 298 K, 500 MHz): δ 3.67 (t, J = 4.9 Hz, 4H); 3.64 (t, J=3.5 Hz 3H); 3.52-3.48 (br, 2H); 3.44 (dd, J = 6.5 Hz, J = 7.0 Hz, 2H); 3.40-3.32 (br, 2H); 3.29 (br, 2H); 2.39 (br, 4H); 2.29 (br, 2H); 1.14 (t, 3 J = 7.0 Hz, 3H). 13C NMR (CDCl3, 298 K, 125 MHz): δ 157.5 (1C, NC(O)O); 156.8 (1C, OC(O)O); 69.0 (1C, COCH2CH3); 67.0 (2C, OCH2 morpholine); 66.5 (1C, OCH2CH3); 56.1 (1C, NmorpholineCH2); 53.7 (2C, NCH2 morpholine); 52.5 (OCH3); 47.5 (1C, NCH2CH2OCH3); 46.7 (1C, NmorpholineCH2CH2CH2); 25.4 (1C, NmorpholineCH2CH2); 15.2 (1C, OCH2CH3). BR-SIMS m/z calcd for C14H24N2O6 [M-H]- 317.2, found 317.2

ESI-FTMS m/z calcd for C14H26N2O6 [M+H]+ 319.1869, found 319.1840

Ex situ Synthesis of reference compound E(2)tBu

The procedure followed was adapted from previous report.4 In an ice bath cooled round bottom flask under argon atmosphere, Boc2O (2 equiv) was dissolved in 3 mL of MeCN, and DMAP (0.5 equiv) was added. After 5 min, a solution of EEMPA (0.5 mmol) in 2 mL of MeCN was added slowly during 2 min, and after an additional 1-5 min, chloroform (10 mL) was added and the solution was washed immediately with water (2 × 50 mL), dried with MgSO4, and evaporated to give a colorless oil.

1H NMR (CD3CN, 298 K, 500 MHz): δ 3.55 (t, br, 4H); 3.42-3.37 (m, 4H); 3.29 (br, 2H); 3.21 (t, 3J = 6.5 Hz, 2H); 2.31 (br, 4H); 2.23 (t, 3J = 6.75 Hz, 2H); 1.66 (m, br, 2H); 1.38 (s, 9H); 1.10 (t, 3 J = 7 Hz, 3H). 13C NMR (CD3CN, 298 K, 125 MHz): δ 155.7 (1C, NC(O)O); 141.2 (1C, OC(O)O); 78.3 (1C, OC(CH3)3); 69.2 (1C, OCH2CH2); 66.6 (2C, OCH2 morpholine); 65.8 (1C, NmorpholineCH2); 55.8 (1C, OCH2CH3); 53.7 (2C, CH2Nmorpholine); 47.3 (1C, OCH2CH2); 46.1 (1C, NCH2CH2CH2); 28.2 (3C, OC(CH3)3); 25.3 (1C, NCH2CH2CH2); 15.1 (1C, OCH2CH3). ESI-FTMS m/z calcd for C17H32N2O6 [M+H]+ 361.2333, found 361.2336.

Analytical procedures

PVT Measurements

To support and complement the NMR-based speciation, CO2-binding isotherm was measured using a custom “Pressure Volume Temperature (PVT) cell.” This apparatus was designed for simultaneous measurement of vapor liquid equilibrium, sorption kinetics and solvent viscosity 5 (Scheme S2) as described in our previous published work on EEMPA 6. Vapor-Liquid Equilibrium measurements on CO2 sorption were collected on the static synthetic method 7 which known amounts of CO2 are injected into the cell of known volume and allowed to come to equilibrium pressure at a fixed temperature. The PVT apparatus provides direct measurement of total pressure in the cell, however the equilibrium partial pressure of CO2 (P*) is calculated by subtracting the vapor pressure of EEMPA from the total pressure equilibrated at a given temperature of the measurement. EEMPA’s vapor pressure is ~1 mbar at 25 ˚C, resulting in the total equilibrium pressure Pf to be equivalent to Pf* of CO2 (Fig. S2).

Scheme. S2.

Schematic of the PVT apparatus used for isotherm measurements.

Polarity measurement

Solvatochromic Measurements and Analysis. Kamlet-Taft polarity parameters provide insight into a solvent’s hydrogen bond donating ability or acidity (α), hydrogen bond accepting or basicity (β) and polarity-polarizability (π*), based on the absorbance maximum exhibited by the solvatochromic dye. The polarity-polarizability scale of π* is an indicator of a solvent’s ability to stabilize charge or dipole moment 8. Linear relationships between the dielectric constant with π* include a contribution from the refractive index 8,9 which introduces an inherent difficulty to calculate the dielectric constant of an unknown solvent from π* when the refractive index is also unknown. Similar attempts to correlate the Reichardt’s parameter, ET(30), with the solvent dielectric constant have also been proposed with contradicting results 10. More importantly, Reichardt’s dye bleaches in amine solvents, limiting the ability to apply the ET(30) scale to CO2 capture solvents like EEMPA. Alternatively, Nile Red dye was used to determine the polarity of the EEMPA solvent based on the λmax­. Nile red dye was dissolved to produce a concentration of 10-4 M.

EPR Measurements and Spectroscopy. TEMPOL (4-Hydroxy-TEMPO, Sigma-Aldrich) was dissolved in all solvents to produce stock solutions of 10 mM which were subsequently diluted to produce 100 μM solutions. The samples were purged with helium to remove dissolved oxygen. All EPR measurements were performed on a Bruker ELEXSYS E580 spectrometer equipped with an SHQE resonator. A quartz capillary, ID 0.8 mm and OD 1 mm, was used to hold the solution in the EPR cavity with both ends sealed by Critoseal. Instrument settings used to record the spectra were microwave frequency = 9.32 GHz, sweep width = 50 G, sweep time = 42s, microwave power = 2 mW, field modulation amplitude = 0.1 G, with four scans averaged. Spin Hamiltonian parameters for the spectra were obtained by fitting with EasySpin 1.

Loading assessment and EEMPA-CO2 covalent adduct detection/quantification

NMR samples preparation at 1 atm for EEMPA-CO2 isotherms and speciation (Fig. S2, S6-S12)

All spectra were run neat and referenced to external capillary (1H Proton to C3H3(CF3)3 at 8.5 ppm and 13C Carbon to CD3CN at 118.26 ppm). EEMPA samples containing varying EEMPA/ CO2 ratios x0 were obtained as previously described in the literature 11,12 , by combining pressure and gravimetric monitoring on tubes containing a known amount of EEMPA. The following parameters were used for spectra recording : D1 = 75 sec; # scans 512 except for free 2-EEMPA, which was 128.

NMR sample preparation above 1 atm for EEMPA-CO2 isotherms and speciation (Fig S2, S6-S12.). The general methodology followed correspond to procedures validated in the literature. 12 For pressurized experiments, rotors of the WHiMS design were used 13 . This material allows for gas to be added via a one way check valve. The liquid portion of the sample was pipetted into a tared rotor which was then sealed with the o-ring containing bushing. After this reference mass was recorded, the rotor was placed in a pressure vessel. After removal of the surrounding air with a vacuum pump, the vessel (and therefore the rotor) was pressurized with CO2 using an Isco syringe pump. After equilibration (~15 minutes), the rotor was removed and the new mass recorded. We utilized a home-built custom HX probe as described previously 14 where the design is compatible with our in-house-developed WHiMS rotor system, additionally, magnetic susceptibility matched wire for the RF coil further enhances the spectral resolution of the mixed-phase samples. The 90° pulse width calibrated for the probe was 5 µs for both 1H and 13C, and the relaxation delay was optimized for full relaxation before each scan. 1H and 13C spin-lattice relaxation time (T1) measurements utilized the inversion-recovery (180° - tdelay - 90° - acquisition) sequence. 1H-1H and 13C-13C exchange spectroscopy (EXSY) experiments using a standard pulse sequence were carried out with several mixing times (5, 50, and 100 ms).

Qualitative monitoring of the equilibrium between dissolved CO2 and EEMPA-bound CO2 (E(1)(-)) by MAS 13C NMR. EEMPA was sparged with CO2 gas at ambient pressure for 3 minutes and then pressurized for 1 minute around 20 bars (corresponding to an initial CO2/EEMPA stoichiometry x0 of 0.99). The sample was subjected to static and MAS 13C and 1H NMR (Fig. S16). At low loading (a = 0.47), mixing of CO2 and EEMPA happens quickly, so the spinning of the rotor simply offers higher resolution for the spectra. At higher loading (a = 0.75), the complete mixing and reaction of CO2 and EEMPA takes more than 30 minutes, therefore the rotor spinning provides a fast complete mixing between CO2 and EEMPA, as seen in the sudden conversion of E(1)- (carbamate) into E(1) (carbamic acid) corresponding to the shielding of the 13C CO signal by more than 1 ppm (see also Fig. S11)

Qualitative monitoring of the anhydride E2- conversion into carbamic acid pairs E1-E1 (Fig. S33-35) 3 mL of 2-EEMPA was put in a 25 mL Parr reactor, which was closed and evacuated. Next, Parr reactor overhead stirrer was switched on and 2-EEMPA was pressurized with P*0 = 20 bars (x0 = 0.99) of CO2 delivered by high pressure Teledyne ISCO pump and kept at constant pressure by continuous CO2 delivery. During pressurization, temperature increased from ambient to ~49 °C as CO2 was being consumed. After temperature decreased back down and stabilized at 26 °C, stirring was switched off and the mixture was pushed with P*0 = 35 bars of CO2 (x0 = 1.6) through the PEEK tube into the PEEK NMR cell without opening it to air. Spectra were recorded immediately over a period of 19h.

Direct observation of E(2)- and of its capping by TMCHN2 by 13C NMR (Fig S41-42)

2.6 g (0.011 mol) of 2-EEMPA was mixed with 10 mL of ~0.6 M trimethylsilyldiazomethane in hexanes. This mixture was placed in an Ace pressure tube with a plunger and a front seal and put under vacuum until more than 90% of hexanes has been removed (this amount is just an estimate, but, at this point, TMS-diazomethane started to be evaporated as well, as evidenced by yellow color of the condensate in the trap). Next, 13CO2 was added to the mixture, which was briefly cooled down to condense as much 13CO2 as possible. Reaction was left stirring for 2 h. Upon warming up of the solution, it bubbles and releases excess of condensed 13CO2. At this stage, the pressure flask was opened, an aliquot was drawn and analyzed by 13C NMR, confirming initial formation of the carbamate. Small doublet of doublets assigned to E(2)Me starts to be already visible as well (Fig. S41). After the NMR spectra were collected, the faint yellow solution of moderate viscosity (due to the presence of hexanes) was put back in the reaction vessel, which was briefly evacuated to remove headspace gas. The vessel was then back-filled with CO2 and allowed to stand overnight. 13C NMR analysis confirmed full disappearance of signal @ 161.2 ppm, which was attributed to the carbamate species. This disappearance is concurrent with growth of doublet of doublets, which we attribute to the E(2)Me species

Pressurized Sample preparation and FT-IR spectra acquisition. A Nicolet is10 FT-IR equipped with a DTGS KBr detector coupled with a high-pressure demountable liquid cell was used to record transmission spectra. The spectrum of the empty transmission cell was used as background to remove residual signals of atmospheric water and CO2. EEMPA was loaded into the transmission cell using a syringe to create a thin layer on the KBr windows. The cell was connected to a Parr reactor that was pressurized with CO2. The transmission cell was initially evacuated before a two-way valve was opened pressurize the cell to 1328 psi. Pressure was monitored using a pressure transducer provided by a Parr reactor controller that was connected to the Parr reactor. The pressure was held constant for 20 minutes before the pressure was released by opening a two-way valve to a second Parr reactor that was under vacuum. Spectra were recorded throughout the isobaric process and during the stepwise decrease in pressure. All spectra were recorded using 32 scans at a resolution of 4 cm-1. Initial pressure P0 to stoichiometry x0 to loading a conversions were conducted using reference curves displayed at Fig S2.

Matlab data Analysis

Dimeric EEPMA model

xE0.52= K0.52∙xE02∙xCO2 							(1)

xE12= K12∙xE0.52∙xCO2							(2)

xE02+ xE0.52+xE12=1 							(3)

xCO2=x0-α2								(4)

With xCO2 being the molar fraction of unbound CO2 (in the headspace and as a dissolved gas) and xEi2 the molar fraction of i-loaded dimer (i = 0, unloaded, i = 0.5 half-loaded, i= 1, fully loaded).

The chemical loading a of the sample can be expressed as :

α2=0,5 xE0.52+xE12								(5)

Tetramolecular EEPMA system:

xE0.254= K0.254∙xE04∙xCO2							(6)

xE0.54= K0.54∙xE0.254∙xCO2							(7)

xE0.754= K0.754∙xE0.54∙xCO2							(8)

xE14= K14∙xE0.754∙xCO2							(9)

xE04+xE0.254+xE0.54+xE0.754 + xE14=1					(10)

xCO2=X0-α4								(11)

With xCO2 the amount of unbound CO2 and

α4=0,25 xE0.254+0,5 xE0.54+0,75 xE0.754 + xE14				(12)

Equilibrium constant determination by parametric fitting.

The association constants for both dimeric and tetrameric EEMPA clusters are calculated through the concomitant fitting of the speciation (molar fraction of E(0)(+) and E(1)(-) species quantified by q13C NMR as a function of X0 ) using a MATLAB script with:

The CO2 loading α, determined by q13C NMR or PVT (2 x 49 dataset),

And,

The CO2 loading α, determined by q13C NMR and the chemical shift variation of each of the 18 13C nucleus observed, with respect to its value at X0=0 (20 x 14 dataset),

For any given set of equilibrium constants, the molar fraction of each species of the system is determined by solving the system of equation composed of the CO2 absorption equations and CO2 / EEMPA mass balances (equations (1) to (5) for the dimeric system or equations (6) to (12) for the tetrameric system). During the fitting process, the predicted molar fractions are determined for any set of association constants and total quantity of CO2 X0  using MATLAB’s build-in fsolve function with Levenberg-Marquart algorithm.

The predicted CO2 loading α is obtained through the equation (5) or (12).

When appropriate, for each 13C nucleus, the predicted chemical shift perturbation ∆δin of nucleus i in the n-mer model is obtained through the equations (13) or (14), after parameter adjustments of the chemical shift perturbations dδln through curve fitting with chemical shift variation data, using MATLAB’s build-in lsqcurvefit function with trust region reflective algorithm.

∆δi2=dδ0.5,i2∙xE0.52 + dδ1,i2∙xE12 						   (13)

∆δi4=dδ0.25,i4∙xE0.254+dδ0.5,i4∙xE0.54+dδ0.75,i4∙xE0.754 + dδ1,i4∙xE14 		(14)

Comparison between predicted and observed data permits the parameter adjustment of the association constants using the lsqcurvefit function with trust region reflective algorithm. The fittings are performed on non-normalized datasets. CO2 loading data receives a higher ponderation than chemical shift variation during the fitting to account for their higher reliability (coherent q13C NMR and PVT data) and lower variability. The ponderation of α data was set at 10 against 1 for each 13C nucleus chemical shift variation. To avoid local optimums, initial constant values are randomly generated within the design space over multiple attempts (>30) and fittings displaying the lower residual are conserved.

Scheme. S3.

Schematic representation of the algorithm used for the determination of association constants (here for fitting over CO2 loading α and chemical shift variations, with the tetramolecular EEMPA system).

Computational procedures

DFT Calculations of the stability of the different clusters

Static DFT calculations were performed using ORCA software package15,16 using B3LYP17,18 functional and Grimme-D3 dispersion correction.19 Def2-TZVP basis sets were used for all the atoms and SMD implicit solvent model20 with DMF, t-butanol and 1-propanol solvents were used for structural optimizations. Numerical frequencies were computed in order to confirm the local minimum structure of these structures and to assess the entropy effects (including translational, rotational and vibrational contribution). The energy landscapes of rotamers for the anhydride forms with different capping agents were also evaluated using these setups. Acetonitrile and Methanol was used as implicit solvent for the modeling of the rotamer landscape.

Static DFT calculations of the rotamer energy landscape on E(2)R species

The DFT calculations were carried out on E(2)R (R = H, Li, Me, tBu) using SMD implicit solvent model. Therefore, both intramolecular interactions and intermolecular interactions between E(2)R rotamer and the implicit medium contribute to the overall rotational barrier. No explicit inter-molecular interaction is considered in this solvent modeling, which might be a significant factor that affects the rotational energy barrier in the real experimental system.

AIMD modelling of Tetrameric Structures as a function of time at 25% CO2 loading

AIMD simulations were performed using CP2K software package21 using PBE functional S1322 and Grimme-D3 dispersion correction.19 The simulation box (19.9115 × 19.9115 × 19.9115 Å3) included 20 EEMPA molecules. Out of these 20 EEMPA molecules, 5 of them were loaded with CO2 (i.e. 25% loading, see fig 2). Canonical NVT Ensemble simulations were performed at 298 K. Norm conserving pseudopotentials were used for core electrons23 and DZVP quality Gaussian type basis sets were used for all atoms. An auxiliary plane-wave basis set24 with 450 Ry cutoff criteria was used for calculating electrostatic terms.

WAXS monitoring of the clustering process

WAXS methods measurements and data acquisition at STP. The equilibration of EEMPA samples upon CO2 exposure at each pressure was monitored using WAXS for duration ranging from 1 to 4 hr, the highest initial pressures/stoichiometries requiring the longest equilibration times. Sample scattering data was corrected for sample container background and sample geometry, and normalized by time, transmission coefficient, and the intensity scaled to absolute units of differential scattering cross section per unit volume.

WAXS data processing methodology and fitting. The WAXS curves were fit (SASview version 5.0.4, http://www.sasview.org/) over the measured range using a Teubner-Strey model25,26 for the low-Q region of the spectra and three Gaussian peaks for the higher angle high-intensity peak that consists of various short nearest molecular neighbor correlations. The Teubner-Strey (TS) model applied at low Q describes the periodicity and correlation length due to regions of alternating electron density distribution induced by CO2 absorption.

The full two-phase Teubner-Strey model with three Gaussian peaks (TS+3G) is presented below:

Iq=8πφa1-φa∆ρ2c2/ξa2+c1q2+c2q4+i=13Aie-12q-q0,i2/σi2+bgd

In this model, φa is volume fraction of phase a, ξ is the correlation length, and ∆ρ is the x-ray scattering length density difference between the two phases. Because the term 1-φa∆ρ2 depends on precise knowledge of the density of electron rich core in comparison to its surroundings, this term was held constant and a single scaling parameter. Instead, A_scale represents the growth of the CO2-dependent core phase, presented in Table S1 below.  The parameters a2, c1, c2 are related to the periodicity d and correlation length ξ as follows:

Periodicity d is given by: d=2π12a2c21/2-14c1c2-1/2

And the correlation length is given by: ξ=12a2c21/2+14c1c2-1/2.

The three Gaussian peaks are used to fit the primary broad peak signal between 1.32 and 1.40 Å-1, which is due to a number of nearest neighbor intermolecular correlations. The list of obtained fitting parameters for the TS+3G model are presented in Tables S8-S11.

The amphiphilicity factor, fa, is useful in describing the phase stability of the system and is related to the Teubner-Strey model parameters as:  fa=c14a2c21/2 . The original definition of the amphiphilicity factor is in relation to the water-water structure function. Table S8 describes the physical properties corresponding to different values of  fa (adapted from reference27 The α dependence of the amphiphilicity factor for EEMPA is shown in Fig S27.

Supplementary figures

pKa determination

Fig. S1.

Titration curve of aqueous EEMPA 0.37 M with aqueous HCl 0,5 M (a) and plot the protonation state of E(0) vs pH in aqueous medium (Inflection points correspond to) the pKa value (b).

Gaz-liquid partition

Fig. S2.

EEMPA CO2 binding isotherm. Plot of the chemical loading a vs initial CO2/EEMPA stoichiometry x0. Empty circles : PVT measurements. Full dots : q13C NMR data (black : including dissolved CO2; grey : without dissolved CO2). Perfect match between PVT (white dots) and q13C NMR  measurements (grey dots) cross-validates both quantification methods.

According to IUPAC recommendations, the binding isotherms is defined by the relation, at constant temperature, between a, the fraction of ligand (herein CO2) bound to a receptor (herein EEMPA) vs the amount of unabsorbed ligand (herein free CO2 in the headspace) noted x(unbound). In most set-ups (in particular those devoted to adsorption of CO2 on solid material), the gas is supplied in large excess so that the amount bound can be neglected with respect to the global reservoir. This lead most authors to approximate the amount of unbound gas to the total amount of gas introduced, i.e. the equilibrium partial pressure P*f and the initial partial pressure P*0. Equilibrium pressure measurements confirmed the linear relationship in our set up between x(unbound) and x0 (Fig. S3). This validates our systematic use of x0 as the x-axis of binding isotherms.

Fig. S3.

Gas/liquid CO2 partition upon EEMPA loading. molar fraction of CO2 in the headspace x(headspace) derived from pressure measurements vs initial CO2 to EEMPA molar fraction x0.

P*f can extrapolated from xunbound, the molar fraction of CO2 which was neither dissolved in EEMPA (physically captured) nor reacted with EEMPA (chemically bound). This fraction was calculated by subtracting the total amount of CO2 initially introduced measured by gravimetry to the molar amount CO2 bound into the liquid phase measured by q13C NMR. P*0 can be measured independently (see PVT measurement section) and was found to correlate with x0 through the perfect gas law within less than 2% deviation. Therefore, the same law could reasonably be applied to evaluate Pf from nf.

Fig. S4.

Relationship between initial partial pressure in CO2 P0* and equilibrium/final partial Pf* pressure in CO2 in our setup monitored experimentally.

Polarity measurements

Fig. S5. Polarity measurement on neat EEMPA samples upon gradual CO2 loading. (a) Nitrogen isotropic hyperfine coupling constant aN as a function of dielectric constant for solutions of Tempol in various polar non-hydroxylic solvents including EEMPA at a = 0, 25 and 36 mol% CO2 loading. (b) Comparison of the Nile Red dye λmax and the dielectric constant calculated through EPR as a function of CO2 loading.

Free EEMPA has a polarity like PPG-1200 with an absorption maximum of 526 nm. CO2 loaded EEMPA exhibited a red shift in the absorbance spectrum. At 25 mol% CO2 and 36 mol % CO2 loading, the absorbance maximum are located at 530 and 533 nm, respectively, which indicates an increase in polarity of the medium. Based on the nile red λmax values, the EEMPA solvent polarity shifts from a nile red similar to PPG-1200 to t-butanol (5.0 Mpa CO2, 25 °C) in the loading range mentioned above.

The nitrogen isotropic hyperfine coupling constant, aN, was measured for EEMPA, 25 mol % CO2-loaded EEMPA, and 36 mol % CO2-loaded EEMPA. As the CO2 loading increased, aN increased from an initial value of 15.46 to 15.54 and 15.60 for 25 mol% and 36 mol% CO2 respectively. The increase of aN with CO2 loading was expected from Nile Red experiments as aN is known to increase with increasing solvent polarity due to the stabilization of the nitroxide radical. aN was found to linearly correlate with the various solvent polarity parameters: Wertheim parameter (μρ ln ε/MW), dielectric constant (ε) and the Dimroth-Reichardt parameter (ET­30) 28. The dielectric constant of EEMPA, 25 mol % CO2-loaded EEMPA and 36 mol % CO2-loaded EEMPA was extrapolated from the linear relationship between the dielectric constant of polar non-hydroxylic solvents that include ethyl acetate, acetonitrile and dimethyl sulfoxide (DMSO) and aN shown in Fig. S5a. The linear regression presented in literature gave a better fitting with r = 0.963 with an intercept of 15.45 and a slope of 0.0062 28. The dielectric constant of EEMPA increases from 1.92 to 15.47 and 24.2 with 25 mol% and 36 mol% CO2 loading, respectively, due to the formation of charged species.

Comparison of the dielectric constant calculated through the EPR technique to the solvatochromic technique using Nile red dye show the same trend with increasing CO2 loading, shown in Fig. S5b. The trend confirms the increasing polarity of EEMPA as CO2 loading increased.

Binding isotherms

Fig. S6. Fitting of the experimental chemical loading a values (circles) with increasing CO2/EEMPA stoichiometries x0 with the dimeric model and resulting RMSD value.

Fig. S7. Fitting of the experimental chemical loading a values (circles) with increasing CO2/EEMPA stoichiometries x0 with the tetrameric model and resulting RMSD value.

NMR speciation

Fig. S8. q13C NMR stacked spectra of EEMPA upon gradual loading with CO2 (bottom to top, see table S6 for values) focused on nuclei Ca and Cf. Blue : E(0)(+), Red : E(1)(-)

Fig. S9. q13C NMR stacked spectra of EEMPA upon gradual loading with CO2 (bottom to top, see table S6 for values) focused on nuclei Cd, Cd, Cg and Ch. Blue : E(0)(+), Red : E(1)(-)

Fig. S10. q13C NMR stacked spectra of EEMPA upon gradual loading with CO2 (bottom to top, see table S6 for values) focused on nuclei Cb, Cc and Ci. Blue : E(0)(+), Red : E(1)(-)

Fig. S11. q13C NMR stacked spectra of EEMPA upon gradual loading with CO2 (bottom to top, see table S6 for values) focused on carbonyl and dissolved CO2 signals. Blue : E(0)(+), Red : E(1)(-)

Fig. S12. q1H NMR stacked spectra of EEMPA upon gradual loading with CO2 (bottom to top, see table S6 for values) focused on CH signals (0.8 – 4.2 ppm).

Fig. S13. q1H NMR stacked spectra of EEMPA upon gradual loading with CO2 (bottom to top, see table S6 for values) focused on XH signals (X = N, O; 2.5 – 13 ppm).

13C-NMR  spectra of the carbamic acid E(1)

Fig. S14. 1H-1H NOESY NMR spectra of EEMPA and CO2 (a =0.95) at a mixing time of 10 ms.

This experiment reveals that the carbamic acid proton is closer to a morpholine ring (the correlation between NCOOH and f, e, g, h, i, j, k, as well as the lack of correlation between NCOOH and a, b, c, d). These correlations between a and j, k are consistent with the tetrameric clustering into reverse micelles-like architectures.

Fig. S15. 13C-13C 2D EXSY spectra of CO2-EEMPA (a = 0.95) at a mixing time of 50 ms. cross-peak correlation indicate an equilibrium process between carbamic acid E(1) (159 ppm) and dissolved CO2 (125.2 ppm) but not between carbamic acid E(1) and CO2 gas (124.8 ppm).

Fig S16. Effect of spinning on the carbamate E(1)- + dissolved CO2 to carbamic acid E(1) conversion. 13C of EEMPA and 13C-enriched CO2, at P*0 = 1 bar (x0 = a = 0.47) in static (a) and MAS conditions (b) and at P*0 = 20 bars (x0 = 0.99, a = 0.75) in static (c) and MAS conditions (d).

Fig S17. Literature survey (histogram) of 13C NMR shift of CO signals of amine-CO2 adducts in porous solids29 (red-orange) and organic solutions30,31 (green) by category of adduct : ammonium carbamate (left) and carbamic acid (right) and comparison with measured values in the present study (horizontal lines : values obtained in this work ; blue : ammonium carbamate;  red : carbamic acid; size of the bar reflect the dispersity of the reported values).

Fitted NMR data

Fig. S18. Plot of the Chemical shift perturbation (CSP) Dd(i) of the 13C nuclei of unloaded E(0)(+) upon loading with increasing CO2/EEMPA stoichiometries x0. Color code reflects the amplitude of perturbation. Dots : experimental data. Line : thermodynamic dimer model from Matlab fitting. R2mean = 0.942 is calculated as the average standard deviation of the full fit series displayed in Fig. S18+S19.

Fig. S19. Plot of the Chemical shift perturbation (CSP) Dd(i) of the 13C nuclei of loaded EEMPA E(1)(-) upon loading with increasing CO2/EEMPA stoichiometries x0. Color code reflects the amplitude of perturbation. Dots : experimental data. Line : thermodynamic dimer model from Matlab fitting. R2mean = 0.942 is calculated as the average standard deviation of the full fit series displayed in Fig S18+S19.

Fig. S20. Plot of the Chemical shift perturbation (CSP) Dd(i) of the 13C nuclei of unloaded E(0)(+) upon loading with increasing CO2/EEMPA stoichiometries x0. Color code reflects the amplitude of perturbation. Dots : experimental data. Line : thermodynamic tetramer model from Matlab fitting. R2mean = 0.997 is calculated as the average standard deviation of the full fit series displayed in Fig S20+S21.

Fig. S21. Plot of the Chemical shift perturbation (CSP) Dd(i) of the 13C nuclei of loaded EEMPA E(1)(-) upon loading with increasing CO2/EEMPA stoichiometries x0. Color code reflects the amplitude of perturbation. Dots : experimental data. Line : thermodynamic tetrameric model from Matlab fitting. R2mean = 0.997 is calculated as the average standard deviation of the full fit series displayed in Fig. S18+S19.

Speciation models

Fig. S22. Dimeric model used to fit the NMR data displayed at Fig. S18 and S19 (a) and equilibrium constant obtained (b) (c indicates the cooperativity factor, see 32 and speciation resulting from the model plotted as a function of the chemical loading a (c) or initial stoichiometry x0 (d). This model predicts that the carbamic acid E(1) is only present from a = 0.5.

Fig. S23. Tetrameric model used to fit the NMR data displayed at Fig. S20 and S21 (a) and equilibrium constant obtained (b) (c indicates the cooperativity factor, see ref) and speciation resulting from the model plotted as a function of the chemical loading a or initial stoichiometry x0. This model predicts that the carbamic acid E(1) is present from a = 0.2.

FT-IR data

Fig. S24. FT-IR spectra of EEMPA during pressurized loading CO2 and depressurization. Absorbance spectra of EEMPA during increasing CO2 loading (green : 0.27, purple : 0.58, yellow: 0.89 and red :  0.92; colour code correspond to speciation displayed at Fig S23). (b) Absorbance spectra of EEMPA during depressurization from a CO2 loading of 0.92 (red) to 0.27 upon progressive depressurization

The pattern of signals obtained in FT-IR with EEMPA at low CO2 loading is constituted of two overlaying peaks in the carbonyl region  (around 1655 cm-1  and 1682,  for a = 027, wherein the singly occupied (green) tetramer E4(0.25) is the main species). This band becomes narrower as the CO2 pressure in the Parr reactor and the loading increased (up to a = 0.92, from green to red, Fig. S24a). Depressurization of the Parr reactor (Fig S24b) to release CO2 causes the band at 1682 cm-1 to reform as the band at 1655 cm-1 to decrease until it fully disappears. Interpretation of this phenomenon is proposed at Fig S25.

Fig. S25. Reported carbonyl stretching frequency in FT-IR spectroscopy for carboxylic species (grey) 33 and carbamates as isolated34,35 or herein paired species? X+ is the counter-ion which  accompanies the carbamate/carbamate. Unusual decrease of the CO stretching frequency for EEMPA between a = 0.3 – 0.9 is attributed to E4(0.5) + CO2 -> E4(0.75) which corresponds to the conversion of two (ammonium) carbamates  into (ammonium) carbamate – carbamic acid pair.  Symmetrical and unsymmetrical modes have radically different intensities and only the first one is detected

WAXS data

Fig. S26. WAXS data comparison of unloaded and CO2-loaded EEMPA at STP, against pressure-induced CO2 loading (α) structural data.

Medium range ordering (MRO) with periodicity of ~12 Å begins to appear at low Q for α from 0.58 - 0.72. The MRO becomes better defined at higher pressures for α from 0.58 – 0.86. The I(Q=0) value, which is proportional to isothermal compressibility, initially increases as unbound CO2 expands cavities within the solvent yet drops once the new high-pressure tetramer binds locally available CO2. The curves α: 0.03-0.86 are vertically offset by 0.01 cm-1 on the plot for clarity. The CO2-loaded EEMPA at STP is offset by 0.062 to align with the α=0.36 curve.

Fig. S27. Amphiphilicity factor dependence on CO2 concentration in EEMPA.

During initial CO2 incorporation, the interface becomes less defined up to α = 0.36, when the structure contains similar quantities of the different CO2-bound species and as well as gas phase CO2. Increased pressure drives the system to the primary CO2-loaded tetramer state and the interfaces demarcating the regions of unique binding chemistry are better defined with fa ~ -0.8.

DFT-computed clustering and absorption energies

Fig. S28. DFT modelling of EEMPA-CO2 adducts and absorption enthalpies: (a) optimized structures of monomeric constituents (in DMF) and their dimerization enthalpy (b) into corresponding optimized structures (c, in DMF) and successive enthalpies of CO2 absorption (d) (Atom color code : C (black), H (white), O (red), N (blue); histogram implicit solvent code : DMF (light), t-butanol (medium), propanol (dark). Although all structures were minimized in the three solvents, the choice was made to display minimized structures in the most dissociative one (DMF).

Fig. S29. DFT modelling of EEMPA-CO2 adducts and self-assembling enthalpies. Optimized structures of monomeric constituents (in DMF, left) and their tetramerization enthalpy (right) into corresponding optimized structures (in DMF, middle) (Atom color code : C (black), H (white), O (red), N (blue); histogram implicit solvent code : DMF (light), t-butanol (medium), propanol (dark). Although all structures were minimized in the three solvents, the choice was made to display minimized structures in the most dissociative one (DMF).

Fig. S30. DFT modelling of CO2 absorption by self-assembled tetramers. Optimized structures of monomeric constituents (in DMF, left) and their CO2 absorption enthalpy (right) (Atom color code : C (black), H (white), O (red), N (blue); histogram implicit solvent code : DMF (light), t-butanol (medium), propanol (dark). Although all structures were minimized in the three solvents, the choice was made to display minimized structures in the most dissociative one (DMF).

Structural data from DFT calculations

Fig. S31. DFT modelling of self-assembled tetramers in DMF and quantification of their morphological features. Minimized structures of E4(0.5) (a), E4(1) (b) and of their internal reactive sites (c) and (d). These dimension as well as the computed sizes of tetramer clusters (left bars) and their cavities (right bars) quantified by gyradius (e) match experimental values obtained by WAXS (see Fig.3). (atom color code : blue : nitrogen, red : oxygen; black : carbon; white : hydrogen).

AIMD data for Tetrameric Structures as a function of time at 25% CO2 loading

Figure S32. (top) Snapshots of AIMD simulations of E4(0.25) at different time. (bottom) The dynamics of bond lengths for atoms in the pocket highlighted in the top panel. The tetramer involving one CO2-loaded EEMPA is stable over ~59 ps in full agreement with the NMR model. The C-N bond length is ~6 Å whereas N-N distances are around 8 Å.

Figure S33. (top) Snapshots of AIMD simulations of E4(0.5). at different time. (bottom) The dynamics of bond lengths for atoms in the pocket highlighted in the top panel. The tetramer involving two CO2-loaded EEMPA is quite stable over a timescale of ~48 ps (Figure 12). The C-C and N-N bond lengths are shown in Figure 13. We further analyzed the N-H and O-H bonds present in the tetramer. The N-H bonds are in the range 1.0-1.25 Å whereas the O-H bonds are in the range 1.4-2.1Å. This suggests that the core of the tetramer involving two CO2 loaded EEMPA involves two salt bridges and 4 strong hydrogen bonds.

Figure S34.  Snapshots of AIMD simulations of E4(0.75) at different times. Atoms in the tetramer pocket are highlighted. The tetramer involving 3 CO2-loaded EEMPA constituents is quite dynamic at 25% CO2-loading. It varies between a trimer like structure to a tetramer like structure at 298 K and low CO2 loading. Thus, at low CO2 loading, this tetramer will moderately favored. This agrees with the NMR-based speciation. E4(0.75) has one zwitterionic interaction and 4 hydrogen bonding interactions present.

Figure S35. Snapshots of AIMD simulations in E4(1.0) of different times. Atoms in the tetramer pocket are highlighted. The tetramer involving four CO2-loaded EEMPA constituents is unstable at 25% CO2 loading. As shown in Fig. S35 the tetramer falls apart during the AIMD trajectory. This is consistent with experimental observation as well as cluster DFT calculations where we also see that the E4 (1.0) is unfavored at low CO2 loading, and with the model obtained from NMR speciation.

13C NMR monitoring of E(2) decay

Fig S36. Extension of the 46-72 ppm region of the spectra during decay over time of the free anhydride from a = 0.96 (bottom spectrum) to a = 0.56 (top spectrum) as measured by relative f1 vs f0 integration. (Bottom spectrum : immediately after transfer, then every 10 min for the next six spectra; the last one was recorded 19h after transfer and correspond to equilibrium state at a = 0.56).

Fig. S37.Extension of the 15-28 ppm region of the spectra during decay over time of the free anhydride from a = 0.96 (bottom spectrum) to a = 0.56 (top spectrum) as measured by relative f1 vs f0 integration. (Bottom spectrum : immediately after transfer, then every 10 min for the next six spectra; the last one was recorded 19h after transfer and correspond to equilibrium state at a = 0.56).

Fig. S38. Extension of the 15-28 ppm region of the spectra during decay over time of the free anhydride from a = 0.96 (bottom spectrum) to a = 0.56 (top spectrum) as measured by relative f1 vs f0 integration. (Bottom spectrum : immediately after transfer, then every 10 min for the next six spectra; the last one was recorded 19h after transfer and correspond to equilibrium state at a = 0.56).

DFT analysis of CO2 absorption mechanisms from 50% loading state

Figure S39. DFT computed anhydride-based CO2 absorption mechanisms by EEMPA (a) dimeric pathway, (b) tetrameric pathway. Three implicit solvent environments were considered. Structures displayed correspond to DMF as an implicit solvent.

Figure S40. DFT computed zwitterionic mechanisms of E4(0.5) + CO2 -> E4(1) absorption step E4(0.5) + CO2 -> E4(0.75) absorption step via the zwitterion intermediate E4(0.75)z (a) and E4(0.75) + CO2 -> E4(1) absorption step via the zwitterion intermediate E4(1)z. Three implicit solvent environments were considered.

NMR and MS data of in situ and ex situ E(2)R formation

Fig. S41. 13C NMR spectra of crude mixture obtained from E, TMSCHN2 and 13CO2. See Fig. S42 for assignment of the signals below 80 ppm.

Fig. S42. Extension of the aliphatic region with assignments of known signals (E(1)(+) and E(0)(-) are the major compounds). Unlabeled signals are postulated to belong to E-TMS, main side-product of the reaction when conducted in the absence of MeOH.

Fig. S43.1H NMR of purified E(2)Me. For details about the assignment see Fig. S45.

Fig. S44. 13C NMR of purified (E(2)Me), including numbering scheme. Insert is the expansion of the carbonyl region in dark orange.

Fig. S45. Extension of the relevant region of the HSQC NMR of purified (E(2)Me). Numbers relate to protons/carbons as outlined in previous spectra. HSQC was recorded to establish short-range proton-carbon coupling (1JCH) and assign 1H signal displayed at Fig. S41.

Fig. S46. Extension of relevant region of HMBC NMR of purified (E(2)Me). Capital letters refer to carbon (Cx) nuclei, while lower case letters relate to protons (Hx). HMBC was recorded to establish long-range proton-carbon coupling (nJCH) within ethylenic moieties (see double arrows). No conclusive correlation involving carbon j2 and k2 could be obtained.

Fig. S47. (-)-LR-TOF SIMS spectrum of purified E(2)Me. Found for [E(2)Me-H] (Calcd 317.2).

Fig. S48. (+)-HR-ESI/MS spectrum of purified E(2)Me. Found for E(2)MeH+ : 319.1840 (Calcd  319.1869) and E(2)Me2+ : 333.1996 (calcd : 333.2026).

Fig. S49. 140-160 ppm region of the 13C NMR of E(2)tBu (orange) containing traces of E(1)tBu (red).

Fig. S50. 60-90 ppm region of the 13C NMR of E(2)tBu (orange) containing traces of E(1)tBu (red).

Fig. S51. 40-70 ppm region of the 13C NMR of E(2)tBu (orange) containing traces of E(1)tBu (red).

Figure S52. 10-40 ppm region of the 13C NMR of E(2)tBu (orange) containing traces of E(1)tBu (red). * and # respectively mark traces of AcOEt and tBuOH.

Figure S53. (+) HR-ESI MS spectrum of purified E(2)tBu (a) and comparison between experimental and theoretical pattern (b).

DFT-computed conformational energies on E(2)R derivatives

Fig. S54. (a) Snapshot of minimized structure of E(2)Me and E(2)tBu obtained by DFT calculations. (b) Rotamer landscape for EEMPA anhydrides E(2)R with different capping agents (R = Li, H, Me, tBu). The inset table shows the energy barrier of different rotamers.

Our calculations above (Fig S54) have shown that different implicit solvents have limited impact on stabilities of the intermediates and products of CO2 capture by EEMPA. Furthermore, even though the tetra-butyl cap has a significantly larger size than the other groups, the rotational barrier is not affected much by intra-molecular interaction due to the large space for the rotation of the capping group. On the other hand, the intra-molecular hydrogen bonding in the H capped anhydride results in higher rotational barrier.

Supplementary tables

Abbreviations and schematic representations

Table S1.

Building blocks involved in CO2 capture / Constituents of dynamic covalent system.

| Constituent/building block | Formula | Symbol |
| --- | --- | --- |
| E |  |  |
| CO2 | CO2 |  |

Table S2.

Adducts from capture / resulting from reversible covalent linkage between constituents. Components for non-covalent clustering into dimers/tetramers.

| Component/adduct |  | Formula | Symbol |
| --- | --- | --- | --- |
| E(0)(+) | E(0) |  |  |
|  | E(0)+ |  |  |
| E(1)(-) | E(1)- |  |  |
|  | E(1) |  |  |

Table S3.

Adducts from capture / resulting from reversible covalent linkage between constituents. Components for non-covalent clustering into dimers/tetramers.

| (trapped) Intermediate | Formula | Symbol |
| --- | --- | --- |
| E(2)- |  |  |
| E(1)Z |  |  |
| E(2)Me |  | N/A |
| E(2)tBu |  | N/A |

Table S4.

Dimeric clusters resulting from non-covalent assembling of components/capture adducts

| Dimer/cluster |  | Formula | Symbol |
| --- | --- | --- | --- |
| E2 | E2(0) | E(0)E(0) |  |
|  | E2(0.5) | E(1)-E(0)+ |  |
|  | E2(1)* | E(2)-E(0)+ |  |
|  | E2(1) | E(1)E(1) |  |

Table S5.

Tetrameric clusters resulting from non-covalent assembling of components/capture adducts

| Tetramer/cluster |  | Formula | Symbol |
| --- | --- | --- | --- |
| E4 | E4(0) | E(0) E(0) E(0) E(0) |  |
|  | E4(0.25) | E(1)-E(0)+E(0) E(0) |  |
|  | E4(0.5) | E(1)-E(0)+E(1)-E(0)+ |  |
|  | E4(0.5)N | E(1)-E(0)+ E(1)E(0) |  |
|  | E4(0.75)N | E(1) E(0) E(1) E(1) |  |
|  | E4(0.75)Z | E(1)-E(0)+ E(1)E(1)z |  |
|  | E4(0.75)* | E(2)-E(0)+ E(1)-E(0)+ |  |
|  | E4(0.75) | E(1)-E(0)+E(1)E(1) |  |
|  | E4(1)Z | E(1)E(1)ZE(1)E(1) |  |
|  | E4(1) | E(1)E(1)E(1)E(1) |  |

Binding isotherms and speciation data

Table S6.

EEMPA/CO2 samples analyzed by q13C NMR (Fig. S8-S16). Correspondence between initial stoichiometry x0 and chemical loading a.

| x0 | a |
| --- | --- |
| 0.000 | 0.000 |
| 0.077 | 0.077 |
| 0.143 | 0.143 |
| 0.176 | 0.176 |
| 0.274 | 0.272 |
| 0.359 | 0.356 |
| 0.374 | 0.371 |
| 0.472 | 0.467 |
| 0.534 | 0.520 |
| 0.646 | 0.597 |
| 0.993 | 0.746 |
| 1.616 | 0.809 |
| 2.352 | 0.842 |
| 5.054 | 0.950 |

Table S7. RMSD values of the fit of experimental 13C chemical shifts and loading values vs x0 with the dimeric (left) and tetrameric model. (Right) (Averaged RMSD : bold font, RMSD values below an arbitrary 0.95 cutoff : red).

|  | R²(E2) | R²(E4) |
| --- | --- | --- |
| alpha | 0.9931 | 0.9996 |
| CO | 0.6181 | 0.997 |
| c' | 0.9745 | 0.9914 |
| c | 0.9809 | 0.9991 |
| i,i’ | 0.9626 | 0.9986 |
| b,b’ | 0.9118 | 0.9618 |
| g | 0.9618 | 0.9959 |
| g' | 0.9829 | 0.9895 |
| h | 0.9869 | 0.9895 |
| h' | 0.951 | 0.9825 |
| d | 0.9783 | 0.9979 |
| e | 0.9808 | 0.9979 |
| d' | 0.8848 | 0.9652 |
| e' | 0.9727 | 0.9825 |
| f | 0.969 | 0.9963 |
| f' | 0.935 | 0.9982 |
| a' | 0.9368 | 0.9759 |
| a | 0.9427 | 0.9883 |
|  | 0.9370 | 0.993 |

WAXS parameters

Table S8.  WAXS TS+3G model, Teubner Strey Parameters.

| α | A_scale (cm-1) | A_scale (Err) | A_d (Å) | A_d (Err) | A_xi (Å) | A_xi (Err) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.03 | 0.0035 | 0.0045 | 21.96 | 1.16 | 7.96 | 4.82 |
| 0.08 | 0.0036 | 0.0097 | 20.34 | 2.24 | 6.10 | 6.76 |
| 0.20 | 0.0047 | 0.0071 | 20.58 | 1.31 | 6.71 | 4.46 |
| 0.36 | 0.0086 | 0.0220 | 20.82 | 2.72 | 4.53 | 3.69 |
| 0.58 | 0.1932 | 0.0161 | 11.93 | 0.08 | 4.20 | 0.16 |
| 0.72 | 0.3782 | 0.0083 | 11.90 | 0.02 | 5.92 | 0.08 |
| 0.79 | 0.4228 | 0.0097 | 11.85 | 0.02 | 5.65 | 0.08 |
| 0.84 | 0.4245 | 0.0091 | 11.85 | 0.02 | 5.71 | 0.07 |
| 0.86 | 0.4244 | 0.0088 | 11.81 | 0.02 | 5.81 | 0.07 |

Table S9. WAXS TS+3G model, Gauss Peak 1 parameters.

| α | B_scale (cm-1) | B_scale (Err) | B_peak_pos (Å-1) | B_peak_pos (Err) | B_sigma (Å-1) | B_sigma (Err) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.03 | 0.1319 | 0.0024 | 1.326 | 0.0008 | 0.216 | 0.0026 |
| 0.08 | 0.1309 | 0.0028 | 1.326 | 0.0009 | 0.213 | 0.0029 |
| 0.20 | 0.1315 | 0.0024 | 1.328 | 0.0008 | 0.211 | 0.0026 |
| 0.36 | 0.1354 | 0.0035 | 1.328 | 0.0010 | 0.212 | 0.0035 |
| 0.58 | 0.1274 | 0.0018 | 1.325 | 0.0009 | 0.207 | 0.0028 |
| 0.72 | 0.1277 | 0.0021 | 1.322 | 0.0006 | 0.174 | 0.0019 |
| 0.79 | 0.1239 | 0.0031 | 1.321 | 0.0007 | 0.177 | 0.0026 |
| 0.84 | 0.1277 | 0.0024 | 1.322 | 0.0006 | 0.175 | 0.0022 |
| 0.86 | 0.1307 | 0.0020 | 1.322 | 0.0006 | 0.173 | 0.0018 |

Table S10. WAXS TS+3G model, Gauss Peak 2 parameters.

| α | C_scale (cm-1) | C_scale (Err) | C_peak_pos (Å-1) | C_peak_pos (Err) | C_sigma (Å-1) | C_sigma (Err) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.03 | 0.0341 | 0.0020 | 1.378 | 0.0018 | 0.115 | 0.0026 |
| 0.08 | 0.0332 | 0.0022 | 1.377 | 0.0021 | 0.116 | 0.0029 |
| 0.20 | 0.0304 | 0.0021 | 1.377 | 0.0020 | 0.112 | 0.0030 |
| 0.36 | 0.0302 | 0.0023 | 1.374 | 0.0020 | 0.110 | 0.0031 |
| 0.58 | 0.0342 | 0.0024 | 1.356 | 0.0013 | 0.109 | 0.0026 |
| 0.72 | 0.0208 | 0.0025 | 1.344 | 0.0021 | 0.090 | 0.0044 |
| 0.79 | 0.0283 | 0.0037 | 1.337 | 0.0018 | 0.098 | 0.0044 |
| 0.84 | 0.0242 | 0.0030 | 1.336 | 0.0018 | 0.092 | 0.0043 |
| 0.86 | 0.0234 | 0.0025 | 1.340 | 0.0018 | 0.088 | 0.0039 |

Table S11. WAXS TS+3G model, Gauss Peak 3 parameters.

| α | D_scale (cm-1) | D_scale (Err) | D_peak_pos (Å-1) | D_peak_pos (Err) | D_sigma (Å-1) | D_sigma (Err) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.03 | 0.0613 | 0.0021 | 1.339 | 0.0018 | 0.526 | 0.0330 |
| 0.08 | 0.0631 | 0.0029 | 1.338 | 0.0021 | 0.503 | 0.0373 |
| 0.20 | 0.0656 | 0.0024 | 1.333 | 0.0016 | 0.501 | 0.0302 |
| 0.36 | 0.0609 | 0.0050 | 1.334 | 0.0040 | 0.518 | 0.0721 |
| 0.58 | 0.0669 | 0.0019 | 1.374 | 0.0042 | 0.484 | 0.0107 |
| 0.72 | 0.1032 | 0.0011 | 1.394 | 0.0016 | 0.497 | 0.0046 |
| 0.79 | 0.1065 | 0.0012 | 1.402 | 0.0020 | 0.517 | 0.0054 |
| 0.84 | 0.1079 | 0.0011 | 1.401 | 0.0018 | 0.512 | 0.0050 |
| 0.86 | 0.1087 | 0.0010 | 1.402 | 0.0017 | 0.512 | 0.0046 |

References

1.	Stoll, S. & Schweiger, A. EasySpin, a comprehensive software package for spectral simulation and analysis in EPR. Journal of Magnetic Resonance 178, 42–55 (2006).

2.	Cantu, D. C. et al. Molecular-Level Overhaul of γ-Aminopropyl Aminosilicone/Triethylene Glycol Post-Combustion CO2-Capture Solvents. ChemSusChem 13, 3429–3438 (2020).

3.	Subirats, X., Fuguet, E., Rosés, M., Bosch, E. & Ràfols, C. Methods for pKa Determination (I): Potentiometry, Spectrophotometry, and Capillary Electrophoresis. in Reference Module in Chemistry, Molecular Sciences and Chemical Engineering 1–10 (Elsevier, 2015). doi:10.1016/B978-0-12-409547-2.11559-8.

4.	Basel, Y. & Hassner, A. Di- tert -butyl Dicarbonate and 4-(Dimethylamino)pyridine Revisited. Their Reactions with Amines and Alcohols. J Org Chem 65, 6368–6380 (2000).

5.	Jiang, Y. et al. Techno-economic comparison of various process configurations for post-combustion carbon capture using a single-component water-lean solvent. International Journal of Greenhouse Gas Control 106, 103279 (2021).

6.	Zheng, R. F. et al. A single-component water-lean post-combustion CO2capture solvent with exceptionally low operational heat and total costs of capture-comprehensive experimental and theoretical evaluation. Energy Environ Sci 13, 4106–4113 (2020).

7.	Gibbs, R. E. & Van Ness, H. C. Vapor—Liquid Equilibria from Total-Pressure Measurements. A New Apparatus. Industrial and Engineering Chemistry Fundamentals 11, 410–4130 (1972).

8.	Kamlet, M. J., Abboud, J.-L. M., Abraham, M. H. & Taft, R. W. Solvent Effects in Organic Chemistry. J. J. Am. Chem. Soc 48, 23 (1983).

9.	Brady, J. E. & Carr, P. W. An Analysis of Dielectric Models of Solvatochromism. J. Phys. Chem 89, 5759–5766 (1985).

10.	Teo, W. et al. Nile Red fluorescence spectroscopy reports early physicochemical changes in myelin with high sensitivity. Proc Natl Acad Sci U S A 118, e2016897118 (2021).

11.	Septavaux, J. et al. Simultaneous CO2 capture and metal purification from waste streams using triple-level dynamic combinatorial chemistry. Nat Chem 12, 202–212 (2020).

12.	Pugh, S. M. & Forse, A. C. Nuclear magnetic resonance studies of carbon dioxide capture. Journal of Magnetic Resonance 346, 107343 (2023).

13.	Walter, E. D. et al. Operando MAS NMR Reaction Studies at High Temperatures and Pressures. Journal of Physical Chemistry C 122, 8209–8215 (2018).

14.	Mehta, H. S. et al. A novel high-temperature MAS probe with optimized temperature gradient across sample rotor for in-situ monitoring of high-temperature high-pressure chemical reactions. Solid State Nucl Magn Reson 102, 31–35 (2019).

15.	Neese, F. The ORCA program system. Wiley Interdiscip Rev Comput Mol Sci 2, 73–78 (2012).

16.	Neese, F., Wennmohs, F., Becker, U. & Riplinger, C. The ORCA quantum chemistry program package. Journal of Chemical Physics 152, (2020).

17.	Becke, A. D. Density-functional exchange-energy approximation with correct asymptotic behavior. Phys Rev A  (Coll Park) 38, 3098–3100 (1988).

18.	Lee, C., Yang, eitao & Parr, R. G. Development of the Colic-Salvetti correlation-energy formula into a functional of the electron density. Phys Rev B 37, 785–789 (1988).

19.	Grimme, S. Semiempirical GGA-type density functional constructed with a long-range dispersion correction. J Comput Chem 27, 1787–1799 (2006).

20.	Marenich, A. V., Cramer, C. J. & Truhlar, D. G. Universal solvation model based on solute electron density and on a continuum model of the solvent defined by the bulk dielectric constant and atomic surface tensions. Journal of Physical Chemistry B 113, 6378–6396 (2009).

21.	Vandevondele, J. et al. QUICKSTEP: Fast and accurate density functional calculations using a mixed Gaussian and plane waves approach. Comput Phys Commun 167, 103–128 (2005).

22.	Perdew, J. P., Burke, K. & Ernzerhof, M. Generalized gradient approximation made simple. Phys Rev Lett 77, 3865 (1996).

23.	Goedecker, S., Teter, M. & Hutter, J. Separable dual-space Gaussian pseudopotentials. Phys Rev B 54, 1703–1710 (1996).

24.	Lippert, G., Hutter, J. & Parrinello, M. A hybrid Gaussian and plane wave density functional scheme. Mol Phys 92, 477–488 (1997).

25.	Teubner, M. & Strey, R. Origin of the scattering peak in microemulsions. J Chem Phys 87, 3195 (1987).

26.	Schubert, K. V., Strey, R., Kline, S. R. & Kaler, E. W. Small angle neutron scattering near Lifshitz lines: Transition from weakly structured mixtures to microemulsions. J Chem Phys 101, 5343–5355 (1994).

27.	Morkved, T. L., Stepanek, P., Krishnan, K., Bates, F. S. & Lodge, T. P. Static and dynamic scattering from ternary polymer blends: Bicontinuous microemulsions, Lifshitz lines, and amphiphilicity. Journal of Chemical Physics 114, 7247 (2001).

28.	Ottaviani, M. F., Martini, G. & Nuti, L. Nitrogen Hyperfine Splitting Constant of Nitroxide Solutions: Differently Structured and Charged Nitroxides as Probes of Environmental Properties. Magnetic Resonance in Chemistry 25, 897–904 (1987).

29.	Forse, A. C. et al. Elucidating CO 2 Chemisorption in Diamine-Appended Metal− Organic Frameworks. J Am Chem Soc 140, 18016–18031 (2018).

30.	Kortunov, P. V., Siskin, M., Baugh, L. S. & Calabro, D. C. In Situ Nuclear Magnetic Resonance Mechanistic Studies of Carbon Dioxide Reactions with Liquid Amines in Non-aqueous Systems: Evidence for the Formation of Carbamic Acids and Zwitterionic Species. Energy and Fuels 29, 5940–5966 (2015).

31.	Masuda, K., Ito, Y., Horiguchi, M. & Fujita, H. Studies on the solvent dependence of the carbamic acid formation from ω-(1-naphthyl)alkylamines and carbon dioxide. Tetrahedron 61, 213–229 (2005).

32.	Hunter, C. A. & Anderson, H. L. What is cooperativity? Angewandte Chemie - International Edition 48, 7488–7499 (2009).

33.	Masuda, K., Ito, Y., Horiguchi, M. & Fujita, H. Studies on the solvent dependence of the carbamic acid formation from u-(1-naphthyl)alkylamines and carbon dioxide. Tetrahedron 61, 213–229 (2004).

34.	Switzer, J. R. et al. Reversible Ionic Liquid Stabilized Carbamic Acids: A Pathway Toward Enhanced CO 2 Capture. Ind Eng Chem Res 52, 13159–13163 (2013).

35.	Didas, S. A., Sakwa-Novak, M. A., Foo, G. S., Sievers, C. & Jones, C. W. Effect of Amine Surface Coverage on the Co-Adsorption of CO 2 and Water: Spectral Deconvolution of Adsorbed Species. J Phys Chem Lett 5, 4194–4200 (2014).