Supplementary part

Local Monitoring of Photosensitizer Transient States Provides Feedback for Enhanced Efficiency and Targeting Selectivity in Photodynamic Therapy.

Elin Sandberga, Chinmaya V Srambickala, Joachim Pigueta, Haichun Liua, Jerker Widengrena,*

a Royal Institute of Technology (KTH), Experimental Biomolecular Physics, Dept. Applied Physics, Albanova Univ Center 106 91 Stockholm, Sweden

* Corresponding author: Email:  jwideng@kth.se, Phone: +46-8-7907813

S1. Electronic state model for IR700

Figure S1. IR700 electronic state model, in which the singlet ground state (S0) is excited to the emissive singlet excited state (S1) from where it via intersystem crossing can go to the triplet state T1. From T1 IR700 can again return to S0 or form a photo-reduced state (R), the radical anion state, with an enhanced yield in the presence of electron donors such as ascorbic acid. From the photo-reduced state it can be re-oxidized back to the ground state or the release of a phenol ligand can be triggered.

S2. Electronic state models

Electonic state models for MB

With the electronic state model shown in Figure 2A, the electronic state population dynamics of a MB fluorophore, subject to a constant excitation photon flux of Φexc starting at time t=0, is given by

ddtAt=M∙At			(S1)

Here, At= St, T1t, T1Ht,  RtT represents the population probabilities of the singlet (both S0 and S1), triplet, protonated triplet, and photo-reduced (both protonated and non-protonated) state, respectively.

M=         -kISC´kT1            kISC´-(kT1+kH+kRED)                  kT2                         kOX              -kOH                       00                        kH0                        kRED    -(kT2+kOH+kRED)0kRED-kOX       (S2)

is the model rate matrix describing the transitions between the states. In the matrix, the effective intersystem crossing rate, from S to T1 is given by:

kISC´=kISC∙σexc∙Φexcσexc∙Φexc+k10				(S3)

with σexc denoting the excitation cross section of the singlet ground state, S0, and k10 the decay rate from the excited singlet state, S1, to S0. The recovery rate of the photo-radical state, kOX, is a composite recovery rate of the protonated and non-protonated form of the photo-radical. kRED denotes the reduction rate of the T1 and T1H states and is for simplicity denoted as one and the same rate from both these states.

The initial condition for Eq. (S1) is

A0=1, 0, 0, 0 T				(S4)

, assuming all MB fluorophores are in the singlet (ground) state before onset of excitation at t=0.

For a rectangular excitation pulse, Φexc is constant throughout the excitation duration and the matrix M is not time dependent. The general solution to Eq S1 is then

At=eMt∙A0				(S5)

The dependence of the detected fluorescence at time, t, after onset of excitation is then given by

Ft=qF∙qD∙k10·σexc∙Φexcσexc∙Φexc+k10∙St        		 (S6)

For solutions in which the pH is much higher than the pKA of the MB triplet state (pH >> 7,2) the  protonation of T1 can be neglected. A simplified model (Figure 2C) can then be applied, in which the state population vector of Eq. S1 can then be reduced to At= St, T1t, RtT, and with the model rate matrix given by

M=-kISC´kT1kOX   kISC´-(kT1+kRED)00kRED-kOX			(S7)

For solutions in which the pH is much lower than the pKA of the MB triplet state (pH << 7,2), and in which the buffer concentration is high, such that the triplet protonation rate kH≫kT1, kRED1 and kOH. A simplified model can then be applied, which is analogous to that of Figure 2C, but with At= St, T1Ht, RtT, and with the model rate matrix given by

M=-kISC´kT2kOX   kISC´-(kT2+kRED)00kRED-kOX			(S8)

In Eqs. S7 and S8, kOX refers to the recovery rates of the non-protonated and protonated form of the photo-radical, respectively.

Electronic state model for IR700

With the electronic state model shown in Figure S1,, the electronic state population dynamics of a IR700, subject to a constant excitation photon flux of Φexc starting at time t=0, is likewise described by Eq. S1, with At= St, T1t,  R-tT representing the population probabilities of its singlet (both S0 and S1), triplet , and photo-reduced state. The model rate matrix is, similar to Eqs. S7 and S8, given by:

M=-kISC´kTkOX   kISC´-(kT+kRED)00kRED-kOX				(S9)

In absence of R- formation, only singlet-triplet state transitions have to be considered, with At= St, T1tT, and

M=-kISC´kT   kISC´-kT						(S10)

S3. Fiber-based TRAST measurements

Figure S2. Experimental TRAST curves, recorded via an optical fiber placed in a PBS (12mM) solution. TRAST data is represented by dots and the lines are the fitted curves with residuals below. More details are given in the main text. Fitted rate parameter values are presented in Table 2. (A) Φexc  dependence of IR700 globally fitted with a triplet state model. (B) Φexc  dependence of MB measured at pH 3.5, where data has been fitted by a 4-state model (C) Φexc  dependence of MB measured at pH 9.3, where again data has been fitted by a 4-state model.

S4.   FCS – experimental setup and analysis

FCS measurements were performed on a commercial, epi-illuminated, confocal laser scanning microscope (Olympus FV1200). Solution samples with IR700 were excited by the focused beam (338nm 1/e² radius) of a 640 nm diode laser (LDH-D-C-640, PicoQuant GmbH, Berlin) in continuous wave. The emitted fluorescence was collected back through the microscope objective (UPlanSApo 60x/1.2w, Olympus), passed through a dichroic mirror (ZT405/488/635rpc-UF2, Chroma), an emission filter (HQ720/15, Chroma), and focused onto a pinhole (50µm) in the back focal plane. The fluorescence signal was finally split and directed on two avalanche photodiodes (Picoquant, Tau-spad), whose signals were collected by a data acquisition card (Hydraharp 400, Picoquant, Berlin).

In the FCS measurements, for freely diffusing fluorescent molecules undergoing dark state transitions, the autocorrelation curves of the recorded fluorescence intensity, F(t), can be described by:

Gτ=<FtFt+τ><Ft>2=GDτGTτ+ 1		(S11),

where GDτ denotes the translational diffusion-dependent part GTτ signifies the contribution from photo-induced dark state transitions. GDτ can be expressed as:

GDτ=1Nm1+ττD-1×1+ω0ωz2ττD-12 			(S12),

with ω0 and ωZ denoting the distances from the center of the laser beam focus in the radial and axial direction respectively at which the collected fluorescence intensity has dropped by a factor of 1e2 compared to its peak value. Nm is the mean number of fluorescent molecules within the detection volume. τD is the characteristic diffusion time of the fluorescent molecules, given by the diffusion coefficient D as τD=ω024D.

If no dark state transitions occur, the blinking term GTτ=1. Otherwise, for a fluorophore with n dark transient states, and for τ much longer than the anti-bunching relaxation times of the fluorophores, GTτ can be expressed as a normalized set of relaxation terms [16], averaged over the confocal detection volume, weighted by the square of the detected molecular brightness of the molecules, Wr:

GTτ=W2r1-i=1nAi(r)-Ai(r)e-λi(r)τdVW2ri=1n1-Ai(r)dV  		(S13)

In analogy to Eq. 6, λi(r) are the eigenvalues and Ai(r) the related amplitudes, reflecting the population build-up of the different photo-induced non-fluorescent states. At steady state and with no photobleaching, the sum of the population probabilities for S0 and S1, together with i=1nAi(r) equals one.

S5. Fluorescence lifetime measurements

Time-correlated single photon counting (TCSPC) lifetime measurements were performed on IR700 solutions, prepared as described above, using the same microscope setup as for the FCS measurements, but in this case with the diode laser (638 nm, LDH-D-C-640, PicoQuant GmbH, Berlin) operated in pulsed mode. Instrument response functions (IRFs) were determined from the back-reflected light from the laser excitation pulses. The signals were fed into a data acquisition card (Hydraharp, Picoquant GmbH), deconvoluted and then fit to an exponential decay based on non-linear least squares minimization (Symphotime, Picoquant GmbH).