Enhancement of photo-bromination of phenol by anthraquinone-2-sulphonate and benzophenone: implication for photo-production of organic brominated compounds by dissolved organic matter in marine environment

Hui Liu*1, Xiaojun Qiu 1, Xiaomei Zhu1, Bing Sun1, Xiaoxing Zhang1

1College of Environmental Science and Engineering, Dalian Maritime University, Dalian 116026, China

*Corresponding author: Hui Liu, liuhui@dlmu.edu.cn

Text S1

Chemical analysis

The concentrations of bromophenols were analyzed by gas chromatography mass spectrometry (GC–MS). The sample solution was spiked with 2-hydroxy-5-chlorobiphenyl (100 μg L-1) as the internal standard, and then acidified to pH ~2 using 2.5 mol L-1 H2SO4, followed by extraction with dichloromethane (15 mL, 2x). After dehydration using an anhydrous sodium sulfate column, the extract was concentrated to 500 μL for direct analysis by GC–MS. Analysis was performed using an Agilent 7890 GC and 5975C MSD with an EI source operating in SIM mode, using ions at mass/charge ratios (m/z) 172 and 174 for bromophenols determination and 204 and 206 for 2-hydroxy-5-chlorobiphenyl determination. The recoveries of 2-bromophenol and 4-bromophenol (n=5) were 90 ± 7.9% and 100 ± 10.7%, respectively, and the detection limits (signal to noise ratio, S/N=3) of 2-bromophenol and 4-bromophenol were 65 and 136 ng L−1, respectively. The GC was operated with helium as the carrier gas in the splitless mode with a DB-5ms capillary column (30 m × 0.25 mm × 0.25 μm). The following conditions were used for the analysis: a 1-μL injection, a source temperature of 230°C and an inlet temperature of 270 °C. The column temperature ramp was as follows: 80 °C for 2.0 min, 80–270 °C at 15 °C/min and 270 °C hold for 10 min.

Fig. S1 UV-vis spectra of 10 μmol L-1 AQ2S, BP and 5 mg L-1 SRNOM

Fig. S2  Possible formation pathways of •OH from the photochemical reaction of phenol after irradiation.

Direct excitation of phenol by light leads to the formation of phenoxyl radical (ArO•) and H radical (Dzengel et al. 1999). The single electron on phenoxyl vibrates onto aromatic carbon resulting i and ii which react with molecular oxygen forming aryldioxyl radical (ArOO•), iii (Li and Chignell 1987). Jiang et al. (2020) have also demonstrated that the ArOO• radical is one of a source of •OH via the intermediate iv. Another pathway for •OH is a secondary photochemical reaction of phenol, p-benzoquinone was the main photoproduct of phenol and its excited state would attack water to produce p-benzosemiquinone and •OH. (Ononye and Bolton 1986; Ononye et al. 1986).

Fig. S3 The photodegradatoin of phenol in the presence of 8 mmol L-1 bromide under different conditions.

Fig. S4 Proposed pathways of sensitized oxidation of halide ions in aqueous solutions.

Reference:

Dzengel J, Theurich J, Bahnemann DW (1999) Formation of nitroaromatic compounds in advanced oxidation processes: photolysis versus photocatalysis. Environ Sci Technol 33:294–300

Li ASW, Chignell CF (1987) Spectroscopic studies of cutaneous photosensitizing agents. 12. Spin trapping study of the free-radicals generated during the photolysis of photoallergens bithionol and fentichlor. Photochem Photobiol 46:445–452

Ononye AI, Bolton JR (1986) Mechanism of the photochemistry of p-benzoquinone in aqueous-solutions.2. Optical flash-photolysis studies. The J Phys Chem 90:6270–6274

Ononye AI, Mcintosh AR, Bolton JR (1986) Mechanism of the photochemistry of pbenzoquinone in aqueous-solutions.1. Spin trapping and flash-photolysis electronparamagnetic resonance studies. The J Phys Chem 90:6266–6270