Supplementary data

Integrated Intelligent Supervisory Controller with Fractional Controller Design for Plant-wide Wastewater Treatment Plants

Abdul Gaffar Sheik1*, Indranil Dey2, Arvind Kumar1, Sheena Kumari1, Faizal Bux1, Seshagiri Rao Ambati3

1Institute for Water and Wastewater Technology, Durban University of Technology, Durban-4001, South Africa.

2Department of Chemical Engineering, National Institute of Technology, Warangal – 506 004, Telangana, India

3Department of Chemical Engineering, Indian Institute of Petroleum and Energy, Visakhapatnam – 530 003, Andhra Pradesh, India

*corresponding author, Email: AbdulS@dut.ac.za

Table T1 State variables of ASM2d, units with notations, and average influent data

| Notation | Parameters | Units | Average influent data |
| --- | --- | --- | --- |
| SO | Dissolved oxygen | g O2/ m3 | 0 |
| SF | Fermentable substrate | g/m3 | 69.9 |
| SA | Acetate | g/m3 | 57.4 |
| SI | Soluble inerts | kg/m3 | 26.5 |
| SNH4 | Ammonium | g/m3 | 25.1 |
| SN2 | Dinitrogen | g/m3 | 0 |
| SNOX | Nitrate plus nitrite | g/m3 | 0 |
| SPO4 | Phosphate | g/m3 | 5.6 |
| SI | Saturation index | kg COD /m3 | 84 |
| Xi | Inert particulate organics | g COD /m3 | 94.09 |
| XS | Sulfate reducing bacteria | kg COD /m3 | 369.9 |
| XH | Heterotrophic biomass | g COD /m3 | 51.5 |
| XPAO | Phosphorus accumulating organisms | g COD /m3 | 0 |
| XPP | Polyphosphates | (g /m3 ) (kmol /m3) | 0 |
| XPHA | Polyhydroxy alkanoates | (g COD /m3) (kg COD /m3) | 0 |
| XA | Autotrophic biomass | g COD /m3 | 0 |
| XTSS | Total suspended solids | g SS/m3 | 374.6 |
| SK | Potassium | (g m/m3) (kmol /m3) | 20 |
| SMg | Magnesium | (g m/m3) (kmol m/m3) | 30 |
| Q | Flow | m3/d | 20648 |
| Temp | Temperature | OC | 15 |
| SNa | Sodium | (g /m3) (kmol /m3) | 175 |
| SCl | Chloride | (g /m3) (kmol /m3) | 300 |
| SCa | Calcium | (g /m3) (kmol /m3) | 60 |
| SSO4 | Sulfate | (g /m3) (kmol /m3) | 0 |
| SFe2 | Iron (II) | (g /m3) (kmol /m3) | 0 |
| SFe3 | Iron (III) | (g /m3) (kmol /m3) | 0 |
| SAl | Alkalinity | kmol/m3 | 0 |
| SIS | Inorganic total sulfides | kg COD /m3 | 0 |
| XHFOL | Hydrous ferric oxide with low number of active sites | (g /m3) (kmol /m3) | 0 |
| XHFOH | Hydrous ferric oxide with high number of active sites | (g /m3) (kmol /m3) | 0 |
| XHFOLP | XHFO_L with bounded adsorption sites | g/m3 | 0 |
| XHFOHP | XHFO_H with bounded adsorption sites | g/m3 | 0 |
| XS0 | Elemental sulfur | (g/m3) (kg/m3) | 0 |
| XSRB | Sulfate-reducing bacteria | kg/m3 | 0 |
| XISS | Inorganic suspended soilds | g SS/m3 | 33.5 |

Table T2 Average data of dynamic mass flow rates

| Dynamic mass flow rates | Average data |
| --- | --- |
| Chemical oxygen demand (COD) | 8386 kg /d |
| Nitrogen | 1014 kg N/d |
| Phosphorus | 197 kg P/d |
| S:COD | 0.003 kg S Kg/COD |

Table T3. Weighting factors for EQI.

| Weighting factors of EQI θi |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Weighting factors | iTSS | iCOD | iNKJ | iNO | iBOD5 | iPorg | iPinorg |
| Value | 2 | 1 | 30 | 10 | 2 | 100 | 100 |

Plant Performance Criteria

Plant assessment formulation

In any event, it serves as an example of how numerous treatment plants may be retrofitted to meet tighter effluent limits by modifying their plant layout. All of the unit operations (primary and secondary settler, biological treatment), and sludge lines (thickener, dewatering, struvite, anaerobic digester) with biological control (SO and ammonia) are taken into consideration. These operations result in modifications to the effluent levels along with process operating expenses (Solon et al., 2017 a, b). To evaluate performance of the WWTP for various control topologies, two performance indices were used. The traditional evaluation criteria of EQI and OCI. This value provides a better understanding of the trade-offs and synergies that various nutrient recovery control strategies can have on GHG (nitrous oxide, carbon dioxide, and methane) emissions. EQI (kg. pollution. unit’s d-1) represents the total pollution leaving the plant and is computed using eq. (1-6) as a weighted sum of effluent TSS, BOD, TKN, COD, NO (oxidized forms of nitrogen, including NO2, NO3, NO, NH2OH, and N2O), and inorganic and organic P (Pinorg and Porg respectively) (Gernaey et al., 2011; 2014; Solon et al., 2017 a, b). The pollution concentrations of each individual estimation are elucidated:

EQI=1to*1000tstarttendθTSS*TSSet+θCOD*CODet+θNKJ*SNKJet+θNO*SNOet+θBOD5*BOD5et+θPorg*SPorget+θPinorg*SPinorget*Qet*dt                                                                           (1)

Here

CODef=SFe+SAe+SIe+XIe+XSe+XB,He+XPAOe+XPHAe+XB,Ae+iCODSFeII*SFeIIe+iCODSIS*SISe+iCODXSO*XSeO+XSRBe                                                                                                                                                 (2)

STKNef=SNHef+iNSFSFef+iNSISIef+iNXIXIef+iNXSXSef+iNBM*XB,Hef+XPAOef+XB,Aef+XSRBef                                                                                                                                                                                       (3)

BOD5ef=0.25SFef+SAef+1-fSI*XSef+1-fXIH*XB,Hef+1-fXIP*XPAOef+XPHAef+1-fXIA*XB,Aef+XSRBef                                                                                                                                                   (4)

SPorgef=XPPef+iPSF*SFef+iPSI*SIef+iPXI*XIef+iPXS*XSef+iPBM*XB,Hef+XPAOef+XB,Aef+XSRBef                                                                                                                                                                                       (5)

SPinorgef=SPO4ef                                                                                                                                                 (6)

TN=STKNef+SNOef                                                                                                                                                              (7)

TP=SPinorgef+SPorgef                                                                                                                                                      (8)

SNH4ef=SNH4ef                                                                                                                                                                       (9)

XTSSef=XTSSef                                                                                                                                                                     (10)

Where tobs denotes the whole evaluation period, and θi denotes the weighting factors used to transform distinct contaminants into general pollution units (Gernaey et al., 2011; 2014; Solon et al., 2017 a, b; Sheik et al., 2022b). those are reported in the supplementary data Table T3. Qe denotes the effluent flow rate in m3/d. From eq. (11), the OCI is computed as a weighted total of the expenses of pumping, mixing, aeration and heating energy, external carbon source, sludge production disposal, chemicals, as well as the possible advantages in the production of methane and nutrients recovered (e.g. struvite).

| OCI=AE+PE+GPS∙SP+GEC .EC+ME-GMP∙MP+max(0,HE-7MP)+GMD .MD+GMeg .Meg-GSrecover. Srecover+AEstripp | (11) |
| --- | --- |

The AE refers to the aeration energy, the PE refers to pump energy, the SP refers to sludge production, and the EC defined by external carbon additions. Me represents mixing energy, MP refers to methane production, HE defined by to AD energy required, MA refers to metal addition, Mg stands for magnesium addition, and Srecovered refers to struvite recovered. The Gi is weighting factors that were chosen in the same way that Gernaey et al. (2014) and Solon et al. (2014) and (2017b). Finally, other regulatory criteria were utilized to determine the plant's performance, such as the percentage of violations in the plant (PV), which refers to when effluent levels exceed discharge limitations for particular nutrients in the effluent. Nevertheless, only the data gathered during the final 364 days of dynamic simulations are used to assess the control techniques that were used. All dynamic simulations (609 days) are accompanied by a steady-state simulation (300 days). A brief explanation of all the parameters of OCI is reported in the supplementary data.

The aeration energy (AE) is described as (kWh/d):

AE= SOsat1800∙Ttotfi=17Vi∙KLait dt                                                                                                                                  12

Where KLai signifies the oxygen mass transfer coefficient, Vi notifies the volume of the reactors and oxygen saturation coefficient. T is the length of evaluation time (364 days)

The pumping energy (PE) is defined as (kWh/d):

PE= 1Ttotf0.004∙Qint+0.008∙Qr+0.050∙Qw+0.075∙QPU+0.060 QTU+0.004∙QDOdt                  (13)

Where Qintr is the internal recycle (m3/d), Qexr is the external recycle (m3/d), Qw is the waste flow (m3/d), QPU is the primary clarifier underflow, QTU is the thickener underflow and QDO is the dewatering overflow.

The sludge production (SP) is expressed as (kg/d):

SP=1T∙1000∙XTSStf-XTSSto+totfTSSXtQX(t) dt                                                                                14

XTSSt= XTSS,  ASUt+XTSS,  SSUt+XTSS,  PSUt+XTSS,  ADUt+XTSS,  SUt

With XTSS,Xt= TSSXt∙VX

Where QX (t) is the sludge flow and TSSX is the total amount of solids in the sludge flow stream (after dewatering in BSM2-P). XTSS is elucidated as the sum of TSS mass present in an individual process unit. The subscripts refer to the concern process units.

The mixing energy (ME) is defined as (kWh/d):

The mixing is highly necessary to avoid the biomass settling in the non-aerated and aerated reactors like all ASU tanks and anaerobic digester and the mixing energy (ME) is defined as (kWh/d):

ME=MEASU+MEADU                                                                                                                                                       (15)

Where

MEASU=24Ttotfi-1i=7if KLai<20d-10.005∙Viif KLai≥20d-10∙dt

MEADU=24∙0.005∙VADU

Where, Vi is the ith tank volume (m3) and 0.005 kW/m3 is the mixing power consumption factor in ASU. VADU is the volume of liquid in ADU and the mixing power consumption factor 0.005 kW/m3.

Methane production (kg CH4/d) is defined as:

The average methane production per day value is defined by using the eq. (5).

MP=Patm∙16T∙R∙TOTtotf1Ptgt∙Pg,CH4t∙Qgt∙dt                                                                                                              (16)

Where, PgCH4 (bar) partial pressure of methane gas produced in the headspace, R denotes the universal gas law constant i.e 8.3145.10-2 bar m3 kmol-1 k-1, TOT represents the operating temperature of the digester (308.15 K), Ptg is the total gas pressure in the headspace, Patm is atmospheric pressure (1.013 bar) and Qg is the gas flow rate of produced gas.

Net heating energy is described as:

HEnet=max0, HE-7∙PM                                                                                                                                        (17)

Where HE is the amount of energy required to get the anaerobic digester up to operating temperature, as shown in the equation below.

HE=2486400∙TtotfPH2O∙CH2O∙TOT-Tadu,it∙Qadt.dt                                                                                    (18)

TADU,i=TPSU∙QPSUt+TTHKt∙QTHKtQADUt

Here, QADUt=QPSUt+QTHKt

Where, PH2O is the density of water (1000 kg/m3), CH2O is the specific heat capacity of water (4.186 KJ kg-1 oC-1). Tad,i is the temperature of ADU influent, TOP is the optimal temperature of ADU. Qad is the flow rate to the ADU (m3/d).

Table T4 Linguistic functions and MF’s for control inputs and outputs.

| Linguistic Variable (Output) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Linguistic value |  | Range | MF | Characteristic ranges |  |  |  |
| 1 |  | Lower | Gaussian bell-shaped shaped | 0.175 | 5.4 | 0.11 |  |
| 2 |  | Medium | Gaussian bell-shaped shaped | 1.06 | 5.87 | 1.36 |  |
| 3 |  | Higher | Gaussian bell-shaped shaped | 3.56 | 18 | 6 |  |
| Linguistic Variable (Input) |  |  |  |  |  |  |  |
| Linguistic value |  | Range | MF | Characteristic ranges |  |  |  |
| 1 |  | Lower | Gaussian bell-shaped shaped | 1.89 | 9.18 | 0.034 |  |
| 2 |  | Medium | Gaussian bell-shaped shaped | 1.02 | 7.75 | 2.96 |  |
| 3 |  | Higher | Gaussian bell-shaped shaped | 8.26 | 42.2 | 12.36 |  |
| - Membership function of the input (ammonia) |  |  |  |  |  |
| - Membership function of output (Dissolved oxygen) |  |  |  |  |  |
| Figure T1 Membership functions of (A) input, and (B) output |  |  |  |  |  |

| AE Aeration energy rate (Kwh/d) ASM Activated sludge model BNR Biological nutrient removal BSM Benchmark simulation model BOD5 Biological oxygen demand COD Chemical oxygen demand CSTR Continuous stirred tank reactor ConcMeg Magnesium added concentration (kg.m-3) Concstru Struvite concentration (kg.m-3) EQI Effluent quality index (kg. polluts. units.d-1) EC External carbon source (kg.d-1) FLC Fuzzy logic controller FOM First order model FPI Fractional proportional integral Fe Iron HE Heating energy (kwh.d-1) GHG Greenhouse gas emission IQI Influent quality index (kg. polluts. units.d-1) Kc Proportional gain KLa Oxygen mass transfer coefficient MD Metal dosages (kg.d-1) ME Mixing energy (kwh.d-1) MP Methane Production (kg CH4.d-1) Meg Magnesium dosage (Kg.d-1) NO Nitrate | PE Pumping energy consumption (kWh/d) HUk Pollutant load corresponding to component Qin Influent flow rate (m3/d) Qintr Internal recycle flow rate (m3.d-1) Qr Return sludge flow rate (m3.d-1) Qw Waste sludge flow rate (m3.d-1) QMeg Flow rate of magnesium added (m3.d-1) Qout Waste sludge flow rate (m3.d-1) Qstru Struvite recovery flow rate (m3.d-1) Srecover Struvite recovery (kg.d-1) SO Dissolved oxygen S Sulphur SOsat Saturation oxygen concentration (kg O2. m-3) SP Sludge production (kg SS.d-1) to Start time tf End time TN Total nitrogen TKN Total Kjeldahl Nitrogen TP Total phosphorus TSS Total suspended solids WWTP Waste water treatment plant Gi Commercial prices (i= Mg, SP, MP, EC, and Srecover) Zi Mass transfer between the liquid and the gas volume (i= CO2, ZMg2, not ZNH4 and ZPO34) |
| --- | --- |

Nomenclature