Supporting information

Fig. S1 The total ion current (TIC) profiles of three mycotoxins using LC-MS/MS analysis in MRM mode: 313.2→285.1 (CE 30.7V, CP17.0V) and 313.2→241.0 (CE 48.2V, CP17.0V) for AFB1, 319.1→283.0 (CE 164V, CP 17.0V) and 319.1→187.0 (CE 25.9V, CP 12.0V) for ZEA, 404.1→239.0 (CE 31.2V, CP 15.0V) and 404.1→358.0 (CE 19.1V, CP 18.0V) for OTA.

Table S1 Response surface analysis (RSA) design chart of two factors at five levels for optimizing UV wavelength and Energy density for PD process of mycotoxins

| Run | A: UV wavelength (nm) | B: Energy density (mJ cm-2) | R1: The fluorescent intensity of AFB1 (mAu) |
| --- | --- | --- | --- |
| 1 | 300 | 15 | 8.52×104 |
| 2 | 400 | 15 | 8.51×104 |
| 3 | 300 | 35 | 8.57×104 |
| 4 | 400 | 35 | 8.71×104 |
| 5 | 300 | 15 | 9.21×104 |
| 6 | 400 | 15 | 9.26×104 |
| 7 | 300 | 35 | 9.35×104 |
| 8 | 400 | 35 | 9.51×104 |
| 9 | 200 | 25 | 8.89×104 |
| 10 | 500 | 25 | 9.03×104 |
| 11 | 350 | 5 | 8.70×104 |
| 12 | 350 | 50 | 8.85×104 |
| 13 | 350 | 25 | 8.66×104 |
| 14 | 350 | 25 | 9.33×104 |
| 15 | 350 | 25 | 9.07×104 |
| 16 | 350 | 25 | 9.25×104 |
| 17 | 350 | 25 | 9.11×104 |
| 18 | 350 | 25 | 9.15×104 |
| 19 | 350 | 25 | 9.12×104 |
| 20 | 350 | 25 | 9.25×104 |

Fig. S2 the iterations plots of three algorithm, ATLD, PARAFAC and MCR-ALS

Table S2 the test of goodness of fit between the resolved results (matrix A) and real normalized spectra of targets using ATLD, PARAFAC and MCR-ALS algorithm in different number of factors

| Number of factors (n) | ATLD |  |  |  | PARAFAC |  |  |  | MCR-ALS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AFB1 | ZEA | OTA |  | AFB1 | ZEA | OTA |  | AFB1 | ZON | OTA |
| 4 | 82.44% | 88.41% | 92.03% |  | 66.83% | 64.19% | 76.85% |  | 62.62% | 70.50% | 79.43% |
| 5 | 89.71% | 88.45% | 94.84% |  | 78.04% | 69.00% | 77.18% |  | 70.86% | 75.30% | 79.46% |
| 6 | 92.48% | 86.29% | 96.94% |  | 71.23% | 84.17% | 80.94% |  | 76.60% | 87.77% | 92.07% |
| 7 | 99.47% | 99.20% | 99.65% |  | 99.07% | 99.35% | 99.08% |  | 99.13% | 99.60% | 99.01% |
| 8 | 99.14% | 99.03% | 99.74% |  | 90.30% | 86.09% | 88.86% |  | 89.98% | 92.29% | 84.29% |
| 9 | 99.33% | 99.93% | 99.81% |  | 87.95% | 96.45% | 85.58% |  | 86.11% | 84.19% | 89.34% |
| 10 | 99.38% | 99.33% | 99.53% |  | 90.76% | 90.65% | 86.11% |  | 83.10% | 77.54% | 82.01% |

Table S3 Calibration parameters, limit of detection and quantitation (LOD and LOQ) for mycotoxin analysis using LC-MS/MS using clean-up with a multi-antibody immunoaffinity column (IAC).

| Target | Concentration range (μg kg-1) | Slope | Intercept | R2 | LOD (μg kg-1) | LOQ (μg kg-1) |
| --- | --- | --- | --- | --- | --- | --- |
| AFB1 | 1-50 | 1.5×104 | 2.8×103 | 0.99 | 0.3 | 1.0 |
| ZEA | 5-200 | 1.7×104 | 3.1×103 | 0.99 | 2.0 | 5.0 |
| OTA | 5-200 | 1.3×104 | 2.1×103 | 0.99 | 0.6 | 2.0 |

Table S4 the recovery accuracies of five mycotoxins spiked in the matrices of table grape, raisin, wine grape and wine using LC-MS/MS with IAC-cleanup.

| Matrix type | AFB1/AFB2a |  |  |  | ZEA |  |  |  | OTA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Spiked (μg kg-1) | Found (μg kg-1) | Recoverya |  | Spiked (μg kg-1) | Found (μg kg-1) | Recoverya |  | Spiked (μg kg-1) | Found (μg kg-1) | Recoverya |
| Table grape | 0 | 0.06 | - |  | 0 | 0.05 | - |  | 0 | 0.05 | - |
|  | 2 | 2.24 | 109.0% |  | 10 | 8.08 | 80.3% |  | 10 | 8.12 | 80.7% |
|  | 5 | 3.96 | 72.0% |  | 20 | 16.42 | 81.9% |  | 20 | 16.39 | 81.7% |
|  | 10 | 7.55 | 74.9% |  | 50 | 44.07 | 88.0% |  | 50 | 40.33 | 80.1% |
| Raisin | 0 | 0.05 | - |  | 0 | 0.03 | - |  | 0 | 0.03 | - |
|  | 2 | 1.74 | 84.5% |  | 10 | 8.57 | 85.4% |  | 10 | 7.96 | 79.3% |
|  | 5 | 4.07 | 80.4% |  | 20 | 16.63 | 83.0% |  | 20 | 15.95 | 79.6% |
|  | 10 | 8.11 | 80.6% |  | 50 | 40.02 | 80.0% |  | 50 | 41.88 | 83.7% |
| Wine grape | 0 | 0.11 | - |  | 0 | 0.21 | - |  | 0 | 0.08 | - |
|  | 1 | 1.25 | 114.0% |  | 5 | 3.58 | 67.4% |  | 5 | 4.09 | 80.2% |
|  | 2 | 1.63 | 76.0% |  | 15 | 12.50 | 81.9% |  | 15 | 12.71 | 84.2% |
|  | 5 | 4.05 | 79.8% |  | 20 | 17.44 | 86.2% |  | 20 | 16.33 | 81.2% |
| Wine | 0 | 0.07 | - |  | 0 | 0.19 | - |  | 0 | 0.01 | - |
|  | 1 | 1.12 | 105.0% |  | 5 | 4.01 | 76.4% |  | 5 | 3.83 | 76.4% |
|  | 2 | 1.45 | 69.0% |  | 15 | 11.99 | 78.7% |  | 15 | 12.66 | 84.3% |
|  | 5 | 4.07 | 80.0% |  | 20 | 16.52 | 81.7% |  | 20 | 15.92 | 79.6% |

a: The recovery is calculated by the equation: Ci-C0Cspiked×100%, Ci and C0 is the concentration of mycotoxin in spiked and unspiked samples, respectively. Cspikes is the added concentration.