Supplementary Table 1

Table 1. Parameters of MRI Sequence

| Sequence | Plane | TR | TE | Slice thickness | Gap between slices | NEX | FOV | Matrix size | Sequence |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T2WI with fat suppression | coronal | 1280 | 85 | 3-4 | 1 | 4 | 18 | 288×192 | FRFSE |
| T1WI | axial | 460 | 8 | 3-4 | 0.5 | 2 | 25 | 288×192 | FSE |
| T2WI with fat suppression | axial | 3000 | 85 | 3-4 | 0.5 | 4 | 25 | 320×224 | FRFSE |
| DWI | axial | 6550 | minimum | 3-4 | 0.5 | 6 | 25 | 128×128 | SS-EPI |
| Contrast-enhanced T1WI | axial | 5.7 | 1.7 | 3-4 | 0.5 | 1 | 25 | 192×256 | FSPGR |
| Units |  | ms | ms | mm | mm |  | cm |  |  |

Abbreviations: TR, repetition time; TE, echo time; NEX, number of excitations; FOV, field of view; T2WIWI, T2WI-weighted image; T1WI, T1-weighted image; DWI, diffusion-weighted imaging; FRFSE, fast recovery fast spin echo; FSE, fast spin echo; SS-EPI, single shot echo-planar imaging; FSPGR, fast spoiled gradient echo.

Combined thresholds

The parameters of ADCmin and DWI_SIR were combined to provide a single integrated diagnostic marker. Thresholds were established by receiver operating characteristic analysis (ROC) curves to maximize the sensitivity and specificity of benign and malignant lesions. The threshold for ADCmin is 1.13×10-3 mm2/sec and for DWI_SIR is 1.20. If both ADCmin was less than 1.13×10-3 mm2/sec and DWI_SIR was greater than 1.20, the combination threshold is determined to be malignant; otherwise, it is determined to be benign. Similarly, instead of ADCmin combined DWI_SIR with ADC_SIR to provide a single integrated diagnostic marker. The threshold for ADC_SIR is 1.25. If both ADC_SIR was less than 1.25 and DWI_SIR was greater than 1.20, the combination threshold is determined to be malignant; otherwise, it is determined to be benign. The diagnostic efficacy of the combined thresholds was then calculated and compared with ACR-TIRADS.

Supplementary Table 2

| Model | P |
| --- | --- |
| Prediction model VS Combined thresholds DWI_SIR and ADC_SIR | 0.036* |
| Prediction model VS Combined thresholds DWI_SIR and ADCmin | 0.500 |
| Combined thresholds DWI_SIR and ADC_SIR VS Combined thresholds DWI_SIR and ADCmin | 0.018* |
| ACR TIRADS VS Prediction model | 0.050* |
| ACR TIRADS VS Combined thresholds DWI_SIR and ADC_SIR | 0.475 |
| ACR TIRADS VS Combined thresholds DWI_SIR and ADCmin | 0.041* |

Abbreviations: DWI, diffusion weighted imaging; ADC, apparent diffusion coefficient;  AUC, area under the receiver operating characteristic curve; SIR, signal intensity ratio of lesion to contralateral-normal thyroid tissue; ACR-TIRADS, American College of Radiology Thyroid Imaging Reporting and Data System.