# Materials and Methods (Online)

## 1. Study Design

This study investigated the use of the Sentry Raman spectroscopy System for intraoperative use in 67 adult patients undergoing open brain surgery at the Montreal Neurological Institute-Hospital (MNI-H, Montreal, Canada) and Mount Sinai Hospital (MSH, New York, USA). The cohort included patients with glioblastoma, metastatic cancer, and meningioma (Table S2). Forty-nine patients were recruited from the MNI-H and 18 from MSH. The study was approved by the Ethics Review Boards at MNI-H (ODS Sentry System-1000 / 2019-5313) and MSH (HS #: STUDY-20-01371), and informed consent was obtained from all subjects. The methods were carried out in accordance with the approved guidelines and regulations. Standard clinical imaging prior to surgery by magnetic resonance imaging (MRI) was followed, as well as a complete preoperative neurologic examination. The surgeons were blinded to information about the in situ spectral fingerprint measurements acquired during surgery.

## 2. Handheld Raman spectroscopic probe

The Sentry System from Reveal Surgical (Montreal, Canada) was used. It was composed of a handheld probe connected to a near-infrared (NIR) laser and a spectrometer through a fibre optic cable of length 3 meters. The probe was sterilizable, reusable and had the shape of a stylet of length 12 cm. There is a conical tip of outer diameter 2.1 mm where the instrument contacts the tissue. The probe contains 9 light detection optical fibres that are circumferentially distributed around one optical fibre dedicated to tissue laser excitation. A lens at the tip of the probe ensured that both the laser spot size on the tissue surface and the area viewed through the detection fibres had a diameter of 500 micrometres. The excitation fibre was connected to a NIR spectrum-stabilized continuous-wave laser emitting at 785 nm with a maximum power of 350 mW (Innovative Photonic Solutions, Plainsboro, NJ, USA). Light scattered within the tissue and re-emitted from its surface was detected using a charge-coupled device (CCD) sensor (Newton model, Andor Technology, Belfast, UK) through a spectrometer slit of width 100 micrometres and a volume phase diffraction transmission holographic grating (Emvision LLC, model EM-VPHG-50.8-6002). The sensor was pre-cooled to -80oC before being used in surgery. Each spectrum acquired with the system covered a range of spectral shifts from 400 to 2000 cm−1, with a spectral resolution of approximately 1.8 cm−1. A preliminary laboratory version of the instrument from which the Sentry System was designed has been described in Jermyn et al.1

## 3. Raman spectral acquisition and intraoperative workflow

The probe was steam sterilised prior to intraoperative use and spectral fingerprint detection. An average of 30 spectra (minimum number: 1, maximum number: 80, standard deviation: 15) were acquired during each neurosurgical procedure (Figure 1A). The number of spectral fingerprints collected for each patient is also shown graphically as individual dots in Figure S7A, where each band of a different colour (either grey or white) represents a different patient. Region-of-interest selection for each measurement was based on pre-operative information from magnetic resonance imaging (MRI) and visual assessment by the surgeon using a surgical microscope (OPMI Pentero or Kinevo model, Zeiss, Germany). The study design ensured the number of measurements made in tumor and non-tumoral brain was balanced. During brain tumor surgery, it is common to remove non-pathological brain as part of the tumor resection. In this study, that non-pathological brain was interrogated prior to resection.

For each spectral fingerprint acquisition, the probe was placed in direct contact with the tissue and the light source of the surgical microscope was momentarily turned off. Each spectrum consisted of 20 successive spectra (repeat measurements at the same location) that were averaged to increase the signal-to-noise ratio. Each successive spectrum was obtained with a laser power of 75 mW at the probe tip with a 100 ms acquisition time. For each measurement, a biopsy was taken from the same region that approximately had the shape of a cylinder with a diameter of 0.5 mm and a height of 3 mm.

## 4. Histopathology analyses and sample classification

Each biopsy sample was fixed in formalin, embedded in paraffin, and sectioned prior to deposition onto a glass slide. Three serial sections of each specimen were cut at a thickness of 4 micrometres. Each section was stained using haematoxylin and eosin (H&E). All sections were visualised using brightfield microscopy and a trained neuropathologist. Samples used in this study were those classified as either tumor, if they contained only bulk tumor, or non-tumoral brain, if no tumor cells were present. 675 were tumor and 666 samples were non-tumoral brain (Table S1). From bulk tumor and non-tumor tissue, 553 spectral fingerprints were acquired in patients with glioblastomas (518 at MNI-H, 35 at MSH), 313 in patients with metastatic cancer (243 at MNI-H, 70 at MSH) and 475 in patients with meningiomas (446 at MNI-H, 29 at MSH).

5. Power studies

A power analysis was conducted to estimate the number of samples required to determine the likelihood that basic statistical tests (e.g., t-test) could find a statistically significant difference between non-tumoral brain and tumor tissue (either glioblastoma, metastasis, or meningioma). The software G*Power was used to perform the analysis.2 The computation was based on a moderate effect size of 0.5 which is consistent with prior Raman spectroscopy studies.3,4 The effect size was computed based on the average and standard deviation associated with the Raman bands at 1441 cm-1 (lipids and proteins) and 1004 cm-1 (phenylalanine).The computation revealed that the development of two-class models (e.g., non-tumoral brain versus glioblastoma) required 100 measurements per category for a statistical power of 1 -  = 95% and a value  of 0.05, where  and  are Type I and Type II errors, respectively.

A posteriori analysis of the data presented in this manuscript led to an effect size >1.8 for the models associated with specific pathologies and 1.12 for the models discriminating non-tumoral brain from tumors of any kind. All models trained/validated and tested in this study were associated with more than 100 samples per category, effectively guaranteeing a statistical confidence >95% in our ability to reject the null hypothesis, namely that the spectral fingerprints associated with non-tumoral measurements are different than the measurements made in tumor tissue.

6. Spectral fingerprint measurements and intraoperative workflow

### 6.1 Spectral pre-processing

For data analysis, Python 3.7.10 with Scikit-Learn 1.0.2 were used. Code repository for spectral pre-processing is publicly available in the paper "Open-sourced Raman spectroscopy data processing package implementing a novel baseline removal algorithm validated from multiple datasets acquired in human tissue and biofluids" Sheehy et al., Journal of Biomedical Optics, 28 (2), 025002 (2023)5 and also on Github (https://github.com/mr-sheg/orpl).

The following standard data pre-processing steps were applied to each spectroscopic measurement (Figure S2)6: 1) subtraction of a ‘dark count’ background measurement acquired with the laser turned off prior to each repeat acquisition (i.e., laser-off background), 2) removal of cosmic ray events, 3) truncation of pixels with lower Raman scattering photonic counts (400-800 cm-1, all wavenumbers above 1750 cm-1), resulting in a spectrum with 521 spectral bins, 4) x-axis calibration using the known positions of Raman peaks from a reference material (polycarbonate resin sample7), 5) instrument response correction from spectral measurements acquired from a calibration material (NIST 785 nm Raman standard), 6) averaging of 20 successive measurements acquired at the same location, 7) baseline subtraction using the BubbleFill algorithm8 with a minimum ‘bubble’ diameter of 60 cm-1, 8) curve smoothing using a Savitzky-Golay filter of order 3 with a window size of 11 and 9) standard normal variate (SNV) normalization.

The BubbleFill algorithm is an iterative procedure that grows ‘bubbles’ with a diameter ranging from the full spectrum’s length up to a pre-set minimum size.5 The diameter is expressed in wavenumber units (cm-1). To avoid user bias and ensure the pre-processing process could be automatically applied uniformly to the whole dataset (prior to machine learning), no fine tuning of the threshold minimum size was done. Rather, it was pre-set to correspond to the width in cm-1 of a Raman band ubiquitously observed in all collected Raman spectra, namely the lipid/protein band around 1441 cm-1. This methodological aspect of the study may explain differences in band ratios when comparing the spectra in this study with other Raman spectroscopy work studying brain.9,10

### 6.2 Spectral quality factor

A spectral quality factor (QF) metric was computed for each SNV-normalized spectral fingerprint. It consisted of a number of maximum value 1 quantifying the likelihood the signal was associated with a random probability distribution.8 A random signal would have had a value of QF close to 0 while signals containing Raman spectroscopy (inelastic scattering) information were associated with QF > 0. Lower QF measurements were associated with lower inelastic scattering photonic counts and higher levels of stochastic noise, reducing their ability to reliably capture the spectral fingerprint of the tissue. The quality factor (QF) metric used in this work was defined as the average signed squared intensity:5

where r is an SNV-normalized Raman spectrum and sgn(x) is the sign function of x, returning -1 or 1 depending on whether x is negative or positive, respectively. Examples of individual spectra (i.e., one location in the brain for one patient) are shown corresponding to a low QF value (Figure S2E) and a high QF value (Figure S2F). The QF value of all spectral fingerprints acquired as part of this study are shown (Figure S7A) along with the actual individual spectra for non-tumoral and tumor samples, in the form of spectrograms (Figure S7B, Figure S7C).

6.3 Machine learning models

Machine learning models were developed for the detection of glioblastoma, metastatic cancer, or meningioma, and one all-encompassing tumor detection model was developed from all measurements, independent of tumor type (Figure S1). Each of the four classification models was developed from a training set composed of 80% of the spectral fingerprint measurements from the MNI-H and MSH. For each model, a testing set (i.e., holdout set) associated with the remaining 20% of all spectral fingerprint measurements was held out to evaluate the performance of the models on an independent dataset. The constitution of the testing sets was such that they had approximately the same percentage of samples from MNH-H and MSH patients as in the training sets. All samples from a given patient were either in the training or the holdout set, to remove potential biases arising from sharing patient samples between the training/validation and testing phases.

Prior to machine learning model training/validation and testing, a Gaussian fitting technique was applied to each spectral fingerprint measurement that was described in Plante et al.11 Briefly, this technique fitted a Gaussian function on any peak with a prominence of 0.1, a height of 0.5 (relative to the lowest value in the SNV-normalized spectrum), and a tolerance of 2 cm-1 on the position of the peak, considering that the Raman spectrum intensity ranges from -2 to 7 in normalized intensity (SNV normalisation). Only the peaks that were present in 50% of all measurements were retained as potential features.11 This procedure extracted the position in wavenumbers, the height, and the width of up to 11 different peaks. The specific number of peaks retained depended on the pathology type, i.e., on which machine learning model was trained. The height and width of those peaks (up to 22 variables in total) –herein labelled the peak features– along with the relative intensity of the 521 individual bands within each spectrum, constituted the set of potential spectral features from which machine learning models could be trained. Prior to model training/validation and testing, the number of features was reduced to include only those that contributed the most to the variance between non-tumoral brain and tumor. This feature selection step was accomplished using a random forest algorithm with 200 estimators where the maximum number of features (N) was the only floating hyperparameter.12 This technique was used by our group in multiple Raman spectroscopy publications, both for cancer detection in tissue13 and for biofluid interrogation to detect COVID-19 infection.14 The feature selection process is essentially a dimensional reduction step implemented prior to machine learning model training/validation and testing. A different method that is commonly used by other Raman spectroscopy groups is principal component analysis (PCA).15

Machine learning model training from the dimensionally-reduced features set was done using linear SVM with the regularization parameter C. Unbalanced classes in each model are accounted for with a class weight parameter adjusted to reflect the ratio between non-tumoral and tumoral brain samples.13,16  Each time a model was trained, hyperparameters were selected by carrying out a grid search across many combinations (N, C). The regularization parameter C was varied between 0.01 and 5, the number of individual band features was varied between 5 and 25 and the number of peak features varied between 2 to 20, such that N (i.e., the total number of features) ranged between 7 and 45. For each combination, performance was assessed using five-fold cross validation based on the number of false/true positives and false/true negatives, by comparing the model prediction with the assigned pathological label (tumor or non-tumoral brain). Specifically, the training dataset was split into five non-overlapping subsets (folds). Each fold consisted in training a model from 4 of the 5 subsets, while the remaining subset (validation set) was used to assess performance. This resulted in one set of hyperparameters (N, C) (i.e., a model) that minimized the number of false positive and false negative predictions. The final model was applied to the holdout data subset and performances were reported as a receiver operating characteristic (ROC) analysis. Accuracy, sensitivity, and specificity were calculated from the ROC curve and the ROC curve area under curve (AUC) was reported.

Two sets of predictive models were developed, one set without any QF threshold (i.e., no spectral quality cutoff) applied to the spectral fingerprint data and one keeping only higher quality data. Models with no QF threshold consisted of i) 188 non-tumoral brain and 365 glioblastoma samples, ii) 194 non-tumoral brain and 119 metastasis samples, iii) 284 non-tumoral brain and 191 meningioma samples, and iv) a total of 666 non-tumoral brain and 675 tumor samples (Figure S8). Higher quality models consisted of i) 110 non-tumoral brain and 267 glioblastoma samples, ii) 137 non-tumoral brain and 107 metastases samples, iii) 173 non-tumoral brain and 191 meningioma samples, and iv) a total of 420 non-tumoral brain and 565 tumor samples (Figure 2, Figures S4–S7). The higher quality dataset consisted of spectra with QF > 0.5 for glioblastoma and metastatic patients and QF > 0.3 for meningioma patients.

References

1.	Jermyn, M. et al. Intraoperative brain cancer detection with Raman spectroscopy in humans. Science Translational Medicine 7, 1–10 (2015).

2.	Faul, F., Erdfelder, E., Buchner, A. & Lang, A.-G. Statistical power analyses using G*Power 3.1: tests for correlation and regression analyses. Behav Res Methods 41, 1149–1160 (2009).

3.	David, S. et al. In situ Raman spectroscopy and machine learning unveil biomolecular alterations in invasive breast cancer. Journal of Biomedical Optics 28, 036009 (2023).

4.	Lemoine, É. et al. Feature engineering applied to intraoperative in vivo Raman spectroscopy sheds light on molecular processes in brain cancer: a retrospective study of 65 patients. Analyst 144, 6517–6532 (2019).

5.	Sheehy, G. et al. Open-sourced Raman spectroscopy data processing package implementing a baseline removal algorithm validated from multiple datasets acquired in human tissue and  biofluids. J Biomed Opt 28, 025002 (2023).

6.	Grosset, A. A. et al. Identification of intraductal carcinoma of the prostate on tissue specimens using Raman micro-spectroscopy: A diagnostic accuracy case–control study with multicohort validation. PLoS Medicine 17, 1–20 (2020).

7.	de Groot, S., Van Oosten, Th. B., Te Nijenhuis, B., Lagana, A. & Keneghan, B. RSR00014, Polycarbonate. Infrared and Raman Users Group Spectral Database www.irug.org (2013).

8.	Sheehy, G. et al. Open-sourced Raman spectroscopy data processing package implementing a novel baseline removal algorithm validated from multiple datasets acquired in human tissue and biofluids. Journal of Biomedical Optics (Submitted).

9.	Auner, G. W. et al. Applications of Raman spectroscopy in cancer diagnosis. Cancer Metastasis Rev 37, 691–717 (2018).

10.	Aguiar, R. P., Falcão, E. T., Pasqualucci, C. A. & Silveira, L. Use of Raman spectroscopy to evaluate the biochemical composition of normal and tumoral human brain tissues for diagnosis. Lasers in Medical Science 37, 121–133 (2022).

11.	Plante, A. et al. Dimensional reduction based on peak fitting of Raman micro spectroscopy data improves detection of prostate cancer in tissue specimens. J Biomed Opt 26, (2021).

12.	Breiman, L. Random Forests. Machine Learning 45, 5–32 (2001).

13.	Fabien Picot et al. Image-guided Raman spectroscopy navigation system to improve transperineal prostate cancer detection. Part 1: Raman spectroscopy fiber-optics system and in situ tissue characterization. Journal of Biomedical Optics 27, 095003 (2022).

14.	Ember, K. et al. Saliva-based detection of COVID-19 infection in a real-world setting using reagent-free Raman spectroscopy and machine learning. Journal of biomedical optics 27, 25002 (2022).

15.	Jermyn, M. et al. A review of Raman spectroscopy advances with an emphasis on clinical translation challenges in oncology. Physics in Medicine & Biology 61, R370 (2016).

16.	David, S. et al. Multispectral label-free Raman spectroscopy can detect ovarian and endometrial cancer with high accuracy. J Biophotonics 15, e202100198 (2022).