Supplementary Materials

Experimental and theoretical revealing of piezo-photocatalyst Bi2O2CO3 for ciprofloxacin degradation in water

Xiaofeng Sun1, Guorong Liu2, Ruishan Li2, Liexiao Li2, Jianfeng Dai2, Hua Yang1,2*

1State Key Laboratory of Advanced Processing and Recycling of Non-ferrous Metals, Lanzhou University of Technology, Lanzhou 730050, China

2School of Science, Lanzhou University of Technology, Lanzhou 730050, China

Characterization of Bi2O2CO3

Scanning electron microscopy (SEM) observation was performed on a JSM-6701F field-emission scanning electron microscope (JEOL Ltd.). Transmission electron microscopy (TEM) analysis was performed on JEM-1200EX field-emission transmission electron microscope (JEOL Ltd.). X-ray powder diffraction (XRD) analysis was carried out on a Bruker D8 Advance X-ray powder diffractometer with Cu Kα radiation (λ = 0.15406 nm). Fourier transform infrared (FTIR) spectroscopy analysis was carried out on a Spectrum Two FTIR spectrophotometer (PerkinElmer) in the range of 500–4000 cm−1 using KBr powder. Ultraviolet-visible diffuse reflectance spectroscopy (UV-vis DRS) measurement was done on a TU-1901 double beam spectrophotometer (Beijing Purkinje General Instrument Co. Ltd.). X-ray photoelectron spectroscopy (XPS) investigation was performed a PHI-5702 multi-functional X-ray photoelectron spectrometer (Physical Electronics) with Mg Kα radiation (hv = 1253.6eV and power = 250 W). Electron paramagnetic resonance (EPR) signals were monitored on a Bruker microESR spectrometer. Liquid chromatography‒mass spectrometry (LC‒MS) analysis was performed on a thermos TSQ Quantum Ultra mass spectrometer and Agilent 1100 liquid chromatograph.

DFT calculation

Vienna Ab-initio Simulation Package (VASP) software was used to calculate the band structure and density of states of Bi2O2CO3 based on the density functional theory (DFT) [1,2]. The interaction between electrons and ions was described by the projection augmented plane wave (PAW) method and Perdew–Burke–Ernzerhof (PBE) exchange correlation functional within a generalized gradient approximation (GGA). A truncation energy of the valence electron plane was set as 400 eV and the first Brillouin zone was sampled using a 6 × 1 × 6 Monkhorst-Pack k-points mesh. In our calculations, the lattice parameters of Bi2O2CO3 are a = 5.55541 Å, b = 28.02602 Å, c = 5.53552 Å and α = β = γ = 90.0000o. In addition, the convergence for energy was chosen as 10−5 eV and the structural optimization was obtained until the Hellmann–Feynman forces acting on each atom was less than 0.03 eV/Å.

For calculating the elasticity matrix (cE), relative permittivity matrix (εr), piezoelectric coupling matrix (eES) and volume density (ρ) of Bi2O2CO3, the POSCAR, POTCAR and KPOINTS files are consistent with those in the state density calculation, but the INCAR files are different. In the process of calculating cE, IBRION = 6, NFREE = 2, ISIF = 3 and ENCUT = 650 eV. In the process of calculating eES and εr, LEPSILON = T, LPEAD = T, KSPACING = 0.2, LREAL = Auto, ISIF=3, SIGMA = 0.1 and ENCUT = 400 [3].

Additionally, the q(N−1) (the molecule lost an electron), q(N) (the neutral molecule), q(N+1) (the molecule getting an electron), the Laplacian bond order (LBO), the lowest unoccupied molecular orbital (LUMO) and highest occupied molecular orbital (HOMO) of CIP were calculated by the Multiwfn software [4,5]. The condensed Fukui index including the nucleophilic (f +), electrophilic (f −), radical attack (f 0) and condensed dual descriptors (CDD) were calculated and applied to evaluate the reactive sites on the CIP molecule.

FEM simulation

The finite-element method (FEM) simulation involved a dynamic 3D analysis of a piezoelectric device using COMSOL Multiphysics software. The parameters cE, εr and eES used for the simulation of piezoelectric potential distributions in the Bi2O2CO3 nanoplates are presented in the body text. The volume density of Bi2O2CO3 is ρ = 6860 kg/m3. The center of the Bi2O2CO3 crystals was fixed and grounded. The polarization was aligned along the global co-ordinate z-axis.

Reference:

[1] G. Kresse, J. Furthmüller, Efficiency of ab-initio total energy calculations for metals and semiconductors using a plane-wave basis set, Comp. Mater. Sci. 6 (1996) 15‒50.

[2] G. Kresse, J. Furthmüller, Efficient iterative schemes for ab initio total-energy calculations using a plane-wave basis set, Phys. Rev. B 54 (1996) 11169.

[3] Y.L. Page, P. Saxe, Symmetry-general least-squares extraction of elastic data for strained materials from ab initio calculations of stress, Phys. Rev. B 65 (2002) 104104.

[4] T. Lu, F. Chen, Multiwfn: A multifunctional wavefunction analyzer, J. Comput. Chem. 33 (2012) 580–592.

[5] T. Lu, F. Chen, Bond order analysis based on the Laplacian of electron density in fuzzy overlap space, J. Phys. Chem. A 117 (2013) 3100–3108.

Table S1. Fathead minnow LC50, daphnia magna LC50, development toxicity and mutagenicity of CIP and its degradation intermediates analyzed by Toxicity Estimation Software Tool (TEST, version 5.1.1).

| Compounds | Fathead minnow LC50 (96 h) (mg/L) | Daphnia magna LC50 (48 h) (mg/L) | Development toxicity | Mutagenicity |
| --- | --- | --- | --- | --- |
| CIP | 0.19 | 2.84 | 1.09 | 0.62 |
| P1 | 0.31 | 58.06 | 1.22 | 0.58 |
| P2 | 0.14 | 6.59 | 1.01 | 0.39 |
| P3 | 0.13 | 16.08 | 1.02 | 0.47 |
| P4 | 0.20 | 10.36 | 0.90 | 0.29 |
| P5 | 0.45 | 1.21 | 0.69 | 0.36 |
| P6 | 3208.38 | 85.13 | 0.59 | 0.71 |
| P7 | 36.59 | 39.05 | 0.34 | 0.08 |
| P8 | 31128.35 | 23217.87 | 0.50 | 0.012 |
| P9 | 14.80 | 18.77 | 0.36 | 0.00 |
| P10 | 18.68 | 12.11 | 0.43 | 0.00 |
| P11 | 7.56 | 5.37 | 0.46 | 0.00 |
| P12 | 232.00 | 241.22 | 0.71 | -0.02 |
| P13 | 227.57 | 28.90 | 0.66 | -0.05 |
| P14 | 13493.12 | 9614.39 | 0.49 | 0.23 |
| P15 | 762.37 | 1035.97 | 0.71 | -0.06 |
| P16 | 0.23 | 18.80 | 1.02 | 0.58 |
| P17 | 0.38 | 28.36 | 0.92 | 0.37 |
| P18 | 0.21 | 9.73 | 0.96 | 0.34 |
| P19 | 0.41 | 2.52 | 0.89 | 0.57 |
| P20 | 9.95 | 273.32 | 0.74 | 0.13 |
| P21 | 117.12 | 966.21 | 0.69 | 0.03 |
| P22 | 52.64 | 567.54 | 0.87 | 0.14 |
| P23 | 152.30 | 76.33 | 0.64 | 0.06 |
| P24 | 946.72 | 437.38 | 0.37 | 0.07 |
| P25 | 1401.32 | 677.53 | 0.51 | 0.28 |
| P26 | 0.27 | 51.24 | 1.13 | 0.63 |
| P27 | 0.44 | 315.87 | 0.99 | 0.48 |
| P28 | 2.78 | 97.53 | 0.53 | 0.05 |
| P29 | 7.24 | 41.84 | 0.73 | 0.21 |
| P30 | 7.10 | 45.03 | 0.77 | 0.48 |
| P31 | 23.07 | 1146.50 | 0.72 | 0.21 |
| P32 | 57.70 | 1629.24 | 0.65 | 0.28 |
| P33 | 4355.71 | 1283.45 | 0.43 | 0.29 |
| P34 | 404.63 | 549.65 | 0.72 | 0.08 |

Fig. S1. Mass spectral database for piezo-photocatalytic decomposition products of CIP over Bi2O2CO3 nanoplates.