Supplementary Information for

Three-dimensional single-particle tracking by interferometric scattering microscopy with a double-helix point spread function

Mingchuan Huang1†, Qiankun Chen1†, Yang Liu1, Chi Zhang2, Rongjin Zhang2, Junhua Yuan2*, and Douguo Zhang2*

1Advanced Laser Technology Laboratory of Anhui Province, Department of Optics and Optical Engineering, University of Science and Technology of China, Hefei, Anhui, 230026, China

2Department of Physics, University of Science and Technology of China, Hefei, Anhui, 230026, China

†Mingchuan Huang and Qiankun Chen contributed equally to this work.

*Correspondence and requests for materials should be addressed to: dgzhang@ustc.edu.cn (Douguo Zhang) or jhyuan@ustc.edu.cn (Junhua Yuan).

Section 1: Numerical calculations of the iSCAT microscopy image

The calculation procedure for the iSCAT microscopy image can be divided into two steps. In the first step, the near-field electromagnetic field distribution is obtained using the Finite-Difference Time-Domain (FDTD) method. In the second step, the electric field at the back focal plane (BFP) is obtained through a near-to-far-field transformation 1. The iSCAT image (front focal plane image, FFP) is obtained through operating a two-dimensional Fourier transformation of previous BFP image taking account of the phase delay and polarization selection that are induced by the quarter waveplate and the polarizer.

Step 1: The simulation the near-field distribution

As shown in Figure S1, the perfectly matched layer (PML) absorbing boundaries are used in three dimensions, the boundaries form a simulation area with a size of 110 μm × 110 μm × 5 μm. Two light sources are used in the simulation. One is a Gaussian beam polarized along the X-axis and is focused by a lens of numerical aperture (NA) 0.13. The dimensions of this light source are set as 110 μm × 110 μm, at a height of Z = 1.2 μm. As illustrated in Figure S1, the beam wavelength was set at 635 nm, and the surrounding medium was a vacuum. The other light source is also a Gaussian beam with polarization orientation along the Y-axis, and its phase is 90o ahead of the first source. The two linearly-polarized light sources are combined to simulate the circularly polarized light. Below the light source is a polystyrene particle of 500 nm in diameter (refractive index n = 1.59), whose center is located at Z = 0.25 μm. The particle is placed on a well-designed dielectric multilayer chip (alternative 20 pairs of 59 nm thick silicon nitride layer (n = 2.53) and 72 nm thick silicon dioxide layer (n = 1.46)), the structural dimensions of the dielectric multilayer chip were the same as those used in the experiments. The monitor is placed in the glass (n = 1.52), whose size is 110 μm × 110 μm, located at Z = -3 μm. This monitor can get the near-field electric-field distribution  of the iSCAT signal.

Step 2: The calculation of electric field distribution at the BFP  and FFP .

As shown in Figure S2, the calculation of far field  can be derived through near-to-far-field transformation. According to the vector ray tracing 2, 3, the electric field at the BFP of the imaging objective lens  is expressed,

|  |  | (S1.1) |
| --- | --- | --- |
|  |  | (S1.2) |

where  is the transformation matrix , matrix describes the coordinate transformation for rotation around the Z-axis and  (azimuthal angle) is the angle of the meridional plane to the X-axis.

|  |  | (S1.3) |
| --- | --- | --- |

where the matrix  describes the refraction of the ray as it traverses a lens and  (radial angle) is the angle by which the ray is rotated in a direction perpendicular to the meridional plane:

|  |  | (S1.4) |
| --- | --- | --- |

The  can be decomposed into two components  in X-direction and  in Y-direction, and the  is then superimposed with a phase delay of 90o to simulate the function of quarter wave plate (QWP) used in the experiment. And after an analyzer that is set to an angle  with respect to the X-axis, the electric field  can be written as:

|  |  | (S1.5) |
| --- | --- | --- |

Finally, the electric field  at the image plane is obtained through a two-dimensional Fourier transformation

|  |  | (S1.6) |
| --- | --- | --- |

where  is the lateral coordinates on the imaging plane of the tube lens and represents the lateral coordinates on the momentum space.

Section 2: Theoretical analysis on the spin-to-orbital angular momentum conversion enhanced by the dielectric multilayer chip

Let’s consider an optical imaging system by using vector ray tracing, the light scattered by single particle passes through a multilayer or a glass substrate and is collected by a high numerical aperture (NA) objective lens. On a circular coordinate basis 4, the transformation matrix of the electric field takes the form

|  |  | (S2.1) |
| --- | --- | --- |
|  |  | (S2.2) |

The angle and polarization dependent transmittance through the glass or multilayer substrate is described by the matrix :

|  |  | (S2.3) |
| --- | --- | --- |

The matrix  is a unitary transformation from the Cartesian to circular basis:

|  | (S2.4) |
| --- | --- |

In our work, is left circularly polarized (LCP, incident beam),  is right circularly polarized (RCP, output beam), then the transfer function  can be given as following:

|  |  | (S2.5) |
| --- | --- | --- |

From Eq. (S2.5), it is noticed that the transfer function exhibits a nontrivial topological charge of two 5. Based on the angular momentum conservation law, the conversion from the LCP incident beam to RCP output beam is induced by the spin-to-orbital angular momentum conversion, where the spin angular momentum of the incident beam is conversed to the orbital angular momentum (OAM) of the RCP light scattering from a single particle of interest. Based on this equation, the conversion efficiency is dependent on the value , the higher this value, the higher conversion efficiency. According to the calculated curves shown in Figure 1f, the chip substrate increases the coefficient .

Section 3: Detailed descriptions on the data analysis procedures for the captured iSCAT microscopy images

The captured original iSCAT image of a particle that is in the vicinity of the focal plane is shown in Figure S5a, which show two lobes. On this image, the non-uniform background sometimes hinders the determination of the center of the two-lobes. In order to extract the position information of the two lobes of the double-helix pattern, a threshold value is defined that is of an appropriate ratio vs the brightest pixel intensity in the image. The rule of selecting this specific threshold value is to ensure that two lobes in the image have significant areas while avoiding their fusion 6. Only the pixels with intensities higher than this threshold value are retained, while the others are discarded. Subsequently, the intensity-weighted centroids of the two lobes are individually calculated, then the tilt angle of the two lobes can be derived, as shown in Figure S5b.

The two lobes rotate with the axial-location of this particle, so we can estimate the axial-location of the particle through the calibration curve of tilt angle vs. axial-location as shown in Figure 4a4 and 4b4. The X and Y positions of this particle can be derived through the midpoint of two lobes, and the Z position can be derived by mapping the tilt angle of lobes to its axial-location.

Due to the enhanced imaging contrast by the multilayer chip, the interference patterns of a particle are still clearly when it is far away from the focal plane of the objective lens, as shown in Figure S6a, making it is possible to determine its axial location. A cross-sectional analysis on this iSCAT image is conducted and the distance (D) between two selected points on this white-line is defined for extracting the axial location of this particle, as shown in Figure S6b. It is found that the value of D is changed with the axial location of the particle, as shown in Figure 4c5, and 4d5.

In addition, the rotating directions of this spiral fringes above and below the focal plane are reversed (Figure 2j and 2k), which provides a simple method to determine whether the particle is above or below the focal plane. In this case, the Z-position of this particle is extracted through distance of these two points (D), and the lateral coordinates X and Y were then obtained from the center position of this spiral pattern.

Section 4: The estimation of localization precision

The above is the qualitative description, a more precise explanation is presented here. Eqs. S1.1 and S1.2 can be rewritten as the vector field forms: We recorded the 3D positions of four particles at different positions 50 times and the histograms of the spatial coordinates are shown in Figure S8, from which we determined the standard deviations of the particles’ positions. When the particles located within a range of 2 μm near the focal plane, the standard deviations were estimated as , , which can be considered as the localization precision of a single measurement 6, 7.

The standard deviations of particles at a distance about Z = -5 μm and Z = 20 μm from the focal plane (Z = 0 μm) were , respectively. In the future, the localization precision can be further improved through a more stable optical system, drift correction and other improvements.

Section 5: Theoretical analysis on formation of the axial-location dependent point spread function of the iSCAT microscopy

The intensity of the particle image  is the sum of the transmitted background, the signal, and their interference, which can be written as

|  |  | (S5.1) |
| --- | --- | --- |

where , , and are the transmitted light, the forward scattering signal of the particle, and their relative phase difference, respectively.

The transmitted light is a weakly focused Gaussian beam, approximately a plane wave, and the phase of transmitted light  can be expressed as:

|  |  | (S5.2) |
| --- | --- | --- |

Whereis the spherical wavefront of transmitted light， is the Gouy phase and  is the Rayleigh range of the transmitted Gaussian beam.

The light scattered by particle has a second-order phase vortex, which is approximately a second-order Laguerre Gaussian beam 6, 8, 9, 10. The phase of scattered light  can be written as

|  |  | (S5.3) |
| --- | --- | --- |

Where  is the second-order phase vortex,is the spherical wavefront of scattered light， is the Gouy phase and  is the Rayleigh range of the Laguerre Gaussian beam.

Hence the total phase difference  can be written as:

|  |  | (S5.4) |
| --- | --- | --- |

In the vicinity of the focus, as the axial position of the particle changes, the Gouy phase shifts. Thus, the two lobes generated by interference between scattered light and transmitted light, will rotate around the optical axis (Z-axis). In the area far away from the focus (such as Z = -15 μm or Z = 15 μm), the spherical wavefront of the scattered light above and below the focal plane are opposite, as a result, the rotating directions of spiral fringes are reversed (as shown in Figure 2j and 2k, Figure 3g and 3h)

References for SI reference citations

1.	Yang JJ, Hugonin JP, Lalanne P. Near-to-Far Field Transformations for Radiative and Guided Waves. Acs Photonics 3, 395-402 (2016).

2.	Foreman MR, Török P. Computational methods in vectorial imaging. Journal of Modern Optics 58, 339-364 (2011).

3.	Torok P, Higdon PD, Wilson T. On the general properties of polarised light conventional and confocal microscopes. Optics Communications 148, 300-315 (1998).

4.	Bliokh KY, Ostrovskaya EA, Alonso MA, Rodriguez-Herrera OG, Lara D, Dainty C. Spin-to-orbital angular momentum conversion in focusing, scattering, and imaging systems. Opt Express 19, 26132-26149 (2011).

5.	Long OY, Guo C, Wang H, Fan S. Isotropic topological second-order spatial differentiator operating in transmission mode. Opt Lett 46, 3247-3250 (2021).

6.	Pavani SR, et al. Three-dimensional, single-molecule fluorescence imaging beyond the diffraction limit by using a double-helix point spread function. Proc Natl Acad Sci U S A 106, 2995-2999 (2009).

7.	Thompson RE, Larson DR, Webb WW. Precise nanometer localization analysis for individual fluorescent probes. Biophys J 82, 2775-2783 (2002).

8.	Pavani SR, Piestun R. Three dimensional tracking of fluorescent microparticles using a photon-limited double-helix response system. Opt Express 16, 22048-22057 (2008).

9.	Piestun R, Schechner YY, Shamir J. Propagation-invariant wave fields with finite energy. J Opt Soc Am A Opt Image Sci Vis 17, 294-303 (2000).

10.	Schechner YY, Piestun R, Shamir J. Wave propagation with rotating intensity distributions. Phys Rev E Stat Phys Plasmas Fluids Relat Interdiscip Topics 54, R50-R53 (1996).

Figure S1. Schematic diagram of simulation model for calculating the near-field distribution of the iSCAT signals.

Figure S2. Schematic diagram of simulation model for calculating the electric field distribution at the BFP and FFP of the iSCAT signals.

Figure S3. The measurement of back focal plane (BFP) images. (a, b) the measured experimentally angular-dependent distributions of the total electric field intensity (ITotal) from the single particle when the particle was placed on the glass (a) or chip substrate (b), here the combination of QWP2 and P2 was removed from the optical setup. (c, d) the measured experimentally angular-dependent distributions of the RCP light (IRCP) for the particle on the glass (c) or chip substrate (d), where the combination of QWP2 and P2 was placed before the camera to allow only RCP light passing through. The appearance of the center bright spots (c, d) can be attributed to the depolarization effects during the imaging process, and also the directly transmission of the illumination LCP light that has not been fully blocked by the combination of QWP2 and P2.

Figure S4. The comparisons of the out-of-focus iSCAT microscopy images. (a) The out-of-focus iSCAT images of the particle (on the glass substrate) with its axial position changing from Z = -20 μm to 20 μm on the glass substrate. They are the conventional iSCAT images without used of images postprocessing.  (b) The out-of-focus iSCAT images of the particle (with a pair of nearly-orthogonal circular-polarizers on the chip substrate, the proposed iSCAT microscopy) with its axial position changing from Z = -20 μm to 20 μm.

Figure S5. Calibration method for particle that is locating near the focal plane of the objective lens. (a) The original iSCAT image with a double-helix pattern. (b) The iSCAT image after threshold setting and lobes weighting center processing. The particle is in the vicinity of the focal plane. Scale bar, 1 µm.

Figure S6. Calibration method for particle that is far-away from the focal plane of the objective lens. (a)The iSCAT image of a particle with distance far away from the focal plane of the objective lens (Z = 12.5 μm), a value D is used to defined the distance between two points along the white line on this image. (b) The intensity profile along the white line on (a). Scale bar, 1 µm.

Figure S7. (a-h) A series of iSCAT images with orientation of the polarizer 2 changing from 35o to -35o, when the particle is above (Z = 3 μm) the focal plane of the imaging objective. Scale bar, 1 µm.

Figure S8. Histogram of 50 localizations of four particles at different position in X, Y, and Z. (a, b) The histogram of particles located within a range of 2 μm near the focal plane of the objective lens, (c, d) The histogram of particles at an axial location about Z = -5 μm and Z = 20 μm from the focal plane (Z = 0 μm).

Figure S9. (a-c) The iSCAT images with three-helix pattern when the sample is below (Z = 3 μm), on (Z = 0 μm) or above (Z = 3 μm) the focal plane of the imaging objective. Scale bar, 1 µm.

Figure S10. (a-c) The iSCAT images of particle with diameter 200 nm when the particle is below (Z = -3 μm), on (Z = 0 μm) or above (Z = 3 μm) the focal plane of the imaging objective. (d-f) The iSCAT images of particle with diameter 100 nm when the particle is below (Z = -3 μm), on (Z = 0 μm) or above (Z = 3 μm) the focal plane of the imaging objective. Scale bars, 1 µm.

Video 1: The demonstration of the rotation of the double helix pattern with the axial location of the particle (near the focal plane), and the formation of the calibration curve (numerical simulation results).

Video 2: The demonstration of the rotation of the double helix pattern with the axial location of the particle (near the focal plane), and the formation of the calibration curve (Experimental results).

Video 3: The demonstration of the evolution of the double spiral pattern with the axial location of the particle (in the long axial-range of -20 μm to 20 μm), and the formation of the calibration curve (numerical simulation results).

Video 4: The demonstration of the evolution of the double spiral pattern with the axial location of the particle (in the long axial-range of -20 μm to 20 μm), and the formation of the calibration curve (experimental results).

Video 5: The formation of the 3D trajectories of the particle attached to the filament stub of a living bacterium.