| Table 2 Morphometric parameters and their standard formulae Sl. No. | Morphometric Parameter | Formula | Variables | Reference |
| --- | --- | --- | --- | --- |
| 1 | Relative Relief (Rr) | Rr = Hmax - Hmin | Hmax = Maximum Elevation and Hmin = Minimum Elevation | Strahler (1952) |
| 2 | Relief Ratio (Rh) | Rh = Rr/Lb | Lb = Basin Length | Schumm (1956) |
| 3 | Basin Circularity (Rc) | Rc = 4πA/P2 | A = Basin Area, P = Perimeter of the basin, π = 3.142 | Schumm (1956) |
| 4 | Elongation Ratio (Re) | Re = 2*(A/π)0.5/Lb | A = Basin Area, π = 3.142, Lb = Basin Length | Bull and McFadden (1977) |
| 5 | Form Factor (Rf) | Rf = A/Lb 2 | A = Basin Area, Lb = Basin Length | Horton (1932) |
| 6 | Dissection Index (DI) | DI = Rr / Hmax | Rr = Relative Relief and Hmax = Maximum Elevation | Nir (1957) |
| 7 | Bifurcation Ratio (Rb) | Rb = Nu/Nu+1 | Nu = Number of streams of order u, Nu+1 = Number of streams of order u+1 | Horton (1945) |
| 88 | Stream Length Ratio (Rl) | Rl =Lu/Lu-1 | Lu = Length of streams of order u, Lu-1 = Length of streams of order u-1, | Horton (1945) |
| 9 | Texture Ratio (T) | T = N/P | N = Number of streams of all orders, P = Perimeter of the basin | Strahler (1957) |
| 10 | Hypsometric Integral (HI) | HI = (Hmean – Hmin)/ (Hmax – Hmin) | Hnean = Mean elevation of the basin, Hmin = Minimum elevation of the basin and Hmax = Maximum elevation of the basin | Pike and Wilson (1971) |
| 11 | Slope(θ) | 1st derivative of DEM surface |  | Evans (1980) |
| 12 | Curvature (Cv) | 2nd derivative of DEM surface |  | Evans (1980) |
| 13 | Drainage density (Dd) | Dd = Lu / A | Lu = Total length of all the streams in the basin, A = Basin area | Horton (1945) |
| 14 | Stream frequency (Fs) | Fs = Nu / A | Nu = Total number of all streams, Wa = Basin area | Horton (1945) |
| 15 | Constant of Channel Maintenance (CCM) | CCM = 1/Dd | Dd = Drainage Density | Schumm (1956) |
| 16 | Length of Overland Flow (LOF) | LOF = 1/Dd *2 | Dd = Drainage Density | Horton (1945) |
| 17 | Infiltration Number (IN) | IN = Dd * Sf | Dd = Drainage Density Sf = Stream frequency | Faniran (1968) |
| 18 | Ruggedness Number (Rn) | Rn = (Rr * Dd) / 1000 | Rr = Relative Relief, Dd = Drainage Density | Melton (1965) |
| 19 | Topographic Wetness Index (TWI) | TWI = ln [As / tan(θ)] | As = Specific Catchment Area derived from the accumulation matrix, θ = Slope in radians | Moore and Burch (1986) |
| 20 | Stream Power Index (SPI) | SPI = ln [As * tan (θ)] | As = Specific Catchment Area derived from the accumulation matrix, θ = Slope in radians | Moore et al. (1991) |