Supplementary Materials for

Optics miniaturization strategy for demanding Raman spectroscopy applications

Oleksii Ilchenko1,2, Yurii Pilhun2,3, Andrii Kutsyk2,3,4, Denys Slobodianiuk3,4, Yaman Goksel1, Elodie Dumont1, Lukas Vaut1, Chiara Mazzoni1, Lidia Morelli1, Tomas Rindzevicius1, Thomas Emil Andersen6, Mikael Lassen7, Hemanshu Mundhada8, Christian Bille Jendresen8, Peter Alshede Philipsen9, Merete Hædersdal9, Anja Boisen1

1Technical University of Denmark, Department of Health Technology, Center for Intelligent Drug Delivery and Sensing Using Microcontainers and Nanomechanics, Kgs. Lyngby 2800, Denmark

2Lightnovo ApS, Birkerød 3460, Denmark

3Taras Shevchenko National University of Kyiv, Kyiv 01601, Ukraine

4Institute of Magnetism, Kyiv 03142, Ukraine

5Technical University of Denmark, Department of Energy Conversion and Storage, Kgs. Lyngby 2800, Denmark

6 Department of Clinical Microbiology, Odense University Hospital and Research Unit for Clinical Microbiology, University of Southern Denmark, Odense, Denmark

7 Danish Fundamental Metrology, Hørsholm 2970, Denmark

8Cysbio ApS, Hørsholm 2970, Denmark

9Department of Dermatology, Copenhagen University Hospital, Copenhagen 2400, Denmark

Correspondence to: olil@dtu.dk

This PDF file includes:

Supplementary Text

Figs. S1 to S28

Tables S1 to S5

Captions for Movie S1

Captions for Data S1 to S7

References

Supplementary Text

S1 “Mode hop” deconvolution.

Blurred signal can be written as convolution of PSF and real (ideal) signal:

fx=-∞∞psfx-χfχdχ=psf*f#1

In matrix form

f=Apsff=Af⋅psf#2

The above equation represents symmetry properties of convolution. It allows us to estimate PSF using ideal and blurred signals. The obtained PSF can be used for the deblurring of measured signals.

Since measured blurred signal contains noise, the solution should be found in sense of least squares.

The PSF estimation problem can be rewritten as classical least squares problem:

psf=argminxy-Ax22#3

To constrain result to be non-negative, NNLS should be used:

psf=argminxy-Ax22subject to x≥0#4

These problems are ill-posed and regularization steps are required. We considered several types of regularization1:

- L1 regularization

psf=argminxy-Ax22+λ2 x12#5

- L2 regularization (robust regression)

psf=argminxy-Ax22+λ2 x22#6

- L2 smooth regularization

psf=argminxy-Ax22+λ2D2x22#7

Here D2 is a second derivative matrix.

When the ‘ideal’ reference channel spectrum is available, the ‘mod hop’ deconvolution algorithm is the following:

- Considering the reference channel spectrum at each step as blurred one, we can obtain PSF using the ‘ideal’ reference spectrum.

- Data channel spectrum is deblurred using the obtained PSF.

We used the same value of the regularization parameter λ=0.1. As the example, the ‘ideal’ reference channel spectrum (Fig. S2) and blurred reference channel spectrum (Fig. S3) were used to estimate the PSF (Fig. S4). The deblurred data channel spectrum is shown in Fig. S5. We used different techniques of regularization. The best result is obtained for L2 smooth regularization (Eq. 7)

The same procedure were applied to the time map (Fig. S6).

S2. Spectral deblurring

Diamond peak at 1332 cm-1 (Fig. S7) was used as for the estimation of PSF. Raman spectrum of diamond was baseline corrected using asymmetrical least squares2 (Fig. S8). Parameters used for baseline estimation is the following: λ=104,p=0.001. The obtained PSF is shown in Fig. S9.

We used L2 nonnegative regularization (Eq. 6). FISTA algorithm implemented in IRTools3 was used to solve this problem. Raman spectrum of toluene was used to test this deconvolution approach (Fig. S10). Background was estimated using asymmetric least squares with λ=105,p=0.001. The obtained results are shown in Fig. S11. It worth to be noted that background correction is a crucial step. Otherwise, additional artefacts might occur. As it can be seen from the Fig. S12, such approach allows increasing spectral resolution in 1.5 times.

Fig. S1. Raw data and reference spectra.

Fig. S2. Reference channel spectrum used as ideal.

Fig. S3. ‘Raw’ and ‘ideal’ reference channel spectra

Fig.S4. The estimated PSF.

Fig. S5. Deblurred vs. Raw data channel spectra

Fig. S6. Deblurred time maps

Fig. S7. Raman spectrum of diamond. The highlighted area is used for the PSF estimation.

Fig. S8. The calculated baseline.

Fig. S9. The estimated PSF.

Fig. S10. Raman spectrum of toluene

Fig.S11. Raw and deblurred spectra of toluene.

Fig. S12. Peak width at half height for the raw and deblurred spectra. They were peak height normalized.

Fig. S13. Raman spectra of water ethanol solution with different concentration of methanol. Data obtained at laser excitation 785nm.

Fig. S14. Methanol quantification results obtained from laser excitation 785nm. Methanol detection was performed by PLS regression using MVC1 toolbox4,5.

Fig. S15. Raman spectra of water ethanol solution with different concentration of methanol. Data obtained at laser excitation 675nm.

Fig. S16. Methanol quantification results obtained from laser excitation 675nm. Methanol detection was performed by PLS regression using MVC1 toolbox4,5.

Fig. S17. PLS calibration based on Raman data for liquid samples of pHCA diluted in PBS

Fig. S18. Raman spectra of the different concentrations of Serine diluted in PBS

Fig. S19. Peak intensity spatial distribution of MTX

Fig. S20. Photograph of silver coated NP SERS chips inserted into solution with MTX.

Fig. S21. Map averaged MTX spectra.

Fig. S22. Background corrected spectra. Background was eliminated using asymmetrical least squares with λ=105,p=0.01 with further mean-centering. Spectral range 626-1412 cm-1 was used for calibration.

Fig. S23. MTX PLS calibration results. Analysis was performed by PLS regression using MVC1 toolbox4,5.

Fig. S24. MTX. PLS validaiton results. Analysis was performed by PLS regression using MVC1 toolbox4,5.

Fig. S25. ML of bacteria data

Fig. S26. ML of bacteria data

Fig. S27. The optomechanical design of miniaturized Raman spectrometer with spectral resolution 18cm-1 in the range 400-2000cm-1 operated from laser wavelength 785nm.

Fig. S28. Photograph of the prototype of miniaturized Raman spectrometer with spectral resolution 18cm-1 in the range 400-2000cm-1 operated from laser wavelength 785nm.

Tables

Table S1. Methanol quantification from 785nm

| Parameter | Value |
| --- | --- |
| RMSECV | 0.511 |
| RMSEP | 0.4411 |
| R2 prediction | 0.998 |
| LOD min | 0.2746 |
| LOD max | 0.7739 |
| LOQ min | 0.8238 |
| LOQ max | 2.3217 |

Table S2. Methanol quantification from 675nm

| Parameter | Value |
| --- | --- |
| RMSECV | 0.1 |
| RMSEP | 0.4411 |
| R2 prediction | 1.0 |
| LOD min | 0.0711. |
| LOD max | 0.1607. |
| LOQ min | 0.2134 |
| LOQ max | 0.4822 |

Table S3. pHCA calibration Raman

| pHCA concentration (µM) | 1 | 2 |  | average | std dev |
| --- | --- | --- | --- | --- | --- |
| 1000 | 47.2105 | 47.2451 |  | 47.2278 | 0.024466 |
| 500 | 24.4478 | 24.402 |  | 24.4249 | 0.032385 |
| 250 | 12.546 | 12.5231 |  | 12.53455 | 0.016193 |
| 125 | 6.278 | 6.2729 |  | 6.27545 | 0.003606 |
| 62.5 | 3.10259 | 3.102102 |  | 3.102346 | 0.000345 |
| 31.25 | 1.6234 | 1.6227 |  | 1.62305 | 0.000495 |

Table S4. Serine calibration Raman

| g/L | Intensity | Intensity | Intensity |
| --- | --- | --- | --- |
| 50 | 102 | 103 | 105 |
| 25 | 56 | 55.5 | 56 |
| 12.5 | 28 | 29 | 28.5 |
| 6.25 | 16 | 17 | 16.8 |
| 1 | 5 | 5.8 | 5.3 |

Table S5. MTX quantification by SERS

| RMSECV | 8.7 |
| --- | --- |
| RMSEP | 4.0475 |
| R2 pred | 0.9931 |
| LOD min | 2.9842 |
| LOD max | 6.875 |
| LOQ min | 8.9527 |
| LOQ max | 20.6251 |

References

- Eilers, P. H. C. Penalties in Chemometrics. in Encyclopedia of Analytical Chemistry 1–12 (Wiley, 2020). doi:10.1002/9780470027318.a9658.

- Eilers, P. H. C. & Boelens, H. F. M. Baseline Correction with Asymmetric Least Squares Smoothing. https://zanran_storage.s3.amazonaws.com/www.science.uva.nl/ContentPages/443199618.pdf (2005).

- Gazzola, S., Hansen, P. C. & Nagy, J. G. IR Tools: a MATLAB package of iterative regularization methods and large-scale test problems. Numer Algorithms 81, 773–811 (2019).

- Olivieri, A. C., Goicoechea, H. C. & Iñón, F. A. MVC1: An integrated MatLab toolbox for first-order multivariate calibration. Chemometrics and Intelligent Laboratory Systems 73, 189–197 (2004).

- Olivieri, A. & Olivieri, A. C. Introduction to Multivariate Calibration A Practical Approach Introduction to Multivariate Calibration Introduction to Multivariate Calibration.

Movie S1.

Demonstration of SORS performance in experiment of measuring whiskey content through the glass bottle

Data S1.

Kinetic Raman experiments that demonstrate real-time calibration based on reference channel for laser excitation wavelengths 675 and 785nm.

Data S2.

Quantification of methanol in water-ethanol solution

Data S3.

Raman spectra and PLS quantification of pHCA, Glucose and salts

Data S4.

Raman spectra and PLS quantification of Serine

Data S5.

Raman spectra of human skin.

Data S6.

SERS spectra, calibration and validation data for MTX quantification.

Data S7.

Raw Raman spectra of bacteria obtained via Raman mapping