# Supplementary Information

# Towards carbon-neutral cities: an assessment of urban CO2 removal and albedo management

Authors: Quirina Rodriguez Mendeza,b*, Felix Creutziga,c, Sabine Fussa,b, Sarah Lücka

- Mercator Research Institute on Global Commons and Climate Change, Berlin, Germany

- Geographisches Institut, Humboldt-Universität zu Berlin, Berlin, Germany

- Technische Universität Berlin, Berlin, Germany

*Corresponding author: Rodriguez@mcc-berlin.net

# Index

I.	Urban Vegetation	1

II.	Biochar	6

i. Urban Soils	7

ii. Additive to Construction Materials	12

III.	Wood in Construction	18

IV.	Decentralised Direct Air Capture	20

V.	Surface Albedo Management	24

VI.	Co-Benefits	28

VII.	Barriers to Implementation	30

Bibliography	33

## Urban Vegetation

Table 1: Carbon content estimates of urban vegetation strategies in the reviewed studies. Values reported in units of C have been included and converted to CO2.

| Reference | Scale | Location | Metric |  | Reported Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Annual Sequestration | Cumulative Storage | C | CO2 |
| Urban Forests & Parks |  |  |  |  |  |  |
| (Nowak et al., 2013) | National | US | ✓ |  | 0.28 tC/ha·yr | 10.3 tCO2/ha·yr |
| (Nowak et al., 2013) | National | US |  | ✓ (per area of cover) | 76.9 tC/ha | 282 tCO2/ha |
| (Kiss et al., 2015) | City | Szeged, Hungary | ✓ |  |  | 2.32 tCO2/ha·yr |
| (Shadman et al., 2022) | Park | Dhaka, Bangladesh | ✓ |  |  | 19.5 tCO2/ha·yr |
| (Othman, 2019) | Urban district | Selangor, Malaysia | ✓ |  |  | 10.7 tCO2/ha·yr |
| (Trlica et al., 2020) | City | Boston, US | ✓ |  |  | 1.8 (0 – 11.3) tCO2/ha·yr |
| (Almeida et al., 2018) | Park | São Paulo, Brazil | ✓ |  |  | 10 tCO2/ha·yr |
| (C. Liu & Li, 2012) |  | Shenyang, China | ✓ |  | 2.84 tC/ha·yr | 10.41 tCO2/ha·yr |
| (Reynolds et al., 2017) | City | Medellin, Colombia | ✓ |  |  | 0.89 tCO2/ha·yr |
| (Reynolds et al., 2017) | City | Medellin, Colombia |  | ✓ (per area of cover) |  | 48.2 tCO2/ha |
| (Escobedo et al., 2010) | City | US | ✓ |  |  | 2.57 (Gainesville) & 0.855 (Miami) tCO2/ha·yr |
| (Nagar et al., 2019) |  | Delhi, India | ✓ |  |  | 3.45 tCO2/ha·yr |
| (Fan & Wei, 2022) | Regional | Hangzhou, China | ✓ |  | 1.45 tC/ha·yr | 3.45 tCO2/ha·yr |
| (M. Zhao et al., 2010) | City | Hangzhou, China |  |  | 1.66 tC/ha·yr | 6.09 tCO2/ha·yr |
| (Milnar & Ramaswami, 2020) | National (11 Cities) | US | ✓ |  |  | 0.87 to 2.1 tCO2/ha·yr sequestered (-3.4 to 9.8 tCO2/ha·yr emitted from land conversion) |
| (Fares et al., 2020) | Parks | Italy | ✓ |  |  | 1 (Rome) & 0.5 (Turin) tCO2/ha·yr |
| (W. Y. Chen, 2015) | National | China (35 cities) | ✓ |  |  | Average of 2.16 tCO2/ha·yr (total 1.9 MtCO2/yr) |
| (Y. Wang et al., 2021) | City | Beijing, China | ✓ |  | 20.6 tC/ha·yr | 75.5 tCO2/ha·yr |
| (Jiang et al., 2020) | City | Shanghai, China |  | ✓ (per area of cover) | 44.8 tC/ha | 164.3 tCO2/ha·yr |
| (Moussa et al., 2019) | City | Niger |  | ✓ (per area of cover) | 34 (Nimey) & 66.5 (Maradi) tC/ha | 125 (Niamey) & 243.8 (Maradi) tCO2/ha |
| (Pradhan et al., 2022) | Park | Cooch Behar, India (Himalayas) |  | ✓ (per area of cover) | 485.55 tC/ha | 1780.5 tCO2/ha |
| (Muluneh & Worku, 2022) | City | Dessie city, Ethiopia |  | ✓ (per area of cover) |  | 2734.79 ± 627.06 tCO2/ha |
| (Richter et al., 2020) | City | Berlin, Germany |  | ✓ (per area of cover) | 270 tC/ha | 990 tCO2/ha |
| (Pregitzer et al., 2022) | City | New York, US |  | ✓ (per area of cover) | 263.04 tC/ha | 964 tCO2/ha |
| (D.-H. Lee et al., 2019) | City | Seoul, South Korea |  | ✓ (per area of cover) | 63.19 tC/ha | 231.7 tCO2/ha |
| (Nero, 2017) | City | Kumasi metropolis, Ghana. |  | ✓ (per area of cover) | 270 ± 22 tC/ha | 990 ± 81 tCO2/ha |
| (Raciti et al., 2014) | City | Boston, US |  | ✓ (per area of cover) | 28.8 tC/ha | 105.6 tCO2/ha |
| (Lindén et al., 2020) | City | Helsinki, Finland |  | ✓ (per area of cover) | 22.1 – 28.1 tC/ha | 81 – 103 tCO2/ha |
| (Strohbach et al., 2012) | Park | Leipzig, Germany |  | ✓ (per area of cover) |  | 137 – 162 tCO2/ha |
| (Veerkamp et al., 2020) | Continental | Europe (775 cities) |  | ✓ (per area of cover) | 87.59 tC/ha | 321.2 tCO2/ha |
| (Cortinovis et al., 2022) | City | Spain; Sweden; The Netherlands |  | ✓ (per area of cover) |  | 269 (Barcelona), 318 (Malmö), & 369 (Utrecht) tCO2/ha |
| (Dorendorf et al., 2015) | City | Hamburg, Germany |  | ✓ (per area of land) | 123.2 tC/ha | 451.7 tCO2/ha |
| (Y.-C. Wang & Lin, 2012) | National | Taiwan |  | ✓ (per area of land) | 9.25 tC/ha | 33.9 tCO2/ha |
| (Mexia et al., 2018) | City | Almada, Spain |  | ✓ (per area of land) | 262.4 tC/ha | 962 tCO2/ha |
| (Timilsina et al., 2014) | City | Florida, US |  | ✓ (per area of land) | 63 tC/ha | 231 tCO2/ha |
| (Z. G. Davies et al., 2011) | City | Leicester, UK |  | ✓ (per area of land) | 31.6 tC/ha | 115.9 tCO2/ha |
| Urban Afforestation in available areas |  |  |  |  |  |  |
| (Chakraborty et al., 2022) | State | California, US | ✓ |  |  | Increase by 4.5 MtCO2/yr |
| (Brilli et al., 2022) | Municipality | Prato, Italy | ✓ |  |  | Increase by 18 ktCO2/yr |
| (Teo et al., 2021) | Global |  | ✓ |  |  | Increase by 82.4 ± 25.7 MtCO2/year |
| (Mandal et al., 2022) | City | Asansol-Durgapur, India |  | ✓ (per area of cover) | 41.77 tC/ha | No quantitative information on annual increase. |
| (Sandre et al., 2021) | City | São Paulo |  |  |  | No quantitative information on C: ”Potential to increase vegetation cover by 30%.” |
| Street trees |  |  |  |  |  |  |
| (Lin et al., 2022) | City | New York, US |  | ✓ (per area of cover) | 13.8 tC/ha | 50.6 tCO2/ha |
| (Tang et al., 2016) |  | Beijing, China | ✓ |  | 0.5 ± 0.3 tC/ha·yr | 1.8 ± 1.1 tCO2/ha·yr |
| (O’Donoghue & Shackleton, 2013) | Parking | South Africa | ✓ |  | 0.39 ± 2.5 tC/ha | 1.4 ± 9.2 tCO2/ha·yr |
| Green Roofs |  |  |  |  |  |  |
| (Heusinger & Weber, 2017) | City | Berlin, Germany | ✓ |  |  | 0.313 kgCO2/m2·yr |
| (George, 2012) | Roof | Not location-specific | ✓ |  |  | 1.22 kgCO2/m2·yr |
| (Cascone et al., 2018) | City | Catania, Italy | ✓ |  |  | 1.35 kgCO2/m2·yr |
| (Kuronuma et al., 2018) | Roof | Japan | ✓ |  |  | 1.70 – 1.89 kgCO2/m2·yr |
| (Whittinghill et al., 2014) | Roof | Michigan, US | ✓ |  | 3.13 – 9.82 kgC/m2·yr | 11.5 – 36.4 kgCO2/m2·yr |
| (Luo et al., 2015) | Roof | Chengdu, China | ✓ |  | 4.73 – 7.11 kgC/m2·yr | 17.3 – 26.1 kgCO2/m2·yr |
| (Ondoño et al., 2016b) | Roof | Spain | ✓ |  | -4.6 – 4.4 kgC/m2·yr (from source to sink) | -16.9 – 16.1 kgCO2/m2·yr (from source to sink) |
| (Ondoño et al., 2016a) | Roof | Spain | ✓ |  | 1.9 – 4.4 kgC/m2·yr | 7.0 – 16.1 kgCO2/m2·yr |
| (Kuronuma & Watanabe, 2017) | Roof | Japan | ✓ |  | 0.28 – 0.67 kgC/m2·yr | 1.0 – 2.5 kgCO2/m2·yr |
| (Sultana et al., 2021) | City | Dhaka, Bangladesh | ✓ |  |  | 7.6 – 46 kgCO2/m2·yr |
| (Getter et al., 2009) | Roof | Michigan, US | ✓ |  |  | 0.3 kgCO2/m2·yr (0.2 kgCO2/m2 over 2 years) |
| (Ismail et al., 2012) | Roof | Malaysia | ✓ |  | 0.23 kgC/m2·yr | 0.84 kgCO2/m2·yr |
| (Peng & Jim, 2015) | Roof | Hong Kong | ✓ |  |  | 9.4 kgCO2/m2·yr |

## Biochar

Table 2: Carbon content of biochar from reviewed literature. If available, two different metrics referring to biochar’s carbon content are included. A negative sign denotes a carbon sink.

| Reference | Biomass Feedstock | Carbon Content |  |
| --- | --- | --- | --- |
|  |  | (%C) | (kgCO2eq/kg biochar) |
| (Kumar Mishra et al., 2023) | Multiple (Review study) | 41.90 to 87.19% | − 1.54 to − 3.20 kgCO2eq/kg biochar |
| (Uday et al., 2022) | Multiple (Review study) | 8.7 to 92.9% | − 0.69 to − 3.41 kgCO2eq/kg biochar |
| (Alhashimi & Aktas, 2017) | Multiple (Meta-analysis) |  | Average of − 0.9 kgCO2eq/kg biochar (ranges from − 0.1 for cardboard biochar to − 3.5 for grass-derived biochar) |
| (Azzi et al., 2022) | Multiple (wood pellets, garden waste, logging residues, willow chips) | 69.9 to 93.4% | − 2.0 to − 3.3 kgCO2eq/kg biochar in a decarbonised energy system (includes cradle-to-gate emissions and energy substitution effects); − 4.9 to 0.5 kgCO2eq/kg biochar in a fossil energy system |
| (Ariluoma et al., 2021) | Not Specified. | 85% | − 3.1 kgCO2eq/kg biochar |

A carbon content in biochar of 41% to 93% C corresponds to 1.5 – 3.4 kgCO2/kg biochar.

### i. Urban Soils

The negative emissions potential of biochar applications to urban soils principally depends on three different factors: i. the life-cycle carbon content and ii. the bulk density of the biochar itself, and iii. the optimal quantity of this carbonaceous material that can be applied to a given volume of soil. For instance, carbonised wood pellets have a larger carbon content and density (93% and 500 kg/m3, respectively) and hence large C sink relative to garden waste-derived biochar (70% and 242 kg/m3) (Azzi et al., 2022).

From a life-cycle perspective, the amount of CO2 per kilogram of biochar is dependent on the carbon content of pyrolised biomass feedstock, and the yield and stability of biochar, as well as the energy conversion efficiency of pyrolytic gas (Q. Yang et al., 2021). Additional emissions associated to biochar production include transportation of feedstocks and products, although these could be locally optimised to be kept at a minimum(Maljaee et al., 2021).

The density and carbon stored in biochar are, in turn, dependent on the methods (eg. pyrolisis, gasification) and conditions (eg. temperature, residence time) of production, as well as biomass feedstock used, resulting in a large variety of estimates reported in the literature (Table 2). From these values, a carbon content of 1.5 – 3.4 kgCO2/kg biochar and density of 200 – 500 kg/m3 of biochar have been assumed in this study.

To estimate the potential of storing C in urban soils, we estimate the available global area suitable for biochar amendments. We consider three urban nature-based solutions: urban green spaces, including landscaping soils such as urban forests, parks and residential, as well as street trees and green roofs. Biochar dosages and soil depths for each application have been derived from the literature, summarised in Table 3.

A key study on biochar applications to urban soils quantifies the potential of biochar in a wide range of urban applications in Uppsala, Sweden (Azzi et al., 2022). These include green roofs, urban soils, and biochar as biofilm carrier for water purification at rates of 2.5%, 20% and 100% of biochar per volume, respectively. The resultant average C sink for the construction of Uppsala’s new city-district is estimated at 0.87 to 2.4 MtCO2/year, which in terms of area is equivalent to 19.8 to 53.8 kgCO2/m2·year.

Table 3: Case studies on urban biochar soil applications and key relevant parameters.

| Reference | Scale | Location | Biochar Application | Biochar dosage | C sink contribution from biochar |
| --- | --- | --- | --- | --- | --- |
| (Azzi et al., 2022) | Urban District | Uppsala, Sweden | Green roofs | 2.5% bulk volume parts (bvp) biochar | Reports results for 6 biochar applications and 7 biochar supply chains in 2 energy systems. |
|  |  |  | Structural soil for street trees | 20%bvp biochar-compost (ie. 10% bvp biochar) and 80%bvp macadam 32–64 mm mix in equal parts |  |
|  |  |  | Urban soil (green spaces) | 20% bvp biochar |  |
|  |  |  | Biofilm Carrier | 100% raw biochar |  |
| (Ariluoma et al., 2021) | Urban District | Helsinki, Finland | Urban soil (trees, perennials, shrubs, meadows, lawn) | From 10 to 20% per volume of topsoil with soil depths ranging from 0.2 to 0.8 m. | 25 to 50 kgCO2/m2 |
|  |  |  | Green roofs | From 10 to 20% per volume of soil with depth of 10 cm. | 9.4 to 18.7 kgCO2/m2 |
| (Seguin et al., 2018) | Pot Experiment | Montreal, Canada | Structural soil for street trees | 0 to 7.5% biochar (density 0.30 g cm−3) by dry weight | NA |
| (Yoo et al., 2020) | Pot Experiment | Chungchungnam-do, Korea | Structural soil for street trees | 2.5% by weight of biochar | NA |
| (Scharenbroch et al., 2013) | Pot Experiment | Illinois, US | Structural soil for street trees | 25 Mg biochar/ha·yr = 2.5 kg/m2·yr | 5.22 ± 0.5 kgC/m2 |
| (Ow et al., 2018) | 1559 trees across city | Singapore | Structural soil for street trees | 3:1 soil & biochar addition |  |
| (Y. J. Kim et al., 2021) | Urban District | Suwon, Korea | Structural soil for street trees | 2.5% wt biochar (100 - 250 kg/m3) mixed with surface soil (<10cm depth, ~100 kg/m3) | 44 to 50 kgCO2/m2 |
| (Schaffert & Percival, 2016) | Pot Experiment | Hampshire, UK | Structural soil for street trees | 2.5% biochar per volume |  |
| (Stockholm Stad, 2017) | City | Stockholm, Sweden | Structural soil for street trees | 10% - 25% biochar (250 kg/m3) by volume |  |
| (Lo Piccolo et al., 2022) | Pot Experiment | Pisa, Italy | Structural soil for street trees | 1.5 % (w/w) biochar | 259 gCO2 out of 544 gCO2 per pot |
| (Embren, 2016) | City | Stockholm, Sweden | Urban soil (perennials & bushes in parks) | 25% biochar per soil volume in top 30cm layer |  |
| (Yue et al., 2017) | Pot Experiment | Beijing, China | Urban soil (turfgrass) | 50% biochar per volume of soil | Increase soil organic C by 1.9 |
| (Y. Sun et al., 2019) | Pot Experiment | Nanjing, China | Urban soil (green spaces) | 2% weight to weight (w/w) biochar to soil | Increase in soil C by 5.8 to 9.4 gC/kg soil |
| (B. Yang et al., 2020) | Pot Experiment | Xuchang, China | Urban soil (green spaces) | 0 to 20% biochar by weight (recommend 5%) |  |
| (Sigua et al., 2016) | Pot Experiment | South Carolina, US | Urban soil (green spaces) | 40 Mg biochar/ha |  |
| (Novak et al., 2016) | Pot Experiment | South Carolina, US | Urban soil (green spaces) | 20 g biochar per kg of subsoil |  |
| (Ghosh et al., 2012) | Park | Singapore | Urban soil (with specific focus on turfgrass) | Char briquettes made from sawdust mixed at rates of 25, 50 and 75% by volume with soil |  |
| (Ghosh et al., 2015) | Pot Experiment | Singapore | Urban soil (tree saplings) | 20 – 25% biochar per volume of soil | +32.5 MgC/ha |
| (Di Bonito et al., 2018) | Pot Experiment | Milan, Italy | Green roofs | 5% biochar per volume of soil with 17 cm depth |  |
| (Farrell et al., 2016) | Pot Experiment | Melbourne, Australia | Green roofs | 10, 20, 30 and 40% bvp of biochar to green roof substrate |  |
| (Huang et al., 2020) | Pot Experiment | Nanning City, CIna | Green roofs | 10% of biochar by weight of soil |  |
| (H. Chen et al., 2018) | Roof | Nanjing, China | Green roofs | 10% biochar (970 kg/m3) | 11.99 kgC/m2 |
| (J. Wang et al., 2023) | National Green Infrastructure Facility | Newcastle, UK | Blue-green infrastructure spaces | 4.8 kg biochar/m2 in a 15 cm deep topsoil layer | 30 tC/ha (2.91 ± 2.48 kgC·m−2) |

Based on the soil depths reported in Table 3, the available area for each application has been estimated as follows:

- Urban green spaces: refers to landscaping soils, including residential gardens, urban forests and parks.

  - Global area of urban green spaces was estimated at 210·109 m2 in 2015 (Richards & Belcher, 2020).

  - Assuming 10% of the global urban green spaces can be amended with biochar and a soil depth of 30 cm (based on literature reported values, see Table 3 above) would result in  2.1·1011 m2 · 0.1 · 0.3 = 6.3·109 m3 of soil.

- Street trees:

  - (Boeing, 2022) Analyzes the street networks of 8,914 urban areas in 178 countries, with data acquired and modeled from OpenStreetMap. The study found that the total street length amounts to 5.8·109 m. Assuming that approximately 50% of the total street network are local roads (own assumption) results in 3·109 m of local roads in cities globally.

  - (Smart et al., 2020) study focuses on five capital cities (Ottawa, Stockholm, Buenos Aires, Paris, and Washington, DC) situated in two climate zone. The average of urban street trees across different climates and regions was reported as approximately 5 trees/100m on local roads.

  - By assuming that each tree has a soil volume of 3 to 5 m3  → 3·109 m · 5 trees100 m · 3 - 5 m3tree = 4.5 to 7.5·108 m3 of soil.

- Green roofs:

  - (Z. Sun et al., 2022) Total area of global impervious surfaces ∼ 1.29 million km2 in 2018 (1.29 · 1012 m2)

  - Roofs represent up to 32% of the horizontal surface of built-up areas (Frazer, 2005), this figure is lower for (Jacobson & Hoeve, 2012) and (Akbari et al., 2009) who report 28% and 25%, respectively. Taking a conservative estimate of 25% results in 3.22 · 1011 m2. This number is comparable to the estimate from (Akbari et al., 2009), who estimate a global roof area of 3.725 · 1011 m2.

  - Assuming green roofs can be installed in 10% of global roof area and a soil depth of 15cm would result in 3.22 · 1011 m2 · 0.1 · 0.15 = 4.84 · 109 m3 of soil.

Given that volumetric dosages have been considered, not only a range of biochar carbon contents but also of densities are evaluated to account for the different carbon storage potential per meter cube of biochar. The cumulative carbon storage potentials have been estimated in Table 4, from which annualised global potentials were estimated by considering a 100-year biochar stability of 80% and a 50 years project lifetime, as recommended by the consulted experts.

Table 4: Estimating global potential of biochar amendments in urban green spaces, street trees and green roofs.

|  | Urban Green Spaces | Street Trees | Green Roofs |
| --- | --- | --- | --- |
| i. | 1.5 – 3.4 kgCO2/kg biochar |  |  |
| ii. | 200 – 500 kg/m3 of biochar |  |  |
| Dosage | 20% bvp | 10% bvp | 2-5 – 10% bvp |
| Available soil volume | 6.3·109 m3 of soil | 4.5 – 7.5·108 m3 of soil | 4.8·109 m3 of soil |
| iii. | 1.26·109 m3 of biochar | 4.5 – 7.5·107 m3 of biochar | 1.2 – 4.8·108 m3 of biochar |
| Total | (1.26·109 m3 of biochar) · (200 – 500 kg/m3 of biochar) · (1.5 – 3.4 kgCO2/kg biochar) = 378 to 2142 MtCO2 | (4.5 – 7.5·107 m3 of biochar) · (200 – 500 kg/m3 of biochar) · (1.5 – 3.4 kgCO2/kg biochar) = 14 to 128 MtCO2 | (1.2 – 4.8·108 m3 of biochar) · (200 – 500 kg/m3 of biochar) · (1.5 – 3.4 kgCO2/kg biochar) = 36 to 822 MtCO2 |
|  | = 459 – 3092 MtCO2 = 306 – 910 Mt biochar |  |  |
| With 80% stability & 50 years lifetime* | = 7 – 49 MtCO2·year-1 |  |  |

*Values from expert consultation.

### ii. Additive to Construction Materials

The adsorption mechanisms through which biochar has the ability to capture CO2 from the atmosphere can be divided into physical and chemical adsorption(J. Liu et al., 2022). On the one hand, physical effects generated by intermolecular interactions (eg. van der Waals forces) primarily cause CO2 to adhere to biochar’s highly porous surface(Dissanayake et al., 2020). This latter characteristic of biochar is essential in determining its CO2 adsorption capacity, given that the higher the specific surface area is, the more active sites are available for CO2 adsorption(Creamer & Gao, 2016). On the other hand, this carbonaceous material can naturally have functional groups or be chemically modified through the addition of metal oxides required to attach CO2 molecules to its surface(Shafeeyan et al., 2010). Biochar’s CO2 adsorption capacity is thus determined by its specific surface area, microporosity, aromaticity, hydrophobicity and the presence of basic functional groups suitable for retaining CO2 (Dissanayake et al., 2020).

Due to its CO2 surface adhesion mechanism, the direct carbon footprint reduction through biochar construction applications has been extensively quantified in the literature in units of mmol CO2 adsorbed per gram of biochar. For instance, a review study (J. Liu et al., 2022) reports estimates with an average of 119 mg CO2 adsorbed per gram of biochar at 25°C and 1 bar, ranging from 20.25 (Promraksa & Rakmak, 2020) to 250.4 (Ben Salem et al., 2021). Another review (Abuelnoor et al., 2021) reports higher values2 averaging at 163 mgCO2/g of biochar and ranging from 22.9 (Ding & Liu, 2020) to 475.3 (Ogungbenro et al., 2018) mmol CO2/g.

Table 5: Application rates of biochar into cementitious composites.

| Reference | Dosage of % biochar by weight of cement | Notes & Key Findings |
| --- | --- | --- |
| (Gupta & Kashani, 2021) | 3% | Net avoidance of 0.72 kgCO2/kg biochar, ie. addition of biochar can reduce the equivalent carbon associated with cementitious building materials by 2–6%. |
| (Gupta et al., 2021) | 1% and 3% | 56% of C in biochar. Biochar-cement composite has higher (+5 to 7%) carbonate mineralization potential compared to control with some improvements in mechanical and durability properties. |
| (Praneeth et al., 2021) | 10% – 40% | Aim of biochar addition is to improve toughness, lower thermal conductivity and density of the cement composites. |
| (Praneeth et al., 2020) | 2%, 4%, 6% and 8% | 4 - 6% is recommended for optimal compressive strength. |
| (X. Yang & Wang, 2021) | 2% and 5% | Confirms feasibility of C sequestration using accelerated carbonation curing (ACC) of biochar-blended mortar. |
| (Qin et al., 2021) | 0.65%, 3.2%, 6.5%, 9.5%, and 13.5% | 6.5% recommended to avoid compromising on compressive strength and splitting tensile strength. |
| (Suarez-Riera et al., 2020) | 2% | Sufficient to increase the strength and toughness of the cement and mortar composites/maintain the mechanical properties. |
| (Gupta, Kua, & Koh, 2018) | 1%, 2% and 5% | 2% recommended for improved compressive strength. |
| (Gupta, Kua, & Low, 2018) | 2% | Fresh biochar offers significantly higher mechanical strength and improved permeability compared to biochar saturated with carbon dioxide. |
| (Asadi Zeidabadi et al., 2018) | 5% and 10% | Best results in terms of compressive and tensile strength wth 5% treated bagasse biochar. |
| (Restuccia & Ferro, 2016) | 0.5%, 0.8%, and 1% | 0.8% recommended for improved mechanical properties. |
| (Restuccia et al., 2020) | 0.8%, 1%, 1.5%, 2% and 2.5% | Biochar nano- and micro-particles can increase the strength and toughness of the composites at a recommended application of 2%. |
| (Sirico et al., 2020) | 0 - 2.5% | Compressive and flexural strengths comparable to those of control specimens; slight improvement of fracture energy by using 1 wt% addition of biochar. |
| (Belletti et al., 2019) | 1% | Comparable compressive strength, flexural strength, toughness and ductility with respect to traditional cement mortars |
| (Singhal, 2023) – Review | 1% – 3% | Reported value for “optimum concentration for enhancing performance in building constituents” from review of relevant literature. |
| (Maljaee et al., 2021) – Review | 2% | Optimum biochar content contributing to the improvement of mechanical properties. |
| (K. Tan et al., 2022) | 1–3% | Higher compressive/flexural strength of modified pervious concrete relative to biochar-free, but strength will be compromised if the content exceeds these values. |
| (Azzi et al., 2022) | 15 – 100% | “Charcrete”: biochar to cement mass ratio within the range 0.15–1. |
| (Akhtar & Sarmah, 2018a) | 0.1 – 0.75% | Biochar at 0.1% produced optimum results with 20% increment on flexural strength. |
| (Ofori-Boadu et al., 2017) | 0 – 10% | Investigated effect of biochar on water adsorption of cement |
| (Dixit et al., 2019) | 2%, 5% and 8% | Biochar application forUltra High Performance Cement. |
| (Cuthbertson et al., 2019) | 1 to 15% | Addition of biochar considerably increased sound absorption coefficient of concrete, and reduced its thermal conductivity and cp,[ressove strength. |
| (L. Wang et al., 2019) | 1% and 2% | Biochar incorporation slightly enhanced the cement hydration reaction, while it also induced microcracks and weakened the strength of sediment products. |
| (H. Lee et al., 2019) | 1%, 2%, 5% and 10% | Biochar was shown to reduce thermal conductivity and water vapor permeability |
| Alternatives to Cement |  |  |
| (Das et al., 2015) | 6%, 12%, 18%, 24%, and 30% by total weight of composite | Not cementitious materials – Wood polypropylene composite added with biochar enhances flexural strength and thermal stability. |
| (L. Chen et al., 2022) | 30% by weight of composite mortar | The most promising mixture (mechanical performance, resource availability, negative CO2 emissions, and economic profits) was 30% wt biochar & 9% wt metakaolin as a binder. |
| (Jeon et al., 2021) | 2%, 4%, 6%, 8%, 10% by total weight of composite | Biochar proved to improve thermal insulation performance and water stability |
| (S. Zhao et al., 2014) | 5 and 10% by weight of asphalt | Biochar proved to be an effective modifier in reducing the temperature susceptibility of the binder. |
| (Kumar et al., 2018) | 5%, 10%, 15% and 20% by weight of binder | Biochar as asphalt modifier resulted in improved binder performance against the effects of aging and deformation. |
| (Akhtar & Sarmah, 2018b) | 0.1 – 1% by volume of concrete | Conclude that waste-derived char can potentially enhance the properties of recycled aggregate concrete. |

The International Energy Agency estimates that global cement production is set to grow by 12 – 23% by 2050 from 2018 levels (IEA, 2018), estimated to be 4282 Mt worldwide (IEA, 2022a) and thus leading to 4796 – 5267 Mt of cement by 2050. Taking the lower bound of this estimate, the process for estimating the global potential of biochar as an additive to cementitious materials is described in Fig. 1 below.

Fig. 1: Estimation of the global mitigation potential of biochar as a construction additive.

Alternatively, we also considered the projected demand for cement under large timber in construction deployments. In their study of buildings as carbon sinks, Churkina et al. (2020a) estimate the emissions from new constructions of residential and commercial buildings by 2050. Four different scenarios are considered: a business-as-usual scenario in which the majority of new buildings (99.5%) are constructed with steel and concrete and 0.5% with timber; and three scenarios where 10%, 50% and 90% of new urban dwellers will be constructed with timber. The uptake of timber as a construction material would reduce the projected demand for concrete and cement, which might in turn reduce the potential of storing C in cementitious materials through biochar additions. To compute future concrete demand, Churkina et al. (2020a) assumed an urban population increase of 2.3 billion people from 2020 to 2050, per capita floor areas of 9.2, 30 and 79.1 m2 with associated demand for concrete of 2567.2, 10587.9 and 27948.3 kg of concrete per capita, respectively. We have assumed a cement ratio of 10% of concrete by mass to compute estimates of annual cement demand by 2050. Based on the lower and upper bound estimates of cement demand, this would correspond to 0.1 to 31 MtCO2 per year, several fewer orders of magnitude relative to estimates from other sources. Additionally, a key takeaway from this alternative computation of cement future demand is that with the widespread of timber in construction, the potential for storing C in cementitious materials through biochar additions might be reduced by one order of magnitude relative to a business-as-usual scenario.

This large discrepancy might be explained due to IEA estimates considering worldwide cement consumption, whereas Churkina et al.’s study focuses exclusively on urban dwellings.

Table 6: Estimated future concrete and cement demand by 2050, based on scenarios and parameters described in (Churkina et al., 2020a).

| Scenario | Floor area per capita (m2/cap) | Concrete demand from Primary Structure from 2020 to 2050 (Gt concrete) | Annual Concrete demand (Gt concrete·year-1) | Annual Cement demand (Gt cement·year-1) | Annual CO2 stored in cement materials through biochar additions (MtCO2·year-1) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min | Max |
| Business- as-usual | 9.2 | 8.90 | 0.20 | 0.02 | 0.7 | 2.8 |
|  | 30 | 24.35 | 0.81 | 0.08 | 2.7 | 11.6 |
|  | 79.1 | 64.28 | 2.14 | 0.21 | 7.1 | 30.5 |
| 10% to timber | 9.2 | 5.31 | 0.18 | 0.02 | 0.6 | 2.5 |
|  | 30 | 21.92 | 0.73 | 0.07 | 2.4 | 10.4 |
|  | 79.1 | 57.85 | 1.93 | 0.19 | 6.4 | 27.5 |
| 50% to timber | 9.2 | 2.95 | 0.10 | 0.01 | 0.3 | 1.4 |
|  | 30 | 12.18 | 0.41 | 0.04 | 1.3 | 5.8 |
|  | 79.1 | 32.14 | 1.07 | 0.11 | 3.6 | 15.3 |
| 90% to timber | 9.2 | 0.59 | 0.02 | 0.002 | 0.1 | 0.3 |
|  | 30 | 2.44 | 0.08 | 0.01 | 0.3 | 1.2 |
|  | 79.1 | 6.43 | 0.21 | 0.02 | 0.7 | 3.1 |

## Wood in Construction

Table 7: Case studies quantifying the climate change benefits of the use of wood in construction at a continental or global scale.

| Reference | Scale | Metric | Reported Value | Timeframe | Notes |
| --- | --- | --- | --- | --- | --- |
| (Mishra et al., 2022) | Global | Annual Emission Savings + C Storage + Land-use emissions | 0.18 - 1.32 GtCO2/year | 2020 - 2100 | Savings = (Long-term carbon storage in harvested wood products) + (Emission from land and forest management) + (Emissions from manufacturing and processing of raw materials for construction of new urban buildings) |
|  |  |  | 0.08 - 0.33 GtCO2/year | 2020 - 2050 | Same as above but values for 2020 - 2050. |
| (Himes & Busby, 2020) | Global | Annual Emission Savings | 0.28 GtCO2/year | By 2030 | Combines population growth, 30 m2 per capita and 216 kgCO2e/m2 estimates to reach global emission reduction potential. |
| (Smith et al., 2023) | Global | Annual NE | 0.2 - 1.3 GtCO2/year | By 2050 | Estimates from (Himes & Busby, 2020; Mishra et al., 2022) - “durable harvested wood products” |
| (Churkina et al., 2020b) | Global | Annual C Storage + C Storage | 0.01–0.68 GtC/year (0.037 - 2.49 GtCO2/year) | 2020 - 2050 | Yearly transfer of organic carbon in primary structure and enclosure of buildings for 9.2 – 79.1 m2/cap for 10 – 90% of new constructions from wood. |
| (Churkina, 2016) | Global | Cumulative Storage | 6.7 GtC (24.57 GtCO2) (range 1.3 – 12.1 GtC) | To 2015 | Calculates the carbon density of buildings in cities globally. |
| (Heräjärvi, 2019) | Global | Cumulative Storage | 450 million m3 of wood products (=85% of global production) | / | Computes the required volume of wood for the biogenic carbon stored in wooden building structures every year to be equivalent to 1% of annual anthropogenic emissions. |
| (McLaren, 2012) | Global | Annual NE | 0.5 - 1.4 GtCO2/year | 2030 - 2050 | Technical, rather than commercial, estimates. |
| (D’Amico et al., 2021) | Global | Cumulative Emission Savings | 20 - 80 MtCO2eq | By 2050 | Avoided emissions by replacing steel-concrete floor slabs in steel structures with CLT slabs. Does not account for carbon sequestration potential in timber. |
| (Hildebrandt et al., 2017) | Europe | Annual (net) carbon storage | 46 MtCO2eq/year | By 2030 | Reported results for “Policy Intervention” scenario calculated using material intensities of wood products. |
| (Pittau et al., 2019) | Europe | Cumulative Storage | 76.6 MtC | 2018 - 2100 | Despite large amount of biogenic carbon stored in timber products, not all emissions can be compensated by 2050 and the result is also a net positive climate change impact. |
| (Pittau et al., 2019) | Europe | Cumulative Storage | Straw I-frame: 281 MtCO2 Hemprecete frame: 54 MtCO2 Hempcrete blocks: 84 MtCO2 | 2018 - 2100 | “Contrarily to timber, the carbon removals from straw and hempcrete-based materials outweigh the emissions before the end of the century, resulting in net negative emissions”. |
| (Johnston & Radeloff, 2019) | Global | Annual NE | 317 – 441 MtCO2/year | By 2030 | Net annual removals by pool of global HWP based on future socioeconomic conditions. If assume 76% of HWPs are durable, the reported estimate comes down to 241 – 335 MtCO2/year (Arehart et al., 2021; Zeitz et al., 2019) |

## Decentralised Direct Air Capture

Table 8: Case studies on decentralised DAC applications.

| Reference | Scale | Focus of study | Reported NE Potential | Methodology |
| --- | --- | --- | --- | --- |
| (Fujikawa & Selyanchyn, 2022) | Building (Roppongi Hills Mori Tower Building, Tokyo) | Review of DAC by membrane separation. | 4200 tCO2/year | Assumptions: 179,013 m2 and 3m high ceilings, 600 ppm, air exchange rate of 5 h-1, 25% of CO2 recovered. |
| (Baus & Nehr, 2022) | Building (Germany) | HVAC/DAC-coupling for CO2-depleted air recirculation. | 20.59 - 35.07 tCO2/year | Reports CO2 separation potential for a 1000 m2 & 100 occupants building operational 250 days a year for average ventilation of 12 h per day. Assumptions: 32.4 gCO2·person-1·h-1, air exchange rate of 2.7 - 5.4 h-1, 60% CO2 separation rate, 80% recirculation rate. |
| (Dittmeyer et al., 2019) | Office & residential buildings, Grocery stores, (Germany) | Retrofitting air conditioning units for decentralized CO2 conversion. | NA (reports production rate of liquid hydrocarbon fuels). | Estimates the volume of air in circulation, and assumes air exchange rate and indoor CO2 concentration of 400 ppm. |
| (Kuittinen et al., 2021) interpreting results from (Dittmeyer et al., 2019) | Building (Germany) | Review of the potential of C storage in the built environment. | Office Building: 104 - 209 kgCO2/m2·year Grocery Store: 210 - 319 kgCO2/m2·year Residential: 10 - 20 kgCO2/m2·year | Values computed for a yearly operation of 8760 hours (full time). Areas of 63000, 1200 and 42 m2 for the office, grocery store and residential buildings, respectively. |
| (Borchers et al., 2022) | National (Germany) | Decentralised DAC as part of a portfolio of CDR options in Germany. | 15 – 100 MtCO2/year | Lower-bound estimate based on equipping only large buildings (>2,500 m2 of floor space) with DAC, higher-bound on per capita estimates. |
| (Bryan & Salamah, 2019) | National (US) | Building-integrated carbon capture with Moisture Swing Air Capture Technology in buildings' mechanical systems. | 5 MtCO2/year | Assumptions: total commercial floor space in the U.S. is 10 billion m2, of which 18% is commercial office buildings. Implement in 50% of office buildings. Operation is 10 hours a day for 250 days a year. 100% CO2 captured. |
| (Sinha et al., 2022) | Building (New York City, US) | DAC (adsorption beds) optimised for air quality control and energy savings. | NA (does not quantify potentials) | Assumptions: 5-floor building with an average space allocation of 5 m2 per person and 14h of human occupancy. |
| (Sinha et al., 2018) | Micro-scale study | Establishes DAC as feasible strategy for reduction of energy requirements from enclosed environments | NA (does not quantify potentials) |  |
| (M. K. Kim et al., 2015) & (M. K. Kim & Leibundgut, 2015) | Micro-scale studies | HVAC/DAC-coupling for energy savings. | NA (does not quantify potentials) |  |
| (Ji et al., 2022) | Micro-scale study | HVAC/DAC-coupling adopted in large commercial buildings | NA (does not quantify potentials) |  |
| (R. Zhao et al., 2019) | Micro-scale study | Thermodynamic study of HVAC/DAC-coupling for energy savings. | NA (does not quantify potentials) |  |
| (Chu et al., 2021) | Micro-scale study | HVAC/DAC-coupling for CO2-depleted air recirculation. | NA (does not quantify potentials) |  |
| (Gall et al., 2016) | Micro-scale study | Microstructural analyses | NA (does not quantify potentials) |  |
| (S. C. Lee et al., 2013) | Micro-scale study | Investigated use of Y-type zeolites for DAC in enclosed environments. | NA (does not quantify potentials) |  |

From the reviewed literature, Table 9 includes the parameters and variables used in the estimation of the global deployment of DAC decentralised in office buildings following eq. 1.

(m2∙ m ∙ %DAC) ∙ 1h  ∙ m3CO2m3air ∙ 106 ∙ kgCO2m3CO2 ∙ MtCO2109  kgCO2 ∙ hyear ∙ η=MtCO2year										(1)

Table 9: Parameters used in decentralised DAC global potential

| Parameter | Value | Source | Notes |
| --- | --- | --- | --- |
| Global office floor area projection for 2050 | 1.46 · 1010 m2 | (Deetman et al., 2020) | Projections based on estimated annual demand for construction materials worldwide combining both regresion and dynamic stock models. |
| Ceiling Height | 3 m | (Dittmeyer et al., 2019; Fujikawa & Selyanchyn, 2022) | Taken as average office ceiling height. |
| % DAC retrofit | 10 – 50% | Own assumption | Percentage of office buildings with DAC/HVAC retrofit from global 2050 office floor area projections. |
| Operational ventilation parameters | 3600 h/year | Own assumption | Corresponds to 12h/day for 300 days/year. |
| Air recirculation rate | 80% | (Baus & Nehr, 2022) | Ratio of air re-entering the building to air passing through the DAC module. Value not used in calculation above, but implicit assumption of the authors in the computation of steady-state CO2 indoor concentrations. |
| CO2 separation rate (η) | 60% | (Baus & Nehr, 2022) | Ratio of captured to available CO2. |
| Air Density | 1.815 kg/m3 | (Baus & Nehr, 2022) |  |
| Air exchange rate (h-1) | 2.7 – 5.4 (h-1) | (Baus & Nehr, 2022) | Varies for different occupancy patterns and assumptions about the age of the building (eg. modern buildings would tend towards higher ventilation air exchange rates). |
| Steady-state indoor CO2 concentrations* | 258 – 678 ppvm | (Baus & Nehr, 2022) | Stedy-state mass of CO2 indoors computed for different scenarios with varying occupancy (100 to 200 people in 3500 m3 building, 32.4 gCO2·person·h-1) and ventilation rates (9450 – 18900 m3·h-1). |

*Baus & Nehr (2022) estimate steady-state indoor CO2 concentrations for multiple 3500 m3 buildings, represented as different scenarios. Relevant to the assessment of CO2 loads, these scenarios vary in the number of daily average occupants (100 – 200) and air exchange rates (2.7 – 5.4 h-1). Steady-state indoor CO2 concentration is reached after the CO2 sources (ambient air, occupancy-related emissions and recirculated air after DAC system) and sinks (DAC system and exhaust air) stabilise, with the system resetting every day. We assume the computed indoor CO2 concentrations (258 – 678 ppvm) refer to daily averages, meaning that the transition period to steady-state is considered negligible and that the number of occupants used in the calculations refers to the daily average occupancy during operation hours (12 h·day-1).

## Surface Albedo Management

Table 10: Cool Surface Strategies identified from the reviewed literature

|  | Pigments | Phase change material | Retro-reflective materials | Others |
| --- | --- | --- | --- | --- |
| Description | High solar reflectance potential incorporated into glazes, fillers, and engobes | Change their physical characteristics as a consequence of heat release or absorption | Reflect an incoming light beam to a surface back towards its source | Complementary ceramic and glass materials as coatings |
| Different types | Cool (low absorptance) & high near-infrared transmittance pigments | Bricks & wall-integrated materials | Surface coating and tiles | Tiles, glazes & bricks |
| Suitable Application | Roofs, building envelopes & pavements | Roofs, building envelopes & pavements | Building envelopes, walls & pavements | Tiles in roofs (durable), glazes (roofs & pavements) |
| Literature Examples | (Gali et al., 2017; Oleson et al., 2010) | (Gao & Meng, 2023; Yoon et al., 2018) | (C. Wang et al., 2021) | (Enríquez et al., 2017; Pisello et al., 2014) |

Table 11: Studies quantifying the temperature impacts of surface albedo modifications albedo in global urban areas.

| Reference | System | Albedo change | Metric Used | ΔT (˚C) | Timeframe / Period |
| --- | --- | --- | --- | --- | --- |
| (Menon et al., 2010) | Increasing albedo in all roofs globally | 0.1 | Global Mean Temperature | –0.008 | Boreal Summer |
| (Zhang et al., 2016) | Increasing albedo in all roofs globally | by 0.75, from 0.15 to 0.9 | Heat Island Intensitya | –1.6 to –1.2 | Yearly average |
| (Zhang et al., 2016) | Increasing albedo in all roofs globally | by 0.75, from 0.15 to 0.9 | Global Mean Temperature | −0.0021 ±0.026 | Yearly average |
| (Jacobson & Hoeve, 2012) | Increasing albedo in all roofs globally | from 0.12 to 0.65 | Global Urban Mean Temperature | –0.02 | 20-year simulation period |
| (Jacobson & Hoeve, 2012) | Increasing albedo in all roofs globally | from 0.12 to 0.65 | Global Mean Temperature | +0.07 | 20-year simulation period |
| (Oleson et al., 2010) | Increasing albedo in all roofs globally | from 0.32 to 0.9 | Maximum Daily Temperature | –0.56 | Yearly average |
| (Oleson et al., 2010) | Increasing albedo in all roofs globally | from 0.32 to 0.9 | Heat Island Intensitya | –0.4 | Yearly average |
| (Rawat & Singh, 2022) | Review on cool roof technologies studies worldwide | – | Roof Surface Temperature | –1.4 to –4.7b | Yearly average |
| (Ouyang et al., 2022) | Impact of global albedo change due to projected urban expansion | −0.001 to +0.001 | Global Mean Temperature | +0.00104 to +0.00128 | By 2050 relative to 2018 |

a Defined as the urban canopy temperature (i.e. air temperature at 2 m) difference between urban and rural areas: Turban-Trural

b In different climatic zones

In efforts to account for the effects of surface albedo alterations accompanying pertubations in the Earth’s carbon cycle, a variety of methodologies have been suggested to express surface albedo changes (Δα) into CO2 equivalence (Susca, 2018). Relevant parameters with significant implications for the different accounting mechanisms include whether the Δα scenario has a time-dependency, is composed of single or time-series of CO2-equivalent pulses or is compatible with IPCC reporting approaches (Bright & Lund, 2021). For instance, Akbari et al. report that increasing the albedo of roofs and pavements in global urban areas by 0.25 and 0.15, respectively, could be compared to offsetting 44 GtCO2 of emissions (Akbari et al., 2009). This is analogous to a single-pulse time-independent scenario, which cannot be compared to an emission scenario (Bright & Lund, 2021). This metric has been defined as “emissions equivalent of shortwave forcing” (EESF) by (Betts, 2000). Zaragoza & Bartolom similarly note that 7.6 GtCO2 of equivalent emissions reduction could be achieved through an increase in 0.15 surface albedo of global paved roads (Zaragoza & Bartolom, 2012).

Alternatively, the “time-dependent emissions equivalence” (TDEE) column vector of CO2-equivalent emission pulses is insensitive to the time horizon chosen, thereby being compatible with cumulative emissions policy targets (Bright & Lund, 2021). More recently, the literature has favoured the use of IPCC-aligned metrics such as Global Warming Potential (GWP), which calculates location-specific RF impacts due to changes in surface albedo as a function of the intensity of incoming radiation, atmospheric transmittance, and the change in albedo (AzariJafari et al., 2019, 2021; Guest et al., 2020; Loijos et al., 2013; Ouyang et al., 2022; Xu et al., 2020). The change in RF is then normalised to the temporally accumulated radiative forcing following a unit pulse CO2 emission over a certain Time Horizon (TH) of choice. However, no study has reported the GWP resultant from increasing the albedo of impervious surfaces at a global scale.

Fig. 2: Overview of distinguishing attributes, methodological differences, drawbacks, and merits of the six Δα-to-CO2-equivalence metrics applied in the scientific literature included in the review conducted by (Bright & Lund, 2021).

Based on the Δα-to-CO2-equivalence metrics described in Fig. 2, we have classified relevant reviewed studies quantifying the effect of modifying urban surface albedo for different systems and scales in Table 12.

Table 12: Summary of relevant literature quantifying the CO2-equivalent effect of modifying urban surface albedo for different systems and scales (in chronological order). AF = Airborne fraction; RF = Radiative forcing; TH = Time Horizon.

| Reference | System | Scale | Metric | Reported Value | Notes |
| --- | --- | --- | --- | --- | --- |
| (Akbari et al., 2009) | Roofs & Pavements | Global | EESF | 44 GtCO2 (– 2.55 kgCO2/m2) | AF = 0.55; RF = – 1.27 W/m2. Increase in urban roof and pavement albedos by 0.25 and 0.15, respectively. |
| (Santero & Horvath, 2009) | Pavements | Scalable | EESF | 220 MgCO2e/lane-km (– 2.55 kgCO2/m2) | AF = 0.55; RF = – 1.27 W/m2; increasing urban pavement albedo by 0.35. |
| (Menon et al., 2010) | Roofs & Pavements | Global | EESF | 57 GtCO2 (during boreal summer) (– 3.26 kgCO2/m2) | AF = 0.55; RF = – 1.63 W/m2; increasing urban roof and pavement albedos by 0.25 and 0.15, respectively. |
| (Millstein & Menon, 2011) | Roofs & Pavements | National (USA) | EESF | 3.3 ± 0.5 Gt CO2 (-101 - 208 kgCO2/m2) | AF = 0.55; RF = -1.8 W/m2 (averaged over full annual cycle). Increase in urban roof and pavement albedos by about 0.25 and 0.15, respectively. |
| (Susca et al., 2011) | Roofs | Scalable | EESF | 140 kgCO2e/m2 for a 50-year time horizon (– 2.55 kgCO2/m2) | Increase in albedo to 0.6 relative to an average of 0.05. |
| (Zaragoza & Bartolom, 2012) | Pavements | Global | EESF | 7.6 GtCO2 (– 2.55 kgCO2/m2) | Total equivalent reduction by asphalt concrete substitution (increase in 0.15 surface albedo) of global paved roads Total global paved surface was taken as 203,104 km2 in 2006. |
| (Akbari & Matthews, 2012) | Roofs & Pavements | Global | EESF/TH | 80 GtCO2e for 50 years, and 160 GtCO2e in year 2200 (6.5 - 7.5 kgCO2e/m2·0.01 albedo change) | AF = 0.55; RF = – 1.27 W/m2, increasing urban roof and pavement albedos by 0.25 and 0.15, respectively. Reports not only cumulative effect but also one-time offset: 25 –150 Gt CO2e emissions. |
| (Susca, 2012a) | Roofs | Scalable | TDEE | 110 - 184 kgCO2e/m2 in 50 years and 131 - 217 kgCO2e/m2 in 100 years | RF = – 1.27 W/m2; increase in albedo to 0.6 and 0.8 relative to an average of 0.05 and 0.15. |
| (Susca, 2012b) | Roofs | Scalable | TDEE | 142 - 167 kgCO2e/m2 for 50 and 100 years, respectively | RF = – 1.27 W/m2; increase in albedo to 0.9 relative to an average of 0.32. |
| (Yu & Lu, 2014) | Pavements | Regional (Michigan USA) | TDEE | 3137 tCO2e; or 12.3 kgCO2e/m2 of pavement for a 40 years lifespan | RF = – 1.54 W/m2; increase in albedo to 0.3 relative to an average of 0.25 after reconstruction at the 20th year until the 40th year. |
| (AzariJafari et al., 2019) | Pavements | City (Montreal, Canada) | GWP | 0.34 kgCO2e/m2 of pavement per 0.2 albedo increase | TH = 50 years; Transmittance = 0.854 |
| (Xu et al., 2020) | Pavements | National (USA) | GWP | 17 MtCO2/year | AP=50 years; TH = 100 years; increasing pavement albedo by 0.2 for all urban and rural asphalt roads across the U.S. |
| (AzariJafari et al., 2021) | Pavements | Urban (Phoenix & Boston, USA) | GWP | 0.4 to 25.9 ktCO2e emission savings, depending on location within city (= 0.6 to 2.1 kgCO2e/0.01 albedo change·m2 pavement) | Location-specific RF impacts due to changes in pavement albedo calculated as a function of the intensity of incoming radiation, atmospheric transmittance, and the change in albedo. |

## Co-Benefits

Table 13:  Identified co-benefits of the five carbon storage and adaptation options for cities. References indicate example case studies identifying the attributed co-benefit.

| Urban Option | Human health and well-being | Biophysical & Environmental | Economic opportunities |
| --- | --- | --- | --- |
| Storing C in Vegetation | - Physical and psychological health benefits at individual level (Suppakittpaisarn et al., 2017); Recreation opportunities (Keeler et al., 2019); Positively influence social activity and interaction (Sullivan et al., 2004) - Thermal human comfort (Ottelé et al., 2011) (Sultana et al., 2021) - Help raise awareness around climate change by routinely intertwining social and ecological functions (Hsu et al., 2022) - Help combat social exclusion and isolation (Green Infrastructure (GI) — Enhancing Europe’s Natural Capital, 2013) (Frantzeskaki, 2019) - Relieve anxiety about rapid urbanization (Y. Wang et al., 2021) - Aesthetic appreciation (O’Sullivan et al., 2017) | - Vegetative cooling: land and ambient urban temperature stabilisation, hence UHI mitigation (Macauley, 2020), (Cheela et al., 2021) - Improved air quality (McDonald et al., 2016) - Support biodiversity conservation (Goddard et al., 2010), (Sagar et al., 2022) - Coastal risk reduction (Richards et al., 2022) - Moderation of extreme events (eg. flooding, peak flows) through storm water management (Cortinovis et al., 2022) - Improved water quality (Davis et al., 2009) - Maintenance of soil fertility (Furey & Tilman, 2021) | - Building energy savings from vegetative cooling (Milnar & Ramaswami, 2020), (Shafique et al., 2020), especially for green roofs (Dong et al., 2020) - Green jobs (Raymond et al., 2017) - Source of natural products and raw materials (European Commission, 2015) (O’Sullivan et al., 2017) - Bottom-up scalability: local cost-effective solution (Cousins, 2021) (Tahvonen & Airaksinen, 2018) |
| Storing C in Soils | - Indirect benefits and ecosystem services from increase vegetation productivity (Fransson et al., 2020) | - Increase in soil nutrient concentrations (Ghosh et al., 2015), soil fertility and crop yields (Lu et al., 2020) - Soil remediation (Singh Yadav et al., 2023) and removal of pollutants from aqueous solutions (X. Tan et al., 2015) - Increase water holding capacity (H. Chen et al., 2018) - Reduction in need for fertiliser and irrigation (Renforth et al., 2011) | - Foster circular bioeconomy and waste reduction (Lo Piccolo et al., 2022) eg. biochar from urban wastewater (J. Liu et al., 2023) - Delivery of heat services from biochar production in cities (Bydén & Fridlund, 2020) - Cost savings from reduced irrigation needs (Bordoloi et al., 2019) - Availability of abundant feedstock materials (Legan et al., 2022) |
| Storing C in the built environment | - Construction reduces noise pollution and the production of dust and waste (Švajlenka et al., 2017) - Aesthetic and well-being benefits of wood (Lähtinen et al., 2019) (eg. naturalness, cosiness and relaxing features of wood (Häyrinen et al., 2020)) | - Substitution of carbon-intensive conventional construction materials (D’Amico et al., 2021) - Mitigate variations of relative humidity inside the building for both wood (Rahim et al., 2023) and biochar (Gupta & Kua, 2017) construction applications - Earthquake resistance (Vieux-Champagne et al., 2014) - Waste management benefits from application of waste-derived biochar to construction (Akinyemi & Adesina, 2020) | - Lower construction times (Kremer & Ritchie, 2018; Talvitie et al., 2021) - Potential to reuse wood products at end-of-life (Woodard & Milner, 2016) - Economic benefit from reduced waste through pysolysing biomass for biochar production (Cazemier, 2017) |
| Storing C below-ground | - Enhanced indoor air quality (López et al., 2023) - Lack of aesthetic conflict | - Production of water in solid sorbent DAC designs (Smith et al., 2023), and consequent potential synergy with green roofs - Potential reduction of energy use due to air recirculation (Sinha & Realff, 2019) | - Retrofit of existing infrastructure (Dittmeyer et al., 2019) - Thermodynamic advantage relative due to higher CO2 concentrations (Fujikawa & Selyanchyn, 2022) - Facilitates access to unutilized CO2-resources (Baus & Nehr, 2022) |
| Surface Albedo Management | - Reduction of heat related mortality (Santamouris & Fiorito, 2021) - Lack of aesthetic conflict (Akbari & Matthews, 2012) | - UHI Reduction (C. Wang et al., 2021) - Rapid temperature reduction effect after deployment (Royal Society, 2009) - Localised and non-uniform effect, thus unlikely to modify weather patterns despite large-scale deployment (Taha, 2015) - Reduced photochemical reaction rates (Taha, 2008) | - Reduced building energy demand (Xu et al., 2020) (especially in tropical climates, eg. (Cubi et al., 2016)) leading to economic savings (Sproul et al., 2014) - Easily applied on existing roofs by conventional low cost methods (Rawat & Singh, 2022) - Cool paintings are a known technology, no research and development required (Royal Society, 2009) |

## Barriers to Implementation

Table 14: Identified considerations relevant to the implementation of urban carbon removal and surface albedo management options. References indicate example case studies identifying the attributed barrier to implementation

| Urban Option | Uncertainty | Adverse Effects | Economic & Cost Barriers | Governance issue |
| --- | --- | --- | --- | --- |
| Storing C in Vegetation | - Biome and vegetation type: different C sequestration (Churkina, 2016) - Impact of climate change on long-term storage potential (Esperon-Rodriguez et al., 2022; Sander, 2022) - Long-term effect of CO2 fertilization on sequestration potential (Huntingford & Oliver, 2021) - Challenging to measure actual (Kabisch et al., 2022) and estimated CO2 sequestration due to: urban scene complexity (Zhuang et al., 2022) and insufficient measurement accuracy (Fan & Wei, 2022) | - Maintenance and irrigation emissions (lifecycle emissions) (Milnar & Ramaswami, 2020) - Water usage (Pataki et al., 2021) - Non-CO2 GHGs emissions (Pataki et al., 2021) - High allergenic potential (Kasprzyk et al., 2019) - Reduced air mixing due to large trees leading to increase in pollution (Vos et al., 2013) - Reduction in surface albedo (Rose & Levinson, 2013) | - Large variability of costs depending on vegetation strategy (McDonald et al., 2016) - Dense cities with restricted available land might not prioritise the development of green spaces based on cost-efficient allocation (Grace et al., 2021) | - Design multi-purpose urban green spaces (Kabisch et al., 2016) - Competing land usage and green gentrification (Cousins, 2021) - Limited potential in pre-existing high-density neighbourhoods, which are also most vulnerable to climate change (Grace et al., 2021) - Stakeholder involvement in the design of urban green spaces (Raymond et al., 2017) - GHG accounting protocol for urban vegetation currently unavailable (Kovac et al., 2020) - Path dependencies and institutional barriers of existing urban systems (C. Davies & Lafortezza, 2019) - Disconnect between short-term actions and long-term goals (Kabisch et al., 2016) |
| Storing C in Soils | - Large variability of C content in biochar depending on feedstock (Uday et al., 2022) - Stability of biochar in diverse urban soils (eg. structural soils, green roofs (Fransson et al., 2020)) - Variability of soil improvement potential depending on feedstock and biochar form (eg. pellet vs dust) (Mohammadi, 2021) - Long-term effect of biochar on soils under real conditions (Kuppusamy et al., 2016) - Lack of studies on the effects of biochar on soil microbial function and enzyme activity (Abhishek et al., 2022) | - Potential ground- and surface water contamination (EBC, 2022) - Direct and indirect LUC from biochar production (Minx et al., 2018) - Lifecycle emissions from deployment and maintenance (Azzi et al., 2022) - Increase in albedo RF from urban biochar use (Meyer et al., 2012) - Potential increase plant vulnerability against insects and pathogens (Minx et al., 2018) | - Range of costs depending on feedstock and transportation requirements (Winters et al., 2022) - Need for a stable biochar price (Salo, 2018) - Current lack of incentives: need to link economic benefits with social and environmental issues (Basak et al., 2022) | - Conflict with alternative land use (Shackley et al., 2011) - Lack of widespread knowledge of benefits and applications of biochar (Krull, 2018) - Knowledge and effort in installing and maintaining biochar soils (Fransson et al., 2020) - Need for environmental risk assessment (Abhishek et al., 2022) |
| Storing C in the built environment | - Methodological discrepancies: large mitigation variation depending on temporal and material boundaries of studies (Moncaster et al., 2018) - Lifecycle emissions cascading from forest growth to wood disposal (Mehr et al., 2018) - Impact of future biogeochemical changes on CO2 sequestration potential (Mishra et al., 2022) - Climate impact of albedo changes (Schwaiger & Bird, 2010) - Future estimations of construction of new dwellings, urban population growth, and area per capita (Churkina et al., 2020b) - Lack of long-term and large-scale testing of biochar adsorption properties in construction materials (Abuelnoor et al., 2021) | - Negative biodiversity impacts on forests maximising CO2 sequestration (Oliver et al., 2014) - Competition for limited land resources (Mishra et al., 2022) and wood applications (Hoxha et al., 2020) - Increased use of gluing products (Ramage et al., 2017) | - Stranded assets of conventional mineral construction (Churkina et al., 2020b) - Location-specificity of associated ecoonomic benefits (Talvitie et al., 2021) - Alternative bio-based materials to wood require economic incentives to unlock upscale (Kuittinen et al., 2021) | - Ensure sustainable forest management practices (Pomponi et al., 2020) - Skillset & workforce retraining (Churkina et al., 2020b) - Architectural codes and standards: avoid double-counting of carbon by cities and forests (Kinnunen et al., 2022) & CO2 reduction from biochar use within concrete (Winters et al., 2022) - Monitoring of end-of-life disposal of wood products (recycling, product lifetime extension) (Budzinski et al., 2020) - Addressing fire safety concern of end users (Kuzmanovska et al., 2018) - Implicit value judgement in time horizon considered (Levasseur et al., 2012) - Guidance on how to interpret LCA studies in building and city contexts (Minunno et al., 2021; Rasmussen et al., 2018) - Standards for like-for-like comparisons between studies (Arehart et al., 2021) - Legal issues with regards to responsibility and risk associated with CO2 storage (Habert et al., 2020) - Lack of supply chain education and communication (Qu et al., 2012) |
| Storing C below-ground | - Low technical readiness: potential to miniaturize and optimize the capture and conversion units to be proven (López et al., 2023) - Further testing of DAC technology required in different locations and climates (IEA, 2022b) - Availability of green energy heat or electricity sources (Baus & Nehr, 2022) - Impact of DAC retrofit on cooling tower’s performance (Noya, 2022) - Operational potential of DAC in ventilation systems: uncertain ventilation times, air CO2 concentrations, outdoor temperatures (Baus & Nehr, 2022; Sinha et al., 2018) - Lack of space that would allow for the installation of additional instrumentation and processes (López et al., 2023) | - Increased energy use and land requirements (Smith et al., 2016), leading to either GHG emissions or competition for renewable energy (Smith et al., 2023) - Sourcing and manufacture of sorbents for DACCS at the scales required (Realmonte et al., 2019)(Realmonte et al., 2019) - Effect of air quality parameters (other than CO2) with air recirculation (Baus & Nehr, 2022) - Increase water use with some DAC approaches (Smith et al., 2023) | - Cost and mechanisms of transport and storage of captured CO2 (Abdullatif et al., 2023) - Increased energy requirements for CO2 liquefaction and transport (despite energy savings from recirculation) (Baus & Nehr, 2022) - Feasibility strongly depends on affordable and long-lived sorbents (Baus & Nehr, 2022) - Revenue from CO2 utilisation (Fujikawa & Selyanchyn, 2022) | - CO2 market mechanisms unavailable or underdeveloped (Gambhir & Tavoni, 2019) - Co-location of DAC plants to benefit from shared resources and transport infrastructure (McQueen et al., 2022) - Currently no IPCC accounting methodology for DAC, thus cannot be counted towards meeting international mitigation targets under the UNFCCC (IEA, 2022b) |
| Surface Albedo Management | - Uncertainty and location-dependence of balance of energy cooling savings and heating penalties (Oleson et al., 2010) - Reduction in atmospheric temperature from reduced surface temperature (J. Yang et al., 2015) - Impact of local weather and urban geometry on CO2-equivalent savings (Morini et al., 2017) - Impact of increasing albedo on the urban environments (eg. on canopy air temperature, (Nazarian et al., 2019), or building walls) - Uncertain feedbacks at high latitudes over snow and sea ice might result in local cooling but global warming (Jacobson & Hoeve, 2012) | - Environmental impact over the entire material lifecycle (Gilbert et al., 2016) - Slower wind & increased levels of pollutants (Falasca et al., 2022) - Potential increase in heating energy consumption (Cubi et al., 2016) - Offset CO2-induced warming but cannot reverse effects of CO2 fertilization of plants (Govindasamy et al., 2002) - Direct tradeoff between surface temperature reduction and human heat load, reducing human comfort (Middel et al., 2020) | - Low upfront installation costs versus maintenance costs as materials degrade (Gilbert et al., 2016) - Existing CO2 markets do not allow trading of CO2-equivalent offsets for albedo management (Akbari & Matthews, 2012) - Lack of economic incentives: need to link environmental and socio-economic benefits of surface albedo increase - Economic valuation of surface albedo modification based on CO2 metrics is challenging due to lack of agreement on conversion methodology | - Lack of comparability of studies (regional, time horizon, and methodological variations & different lifecycles considered) (AzariJafari et al., 2021) - Implementation challenges related to fragmented ownership of different roofs (GCCA, 2012) - Lack of widespread labelling and certification requirements for roofing products (Akbari & Matthews, 2012) - Complexity of environmental benefits of coof roofs and pavements hinder widespread implementation (C. Wang et al., 2021) - Trade-offs between surface, air, and radiant temperature across times, locations, and scales is acknowledged by scientific community but less nuanced in policy applications (Middel et al., 2020) - Lack of agreement on the methodology used to convert surface albedo variation into CO2-equivalent metrics, lack of transparency due to user-dependent methodology choice (Bright & Lund, 2021) - Currently no existing (urban) carbon accounting framework considering the CO2-equivalent effect of albedo increase |

# Bibliography

Abdullatif, Y., Sodiq, A., Mir, N., Bicer, Y., Al-Ansari, T., H. El-Naas, M., & I. Amhamed, A. (2023). Emerging trends in direct air capture of CO 2: A review of technology options targeting net-zero emissions. RSC Advances, 13(9), 5687–5722. https://doi.org/10.1039/D2RA07940B

Abhishek, K., Shrivastava, A., Vimal, V., Gupta, A. K., Bhujbal, S. K., Biswas, J. K., Singh, L., Ghosh, P., Pandey, A., Sharma, P., & Kumar, M. (2022). Biochar application for greenhouse gas mitigation, contaminants immobilization and soil fertility enhancement: A state-of-the-art review. Science of The Total Environment, 853, 158562. https://doi.org/10.1016/j.scitotenv.2022.158562

Abuelnoor, N., AlHajaj, A., Khaleel, M., Vega, L. F., & Abu-Zahra, M. R. M. (2021). Activated carbons from biomass-based sources for CO2 capture applications. Chemosphere, 282, 131111. https://doi.org/10.1016/j.chemosphere.2021.131111

Akbari, H., & Matthews, H. D. (2012). Global cooling updates: Reflective roofs and pavements. Energy and Buildings, 55, 2–6. https://doi.org/10.1016/j.enbuild.2012.02.055

Akbari, H., Menon, S., & Rosenfeld, A. (2009). Global cooling: Increasing world-wide urban albedos to offset CO2. Climatic Change, 94(3), 275–286. https://doi.org/10.1007/s10584-008-9515-9

Akhtar, A., & Sarmah, A. K. (2018a). Novel biochar-concrete composites: Manufacturing, characterization and evaluation of the mechanical properties. Science of The Total Environment, 616–617, 408–416. https://doi.org/10.1016/j.scitotenv.2017.10.319

Akhtar, A., & Sarmah, A. K. (2018b). Strength improvement of recycled aggregate concrete through silicon rich char derived from organic waste. Journal of Cleaner Production, 196, 411–423. https://doi.org/10.1016/j.jclepro.2018.06.044

Akinyemi, B. A., & Adesina, A. (2020). Recent advancements in the use of biochar for cementitious applications: A review. Journal of Building Engineering, 32, 101705. https://doi.org/10.1016/j.jobe.2020.101705

Alhashimi, H. A., & Aktas, C. B. (2017). Life cycle environmental and economic performance of biochar compared with activated carbon: A meta-analysis. Resources, Conservation and Recycling, 118, 13–26. https://doi.org/10.1016/j.resconrec.2016.11.016

Almeida, C. M. V. B., Mariano, M. V., Agostinho, F., Liu, G. Y., & Giannetti, B. F. (2018). Exploring the potential of urban park size for the provision of ecosystem services to urban centres: A case study in São Paulo, Brazil. Building and Environment, 144, 450–458. https://doi.org/10.1016/j.buildenv.2018.08.036

Arehart, J. H., Hart, J., Pomponi, F., & D’Amico, B. (2021). Carbon sequestration and storage in the built environment. Sustainable Production and Consumption, 27, 1047–1063. https://doi.org/10.1016/j.spc.2021.02.028

Ariluoma, M., Ottelin, J., Hautamäki, R., Tuhkanen, E.-M., & Mänttäri, M. (2021). Carbon sequestration and storage potential of urban green in residential yards: A case study from Helsinki. Urban Forestry & Urban Greening, 57, 126939. https://doi.org/10.1016/j.ufug.2020.126939

Asadi Zeidabadi, Z., Bakhtiari, S., Abbaslou, H., & Ghanizadeh, A. R. (2018). Synthesis, characterization and evaluation of biochar from agricultural waste biomass for use in building materials. Construction and Building Materials, 181, 301–308. https://doi.org/10.1016/j.conbuildmat.2018.05.271

AzariJafari, H., Xu, X., Gregory, J., & Kirchain, R. (2021). Urban-Scale Evaluation of Cool Pavement Impacts on the Urban Heat Island Effect and Climate Change. Environmental Science & Technology, 55(17), 11501–11510. https://doi.org/10.1021/acs.est.1c00664

AzariJafari, H., Yahia, A., & Amor, B. (2019). Removing Shadows from Consequential LCA through a Time-Dependent Modeling Approach: Policy-Making in the Road Pavement Sector. Environmental Science & Technology, 53(3), 1087–1097. https://doi.org/10.1021/acs.est.8b02865

Azzi, E. S., Karltun, E., & Sundberg, C. (2022). Life cycle assessment of urban uses of biochar and case study in Uppsala, Sweden. Biochar, 4(1), 18. https://doi.org/10.1007/s42773-022-00144-3

Basak, B. B., Sarkar, B., Saha, A., Sarkar, A., Mandal, S., Biswas, J. K., Wang, H., & Bolan, N. S. (2022). Revamping highly weathered soils in the tropics with biochar application: What we know and what is needed. Science of The Total Environment, 822, 153461. https://doi.org/10.1016/j.scitotenv.2022.153461

Baus, L., & Nehr, S. (2022). Potentials and limitations of direct air capturing in the built environment. Building and Environment, 208, 108629. https://doi.org/10.1016/j.buildenv.2021.108629

Belletti, B., Bernardi, P., Malcevschi, A., & Sirico, A. (2019). Experimental research on mechanical properties of biochar-added cementitious mortars. Proceedings of the Fib Symposium 2019: Concrete - Innovations in Materials, Design and Structures. https://www.scopus.com/record/display.uri?eid=2-s2.0-85066072604&origin=inward&txGid=6e01aa36a25de3c900b5b4377e302c85

Ben Salem, I., El Gamal, M., Sharma, M., Hameedi, S., & Howari, F. M. (2021). Utilization of the UAE date palm leaf biochar in carbon dioxide capture and sequestration processes. Journal of Environmental Management, 299, 113644. https://doi.org/10.1016/j.jenvman.2021.113644

Betts, R. A. (2000). Offset of the potential carbon sink from boreal forestation by decreases in surface albedo. Nature, 408(6809), Article 6809. https://doi.org/10.1038/35041545

Boeing, G. (2022). Street Network Models and Indicators for Every Urban Area in the World. Geographical Analysis, 54(3), 519–535. https://doi.org/10.1111/gean.12281

Borchers, M., Thrän, D., Chi, Y., Dahmen, N., Dittmeyer, R., Dolch, T., Dold, C., Förster, J., Herbst, M., Heß, D., Kalhori, A., Koop-Jakobsen, K., Li, Z., Mengis, N., Reusch, T. B. H., Rhoden, I., Sachs, T., Schmidt-Hattenberger, C., Stevenson, A., … Yeates, C. (2022). Scoping carbon dioxide removal options for Germany–What is their potential contribution to Net-Zero CO2? Frontiers in Climate, 4. https://www.frontiersin.org/articles/10.3389/fclim.2022.810343

Bordoloi, S., Gopal, P., Boddu, R., Wang, Q., Cheng, Y.-F., Garg, A., & S, S. (2019). Soil-biochar-water interactions: Role of biochar from Eichhornia crassipes in influencing crack propagation and suction in unsaturated soils. Journal of Cleaner Production, 210, 847–859. https://doi.org/10.1016/j.jclepro.2018.11.051

Bright, R. M., & Lund, M. T. (2021). CO2-equivalence metrics for surface albedo change based on the radiative forcing concept: A critical review. Atmospheric Chemistry and Physics, 21(12), 9887–9907. https://doi.org/10.5194/acp-21-9887-2021

Brilli, L., Carotenuto, F., Chiesi, M., Fiorillo, E., Genesio, L., Magno, R., Morabito, M., Nardino, M., Zaldei, A., & Gioli, B. (2022). An integrated approach to estimate how much urban afforestation can contribute to move towards carbon neutrality. Science of The Total Environment, 842, 156843. https://doi.org/10.1016/j.scitotenv.2022.156843

Bryan, H., & Salamah, F. B. (2019). Building-integrated carbon capture: : Carbon Dioxide Removal through Buildings’ Mechanical Systems. 320–327. http://www.scopus.com/inward/record.url?scp=85093975037&partnerID=8YFLogxK

Budzinski, M., Bezama, A., & Thrän, D. (2020). Estimating the potentials for reducing the impacts on climate change by increasing the cascade use and extending the lifetime of wood products in Germany. Resources, Conservation & Recycling: X, 6, 100034. https://doi.org/10.1016/j.rcrx.2020.100034

Bydén, W., & Fridlund, D. (2020). Carbon Negative Heat and Power with Biochar Production: An Economic Analysis of a Combined Pyrolysis and CHP plant.

Cascone, S., Catania, F., Gagliano, A., & Sciuto, G. (2018). A comprehensive study on green roof performance for retrofitting existing buildings. Building and Environment, 136, 227–239. https://doi.org/10.1016/j.buildenv.2018.03.052

Cazemier, D. S. (2017). Comparing cross laminated timber with concrete and steel: A financial analysis of two buildings in Australia. Modular and Offsite Construction (MOC) Summit Proceedings. https://doi.org/10.29173/mocs63

Chakraborty, T., Biswas, T., Campbell, L. S., Franklin, B., Parker, S. S., & Tukman, M. (2022). Feasibility of afforestation as an equitable nature-based solution in urban areas. Sustainable Cities and Society, 81, 103826. https://doi.org/10.1016/j.scs.2022.103826

Cheela, V. R. S., John, M., Biswas, W., & Sarker, P. (2021). Combating Urban Heat Island Effect—A Review of Reflective Pavements and Tree Shading Strategies. Buildings, 11(3), Article 3. https://doi.org/10.3390/buildings11030093

Chen, H., Ma, J., Wang, X., Xu, P., Zheng, S., & Zhao, Y. (2018). Effects of Biochar and Sludge on Carbon Storage of Urban Green Roofs. Forests, 9(7), 413. https://doi.org/10.3390/f9070413

Chen, L., Zhang, Y., Wang, L., Ruan, S., Chen, J., Li, H., Yang, J., Mechtcherine, V., & Tsang, D. C. W. (2022). Biochar-augmented carbon-negative concrete. Chemical Engineering Journal, 431, 133946. https://doi.org/10.1016/j.cej.2021.133946

Chen, W. Y. (2015). The role of urban green infrastructure in offsetting carbon emissions in 35 major Chinese cities: A nationwide estimate. Cities, 44, 112–120. https://doi.org/10.1016/j.cities.2015.01.005

Chu, Y., Xu, P., Jia, Y., Lin, M., Peng, C., & Dou, Q. (2021). An adsorption-based CO2 treatment unit for ultra-low fresh air HVAC system using solid amine. Energy and Buildings, 247, 111148. https://doi.org/10.1016/j.enbuild.2021.111148

Churkina, G. (2016). The Role of Urbanization in the Global Carbon Cycle. Frontiers in Ecology and Evolution, 3. https://www.frontiersin.org/articles/10.3389/fevo.2015.00144

Churkina, G., Organschi, A., Reyer, C. P. O., Ruff, A., Vinke, K., Liu, Z., Reck, B. K., Graedel, T. E., & Schellnhuber, H. J. (2020a). Buildings as a global carbon sink. Nature Sustainability, 3(4), 269–276. https://doi.org/10.1038/s41893-019-0462-4

Churkina, G., Organschi, A., Reyer, C. P. O., Ruff, A., Vinke, K., Liu, Z., Reck, B. K., Graedel, T. E., & Schellnhuber, H. J. (2020b). Buildings as a global carbon sink. Nature Sustainability, 3(4), 269–276. https://doi.org/10.1038/s41893-019-0462-4

Cortinovis, C., Olsson, P., Boke-Olén, N., & Hedlund, K. (2022). Scaling up nature-based solutions for climate-change adaptation: Potential and benefits in three European cities. Urban Forestry & Urban Greening, 67, 127450. https://doi.org/10.1016/j.ufug.2021.127450

Cousins, J. J. (2021). Justice in nature-based solutions: Research and pathways. Ecological Economics, 180, 106874. https://doi.org/10.1016/j.ecolecon.2020.106874

Creamer, A. E., & Gao, B. (2016). Carbon-Based Adsorbents for Postcombustion CO2 Capture: A Critical Review. Environmental Science & Technology, 50(14), 7276–7289. https://doi.org/10.1021/acs.est.6b00627

Cubi, E., Zibin, N. F., Thompson, S. J., & Bergerson, J. (2016). Sustainability of Rooftop Technologies in Cold Climates: Comparative Life Cycle Assessment of White Roofs, Green Roofs, and Photovoltaic Panels. Journal of Industrial Ecology, 20(2), 249–262. https://doi.org/10.1111/jiec.12269

Cuthbertson, D., Berardi, U., Briens, C., & Berruti, F. (2019). Biochar from residual biomass as a concrete filler for improved thermal and acoustic properties. Biomass and Bioenergy, 120, 77–83. https://doi.org/10.1016/j.biombioe.2018.11.007

D’Amico, B., Pomponi, F., & Hart, J. (2021). Global potential for material substitution in building construction: The case of cross laminated timber. Journal of Cleaner Production, 279, 123487. https://doi.org/10.1016/j.jclepro.2020.123487

Das, O., Sarmah, A. K., & Bhattacharyya, D. (2015). A novel approach in organic waste utilization through biochar addition in wood/polypropylene composites. Waste Management, 38, 132–140. https://doi.org/10.1016/j.wasman.2015.01.015

Davies, C., & Lafortezza, R. (2019). Transitional path to the adoption of nature-based solutions. Land Use Policy, 80, 406–409. https://doi.org/10.1016/j.landusepol.2018.09.020

Davies, Z. G., Edmondson, J. L., Heinemeyer, A., Leake, J. R., & Gaston, K. J. (2011). Mapping an urban ecosystem service: Quantifying above-ground carbon storage at a city-wide scale. Journal of Applied Ecology, 48(5), 1125–1134. https://doi.org/10.1111/j.1365-2664.2011.02021.x

Davis, A. P., Hunt, W. F., Traver, R. G., & Clar, M. (2009). Bioretention Technology: Overview of Current Practice and Future Needs. Journal of Environmental Engineering, 135(3), 109–117. https://doi.org/10.1061/(ASCE)0733-9372(2009)135:3(109)

Deetman, S., Marinova, S., van der Voet, E., van Vuuren, D. P., Edelenbosch, O., & Heijungs, R. (2020). Modelling global material stocks and flows for residential and service sector buildings towards 2050. Journal of Cleaner Production, 245, 118658. https://doi.org/10.1016/j.jclepro.2019.118658

Di Bonito, R., Biagiotti, D., Giagnacovo, G., Viola, C., & Campiotti, C. A. (2018). Sustainable and energy saving urban horticulture on rooftop gardens in Mediterranean climatic conditions. Acta Horticulturae, 1215, 383–388. https://doi.org/10.17660/ActaHortic.2018.1215.70

Ding, S., & Liu, Y. (2020). Adsorption of CO2 from flue gas by novel seaweed-based KOH-activated porous biochars. Fuel, 260, 116382. https://doi.org/10.1016/j.fuel.2019.116382

Dissanayake, P. D., You, S., Igalavithana, A. D., Xia, Y., Bhatnagar, A., Gupta, S., Kua, H. W., Kim, S., Kwon, J.-H., Tsang, D. C. W., & Ok, Y. S. (2020). Biochar-based adsorbents for carbon dioxide capture: A critical review. Renewable and Sustainable Energy Reviews, 119, 109582. https://doi.org/10.1016/j.rser.2019.109582

Dittmeyer, R., Klumpp, M., Kant, P., & Ozin, G. (2019). Crowd oil not crude oil. Nature Communications, 10(1), Article 1. https://doi.org/10.1038/s41467-019-09685-x

Dixit, A., Gupta, S., Pang, S. D., & Kua, H. W. (2019). Waste Valorisation using biochar for cement replacement and internal curing in ultra-high performance concrete. Journal of Cleaner Production, 238, 117876. https://doi.org/10.1016/j.jclepro.2019.117876

Dong, J., Lin, M., Zuo, J., Lin, T., Liu, J., Sun, C., & Luo, J. (2020). Quantitative study on the cooling effect of green roofs in a high-density urban Area—A case study of Xiamen, China. Journal of Cleaner Production, 255, 120152. https://doi.org/10.1016/j.jclepro.2020.120152

Dorendorf, J., Eschenbach, A., Schmidt, K., & Jensen, K. (2015). Both tree and soil carbon need to be quantified for carbon assessments of cities. Urban Forestry & Urban Greening, 14(3), 447–455. https://doi.org/10.1016/j.ufug.2015.04.005

EBC. (2022). Guidelines of the European Biochar Certificate (Version 10.1). Ithaka Institute. https://www.european-biochar.org/media/doc/2/version_en_10_1.pdf

Embren, B. (2016). Planting Urban Trees with Biochar. The Biochar Journal, Arbaz, Switzerland. https://www.biochar-journal.org/en/ct/77

Enríquez, E., Fuertes, V., Cabrera, M. J., Seores, J., Muñoz, D., & Fernández, J. F. (2017). New strategy to mitigate urban heat island effect: Energy saving by combining high albedo and low thermal diffusivity in glass ceramic materials. Solar Energy, 149, 114–124. https://doi.org/10.1016/j.solener.2017.04.011

Escobedo, F., Varela, S., Zhao, M., Wagner, J. E., & Zipperer, W. (2010). Analyzing the efficacy of subtropical urban forests in offsetting carbon emissions from cities. Environmental Science & Policy, 13(5), 362–372. https://doi.org/10.1016/j.envsci.2010.03.009

Esperon-Rodriguez, M., Tjoelker, M. G., Lenoir, J., Baumgartner, J. B., Beaumont, L. J., Nipperess, D. A., Power, S. A., Richard, B., Rymer, P. D., & Gallagher, R. V. (2022). Climate change increases global risk to urban forests. Nature Climate Change, 12(10), Article 10. https://doi.org/10.1038/s41558-022-01465-8

Green Infrastructure (GI)—Enhancing Europe’s Natural Capital, COMMUNICATION FROM THE COMMISSION TO THE EUROPEAN PARLIAMENT, THE COUNCIL, THE EUROPEAN ECONOMIC AND SOCIAL COMMITTEE AND THE COMMITTEE OF THE REGIONS (2013) (testimony of European Commission). https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX:52013DC0249

European Commission. (2015). Towards an EU research and innovation policy agenda for nature-based solutions and re-naturing cities :final report of the Horizon 2020 expert group on ‘Nature based solutions and re naturing cities’: (Full version). (Directorate General for Research and Innovation.). Publications Office. https://data.europa.eu/doi/10.2777/479582

Falasca, S., Zinzi, M., Ding, L., Curci, G., & Santamouris, M. (2022). On the mitigation potential of higher urban albedo in a temperate oceanic metropolis. Sustainable Cities and Society, 81, 103850. https://doi.org/10.1016/j.scs.2022.103850

Fan, Y., & Wei, F. (2022). Contributions of Natural Carbon Sink Capacity and Carbon Neutrality in the Context of Net-Zero Carbon Cities: A Case Study of Hangzhou. Sustainability, 14(5), Article 5. https://doi.org/10.3390/su14052680

Fares, S., Conte, A., Alivernini, A., Chianucci, F., Grotti, M., Zappitelli, I., Petrella, F., & Corona, P. (2020). Testing Removal of Carbon Dioxide, Ozone, and Atmospheric Particles by Urban Parks in Italy. Environmental Science & Technology, 54(23), 14910–14922. https://doi.org/10.1021/acs.est.0c04740

Farrell, C., Cao, C. T. N., Ang, X. Q., & Rayner, J. P. (2016). Use of water-retention additives to improve performance of green roof substrates. Acta Horticulturae, 1108, 271–278. https://doi.org/10.17660/ActaHortic.2016.1108.35

Fransson, A., Gustafsson, M., Malmberg, J., & Paulsson, M. (2020). The Biochar Handbook – for users. https://biokol.org/publikationer/pdf/biochar-handbook-for-users

Frantzeskaki, N. (2019). Seven lessons for planning nature-based solutions in cities. Environmental Science & Policy, 93, 101–111. https://doi.org/10.1016/j.envsci.2018.12.033

Frazer, L. (2005). Paving Paradise: The Peril of Impervious Surfaces. Environmental Health Perspectives, 113(7), A456–A462. https://doi.org/10.1289/ehp.113-a456

Fujikawa, S., & Selyanchyn, R. (2022). Direct air capture by membranes. MRS Bulletin, 47(4), 416–423. https://doi.org/10.1557/s43577-022-00313-6

Furey, G. N., & Tilman, D. (2021). Plant biodiversity and the regeneration of soil fertility. Proceedings of the National Academy of Sciences, 118(49), e2111321118. https://doi.org/10.1073/pnas.2111321118

Gali, M. A., Arnold, M. D., Gentle, A. R., & Smith, G. B. (2017). Super-cool paints: Optimizing composition with a modified four-flux model. Thermal Radiation Management for Energy Applications, 10369, 6–15. https://doi.org/10.1117/12.2273548

Gall, E. T., Sonat, C., Nazaroff, W. W., & Unluer, C. (2016). Investigating CO2 removal by Ca- and Mg-based sorbents with application to indoor air treatment. Building and Environment, 110, 161–172. https://doi.org/10.1016/j.buildenv.2016.10.008

Gambhir, A., & Tavoni, M. (2019). Direct Air Carbon Capture and Sequestration: How It Works and How It Could Contribute to Climate-Change Mitigation. One Earth, 1(4), 405–409. https://doi.org/10.1016/j.oneear.2019.11.006

Gao, Y., & Meng, X. (2023). A comprehensive review of integrating phase change materials in building bricks: Methods, performance and applications. Journal of Energy Storage, 62, 106913. https://doi.org/10.1016/j.est.2023.106913

GCCA. (2012). A Practical Guide to Cool Roofs and Cool Pavements Retrieved. Global Cool Cities Alliance (GCCA) and R20 Regions of Climate Action. https://www.coolrooftoolkit.org/wp-content/pdfs/CoolRoofToolkit_Full.pdf

George, A. M. (2012). The Potential Carbon Offset Represented by a Green Roof [Bachelor of Science in Landscape Architecture, West Virginia University]. https://scholar.archive.org/work/4q3jcdawqvbxbbdht6mqgxbuim/access/wayback/https://libraetd.lib.virginia.edu/downloads/rb68xc135?filename=George_AmmyMarie_2012.pdf

Getter, K. L., Rowe, D. B., Robertson, G. P., Cregg, B. M., & Andresen, J. A. (2009). Carbon Sequestration Potential of Extensive Green Roofs. Environmental Science & Technology, 43(19), 7564–7570. https://doi.org/10.1021/es901539x

Ghosh, S., Ow, L. F., & Wilson, B. (2015). Influence of biochar and compost on soil properties and tree growth in a tropical urban environment. International Journal of Environmental Science and Technology, 12(4), 1303–1310. https://doi.org/10.1007/s13762-014-0508-0

Ghosh, S., Yeo, D., Wilson, B., & Ow, L. F. (2012). Application of char products improves urban soil quality. Soil Use and Management, 28(3), 329–336. https://doi.org/10.1111/j.1475-2743.2012.00416.x

Gilbert, H., Mandel, B. H., & Levinson, R. (2016). Keeping California cool: Recent cool community developments. Energy and Buildings, 114, 20–26. https://doi.org/10.1016/j.enbuild.2015.06.023

Goddard, M. A., Dougill, A. J., & Benton, T. G. (2010). Scaling up from gardens: Biodiversity conservation in urban environments. Trends in Ecology & Evolution, 25(2), 90–98. https://doi.org/10.1016/j.tree.2009.07.016

Govindasamy, B., Thompson, S., Duffy, P. B., Caldeira, K., & Delire, C. (2002). Impact of geoengineering schemes on the terrestrial biosphere. Geophysical Research Letters, 29(22), 18-1-18–4. https://doi.org/10.1029/2002GL015911

Grace, M., Balzan, M., Collier, M., Geneletti, D., Tomaskinova, J., Abela, R., Borg, D., Buhagiar, G., Camilleri, L., Cardona, M., Cassar, N., Cassar, R., Cattafi, I., Cauchi, D., Galea, C., La Rosa, D., Malekkidou, E., Masini, M., Portelli, P., … Dicks, L. V. (2021). Priority knowledge needs for implementing nature-based solutions in the Mediterranean islands. Environmental Science & Policy, 116, 56–68. https://doi.org/10.1016/j.envsci.2020.10.003

Guest, G., Zhang, J., Maadani, O., & Shirkhani, H. (2020). Incorporating the impacts of climate change into infrastructure life cycle assessments: A case study of pavement service life performance. Journal of Industrial Ecology, 24(2), 356–368. https://doi.org/10.1111/jiec.12915

Gupta, S., & Kashani, A. (2021). Utilization of biochar from unwashed peanut shell in cementitious building materials – Effect on early age properties and environmental benefits. Fuel Processing Technology, 218, 106841. https://doi.org/10.1016/j.fuproc.2021.106841

Gupta, S., Kashani, A., Mahmood, A. H., & Han, T. (2021). Carbon sequestration in cementitious composites using biochar and fly ash – Effect on mechanical and durability properties. Construction and Building Materials, 291, 123363. https://doi.org/10.1016/j.conbuildmat.2021.123363

Gupta, S., & Kua, H. W. (2017). Factors Determining the Potential of Biochar As a Carbon Capturing and Sequestering Construction Material: Critical Review. Journal of Materials in Civil Engineering, 29(9), 04017086. https://doi.org/10.1061/(ASCE)MT.1943-5533.0001924

Gupta, S., Kua, H. W., & Koh, H. J. (2018). Application of biochar from food and wood waste as green admixture for cement mortar. Science of The Total Environment, 619–620, 419–435. https://doi.org/10.1016/j.scitotenv.2017.11.044

Gupta, S., Kua, H. W., & Low, C. Y. (2018). Use of biochar as carbon sequestering additive in cement mortar. Cement and Concrete Composites, 87, 110–129. https://doi.org/10.1016/j.cemconcomp.2017.12.009

Habert, G., Miller, S. A., John, V. M., Provis, J. L., Favier, A., Horvath, A., & Scrivener, K. L. (2020). Environmental impacts and decarbonization strategies in the cement and concrete industries. Nature Reviews Earth & Environment, 1(11), Article 11. https://doi.org/10.1038/s43017-020-0093-3

Häyrinen, L., Toppinen, A., & Toivonen, R. (2020). Finnish young adults’ perceptions of the health, well-being and sustainability of wooden interior materials. Scandinavian Journal of Forest Research, 35(7), 394–402. https://doi.org/10.1080/02827581.2020.1813798

Heräjärvi, H. (2019). Wooden buildings as carbon storages – Mitigation or oration? Wood Material Science & Engineering, 14(5), 291–297. https://doi.org/10.1080/17480272.2019.1635205

Heusinger, J., & Weber, S. (2017). Extensive green roof CO2 exchange and its seasonal variation quantified by eddy covariance measurements. Science of The Total Environment, 607–608, 623–632. https://doi.org/10.1016/j.scitotenv.2017.07.052

Hildebrandt, J., Hagemann, N., & Thrän, D. (2017). The contribution of wood-based construction materials for leveraging a low carbon building sector in europe. Sustainable Cities and Society, 34, 405–418. https://doi.org/10.1016/j.scs.2017.06.013

Himes, A., & Busby, G. (2020). Wood buildings as a climate solution. Developments in the Built Environment, 4, 100030. https://doi.org/10.1016/j.dibe.2020.100030

Hoxha, E., Passer, A., Saade, M. R. M., Trigaux, D., Shuttleworth, A., Pittau, F., Allacker, K., & Habert, G. (2020). Biogenic carbon in buildings: A critical overview of LCA methods. Buildings and Cities, 1(1), Article 1. https://doi.org/10.5334/bc.46

Hsu, A., Logan, K., Qadir, M., Booysen, M. J. (Thinus), Montero, A. M., Tong, K. (K. K. ), Broadbent, G., Wiedmann, T., Sin Woon, V. K., Good, C., Colding, J., Foliente, G., & Kılkış, Ş. (2022). Opportunities and barriers to net-zero cities. One Earth, 5(7), 739–744. https://doi.org/10.1016/j.oneear.2022.06.013

Huang, S., Garg, A., Mei, G., Huang, D., Chandra, R. B., & Sadasiv, S. G. (2020). Experimental study on the hydrological performance of green roofs in the application of novel biochar. Hydrological Processes, 34(23), 4512–4525. https://doi.org/10.1002/hyp.13881

Huntingford, C., & Oliver, R. J. (2021). Constraints on estimating the CO2 fertilization effect emerge. Nature, 600(7888), 224–225. https://doi.org/10.1038/d41586-021-03560-w

IEA. (2018). Technology Roadmap—Low-Carbon Transition in the Cement Industry. International Energy Agency. https://www.iea.org/reports/technology-roadmap-low-carbon-transition-in-the-cement-industry

IEA. (2022a). Cement. International Energy Agency. https://www.iea.org/reports/cement

IEA. (2022b). Direct Air Capture. IEA. https://www.iea.org/reports/direct-air-capture-3

Ismail, A., Abdul Samad, M. H., Rahman, A. M. A., & Yeok, F. S. (2012). Cooling Potentials and CO2 Uptake of Ipomoea Pes-caprae Installed on the Flat Roof of a Single Storey Residential Building in Malaysia. Procedia - Social and Behavioral Sciences, 35, 361–368. https://doi.org/10.1016/j.sbspro.2012.02.099

Jacobson, M. Z., & Hoeve, J. E. T. (2012). Effects of Urban Surfaces and White Roofs on Global and Regional Climate. Journal of Climate, 25(3), 1028–1044. https://doi.org/10.1175/JCLI-D-11-00032.1

Jeon, J., Park, J. H., Yuk, H., Kim, Y. U., Yun, B. Y., Wi, S., & Kim, S. (2021). Evaluation of hygrothermal performance of wood-derived biocomposite with biochar in response to climate change. Environmental Research, 193, 110359. https://doi.org/10.1016/j.envres.2020.110359

Ji, Y., Yong, J. Y., Liu, W., Zhang, X. J., & Jiang, L. (2022). Thermodynamic analysis on direct air capture for building air condition system: Balance between adsorbent and refrigerant. Energy and Built Environment. https://doi.org/10.1016/j.enbenv.2022.02.009

Jiang, W., Xu, J., Cai, Y., & Liu, Z. (2020). Ecological Land Adaptive Planning in Macroscale, Mesoscale, and Microscale of Shanghai. Sustainability, 12(5), Article 5. https://doi.org/10.3390/su12052142

Johnston, C. M. T., & Radeloff, V. C. (2019). Global mitigation potential of carbon stored in harvested wood products. Proceedings of the National Academy of Sciences, 116(29), 14526–14531. https://doi.org/10.1073/pnas.1904231116

Kabisch, N., Frantzeskaki, N., & Hansen, R. (2022). Principles for urban nature-based solutions. Ambio, 51(6), 1388–1401. https://doi.org/10.1007/s13280-021-01685-w

Kabisch, N., Frantzeskaki, N., Pauleit, S., Naumann, S., Davis, M., Artmann, M., Haase, D., Knapp, S., Korn, H., Stadler, J., Zaunberger, K., & Bonn, A. (2016). Nature-based solutions to climate change mitigation and adaptation in urban areas: Perspectives on indicators, knowledge gaps, barriers, and opportunities for action. Ecology and Society, 21(2). https://www.jstor.org/stable/26270403

Kasprzyk, I., Ćwik, A., Kluska, K., Wójcik, T., & Cariñanos, P. (2019). Allergenic pollen concentrations in the air of urban parks in relation to their vegetation. Urban Forestry & Urban Greening, 46, 126486. https://doi.org/10.1016/j.ufug.2019.126486

Keeler, B. L., Hamel, P., McPhearson, T., Hamann, M. H., Donahue, M. L., Meza Prado, K. A., Arkema, K. K., Bratman, G. N., Brauman, K. A., Finlay, J. C., Guerry, A. D., Hobbie, S. E., Johnson, J. A., MacDonald, G. K., McDonald, R. I., Neverisky, N., & Wood, S. A. (2019). Social-ecological and technological factors moderate the value of urban nature. Nature Sustainability, 2(1), Article 1. https://doi.org/10.1038/s41893-018-0202-1

Kim, M. K., Baldini, L., Leibundgut, H., Wurzbacher, J. A., & Piatkowski, N. (2015). A novel ventilation strategy with CO2 capture device and energy saving in buildings. Energy and Buildings, 87, 134–141. https://doi.org/10.1016/j.enbuild.2014.11.017

Kim, M. K., & Leibundgut, H. (2015). Performance of novel ventilation strategy for capturing CO2 with scheduled occupancy diversity and infiltration rate. Building and Environment, 89, 318–326. https://doi.org/10.1016/j.buildenv.2015.02.012

Kim, Y. J., Hyun, J., Yoo, S. Y., & Yoo, G. (2021). The role of biochar in alleviating soil drought stress in urban roadside greenery. Geoderma, 404, 115223. https://doi.org/10.1016/j.geoderma.2021.115223

Kinnunen, A., Talvitie, I., Ottelin, J., Heinonen, J., & Junnila, S. (2022). Carbon sequestration and storage potential of urban residential environment – A review. Sustainable Cities and Society, 84, 104027. https://doi.org/10.1016/j.scs.2022.104027

Kiss, M., Takács, Á., Pogácsás, R., & Gulyás, Á. (2015). The role of ecosystem services in climate and air quality in urban areas: Evaluating carbon sequestration and air pollution removal by street and park trees in Szeged (Hungary). Moravian Geographical Reports, 23(3), 36–46. https://doi.org/10.1515/mgr-2015-0016

Kovac, A., Bhatia, P., Russell, B., Morris, E., van Staden, M., Arikan, Y., Eichel, A., Dickinson, J., Desai, R., Hoornweg, D., Harris, N., Carreño, C., Lee, D., Ramlow, M., Wang, M., Green, R., Rich, D., Singh, N., Reyna, N., … Quinn, R. (2020). Global Protocol for Community-Scale—Greenhouse Gas Inventories (An Accounting and Reporting Standard for Cities, p. 190). World Resources Institute. https://ghgprotocol.org/sites/default/files/standards/GPC_Full_MASTER_RW_v7.pdf

Kremer, P. D., & Ritchie, L. (2018). Understanding Costs and Identifying Value in Mass Timber Construction: Calculating the ‘Total Cost of Project’ (TCP). Mass Timber Construction Journal, 1(1), Article 1.

Krull, K. H. (2018). Biochar Market Analysis For San Juan County And the Pacific Northwest. Northwest Natural Resource Group. https://www.nnrg.org/wp-content/uploads/2018/12/Biochar-Market-Research-Paper-FINAL.pdf

Kuittinen, M., Zernicke, C., Slabik, S., & Hafner, A. (2021). How can carbon be stored in the built environment? A review of potential options. Architectural Science Review, 0(0), 1–17. https://doi.org/10.1080/00038628.2021.1896471

Kumar, A., Choudhary, R., Narzari, R., Kataki, R., & Shukla, S. K. (2018). Evaluation of bio-asphalt binders modified with biochar: A pyrolysis by-product of Mesua ferrea seed cover waste. Cogent Engineering, 5(1), 1548534. https://doi.org/10.1080/23311916.2018.1548534

Kumar Mishra, R., Jaya Prasanna Kumar, D., Narula, A., Minnat Chistie, S., & Ullhas Naik, S. (2023). Production and beneficial impact of biochar for environmental application: A review on types of feedstocks, chemical compositions, operating parameters, techno-economic study, and life cycle assessment. Fuel, 343, 127968. https://doi.org/10.1016/j.fuel.2023.127968

Kuppusamy, S., Thavamani, P., Megharaj, M., Venkateswarlu, K., & Naidu, R. (2016). Agronomic and remedial benefits and risks of applying biochar to soil: Current knowledge and future research directions. Environment International, 87, 1–12. https://doi.org/10.1016/j.envint.2015.10.018

Kuronuma, T., & Watanabe, H. (2017). Relevance of Carbon Sequestration to the Physiological and Morphological Traits of Several Green Roof Plants during the First Year after Construction. American Journal of Plant Sciences, 08(01), 14–27. https://doi.org/10.4236/ajps.2017.81002

Kuronuma, T., Watanabe, H., Ishihara, T., Kou, D., Toushima, K., Ando, M., & Shindo, S. (2018). CO2 Payoff of Extensive Green Roofs with Different Vegetation Species. Sustainability, 10(7), Article 7. https://doi.org/10.3390/su10072256

Kuzmanovska, I., Gasparri, E., Tapias Monné, D., & Aitchison, M. (2018, August). Tall Timber Buildings: Emerging trends and typologies: 2018 World Conference on Timber Engineering.

Lähtinen, K., Harju, C., & Toppinen, A. (2019). Consumers’ perceptions on the properties of wood affecting their willingness to live in and prejudices against houses made of timber. Wood Material Science & Engineering, 14(5), 325–331. https://doi.org/10.1080/17480272.2019.1615548

Lee, D.-H., Kil, S.-H., Jo, H.-K., & Choi, B. (2019). Spatial Distributions of Carbon Storage and Uptake of Urban Forests in Seoul, South Korea. Sensors and Materials, 31(11), 3811. https://doi.org/10.18494/SAM.2019.2556

Lee, H., Yang, S., Wi, S., & Kim, S. (2019). Thermal transfer behavior of biochar-natural inorganic clay composite for building envelope insulation. Construction and Building Materials, 223, 668–678. https://doi.org/10.1016/j.conbuildmat.2019.06.215

Lee, S. C., Hsieh, C. C., Chen, C. H., & Chen, Y. S. (2013). CO2 Adsorption by Y-Type Zeolite Impregnated with Amines in Indoor Air. Aerosol and Air Quality Research, 13(1), 360–366. https://doi.org/10.4209/aaqr.2012.05.0134

Legan, M., Gotvajn, A. Ž., & Zupan, K. (2022). Potential of biochar use in building materials. Journal of Environmental Management, 309, 114704. https://doi.org/10.1016/j.jenvman.2022.114704

Levasseur, A., Brandão, M., Lesage, P., Margni, M., Pennington, D., Clift, R., & Samson, R. (2012). Valuing temporary carbon storage. Nature Climate Change, 2(1), Article 1. https://doi.org/10.1038/nclimate1335

Lin, J., Ma, Q., Ju, Y., Zhang, H., Wang, Q., & Huang, B. (2022). Relationships between urbanization, tree morphology, and carbon density: An integration of remote sensing, allometric models, and field survey. Urban Forestry & Urban Greening, 76, 127725. https://doi.org/10.1016/j.ufug.2022.127725

Lindén, L., Riikonen, A., Setälä, H., & Yli-Pelkonen, V. (2020). Quantifying carbon stocks in urban parks under cold climate conditions. Urban Forestry & Urban Greening, 49, 126633. https://doi.org/10.1016/j.ufug.2020.126633

Liu, C., & Li, X. (2012). Carbon storage and sequestration by urban forests in Shenyang, China. Urban Forestry & Urban Greening, 11(2), 121–128. https://doi.org/10.1016/j.ufug.2011.03.002

Liu, J., Liu, G., Zhang, W., Li, Z., Xing, F., & Tang, L. (2022). Application potential analysis of biochar as a carbon capture material in cementitious composites: A review. Construction and Building Materials, 350, 128715. https://doi.org/10.1016/j.conbuildmat.2022.128715

Liu, J., Zhang, W., Jin, H., Li, Z., Liu, G., Xing, F., & Tang, L. (2023). Exploring the carbon capture and sequestration performance of biochar-artificial aggregate using a new method. Science of The Total Environment, 859, 160423. https://doi.org/10.1016/j.scitotenv.2022.160423

Lo Piccolo, E., Becagli, M., Lauria, G., Cantini, V., Ceccanti, C., Cardelli, R., Massai, R., Remorini, D., Guidi, L., & Landi, M. (2022). Biochar as a soil amendment in the tree establishment phase: What are the consequences for tree physiology, soil quality and carbon sequestration? Science of The Total Environment, 844, 157175. https://doi.org/10.1016/j.scitotenv.2022.157175

Loijos, A., Santero, N., & Ochsendorf, J. (2013). Life cycle climate impacts of the US concrete pavement network. Resources, Conservation and Recycling, 72, 76–83. https://doi.org/10.1016/j.resconrec.2012.12.014

López, L. R., Dessì, P., Cabrera-Codony, A., Rocha-Melogno, L., Kraakman, B., Naddeo, V., Balaguer, M. D., & Puig, S. (2023). CO2 in indoor environments: From environmental and health risk to potential renewable carbon source. Science of The Total Environment, 856, 159088. https://doi.org/10.1016/j.scitotenv.2022.159088

Lu, L., Yu, W., Wang, Y., Zhang, K., Zhu, X., Zhang, Y., Wu, Y., Ullah, H., Xiao, X., & Chen, B. (2020). Application of biochar-based materials in environmental remediation: From multi-level structures to specific devices. Biochar, 2(1), 1–31. https://doi.org/10.1007/s42773-020-00041-7

Luo, H., Liu, X., Anderson, B. C., Zhang, K., Li, X., Huang, B., Li, M., Mo, Y., Fan, L., Shen, Q., Chen, F., & Jiang, M. (2015). Carbon sequestration potential of green roofs using mixed-sewage-sludge substrate in Chengdu World Modern Garden City. Ecological Indicators, 49, 247–259. https://doi.org/10.1016/j.ecolind.2014.10.016

Macauley, N. (2020). Impact of Carbon Sinks on Urban Heat Island Effects: Assessment Using Satellite Data in Water Scarce Region of the Thesis. http://urn.kb.se/resolve?urn=urn:nbn:se:hig:diva-33414

Maljaee, H., Madadi, R., Paiva, H., Tarelho, L., & Ferreira, V. M. (2021). Incorporation of biochar in cementitious materials: A roadmap of biochar selection. Construction and Building Materials, 283, 122757. https://doi.org/10.1016/j.conbuildmat.2021.122757

Mandal, S., Chatterjee, P., Das, N., Banerjee, R., Batabyal, S., Gangopadhyay, S., & Mondal, A. (2022). Modelling the role of urban forest in the regulation of carbon balance in an industrial area of India. Acta Ecologica Sinica, 42(5), 553–564. https://doi.org/10.1016/j.chnaes.2022.05.005

McDonald, R., Kroeger, T., Boucher, T., Longzhu, W., & Salem, R. (2016). Planting Healthy Air—A global analysis of the role of urban trees in addressing particulate matter pollution and extreme heat. The Nature Conservancy in contribution with C40. https://www.nature.org/content/dam/tnc/nature/en/documents/20160825_PHA_Report_Final.pdf

McLaren, D. (2012). A comparative global assessment of potential negative emissions technologies. Process Safety and Environmental Protection, 90(6), 489–500.

McQueen, N., Ghoussoub, M., Mills, J., & Scholten, M. (2022). A scalable direct air capture process based on accelerated weathering of calcium hydroxide [White Paper]. Heirloom. https://uploads-ssl.webflow.com/6041330ff151737fb03fc474/6245ecce7b2d1b514b674816_Heirloom%20White%20Paper.pdf

Mehr, J., Vadenbo, C., Steubing, B., & Hellweg, S. (2018). Environmentally optimal wood use in Switzerland—Investigating the relevance of material cascades. Resources, Conservation and Recycling, 131, 181–191. https://doi.org/10.1016/j.resconrec.2017.12.026

Menon, S., Akbari, H., Mahanama, S., Sednev, I., & Levinson, R. (2010). Radiative forcing and temperature response to changes in urban albedos and associated CO2 offsets. Environmental Research Letters, 5(1), 014005. https://doi.org/10.1088/1748-9326/5/1/014005

Mexia, T., Vieira, J., Príncipe, A., Anjos, A., Silva, P., Lopes, N., Freitas, C., Santos-Reis, M., Correia, O., Branquinho, C., & Pinho, P. (2018). Ecosystem services: Urban parks under a magnifying glass. Environmental Research, 160, 469–478. https://doi.org/10.1016/j.envres.2017.10.023

Meyer, S., Bright, R. M., Fischer, D., Schulz, H., & Glaser, B. (2012). Albedo Impact on the Suitability of Biochar Systems To Mitigate Global Warming. Environmental Science & Technology, 46(22), 12726–12734. https://doi.org/10.1021/es302302g

Middel, A., Turner, V. K., Schneider, F. A., Zhang, Y., & Stiller, M. (2020). Solar reflective pavements—A policy panacea to heat mitigation? Environmental Research Letters, 15(6), 064016. https://doi.org/10.1088/1748-9326/ab87d4

Millstein, D., & Menon, S. (2011). Regional climate consequences of large-scale cool roof and photovoltaic array deployment. Environmental Research Letters, 6(3), 034001. https://doi.org/10.1088/1748-9326/6/3/034001

Milnar, M., & Ramaswami, A. (2020). Impact of Urban Expansion and In Situ Greenery on Community-Wide Carbon Emissions: Method Development and Insights from 11 US Cities. Environmental Science & Technology, 54(24), 16086–16096. https://doi.org/10.1021/acs.est.0c02723

Minunno, R., O’Grady, T., Morrison, G. M., & Gruner, R. L. (2021). Investigating the embodied energy and carbon of buildings: A systematic literature review and meta-analysis of life cycle assessments. Renewable and Sustainable Energy Reviews, 143, 110935. https://doi.org/10.1016/j.rser.2021.110935

Minx, J. C., Lamb, W. F., Callaghan, M. W., Fuss, S., Hilaire, J., Creutzig, F., Amann, T., Beringer, T., Garcia, W. de O., Hartmann, J., Khanna, T., Lenzi, D., Luderer, G., Nemet, G. F., Rogelj, J., Smith, P., Vicente, J. L. V., Wilcox, J., & Dominguez, M. del M. Z. (2018). Negative emissions—Part 1: Research landscape and synthesis. Environmental Research Letters, 13(6), 063001. https://doi.org/10.1088/1748-9326/aabf9b

Mishra, A., Humpenöder, F., Churkina, G., Reyer, C. P. O., Beier, F., Bodirsky, B. L., Schellnhuber, H. J., Lotze-Campen, H., & Popp, A. (2022). Land use change and carbon emissions of a transformation to timber cities. Nature Communications, 13(1), Article 1. https://doi.org/10.1038/s41467-022-32244-w

Mohammadi, A. (2021). Overview of the Benefits and Challenges Associated with Pelletizing Biochar. Processes, 9(9), Article 9. https://doi.org/10.3390/pr9091591

Moncaster, A. M., Pomponi, F., Symons, K. E., & Guthrie, P. M. (2018). Why method matters: Temporal, spatial and physical variations in LCA and their impact on choice of structural system. Energy and Buildings, 173, 389–398. https://doi.org/10.1016/j.enbuild.2018.05.039

Morini, E., Castellani, B., Presciutti, A., Anderini, E., Filipponi, M., Nicolini, A., & Rossi, F. (2017). Experimental Analysis of the Effect of Geometry and Façade Materials on Urban District’s Equivalent Albedo. Sustainability, 9(7), Article 7. https://doi.org/10.3390/su9071245

Moussa, S., Kyereh, B., Tougiani, A., Kuyah, S., & Saadou, M. (2019). West African Sahelian cities as source of carbon stocks: Evidence from Niger. Sustainable Cities and Society, 50, 101653. https://doi.org/10.1016/j.scs.2019.101653

Muluneh, M. G., & Worku, B. B. (2022). Contributions of urban green spaces for climate change mitigation and biodiversity conservation in Dessie city, Northeastern Ethiopia. Urban Climate, 46, 101294. https://doi.org/10.1016/j.uclim.2022.101294

Nagar, P. K., Sharma, M., Gupta, S., & Singh, D. (2019). A framework for developing and projecting GHG emission inventory and preparing mitigation plan: A case study of Delhi City, India. Urban Climate, 28, 100462. https://doi.org/10.1016/j.uclim.2019.100462

Nazarian, N., Dumas, N., Kleissl, J., & Norford, L. (2019). Effectiveness of cool walls on cooling load and urban temperature in a tropical climate. Energy and Buildings, 187, 144–162. https://doi.org/10.1016/j.enbuild.2019.01.022

Nero, B. F. (2017). Urban green spaces enhance carbon sequestration and conserve biodiversity in cities of the Global South [Thesis, Universitäts- und Landesbibliothek Bonn]. https://bonndoc.ulb.uni-bonn.de/xmlui/handle/20.500.11811/7002

Novak, J., Sigua, G., Watts, D., Cantrell, K., Shumaker, P., Szogi, A., Johnson, M. G., & Spokas, K. (2016). Biochars impact on water infiltration and water quality through a compacted subsoil layer. Chemosphere, 142, 160–167. https://doi.org/10.1016/j.chemosphere.2015.06.038

Nowak, D. J., Greenfield, E. J., Hoehn, R. E., & Lapoint, E. (2013). Carbon storage and sequestration by trees in urban and community areas of the United States. Environmental Pollution, 178, 229–236. https://doi.org/10.1016/j.envpol.2013.03.019

Noya. (2022, December 14). Raising our sail to leverage Direct Air Capture tailwinds while they’re blowing. https://www.noya.co/news/raising-our-sail-to-leverage-dac-tailwinds

O’Donoghue, A., & Shackleton, C. M. (2013). Current and potential carbon stocks of trees in urban parking lots in towns of the Eastern Cape, South Africa. Urban Forestry & Urban Greening, 12(4), 443–449. https://doi.org/10.1016/j.ufug.2013.07.001

Ofori-Boadu, A. N., Kelley, R., Aryeetey, F., Fini, E., & Akangah, P. (2017). The influence of swine-waste bio-char on the early-age characteristics of cement pastes. International Journal of Engineering Research and Applications, 7(6), 1–7.

Ogungbenro, A. E., Quang, D. V., Al-Ali, K. A., Vega, L. F., & Abu-Zahra, M. R. M. (2018). Physical synthesis and characterization of activated carbon from date seeds for CO2 capture. Journal of Environmental Chemical Engineering, 6(4), 4245–4252. https://doi.org/10.1016/j.jece.2018.06.030

Oleson, K. W., Bonan, G. B., & Feddema, J. (2010). Effects of white roofs on urban temperature in a global climate model. Geophysical Research Letters, 37(3). https://doi.org/10.1029/2009GL042194

Oliver, C. D., Nassar, N. T., Lippke, B. R., & McCarter, J. B. (2014). Carbon, Fossil Fuel, and Biodiversity Mitigation With Wood and Forests. Journal of Sustainable Forestry, 33(3), 248–275. https://doi.org/10.1080/10549811.2013.839386

Ondoño, S., Martínez-Sánchez, J. J., & Moreno, J. L. (2016a). The composition and depth of green roof substrates affect the growth of Silene vulgaris and Lagurus ovatus species and the C and N sequestration under two irrigation conditions. Journal of Environmental Management, 166, 330–340. https://doi.org/10.1016/j.jenvman.2015.08.045

Ondoño, S., Martínez-Sánchez, J. J., & Moreno, J. L. (2016b). The inorganic component of green roof substrates impacts the growth of Mediterranean plant species as well as the C and N sequestration potential. Ecological Indicators, 61, 739–752. https://doi.org/10.1016/j.ecolind.2015.10.025

O’Sullivan, O. S., Holt, A. R., Warren, P. H., & Evans, K. L. (2017). Optimising UK urban road verge contributions to biodiversity and ecosystem services with cost-effective management. Journal of Environmental Management, 191, 162–171. https://doi.org/10.1016/j.jenvman.2016.12.062

Othman, R. (2019). The influence of urban park green spaces, plant material specifications and spatial design organization and pattern towards carbon sequestration rate. Applied Ecology and Environmental Research, 17(4). https://doi.org/10.15666/aeer/1704_80798088

Ottelé, M., Perini, K., Fraaij, A. L. A., Haas, E. M., & Raiteri, R. (2011). Comparative life cycle analysis for green façades and living wall systems. Energy and Buildings, 43(12), 3419–3429. https://doi.org/10.1016/j.enbuild.2011.09.010

Ouyang, Z., Sciusco, P., Jiao, T., Feron, S., Lei, C., Li, F., John, R., Fan, P., Li, X., Williams, C. A., Chen, G., Wang, C., & Chen, J. (2022). Albedo changes caused by future urbanization contribute to global warming. Nature Communications, 13(1), Article 1. https://doi.org/10.1038/s41467-022-31558-z

Ow, L. F., Ghosh, S., & Mohamed Lokman Mohd., Y. (2018). Effects of varying establishment approaches on the growth of urban street trees. Arboricultural Journal, 40(4), 201–209. https://doi.org/10.1080/03071375.2018.1528092

Pataki, D. E., Alberti, M., Cadenasso, M. L., Felson, A. J., McDonnell, M. J., Pincetl, S., Pouyat, R. V., Setälä, H., & Whitlow, T. H. (2021). The Benefits and Limits of Urban Tree Planting for Environmental and Human Health. Frontiers in Ecology and Evolution, 9. https://www.frontiersin.org/articles/10.3389/fevo.2021.603757

Peng, L. L. H., & Jim, C. Y. (2015). Economic evaluation of green-roof environmental benefits in the context of climate change: The case of Hong Kong. Urban Forestry & Urban Greening, 14(3), 554–561. https://doi.org/10.1016/j.ufug.2015.05.006

Pisello, A. L., Cotana, F., & Brinchi, L. (2014). On a Cool Coating for Roof Clay Tiles: Development of the Prototype and Thermal-energy Assessment. Energy Procedia, 45, 453–462. https://doi.org/10.1016/j.egypro.2014.01.049

Pittau, F., Lumia, G., Heeren, N., Iannaccone, G., & Habert, G. (2019). Retrofit as a carbon sink: The carbon storage potentials of the EU housing stock. Journal of Cleaner Production, 214, 365–376. https://doi.org/10.1016/j.jclepro.2018.12.304

Pomponi, F., Hart, J., Arehart, J. H., & D’Amico, B. (2020). Buildings as a Global Carbon Sink? A Reality Check on Feasibility Limits. One Earth, 3(2), 157–161. https://doi.org/10.1016/j.oneear.2020.07.018

Pradhan, R., Sarkar, B. Ch., Abha Manohar, K., Shukla, G., Tamang, M., Vineeta, Bhat, J. A., Kumar, M., & Chakravarty, S. (2022). Biomass carbon and soil nutrient status in urban green sites at foothills of eastern Himalayas: Implication for carbon management. Current Research in Environmental Sustainability, 4, 100168. https://doi.org/10.1016/j.crsust.2022.100168

Praneeth, S., Guo, R., Wang, T., Dubey, B. K., & Sarmah, A. K. (2020). Accelerated carbonation of biochar reinforced cement-fly ash composites: Enhancing and sequestering CO2 in building materials. Construction and Building Materials, 244, 118363. https://doi.org/10.1016/j.conbuildmat.2020.118363

Praneeth, S., Saavedra, L., Zeng, M., Dubey, B. K., & Sarmah, A. K. (2021). Biochar admixtured lightweight, porous and tougher cement mortars: Mechanical, durability and micro computed tomography analysis. Science of The Total Environment, 750, 142327. https://doi.org/10.1016/j.scitotenv.2020.142327

Pregitzer, C. C., Hanna, C., Charlop-Powers, S., & Bradford, M. A. (2022). Estimating carbon storage in urban forests of New York City. Urban Ecosystems, 25(2), 617–631. https://doi.org/10.1007/s11252-021-01173-9

Promraksa, A., & Rakmak, N. (2020). Biochar production from palm oil mill residues and application of the biochar to adsorb carbon dioxide. Heliyon, 6(5), e04019. https://doi.org/10.1016/j.heliyon.2020.e04019

Qin, Y., Pang, X., Tan, K., & Bao, T. (2021). Evaluation of pervious concrete performance with pulverized biochar as cement replacement. Cement and Concrete Composites, 119, 104022. https://doi.org/10.1016/j.cemconcomp.2021.104022

Qu, M., Pelkonen, P., Tahvanainen, L., Arevalo, J., & Gritten, D. (2012). Experts’ assessment of the development of wood framed houses in China. Journal of Cleaner Production, 31, 100–105. https://doi.org/10.1016/j.jclepro.2012.03.002

Raciti, S. M., Hutyra, L. R., & Newell, J. D. (2014). Mapping carbon storage in urban trees with multi-source remote sensing data: Relationships between biomass, land use, and demographics in Boston neighborhoods. Science of The Total Environment, 500–501, 72–83. https://doi.org/10.1016/j.scitotenv.2014.08.070

Rahim, M., Djedjig, R., Wu, D., Bennacer, R., & Ganaoui, M. E. (2023). Experimental investigation of hygrothermal behavior of wooden-frame house under real climate conditions. Energy and Built Environment, 4(1), 122–129. https://doi.org/10.1016/j.enbenv.2021.09.002

Ramage, M. H., Burridge, H., Busse-Wicher, M., Fereday, G., Reynolds, T., Shah, D. U., Wu, G., Yu, L., Fleming, P., Densley-Tingley, D., Allwood, J., Dupree, P., Linden, P. F., & Scherman, O. (2017). The wood from the trees: The use of timber in construction. Renewable and Sustainable Energy Reviews, 68, 333–359. https://doi.org/10.1016/j.rser.2016.09.107

Rasmussen, F. N., Malmqvist, T., Moncaster, A., Wiberg, A. H., & Birgisdóttir, H. (2018). Analysing methodological choices in calculations of embodied energy and GHG emissions from buildings. Energy and Buildings, 158, 1487–1498. https://doi.org/10.1016/j.enbuild.2017.11.013

Rawat, M., & Singh, R. N. (2022). A study on the comparative review of cool roof thermal performance in various regions. Energy and Built Environment, 3(3), 327–347. https://doi.org/10.1016/j.enbenv.2021.03.001

Raymond, C. M., Frantzeskaki, N., Kabisch, N., Berry, P., Breil, M., Nita, M. R., Geneletti, D., & Calfapietra, C. (2017). A framework for assessing and implementing the co-benefits of nature-based solutions in urban areas. Environmental Science & Policy, 77, 15–24. https://doi.org/10.1016/j.envsci.2017.07.008

Realmonte, G., Drouet, L., Gambhir, A., Glynn, J., Hawkes, A., Köberle, A. C., & Tavoni, M. (2019). An inter-model assessment of the role of direct air capture in deep mitigation pathways. Nature Communications, 10(1), 1–12.

Renforth, P., Edmondson, J., Leake, J. R., Gaston, K. J., & Manning, D. A. C. (2011). Designing a carbon capture function into urban soils. Proceedings of the Institution of Civil Engineers - Urban Design and Planning, 164(2), 121–128. https://doi.org/10.1680/udap.2011.164.2.121

Restuccia, L., & Ferro, G. A. (2016, May 29). Nanoparticles from food waste: A ‘green’ future for traditional building materials. Proceedings of the 9th International Conference on Fracture Mechanics of Concrete and Concrete Structures. 9th International Conference on Fracture Mechanics of Concrete and Concrete Structures. https://doi.org/10.21012/FC9.276

Restuccia, L., Ferro, G. A., Suarez-Riera, D., Sirico, A., Bernardi, P., Belletti, B., & Malcevschi, A. (2020). Mechanical characterization of different biochar-based cement composites. Procedia Structural Integrity, 25, 226–233. https://doi.org/10.1016/j.prostr.2020.04.027

Reynolds, C. C., Escobedo, F. J., Clerici, N., & Zea-Camaño, J. (2017). Does “Greening” of Neotropical Cities Considerably Mitigate Carbon Dioxide Emissions? The Case of Medellin, Colombia. Sustainability, 9(5), Article 5. https://doi.org/10.3390/su9050785

Richards, D. R., Belcher, R. N., Carrasco, L. R., Edwards, P. J., Fatichi, S., Hamel, P., Masoudi, M., McDonnell, M. J., Peleg, N., & Stanley, M. C. (2022). Global variation in contributions to human well-being from urban vegetation ecosystem services. One Earth, 5(5), 522–533. https://doi.org/10.1016/j.oneear.2022.04.006

Richter, S., Haase, D., Thestorf, K., & Makki, M. (2020). Carbon Pools of Berlin, Germany: Organic Carbon in Soils and Aboveground in Trees. Urban Forestry & Urban Greening, 54, 126777. https://doi.org/10.1016/j.ufug.2020.126777

Rose, L. S., & Levinson, R. (2013). Analysis of the effect of vegetation on albedo in residential areas: Case studies in suburban Sacramento and Los Angeles, CA. GIScience & Remote Sensing, 50(1), 64–77. https://doi.org/10.1080/15481603.2013.778557

Royal Society. (2009). Geoengineering the climate: Science, governance and uncertainty. Royal Society.

Sagar, U. S., Singh, Y., Mahalingam, A., & Malladi, T. (2022). Future impacts of Urban and Peri-urban agriculture on carbon stock and land surface temperatures in India. Urban Climate, 45, 101267. https://doi.org/10.1016/j.uclim.2022.101267

Salo, E. (2018). Current state and future perspectives of biochar applications in Finland [Jyväskylä University School of Business and Economics]. https://jyx.jyu.fi/handle/123456789/56920

Sander, H. A. (2022). Closing four remaining gaps in deploying urban vegetation to enable sustainable cities. One Earth, 5(5), 482–484. https://doi.org/10.1016/j.oneear.2022.04.011

Sandre, A. A., Fruehauf, A. L., Miyahara, A. A. L., Rosa, A. A., Maruyama, C. M., Locoselli, G. M., Candido, L. F., Lombardo, M. A., Coelho, M. A., Murolo, R. P., Pombo, R. M. R., Marques, T. H. N., & Pellegrino, P. R. M. (2021). Geodesign Brazil: Trees for the Metropolitan Area of São Paulo. In O. Gervasi, B. Murgante, S. Misra, C. Garau, I. Blečić, D. Taniar, B. O. Apduhan, A. M. A. C. Rocha, E. Tarantino, & C. M. Torre (Eds.), Computational Science and Its Applications – ICCSA 2021 (pp. 463–475). Springer International Publishing. https://doi.org/10.1007/978-3-030-86979-3_33

Santamouris, M., & Fiorito, F. (2021). On the impact of modified urban albedo on ambient temperature and heat related mortality. Solar Energy, 216, 493–507. https://doi.org/10.1016/j.solener.2021.01.031

Santero, N. J., & Horvath, A. (2009). Global warming potential of pavements. Environmental Research Letters, 4(3), 034011. https://doi.org/10.1088/1748-9326/4/3/034011

Schaffert, E., & Percival, G. (2016). The Influence of Biochar, Slow-Release Molasses, and an Organic N:P:K Fertilizer on Transplant Survival of Pyrus communis ‘Williams’ Bon Chrétien’. Arboriculture & Urban Forestry, 42(2). https://doi.org/10.48044/jauf.2016.009

Scharenbroch, B. C., Meza, E. N., Catania, M., & Fite, K. (2013). Biochar and Biosolids Increase Tree Growth and Improve Soil Quality for Urban Landscapes. Journal of Environmental Quality, 42(5), 1372–1385. https://doi.org/10.2134/jeq2013.04.0124

Schwaiger, H. P., & Bird, D. N. (2010). Integration of albedo effects caused by land use change into the climate balance: Should we still account in greenhouse gas units? Forest Ecology and Management, 260(3), 278–286. https://doi.org/10.1016/j.foreco.2009.12.002

Seguin, R., Kargar, M., Prasher, S. O., Grant Clark, O., & Jutras, P. (2018). Remediating Montreal’s Tree Pit Soil Applying an Ash Tree-Derived Biochar. Water, Air, & Soil Pollution, 229(3), 84. https://doi.org/10.1007/s11270-018-3725-1

Shackley, S., Carter, S., Sims, K., & Sohi, S. (2011). Expert Perceptions of the Role of Biochar as a Carbon Abatement Option with Ancillary Agronomic and Soil-Related Benefits. Energy & Environment, 22(3), 167–187. https://doi.org/10.1260/0958-305X.22.3.167

Shadman, S., Ahanaf Khalid, P., Hanafiah, M. M., Koyande, A. K., Islam, Md. A., Bhuiyan, S. A., Sin Woon, K., & Show, P.-L. (2022). The carbon sequestration potential of urban public parks of densely populated cities to improve environmental sustainability. Sustainable Energy Technologies and Assessments, 52, 102064. https://doi.org/10.1016/j.seta.2022.102064

Shafeeyan, M. S., Daud, W. M. A. W., Houshmand, A., & Shamiri, A. (2010). A review on surface modification of activated carbon for carbon dioxide adsorption. Journal of Analytical and Applied Pyrolysis, 89(2), 143–151. https://doi.org/10.1016/j.jaap.2010.07.006

Shafique, M., Xue, X., & Luo, X. (2020). An overview of carbon sequestration of green roofs in urban areas. Urban Forestry & Urban Greening, 47, 126515. https://doi.org/10.1016/j.ufug.2019.126515

Sigua, G. C., Novak, J. M., Watts, D. W., Johnson, M. G., & Spokas, K. (2016). Efficacies of designer biochars in improving biomass and nutrient uptake of winter wheat grown in a hard setting subsoil layer. Chemosphere, 142, 176–183. https://doi.org/10.1016/j.chemosphere.2015.06.015

Singh Yadav, S. P., Bhandari, S., Bhatta, D., Poudel, A., Bhattarai, S., Yadav, P., Ghimire, N., Paudel, P., Paudel, P., Shrestha, J., & Oli, B. (2023). Biochar application: A sustainable approach to improve soil health. Journal of Agriculture and Food Research, 100498. https://doi.org/10.1016/j.jafr.2023.100498

Singhal, S. (2023). Biochar as a cost-effective and eco-friendly substitute for binder in concrete: A review. European Journal of Environmental and Civil Engineering, 27(2), 984–1009. https://doi.org/10.1080/19648189.2022.2068658

Sinha, A., & Realff, M. J. (2019). A parametric study of the techno‐economics of direct CO 2 air capture systems using solid adsorbents. AIChE Journal, 65(7). https://doi.org/10.1002/aic.16607

Sinha, A., Thakkar, H., Rezaei, F., Kawajiri, Y., & Realff, M. J. (2018). Direct Air Capture of CO2 in Enclosed Environments: Design under Uncertainty and Techno-Economic Analysis. In M. R. Eden, M. G. Ierapetritou, & G. P. Towler (Eds.), Computer Aided Chemical Engineering (Vol. 44, pp. 2179–2184). Elsevier. https://doi.org/10.1016/B978-0-444-64241-7.50358-X

Sinha, A., Thakkar, H., Rezaei, F., Kawajiri, Y., & Realff, M. J. (2022). Reduced building energy consumption by combined indoor CO2 and H2O composition control. Applied Energy, 322, 119526. https://doi.org/10.1016/j.apenergy.2022.119526

Sirico, A., Bernardi, P., Belletti, B., Malcevschi, A., Dalcanale, E., Domenichelli, I., Fornoni, P., & Moretti, E. (2020). Mechanical characterization of cement-based materials containing biochar from gasification. Construction and Building Materials, 246, 118490. https://doi.org/10.1016/j.conbuildmat.2020.118490

Smart, N., Eisenman, T. S., & Karvonen, A. (2020). Street Tree Density and Distribution: An International Analysis of Five Capital Cities. Frontiers in Ecology and Evolution, 8. https://www.frontiersin.org/articles/10.3389/fevo.2020.562646

Smith, P., Davis, S. J., Creutzig, F., Fuss, S., Minx, J., Gabrielle, B., Kato, E., Jackson, R. B., Cowie, A., Kriegler, E., & others. (2016). Biophysical and economic limits to negative CO2 emissions. Nature Climate Change, 6(1), 42–50.

Smith, S. M., Geden, O., Nemet, G. F., Gidden, M. J., Lamb, W. F., Powis, C., Bellamy, R., Callaghan, M. W., Cowie, A., Cox, E., Fuss, S., Gasser, T., Grassi, G., Greene, J., Lück, S., Mohan, A., Müller-Hansen, F., Peters, G. P., Pratama, Y., … Minx, J. C. (2023). The State of Carbon Dioxide Removal—1st Edition (pp. 1–108). Available at: https://www.stateofcdr.org

Sproul, J., Wan, M. P., Mandel, B. H., & Rosenfeld, A. H. (2014). Economic comparison of white, green, and black flat roofs in the United States. Energy and Buildings, 71, 20–27. https://doi.org/10.1016/j.enbuild.2013.11.058

Stockholm Stad. (2017). Växtbäddar i Stockholms stad—En handbok 2017 (Planting beds in Stockholm city—Handbook 2017). https://leverantor.stockholm/entreprenad-i-stockholms-offentliga-miljoer/vaxtbaddshandboken/

Strohbach, M. W., Arnold, E., & Haase, D. (2012). The carbon footprint of urban green space—A life cycle approach. Landscape and Urban Planning, 104(2), 220–229. https://doi.org/10.1016/j.landurbplan.2011.10.013

Suarez-Riera, D., Restuccia, L., & Ferro, G. A. (2020). The use of Biochar to reduce the carbon footprint of cement-based materials. Procedia Structural Integrity, 26, 199–210. https://doi.org/10.1016/j.prostr.2020.06.023

Sullivan, W. C., Kuo, F. E., & Depooter, S. F. (2004). The Fruit of Urban Nature: Vital Neighborhood Spaces. Environment and Behavior, 36(5), 678–700. https://doi.org/10.1177/0193841X04264945

Sultana, R., Ahmed, Z., Hossain, Md. A., & Begum, B. A. (2021). Impact of green roof on human comfort level and carbon sequestration: A microclimatic and comparative assessment in Dhaka City, Bangladesh. Urban Climate, 38, 100878. https://doi.org/10.1016/j.uclim.2021.100878

Sun, Y., Xie, S., & Zhao, S. (2019). Valuing urban green spaces in mitigating climate change: A city-wide estimate of aboveground carbon stored in urban green spaces of China’s Capital. Global Change Biology, 25(5), 1717–1732. https://doi.org/10.1111/gcb.14566

Sun, Z., Du, W., Jiang, H., Weng, Q., Guo, H., Han, Y., Xing, Q., & Ma, Y. (2022). Global 10-m impervious surface area mapping: A big earth data based extraction and updating approach. International Journal of Applied Earth Observation and Geoinformation, 109, 102800. https://doi.org/10.1016/j.jag.2022.102800

Suppakittpaisarn, P., Jiang, X., & Sullivan, W. C. (2017). Green Infrastructure, Green Stormwater Infrastructure, and Human Health: A Review. Current Landscape Ecology Reports, 2(4), 96–110. https://doi.org/10.1007/s40823-017-0028-y

Susca, T. (2012a). Enhancement of life cycle assessment (LCA) methodology to include the effect of surface albedo on climate change: Comparing black and white roofs. Environmental Pollution, 163, 48–54. https://doi.org/10.1016/j.envpol.2011.12.019

Susca, T. (2012b). Multiscale Approach to Life Cycle Assessment. Journal of Industrial Ecology, 16(6), 951–962. https://doi.org/10.1111/j.1530-9290.2012.00560.x

Susca, T. (2018). Embodied Carbon of Surfaces: Inclusion of Surface Albedo Accounting in Life-Cycle Assessment. In F. Pomponi, C. De Wolf, & A. Moncaster (Eds.), Embodied Carbon in Buildings: Measurement, Management, and Mitigation (pp. 105–122). Springer International Publishing. https://doi.org/10.1007/978-3-319-72796-7_5

Susca, T., Gaffin, S. R., & Dell’Osso, G. R. (2011). Positive effects of vegetation: Urban heat island and green roofs. Environmental Pollution, 159(8), 2119–2126. https://doi.org/10.1016/j.envpol.2011.03.007

Švajlenka, J., Kozlovská, M., & Spišáková, M. (2017). The benefits of modern method of construction based on wood in the context of sustainability. International Journal of Environmental Science and Technology, 14(8), 1591–1602. https://doi.org/10.1007/s13762-017-1282-6

Taha, H. (2008). Meso-urban meteorological and photochemical modeling of heat island mitigation. Atmospheric Environment, 42(38), 8795–8809. https://doi.org/10.1016/j.atmosenv.2008.06.036

Taha, H. (2015). Cool Cities: Counteracting Potential Climate Change and its Health Impacts. Current Climate Change Reports, 1(3), 163–175. https://doi.org/10.1007/s40641-015-0019-1

Tahvonen, O., & Airaksinen, M. (2018). Low-density housing in sustainable urban planning – Scaling down to private gardens by using the green infrastructure concept. Land Use Policy, 75, 478–485. https://doi.org/10.1016/j.landusepol.2018.04.017

Talvitie, I., Vimpari, J., & Junnila, S. (2021). Economic feasibility of wood-based structures—Improving urban carbon neutrality strategies. Environmental Research: Infrastructure and Sustainability, 1(1), 011002. https://doi.org/10.1088/2634-4505/abfe05

Tan, K., Qin, Y., & Wang, J. (2022). Evaluation of the properties and carbon sequestration potential of biochar-modified pervious concrete. Construction and Building Materials, 314, 125648. https://doi.org/10.1016/j.conbuildmat.2021.125648

Tan, X., Liu, Y., Zeng, G., Wang, X., Hu, X., Gu, Y., & Yang, Z. (2015). Application of biochar for the removal of pollutants from aqueous solutions. Chemosphere, 125, 70–85. https://doi.org/10.1016/j.chemosphere.2014.12.058

Tang, Y., Chen, A., & Zhao, S. (2016). Carbon Storage and Sequestration of Urban Street Trees in Beijing, China. Frontiers in Ecology and Evolution, 4. https://www.frontiersin.org/articles/10.3389/fevo.2016.00053

Teo, H. C., Zeng, Y., Sarira, T. V., Fung, T. K., Zheng, Q., Song, X. P., Chong, K. Y., & Koh, L. P. (2021). Global urban reforestation can be an important natural climate solution. Environmental Research Letters, 16(3), 034059. https://doi.org/10.1088/1748-9326/abe783

Timilsina, N., Staudhammer, C. L., Escobedo, F. J., & Lawrence, A. (2014). Tree biomass, wood waste yield, and carbon storage changes in an urban forest. Landscape and Urban Planning, 127, 18–27. https://doi.org/10.1016/j.landurbplan.2014.04.003

Trlica, A., Hutyra, L. R., Morreale, L. L., Smith, I. A., & Reinmann, A. B. (2020). Current and future biomass carbon uptake in Boston’s urban forest. Science of The Total Environment, 709, 136196. https://doi.org/10.1016/j.scitotenv.2019.136196

Uday, V., Harikrishnan, P. S., Deoli, K., Zitouni, F., Mahlknecht, J., & Kumar, M. (2022). Current trends in production, morphology, and real-world environmental applications of biochar for the promotion of sustainability. Bioresource Technology, 359, 127467. https://doi.org/10.1016/j.biortech.2022.127467

Veerkamp, C., Spoon, M., van Rijn, F., Loreti, M., Dassen, T., Schipper, A., Boke-Olén, N., Cortinovis, C., Hedlund, K., & Olsson, P. (2020). Assessing the potential of nature-based solutions in European cities for addressing climate change challenges. NATURVATION. https://www.naturvation.eu/sites/default/files/result/files/naturvation_deliverable_3.7_full_potential_of_nbs_in_european_cities_report.pdf

Vieux-Champagne, F., Sieffert, Y., Grange, S., Polastri, A., Ceccotti, A., & Daudeville, L. (2014). Experimental analysis of seismic resistance of timber-framed structures with stones and earth infill. Engineering Structures, 69, 102–115. https://doi.org/10.1016/j.engstruct.2014.02.020

Wang, C., Wang, Z.-H., Kaloush, K. E., & Shacat, J. (2021). Cool pavements for urban heat island mitigation: A synthetic review. Renewable and Sustainable Energy Reviews, 146, 111171. https://doi.org/10.1016/j.rser.2021.111171

Wang, J., Manning, D. A. C., Stirling, R., Lopez-Capel, E., & Werner, D. (2023). Biochar benefits carbon off-setting in blue-green infrastructure soils—A lysimeter study. Journal of Environmental Management, 325, 116639. https://doi.org/10.1016/j.jenvman.2022.116639

Wang, L., Chen, L., Tsang, D. C. W., Kua, H. W., Yang, J., Ok, Y. S., Ding, S., Hou, D., & Poon, C. S. (2019). The roles of biochar as green admixture for sediment-based construction products. Cement and Concrete Composites, 104, 103348. https://doi.org/10.1016/j.cemconcomp.2019.103348

Wang, Y., Chang, Q., & Li, X. (2021). Promoting sustainable carbon sequestration of plants in urban greenspace by planting design: A case study in parks of Beijing. Urban Forestry & Urban Greening, 64, 127291. https://doi.org/10.1016/j.ufug.2021.127291

Wang, Y.-C., & Lin, J.-C. (2012). Air quality enhancement zones in Taiwan: A carbon reduction benefit assessment. Forest Policy and Economics, 23, 40–45. https://doi.org/10.1016/j.forpol.2012.07.009

Whittinghill, L. J., Rowe, D. B., Schutzki, R., & Cregg, B. M. (2014). Quantifying carbon sequestration of various green roof and ornamental landscape systems. Landscape and Urban Planning, 123, 41–48. https://doi.org/10.1016/j.landurbplan.2013.11.015

Winters, D., Boakye, K., & Simske, S. (2022). Toward Carbon-Neutral Concrete through Biochar–Cement–Calcium Carbonate Composites: A Critical Review. Sustainability, 14(8), Article 8. https://doi.org/10.3390/su14084633

Woodard, A. C., & Milner, H. R. (2016). Sustainability of timber and wood in construction. In Sustainability of Construction Materials (pp. 129–157). Elsevier. https://doi.org/10.1016/B978-0-08-100370-1.00007-X

Xu, X., AzariJafari, H., Gregory, J., Norford, L., & Kirchain, R. (2020). An integrated model for quantifying the impacts of pavement albedo and urban morphology on building energy demand. Energy and Buildings, 211, 109759. https://doi.org/10.1016/j.enbuild.2020.109759

Yang, B., Xu, K., & Zhang, Z. (2020). Mitigating evaporation and desiccation cracks in soil with the sustainable material biochar. Soil Science Society of America Journal, 84(2), 461–471. https://doi.org/10.1002/saj2.20047

Yang, J., Wang, Z.-H., & Kaloush, K. E. (2015). Environmental impacts of reflective materials: Is high albedo a ‘silver bullet’ for mitigating urban heat island? Renewable and Sustainable Energy Reviews, 47, 830–843. https://doi.org/10.1016/j.rser.2015.03.092

Yang, Q., Mašek, O., Zhao, L., Nan, H., Yu, S., Yin, J., Li, Z., & Cao, X. (2021). Country-level potential of carbon sequestration and environmental benefits by utilizing crop residues for biochar implementation. Applied Energy, 282, 116275. https://doi.org/10.1016/j.apenergy.2020.116275

Yang, X., & Wang, X.-Y. (2021). Strength and durability improvements of biochar-blended mortar or paste using accelerated carbonation curing. Journal of CO2 Utilization, 54, 101766. https://doi.org/10.1016/j.jcou.2021.101766

Yoo, S. Y., Kim, Y. J., & Yoo, G. (2020). Understanding the role of biochar in mitigating soil water stress in simulated urban roadside soil. Science of The Total Environment, 738, 139798. https://doi.org/10.1016/j.scitotenv.2020.139798

Yoon, S. G., Yang, Y. K., Kim, T. W., Chung, M. H., & Park, J. C. (2018). Thermal Performance Test of a Phase-Change-Material Cool Roof System by a Scaled Model. Advances in Civil Engineering, 2018, e2646103. https://doi.org/10.1155/2018/2646103

Yu, B., & Lu, Q. (2014). Estimation of albedo effect in pavement life cycle assessment. Journal of Cleaner Production, 64, 306–309. https://doi.org/10.1016/j.jclepro.2013.07.034

Yue, Y., Cui, L., Lin, Q., Li, G., & Zhao, X. (2017). Efficiency of sewage sludge biochar in improving urban soil properties and promoting grass growth. Chemosphere, 173, 551–556. https://doi.org/10.1016/j.chemosphere.2017.01.096

Zaragoza, A., & Bartolom, C. (2012). Albedo Effect and Energy Efficiency of Cities. In C. Ghenai (Ed.), Sustainable Development—Energy, Engineering and Technologies—Manufacturing and Environment. InTech. https://doi.org/10.5772/29536

Zeitz, A., Griffin, C. T., & Dusicka, P. (2019). Comparing the embodied carbon and energy of a mass timber structure system to typical steel and concrete alternatives for parking garages. Energy and Buildings, 199, 126–133. https://doi.org/10.1016/j.enbuild.2019.06.047

Zhang, J., Zhang, K., Liu, J., & Ban-Weiss, G. (2016). Revisiting the climate impacts of cool roofs around the globe using an Earth system model. Environmental Research Letters, 11(8), 084014. https://doi.org/10.1088/1748-9326/11/8/084014

Zhao, M., Kong, Z., Escobedo, F. J., & Gao, J. (2010). Impacts of urban forests on offsetting carbon emissions from industrial energy use in Hangzhou, China. Journal of Environmental Management, 91(4), 807–813. https://doi.org/10.1016/j.jenvman.2009.10.010

Zhao, R., Liu, L., Zhao, L., Deng, S., Li, S., Zhang, Y., & Li, H. (2019). Thermodynamic exploration of temperature vacuum swing adsorption for direct air capture of carbon dioxide in buildings. Energy Conversion and Management, 183, 418–426. https://doi.org/10.1016/j.enconman.2019.01.009

Zhao, S., Huang, B., Shu, X., & Ye, P. (2014). Laboratory Investigation of Biochar-Modified Asphalt Mixture. Transportation Research Record, 2445(1), 56–63. https://doi.org/10.3141/2445-07

Zhuang, Q., Shao, Z., Gong, J., Li, D., Huang, X., Zhang, Y., Xu, X., Dang, C., Chen, J., Altan, O., & Wu, S. (2022). Modeling carbon storage in urban vegetation: Progress, challenges, and opportunities. International Journal of Applied Earth Observation and Geoinformation, 114, 103058. https://doi.org/10.1016/j.jag.2022.103058

The more recent IEA 2022 report projects future cement production in 2030 from 2021 historic estimates, indicating that 4333 Mt of cement would be produced in 2030. Extrapolating these figures to 2050 would lead to 4473 Mt of cement, a value below the lower bound of 4796 Mt.

Ratio obtained with proportions of cement, sand and coarse aggregate is 1:2:4 by volume, and densities of 2400 and 1440 kg/m3 for concrete and cement, respectively.