Appendix A.  Detailed Laboratory Testing Methodology

IgM and IgG ELISA Testing

IgM and IgG antibodies against ZIKV, DENV and CHIKV were detected in patients’ serum samples using a commercial indirect, colorimetric ELISA assay (Euroimmun AG, Lübeck, Germany). Anti-Zika ELISA IgM and IgG platforms contain micro titer wells coated with recombinant non-structural protein (NS1) of Zika virus; anti-Dengue ELISA IgM and IgG contain micro titer wells coated with dengue virus antigens; and anti-Chikungunya ELISA IgM and IgG contain micro titer wells coated with recombinant Chikungunya virus antigens. The assays were performed according to the manufacturers’ kit inserts. Briefly, 1:100 diluted serum was added to the coated wells together with positive, negative and calibrator samples in duplicates, and incubated for one hour at 37°C. Conjugated anti-human IgM and IgG antibodies were added for IgM and IgG detection, respectively, followed by a substrate. The OD of each well was measured by spectrophotometry at 450 nm, with a 620 nm reference wavelength. Ratios were determined by dividing the OD of the patient’s sample by the OD of the assay calibrator. IgM results were reported semi-quantitatively by calculating a ratio of the extinction value of the control or patient sample over the extinction value of the calibrator. The sample was considered IgM positive if the IgM ratio was ≥1.1, borderline if the ratio was ≥0.8 to <1.1 and negative if <0.8.  IgG results were reported quantitatively using the standard curve obtained by point-to-point plotting of the extinction values measured for the 3 calibration sera against the corresponding units (linear/linear). The sample was considered IgG positive if the result was ≥22 relative units (RU)/mL, borderline if ≥16 to <22 RU/mL, or negative if <16 RU/mL.

RNA extraction and real-time PCR

All IgM and IgG-positive samples were tested for the presence of viral RNA. Details of RNA extraction and real-time PCR methods can be found in Appendix A. Viral RNA was extracted from 140 μL of plasma sample by using QIAmpviral RNA Mini kit (Qiagen Inc., Valencia, CA), according to the manufacturer’s instructions, and then stored at -80°C until further processing. The real time PCR was performed using Altona RealStar Zika, dengue and Chikungunya PCR kits (Altona Diagnostics, Hamburg, Germany). Reaction mixtures A and B and an internal control (IC) were mixed with 10 uL of extracted RNA. Reverse transcription PCR (RT-PCR) was performed using the ViiA7 system (Applied Biosystems, Foster City, CA, USA). Cycling conditions were as follows: 50°C for 10 min (RT step), 95°C for 2 min, 45 cycles of 95°C for 15 s, 55°C for 45 s, and 72°C for 15 s (run time, 120 min). The Altona IC was required to be detected within 45 cycles; samples in which the Altona IC was not amplified were considered not valid. Samples with a cycle threshold >38-40 were considered positive.

Focus Reduction Neutralization Test

A focus reduction neutralization test (FRNT) was used to quantify CHIKV-specific neutralizing antibodies. Briefly, three-fold serial dilutions of heat-inactivated serum samples were incubated with 150 focus forming units (FFU) of CHIKV strain 181/25 for 1 hour at 37°C in technical duplicate.  Serum-virus mixtures were incubated at 37°C with pre-plated Vero cells in 96-well tissue culture plates for 4 hours, then overlaid with 1% (w/v) methylcellulose (Sigma Aldrich) in Opti-MEM (Invitrogen) containing 2% FBS (Atlanta Biologicals) and 1x Penicillin-Streptomycin (Invitrogen), and incubated at 37°C for 18-24 hours.  Following infection, cells were fixed with 1.3% PFA in 1X PBS for 30 minutes and permeabilized using 1x Perm/Wash buffer (BD Biosciences).  CHIKV-infected foci were detected by sequential incubation with 700 ng/ml of a CHIKV-specific mouse monoclonal antibody (mAb CHK-11), 200 ng/ml of horseradish peroxidase (HRP)-conjugated goat anti-mouse IgG (Sigma Aldrich), and TrueBlue peroxidase substrate (KPL, Inc.).  Foci were quantitated using an ImmunoSpot Macroanalyzer (Cellular Technologies, Ltd.).  Nonlinear regression analysis with a variable slope was used to generate serum-dose response curves and calculate the dilution required to inhibit infectivity by 50% (NT50) (GraphPad Prism, version 6.0g; GraphPad Software).