An often-overestimated ecological risk of copper in Chinese surface water: Bioavailable fraction determined by multiple linear regression of water quality parameters

Wei Liao1,2, Yutao Zheng1, Chenglian Feng2, Wenjun Zhong1, Ziwei Zhu1*, Huiyu Xie2,3, Wenpan Li3, Xiaowei Jin3*, John P. Giesy4,5,6

1 Wetland Research Center, Jiangxi Academy of Forestry, Nanchang, 330032, China

2 State Key Laboratory of Environmental Criteria and Risk Assessment, Chinese Research Academy of Environmental Sciences, Beijing, 100012, China

3 China National Environmental Monitoring Centre, Beijing, 100012, China

4 Department of Veterinary Biomedical Sciences and Toxicology Centre, University of Saskatchewan, Saskatoon, Saskatchewan, S7N 5B3, Canada;

5 Department of Environmental Sciences, Baylor University, Waco, 76798-7266, TX, USA;

6 Department of Integrative Biology and Center for Integrative Toxicology, Michigan State University, East Lansing, MI 48824, USA

For development of hardness-based equations, toxicity data for same species spanned a minimum range of 100 mg L−1 for hardness, and the highest hardness was at least three times that of the lowest. The acute toxicity value and hardness were natural log-transformed, the log-transformed toxicity and hardness tended to be linear, and the slopes of the ln(toxicity) vs. ln(hardness) was different depending on the species. For a pooled hardness-based equation, in the normalization procedure, the pooled slope was fitted through all toxicity values in the species-specific models (Equation S1).

ln(Toxicityi)=ki×ln(HD)+Intercepti+errori    Equation (S1)

where HD represents the hardness, expressed in mg/L; toxicity of Cu is expressed in μg/L; and coefficient ki is defined the coefficient of a selected species or all selected species in derive species-specific equations. For the pooled model, coefficient ki defines all of the selected species in deriving a species-specific model.

The acute toxicity value was standardized to a target water condition use the species-specific models or the pooled model, a standardized equation (Equation S2).

LC50 =exp⁡[ln(LC50 meas)-HDslope (lnHDmeasHDtarget)]   			Equation (S2)

where LC50meas is the observed LC50; HDslope forms the species-specific model or the pooled model; HDmeas is the tested water hardness; and HDtarget is the targeted hardness.

Table S1 Concentrations of total dissolved Cu in Chinese surface water of fifteen regions in 2021 (μg/L)

|  | ALL | YZR | YR | PR | SHR | HHR | HaiHR | LHR | ZMR | XBR | XNR | THL | DCL | CHL | PYL | DTL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 3610 | 0.04 | 0.35 | 393 | 272 | 381 | 272 | 207 | 211 | 128 | 144 | 159 | 22 | 30 | 18 | 11 |
| Min | 0.04 | 0.35 | 0.5 | 0.04 | 0.50 | 0.27 | 0.07 | 0.1 | 0.07 | 0.04 | 0.04 | 0.60 | 0.27 | 0.75 | 1.00 | 0.75 |
| 5th | 0.45 | 0.52 | 0.75 | 0.35 | 0.50 | 0.60 | 0.50 | 0.40 | 0.40 | 0.34 | 0.27 | 1.00 | 0.27 | 0.75 | 1.00 | 0.75 |
| 10th | 0.50 | 1.00 | 1.30 | 0.52 | 0.50 | 0.75 | 0.52 | 0.50 | 0.45 | 0.50 | 0.27 | 1.00 | 0.27 | 0.98 | 1.00 | 0.75 |
| 25th | 0.75 | 1.50 | 2.00 | 0.70 | 0.50 | 1.00 | 1.00 | 0.70 | 1.00 | 0.50 | 0.50 | 1.50 | 0.53 | 1.00 | 1.50 | 0.75 |
| 50th | 1.50 | 2.00 | 3.00 | 1.00 | 0.80 | 1.80 | 1.80 | 1.00 | 1.30 | 0.50 | 1.00 | 2.00 | 1.90 | 1.80 | 1.50 | 1.30 |
| 75th | 2.00 | 3.00 | 4.50 | 2.00 | 1.50 | 2.50 | 2.50 | 1.50 | 2.00 | 1.50 | 2.00 | 2.50 | 2.90 | 3.00 | 2.00 | 2.10 |
| 90th | 3.00 | 4.00 | 6.00 | 3.00 | 2.50 | 3.00 | 3.00 | 2.40 | 2.50 | 3.00 | 3.90 | 3.00 | 3.90 | 3.00 | 2.50 | 2.50 |
| 95th | 4.00 | 70.5 | 16.00 | 4.50 | 3.00 | 5.00 | 3.60 | 3.00 | 3.00 | 4.00 | 7.80 | 3.00 | 4.00 | 3.00 | 2.60 | 2.80 |
| Max | 179 | 1.89 | 2.48 | 112 | 4.5 | 45.5 | 12.0 | 5.3 | 7.00 | 179 | 137 | 5.00 | 4.50 | 26.8 | 3.00 | 3.00 |
| AM | 1.96 | 1.34 | 1.94 | 2.10 | 1.17 | 2.25 | 1.95 | 1.22 | 1.46 | 2.58 | 3.16 | 1.99 | 1.87 | 2.67 | 1.70 | 1.52 |
| CV | 2.48 | 0.04 | 0.35 | 2.96 | 0.78 | 1.35 | 0.69 | 0.67 | 0.61 | 6.11 | 3.9 | 0.35 | 0.72 | 1.74 | 0.34 | 0.55 |

Note: YZR=Yangtze River; YR=Yellow River; PR= Pearl River; SHR= Songhua River; HHR= Huaihe river; HaiHR= Haihe River; LHR= Liaohe River; ZMR= Zhemin area; XBR= Xibei area; XNR= Xinan area; THL= Tai lake; DCL= Dianchi lake; CHL= Chao lake; PYL= Poyang lake; DTL= Dongting lake; n= sites numbers; Min= minimal value; 5th = the 5th percentile in Cu exposed concentration data sets; 10th = the 10th percentile in Cu exposed concentration data sets; 25th = the 25th percentile in Cu exposed concentration data sets; 50th = the 50th percentile in Cu exposed concentration data sets; 75th = the 75th percentile in Cu exposed concentration data sets; 90th = the 90th percentile in Cu exposed concentration data sets; 95th = the 95th percentile in Cu exposed concentration data sets; Max= maximum value; AM= arithmetic mean; CV= coefficient of variation.

Table S2 Water quality variables in Chinese surface freshwater of fifteen regions

|  | Hardness/mg/L | pH | DOC/mg/L |
| --- | --- | --- | --- |
| n | 480 | 3578 | 897 |
| Min | 12.08 | 6.24 | 0.5 |
| 5th | 37.29 | 6.90 | 0.9 |
| 10th | 44.38 | 7.09 | 1.2 |
| 20th | 80.96 | 7.29 | 1.5 |
| 25th | 99.09 | 7.39 | 1.7 |
| 50th | 152.7 | 7.76 | 3.0 |
| 75th | 182.2 | 8.16 | 4.9 |
| 90th | 214.0 | 8.50 | 8.5 |
| 95th | 237.9 | 8.67 | 11.7 |
| Max | 1007 | 9.98 | 45.2 |
| AM | 146.8 | 7.77 | 4.3 |
| GM | 125.8 | 7.78 | 3.0 |
| CV | 0.58 | 0.07 | 1.1 |

Note: n= sites numbers; Min= minimal value; 5th = the 5th percentile in water quality variables data sets; 10th = the 10th percentile in water quality variables data sets; 25th = the 25th percentile in water quality variables data sets; 50th = the 50th percentile in water quality variables data sets; 75th = the 75th percentile in water quality variables data sets; 90th = the 90th percentile in water quality variables data sets; 95th = the 95th percentile in water quality variables data sets; Max= maximum value; AM= arithmetic mean; GM= geometric mean; CV= coefficient of variation.

Table S3 Suggestions of hardness, pH and DOC values for China, compared to previous works.

| Normalization chemistry | Tbalble | pH | DOC (mg/L) | Ca2+ (mg/L) | Mg2+ (mg/L) | Na+ (mg/L) | K+ (mg/L) | SO42- (mg/L) | Cl- (mg/L) | Alkalinity (mg/L) | S2- (μg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5th 1 in data sets | 37.3 | 6.9 | 0.9 |  |  |  |  |  |  |  |  |
| 10th in data sets | 44.4 | 7.1 | 1.2 |  |  |  |  |  |  |  |  |
| 20th in data sets | 81.0 | 7.3 | 1.5 |  |  |  |  |  |  |  |  |
| 50th in data sets | 152.7 | 7.8 | 3.0 |  |  |  |  |  |  |  |  |
| Hardness-based model in China | 150 |  |  |  |  |  |  |  |  |  |  |
| MLR model in China | 150 | 7.8 | 3.0 |  |  |  |  |  |  |  |  |
| Tai Lake2 in China | 169 | 8.1 | 4.9 |  |  |  |  |  |  |  |  |
| Median values in this study3 | 162.3 | 7.9 | 3.7 | 54 | 8 | 12.8 | 2.6 | 36.7 | 16 | 98 | Null5 |
| Geomean in 136 samples | 133.8 | 7.8 | 3.48 | 45 | 7.6 | 17 | 0.9 | 40.7 | 19 | 93 | Null |
| BLM model in China | 150 | 7.8 | 3.0 | 45 | 9 | 13 | 2.6 | 55 | 20 | 95 | 0.3 |
| USEPA4 | 85 | 7.5 | 0.5 | 14 | 12.1 | 26.3 | 2.1 | 81.4 | 1.9 | 65 | 0.3 |

1, 5th = the 5th centile in water chemistry data sets.

2, It was reported by Zhang et al.1

3, 136 samples was surveyed from 2018-2020.

4, It was reported by USEPA.

5, Null=no detection.

MLR= Multiple linear regression; BLM= Biotic ligand model

- Zhang, Y.H., Zang, W.C., Qin, L.M., Zheng, L., Cao, Y., Yan, Z.G., et al. 2017. Water quality criteria for copper based on the BLM approach in the freshwater in China. Plos One, 12: e0170105. http://doi.org/10.1371/journal.pone.0170105

Table S4 Species-specific multiple linear regression model coefficients

|  | Species | n | intercept | ln(hardness) | pH | ln(DOC) | R |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ceriodaphnia dubia | 78 | -5.826 |  | 1.113 | 0.646 | 0.646 |
| 2 | Daphnia magna | 330 | -6.758 | 0.445 | 1.028 | 0.922 | 0.856 |
| 3 | Daphnia obtusa | 53 | -2.700 | 0.238 | 0.651 | 0.843 | 0.826 |
| 4 | Daphnia pulex | 31 | -10.758 |  | 1.716 | 0.956 | 0.788 |
| 5 | Daphnia pulicaria | 24 | 3.002 | 0.541 | -0.446 | 0.874 | 0.954 |
| 6 | Oncorhynchus clarki | 11 | 4.228 |  |  | 0.873 | 0.437 |
| 7 | Oncorhynchus mykiss | 37 | -0.919 | 1.090 |  | 0.314 | 0.788 |
| 8 | Oryzias latipes | 16 | 0.005 |  | 0.728 | 0.854 | 0.750 |
| 9 | Pimephales promelas | 219 | -6.326 | 0.953 | 0.939 | 0.669 | 0.749 |
|  | pooled | 831 | -2.092 | 0.555 | 0.487 | 0.290 | 0.262 |

The standard of normalized pooled MLR model was expressed Standardized LC50  =exp⁡[ln(LC50)-0.555 (lnHDmeas150)-0.487 (pHmeas- 7.8) - 0.29 (lnDOCmeas3.0)]

Table S5 Species-specific hardness-based model coefficients

|  | Species | n | slopes ln(hardness) | 95% Contidence limits | Degrees of freedom | R2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Ceriodaphnia dubia | 78 | 0.8198 | 0.7910, 0.8486 | 76 | 0.912 |
| 2 | Ctenopharyngodon idella | 4 | 1.1179 | 0.4743, 1.7615 | 2 | 0.990 |
| 3 | Cyclops Cyclopoidea | 25 | 1.3412 | 1.3130, 1.3694 | 23 | 0.990 |
| 4 | Daphnia magna | 330 | 0.8678 | 0.8519, 0.8837 | 328 | 0.901 |
| 5 | Daphnia obtusa | 53 | 0.8312 | 0.7850, 0.8774 | 51 | 0.862 |
| 6 | Daphnia pulex | 31 | 0.7195 | 0.6663, 0.7727 | 29 | 0.859 |
| 7 | Daphnia pulicaria | 24 | 0.847 | 0.7923, 0.9017 | 22 | 0.913 |
| 8 | Hyalella azteca | 7 | 0.6824 | 0.6084, 0.7564 | 5 | 0.934 |
| 9 | Hypophthalmichthys molitrix | 5 | 1.2254 | 1.0826, 1.3682 | 3 | 0.949 |
| 10 | Lampsilis siliquoidea | 28 | 0.8435 | 0.8102, 0.8768 | 26 | 0.960 |
| 11 | Oncorhynchus clarki | 11 | 0.998 | 0.9476, 1.0483 | 9 | 0.975 |
| 12 | Oncorhynchus mykiss | 37 | 0.8883 | 0.8611, 0.9155 | 35 | 0.967 |
| 13 | Oncorhynchus tshawytscha | 12 | 0.8384 | 0.7979, 0.8789 | 10 | 0.975 |
| 14 | Oryzias latipes | 16 | 1.5696 | 1.4367, 1.7025 | 14 | 0.903 |
| 15 | Pimephales promelas | 219 | 1.1998 | 1.1814, 1.2182 | 217 | 0.951 |
|  | All of above | 880 | 0.9616 | 0.9506, 0.9726 | 878 | 0.897 |
|  | Mean slopes | 880 | 0.9860 |  | 878 |  |

Table S6 Raw and normalization of toxicity data for Cu

| Species | Raw toxicity data (µg/L) | Normalization toxicity data by Hardness based model (µg/L) | Normalization toxicity data by MLR (µg/L) | Normalization toxicity data by BLM (µg/L) |
| --- | --- | --- | --- | --- |
| Lithoglyphus virens | 7.7 | 50.9 | 41.0 | 53.1 |
| Daphnia pulex | 13.3 | 40.0 | 39.7 | 48.3 |
| Juga plicifera | 14.4 | 95.4 | 76.8 | 89.7 |
| Scapholeberis sp. | 17.3 | 47.9 | 48.2 | 73.7 |
| Ptychocheilus oregonensis | 19.5 | 75.6 | 72.8 | 102.5 |
| Gammarus pseudolimnaeus | 19.6 | 62.6 | 53.8 | 72.9 |
| Hyalella azteca | 20.0 | 35.7 | 61.3 | 87.5 |
| Daphnia obtusa | 22.9 | 76.2 | 167.9 | 79.5 |
| Oncorhynchus tshawytscha | 23.0 | 69.2 | 90.5 | 150.5 |
| Actinonaias pectorosa | 26.9 | 48.7 | 48.7 | 83.8 |
| Physa integra | 37.4 | 119.3 | 102.6 | 132.3 |
| Ceriodaphnia dubia | 38.1 | 64.6 | 36.7 | 37.4 |
| Villosa iris | 38.7 | 57.4 | 39.0 | 24.0 |
| Daphnia magna | 39.0 | 78.0 | 90.4 | 69.1 |
| Daphnia pulicaria | 39.6 | 75.3 | 27.4 | 22.2 |
| Oncorhynchus mykiss | 45.9 | 81.2 | 132.6 | 135.6 |
| Oncorhynchus kisutch | 59.0 | 290.4 | 200.7 | 140.0 |
| Lampsilis siliquoidea | 64.7 | 70.3 | 60.7 | 50.1 |
| Oncorhynchus clarki henshawi | 66.5 | 59.3 | 87.2 | 169.0 |
| Oncorhynchus apache | 67.2 | 59.9 | 95.9 | 182.2 |
| Utterbackia imbecillis | 71.8 | 130.8 | 161.6 | 250.3 |
| Oncorhynchus clarki | 91.3 | 194.0 | 157.4 | 183.2 |
| Salvelinus confluentus | 102.7 | 111.1 | 239.6 | 291.0 |
| Ctenopharyngodon idella | 114.2 | 291.6 | 371.6 | / |
| Bufo boreas | 115.2 | 103.9 | 173.8 | 231.4 |
| Oncorhynchus gorbuscha | 129.9 | 229.2 | 204.6 | 206.3 |
| Pimephales promelas | 130.0 | 402.4 | 817.2 | 273.5 |
| Acrocheilus alutaceus | 137.3 | 366.7 | 413.0 | 664.3 |
| Hypophthalmichthys molitrix | 146.0 | 635.1 | 535.3 | 666.8 |
| Poeciliopsis occidentalis occidentalis | 153.6 | 138.5 | 220.7 | 257.0 |
| Scaphirhynchus platorynchus | 153.6 | 138.5 | 268.2 | 294.8 |
| Oncorhynchus nerka | 156.5 | 276.2 | 246.5 | 253.8 |
| Gila elegans | 192.0 | 167.4 | 264.1 | 277.1 |
| Cyclops Cyclopoidea | 247.1 | 867.3 | 2607 | 2542 |
| Lumbriculus variegatus | 249.5 | 132.4 | 358.7 | 234.2 |
| Xyrauchen texanus | 262.6 | 228.9 | 352.4 | 320.0 |
| Scenedesmus obliquus | 272.4 | 425.1 | 365.5 | 349.8 |
| Etheostoma lepidum | 314.0 | 280.0 | 470.7 | 414.4 |
| Etheostoma flabellare | 343.8 | 304.8 | 532.8 | 437.8 |
| Ptychocheilus lucius | 410.0 | 357.4 | 644.7 | 462.3 |
| Oryzias latipes | 500.0 | 4590 | 750.6 | 1019 |
| Chironomus decorus | 709.4 | 2307 | 2863 | 4767 |
| Lepomis macrochirus | 1305 | 3493 | 3807 | 5273 |
| Campeloma decisum | 1606 | 5124 | 4407 | 8175 |
| Acroneuria lycorias | 7968 | 27100 | 31896 | 42157 |
| Notemigonus crysoleucas | 81216 | 164060 | 172438 | 251264 |

Table S7 The MOS10 of Cu in surface water of China under different water quality parameters condition.

|  | Exposure data C0.9/μg/L | Toxicity data C0.1/μg/L | MOS10 | Num. 1 | % of sites2 |
| --- | --- | --- | --- | --- | --- |
|  | 3.21 (3.20-3.22) |  |  |  |  |
| Standard |  | 12.85 (11.88-13.65) | 3.76 | 25 | 0.71% |
| Raw |  | 5.04 (4.42-5.57) | 1.48 | 99 | 2.81% |
| 5th |  | 2.70 (2.49-2.89) | 0.79 | 582 | 16.52% |
| 10th |  | 3.55 (3.28-3.76) | 1.04 | 219 | 6.22% |
| 20th |  | 5.82 (5.38-6.18) | 1.71 | 87 | 2.47% |
| 50th |  | 12.98 (11.91-13.84) | 3.80 | 25 | 0.71% |

1 The site C0.1 exceedance C0.9 numbers.

2 The site C0.1 exceedance C0.9 percentage.

MOS= margin of safety.

Table S8 Comparison of species-specific and pooled Cu MLR in Species and numbers of toxicity data to previous work

| Number Species |  | Brix et al., 20171 | Brix et al., 20212 | This research |
| --- | --- | --- | --- | --- |
| Invertebrates | 1 | Ceriodaphnia dubia | C. dubia (n=113) | C. dubia (n=78) |
|  | 2 | Daphnia magna | D. magna (n=408) | D. magna (n=330) |
|  | 3 | Daphnia obtusa | D. obtuse (n=50) | D. obtuse (n=53) |
|  | 4 | Daphnia pulex | D. pulex (n=33) | D. pulex (n=31) |
|  | 5 | / | / | Daphnia pulicaria (n=24) |
| Fish | 6 | / | / | Oncorhynchus clarki (n=11) |
|  | 7 | / | Oncorhynchus mykiss (n=89) | O. mykiss (n=37) |
|  | 8 | / | / | Oryzias latipes (n=16) |
|  | 9 | Pimephales promelas | P. promelas (n=113) | P. promelas (n=219) |
|  |  |  | Pooled (n =899) | Pooled (n =831) |

“/” = None data

“n”= Data numbers

- Brix, K.V., DeForest, D.K., Tear, L., Grosell, M. and Adams, W.J., 2017. Use of multiple linear regression models for setting water quality criteria for copper: a complimentary approach to the biotic ligand model. Environ. Sci. Technol. 51, 5182‑5192.
 https://doi.org/10.1021/acs.est.6b05533

- Brix, K.V., Tear L., Santore, R.C., Croteau, K. and DeForest, D.K. 2021. Comparative performance of multiple linear regression and biotic ligand models for estimating the bioavailability of copper in freshwater. Environ. Toxicol. Chem. https://doi.org/10.1002/etc.5012

Fig. S1 Comparison of measured versus MLR-, hardness-based- and BLM-predicted for 9 species. Solid line is the line of perfect agreement between predicted and observed median effect concentrations. Dashed lines indicate a factor of ±2. MLR = multiple linear regression; BLM = biotic ligand model; RFx,2.0 = percentage of data within a factor of 2 using the slope rating formula.

R2=0.98746

Adj R2=0.98657

R2=0.98746

Adj R2=0.98657

R2=0.99207

Adj R2=0.9917

R2=0.9941

Adj R2=0.99368

Fig. S2 Comparison of fitted models to derive the HC5 based on MLR models

Table S9 Acute copper toxicity data used for normalization models development

| Species | Hardness (mg/L) | Temp (°C) | pH | DOC (mg/L) | Ca2+ (mg/L) | Mg2+ (mg/L) | Na+ (mg/L) | K+ (mg/L) | SO42- (mg/L) | Cl- (mg/L) | Alkalinity (mg/L) | S2- (mg/L) | LC50 (µg/L) | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acrocheilus alutaceus | 54.00 | 10.50 | 7.30 | 1.10 | 15.09 | 3.64 | 6.88 | 0.70 | 12.16 | 9.69 | 43.00 | 0.0003 | 137.28 | Andros and Garton 1980 |
| Acroneuria lycorias | 42.00 | 18.50 | 7.00 | 1.10 | 12.34 | 2.72 | 1.30 | 0.57 | 3.40 | 1.20 | 47.00 | 0.0003 | 7968.00 | Warnick and Bell 1969 |
| Actinonaias pectorosa | 96.00 | 25.00 | 8.35 | 0.50 | 15.84 | 13.73 | 29.73 | 2.38 | 92.16 | 2.15 | 102.00 | 0.0003 | 25.92 | Keller 2000 |
| A. pectorosa | 68.00 | 25.00 | 8.35 | 0.50 | 11.22 | 9.72 | 21.06 | 1.68 | 65.28 | 1.53 | 108.00 | 0.0003 | 27.84 | Keller 2000 |
| Bufo boreas | 167.00 | 22.00 | 7.90 | 0.50 | 27.56 | 23.88 | 51.72 | 4.13 | 160.32 | 3.75 | 115.00 | 0.0003 | 115.20 | Dwyer et al. 1999 |
| Campeloma decisum | 44.90 | 15.00 | 7.70 | 1.10 | 13.20 | 2.91 | 1.27 | 0.56 | 3.32 | 1.20 | 42.70 | 0.0003 | 1920.00 | Arthur and Leonard 1970 |
| C. decisum | 44.90 | 15.00 | 7.70 | 1.10 | 13.20 | 2.91 | 1.27 | 0.56 | 3.32 | 1.20 | 42.70 | 0.0003 | 1344.00 | Arthur and Leonard 1970 |
| Ceriodaphnia dubia | 97.60 | 25.00 | 8.00 | 2.00 | 24.07 | 9.13 | 5.44 | 1.60 | 20.80 | 6.40 | 74.20 | 0.0003 | 28.00 | Belanger and Cherry 1990 |
| C. dubia | 182.00 | 25.00 | 8.00 | 2.30 | 50.95 | 13.34 | 14.56 | 2.40 | 23.05 | 11.16 | 144.30 | 0.0003 | 84.00 | Belanger and Cherry 1990 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 25.00 | Belanger et al. 1989 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 17.00 | Belanger et al. 1989 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 30.00 | Belanger et al. 1989 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 24.00 | Belanger et al. 1989 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 28.00 | Belanger et al. 1989 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 32.00 | Belanger et al. 1989 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 23.00 | Belanger et al. 1989 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 20.00 | Belanger et al. 1989 |
| C. dubia | 45.00 | 25.00 | 7.72 | 1.50 | 11.10 | 4.21 | 9.50 | 1.60 | 46.00 | 34.00 | 39.70 | 0.0003 | 19.00 | Belanger et al. 1989 |
| C. dubia | 94.10 | 25.00 | 8.15 | 2.00 | 23.21 | 8.80 | 5.24 | 1.60 | 20.05 | 6.17 | 69.60 | 0.0003 | 26.00 | Belanger et al. 1989 |
| C. dubia | 94.10 | 25.00 | 8.15 | 2.00 | 23.21 | 8.80 | 5.24 | 1.60 | 20.05 | 6.17 | 69.60 | 0.0003 | 21.00 | Belanger et al. 1989 |
| C. dubia | 94.10 | 25.00 | 8.15 | 2.00 | 23.21 | 8.80 | 5.24 | 1.60 | 20.05 | 6.17 | 69.60 | 0.0003 | 27.00 | Belanger et al. 1989 |
| C. dubia | 94.10 | 25.00 | 8.15 | 2.00 | 23.21 | 8.80 | 5.24 | 1.60 | 20.05 | 6.17 | 69.60 | 0.0003 | 37.00 | Belanger et al. 1989 |
| C. dubia | 94.10 | 25.00 | 8.15 | 2.00 | 23.21 | 8.80 | 5.24 | 1.60 | 20.05 | 6.17 | 69.60 | 0.0003 | 34.00 | Belanger et al. 1989 |
| C. dubia | 179.00 | 25.00 | 8.31 | 2.30 | 50.11 | 13.12 | 14.32 | 2.40 | 22.67 | 10.98 | 140.10 | 0.0003 | 67.00 | Belanger et al. 1989 |
| C. dubia | 179.00 | 25.00 | 8.31 | 2.30 | 50.11 | 13.12 | 14.32 | 2.40 | 22.67 | 10.98 | 140.10 | 0.0003 | 38.00 | Belanger et al. 1989 |
| C. dubia | 179.00 | 25.00 | 8.31 | 2.30 | 50.11 | 13.12 | 14.32 | 2.40 | 22.67 | 10.98 | 140.10 | 0.0003 | 78.00 | Belanger et al. 1989 |
| C. dubia | 179.00 | 25.00 | 8.31 | 2.30 | 50.11 | 13.12 | 14.32 | 2.40 | 22.67 | 10.98 | 140.10 | 0.0003 | 81.00 | Belanger et al. 1989 |
| C. dubia | 52.00 | 24.50 | 7.50 | 1.10 | 15.28 | 3.37 | 1.50 | 0.57 | 3.80 | 1.40 | 55.00 | 0.0003 | 18.24 | Carlson et al. 1986 |
| C. dubia | 52.00 | 24.50 | 7.50 | 1.10 | 15.28 | 3.37 | 1.50 | 0.57 | 3.80 | 1.40 | 55.00 | 0.0003 | 16.32 | Carlson et al. 1986 |
| C. dubia | 80.08 | 20.00 | 7.60 | 0.50 | 13.20 | 11.44 | 24.78 | 1.98 | 76.80 | 1.80 | 53.00 | 0.0003 | 3.70 | Diamond et al. 1997 |
| C. dubia | 80.08 | 20.00 | 7.60 | 0.50 | 13.20 | 11.44 | 24.78 | 1.98 | 76.80 | 1.80 | 53.00 | 0.0003 | 7.80 | Diamond et al. 1997 |
| C. dubia | 80.08 | 20.00 | 7.60 | 0.50 | 13.20 | 11.44 | 24.78 | 1.98 | 76.80 | 1.80 | 53.00 | 0.0003 | 8.40 | Diamond et al. 1997 |
| C. dubia | 80.08 | 20.00 | 7.60 | 0.50 | 13.20 | 11.44 | 24.78 | 1.98 | 76.80 | 1.80 | 53.00 | 0.0003 | 7.10 | Diamond et al. 1997 |
| C. dubia | 63.94 | 24.80 | 7.80 | 3.25 | 20.00 | 3.40 | 1.00 | 0.87 | 5.00 | 2.00 | 64.00 | 0.0003 | 41.60 | GLEC 2006 |
| C. dubia | 74.29 | 25.00 | 7.70 | 1.03 | 22.00 | 4.70 | 2.50 | 0.73 | 7.60 | 3.80 | 72.00 | 0.0003 | 10.20 | GLEC 2006 |
| C. dubia | 73.88 | 24.70 | 8.10 | 1.63 | 22.00 | 4.60 | 2.40 | 0.72 | 7.60 | 4.00 | 60.00 | 0.0003 | 12.10 | GLEC 2006 |
| C. dubia | 76.79 | 24.90 | 7.90 | 1.15 | 23.00 | 4.70 | 2.50 | 0.79 | 7.40 | 3.70 | 68.00 | 0.0003 | 12.10 | GLEC 2006 |
| C. dubia | 76.79 | 24.30 | 7.80 | 0.92 | 23.00 | 4.70 | 2.40 | 0.69 | 7.80 | 4.20 | 76.00 | 0.0003 | 6.30 | GLEC 2006 |
| C. dubia | 77.61 | 24.70 | 7.80 | 0.79 | 23.00 | 4.90 | 2.50 | 0.67 | 7.20 | 3.50 | 80.00 | 0.0003 | 12.70 | GLEC 2006 |
| C. dubia | 50.61 | 24.60 | 7.80 | 8.75 | 14.00 | 3.80 | 1.60 | 0.85 | 5.00 | 2.00 | 48.00 | 0.0003 | 57.90 | GLEC 2006 |
| C. dubia | 140.18 | 24.10 | 8.20 | 5.09 | 38.00 | 11.00 | 1.00 | 0.73 | 5.00 | 2.00 | 136.00 | 0.0003 | 27.50 | GLEC 2006 |
| C. dubia | 110.22 | 24.30 | 8.30 | 4.95 | 26.00 | 11.00 | 11.00 | 1.00 | 7.80 | 5.90 | 228.00 | 0.0003 | 18.60 | GLEC 2006 |
| C. dubia | 103.61 | 24.50 | 8.00 | 6.84 | 25.00 | 10.00 | 25.00 | 1.50 | 24.00 | 10.00 | 132.00 | 0.0003 | 56.50 | GLEC 2006 |
| C. dubia | 92.85 | 24.40 | 7.90 | 13.50 | 23.00 | 8.60 | 4.10 | 0.68 | 10.00 | 3.80 | 78.00 | 0.0003 | 99.70 | GLEC 2006 |
| C. dubia | 134.31 | 24.20 | 8.40 | 14.20 | 34.00 | 12.00 | 75.00 | 5.30 | 74.00 | 34.00 | 180.00 | 0.0003 | 84.50 | GLEC 2006 |
| C. dubia | 125.20 | 24.50 | 8.20 | 17.10 | 32.00 | 11.00 | 66.00 | 3.90 | 69.00 | 28.00 | 180.00 | 0.0003 | 162.00 | GLEC 2006 |
| C. dubia | 65.07 | 24.60 | 7.70 | 15.30 | 16.00 | 6.10 | 13.00 | 1.20 | 18.00 | 6.80 | 56.00 | 0.0003 | 147.80 | GLEC 2006 |
| C. dubia | 110.22 | 24.80 | 8.10 | 5.88 | 26.00 | 11.00 | 1.10 | 1.30 | 7.70 | 2.00 | 108.00 | 0.0003 | 64.70 | GLEC 2006 |
| C. dubia | 194.73 | 24.60 | 7.90 | 9.37 | 45.00 | 20.00 | 2.90 | 0.82 | 10.00 | 5.00 | 192.00 | 0.0003 | 78.50 | GLEC 2006 |
| C. dubia | 121.83 | 24.80 | 8.30 | 13.00 | 29.00 | 12.00 | 1.50 | 0.64 | 6.50 | 2.50 | 120.00 | 0.0003 | 122.20 | GLEC 2006 |
| C. dubia | 182.99 | 24.70 | 8.40 | 6.55 | 37.00 | 22.00 | 2.40 | 1.10 | 7.20 | 3.30 | 188.00 | 0.0003 | 48.30 | GLEC 2006 |
| C. dubia | 198.10 | 24.20 | 8.10 | 11.00 | 48.00 | 19.00 | 2.70 | 0.74 | 25.00 | 4.90 | 176.00 | 0.0003 | 80.00 | GLEC 2006 |
| C. dubia | 121.83 | 24.70 | 8.00 | 15.70 | 29.00 | 12.00 | 1.60 | 0.56 | 14.00 | 2.20 | 102.00 | 0.0003 | 167.70 | GLEC 2006 |
| C. dubia | 66.33 | 24.90 | 7.60 | 30.20 | 17.00 | 5.80 | 0.40 | 0.10 | 6.50 | 2.00 | 44.00 | 0.0003 | 210.00 | GLEC 2006 |
| C. dubia | 51.82 | 24.30 | 7.40 | 20.05 | 13.00 | 4.70 | 0.40 | 0.10 | 5.00 | 2.00 | 44.00 | 0.0003 | 136.40 | GLEC 2006 |
| C. dubia | 38.38 | 24.60 | 7.30 | 18.20 | 9.60 | 3.50 | 0.60 | 0.33 | 5.00 | 2.00 | 36.00 | 0.0003 | 133.90 | GLEC 2006 |
| C. dubia | 97.09 | 24.60 | 7.50 | 6.67 | 28.00 | 6.60 | 2.00 | 0.84 | 17.00 | 2.60 | 84.00 | 0.0003 | 63.80 | GLEC 2006 |
| C. dubia | 54.37 | 24.50 | 7.80 | 9.90 | 16.00 | 3.50 | 1.30 | 0.49 | 10.00 | 2.00 | 48.00 | 0.0003 | 101.90 | GLEC 2006 |
| C. dubia | 105.82 | 24.60 | 8.10 | 4.87 | 31.00 | 6.90 | 2.00 | 0.70 | 20.00 | 2.30 | 68.00 | 0.0003 | 28.20 | GLEC 2006 |
| C. dubia | 96.27 | 24.20 | 7.70 | 7.24 | 28.00 | 6.40 | 2.00 | 0.63 | 19.00 | 2.40 | 84.00 | 0.0003 | 48.00 | GLEC 2006 |
| C. dubia | 55.19 | 24.70 | 7.50 | 11.40 | 16.00 | 3.70 | 1.30 | 0.45 | 12.00 | 2.00 | 46.00 | 0.0003 | 77.50 | GLEC 2006 |
| C. dubia | 45.64 | 25.00 | 7.40 | 25.20 | 13.00 | 3.20 | 1.20 | 0.49 | 5.00 | 2.00 | 40.00 | 0.0003 | 185.10 | GLEC 2006 |
| C. dubia | 39.41 | 24.30 | 7.30 | 11.60 | 11.00 | 2.90 | 1.30 | 0.56 | 5.00 | 2.00 | 40.00 | 0.0003 | 103.40 | GLEC 2006 |
| C. dubia | 20.41 | 24.80 | 6.80 | 17.10 | 5.70 | 1.50 | 0.90 | 0.45 | 5.00 | 2.00 | 12.00 | 0.0003 | 116.90 | GLEC 2006 |
| C. dubia | 123.45 | 25.20 | 8.30 | 6.76 | 28.00 | 13.00 | 16.00 | 2.20 | 20.00 | 9.20 | 120.00 | 0.0003 | 29.30 | GLEC 2006 |
| C. dubia | 17.18 | 24.90 | 7.30 | 3.82 | 4.90 | 1.20 | 1.00 | 0.38 | 5.00 | 2.00 | 20.00 | 0.0003 | 14.40 | GLEC 2006 |
| C. dubia | 172.25 | 24.70 | 8.30 | 10.50 | 36.00 | 20.00 | 1.30 | 1.30 | 6.90 | 2.00 | 156.00 | 0.0003 | 41.60 | GLEC 2006 |
| C. dubia | 62.22 | 24.80 | 7.50 | 6.57 | 17.00 | 4.80 | 2.60 | 1.20 | 5.00 | 2.40 | 64.00 | 0.0003 | 59.40 | GLEC 2006 |
| C. dubia | 31.94 | 24.90 | 7.50 | 10.80 | 9.00 | 2.30 | 1.50 | 0.88 | 5.00 | 2.00 | 28.00 | 0.0003 | 117.10 | GLEC 2006 |
| C. dubia | 113.59 | 24.40 | 7.90 | 10.70 | 29.00 | 10.00 | 25.00 | 1.40 | 16.00 | 2.50 | 88.00 | 0.0003 | 41.10 | GLEC 2006 |
| C. dubia | 46.02 | 24.90 | 7.60 | 9.64 | 12.00 | 3.90 | 1.60 | 0.89 | 5.00 | 2.00 | 48.00 | 0.0003 | 66.90 | GLEC 2006 |
| C. dubia | 58.51 | 24.50 | 7.70 | 6.97 | 17.00 | 3.90 | 2.30 | 0.77 | 5.00 | 2.60 | 52.00 | 0.0003 | 65.70 | GLEC 2006 |
| C. dubia | 43.42 | 25.00 | 6.50 | 1.00 | 7.00 | 6.30 | 14.00 | 1.10 | 53.00 | 4.50 | 22.00 | 0.0003 | 1.60 | Markich et al. 2005 |
| C. dubia | 373.43 | 25.00 | 6.50 | 1.00 | 60.20 | 54.18 | 14.00 | 1.10 | 369.55 | 4.50 | 22.00 | 0.0003 | 1.60 | Markich et al. 2005 |
| C. dubia | 201.82 | 25.00 | 8.55 | 0.50 | 33.00 | 29.00 | 63.00 | 5.50 | 194.00 | 5.00 | 132.00 | 0.0003 | 105.00 | Naddy et al. 2015 |
| C. dubia | 201.82 | 25.00 | 8.60 | 0.50 | 33.00 | 29.00 | 63.00 | 5.50 | 194.00 | 5.00 | 129.00 | 0.0003 | 79.80 | Naddy et al. 2015 |
| C. dubia | 201.82 | 25.00 | 8.60 | 0.50 | 33.00 | 29.00 | 63.00 | 5.50 | 194.00 | 5.00 | 119.00 | 0.0003 | 74.40 | Naddy et al. 2015 |
| C. dubia | 35.57 | 25.00 | 7.50 | 0.50 | 6.00 | 5.00 | 13.00 | 1.00 | 40.00 | 1.00 | 28.00 | 0.0003 | 16.60 | Naddy et al. 2015 |
| C. dubia | 57.10 | 25.00 | 8.18 | 0.50 | 9.42 | 8.17 | 17.69 | 1.41 | 54.82 | 1.28 | 81.00 | 0.0003 | 12.86 | Oris et al. 1991 |
| C. dubia | 57.16 | 25.00 | 8.18 | 0.50 | 9.42 | 8.17 | 17.69 | 1.41 | 54.82 | 1.28 | 81.00 | 0.0003 | 12.90 | Oris et al. 1991 |
| C. dubia | 290.24 | 25.00 | 8.61 | 0.50 | 47.69 | 41.56 | 105.06 | 8.37 | 325.65 | 7.62 | 235.18 | 0.0003 | 200.00 | Schubauer-Berigan et al. 1993 |
| C. dubia | 99.93 | 25.00 | 8.40 | 0.40 | 26.00 | 8.50 | 8.70 | 1.00 | 18.00 | 9.10 | 100.00 | 0.0003 | 25.00 | Wang et al. 2011 |
| C. dubia | 99.93 | 25.00 | 8.30 | 5.80 | 26.00 | 8.50 | 9.20 | 1.00 | 20.00 | 11.00 | 94.00 | 0.0003 | 157.00 | Wang et al. 2011 |
| C. dubia | 99.93 | 25.00 | 8.30 | 5.80 | 26.00 | 8.50 | 9.20 | 1.00 | 20.00 | 11.00 | 93.00 | 0.0003 | 147.00 | Wang et al. 2011 |
| C. dubia | 100.34 | 25.00 | 8.30 | 10.00 | 26.00 | 8.60 | 9.40 | 1.10 | 29.00 | 16.00 | 96.00 | 0.0003 | 267.00 | Wang et al. 2011 |
| Chironomus decorus | 44.00 | 20.00 | 7.40 | 0.50 | 6.99 | 6.06 | 13.10 | 1.05 | 40.70 | 0.95 | 32.50 | 0.0003 | 709.44 | Kosalwat and Knight 1987 |
| Ctenopharyngodon idella | 10.66667 | 24.50 | 7.40 | 0.50 | 1.75 | 1.51 | 3.28 | 0.26 | 10.20 | 0.24 |  | 0.0003 | 31.00 | Wang 2014 |
| C. idella | 44.975 | 24.50 | 7.40 | 0.50 | 6.99 | 6.60 | 13.10 | 1.50 | 40.70 | 0.95 |  | 0.0003 | 41.00 | Wang 2014 |
| C. idella | 144.1 | 24.50 | 7.40 | 0.50 | 23.14 | 20.70 | 43.54 | 3.48 | 135.19 | 3.15 |  | 0.0003 | 296.00 | Wang 2014 |
| C. idella | 255.875 | 24.50 | 7.40 | 0.50 | 41.85 | 36.30 | 78.75 | 6.30 | 244.50 | 5.70 |  | 0.0003 | 452.00 | Wang 2014 |
| Cyclops Cyclopoidea | 49.17 | 25.00 | 5.89 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 287 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 5.95 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 288 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.33 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 335 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.51 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 205 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.62 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 309 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.76 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 281 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 7.15 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 322 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 7.55 | 0.01 | 18.00 | 1.00 | 10.20 | 2.00 | 10.10 | 33.70 | 44.40 | 0.0003 | 397 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 7.93 | 0.01 | 18.00 | 1.00 | 19.20 | 2.00 | 10.10 | 33.70 | 83.60 | 0.0003 | 551 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.92 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 169 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.95 | 0.50 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 160 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.89 | 0.75 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 140 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.91 | 1.13 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 202 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.93 | 1.69 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 253 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.98 | 2.53 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 288 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.93 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 241 | Chen et al.2017 |
| C. Cyclopoidea | 71.67 | 25.00 | 6.89 | 0.01 | 27.00 | 1.00 | 4.00 | 2.00 | 10.10 | 49.70 | 17.40 | 0.0003 | 199 | Chen et al.2017 |
| C. Cyclopoidea | 106.67 | 25.00 | 6.94 | 0.01 | 41.00 | 1.00 | 4.00 | 2.00 | 10.10 | 74.50 | 17.40 | 0.0003 | 245 | Chen et al.2017 |
| C. Cyclopoidea | 156.67 | 25.00 | 6.92 | 0.01 | 61.00 | 1.00 | 4.00 | 2.00 | 10.10 | 110.00 | 17.40 | 0.0003 | 285 | Chen et al.2017 |
| C. Cyclopoidea | 231.67 | 25.00 | 6.95 | 0.01 | 91.00 | 1.00 | 4.00 | 2.00 | 10.10 | 163.00 | 17.40 | 0.0003 | 339 | Chen et al.2017 |
| C. Cyclopoidea | 49.17 | 25.00 | 6.94 | 0.01 | 18.00 | 1.00 | 4.00 | 2.00 | 10.10 | 33.70 | 17.40 | 0.0003 | 161.294 | Chen et al.2017 |
| C. Cyclopoidea | 51.25 | 25.00 | 6.92 | 0.01 | 18.00 | 1.50 | 4.00 | 2.00 | 15.20 | 33.70 | 17.40 | 0.0003 | 173.276 | Chen et al.2017 |
| C. Cyclopoidea | 54.58 | 25.00 | 6.89 | 0.01 | 18.00 | 2.30 | 4.00 | 2.00 | 23.30 | 33.70 | 17.40 | 0.0003 | 192.4472 | Chen et al.2017 |
| C. Cyclopoidea | 59.17 | 25.00 | 6.95 | 0.01 | 18.00 | 3.40 | 4.00 | 2.00 | 34.40 | 33.70 | 17.40 | 0.0003 | 218.8076 | Chen et al.2017 |
| C. Cyclopoidea | 65.83 | 25.00 | 6.97 | 0.01 | 18.00 | 5.00 | 4.00 | 2.00 | 50.60 | 33.70 | 17.40 | 0.0003 | 257.15 | Chen et al.2017 |
| Daphnia magna | 170.00 | 20.00 | 7.80 | 0.50 | 27.94 | 24.24 | 52.51 | 4.20 | 162.74 | 3.80 | 115.00 | 0.0003 | 39.55 | Baird et al. 1991 |
| D. magna | 170.00 | 20.00 | 7.80 | 0.50 | 27.94 | 24.24 | 52.51 | 4.20 | 162.74 | 3.80 | 115.00 | 0.0003 | 10.08 | Baird et al. 1991 |
| D. magna | 170.00 | 20.00 | 7.80 | 0.50 | 27.94 | 24.24 | 52.51 | 4.20 | 162.74 | 3.80 | 115.00 | 0.0003 | 19.78 | Baird et al. 1991 |
| D. magna | 170.00 | 20.00 | 7.80 | 0.50 | 27.94 | 24.24 | 52.51 | 4.20 | 162.74 | 3.80 | 115.00 | 0.0003 | 16.61 | Baird et al. 1991 |
| D. magna | 170.00 | 20.00 | 7.80 | 0.50 | 27.94 | 24.24 | 52.51 | 4.20 | 162.74 | 3.80 | 115.00 | 0.0003 | 67.87 | Baird et al. 1991 |
| D. magna | 170.00 | 20.00 | 7.80 | 0.50 | 27.94 | 24.24 | 52.51 | 4.20 | 162.74 | 3.80 | 115.00 | 0.0003 | 30.05 | Baird et al. 1991 |
| D. magna | 52.00 | 18.20 | 7.80 | 1.10 | 14.00 | 3.50 | 12.00 | 2.90 | 23.00 | 11.00 | 45.00 | 0.0003 | 24.96 | Chapman et al. 1980 |
| D. magna | 105.00 | 20.30 | 7.90 | 1.10 | 29.00 | 6.80 | 29.00 | 5.30 | 57.00 | 21.00 | 79.00 | 0.0003 | 28.80 | Chapman et al. 1980 |
| D. magna | 106.00 | 19.70 | 8.10 | 1.10 | 29.00 | 6.80 | 29.00 | 5.30 | 57.00 | 21.00 | 82.00 | 0.0003 | 36.48 | Chapman et al. 1980 |
| D. magna | 207.00 | 19.90 | 8.30 | 1.10 | 58.00 | 13.00 | 62.00 | 8.20 | 127.00 | 40.00 | 166.00 | 0.0003 | 66.24 | Chapman et al. 1980 |
| D. magna | 250.19 | 20.00 | 6.90 | 3.04 | 80.16 | 12.15 | 48.74 | 3.05 | 48.00 | 144.64 | 9.47 | 0.0003 | 81.80 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.92 | 4.53 | 80.16 | 12.15 | 49.89 | 3.05 | 48.00 | 146.05 | 8.31 | 0.0003 | 128.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.91 | 8.96 | 80.16 | 12.15 | 53.80 | 3.05 | 48.00 | 150.31 | 9.60 | 0.0003 | 295.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 7.01 | 1.95 | 80.16 | 12.15 | 49.20 | 3.05 | 48.19 | 142.51 | 9.56 | 0.0003 | 129.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 7.00 | 5.11 | 80.16 | 12.15 | 54.26 | 3.05 | 48.48 | 144.28 | 8.21 | 0.0003 | 261.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.99 | 11.70 | 80.16 | 12.15 | 64.83 | 3.05 | 48.96 | 147.83 | 9.35 | 0.0003 | 638.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.98 | 1.97 | 80.16 | 12.15 | 52.65 | 3.05 | 48.19 | 142.15 | 10.57 | 0.0003 | 86.60 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.97 | 5.35 | 80.16 | 12.15 | 63.91 | 3.05 | 52.80 | 142.86 | 7.54 | 0.0003 | 192.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.99 | 10.80 | 80.16 | 12.15 | 81.84 | 3.05 | 48.96 | 143.93 | 8.42 | 0.0003 | 332.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.94 | 1.95 | 80.16 | 12.15 | 88.28 | 3.05 | 48.10 | 220.85 | 7.99 | 0.0003 | 53.80 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.92 | 8.54 | 80.16 | 12.15 | 232.20 | 3.05 | 48.29 | 478.58 | 7.64 | 0.0003 | 311.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.99 | 15.40 | 80.16 | 12.15 | 374.74 | 3.05 | 48.48 | 748.00 | 8.99 | 0.0003 | 542.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 7.08 | 2.08 | 80.16 | 12.15 | 57.48 | 3.05 | 48.00 | 163.78 | 11.08 | 0.0003 | 50.60 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 7.03 | 9.22 | 80.16 | 12.15 | 97.02 | 3.05 | 48.00 | 229.01 | 9.88 | 0.0003 | 275.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 6.98 | 16.90 | 80.16 | 12.15 | 139.32 | 3.05 | 48.10 | 299.20 | 8.79 | 0.0003 | 607.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 7.06 | 2.58 | 80.16 | 12.15 | 60.92 | 3.05 | 54.82 | 164.49 | 10.58 | 0.0003 | 60.60 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 7.07 | 13.70 | 80.16 | 12.15 | 125.07 | 3.05 | 84.38 | 249.57 | 10.83 | 0.0003 | 212.00 | De Schamphelaere et al. 2004 |
| D. magna | 250.19 | 20.00 | 7.10 | 17.80 | 80.16 | 12.15 | 148.75 | 3.05 | 95.23 | 281.12 | 11.59 | 0.0003 | 372.00 | De Schamphelaere et al. 2004 |
| D. magna | 5.13 | 25.00 | 8.30 | 0.50 | 0.90 | 0.70 | 26.20 | 2.20 | 5.00 | 2.00 | 60.00 | 0.0003 | 2.00 | Long et al. 2004 |
| D. magna | 10.67 | 25.00 | 8.20 | 0.50 | 1.80 | 1.50 | 26.30 | 2.20 | 10.10 | 2.00 | 56.00 | 0.0003 | 2.00 | Long et al. 2004 |
| D. magna | 21.09 | 25.00 | 8.30 | 0.50 | 3.50 | 3.00 | 26.30 | 2.20 | 20.30 | 2.00 | 56.00 | 0.0003 | 10.00 | Long et al. 2004 |
| D. magna | 46.82 | 25.00 | 8.20 | 0.50 | 7.70 | 6.70 | 26.30 | 2.20 | 44.70 | 2.00 | 56.00 | 0.0003 | 11.10 | Long et al. 2004 |
| D. magna | 5.13 | 25.00 | 5.60 | 0.50 | 0.90 | 0.70 | 26.20 | 2.20 | 5.00 | 2.00 | 24.00 | 0.0003 | 2.00 | Long et al. 2004 |
| D. magna | 5.13 | 25.00 | 7.00 | 0.50 | 0.90 | 0.70 | 26.20 | 2.20 | 5.00 | 2.00 | 52.00 | 0.0003 | 2.80 | Long et al. 2004 |
| D. magna | 5.13 | 25.00 | 8.60 | 0.50 | 0.90 | 0.70 | 26.20 | 2.20 | 5.00 | 2.00 | 56.00 | 0.0003 | 4.80 | Long et al. 2004 |
| D. magna | 21.09 | 25.00 | 5.50 | 0.50 | 3.50 | 3.00 | 26.30 | 2.20 | 20.30 | 2.00 | 18.00 | 0.0003 | 2.00 | Long et al. 2004 |
| D. magna | 21.09 | 25.00 | 7.00 | 0.50 | 3.50 | 3.00 | 26.30 | 2.20 | 20.30 | 2.00 | 60.00 | 0.0003 | 7.40 | Long et al. 2004 |
| D. magna | 21.09 | 25.00 | 8.50 | 0.50 | 3.50 | 3.00 | 26.30 | 2.20 | 20.30 | 2.00 | 64.00 | 0.0003 | 6.50 | Long et al. 2004 |
| D. magna | 109.90 | 21.00 | 6.93 | 2.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 12.50 | 0.0003 | 6.82 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 6.93 | 3.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 12.50 | 0.0003 | 15.74 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 7.43 | 3.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 13.88 | 0.0003 | 38.30 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 7.43 | 2.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 13.88 | 0.0003 | 17.95 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 7.82 | 2.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 14.50 | 0.0003 | 18.14 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 7.82 | 3.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 14.50 | 0.0003 | 38.11 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 6.93 | 4.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 12.50 | 0.0003 | 44.16 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 6.93 | 6.10 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 12.50 | 0.0003 | 69.02 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 7.43 | 4.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 13.88 | 0.0003 | 54.91 | Meador 1991 |
| D. magna | 109.90 | 21.00 | 7.82 | 4.40 | 40.00 | 2.43 | 85.10 | 1.23 | 10.00 | 106.00 | 14.50 | 0.0003 | 65.09 | Meador 1991 |
| D. magna | 94.07 | 22.00 | 7.60 | 3.00 | 14.32 | 14.16 | 23.70 | 2.25 | 102.83 | 2.04 | 51.54 | 0.0003 | 108.17 | Meyer et al. 2015 |
| D. magna | 79.10 | 22.00 | 7.60 | 3.00 | 12.63 | 11.55 | 26.04 | 2.06 | 90.70 | 1.87 | 56.63 | 0.0003 | 78.80 | Meyer et al. 2015 |
| D. magna | 83.15 | 22.00 | 7.60 | 3.00 | 13.69 | 11.89 | 23.39 | 1.90 | 90.71 | 1.72 | 50.87 | 0.0003 | 102.25 | Meyer et al. 2015 |
| D. magna | 73.14 | 22.00 | 7.60 | 3.00 | 12.31 | 10.30 | 17.73 | 1.77 | 82.49 | 1.60 | 38.56 | 0.0003 | 76.90 | Meyer et al. 2015 |
| D. magna | 80.48 | 22.00 | 7.60 | 3.00 | 13.28 | 11.49 | 24.72 | 2.10 | 96.03 | 1.91 | 53.77 | 0.0003 | 91.28 | Meyer et al. 2015 |
| D. magna | 96.45 | 22.00 | 7.60 | 3.00 | 15.65 | 13.93 | 30.61 | 2.51 | 99.19 | 2.27 | 66.57 | 0.0003 | 106.90 | Meyer et al. 2015 |
| D. magna | 77.32 | 22.00 | 7.60 | 3.00 | 13.03 | 10.88 | 18.37 | 2.17 | 109.67 | 1.97 | 56.00 | 0.0003 | 80.60 | Meyer et al. 2015 |
| D. magna | 84.84 | 22.00 | 7.60 | 3.00 | 14.03 | 12.09 | 25.35 | 2.13 | 85.69 | 1.93 | 55.77 | 0.0003 | 89.06 | Meyer et al. 2015 |
| D. magna | 82.05 | 22.00 | 7.60 | 3.00 | 13.47 | 11.76 | 25.62 | 1.91 | 91.28 | 1.73 | 64.00 | 0.0003 | 102.10 | Meyer et al. 2015 |
| D. magna | 74.18 | 22.00 | 7.60 | 3.00 | 12.47 | 10.45 | 23.63 | 1.91 | 84.17 | 1.74 | 57.00 | 0.0003 | 85.47 | Meyer et al. 2015 |
| D. magna | 84.06 | 22.00 | 7.60 | 3.00 | 13.85 | 12.01 | 23.95 | 2.04 | 84.49 | 1.85 | 57.00 | 0.0003 | 68.90 | Meyer et al. 2015 |
| D. magna | 83.24 | 22.00 | 7.60 | 3.00 | 13.96 | 11.75 | 24.97 | 2.42 | 87.93 | 2.20 | 53.00 | 0.0003 | 76.60 | Meyer et al. 2015 |
| D. magna | 86.99 | 22.00 | 7.60 | 3.00 | 14.35 | 12.42 | 27.03 | 2.32 | 88.80 | 2.10 | 53.00 | 0.0003 | 110.04 | Meyer et al. 2015 |
| D. magna | 91.18 | 22.00 | 7.60 | 3.00 | 15.02 | 13.04 | 26.87 | 2.15 | 91.51 | 1.95 | 57.00 | 0.0003 | 102.76 | Meyer et al. 2015 |
| D. magna | 89.00 | 22.00 | 7.60 | 3.00 | 14.57 | 12.77 | 26.23 | 2.17 | 89.37 | 1.96 | 57.00 | 0.0003 | 112.00 | Meyer et al. 2015 |
| D. magna | 81.22 | 22.00 | 7.60 | 3.00 | 13.27 | 11.67 | 24.56 | 1.88 | 77.95 | 1.71 | 56.00 | 0.0003 | 107.55 | Meyer et al. 2015 |
| D. magna | 78.67 | 22.00 | 7.60 | 3.00 | 12.86 | 11.30 | 24.66 | 1.88 | 92.90 | 1.70 | 56.00 | 0.0003 | 132.50 | Meyer et al. 2015 |
| D. magna | 92.03 | 22.00 | 7.60 | 3.00 | 14.98 | 13.27 | 25.60 | 2.08 | 136.73 | 1.88 | 48.00 | 0.0003 | 115.20 | Meyer et al. 2015 |
| D. magna | 91.04 | 22.00 | 7.60 | 3.00 | 14.62 | 13.25 | 24.54 | 2.07 | 84.57 | 1.88 | 48.00 | 0.0003 | 136.33 | Meyer et al. 2015 |
| D. magna | 97.90 | 22.00 | 7.60 | 3.00 | 16.12 | 14.00 | 27.38 | 2.34 | 96.33 | 2.12 | 46.00 | 0.0003 | 129.10 | Meyer et al. 2015 |
| D. magna | 94.88 | 22.00 | 7.60 | 3.00 | 15.59 | 13.59 | 28.44 | 2.39 | 88.39 | 2.17 | 46.00 | 0.0003 | 124.54 | Meyer et al. 2015 |
| D. magna | 39.00 | 20.00 | 7.80 | 1.10 | 10.99 | 2.78 | 5.81 | 0.70 | 7.94 | 7.77 | 51.00 | 0.0003 | 8.74 | Nebeker et al. 1986a |
| D. magna | 39.00 | 20.00 | 7.80 | 1.10 | 10.99 | 2.78 | 5.81 | 0.70 | 7.94 | 7.77 | 51.00 | 0.0003 | 11.23 | Nebeker et al. 1986a |
| D. magna | 38.00 | 20.00 | 7.79 | 1.10 | 10.71 | 2.72 | 5.74 | 0.70 | 7.66 | 7.64 | 50.00 | 0.0003 | 6.34 | Nebeker et al. 1986a |
| D. magna | 38.00 | 20.00 | 7.79 | 1.10 | 10.71 | 2.72 | 5.74 | 0.70 | 7.66 | 7.64 | 50.00 | 0.0003 | 9.50 | Nebeker et al. 1986a |
| D. magna | 39.00 | 20.00 | 6.90 | 1.10 | 10.99 | 2.78 | 5.81 | 0.70 | 7.94 | 7.77 | 30.00 | 0.0003 | 11.23 | Nebeker et al. 1986a |
| D. magna | 39.00 | 20.00 | 6.90 | 1.10 | 10.99 | 2.78 | 5.81 | 0.70 | 7.94 | 7.77 | 30.00 | 0.0003 | 6.43 | Nebeker et al. 1986a |
| D. magna | 26.00 | 20.00 | 7.60 | 1.10 | 7.43 | 2.03 | 4.89 | 0.70 | 4.28 | 6.11 | 24.00 | 0.0003 | 8.74 | Nebeker et al. 1986a |
| D. magna | 27.00 | 20.00 | 7.70 | 1.10 | 7.70 | 2.09 | 4.96 | 0.70 | 4.56 | 6.23 | 24.00 | 0.0003 | 4.99 | Nebeker et al. 1986a |
| D. magna | 86.18 | 25.00 | 6.25 | 2.10 | 13.90 | 12.50 | 3.70 | 2.30 | 84.10 | 5.20 | 4.00 | 0.0003 | 9.30 | Ryan 2005 |
| D. magna | 87.09 | 25.00 | 6.22 | 2.10 | 14.10 | 12.60 | 3.50 | 2.50 | 84.40 | 5.90 | 3.70 | 0.0003 | 6.10 | Ryan 2005 |
| D. magna | 86.60 | 25.00 | 6.19 | 2.10 | 13.90 | 12.60 | 3.50 | 2.60 | 83.90 | 6.20 | 3.50 | 0.0003 | 7.90 | Ryan 2005 |
| D. magna | 87.51 | 25.00 | 6.23 | 2.20 | 14.10 | 12.70 | 3.40 | 2.20 | 84.40 | 5.50 | 3.70 | 0.0003 | 8.30 | Ryan 2005 |
| D. magna | 87.09 | 25.00 | 6.22 | 2.30 | 14.10 | 12.60 | 3.80 | 2.70 | 84.70 | 5.80 | 3.70 | 0.0003 | 9.00 | Ryan 2005 |
| D. magna | 87.76 | 25.00 | 6.22 | 2.20 | 14.20 | 12.70 | 3.80 | 2.50 | 84.90 | 6.80 | 3.90 | 0.0003 | 5.40 | Ryan 2005 |
| D. magna | 87.51 | 25.00 | 6.25 | 2.20 | 14.10 | 12.70 | 3.70 | 2.40 | 84.80 | 6.70 | 3.90 | 0.0003 | 9.00 | Ryan 2005 |
| D. magna | 88.26 | 25.00 | 6.29 | 6.60 | 14.40 | 12.70 | 10.90 | 2.10 | 88.70 | 11.60 | 4.50 | 0.0003 | 28.20 | Ryan 2005 |
| D. magna | 88.67 | 25.00 | 6.20 | 6.60 | 14.40 | 12.80 | 10.90 | 2.30 | 87.30 | 13.40 | 3.70 | 0.0003 | 30.60 | Ryan 2005 |
| D. magna | 88.01 | 25.00 | 6.22 | 6.50 | 14.30 | 12.70 | 10.90 | 2.30 | 86.10 | 14.10 | 4.30 | 0.0003 | 33.70 | Ryan 2005 |
| D. magna | 87.84 | 25.00 | 6.21 | 7.00 | 14.40 | 12.60 | 11.10 | 2.10 | 85.90 | 13.40 | 4.30 | 0.0003 | 28.50 | Ryan 2005 |
| D. magna | 87.76 | 25.00 | 6.20 | 7.20 | 14.20 | 12.70 | 11.20 | 2.00 | 87.40 | 12.50 | 4.30 | 0.0003 | 33.90 | Ryan 2005 |
| D. magna | 88.01 | 25.00 | 6.19 | 6.80 | 14.30 | 12.70 | 12.10 | 2.10 | 87.10 | 16.10 | 4.30 | 0.0003 | 31.30 | Ryan 2005 |
| D. magna | 87.59 | 25.00 | 6.22 | 6.80 | 14.30 | 12.60 | 12.10 | 2.10 | 86.50 | 16.00 | 4.30 | 0.0003 | 34.50 | Ryan 2005 |
| D. magna | 89.08 | 25.00 | 6.20 | 14.70 | 14.40 | 12.90 | 23.00 | 2.30 | 94.30 | 23.60 | 5.30 | 0.0003 | 56.90 | Ryan 2005 |
| D. magna | 90.65 | 25.00 | 6.00 | 14.60 | 14.70 | 13.10 | 23.30 | 2.50 | 92.20 | 27.40 | 5.10 | 0.0003 | 39.00 | Ryan 2005 |
| D. magna | 100.41 | 25.00 | 5.97 | 14.30 | 16.30 | 14.50 | 23.20 | 2.60 | 100.00 | 28.70 | 5.10 | 0.0003 | 59.90 | Ryan 2005 |
| D. magna | 88.01 | 25.00 | 5.95 | 15.30 | 14.30 | 12.70 | 24.60 | 2.20 | 88.40 | 27.00 | 5.10 | 0.0003 | 48.00 | Ryan 2005 |
| D. magna | 94.21 | 25.00 | 5.86 | 15.50 | 15.30 | 13.60 | 24.10 | 2.70 | 97.40 | 25.50 | 5.10 | 0.0003 | 46.80 | Ryan 2005 |
| D. magna | 99.34 | 25.00 | 5.89 | 14.30 | 16.20 | 14.30 | 25.20 | 2.50 | 100.60 | 32.10 | 5.10 | 0.0003 | 42.90 | Ryan 2005 |
| D. magna | 98.27 | 25.00 | 5.90 | 14.20 | 16.10 | 14.10 | 25.20 | 2.40 | 99.70 | 31.40 | 4.80 | 0.0003 | 40.20 | Ryan 2005 |
| D. magna | 89.00 | 25.00 | 6.22 | 2.00 | 14.70 | 12.70 | 3.50 | 2.20 | 86.70 | 5.00 | 4.10 | 0.0003 | 7.00 | Ryan 2005 |
| D. magna | 89.92 | 25.00 | 6.24 | 2.10 | 14.90 | 12.80 | 4.30 | 2.30 | 86.90 | 5.70 | 3.70 | 0.0003 | 6.20 | Ryan 2005 |
| D. magna | 89.67 | 25.00 | 6.26 | 2.00 | 14.80 | 12.80 | 3.80 | 2.40 | 86.50 | 5.90 | 3.90 | 0.0003 | 8.30 | Ryan 2005 |
| D. magna | 88.84 | 25.00 | 6.22 | 2.20 | 14.80 | 12.60 | 3.30 | 2.30 | 86.00 | 5.70 | 3.90 | 0.0003 | 6.80 | Ryan 2005 |
| D. magna | 90.08 | 25.00 | 6.21 | 2.10 | 14.80 | 12.90 | 3.70 | 2.50 | 87.50 | 5.40 | 3.70 | 0.0003 | 5.90 | Ryan 2005 |
| D. magna | 89.25 | 25.00 | 6.20 | 2.10 | 14.80 | 12.70 | 4.00 | 2.40 | 86.20 | 6.50 | 3.70 | 0.0003 | 5.60 | Ryan 2005 |
| D. magna | 89.42 | 25.00 | 6.20 | 2.00 | 14.70 | 12.80 | 3.80 | 2.10 | 86.70 | 6.10 | 3.70 | 0.0003 | 5.60 | Ryan 2005 |
| D. magna | 94.55 | 25.00 | 6.34 | 6.30 | 15.60 | 13.50 | 10.90 | 2.30 | 94.50 | 11.40 | 4.90 | 0.0003 | 26.80 | Ryan 2005 |
| D. magna | 95.05 | 25.00 | 6.14 | 6.40 | 15.80 | 13.50 | 11.10 | 2.40 | 93.40 | 13.10 | 4.50 | 0.0003 | 14.30 | Ryan 2005 |
| D. magna | 94.96 | 25.00 | 6.18 | 6.30 | 15.60 | 13.60 | 11.30 | 2.40 | 92.70 | 13.80 | 4.10 | 0.0003 | 19.40 | Ryan 2005 |
| D. magna | 95.62 | 25.00 | 6.17 | 6.70 | 15.70 | 13.70 | 10.70 | 2.80 | 93.40 | 13.40 | 4.40 | 0.0003 | 22.70 | Ryan 2005 |
| D. magna | 94.55 | 25.00 | 6.12 | 6.70 | 15.60 | 13.50 | 10.70 | 2.20 | 93.50 | 11.90 | 4.20 | 0.0003 | 14.80 | Ryan 2005 |
| D. magna | 94.38 | 25.00 | 6.13 | 6.50 | 15.70 | 13.40 | 11.90 | 2.60 | 93.10 | 15.90 | 4.40 | 0.0003 | 16.00 | Ryan 2005 |
| D. magna | 95.05 | 25.00 | 6.11 | 6.50 | 15.80 | 13.50 | 11.60 | 2.40 | 93.70 | 15.70 | 4.40 | 0.0003 | 16.30 | Ryan 2005 |
| D. magna | 88.51 | 25.00 | 6.26 | 0.70 | 14.50 | 12.70 | 0.50 | 2.20 | 84.80 | 2.00 | 3.50 | 0.0003 | 5.60 | Ryan 2005 |
| D. magna | 89.25 | 25.00 | 6.41 | 13.50 | 14.80 | 12.70 | 22.50 | 2.20 | 93.70 | 21.80 | 6.30 | 0.0003 | 66.50 | Ryan 2005 |
| D. magna | 88.51 | 25.00 | 6.12 | 13.90 | 14.50 | 12.70 | 27.50 | 2.10 | 89.90 | 25.90 | 4.80 | 0.0003 | 60.40 | Ryan 2005 |
| D. magna | 88.34 | 25.00 | 6.19 | 13.80 | 14.60 | 12.60 | 24.30 | 2.30 | 88.40 | 27.40 | 4.80 | 0.0003 | 54.00 | Ryan 2005 |
| D. magna | 88.34 | 25.00 | 6.18 | 14.80 | 14.60 | 12.60 | 23.50 | 2.40 | 88.30 | 26.30 | 4.90 | 0.0003 | 63.90 | Ryan 2005 |
| D. magna | 88.34 | 25.00 | 6.08 | 14.20 | 14.60 | 12.60 | 23.20 | 2.20 | 91.50 | 23.20 | 4.80 | 0.0003 | 58.50 | Ryan 2005 |
| D. magna | 88.51 | 25.00 | 6.01 | 13.70 | 14.50 | 12.70 | 25.90 | 2.30 | 90.20 | 30.60 | 4.40 | 0.0003 | 43.10 | Ryan 2005 |
| D. magna | 88.84 | 25.00 | 5.92 | 13.70 | 14.80 | 12.60 | 23.90 | 2.20 | 90.30 | 30.30 | 4.10 | 0.0003 | 45.50 | Ryan 2005 |
| D. magna | 89.92 | 25.00 | 7.04 | 0.30 | 14.90 | 12.80 | 3.50 | 2.30 | 86.30 | 2.10 | 8.80 | 0.0003 | 3.20 | Ryan 2005 |
| D. magna | 85.95 | 25.00 | 7.18 | 2.10 | 14.30 | 12.20 | 7.40 | 2.30 | 83.80 | 5.20 | 10.70 | 0.0003 | 27.30 | Ryan 2005 |
| D. magna | 86.61 | 25.00 | 7.16 | 2.10 | 14.40 | 12.30 | 7.20 | 2.20 | 83.80 | 5.50 | 10.10 | 0.0003 | 17.40 | Ryan 2005 |
| D. magna | 87.02 | 25.00 | 7.17 | 2.00 | 14.40 | 12.40 | 7.50 | 2.10 | 84.10 | 5.60 | 10.70 | 0.0003 | 17.50 | Ryan 2005 |
| D. magna | 86.11 | 25.00 | 7.19 | 2.20 | 14.20 | 12.30 | 7.80 | 2.20 | 83.00 | 5.50 | 11.20 | 0.0003 | 23.90 | Ryan 2005 |
| D. magna | 86.20 | 25.00 | 7.17 | 2.20 | 14.40 | 12.20 | 7.40 | 2.20 | 83.70 | 5.30 | 10.40 | 0.0003 | 20.60 | Ryan 2005 |
| D. magna | 87.43 | 25.00 | 7.19 | 2.20 | 14.40 | 12.50 | 8.40 | 2.40 | 84.50 | 6.80 | 10.90 | 0.0003 | 22.80 | Ryan 2005 |
| D. magna | 86.45 | 25.00 | 7.20 | 2.20 | 14.50 | 12.20 | 8.00 | 2.30 | 83.90 | 6.70 | 11.50 | 0.0003 | 20.30 | Ryan 2005 |
| D. magna | 89.25 | 25.00 | 6.98 | 0.30 | 14.80 | 12.70 | 3.60 | 2.30 | 85.60 | 2.10 | 8.80 | 0.0003 | 2.20 | Ryan 2005 |
| D. magna | 78.01 | 25.00 | 7.30 | 6.60 | 13.10 | 11.00 | 17.20 | 2.00 | 78.80 | 11.50 | 16.00 | 0.0003 | 112.80 | Ryan 2005 |
| D. magna | 78.58 | 25.00 | 7.34 | 6.70 | 13.00 | 11.20 | 17.80 | 2.10 | 77.90 | 13.30 | 17.10 | 0.0003 | 95.30 | Ryan 2005 |
| D. magna | 78.48 | 25.00 | 7.27 | 6.60 | 12.30 | 11.60 | 18.10 | 2.00 | 77.20 | 14.00 | 14.10 | 0.0003 | 70.40 | Ryan 2005 |
| D. magna | 76.85 | 25.00 | 7.28 | 7.10 | 12.80 | 10.90 | 17.60 | 2.00 | 75.40 | 13.40 | 15.20 | 0.0003 | 102.10 | Ryan 2005 |
| D. magna | 79.91 | 25.00 | 7.30 | 6.80 | 13.20 | 11.40 | 17.70 | 2.30 | 79.70 | 12.20 | 15.50 | 0.0003 | 82.90 | Ryan 2005 |
| D. magna | 82.14 | 25.00 | 7.33 | 6.60 | 13.60 | 11.70 | 19.30 | 2.30 | 81.10 | 15.80 | 15.50 | 0.0003 | 126.00 | Ryan 2005 |
| D. magna | 81.07 | 25.00 | 7.36 | 6.70 | 13.50 | 11.50 | 20.30 | 2.30 | 80.30 | 15.90 | 16.00 | 0.0003 | 103.40 | Ryan 2005 |
| D. magna | 120.43 | 25.00 | 7.06 | 0.30 | 19.70 | 17.30 | 3.50 | 3.40 | 115.50 | 3.10 | 10.40 | 0.0003 | 2.40 | Ryan 2005 |
| D. magna | 86.02 | 25.00 | 7.36 | 15.40 | 14.00 | 12.40 | 32.90 | 2.10 | 91.60 | 24.50 | 22.10 | 0.0003 | 250.40 | Ryan 2005 |
| D. magna | 88.67 | 25.00 | 7.37 | 15.60 | 14.40 | 12.80 | 34.80 | 2.10 | 90.60 | 28.80 | 21.10 | 0.0003 | 221.50 | Ryan 2005 |
| D. magna | 99.18 | 25.00 | 7.31 | 15.10 | 16.30 | 14.20 | 31.70 | 2.80 | 98.80 | 30.30 | 19.50 | 0.0003 | 236.10 | Ryan 2005 |
| D. magna | 85.61 | 25.00 | 7.35 | 15.90 | 14.00 | 12.30 | 31.20 | 2.20 | 86.10 | 27.90 | 20.80 | 0.0003 | 245.50 | Ryan 2005 |
| D. magna | 92.06 | 25.00 | 7.37 | 16.10 | 15.10 | 13.20 | 32.70 | 2.30 | 95.80 | 26.10 | 22.00 | 0.0003 | 229.80 | Ryan 2005 |
| D. magna | 98.93 | 25.00 | 7.40 | 14.50 | 16.20 | 14.20 | 34.90 | 2.40 | 100.10 | 32.40 | 21.10 | 0.0003 | 298.30 | Ryan 2005 |
| D. magna | 98.02 | 25.00 | 7.43 | 14.60 | 16.00 | 14.10 | 37.00 | 2.30 | 99.50 | 32.20 | 22.90 | 0.0003 | 290.90 | Ryan 2005 |
| D. magna | 85.77 | 25.00 | 7.03 | 0.60 | 13.90 | 12.40 | 3.20 | 2.00 | 82.30 | 1.80 | 8.30 | 0.0003 | 2.40 | Ryan 2005 |
| D. magna | 85.86 | 25.00 | 7.16 | 2.20 | 14.10 | 12.30 | 6.90 | 2.20 | 83.90 | 5.20 | 10.40 | 0.0003 | 17.90 | Ryan 2005 |
| D. magna | 86.27 | 25.00 | 7.18 | 2.10 | 14.10 | 12.40 | 6.90 | 2.00 | 83.60 | 5.50 | 10.50 | 0.0003 | 17.10 | Ryan 2005 |
| D. magna | 85.61 | 25.00 | 7.19 | 2.20 | 14.00 | 12.30 | 7.00 | 2.40 | 82.80 | 6.10 | 10.30 | 0.0003 | 19.80 | Ryan 2005 |
| D. magna | 85.20 | 25.00 | 7.18 | 2.20 | 14.00 | 12.20 | 6.90 | 2.10 | 82.00 | 5.50 | 10.40 | 0.0003 | 20.90 | Ryan 2005 |
| D. magna | 86.27 | 25.00 | 7.19 | 2.20 | 14.10 | 12.40 | 6.90 | 2.20 | 83.80 | 5.30 | 10.80 | 0.0003 | 20.60 | Ryan 2005 |
| D. magna | 87.18 | 25.00 | 7.20 | 2.30 | 14.30 | 12.50 | 7.10 | 2.20 | 84.30 | 6.60 | 10.50 | 0.0003 | 21.80 | Ryan 2005 |
| D. magna | 86.93 | 25.00 | 7.21 | 2.20 | 14.20 | 12.50 | 7.40 | 2.10 | 84.10 | 6.50 | 10.90 | 0.0003 | 20.70 | Ryan 2005 |
| D. magna | 94.71 | 25.00 | 7.32 | 6.70 | 15.50 | 13.60 | 17.20 | 2.60 | 95.00 | 12.10 | 13.20 | 0.0003 | 56.10 | Ryan 2005 |
| D. magna | 94.30 | 25.00 | 7.32 | 8.30 | 15.50 | 13.50 | 16.50 | 2.40 | 93.40 | 16.30 | 13.30 | 0.0003 | 55.40 | Ryan 2005 |
| D. magna | 94.30 | 25.00 | 7.33 | 6.80 | 15.50 | 13.50 | 15.80 | 2.30 | 92.20 | 14.50 | 13.20 | 0.0003 | 68.80 | Ryan 2005 |
| D. magna | 94.55 | 25.00 | 7.32 | 7.20 | 15.60 | 13.50 | 16.00 | 2.50 | 92.40 | 14.00 | 12.90 | 0.0003 | 65.20 | Ryan 2005 |
| D. magna | 94.30 | 25.00 | 7.32 | 8.30 | 15.50 | 13.50 | 17.10 | 2.30 | 94.30 | 14.40 | 12.90 | 0.0003 | 67.90 | Ryan 2005 |
| D. magna | 94.96 | 25.00 | 7.31 | 8.20 | 15.60 | 13.60 | 17.30 | 2.30 | 94.10 | 19.30 | 12.80 | 0.0003 | 82.70 | Ryan 2005 |
| D. magna | 94.30 | 25.00 | 7.31 | 6.80 | 15.50 | 13.50 | 16.80 | 2.40 | 92.70 | 16.30 | 13.10 | 0.0003 | 60.20 | Ryan 2005 |
| D. magna | 87.68 | 25.00 | 7.21 | 0.60 | 14.50 | 12.50 | 4.10 | 1.70 | 84.20 | 1.60 | 10.70 | 0.0003 | 2.10 | Ryan 2005 |
| D. magna | 88.59 | 25.00 | 7.21 | 14.40 | 14.70 | 12.60 | 26.40 | 1.70 | 93.60 | 22.70 | 12.80 | 0.0003 | 129.20 | Ryan 2005 |
| D. magna | 87.43 | 25.00 | 7.16 | 14.90 | 14.40 | 12.50 | 27.50 | 1.80 | 89.40 | 27.30 | 12.10 | 0.0003 | 139.50 | Ryan 2005 |
| D. magna | 88.34 | 25.00 | 7.15 | 15.00 | 14.60 | 12.60 | 26.20 | 2.10 | 88.50 | 29.50 | 12.00 | 0.0003 | 126.10 | Ryan 2005 |
| D. magna | 88.59 | 25.00 | 7.16 | 15.50 | 14.70 | 12.60 | 26.60 | 1.70 | 88.80 | 26.90 | 11.90 | 0.0003 | 151.00 | Ryan 2005 |
| D. magna | 89.25 | 25.00 | 7.13 | 15.30 | 14.80 | 12.70 | 26.00 | 1.90 | 92.60 | 24.50 | 11.90 | 0.0003 | 125.70 | Ryan 2005 |
| D. magna | 89.25 | 25.00 | 7.10 | 14.50 | 14.80 | 12.70 | 29.10 | 1.70 | 91.10 | 31.80 | 11.60 | 0.0003 | 125.80 | Ryan 2005 |
| D. magna | 88.34 | 25.00 | 7.15 | 14.90 | 14.60 | 12.60 | 29.10 | 1.90 | 90.20 | 32.40 | 11.70 | 0.0003 | 133.30 | Ryan 2005 |
| D. magna | 90.90 | 25.00 | 7.94 | 0.50 | 14.80 | 13.10 | 24.40 | 2.50 | 87.10 | 2.30 | 52.00 | 0.0003 | 12.50 | Ryan 2005 |
| D. magna | 87.76 | 25.00 | 7.95 | 2.40 | 14.20 | 12.70 | 27.20 | 2.10 | 85.50 | 5.40 | 53.30 | 0.0003 | 121.50 | Ryan 2005 |
| D. magna | 86.27 | 25.00 | 7.95 | 2.40 | 14.10 | 12.40 | 27.30 | 2.10 | 83.80 | 6.10 | 53.30 | 0.0003 | 64.50 | Ryan 2005 |
| D. magna | 87.34 | 25.00 | 7.95 | 2.30 | 14.20 | 12.60 | 29.50 | 2.30 | 84.20 | 6.30 | 52.00 | 0.0003 | 58.90 | Ryan 2005 |
| D. magna | 86.68 | 25.00 | 7.97 | 2.40 | 14.10 | 12.50 | 28.50 | 2.10 | 83.80 | 5.90 | 53.30 | 0.0003 | 86.80 | Ryan 2005 |
| D. magna | 87.59 | 25.00 | 7.97 | 2.50 | 14.30 | 12.60 | 27.80 | 2.20 | 85.00 | 5.60 | 53.30 | 0.0003 | 77.30 | Ryan 2005 |
| D. magna | 87.18 | 25.00 | 7.96 | 2.40 | 14.30 | 12.50 | 27.80 | 2.00 | 84.60 | 6.80 | 52.00 | 0.0003 | 97.50 | Ryan 2005 |
| D. magna | 87.76 | 25.00 | 7.97 | 2.40 | 14.20 | 12.70 | 27.90 | 2.40 | 85.20 | 7.20 | 53.30 | 0.0003 | 91.00 | Ryan 2005 |
| D. magna | 89.99 | 25.00 | 8.01 | 0.40 | 14.60 | 13.00 | 29.00 | 2.40 | 86.50 | 2.20 | 54.70 | 0.0003 | 17.00 | Ryan 2005 |
| D. magna | 76.76 | 25.00 | 8.39 | 6.70 | 12.60 | 11.00 | 84.90 | 2.00 | 77.70 | 11.70 | 150.70 | 0.0003 | 574.80 | Ryan 2005 |
| D. magna | 78.50 | 25.00 | 8.40 | 6.80 | 12.80 | 11.30 | 84.40 | 2.20 | 77.80 | 13.70 | 146.70 | 0.0003 | 239.70 | Ryan 2005 |
| D. magna | 80.73 | 25.00 | 8.30 | 6.80 | 13.20 | 11.60 | 66.90 | 2.10 | 79.00 | 14.50 | 117.30 | 0.0003 | 193.00 | Ryan 2005 |
| D. magna | 76.92 | 25.00 | 8.29 | 7.00 | 12.50 | 11.10 | 64.30 | 2.10 | 75.60 | 13.30 | 114.70 | 0.0003 | 258.50 | Ryan 2005 |
| D. magna | 78.50 | 25.00 | 8.33 | 7.00 | 12.80 | 11.30 | 70.60 | 2.10 | 78.50 | 12.40 | 124.00 | 0.0003 | 301.00 | Ryan 2005 |
| D. magna | 82.05 | 25.00 | 8.33 | 6.80 | 13.40 | 11.80 | 70.20 | 2.30 | 81.20 | 16.30 | 122.70 | 0.0003 | 377.30 | Ryan 2005 |
| D. magna | 81.64 | 25.00 | 8.33 | 6.80 | 13.40 | 11.70 | 70.80 | 2.30 | 80.60 | 16.20 | 122.70 | 0.0003 | 361.80 | Ryan 2005 |
| D. magna | 81.80 | 25.00 | 7.99 | 0.30 | 13.30 | 11.80 | 23.80 | 1.90 | 78.60 | 1.70 | 56.00 | 0.0003 | 6.90 | Ryan 2005 |
| D. magna | 83.79 | 25.00 | 8.06 | 15.00 | 13.60 | 12.10 | 54.30 | 2.00 | 89.30 | 23.90 | 70.70 | 0.0003 | 551.00 | Ryan 2005 |
| D. magna | 83.54 | 25.00 | 8.06 | 15.50 | 13.50 | 12.10 | 54.20 | 2.00 | 85.80 | 28.40 | 66.70 | 0.0003 | 380.50 | Ryan 2005 |
| D. magna | 84.70 | 25.00 | 8.08 | 15.30 | 13.80 | 12.20 | 54.70 | 2.00 | 85.20 | 30.00 | 66.70 | 0.0003 | 367.80 | Ryan 2005 |
| D. magna | 84.70 | 25.00 | 8.10 | 16.10 | 13.80 | 12.20 | 53.80 | 2.30 | 85.10 | 28.30 | 69.30 | 0.0003 | 388.40 | Ryan 2005 |
| D. magna | 84.04 | 25.00 | 8.09 | 15.90 | 13.70 | 12.10 | 53.80 | 1.90 | 88.10 | 25.50 | 66.70 | 0.0003 | 402.80 | Ryan 2005 |
| D. magna | 85.11 | 25.00 | 8.09 | 15.80 | 13.80 | 12.30 | 55.70 | 1.90 | 87.40 | 34.70 | 66.70 | 0.0003 | 530.80 | Ryan 2005 |
| D. magna | 85.20 | 25.00 | 8.09 | 15.70 | 14.00 | 12.20 | 52.70 | 2.10 | 87.50 | 34.40 | 68.00 | 0.0003 | 483.70 | Ryan 2005 |
| D. magna | 90.58 | 25.00 | 8.00 | 0.40 | 15.00 | 12.90 | 23.60 | 2.50 | 86.90 | 2.20 | 53.30 | 0.0003 | 11.60 | Ryan 2005 |
| D. magna | 90.58 | 25.00 | 8.03 | 2.10 | 15.00 | 12.90 | 25.60 | 2.40 | 88.30 | 5.30 | 52.00 | 0.0003 | 114.60 | Ryan 2005 |
| D. magna | 90.99 | 25.00 | 8.03 | 2.10 | 15.00 | 13.00 | 26.10 | 2.30 | 88.00 | 5.70 | 50.70 | 0.0003 | 55.00 | Ryan 2005 |
| D. magna | 95.46 | 25.00 | 8.02 | 2.10 | 15.80 | 13.60 | 26.80 | 2.60 | 92.00 | 6.30 | 50.70 | 0.0003 | 61.70 | Ryan 2005 |
| D. magna | 91.90 | 25.00 | 8.03 | 2.30 | 15.20 | 13.10 | 26.10 | 2.40 | 88.80 | 5.90 | 50.70 | 0.0003 | 54.30 | Ryan 2005 |
| D. magna | 90.17 | 25.00 | 8.03 | 2.30 | 15.00 | 12.80 | 26.90 | 2.30 | 87.70 | 5.50 | 50.70 | 0.0003 | 55.60 | Ryan 2005 |
| D. magna | 95.71 | 25.00 | 8.03 | 2.20 | 15.90 | 13.60 | 29.10 | 2.50 | 92.90 | 6.90 | 50.70 | 0.0003 | 72.60 | Ryan 2005 |
| D. magna | 91.49 | 25.00 | 8.03 | 2.20 | 15.20 | 13.00 | 28.70 | 2.40 | 88.60 | 6.70 | 50.70 | 0.0003 | 80.20 | Ryan 2005 |
| D. magna | 92.56 | 25.00 | 7.84 | 0.50 | 15.30 | 13.20 | 16.80 | 2.30 | 88.70 | 2.10 | 40.00 | 0.0003 | 3.20 | Ryan 2005 |
| D. magna | 94.30 | 25.00 | 7.86 | 6.50 | 15.50 | 13.50 | 28.10 | 2.20 | 94.20 | 11.50 | 44.00 | 0.0003 | 234.90 | Ryan 2005 |
| D. magna | 92.97 | 25.00 | 7.87 | 6.70 | 15.30 | 13.30 | 29.80 | 2.40 | 91.70 | 13.70 | 41.30 | 0.0003 | 153.50 | Ryan 2005 |
| D. magna | 92.56 | 25.00 | 7.87 | 6.60 | 15.30 | 13.20 | 27.70 | 2.30 | 90.60 | 14.20 | 41.30 | 0.0003 | 132.60 | Ryan 2005 |
| D. magna | 93.22 | 25.00 | 7.87 | 7.00 | 15.40 | 13.30 | 28.40 | 2.20 | 91.30 | 13.30 | 41.30 | 0.0003 | 178.20 | Ryan 2005 |
| D. magna | 92.81 | 25.00 | 7.87 | 6.90 | 15.40 | 13.20 | 28.90 | 2.30 | 92.30 | 12.40 | 41.30 | 0.0003 | 166.70 | Ryan 2005 |
| D. magna | 94.55 | 25.00 | 7.86 | 6.70 | 15.60 | 13.50 | 29.70 | 2.20 | 93.10 | 15.90 | 41.30 | 0.0003 | 180.10 | Ryan 2005 |
| D. magna | 94.30 | 25.00 | 7.85 | 6.80 | 15.50 | 13.50 | 29.10 | 2.20 | 92.90 | 16.00 | 41.30 | 0.0003 | 178.40 | Ryan 2005 |
| D. magna | 91.49 | 25.00 | 7.78 | 0.30 | 15.20 | 13.00 | 17.40 | 2.00 | 87.80 | 1.80 | 42.70 | 0.0003 | 10.50 | Ryan 2005 |
| D. magna | 91.08 | 25.00 | 8.04 | 14.70 | 15.20 | 12.90 | 53.40 | 2.20 | 96.30 | 23.50 | 72.00 | 0.0003 | 474.80 | Ryan 2005 |
| D. magna | 90.83 | 25.00 | 8.05 | 15.30 | 15.10 | 12.90 | 54.80 | 2.30 | 92.80 | 28.50 | 72.00 | 0.0003 | 426.80 | Ryan 2005 |
| D. magna | 91.49 | 25.00 | 8.06 | 14.10 | 15.20 | 13.00 | 54.00 | 2.40 | 91.10 | 28.10 | 70.70 | 0.0003 | 355.60 | Ryan 2005 |
| D. magna | 92.97 | 25.00 | 8.07 | 15.00 | 15.30 | 13.30 | 56.30 | 2.20 | 93.10 | 26.40 | 72.00 | 0.0003 | 379.20 | Ryan 2005 |
| D. magna | 92.31 | 25.00 | 8.07 | 15.40 | 15.20 | 13.20 | 57.20 | 2.30 | 95.60 | 25.00 | 69.30 | 0.0003 | 350.90 | Ryan 2005 |
| D. magna | 93.47 | 25.00 | 8.08 | 14.30 | 15.50 | 13.30 | 57.90 | 2.40 | 95.10 | 32.00 | 70.70 | 0.0003 | 415.80 | Ryan 2005 |
| D. magna | 94.55 | 25.00 | 8.08 | 14.60 | 15.60 | 13.50 | 61.10 | 2.40 | 96.20 | 32.40 | 69.30 | 0.0003 | 475.00 | Ryan 2005 |
| D. magna | 10.83 | 25.00 | 6.16 | 0.60 | 1.70 | 1.60 | 2.90 | 2.00 | 10.40 | 1.00 | 5.30 | 0.0003 | 0.50 | Ryan et al. 2009 |
| D. magna | 20.43 | 25.00 | 5.96 | 0.80 | 3.40 | 2.90 | 2.80 | 2.00 | 20.10 | 1.00 | 5.30 | 0.0003 | 1.00 | Ryan et al. 2009 |
| D. magna | 42.35 | 25.00 | 6.34 | 0.50 | 6.90 | 6.10 | 2.80 | 2.00 | 40.40 | 0.90 | 5.30 | 0.0003 | 1.60 | Ryan et al. 2009 |
| D. magna | 9.51 | 25.00 | 7.91 | 0.30 | 1.50 | 1.40 | 27.00 | 2.00 | 9.20 | 1.00 | 61.30 | 0.0003 | 5.20 | Ryan et al. 2009 |
| D. magna | 18.45 | 25.00 | 7.97 | 0.30 | 3.10 | 2.60 | 26.90 | 2.00 | 18.20 | 1.00 | 61.30 | 0.0003 | 2.60 | Ryan et al. 2009 |
| D. magna | 42.35 | 25.00 | 7.95 | 0.60 | 6.90 | 6.10 | 26.90 | 2.00 | 40.50 | 1.10 | 61.30 | 0.0003 | 9.30 | Ryan et al. 2009 |
| D. magna | 8.85 | 25.00 | 6.02 | 2.20 | 1.40 | 1.30 | 10.60 | 2.00 | 10.90 | 4.20 | 6.70 | 0.0003 | 3.20 | Ryan et al. 2009 |
| D. magna | 22.00 | 25.00 | 6.11 | 2.20 | 3.70 | 3.10 | 10.50 | 2.00 | 23.90 | 4.10 | 6.70 | 0.0003 | 4.90 | Ryan et al. 2009 |
| D. magna | 39.95 | 25.00 | 6.17 | 2.30 | 6.60 | 5.70 | 10.60 | 2.00 | 40.80 | 4.20 | 6.70 | 0.0003 | 7.00 | Ryan et al. 2009 |
| D. magna | 10.83 | 25.00 | 6.33 | 8.70 | 1.70 | 1.60 | 33.10 | 2.00 | 19.90 | 13.20 | 6.70 | 0.0003 | 18.80 | Ryan et al. 2009 |
| D. magna | 23.08 | 25.00 | 6.33 | 8.70 | 3.80 | 3.30 | 33.10 | 2.00 | 30.70 | 13.30 | 5.30 | 0.0003 | 26.60 | Ryan et al. 2009 |
| D. magna | 45.00 | 25.00 | 6.33 | 8.40 | 7.30 | 6.50 | 32.10 | 2.00 | 51.90 | 12.90 | 5.30 | 0.0003 | 22.80 | Ryan et al. 2009 |
| D. magna | 8.94 | 25.00 | 7.92 | 2.30 | 1.60 | 1.20 | 35.10 | 2.00 | 11.20 | 4.40 | 58.70 | 0.0003 | 15.30 | Ryan et al. 2009 |
| D. magna | 21.01 | 25.00 | 7.95 | 2.40 | 3.30 | 3.10 | 35.00 | 2.00 | 22.00 | 4.30 | 60.00 | 0.0003 | 16.70 | Ryan et al. 2009 |
| D. magna | 40.20 | 25.00 | 7.98 | 2.10 | 6.70 | 5.70 | 35.40 | 2.00 | 41.20 | 4.10 | 60.00 | 0.0003 | 21.10 | Ryan et al. 2009 |
| D. magna | 9.26 | 25.00 | 7.95 | 8.50 | 1.40 | 1.40 | 56.40 | 2.00 | 18.70 | 12.80 | 66.70 | 0.0003 | 106.40 | Ryan et al. 2009 |
| D. magna | 20.93 | 25.00 | 7.96 | 8.40 | 3.60 | 2.90 | 56.20 | 2.00 | 29.40 | 12.90 | 62.70 | 0.0003 | 130.50 | Ryan et al. 2009 |
| D. magna | 45.08 | 25.00 | 7.98 | 8.60 | 7.50 | 6.40 | 56.60 | 2.00 | 52.00 | 13.10 | 62.70 | 0.0003 | 122.50 | Ryan et al. 2009 |
| D. magna | 10.17 | 25.00 | 6.19 | 2.60 | 1.60 | 1.50 | 11.70 | 2.00 | 12.90 | 4.60 | 5.30 | 0.0003 | 4.00 | Ryan et al. 2009 |
| D. magna | 20.02 | 25.00 | 6.23 | 2.60 | 3.40 | 2.80 | 11.90 | 2.00 | 22.20 | 4.70 | 5.30 | 0.0003 | 8.60 | Ryan et al. 2009 |
| D. magna | 42.02 | 25.00 | 6.29 | 2.60 | 7.10 | 5.90 | 11.70 | 2.00 | 43.00 | 4.60 | 5.30 | 0.0003 | 8.30 | Ryan et al. 2009 |
| D. magna | 9.51 | 25.00 | 6.30 | 8.70 | 1.50 | 1.40 | 33.00 | 2.00 | 18.90 | 13.40 | 6.70 | 0.0003 | 25.90 | Ryan et al. 2009 |
| D. magna | 21.51 | 25.00 | 6.28 | 9.00 | 3.50 | 3.10 | 34.10 | 2.00 | 31.10 | 13.50 | 5.30 | 0.0003 | 25.20 | Ryan et al. 2009 |
| D. magna | 42.60 | 25.00 | 6.42 | 8.70 | 7.00 | 6.10 | 33.20 | 2.00 | 50.50 | 13.50 | 5.30 | 0.0003 | 47.30 | Ryan et al. 2009 |
| D. magna | 8.69 | 25.00 | 7.84 | 2.20 | 1.50 | 1.20 | 34.50 | 2.00 | 11.00 | 4.10 | 58.70 | 0.0003 | 30.80 | Ryan et al. 2009 |
| D. magna | 20.43 | 25.00 | 7.86 | 2.60 | 3.40 | 2.90 | 35.80 | 2.00 | 22.40 | 4.50 | 57.30 | 0.0003 | 38.20 | Ryan et al. 2009 |
| D. magna | 43.92 | 25.00 | 7.87 | 2.60 | 7.20 | 6.30 | 35.80 | 2.00 | 44.90 | 4.60 | 60.00 | 0.0003 | 45.10 | Ryan et al. 2009 |
| D. magna | 10.83 | 25.00 | 7.87 | 8.90 | 1.70 | 1.60 | 57.90 | 2.00 | 20.50 | 13.40 | 61.30 | 0.0003 | 108.80 | Ryan et al. 2009 |
| D. magna | 22.17 | 25.00 | 7.94 | 8.30 | 3.60 | 3.20 | 55.80 | 2.00 | 30.20 | 12.90 | 60.00 | 0.0003 | 187.60 | Ryan et al. 2009 |
| D. magna | 41.44 | 25.00 | 7.98 | 8.80 | 6.70 | 6.00 | 57.10 | 2.00 | 49.60 | 13.30 | 61.30 | 0.0003 | 159.00 | Ryan et al. 2009 |
| D. magna | 230.14 | 20.00 | 8.56 | 0.91 | 66.24 | 15.72 | 52.47 | 1.82 | 141.20 | 27.30 | 140.00 | 0.0003 | 43.10 | Villavicencio et al. 2005 |
| D. magna | 117.04 | 20.00 | 8.16 | 0.71 | 35.08 | 7.15 | 13.83 | 3.02 | 82.90 | 6.70 | 59.00 | 0.0003 | 32.30 | Villavicencio et al. 2005 |
| D. magna | 114.83 | 20.00 | 8.36 | 1.07 | 37.23 | 5.31 | 10.10 | 1.34 | 45.70 | 4.40 | 86.00 | 0.0003 | 48.20 | Villavicencio et al. 2005 |
| D. magna | 64.65 | 20.00 | 8.24 | 0.79 | 21.95 | 2.39 | 5.52 | 0.75 | 17.90 | 2.30 | 66.00 | 0.0003 | 32.00 | Villavicencio et al. 2005 |
| D. magna | 60.17 | 20.00 | 8.17 | 0.76 | 19.43 | 2.83 | 6.87 | 0.79 | 24.70 | 3.50 | 54.00 | 0.0003 | 24.30 | Villavicencio et al. 2005 |
| D. magna | 140.24 | 20.00 | 8.62 | 0.98 | 35.91 | 12.28 | 11.90 | 1.26 | 13.20 | 7.20 | 154.00 | 0.0003 | 56.90 | Villavicencio et al. 2005 |
| D. magna | 109.90 | 20.00 | 8.43 | 0.98 | 32.65 | 6.89 | 9.31 | 1.46 | 36.20 | 2.80 | 97.00 | 0.0003 | 59.90 | Villavicencio et al. 2005 |
| D. magna | 118.26 | 20.00 | 8.45 | 1.67 | 34.91 | 7.55 | 11.66 | 0.91 | 30.80 | 4.00 | 107.00 | 0.0003 | 66.20 | Villavicencio et al. 2005 |
| D. magna | 83.19 | 20.00 | 8.15 | 0.38 | 27.79 | 3.35 | 4.30 | 0.65 | 42.70 | 5.50 | 54.00 | 0.0003 | 12.10 | Villavicencio et al. 2005 |
| D. magna | 59.50 | 20.00 | 8.03 | 0.31 | 18.22 | 3.40 | 7.02 | 0.56 | 30.30 | 5.30 | 42.00 | 0.0003 | 8.30 | Villavicencio et al. 2005 |
| D. magna | 71.21 | 20.00 | 7.94 | 0.31 | 22.68 | 3.54 | 6.14 | 0.83 | 45.00 | 5.90 | 37.00 | 0.0003 | 12.10 | Villavicencio et al. 2005 |
| D. magna | 36.41 | 20.00 | 8.11 | 1.18 | 6.40 | 4.96 | 5.55 | 1.41 | 8.30 | 1.50 | 51.00 | 0.0003 | 27.80 | Villavicencio et al. 2005 |
| D. magna | 22.75 | 20.00 | 7.90 | 0.30 | 5.45 | 2.22 | 4.46 | 1.11 | 9.50 | 0.90 | 31.00 | 0.0003 | 5.70 | Villavicencio et al. 2005 |
| D. magna | 89.19 | 20.00 | 7.83 | 0.73 | 33.41 | 1.40 | 3.09 | 0.61 | 8.30 | 0.80 | 21.00 | 0.0003 | 8.90 | Villavicencio et al. 2005 |
| D. magna | 80.61 | 20.00 | 8.39 | 0.07 | 18.10 | 8.60 | 10.55 | 2.16 | 18.80 | 1.90 | 76.00 | 0.0003 | 28.30 | Villavicencio et al. 2005 |
| D. magna | 29.70 | 20.00 | 8.12 | 0.14 | 7.54 | 2.64 | 5.13 | 1.07 | 9.00 | 1.50 | 41.00 | 0.0003 | 9.20 | Villavicencio et al. 2005 |
| D. magna | 15.93 | 20.00 | 7.80 | 0.72 | 4.30 | 1.26 | 2.68 | 0.67 | 7.10 | 0.50 | 23.00 | 0.0003 | 7.80 | Villavicencio et al. 2005 |
| D. magna | 14.90 | 20.00 | 7.65 | 0.60 | 3.79 | 1.32 | 2.47 | 0.80 | 7.10 | 0.30 | 23.00 | 0.0003 | 7.30 | Villavicencio et al. 2005 |
| D. magna | 20.88 | 20.00 | 7.83 | 0.38 | 6.07 | 1.39 | 4.27 | 0.68 | 11.20 | 0.60 | 24.00 | 0.0003 | 6.20 | Villavicencio et al. 2005 |
| D. magna | 10.14 | 20.00 | 7.50 | 0.34 | 2.74 | 0.80 | 2.11 | 0.54 | 6.80 | 0.30 | 14.00 | 0.0003 | 3.70 | Villavicencio et al. 2005 |
| D. magna | 25.05 | 20.00 | 7.81 | 0.25 | 6.65 | 2.05 | 5.55 | 0.94 | 11.50 | 2.00 | 27.00 | 0.0003 | 6.40 | Villavicencio et al. 2005 |
| D. magna | 17.00 | 20.00 | 7.83 | 0.44 | 4.17 | 1.60 | 3.21 | 0.85 | 9.40 | 1.20 | 27.00 | 0.0003 | 8.40 | Villavicencio et al. 2005 |
| D. magna | 8.64 | 20.00 | 7.57 | 0.53 | 2.42 | 0.63 | 2.23 | 0.35 | 8.30 | 1.00 | 14.00 | 0.0003 | 5.50 | Villavicencio et al. 2005 |
| D. magna | 7.47 | 20.00 | 7.41 | 0.81 | 2.07 | 0.56 | 2.07 | 0.28 | 8.50 | 1.20 | 12.00 | 0.0003 | 4.90 | Villavicencio et al. 2005 |
| D. magna | 21.96 | 20.00 | 7.86 | 0.44 | 4.92 | 2.35 | 9.55 | 0.92 | 9.70 | 9.20 | 29.00 | 0.0003 | 7.20 | Villavicencio et al. 2005 |
| D. magna | 21.86 | 20.00 | 7.83 | 0.60 | 4.78 | 2.41 | 5.60 | 1.00 | 10.10 | 2.20 | 31.00 | 0.0003 | 7.40 | Villavicencio et al. 2005 |
| D. magna | 20.25 | 20.00 | 7.86 | 0.45 | 5.11 | 1.82 | 4.61 | 0.78 | 10.10 | 1.50 | 29.00 | 0.0003 | 7.40 | Villavicencio et al. 2005 |
| D. magna | 14.43 | 20.00 | 7.78 | 0.37 | 3.95 | 1.11 | 3.40 | 0.46 | 3.70 | 4.90 | 16.00 | 0.0003 | 3.80 | Villavicencio et al. 2005 |
| D. magna | 11.07 | 20.00 | 7.55 | 3.27 | 2.14 | 1.39 | 4.20 | 0.70 | 1.40 | 5.30 | 16.00 | 0.0003 | 50.00 | Villavicencio et al. 2005 |
| D. magna | 7.92 | 20.00 | 7.21 | 4.10 | 1.39 | 1.08 | 5.66 | 0.61 | 8.00 | 7.60 | 10.00 | 0.0003 | 21.00 | Villavicencio et al. 2005 |
| D. magna | 7.09 | 20.00 | 7.13 | 7.28 | 1.34 | 0.91 | 3.84 | 0.60 | 7.50 | 5.70 | 10.00 | 0.0003 | 34.40 | Villavicencio et al. 2005 |
| D. magna | 37.33 | 20.00 | 7.00 | 9.01 | 3.19 | 7.13 | 55.90 | 2.45 | 17.30 | 100.10 | 8.00 | 0.0003 | 101.80 | Villavicencio et al. 2005 |
| D. magna | 11.63 | 20.00 | 7.43 | 8.47 | 2.30 | 1.43 | 6.38 | 0.81 | 8.40 | 8.00 | 15.00 | 0.0003 | 118.50 | Villavicencio et al. 2005 |
| D. magna | 10.69 | 20.00 | 7.45 | 9.40 | 2.04 | 1.36 | 4.48 | 0.60 | 8.10 | 4.30 | 14.00 | 0.0003 | 77.30 | Villavicencio et al. 2005 |
| D. magna | 13.78 | 20.00 | 7.70 | 0.96 | 3.59 | 1.17 | 3.60 | 0.51 | 3.20 | 5.30 | 16.00 | 0.0003 | 11.40 | Villavicencio et al. 2005 |
| D. magna | 13.09 | 20.00 | 7.65 | 1.59 | 3.23 | 1.22 | 3.70 | 0.56 | 2.80 | 5.60 | 16.00 | 0.0003 | 19.00 | Villavicencio et al. 2005 |
| D. magna | 12.41 | 20.00 | 7.64 | 2.25 | 2.86 | 1.28 | 3.90 | 0.60 | 2.30 | 5.90 | 16.00 | 0.0003 | 39.00 | Villavicencio et al. 2005 |
| D. magna | 11.72 | 20.00 | 7.62 | 2.75 | 2.50 | 1.33 | 4.00 | 0.65 | 1.90 | 4.90 | 16.00 | 0.0003 | 47.00 | Villavicencio et al. 2005 |
| D. magna | 42.77 | 20.00 | 7.90 | 0.10 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 0.93 | 28.10 | 0.0003 | 8.40 | Villavicencio et al. 2005 |
| D. magna | 42.77 | 20.00 | 7.84 | 1.10 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 0.93 | 28.00 | 0.0003 | 30.50 | Villavicencio et al. 2005 |
| D. magna | 42.77 | 20.00 | 7.85 | 2.10 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 0.93 | 28.02 | 0.0003 | 56.10 | Villavicencio et al. 2005 |
| D. magna | 42.77 | 20.00 | 7.87 | 3.10 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 0.93 | 28.06 | 0.0003 | 72.00 | Villavicencio et al. 2005 |
| D. magna | 42.77 | 20.00 | 7.91 | 4.20 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 4.30 | 28.12 | 0.0003 | 88.80 | Villavicencio et al. 2005 |
| D. magna | 42.77 | 20.00 | 7.87 | 5.20 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 4.60 | 28.06 | 0.0003 | 100.00 | Villavicencio et al. 2005 |
| D. magna | 85.29 | 20.00 | 8.20 | 0.10 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.33 | 0.0003 | 25.30 | Villavicencio et al. 2005 |
| D. magna | 85.29 | 20.00 | 8.20 | 0.87 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.33 | 0.0003 | 60.30 | Villavicencio et al. 2005 |
| D. magna | 85.29 | 20.00 | 8.22 | 1.78 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.34 | 0.0003 | 78.40 | Villavicencio et al. 2005 |
| D. magna | 85.29 | 20.00 | 8.23 | 2.69 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.35 | 0.0003 | 109.70 | Villavicencio et al. 2005 |
| D. magna | 85.29 | 20.00 | 8.12 | 3.63 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.24 | 0.0003 | 104.50 | Villavicencio et al. 2005 |
| D. magna | 85.29 | 20.00 | 8.11 | 4.65 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.22 | 0.0003 | 156.10 | Villavicencio et al. 2005 |
| D. magna | 167.03 | 20.00 | 8.36 | 0.10 | 28.30 | 23.40 | 52.80 | 4.40 | 140.90 | 6.00 | 111.06 | 0.0003 | 60.00 | Villavicencio et al. 2005 |
| D. magna | 167.03 | 20.00 | 8.45 | 0.84 | 28.30 | 23.40 | 52.80 | 4.40 | 140.90 | 6.00 | 110.99 | 0.0003 | 100.00 | Villavicencio et al. 2005 |
| D. magna | 167.03 | 20.00 | 8.46 | 1.57 | 28.30 | 23.40 | 52.80 | 4.40 | 140.90 | 6.00 | 110.97 | 0.0003 | 125.20 | Villavicencio et al. 2005 |
| D. magna | 167.03 | 20.00 | 8.46 | 2.33 | 28.30 | 23.40 | 52.80 | 4.40 | 140.90 | 6.00 | 110.97 | 0.0003 | 206.00 | Villavicencio et al. 2005 |
| D. magna | 167.03 | 20.00 | 8.45 | 3.25 | 28.30 | 23.40 | 52.80 | 4.40 | 140.90 | 0.93 | 110.99 | 0.0003 | 235.90 | Villavicencio et al. 2005 |
| D. magna | 167.03 | 20.00 | 8.39 | 3.94 | 28.30 | 23.40 | 52.80 | 4.40 | 140.90 | 0.93 | 111.05 | 0.0003 | 239.80 | Villavicencio et al. 2005 |
| D. magna | 110.61 | 20.00 | 8.32 | 1.00 | 31.60 | 7.70 | 7.20 | 1.50 | 68.20 | 4.20 | 63.00 | 0.0003 | 15.90 | Villavicencio et al. 2011 |
| D. magna | 121.96 | 20.00 | 8.36 | 0.80 | 34.00 | 9.00 | 13.10 | 1.10 | 59.50 | 10.20 | 98.00 | 0.0003 | 27.40 | Villavicencio et al. 2011 |
| D. magna | 199.92 | 20.00 | 8.59 | 0.70 | 45.10 | 21.20 | 20.60 | 0.90 | 81.00 | 27.90 | 235.00 | 0.0003 | 64.40 | Villavicencio et al. 2011 |
| D. magna | 109.28 | 20.00 | 8.26 | 1.00 | 34.20 | 5.80 | 9.90 | 1.40 | 64.50 | 9.60 | 84.00 | 0.0003 | 30.20 | Villavicencio et al. 2011 |
| D. magna | 70.23 | 20.00 | 8.51 | 0.80 | 21.20 | 4.20 | 6.90 | 0.50 | 34.30 | 2.50 | 59.00 | 0.0003 | 16.90 | Villavicencio et al. 2011 |
| D. magna | 261.32 | 20.00 | 8.53 | 0.60 | 81.40 | 14.10 | 25.70 | 1.90 | 313.00 | 43.60 | 83.00 | 0.0003 | 36.80 | Villavicencio et al. 2011 |
| D. magna | 439.08 | 20.00 | 8.58 | 0.40 | 142.20 | 20.40 | 109.20 | 3.00 | 248.50 | 275.20 | 89.00 | 0.0003 | 40.90 | Villavicencio et al. 2011 |
| D. magna | 75.64 | 20.00 | 7.89 | 0.30 | 26.50 | 2.30 | 10.20 | 1.20 | 48.10 | 12.50 | 29.00 | 0.0003 | 12.00 | Villavicencio et al. 2011 |
| D. magna | 93.74 | 20.00 | 8.02 | 1.30 | 22.70 | 9.00 | 11.80 | 2.10 | 45.00 | 11.90 | 43.00 | 0.0003 | 28.00 | Villavicencio et al. 2011 |
| D. magna | 31.53 | 20.00 | 8.00 | 1.00 | 9.00 | 2.20 | 6.60 | 0.80 | 13.00 | 5.30 | 26.00 | 0.0003 | 5.90 | Villavicencio et al. 2011 |
| D. magna | 14.12 | 20.00 | 7.76 | 0.80 | 4.50 | 0.70 | 1.60 | 0.20 | 4.40 | 0.20 | 17.00 | 0.0003 | 3.60 | Villavicencio et al. 2011 |
| D. magna | 18.43 | 20.00 | 7.99 | 0.50 | 5.40 | 1.20 | 2.90 | 0.40 | 7.60 | 1.30 | 20.00 | 0.0003 | 3.10 | Villavicencio et al. 2011 |
| D. magna | 10.45 | 20.00 | 7.90 | 1.10 | 2.70 | 0.90 | 2.40 | 0.60 | 1.20 | 1.30 | 15.00 | 0.0003 | 8.80 | Villavicencio et al. 2011 |
| D. magna | 11.93 | 20.00 | 7.99 | 1.10 | 2.80 | 1.20 | 2.40 | 0.80 | 3.10 | 1.20 | 19.00 | 0.0003 | 10.10 | Villavicencio et al. 2011 |
| D. magna | 12.86 | 20.00 | 7.97 | 1.80 | 3.50 | 1.00 | 2.80 | 1.10 | 2.20 | 1.10 | 25.00 | 0.0003 | 22.70 | Villavicencio et al. 2011 |
| D. magna | 13.86 | 20.00 | 7.99 | 1.20 | 3.90 | 1.00 | 2.00 | 0.50 | 2.50 | 0.70 | 18.00 | 0.0003 | 13.20 | Villavicencio et al. 2011 |
| D. magna | 25.72 | 20.00 | 8.00 | 0.60 | 7.00 | 2.00 | 4.50 | 0.70 | 6.00 | 5.00 | 28.00 | 0.0003 | 10.70 | Villavicencio et al. 2011 |
| D. magna | 12.11 | 20.00 | 7.97 | 0.70 | 3.20 | 1.00 | 2.20 | 0.40 | 1.50 | 1.10 | 21.00 | 0.0003 | 9.90 | Villavicencio et al. 2011 |
| D. magna | 70.07 | 20.00 | 8.35 | 0.20 | 15.20 | 7.80 | 40.00 | 1.90 | 20.10 | 56.00 | 68.00 | 0.0003 | 22.40 | Villavicencio et al. 2011 |
| D. magna | 32.83 | 20.00 | 8.18 | 0.50 | 8.20 | 3.00 | 6.70 | 1.40 | 5.30 | 2.10 | 45.00 | 0.0003 | 13.30 | Villavicencio et al. 2011 |
| D. magna | 198.95 | 21.00 | 7.57 | 11.12 | 77.13 | 1.47 | 50.47 | 0.46 | 55.00 | 33.00 | 142 | 0.0003 | 197.53 | Zhou et al.2014 |
| D. magna | 168.0917 | 21.00 | 8 | 20.12 | 64.47 | 1.66 | 126.70 | 0.47 | 52.00 | 57.60 | 219 | 0.0003 | 244.45 | Zhou et al.2014 |
| D. magna | 120.5583 | 21.00 | 7.98 | 11.95 | 46.54 | 1.01 | 70.01 | 0.59 | 63.00 | 50.80 | 69 | 0.0003 | 350 | Zhou et al.2014 |
| D. magna | 136.7333 | 21.00 | 7.56 | 10.06 | 53.01 | 1.01 | 58.54 | 0.60 | 55.00 | 44.00 | 79 | 0.0003 | 194.78 | Zhou et al.2014 |
| D. magna | 912.5417 | 21.00 | 7.98 | 4.99 | 66.35 | 179.2 | 35.31 | 0.51 | 76.00 | 43.80 | 193 | 0.0003 | 134.55 | Zhou et al.2014 |
| D. magna | 1007.308 | 21.00 | 8.07 | 10.84 | 78.59 | 194.6 | 64.93 | 0.53 | 90.00 | 72.40 | 258 | 0.0003 | 155.69 | Zhou et al.2014 |
| D. magna | 419.2917 | 21.00 | 7.72 | 7.31 | 104.9 | 37.69 | 128.40 | 14.45 | 67.00 | 66.20 | 87 | 0.0003 | 210.49 | Zhou et al.2014 |
| D. magna | 460.1667 | 21.00 | 7.86 | 6.44 | 111.8 | 43.36 | 140.90 | 15.46 | 66.00 | 54.00 | 98 | 0.0003 | 187.77 | Zhou et al.2014 |
| D. magna | 12.08333 | 21.00 | 8.4 | 1.9 | 4 | 0.5 | 0.75 | 0.35 | 0.10 | 2.00 | 48 | 0.0003 | 71.46 | Zhou et al.2014 |
| Daphnia obtusa | 230.14 | 20.00 | 8.56 | 0.91 | 66.24 | 15.72 | 52.47 | 1.82 | 141.20 | 27.30 | 140.00 | 0.0003 | 57.30 | Villavicencio et al. 2005 |
| D. obtusa | 117.04 | 20.00 | 8.16 | 0.71 | 35.08 | 7.15 | 13.83 | 3.02 | 82.90 | 6.70 | 59.00 | 0.0003 | 28.20 | Villavicencio et al. 2005 |
| D. obtusa | 114.83 | 20.00 | 8.36 | 1.07 | 37.23 | 5.31 | 10.10 | 1.34 | 45.70 | 4.40 | 86.00 | 0.0003 | 62.80 | Villavicencio et al. 2005 |
| D. obtusa | 64.65 | 20.00 | 8.24 | 0.79 | 21.95 | 2.39 | 5.52 | 0.75 | 17.90 | 2.30 | 66.00 | 0.0003 | 34.60 | Villavicencio et al. 2005 |
| D. obtusa | 60.17 | 20.00 | 8.17 | 0.76 | 19.43 | 2.83 | 6.87 | 0.79 | 24.70 | 3.50 | 54.00 | 0.0003 | 30.30 | Villavicencio et al. 2005 |
| D. obtusa | 140.24 | 20.00 | 8.62 | 0.98 | 35.91 | 12.28 | 11.90 | 1.26 | 13.20 | 7.20 | 154.00 | 0.0003 | 52.30 | Villavicencio et al. 2005 |
| D. obtusa | 109.90 | 20.00 | 8.43 | 0.98 | 32.65 | 6.89 | 9.31 | 1.46 | 36.20 | 2.80 | 97.00 | 0.0003 | 41.40 | Villavicencio et al. 2005 |
| D. obtusa | 118.26 | 20.00 | 8.45 | 1.67 | 34.91 | 7.55 | 11.66 | 0.91 | 30.80 | 4.00 | 107.00 | 0.0003 | 68.80 | Villavicencio et al. 2005 |
| D. obtusa | 83.19 | 20.00 | 8.15 | 0.38 | 27.79 | 3.35 | 4.30 | 0.65 | 42.70 | 5.50 | 54.00 | 0.0003 | 19.30 | Villavicencio et al. 2005 |
| D. obtusa | 59.50 | 20.00 | 8.03 | 0.31 | 18.22 | 3.40 | 7.02 | 0.56 | 30.30 | 5.30 | 42.00 | 0.0003 | 17.10 | Villavicencio et al. 2005 |
| D. obtusa | 71.21 | 20.00 | 7.94 | 0.31 | 22.68 | 3.54 | 6.14 | 0.83 | 45.00 | 5.90 | 37.00 | 0.0003 | 14.60 | Villavicencio et al. 2005 |
| D. obtusa | 104.75 | 20.00 | 7.90 | 0.17 | 35.09 | 4.16 | 7.95 | 1.52 | 62.40 | 13.10 | 47.00 | 0.0003 | 19.20 | Villavicencio et al. 2005 |
| D. obtusa | 53.62 | 20.00 | 7.50 | 0.33 | 17.40 | 2.47 | 2.95 | 0.35 | 30.10 | 0.30 | 21.00 | 0.0003 | 8.40 | Villavicencio et al. 2005 |
| D. obtusa | 68.98 | 20.00 | 7.60 | 0.55 | 20.93 | 4.06 | 6.44 | 0.75 | 40.50 | 0.70 | 31.00 | 0.0003 | 6.60 | Villavicencio et al. 2005 |
| D. obtusa | 36.41 | 20.00 | 8.11 | 1.18 | 6.40 | 4.96 | 5.55 | 1.41 | 8.30 | 1.50 | 51.00 | 0.0003 | 10.10 | Villavicencio et al. 2005 |
| D. obtusa | 22.75 | 20.00 | 7.90 | 0.30 | 5.45 | 2.22 | 4.46 | 1.11 | 9.50 | 0.90 | 31.00 | 0.0003 | 11.40 | Villavicencio et al. 2005 |
| D. obtusa | 89.19 | 20.00 | 7.83 | 0.73 | 33.41 | 1.40 | 3.09 | 0.61 | 8.30 | 0.80 | 21.00 | 0.0003 | 15.60 | Villavicencio et al. 2005 |
| D. obtusa | 80.61 | 20.00 | 8.39 | 0.07 | 18.10 | 8.60 | 10.55 | 2.16 | 18.80 | 1.90 | 76.00 | 0.0003 | 17.10 | Villavicencio et al. 2005 |
| D. obtusa | 29.70 | 20.00 | 8.12 | 0.14 | 7.54 | 2.64 | 5.13 | 1.07 | 9.00 | 1.50 | 41.00 | 0.0003 | 9.70 | Villavicencio et al. 2005 |
| D. obtusa | 15.93 | 20.00 | 7.80 | 0.72 | 4.30 | 1.26 | 2.68 | 0.67 | 7.10 | 0.50 | 23.00 | 0.0003 | 9.50 | Villavicencio et al. 2005 |
| D. obtusa | 14.90 | 20.00 | 7.65 | 0.60 | 3.79 | 1.32 | 2.47 | 0.80 | 7.10 | 0.30 | 23.00 | 0.0003 | 10.00 | Villavicencio et al. 2005 |
| D. obtusa | 20.88 | 20.00 | 7.83 | 0.38 | 6.07 | 1.39 | 4.27 | 0.68 | 11.20 | 0.60 | 24.00 | 0.0003 | 8.20 | Villavicencio et al. 2005 |
| D. obtusa | 10.14 | 20.00 | 7.50 | 0.34 | 2.74 | 0.80 | 2.11 | 0.54 | 6.80 | 0.30 | 14.00 | 0.0003 | 5.00 | Villavicencio et al. 2005 |
| D. obtusa | 25.05 | 20.00 | 7.81 | 0.25 | 6.65 | 2.05 | 5.55 | 0.94 | 11.50 | 2.00 | 27.00 | 0.0003 | 8.50 | Villavicencio et al. 2005 |
| D. obtusa | 17.00 | 20.00 | 7.83 | 0.44 | 4.17 | 1.60 | 3.21 | 0.85 | 9.40 | 1.20 | 27.00 | 0.0003 | 10.50 | Villavicencio et al. 2005 |
| D. obtusa | 8.64 | 20.00 | 7.57 | 0.53 | 2.42 | 0.63 | 2.23 | 0.35 | 8.30 | 1.00 | 14.00 | 0.0003 | 4.40 | Villavicencio et al. 2005 |
| D. obtusa | 7.47 | 20.00 | 7.41 | 0.81 | 2.07 | 0.56 | 2.07 | 0.28 | 8.50 | 1.20 | 12.00 | 0.0003 | 6.00 | Villavicencio et al. 2005 |
| D. obtusa | 21.96 | 20.00 | 7.86 | 0.44 | 4.92 | 2.35 | 9.55 | 0.92 | 9.70 | 9.20 | 29.00 | 0.0003 | 8.90 | Villavicencio et al. 2005 |
| D. obtusa | 21.86 | 20.00 | 7.83 | 0.60 | 4.78 | 2.41 | 5.60 | 1.00 | 10.10 | 2.20 | 31.00 | 0.0003 | 12.60 | Villavicencio et al. 2005 |
| D. obtusa | 20.25 | 20.00 | 7.86 | 0.45 | 5.11 | 1.82 | 4.61 | 0.78 | 10.10 | 1.50 | 29.00 | 0.0003 | 12.10 | Villavicencio et al. 2005 |
| D. obtusa | 14.43 | 20.00 | 7.78 | 0.37 | 3.95 | 1.11 | 3.40 | 0.46 | 3.70 | 4.90 | 16.00 | 0.0003 | 4.30 | Villavicencio et al. 2005 |
| D. obtusa | 11.07 | 20.00 | 7.55 | 3.27 | 2.14 | 1.39 | 4.20 | 0.70 | 1.40 | 5.30 | 16.00 | 0.0003 | 69.90 | Villavicencio et al. 2005 |
| D. obtusa | 7.92 | 20.00 | 7.21 | 4.10 | 1.39 | 1.08 | 5.66 | 0.61 | 8.00 | 7.60 | 10.00 | 0.0003 | 47.20 | Villavicencio et al. 2005 |
| D. obtusa | 7.09 | 20.00 | 7.13 | 7.28 | 1.34 | 0.91 | 3.84 | 0.60 | 7.50 | 5.70 | 10.00 | 0.0003 | 95.70 | Villavicencio et al. 2005 |
| D. obtusa | 37.33 | 20.00 | 7.00 | 9.01 | 3.19 | 7.13 | 55.90 | 2.45 | 17.30 | 100.10 | 8.00 | 0.0003 | 107.50 | Villavicencio et al. 2005 |
| D. obtusa | 11.63 | 20.00 | 7.43 | 8.47 | 2.30 | 1.43 | 6.38 | 0.81 | 8.40 | 8.00 | 15.00 | 0.0003 | 157.40 | Villavicencio et al. 2005 |
| D. obtusa | 10.69 | 20.00 | 7.45 | 9.40 | 2.04 | 1.36 | 4.48 | 0.60 | 8.10 | 4.30 | 14.00 | 0.0003 | 176.80 | Villavicencio et al. 2005 |
| D. obtusa | 13.78 | 20.00 | 7.70 | 0.96 | 3.59 | 1.17 | 3.60 | 0.51 | 3.20 | 5.30 | 16.00 | 0.0003 | 23.40 | Villavicencio et al. 2005 |
| D. obtusa | 13.09 | 20.00 | 7.65 | 1.59 | 3.23 | 1.22 | 3.70 | 0.56 | 2.80 | 5.60 | 16.00 | 0.0003 | 38.40 | Villavicencio et al. 2005 |
| D. obtusa | 12.41 | 20.00 | 7.64 | 2.25 | 2.86 | 1.28 | 3.90 | 0.60 | 2.30 | 5.90 | 16.00 | 0.0003 | 52.90 | Villavicencio et al. 2005 |
| D. obtusa | 11.72 | 20.00 | 7.62 | 2.75 | 2.50 | 1.33 | 4.00 | 0.65 | 1.90 | 4.90 | 16.00 | 0.0003 | 54.10 | Villavicencio et al. 2005 |
| D. obtusa | 14.43 | 20.00 | 7.42 | 3.50 | 3.95 | 1.11 | 3.40 | 0.46 | 3.70 | 4.90 | 16.00 | 0.0003 | 50.50 | Villavicencio et al. 2005 |
| D. obtusa | 42.77 | 20.00 | 7.77 | 0.10 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 0.93 | 27.86 | 0.0003 | 2.10 | Villavicencio et al. 2005 |
| D. obtusa | 42.77 | 20.00 | 7.87 | 1.00 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 0.93 | 28.06 | 0.0003 | 30.70 | Villavicencio et al. 2005 |
| D. obtusa | 42.77 | 20.00 | 7.91 | 2.00 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 0.93 | 28.12 | 0.0003 | 35.60 | Villavicencio et al. 2005 |
| D. obtusa | 42.77 | 20.00 | 7.92 | 3.00 | 7.40 | 5.90 | 13.60 | 1.10 | 38.60 | 0.93 | 28.13 | 0.0003 | 58.20 | Villavicencio et al. 2005 |
| D. obtusa | 85.29 | 20.00 | 8.11 | 0.10 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.22 | 0.0003 | 7.50 | Villavicencio et al. 2005 |
| D. obtusa | 85.29 | 20.00 | 7.95 | 0.83 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 57.89 | 0.0003 | 27.50 | Villavicencio et al. 2005 |
| D. obtusa | 85.29 | 20.00 | 8.12 | 1.66 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.24 | 0.0003 | 41.10 | Villavicencio et al. 2005 |
| D. obtusa | 85.29 | 20.00 | 8.16 | 2.39 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.29 | 0.0003 | 69.90 | Villavicencio et al. 2005 |
| D. obtusa | 85.29 | 20.00 | 8.15 | 3.23 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.27 | 0.0003 | 100.10 | Villavicencio et al. 2005 |
| D. obtusa | 85.29 | 20.00 | 8.15 | 4.12 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.27 | 0.0003 | 131.00 | Villavicencio et al. 2005 |
| D. obtusa | 85.29 | 20.00 | 8.21 | 0.10 | 14.20 | 12.10 | 27.30 | 2.10 | 80.70 | 3.19 | 58.34 | 0.0003 | 7.70 | Villavicencio et al. 2005 |
| Daphnia pulex | 230.14 | 20.00 | 8.56 | 0.91 | 66.24 | 15.72 | 52.47 | 1.82 | 141.20 | 27.30 | 140.00 | 0.0003 | 44.50 | Villavicencio et al. 2005 |
| D. pulex | 117.04 | 20.00 | 8.16 | 0.71 | 35.08 | 7.15 | 13.83 | 3.02 | 82.90 | 6.70 | 59.00 | 0.0003 | 20.00 | Villavicencio et al. 2005 |
| D. pulex | 114.83 | 20.00 | 8.36 | 1.07 | 37.23 | 5.31 | 10.10 | 1.34 | 45.70 | 4.40 | 86.00 | 0.0003 | 52.50 | Villavicencio et al. 2005 |
| D. pulex | 64.65 | 20.00 | 8.24 | 0.79 | 21.95 | 2.39 | 5.52 | 0.75 | 17.90 | 2.30 | 66.00 | 0.0003 | 26.00 | Villavicencio et al. 2005 |
| D. pulex | 60.17 | 20.00 | 8.17 | 0.76 | 19.43 | 2.83 | 6.87 | 0.79 | 24.70 | 3.50 | 54.00 | 0.0003 | 21.60 | Villavicencio et al. 2005 |
| D. pulex | 140.24 | 20.00 | 8.62 | 0.98 | 35.91 | 12.28 | 11.90 | 1.26 | 13.20 | 7.20 | 154.00 | 0.0003 | 49.00 | Villavicencio et al. 2005 |
| D. pulex | 109.90 | 20.00 | 8.43 | 0.98 | 32.65 | 6.89 | 9.31 | 1.46 | 36.20 | 2.80 | 97.00 | 0.0003 | 44.20 | Villavicencio et al. 2005 |
| D. pulex | 118.26 | 20.00 | 8.45 | 1.67 | 34.91 | 7.55 | 11.66 | 0.91 | 30.80 | 4.00 | 107.00 | 0.0003 | 51.70 | Villavicencio et al. 2005 |
| D. pulex | 83.19 | 20.00 | 8.15 | 0.38 | 27.79 | 3.35 | 4.30 | 0.65 | 42.70 | 5.50 | 54.00 | 0.0003 | 8.10 | Villavicencio et al. 2005 |
| D. pulex | 59.50 | 20.00 | 8.03 | 0.31 | 18.22 | 3.40 | 7.02 | 0.56 | 30.30 | 5.30 | 42.00 | 0.0003 | 9.70 | Villavicencio et al. 2005 |
| D. pulex | 36.41 | 20.00 | 8.11 | 1.18 | 6.40 | 4.96 | 5.55 | 1.41 | 8.30 | 1.50 | 51.00 | 0.0003 | 18.10 | Villavicencio et al. 2005 |
| D. pulex | 22.75 | 20.00 | 7.90 | 0.30 | 5.45 | 2.22 | 4.46 | 1.11 | 9.50 | 0.90 | 31.00 | 0.0003 | 5.00 | Villavicencio et al. 2005 |
| D. pulex | 89.19 | 20.00 | 7.83 | 0.73 | 33.41 | 1.40 | 3.09 | 0.61 | 8.30 | 0.80 | 21.00 | 0.0003 | 5.90 | Villavicencio et al. 2005 |
| D. pulex | 80.61 | 20.00 | 8.39 | 0.07 | 18.10 | 8.60 | 10.55 | 2.16 | 18.80 | 1.90 | 76.00 | 0.0003 | 19.50 | Villavicencio et al. 2005 |
| D. pulex | 29.70 | 20.00 | 8.12 | 0.14 | 7.54 | 2.64 | 5.13 | 1.07 | 9.00 | 1.50 | 41.00 | 0.0003 | 5.40 | Villavicencio et al. 2005 |
| D. pulex | 15.93 | 20.00 | 7.80 | 0.72 | 4.30 | 1.26 | 2.68 | 0.67 | 7.10 | 0.50 | 23.00 | 0.0003 | 6.10 | Villavicencio et al. 2005 |
| D. pulex | 14.90 | 20.00 | 7.65 | 0.60 | 3.79 | 1.32 | 2.47 | 0.80 | 7.10 | 0.30 | 23.00 | 0.0003 | 6.40 | Villavicencio et al. 2005 |
| D. pulex | 20.88 | 20.00 | 7.83 | 0.38 | 6.07 | 1.39 | 4.27 | 0.68 | 11.20 | 0.60 | 24.00 | 0.0003 | 5.40 | Villavicencio et al. 2005 |
| D. pulex | 10.14 | 20.00 | 7.50 | 0.34 | 2.74 | 0.80 | 2.11 | 0.54 | 6.80 | 0.30 | 14.00 | 0.0003 | 3.30 | Villavicencio et al. 2005 |
| D. pulex | 25.05 | 20.00 | 7.81 | 0.25 | 6.65 | 2.05 | 5.55 | 0.94 | 11.50 | 2.00 | 27.00 | 0.0003 | 4.80 | Villavicencio et al. 2005 |
| D. pulex | 17.00 | 20.00 | 7.83 | 0.44 | 4.17 | 1.60 | 3.21 | 0.85 | 9.40 | 1.20 | 27.00 | 0.0003 | 4.00 | Villavicencio et al. 2005 |
| D. pulex | 8.64 | 20.00 | 7.57 | 0.53 | 2.42 | 0.63 | 2.23 | 0.35 | 8.30 | 1.00 | 14.00 | 0.0003 | 3.10 | Villavicencio et al. 2005 |
| D. pulex | 7.47 | 20.00 | 7.41 | 0.81 | 2.07 | 0.56 | 2.07 | 0.28 | 8.50 | 1.20 | 12.00 | 0.0003 | 3.40 | Villavicencio et al. 2005 |
| D. pulex | 21.96 | 20.00 | 7.86 | 0.44 | 4.92 | 2.35 | 9.55 | 0.92 | 9.70 | 9.20 | 29.00 | 0.0003 | 3.60 | Villavicencio et al. 2005 |
| D. pulex | 21.86 | 20.00 | 7.83 | 0.60 | 4.78 | 2.41 | 5.60 | 1.00 | 10.10 | 2.20 | 31.00 | 0.0003 | 5.20 | Villavicencio et al. 2005 |
| D. pulex | 20.25 | 20.00 | 7.86 | 0.45 | 5.11 | 1.82 | 4.61 | 0.78 | 10.10 | 1.50 | 29.00 | 0.0003 | 4.10 | Villavicencio et al. 2005 |
| D. pulex | 7.92 | 20.00 | 7.21 | 4.10 | 1.39 | 1.08 | 5.66 | 0.61 | 8.00 | 7.60 | 10.00 | 0.0003 | 30.60 | Villavicencio et al. 2005 |
| D. pulex | 7.09 | 20.00 | 7.13 | 7.28 | 1.34 | 0.91 | 3.84 | 0.60 | 7.50 | 5.70 | 10.00 | 0.0003 | 37.80 | Villavicencio et al. 2005 |
| D. pulex | 37.33 | 20.00 | 7.00 | 9.01 | 3.19 | 7.13 | 55.90 | 2.45 | 17.30 | 100.10 | 8.00 | 0.0003 | 65.60 | Villavicencio et al. 2005 |
| D. pulex | 11.63 | 20.00 | 7.43 | 8.47 | 2.30 | 1.43 | 6.38 | 0.81 | 8.40 | 8.00 | 15.00 | 0.0003 | 71.60 | Villavicencio et al. 2005 |
| D. pulex | 10.69 | 20.00 | 7.45 | 9.40 | 2.04 | 1.36 | 4.48 | 0.60 | 8.10 | 4.30 | 14.00 | 0.0003 | 86.20 | Villavicencio et al. 2005 |
| Daphnia pulicaria | 48.00 | 18.00 | 8.03 | 2.29 | 14.11 | 3.11 | 1.36 | 0.57 | 3.55 | 1.25 | 42.00 | 0.0003 | 10.94 | Lind et al. 1978 |
| D. pulicaria | 48.00 | 18.00 | 8.03 | 2.82 | 14.11 | 3.11 | 1.36 | 0.57 | 3.55 | 1.25 | 42.00 | 0.0003 | 8.70 | Lind et al. 1978 |
| D. pulicaria | 48.00 | 18.00 | 8.01 | 2.73 | 14.11 | 3.11 | 1.36 | 0.57 | 3.55 | 1.25 | 44.00 | 0.0003 | 6.95 | Lind et al. 1978 |
| D. pulicaria | 44.00 | 18.00 | 8.04 | 3.08 | 12.93 | 2.85 | 1.24 | 0.57 | 3.25 | 1.15 | 42.00 | 0.0003 | 10.37 | Lind et al. 1978 |
| D. pulicaria | 31.00 | 18.00 | 6.66 | 12.21 | 7.37 | 3.06 | 1.68 | 0.50 | 6.33 | 1.29 | 27.00 | 0.0003 | 53.18 | Lind et al. 1978 |
| D. pulicaria | 29.00 | 18.00 | 6.97 | 11.34 | 6.90 | 2.87 | 1.57 | 0.50 | 5.92 | 1.21 | 27.00 | 0.0003 | 53.09 | Lind et al. 1978 |
| D. pulicaria | 28.00 | 18.00 | 7.20 | 11.34 | 6.66 | 2.77 | 1.52 | 0.50 | 5.72 | 1.17 | 22.00 | 0.0003 | 51.17 | Lind et al. 1978 |
| D. pulicaria | 88.00 | 18.00 | 7.01 | 24.42 | 20.95 | 8.52 | 16.47 | 1.88 | 22.63 | 18.99 | 20.00 | 0.0003 | 93.31 | Lind et al. 1978 |
| D. pulicaria | 100.00 | 18.00 | 7.55 | 29.65 | 23.93 | 9.47 | 21.21 | 2.16 | 25.98 | 23.28 | 20.00 | 0.0003 | 191.04 | Lind et al. 1978 |
| D. pulicaria | 82.00 | 18.00 | 6.99 | 27.91 | 19.45 | 8.04 | 14.10 | 1.74 | 20.95 | 16.84 | 18.00 | 0.0003 | 204.48 | Lind et al. 1978 |
| D. pulicaria | 84.00 | 18.00 | 7.01 | 27.91 | 19.95 | 8.20 | 14.89 | 1.78 | 21.51 | 17.56 | 17.00 | 0.0003 | 158.40 | Lind et al. 1978 |
| D. pulicaria | 16.00 | 18.00 | 7.39 | 11.61 | 4.14 | 1.38 | 0.16 | 0.30 | 6.72 | 0.32 | 11.00 | 0.0003 | 34.08 | Lind et al. 1978 |
| D. pulicaria | 151.00 | 18.00 | 7.76 | 12.58 | 36.79 | 14.40 | 10.79 | 1.40 | 62.02 | 19.68 | 44.00 | 0.0003 | 75.65 | Lind et al. 1978 |
| D. pulicaria | 96.00 | 18.00 | 8.10 | 27.10 | 22.09 | 9.94 | 6.86 | 1.40 | 19.91 | 4.27 | 91.00 | 0.0003 | 108.48 | Lind et al. 1978 |
| D. pulicaria | 26.00 | 18.00 | 7.24 | 24.19 | 7.38 | 1.84 | 0.26 | 0.30 | 11.62 | 2.60 | 4.00 | 0.0003 | 73.34 | Lind et al. 1978 |
| D. pulicaria | 84.00 | 18.00 | 7.08 | 12.58 | 20.46 | 8.01 | 6.00 | 1.40 | 34.50 | 10.95 | 13.00 | 0.0003 | 81.31 | Lind et al. 1978 |
| D. pulicaria | 92.00 | 18.00 | 7.22 | 20.32 | 22.41 | 8.77 | 6.57 | 1.40 | 37.79 | 11.99 | 19.00 | 0.0003 | 176.64 | Lind et al. 1978 |
| D. pulicaria | 47.00 | 18.00 | 8.03 | 2.73 | 13.81 | 3.05 | 1.33 | 0.57 | 3.47 | 1.23 | 42.50 | 0.0003 | 8.93 | Lind et al. 1978 |
| D. pulicaria | 97.00 | 18.00 | 8.03 | 2.73 | 34.00 | 2.90 | 1.30 | 0.57 | 51.30 | 1.20 | 42.50 | 0.0003 | 17.09 | Lind et al. 1978 |
| D. pulicaria | 147.00 | 18.00 | 8.03 | 2.73 | 54.00 | 2.90 | 1.30 | 0.57 | 99.30 | 1.20 | 42.50 | 0.0003 | 22.75 | Lind et al. 1978 |
| D. pulicaria | 247.00 | 18.00 | 8.03 | 2.73 | 94.00 | 2.90 | 1.30 | 0.57 | 147.30 | 1.20 | 42.50 | 0.0003 | 26.21 | Lind et al. 1978 |
| D. pulicaria | 97.00 | 18.00 | 8.03 | 2.73 | 13.60 | 15.20 | 1.30 | 0.57 | 51.30 | 1.20 | 42.50 | 0.0003 | 24.19 | Lind et al. 1978 |
| D. pulicaria | 147.00 | 18.00 | 8.03 | 2.73 | 13.60 | 27.50 | 1.30 | 0.57 | 99.30 | 1.20 | 42.50 | 0.0003 | 24.10 | Lind et al. 1978 |
| D. pulicaria | 247.00 | 18.00 | 8.03 | 2.73 | 13.60 | 51.90 | 1.30 | 0.57 | 147.30 | 1.20 | 42.50 | 0.0003 | 24.10 | Lind et al. 1978 |
| Etheostoma flabellare | 170.00 | 20.00 | 7.80 | 0.50 | 27.90 | 24.20 | 52.50 | 4.20 | 163.00 | 3.80 | 115.00 | 0.0003 | 316.80 | Lydy and Wissing 1988 |
| E. flabellare | 170.00 | 20.00 | 7.80 | 0.50 | 27.90 | 24.20 | 52.50 | 4.20 | 163.00 | 3.80 | 115.00 | 0.0003 | 327.36 | Lydy and Wissing 1988 |
| E. flabellare | 170.00 | 20.00 | 7.90 | 0.50 | 27.90 | 24.20 | 52.50 | 4.20 | 163.00 | 3.80 | 115.00 | 0.0003 | 358.08 | Lydy and Wissing 1988 |
| E. flabellare | 170.00 | 20.00 | 7.80 | 0.50 | 27.90 | 24.20 | 52.50 | 4.20 | 163.00 | 3.80 | 115.00 | 0.0003 | 376.32 | Lydy and Wissing 1988 |
| Etheostoma lepidum | 167.00 | 22.00 | 8.00 | 0.50 | 27.56 | 23.88 | 51.72 | 4.13 | 160.32 | 3.75 | 115.00 | 0.0003 | 249.60 | Dwyer et al. 1999 |
| E. nigrum | 170.00 | 20.00 | 7.80 | 0.50 | 27.90 | 24.20 | 52.50 | 4.20 | 163.00 | 3.80 | 115.00 | 0.0003 | 473.28 | Lydy and Wissing 1988 |
| E. nigrum | 170.00 | 20.00 | 7.80 | 0.50 | 27.90 | 24.20 | 52.50 | 4.20 | 163.00 | 3.80 | 115.00 | 0.0003 | 463.68 | Lydy and Wissing 1988 |
| E. nigrum | 170.00 | 20.00 | 7.80 | 0.50 | 27.90 | 24.20 | 52.50 | 4.20 | 163.00 | 3.80 | 115.00 | 0.0003 | 577.92 | Lydy and Wissing 1988 |
| E. nigrum | 170.00 | 20.00 | 7.80 | 0.50 | 27.90 | 24.20 | 52.50 | 4.20 | 163.00 | 3.80 | 115.00 | 0.0003 | 526.08 | Lydy and Wissing 1988 |
| Etheostoma rubrum | 167.00 | 22.00 | 8.20 | 0.50 | 27.56 | 23.88 | 51.72 | 4.13 | 160.32 | 3.75 | 115.00 | 0.0003 | 57.60 | Dwyer et al. 1999 |
| Gammarus pseudolimnaeus | 44.90 | 15.00 | 7.70 | 1.10 | 13.20 | 2.91 | 1.27 | 0.57 | 3.32 | 1.17 | 42.70 | 0.0003 | 21.12 | Arthur and Leonard 1970 |
| G. pseudolimnaeus | 44.90 | 15.00 | 7.70 | 1.10 | 13.20 | 2.91 | 1.27 | 0.57 | 3.32 | 1.17 | 42.70 | 0.0003 | 18.24 | Arthur and Leonard 1970 |
| Gila elegans | 173.00 | 22.00 | 8.05 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 192.00 | Dwyer et al. 1995 |
| Hyalella azteca | 290.00 | 25.00 | 6.23 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 16.32 | Schubauer-Berigan et al. 1993 |
| H. azteca | 290.00 | 25.00 | 7.51 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 23.04 | Schubauer-Berigan et al. 1993 |
| H. azteca | 290.00 | 25.00 | 8.38 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 83.52 | Schubauer-Berigan et al. 1993 |
| H. azteca | 20.50 | 21.00 | 7.15 | 2.80 | 5.10 | 1.90 | 5.30 | 0.80 | 9.30 | 10.00 | 6.70 | 0.0003 | 23.33 | Welsh 1996 |
| H. azteca | 20.50 | 21.00 | 7.15 | 2.80 | 5.10 | 1.90 | 5.30 | 0.80 | 9.30 | 10.00 | 6.70 | 0.0003 | 22.85 | Welsh 1996 |
| H. azteca | 20.60 | 21.00 | 7.14 | 0.50 | 5.30 | 1.80 | 5.50 | 0.80 | 7.00 | 9.70 | 11.00 | 0.0003 | 7.87 | Welsh 1996 |
| Hyalella azteca | 20.60 | 21.00 | 7.14 | 0.50 | 5.30 | 1.80 | 5.50 | 0.80 | 7.00 | 9.70 | 11.00 | 0.0003 | 9.60 | Welsh 1996 |
| Hypophthalmichthys molitrix | 10.66667 | 24.50 | 7.40 | 0.50 | 1.75 | 1.51 | 3.28 | 0.26 | 10.20 | 0.24 |  | 0.0003 | 39.00 | Wang 2015a |
| H. molitrix | 44.975 | 24.50 | 7.40 | 0.50 | 6.99 | 6.60 | 13.10 | 1.50 | 40.70 | 0.95 |  | 0.0003 | 67.00 | Wang 2015a |
| H. molitrix | 144.1 | 24.50 | 7.40 | 0.50 | 23.14 | 20.70 | 43.54 | 3.48 | 135.19 | 3.15 |  | 0.0003 | 307.00 | Wang 2015a |
| H. molitrix | 255.875 | 24.50 | 7.40 | 0.50 | 41.85 | 36.30 | 78.75 | 6.30 | 244.50 | 5.70 |  | 0.0003 | 460.00 | Wang 2015a |
| H. molitrix | 10.66667 | 26.00 | 7.54 | 1.60 | 1.75 | 1.51 | 3.28 | 0.26 | 10.20 | 0.24 |  | 0.0003 | 180.00 | Wang 2015a |
| Juga plicifera | 21.00 | 15.00 | 7.20 | 1.10 | 6.06 | 1.75 | 4.53 | 0.70 | 2.87 | 5.47 | 26.00 | 0.0003 | 14.40 | Nebeker et al. 1986b |
| L. siliquoidea | 138.00 | 19.40 | 8.40 | 0.90 | 32.00 | 12.00 | 20.00 | 3.10 | 18.00 | 24.00 | 123.00 | 0.0003 | 27.00 | Wang et al. 2009 |
| L. siliquoidea | 139.00 | 19.40 | 8.40 | 3.60 | 32.00 | 12.00 | 20.00 | 3.10 | 18.00 | 24.00 | 126.00 | 0.0003 | 56.00 | Wang et al. 2009 |
| L. siliquoidea | 141.00 | 19.40 | 8.40 | 5.60 | 32.00 | 12.00 | 20.00 | 3.10 | 18.00 | 24.00 | 132.00 | 0.0003 | 56.00 | Wang et al. 2009 |
| L. siliquoidea | 140.00 | 19.40 | 8.40 | 10.80 | 32.00 | 12.00 | 20.00 | 3.10 | 18.00 | 24.00 | 144.00 | 0.0003 | 127.00 | Wang et al. 2009 |
| L. siliquoidea | 188.00 | 19.50 | 8.70 | 0.90 | 51.00 | 17.00 | 178.00 | 35.00 | 189.00 | 296.00 | 163.00 | 0.0003 | 53.00 | Wang et al. 2009 |
| L. siliquoidea | 192.00 | 19.50 | 8.60 | 3.10 | 51.00 | 17.00 | 178.00 | 35.00 | 189.00 | 296.00 | 168.00 | 0.0003 | 113.00 | Wang et al. 2009 |
| L. siliquoidea | 188.00 | 19.50 | 8.60 | 5.70 | 51.00 | 17.00 | 178.00 | 35.00 | 189.00 | 296.00 | 165.00 | 0.0003 | 149.00 | Wang et al. 2009 |
| L. siliquoidea | 186.00 | 19.50 | 8.60 | 9.50 | 51.00 | 17.00 | 178.00 | 35.00 | 189.00 | 296.00 | 160.00 | 0.0003 | 298.00 | Wang et al. 2009 |
| L. siliquoidea | 44.00 | 19.90 | 8.10 | 0.80 | 18.00 | 4.30 | 3.30 | 2.10 | 14.70 | 5.00 | 41.00 | 0.0003 | 9.90 | Wang et al. 2009 |
| L. siliquoidea | 42.00 | 19.90 | 8.00 | 2.80 | 18.00 | 4.30 | 3.30 | 2.10 | 14.70 | 5.00 | 42.00 | 0.0003 | 58.00 | Wang et al. 2009 |
| L. siliquoidea | 45.00 | 19.90 | 8.00 | 5.30 | 18.00 | 4.30 | 3.30 | 2.10 | 14.70 | 5.00 | 40.00 | 0.0003 | 75.00 | Wang et al. 2009 |
| L. siliquoidea | 42.00 | 19.90 | 8.10 | 9.80 | 18.00 | 4.30 | 3.30 | 2.10 | 14.70 | 5.00 | 41.00 | 0.0003 | 144.00 | Wang et al. 2009 |
| L. siliquoidea | 215.00 | 19.40 | 8.50 | 0.30 | 76.00 | 40.00 | 341.00 | 42.00 | 192.00 | 5.70 | 133.00 | 0.0003 | 44.00 | Wang et al. 2009 |
| L. siliquoidea | 199.00 | 19.40 | 8.50 | 2.90 | 76.00 | 40.00 | 341.00 | 42.00 | 192.00 | 5.70 | 130.00 | 0.0003 | 114.00 | Wang et al. 2009 |
| L. siliquoidea | 215.00 | 19.40 | 8.50 | 5.90 | 76.00 | 40.00 | 341.00 | 42.00 | 192.00 | 5.70 | 134.00 | 0.0003 | 186.00 | Wang et al. 2009 |
| L. siliquoidea | 216.00 | 19.40 | 8.50 | 10.50 | 76.00 | 40.00 | 341.00 | 42.00 | 192.00 | 5.70 | 129.00 | 0.0003 | 396.00 | Wang et al. 2009 |
| L. siliquoidea | 164.00 | 20.40 | 8.60 | 0.30 | 31.00 | 21.00 | 51.00 | 4.50 | 188.00 | 6.20 | 126.00 | 0.0003 | 24.00 | Wang et al. 2009 |
| L. siliquoidea | 166.00 | 20.40 | 8.50 | 1.30 | 31.00 | 21.00 | 51.00 | 4.50 | 188.00 | 6.20 | 123.00 | 0.0003 | 66.00 | Wang et al. 2009 |
| L. siliquoidea | 166.00 | 20.40 | 8.50 | 1.60 | 31.00 | 21.00 | 51.00 | 4.50 | 188.00 | 6.20 | 123.00 | 0.0003 | 72.00 | Wang et al. 2009 |
| L. siliquoidea | 167.00 | 20.40 | 8.50 | 2.30 | 31.00 | 21.00 | 51.00 | 4.50 | 188.00 | 6.20 | 124.00 | 0.0003 | 133.00 | Wang et al. 2009 |
| L. siliquoidea | 168.00 | 20.40 | 8.50 | 3.50 | 31.00 | 21.00 | 51.00 | 4.50 | 188.00 | 6.20 | 123.00 | 0.0003 | 196.00 | Wang et al. 2009 |
| L. siliquoidea | 52.00 | 20.90 | 8.10 | 0.30 | 7.00 | 6.10 | 13.20 | 1.00 | 40.70 | 1.00 | 52.00 | 0.0003 | 21.00 | Wang et al. 2009 |
| L. siliquoidea | 87.00 | 20.40 | 8.20 | 0.30 | 14.00 | 12.10 | 26.30 | 2.10 | 81.40 | 1.90 | 68.00 | 0.0003 | 30.00 | Wang et al. 2009 |
| L. siliquoidea | 176.00 | 20.40 | 8.60 | 0.30 | 28.00 | 24.20 | 52.60 | 4.20 | 162.70 | 3.80 | 129.00 | 0.0003 | 26.00 | Wang et al. 2009 |
| L. siliquoidea | 297.00 | 20.40 | 8.80 | 0.30 | 56.00 | 48.50 | 105.20 | 8.40 | 325.40 | 7.60 | 208.00 | 0.0003 | 30.00 | Wang et al. 2009 |
| L. siliquoidea | 175.00 | 19.40 | 8.40 | 0.30 | 31.00 | 21.00 | 51.00 | 4.50 | 188.00 | 6.20 | 128.00 | 0.0003 | 31.00 | Wang et al. 2009 |
| L. siliquoidea | 180.00 | 19.50 | 8.60 | 0.30 | 31.00 | 21.00 | 51.00 | 4.50 | 188.00 | 6.20 | 127.00 | 0.0003 | 43.00 | Wang et al. 2009 |
| L. siliquoidea | 173.00 | 19.90 | 8.60 | 0.30 | 31.00 | 21.00 | 51.00 | 4.50 | 188.00 | 6.20 | 125.00 | 0.0003 | 39.00 | Wang et al. 2009 |
| L. macrochirus | 45.00 | 20.00 | 7.50 | 1.10 | 13.23 | 2.92 | 1.30 | 0.57 | 3.40 | 1.20 | 43.00 | 0.0003 | 1056.00 | Benoit 1975 |
| L. macrochirus | 85.00 | 20.20 | 7.30 | 1.10 | 23.90 | 6.50 | 0.64 | 0.46 | 4.32 | 1.50 | 82.00 | 0.0003 | 2200.00 | Blaylock et al. 1985 |
| L. macrochirus | 85.00 | 21.85 | 7.45 | 1.10 | 23.90 | 6.50 | 0.64 | 0.46 | 4.32 | 1.50 | 82.00 | 0.0003 | 1300.00 | Blaylock et al. 1985 |
| L. macrochirus | 25.90 | 19.00 | 7.03 | 1.50 | 6.39 | 2.42 | 5.47 | 1.60 | 26.49 | 19.43 | 27.10 | 0.0003 | 960.00 | Cairns et al. 1981 |
| Lithoglyphus virens | 21.00 | 15.00 | 7.20 | 1.10 | 6.06 | 1.75 | 4.53 | 0.70 | 2.87 | 5.47 | 26.00 | 0.0003 | 7.68 | Nebeker et al. 1986b |
| L. variegatus | 290.00 | 25.00 | 6.57 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 124.80 | Schubauer-Berigan et al. 1993 |
| L. variegatus | 290.00 | 25.00 | 7.29 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 259.20 | Schubauer-Berigan et al. 1993 |
| L. variegatus | 290.00 | 25.00 | 8.25 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 480.00 | Schubauer-Berigan et al. 1993 |
| Notemigonus crysoleucas | 72.20 | 25.00 | 7.50 | 1.50 | 17.81 | 6.75 | 15.26 | 1.60 | 73.84 | 54.15 | 42.50 | 0.0003 | 81216.00 | Hartwell et al. 1989 |
| Oncorhynchus apache | 169.00 | 12.00 | 8.00 | 0.50 | 27.89 | 24.17 | 52.34 | 4.18 | 162.24 | 3.79 | 117.00 | 0.0003 | 67.20 | Dwyer et al. 1995 |
| O. clarki | 205.00 | 13.70 | 7.73 | 3.30 | 49.80 | 19.60 | 4.00 | 0.64 | 10.00 | 0.44 | 178.00 | 0.0003 | 367.00 | Chakoumakos et al. 1979 |
| O. clarki | 69.90 | 13.70 | 8.54 | 1.50 | 18.40 | 5.80 | 1.41 | 0.22 | 3.51 | 0.15 | 174.00 | 0.0003 | 186.00 | Chakoumakos et al. 1979 |
| O. clarki | 18.00 | 13.70 | 8.07 | 0.75 | 4.80 | 1.50 | 0.36 | 0.06 | 0.90 | 0.04 | 183.00 | 0.0003 | 36.80 | Chakoumakos et al. 1979 |
| O. clarki | 204.00 | 13.70 | 7.61 | 3.30 | 64.70 | 10.30 | 4.10 | 0.66 | 10.25 | 0.45 | 77.90 | 0.0003 | 232.00 | Chakoumakos et al. 1979 |
| O. clarki | 83.00 | 13.70 | 7.40 | 1.70 | 20.40 | 7.80 | 1.67 | 0.27 | 4.17 | 0.18 | 70.00 | 0.0003 | 162.00 | Chakoumakos et al. 1979 |
| O. clarki | 31.40 | 13.70 | 8.32 | 0.94 | 7.90 | 2.70 | 0.63 | 0.10 | 1.58 | 0.07 | 78.30 | 0.0003 | 73.60 | Chakoumakos et al. 1979 |
| O. clarki | 160.00 | 13.70 | 7.53 | 2.80 | 57.50 | 4.00 | 3.22 | 0.51 | 8.04 | 0.35 | 26.00 | 0.0003 | 91.00 | Chakoumakos et al. 1979 |
| O. clarki | 74.30 | 13.70 | 7.57 | 1.50 | 24.70 | 3.10 | 1.49 | 0.24 | 3.73 | 0.16 | 22.70 | 0.0003 | 44.40 | Chakoumakos et al. 1979 |
| O. clarki | 26.40 | 13.70 | 7.64 | 0.87 | 6.00 | 2.80 | 0.53 | 0.08 | 1.33 | 0.06 | 20.10 | 0.0003 | 15.70 | Chakoumakos et al. 1979 |
| O. clarki henshawi | 169.00 | 12.00 | 8.10 | 0.50 | 27.89 | 24.17 | 52.34 | 4.18 | 162.24 | 3.79 | 117.00 | 0.0003 | 76.80 | Dwyer et al. 1995 |
| O. clarki henshawi | 169.00 | 12.00 | 8.25 | 0.50 | 27.89 | 24.17 | 52.34 | 4.18 | 162.24 | 3.79 | 117.00 | 0.0003 | 57.60 | Dwyer et al. 1995 |
| Oncorhynchus gorbuscha | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 137.28 | Servizi and Martens 1978 |
| O. gorbuscha | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 83.52 | Servizi and Martens 1978 |
| O. gorbuscha | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 191.04 | Servizi and Martens 1978 |
| Oncorhynchus kisutch | 33.00 | 13.50 | 7.29 | 2.50 | 8.78 | 2.70 | 7.32 | 1.15 | 6.14 | 6.81 | 29.00 | 0.0003 | 157.44 | Buckley 1983 |
| O. kisutch | 25.00 | 12.00 | 7.30 | 1.30 | 6.80 | 1.80 | 5.00 | 0.60 | 4.20 | 6.00 | 24.00 | 0.0003 | 31.68 | Chapman 1975 |
| O. kisutch | 20.00 | 9.40 | 7.29 | 1.30 | 5.78 | 1.69 | 4.46 | 0.70 | 2.59 | 5.34 | 22.00 | 0.0003 | 44.16 | Chapman and Stevens 1978 |
| O. kisutch | 31.10 | 13.30 | 7.30 | 3.20 | 8.02 | 2.70 | 5.12 | 0.65 | 4.00 | 4.50 | 29.60 | 0.0003 | 49.00 | Mudge et al. 1993 |
| O. kisutch | 31.10 | 13.30 | 7.30 | 3.20 | 8.02 | 2.70 | 5.12 | 0.65 | 4.00 | 4.50 | 29.60 | 0.0003 | 51.00 | Mudge et al. 1993 |
| O. kisutch | 31.60 | 15.70 | 7.50 | 3.20 | 8.15 | 2.74 | 5.12 | 0.65 | 3.50 | 4.20 | 30.40 | 0.0003 | 58.00 | Mudge et al. 1993 |
| O. kisutch | 31.00 | 15.30 | 7.20 | 3.20 | 7.99 | 2.69 | 5.12 | 0.65 | 2.30 | 3.10 | 29.70 | 0.0003 | 78.00 | Mudge et al. 1993 |
| Oncorhynchus mykiss | 44.10 | 11.50 | 7.70 | 2.00 | 9.07 | 4.10 | 4.75 | 1.02 | 3.30 | 1.56 | 49.70 | 0.0003 | 40.00 | Cacela et al. 1996 |
| O. mykiss | 44.60 | 11.50 | 7.80 | 0.99 | 7.37 | 6.10 | 6.24 | 0.80 | 1.31 | 3.82 | 53.10 | 0.0003 | 19.00 | Cacela et al. 1996 |
| O. mykiss | 36.10 | 11.40 | 7.60 | 1.31 | 4.03 | 7.13 | 1.56 | 0.26 | 1.49 | 0.88 | 36.60 | 0.0003 | 18.00 | Cacela et al. 1996 |
| O. mykiss | 36.20 | 11.50 | 6.10 | 1.36 | 3.93 | 7.27 | 1.57 | 0.28 | 1.47 | 0.87 | 8.50 | 0.0003 | 12.00 | Cacela et al. 1996 |
| O. mykiss | 20.40 | 11.70 | 7.50 | 0.15 | 3.13 | 2.77 | 2.62 | 0.25 | 0.36 | 1.48 | 23.00 | 0.0003 | 5.70 | Cacela et al. 1996 |
| O. mykiss | 45.20 | 11.70 | 7.70 | 1.23 | 9.70 | 4.43 | 5.33 | 0.97 | 3.41 | 1.47 | 50.00 | 0.0003 | 35.00 | Cacela et al. 1996 |
| O. mykiss | 45.40 | 11.80 | 6.30 | 1.22 | 9.70 | 4.43 | 5.02 | 0.98 | 3.37 | 1.37 | 10.90 | 0.0003 | 18.00 | Cacela et al. 1996 |
| O. mykiss | 41.90 | 12.30 | 7.90 | 0.33 | 6.60 | 5.97 | 5.89 | 0.63 | 1.11 | 3.37 | 48.30 | 0.0003 | 17.00 | Cacela et al. 1996 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 169.00 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 85.30 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 83.30 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 103.00 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 274.00 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 128.00 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 221.00 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 165.00 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 197.00 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 514.00 | Chakoumakos et al. 1979 |
| O. mykiss | 194.00 | 12.80 | 7.84 | 3.30 | 55.10 | 13.70 | 4.00 | 0.64 | 10.00 | 0.44 | 174.00 | 0.0003 | 243.00 | Chakoumakos et al. 1979 |
| O. mykiss | 23.00 | 12.20 | 7.10 | 1.40 | 6.10 | 1.80 | 4.40 | 0.40 | 5.80 | 6.00 | 22.00 | 0.0003 | 26.88 | Chapman 1975, 1978 |
| O. mykiss | 23.00 | 12.20 | 7.10 | 1.40 | 6.10 | 1.80 | 4.40 | 0.40 | 5.80 | 6.00 | 22.00 | 0.0003 | 16.32 | Chapman 1975, 1978 |
| O. mykiss | 23.00 | 12.20 | 7.40 | 1.30 | 6.80 | 1.80 | 5.00 | 0.60 | 4.20 | 6.00 | 22.00 | 0.0003 | 17.28 | Chapman 1975, 1978 |
| O. mykiss | 23.00 | 12.20 | 7.10 | 1.30 | 6.80 | 1.80 | 5.00 | 0.60 | 4.20 | 6.00 | 22.00 | 0.0003 | 27.84 | Chapman 1975, 1978 |
| O. mykiss | 9.20 | 15.50 | 6.96 | 0.50 | 2.30 | 0.70 | 2.00 | 0.20 | 4.60 | 2.10 | 11.00 | 0.0003 | 2.69 | Cusimano et al. 1986 |
| O. mykiss | 169.00 | 12.00 | 8.20 | 0.50 | 27.89 | 24.17 | 52.34 | 4.18 | 162.24 | 3.79 | 117.00 | 0.0003 | 105.60 | Dwyer et al. 1995 |
| O. mykiss | 169.00 | 12.00 | 7.95 | 0.50 | 27.89 | 24.17 | 52.34 | 4.18 | 162.24 | 3.79 | 117.00 | 0.0003 | 48.00 | Dwyer et al. 1995 |
| O. mykiss | 169.00 | 12.00 | 7.95 | 0.50 | 27.89 | 24.17 | 52.34 | 4.18 | 162.24 | 3.79 | 117.00 | 0.0003 | 57.60 | Dwyer et al. 1995 |
| O. mykiss | 214.00 | 7.64 | 7.94 | 0.27 | 49.40 | 24.10 | 10.30 | 1.75 | 18.90 | 5.28 | 198.00 | 0.0003 | 96.96 | Hansen et al. 2000 |
| O. mykiss | 220.00 | 7.74 | 7.92 | 0.36 | 51.20 | 25.50 | 8.36 | 2.10 | 24.00 | 4.64 | 197.00 | 0.0003 | 295.68 | Hansen et al. 2000 |
| O. mykiss | 105.00 | 7.77 | 7.82 | 0.10 | 23.10 | 11.80 | 3.54 | 3.22 | 17.10 | 2.91 | 94.10 | 0.0003 | 89.28 | Hansen et al. 2000 |
| O. mykiss | 98.20 | 8.49 | 7.89 | 0.05 | 22.30 | 11.20 | 3.58 | 0.90 | 11.50 | 2.85 | 87.90 | 0.0003 | 34.46 | Hansen et al. 2000 |
| O. mykiss | 104.00 | 16.30 | 7.83 | 0.28 | 22.40 | 11.40 | 3.76 | 2.72 | 12.40 | 3.01 | 97.60 | 0.0003 | 52.22 | Hansen et al. 2000 |
| O. mykiss | 31.00 | 15.30 | 7.20 | 3.20 | 7.99 | 2.69 | 5.12 | 0.65 | 2.30 | 3.10 | 29.70 | 0.0003 | 68.00 | Mudge et al. 1993 |
| O. mykiss | 38.70 | 12.00 | 7.62 | 0.33 | 2.37 | 8.65 | 13.70 | 0.15 | 0.36 | 20.30 | 40.00 | 0.0003 | 3.40 | Welsh et al. 2000 |
| O. mykiss | 39.30 | 12.00 | 7.61 | 0.36 | 14.10 | 1.80 | 13.20 | 0.10 | 0.36 | 19.90 | 41.70 | 0.0003 | 8.10 | Welsh et al. 2000 |
| O. mykiss | 89.50 | 12.00 | 8.21 | 0.35 | 15.00 | 11.85 | 10.05 | 1.00 | 0.36 | 6.73 | 97.50 | 0.0003 | 17.20 | Welsh et al. 2000 |
| O. mykiss | 89.67 | 12.00 | 8.15 | 0.35 | 28.90 | 3.15 | 32.50 | 0.50 | 0.36 | 45.20 | 97.25 | 0.0003 | 32.00 | Welsh et al. 2000 |
| Oncorhynchus nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 182.40 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 192.00 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 96.00 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 105.60 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 124.80 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 144.00 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 201.60 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 163.20 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 182.40 | Servizi and Martens 1978 |
| O. nerka | 83.10 | 7.15 | 7.63 | 2.58 | 22.34 | 6.31 | 10.26 | 7.50 | 25.10 | 9.99 | 62.50 | 0.0003 | 230.40 | Servizi and Martens 1978 |
| Oncorhynchus tshawytscha | 23.00 | 12.20 | 7.40 | 1.30 | 6.80 | 1.80 | 5.00 | 0.60 | 4.20 | 6.00 | 22.00 | 0.0003 | 24.96 | Chapman 1975, 1978 |
| O. tshawytscha | 23.00 | 12.20 | 7.40 | 1.30 | 6.80 | 1.80 | 5.00 | 0.60 | 4.20 | 6.00 | 22.00 | 0.0003 | 18.24 | Chapman 1975, 1978 |
| O. tshawytscha | 23.00 | 12.20 | 7.10 | 1.40 | 6.10 | 1.80 | 4.40 | 0.40 | 5.80 | 6.00 | 22.00 | 0.0003 | 36.48 | Chapman 1975, 1978 |
| O. tshawytscha | 23.00 | 12.20 | 7.10 | 1.30 | 6.80 | 1.80 | 5.00 | 0.60 | 4.20 | 6.00 | 22.00 | 0.0003 | 24.96 | Chapman 1975, 1978 |
| O. tshawytscha | 13.00 | 12.00 | 7.15 | 0.50 | 2.15 | 1.86 | 4.03 | 0.32 | 12.48 | 0.29 | 12.00 | 0.0003 | 9.79 | Chapman and McCrady 1977 |
| O. tshawytscha | 46.00 | 12.00 | 7.55 | 0.50 | 7.59 | 6.58 | 14.25 | 1.14 | 44.16 | 1.03 | 35.00 | 0.0003 | 23.14 | Chapman and McCrady 1977 |
| O. tshawytscha | 182.00 | 12.00 | 8.12 | 0.50 | 30.04 | 26.03 | 56.37 | 4.50 | 174.72 | 4.08 | 125.00 | 0.0003 | 79.20 | Chapman and McCrady 1977 |
| O. tshawytscha | 359.00 | 12.00 | 8.49 | 0.50 | 59.25 | 51.34 | 111.19 | 8.89 | 344.64 | 8.06 | 243.00 | 0.0003 | 123.26 | Chapman and McCrady 1977 |
| O. tshawytscha | 36.60 | 12.00 | 7.71 | 0.06 | 6.36 | 4.73 | 4.84 | 0.22 | 0.94 | 2.79 | 40.80 | 0.0003 | 7.40 | Welsh et al. 2000 |
| O. tshawytscha | 34.60 | 12.00 | 7.79 | 0.19 | 7.82 | 3.17 | 9.98 | 0.11 | 0.73 | 8.34 | 40.60 | 0.0003 | 12.50 | Welsh et al. 2000 |
| O. tshawytscha | 38.30 | 12.00 | 7.71 | 0.24 | 6.33 | 5.10 | 5.27 | 0.60 | 0.99 | 2.96 | 43.60 | 0.0003 | 14.30 | Welsh et al. 2000 |
| O. tshawytscha | 35.70 | 12.00 | 7.74 | 0.17 | 8.15 | 3.38 | 10.00 | 0.37 | 0.76 | 9.10 | 43.30 | 0.0003 | 18.30 | Welsh et al. 2000 |
| Oryzias latipes | 57.8 | 24 | 8.03 | 0.9 | 19.32 | 2.28 | 3.71 | 2.09 | 10.77 | 4.84 | 85.64 | 0.0003 | 296.41 | Wang et al.2013a |
| O. latipes | 61.70833 | 24.6 | 7.85 | 0.9 | 20.2 | 2.69 | 6.13 | 3.16 | 20.25 | 15.01 | 64.23 | 0.0003 | 305.94 | Wang et al.2013a |
| O. latipes | 141.3167 | 23.5 | 7.74 | 3.6 | 42.51 | 8.41 | 15.33 | 0.015 | 35.26 | 17.24 | 103.07 | 0.0003 | 1086.23 | Wang et al.2013b |
| O. latipes | 98.47083 | 21 | 7.79 | 8.9 | 28.91 | 6.287 | 8.71 | 0.015 | 25.23 | 15.5 | 82.45 | 0.0003 | 1313.93 | Wang et al.2013b |
| O. latipes | 181.5417 | 24.2 | 8.1 | 2.885 | 46.4 | 15.73 | 9.04 | 7.47 | 62.84 | 37.29 | 135.59 | 0.0003 | 1142.69 | Wang et al.2013b |
| O. latipes | 187.5417 | 23.8 | 8.21 | 4.348 | 42.95 | 19.24 | 9.42 | 8.7 | 96.07 | 63.93 | 142.73 | 0.0003 | 1326.42 | Wang et al.2013b |
| O. latipes | 57.79583 | 24 | 8.03 | 0.863 | 19.32 | 2.279 | 3.71 | 2.09 | 10.77 | 4.84 | 85.64 | 0.0003 | 296.41 | Wang et al.2013b |
| O. latipes | 61.69167 | 24.6 | 7.85 | 0.904 | 20.2 | 2.686 | 6.13 | 3.16 | 20.25 | 15.01 | 64.23 | 0.0003 | 305.94 | Wang et al.2013b |
| O. latipes | 59.35417 | 29 | 6.79 | 1.8 | 19.86 | 2.329 | 7 | 0.015 | 13.29 | 20.83 | 21.17 | 0.0003 | 192.82 | Wang et al.2013b |
| O. latipes | 64.20417 | 31 | 7.62 | 1.6 | 22.83 | 1.711 | 4.84 | 0.015 | 28.07 | 14.05 | 37.47 | 0.0003 | 141.16 | Wang et al.2013b |
| O. latipes | 9.108333 | 22 | 7.56 | 2.56 | 2.56 | 0.65 | 4.47 | 2.92 | 56.73 | 18.17 | 80.15 | 0.0003 | 565.29 | Wang et al.2013b |
| O. latipes | 8.758333 | 23 | 7.41 | 2.42 | 2.42 | 0.65 | 4.48 | 2.98 | 59.34 | 20.78 | 78.35 | 0.0003 | 517.13 | Wang et al.2013b |
| O. latipes | 8.683333 | 27.9 | 7.47 | 2.39 | 2.39 | 0.65 | 4.5 | 2.96 | 47.52 | 23.66 | 80.34 | 0.0003 | 468.89 | Wang et al.2013b |
| O. latipes | 8.608333 | 24.2 | 7.34 | 2.36 | 2.36 | 0.65 | 4.5 | 3.05 | 51.52 | 31.76 | 80.15 | 0.0003 | 455.19 | Wang et al.2013b |
| O. latipes | 8.583333 | 20.2 | 7.35 | 2.35 | 2.35 | 0.65 | 4.5 | 3.06 | 51.84 | 30.57 | 84.5 | 0.0003 | 616.51 | Wang et al.2013b |
| O. latipes | 8.558333 | 23 | 7.45 | 2.34 | 2.34 | 0.65 | 4.48 | 2.91 | 61.08 | 45.24 | 86.92 | 0.0003 | 819.76 | Wang et al.2013b |
| Physa integra | 44.90 | 15.00 | 7.70 | 1.10 | 13.20 | 2.91 | 1.27 | 0.56 | 3.32 | 1.20 | 42.70 | 0.0003 | 39.36 | Arthur and Leonard 1970 |
| P. integra | 44.90 | 15.00 | 7.70 | 1.10 | 13.20 | 2.91 | 1.27 | 0.56 | 3.32 | 1.20 | 42.70 | 0.0003 | 35.52 | Arthur and Leonard 1970 |
| Pimephales promelas | 103.00 | 22.00 | 7.40 | 0.50 | 28.47 | 7.77 | 27.78 | 2.64 | 29.60 | 53.02 | 65.00 | 0.0003 | 297.60 | Birge et al. 1983 |
| P. promelas | 103.00 | 22.00 | 7.40 | 0.50 | 28.47 | 7.77 | 27.78 | 2.64 | 29.60 | 53.02 | 65.00 | 0.0003 | 115.20 | Birge et al. 1983 |
| P. promelas | 263.00 | 22.00 | 7.40 | 0.50 | 72.69 | 19.85 | 36.49 | 3.46 | 77.90 | 130.77 | 65.00 | 0.0003 | 374.40 | Birge et al. 1983; Benson and Birge 1985 |
| P. promelas | 52.00 | 24.50 | 7.40 | 1.10 | 15.28 | 3.37 | 1.47 | 0.57 | 3.84 | 1.36 | 55.00 | 0.0003 | 52.80 | Carlson et al. 1986 |
| P. promelas | 52.00 | 24.50 | 7.40 | 1.10 | 15.28 | 3.37 | 1.47 | 0.57 | 3.84 | 1.36 | 55.00 | 0.0003 | 81.60 | Carlson et al. 1986 |
| P. promelas | 173.00 | 22.00 | 8.25 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 278.40 | Dwyer et al. 1995 |
| P. promelas | 173.00 | 22.00 | 8.10 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 604.80 | Dwyer et al. 1995 |
| P. promelas | 173.00 | 22.00 | 8.15 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 384.00 | Dwyer et al. 1995 |
| P. promelas | 173.00 | 22.00 | 7.30 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 374.40 | Dwyer et al. 1995 |
| P. promelas | 45.54 | 22.00 | 7.93 | 1.10 | 13.49 | 2.89 | 1.61 | 0.39 | 3.36 | 1.42 | 42.04 | 0.0003 | 53.96 | Erickson et al. 1996a,b |
| P. promelas | 45.54 | 22.00 | 7.93 | 1.10 | 13.49 | 2.89 | 91.27 | 0.39 | 3.36 | 143.23 | 42.04 | 0.0003 | 165.18 | Erickson et al. 1996a,b |
| P. promelas | 44.54 | 22.00 | 7.98 | 1.10 | 13.19 | 2.82 | 1.61 | 0.39 | 3.36 | 1.42 | 42.04 | 0.0003 | 59.46 | Erickson et al. 1996a,b |
| P. promelas | 44.54 | 22.00 | 7.98 | 1.10 | 13.19 | 2.82 | 45.98 | 0.39 | 3.36 | 72.32 | 44.04 | 0.0003 | 146.46 | Erickson et al. 1996a,b |
| P. promelas | 44.54 | 22.00 | 7.99 | 1.10 | 13.19 | 2.82 | 1.61 | 0.39 | 3.36 | 1.42 | 42.54 | 0.0003 | 82.04 | Erickson et al. 1996a,b |
| P. promelas | 45.54 | 22.00 | 7.96 | 1.10 | 13.49 | 2.89 | 1.61 | 0.39 | 3.36 | 36.87 | 43.04 | 0.0003 | 124.43 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.79 | 1.10 | 13.34 | 2.86 | 1.61 | 0.39 | 3.36 | 1.42 | 46.04 | 0.0003 | 103.76 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.81 | 1.10 | 13.34 | 2.86 | 47.59 | 0.39 | 99.42 | 1.42 | 46.04 | 0.0003 | 167.32 | Erickson et al. 1996a,b |
| P. promelas | 138.12 | 22.00 | 7.79 | 1.10 | 12.89 | 25.76 | 1.61 | 0.39 | 3.36 | 72.32 | 43.04 | 0.0003 | 120.02 | Erickson et al. 1996a,b |
| P. promelas | 151.13 | 22.00 | 7.78 | 1.10 | 14.11 | 28.18 | 1.61 | 0.39 | 99.42 | 1.42 | 43.04 | 0.0003 | 169.42 | Erickson et al. 1996a,b |
| P. promelas | 138.12 | 22.00 | 8.02 | 1.10 | 12.89 | 25.76 | 1.61 | 0.39 | 3.36 | 1.42 | 149.13 | 0.0003 | 268.22 | Erickson et al. 1996a,b |
| P. promelas | 139.12 | 22.00 | 7.78 | 1.10 | 51.18 | 2.78 | 1.61 | 0.39 | 99.42 | 1.42 | 43.04 | 0.0003 | 242.44 | Erickson et al. 1996a,b |
| P. promelas | 47.04 | 22.00 | 7.78 | 1.10 | 13.43 | 4.01 | 1.61 | 0.39 | 3.36 | 1.42 | 43.04 | 0.0003 | 113.35 | Erickson et al. 1996a,b |
| P. promelas | 37.03 | 22.00 | 7.79 | 0.88 | 11.02 | 3.28 | 2.99 | 0.39 | 3.36 | 1.42 | 43.04 | 0.0003 | 77.88 | Erickson et al. 1996a,b |
| P. promelas | 60.05 | 22.00 | 7.80 | 1.10 | 15.23 | 5.95 | 1.61 | 0.39 | 17.77 | 1.42 | 43.04 | 0.0003 | 128.02 | Erickson et al. 1996a,b |
| P. promelas | 76.07 | 22.00 | 7.80 | 1.10 | 18.84 | 7.41 | 1.61 | 0.39 | 32.18 | 1.77 | 42.04 | 0.0003 | 151.13 | Erickson et al. 1996a,b |
| P. promelas | 103.09 | 22.00 | 7.81 | 1.10 | 25.05 | 10.21 | 2.07 | 0.39 | 60.04 | 1.77 | 43.04 | 0.0003 | 166.62 | Erickson et al. 1996a,b |
| P. promelas | 103.09 | 22.00 | 7.78 | 1.10 | 32.06 | 4.01 | 1.84 | 0.39 | 58.12 | 1.77 | 40.04 | 0.0003 | 163.83 | Erickson et al. 1996a,b |
| P. promelas | 107.10 | 22.00 | 7.79 | 1.10 | 18.24 | 15.43 | 1.61 | 0.39 | 61.96 | 1.77 | 43.04 | 0.0003 | 157.48 | Erickson et al. 1996a,b |
| P. promelas | 134.12 | 22.00 | 7.80 | 1.10 | 32.26 | 13.00 | 1.61 | 0.39 | 88.85 | 1.77 | 43.04 | 0.0003 | 199.71 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.82 | 1.10 | 14.03 | 2.19 | 1.38 | 0.39 | 3.36 | 1.06 | 41.04 | 0.0003 | 128.52 | Erickson et al. 1996a,b |
| P. promelas | 46.04 | 22.00 | 7.82 | 1.10 | 14.03 | 2.19 | 6.21 | 1.56 | 5.76 | 7.09 | 42.04 | 0.0003 | 150.88 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.82 | 1.10 | 14.03 | 2.19 | 15.17 | 1.56 | 10.57 | 15.25 | 41.04 | 0.0003 | 131.06 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.81 | 1.10 | 14.23 | 2.19 | 35.17 | 1.56 | 21.61 | 36.16 | 41.04 | 0.0003 | 160.21 | Erickson et al. 1996a,b |
| P. promelas | 44.04 | 22.00 | 7.82 | 1.10 | 15.03 | 2.19 | 62.99 | 1.56 | 40.83 | 70.91 | 40.04 | 0.0003 | 182.88 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.81 | 1.10 | 14.43 | 2.19 | 101.39 | 1.95 | 59.08 | 107.78 | 41.04 | 0.0003 | 180.85 | Erickson et al. 1996a,b |
| P. promelas | 46.04 | 22.00 | 7.81 | 1.10 | 14.23 | 2.19 | 57.02 | 19.16 | 40.83 | 71.97 | 42.04 | 0.0003 | 176.78 | Erickson et al. 1996a,b |
| P. promelas | 189.17 | 22.00 | 7.82 | 1.10 | 55.11 | 15.80 | 1.61 | 0.78 | 152.25 | 1.06 | 42.04 | 0.0003 | 188.91 | Erickson et al. 1996a,b |
| P. promelas | 46.04 | 22.00 | 7.87 | 1.10 | 14.63 | 3.16 | 1.38 | 0.39 | 3.36 | 1.06 | 42.04 | 0.0003 | 125.60 | Erickson et al. 1996a,b |
| P. promelas | 75.07 | 22.00 | 7.87 | 1.10 | 24.45 | 5.95 | 1.38 | 0.39 | 30.74 | 1.06 | 41.04 | 0.0003 | 117.35 | Erickson et al. 1996a,b |
| P. promelas | 46.04 | 22.00 | 7.87 | 1.10 | 14.43 | 3.16 | 19.77 | 0.39 | 12.49 | 18.44 | 41.04 | 0.0003 | 114.55 | Erickson et al. 1996a,b |
| P. promelas | 74.07 | 22.00 | 7.85 | 1.10 | 24.45 | 6.08 | 18.39 | 0.39 | 38.90 | 18.44 | 42.04 | 0.0003 | 126.49 | Erickson et al. 1996a,b |
| P. promelas | 133.12 | 22.00 | 7.85 | 1.10 | 41.08 | 11.67 | 18.39 | 0.39 | 98.94 | 18.44 | 42.04 | 0.0003 | 172.72 | Erickson et al. 1996a,b |
| P. promelas | 76.07 | 22.00 | 7.85 | 1.10 | 24.05 | 6.08 | 47.59 | 0.78 | 58.12 | 52.12 | 43.04 | 0.0003 | 167.32 | Erickson et al. 1996a,b |
| P. promelas | 134.12 | 22.00 | 7.84 | 1.10 | 40.88 | 11.67 | 49.20 | 0.78 | 118.63 | 51.05 | 43.04 | 0.0003 | 226.70 | Erickson et al. 1996a,b |
| P. promelas | 52.05 | 22.00 | 7.96 | 0.30 | 12.02 | 4.13 | 1.61 | 0.39 | 10.57 | 1.77 | 42.04 | 0.0003 | 84.20 | Erickson et al. 1996a,b |
| P. promelas | 51.05 | 22.00 | 7.96 | 0.30 | 11.22 | 3.89 | 2.76 | 0.78 | 10.57 | 3.55 | 41.04 | 0.0003 | 97.79 | Erickson et al. 1996a,b |
| P. promelas | 50.04 | 22.00 | 7.95 | 0.30 | 11.02 | 3.77 | 5.98 | 1.56 | 12.01 | 8.15 | 41.04 | 0.0003 | 70.08 | Erickson et al. 1996a,b |
| P. promelas | 51.05 | 22.00 | 7.97 | 0.30 | 11.22 | 3.89 | 11.96 | 2.35 | 15.37 | 15.25 | 42.04 | 0.0003 | 81.58 | Erickson et al. 1996a,b |
| P. promelas | 51.05 | 22.00 | 7.96 | 0.30 | 11.22 | 3.77 | 23.22 | 3.13 | 21.61 | 30.14 | 41.04 | 0.0003 | 77.43 | Erickson et al. 1996a,b |
| P. promelas | 53.05 | 22.00 | 7.97 | 0.30 | 11.22 | 3.77 | 46.90 | 4.69 | 33.62 | 59.21 | 41.54 | 0.0003 | 110.87 | Erickson et al. 1996a,b |
| P. promelas | 53.05 | 22.00 | 7.96 | 0.30 | 11.62 | 3.89 | 117.94 | 7.04 | 68.20 | 141.81 | 42.04 | 0.0003 | 151.89 | Erickson et al. 1996a,b |
| P. promelas | 52.05 | 22.00 | 7.94 | 0.30 | 11.42 | 3.77 | 236.79 | 10.95 | 128.24 | 279.72 | 43.04 | 0.0003 | 175.26 | Erickson et al. 1996a,b |
| P. promelas | 47.04 | 25.00 | 7.82 | 1.10 | 13.94 | 2.98 | 1.61 | 0.39 | 3.36 | 1.42 | 42.54 | 0.0003 | 145.29 | Erickson et al. 1996a,b |
| P. promelas | 47.04 | 20.00 | 7.82 | 1.10 | 13.94 | 2.98 | 1.61 | 0.39 | 3.36 | 1.42 | 43.04 | 0.0003 | 111.76 | Erickson et al. 1996a,b |
| P. promelas | 47.04 | 15.00 | 7.82 | 1.10 | 13.94 | 2.98 | 1.61 | 0.39 | 3.36 | 1.42 | 42.54 | 0.0003 | 79.18 | Erickson et al. 1996a,b |
| P. promelas | 47.04 | 10.00 | 7.82 | 1.10 | 13.94 | 2.98 | 1.61 | 0.39 | 3.36 | 1.42 | 42.54 | 0.0003 | 60.01 | Erickson et al. 1996a,b |
| P. promelas | 140.12 | 22.00 | 8.03 | 0.30 | 29.06 | 12.03 | 25.06 | 4.30 | 60.04 | 25.88 | 98.09 | 0.0003 | 370.08 | Erickson et al. 1996a,b |
| P. promelas | 88.08 | 22.00 | 7.97 | 0.30 | 19.04 | 7.05 | 14.94 | 2.74 | 37.94 | 17.02 | 63.06 | 0.0003 | 292.10 | Erickson et al. 1996a,b |
| P. promelas | 59.05 | 22.00 | 7.89 | 0.30 | 12.02 | 4.62 | 9.20 | 0.78 | 23.05 | 9.93 | 39.03 | 0.0003 | 101.47 | Erickson et al. 1996a,b |
| P. promelas | 41.04 | 22.00 | 7.83 | 0.30 | 8.22 | 3.04 | 7.59 | 2.74 | 13.93 | 6.38 | 29.03 | 0.0003 | 62.51 | Erickson et al. 1996a,b |
| P. promelas | 27.02 | 22.00 | 7.75 | 0.30 | 5.61 | 1.82 | 4.60 | 2.35 | 8.65 | 4.25 | 23.02 | 0.0003 | 42.06 | Erickson et al. 1996a,b |
| P. promelas | 43.04 | 22.00 | 7.89 | 1.10 | 10.42 | 2.67 | 1.61 | 0.78 | 2.88 | 1.42 | 42.04 | 0.0003 | 172.47 | Erickson et al. 1996a,b |
| P. promelas | 25.02 | 22.00 | 7.57 | 0.30 | 6.69 | 2.03 | 3.45 | 1.17 | 4.32 | 4.96 | 16.01 | 0.0003 | 12.43 | Erickson et al. 1996a,b |
| P. promelas | 107.10 | 22.00 | 8.11 | 0.30 | 28.69 | 8.63 | 14.25 | 1.95 | 19.21 | 16.31 | 80.07 | 0.0003 | 271.27 | Erickson et al. 1996a,b |
| P. promelas | 87.08 | 22.00 | 7.06 | 0.30 | 23.33 | 7.02 | 13.56 | 1.95 | 19.21 | 15.95 | 58.05 | 0.0003 | 71.12 | Erickson et al. 1996a,b |
| P. promelas | 85.08 | 22.00 | 7.33 | 0.30 | 22.79 | 6.86 | 13.79 | 1.95 | 19.21 | 15.95 | 58.05 | 0.0003 | 79.63 | Erickson et al. 1996a,b |
| P. promelas | 88.08 | 22.00 | 7.61 | 0.30 | 23.60 | 7.10 | 13.56 | 1.95 | 19.21 | 15.95 | 59.05 | 0.0003 | 99.54 | Erickson et al. 1996a,b |
| P. promelas | 87.08 | 22.00 | 7.75 | 0.30 | 23.33 | 7.02 | 14.48 | 1.95 | 18.73 | 15.95 | 59.05 | 0.0003 | 132.72 | Erickson et al. 1996a,b |
| P. promelas | 87.08 | 22.00 | 8.07 | 0.30 | 23.33 | 7.02 | 12.64 | 1.95 | 18.73 | 15.95 | 59.05 | 0.0003 | 137.16 | Erickson et al. 1996a,b |
| P. promelas | 87.08 | 22.00 | 8.38 | 0.30 | 23.33 | 7.02 | 13.33 | 1.95 | 18.73 | 15.95 | 59.05 | 0.0003 | 182.25 | Erickson et al. 1996a,b |
| P. promelas | 87.08 | 22.00 | 8.73 | 0.30 | 23.33 | 7.02 | 14.25 | 1.95 | 18.73 | 14.89 | 59.05 | 0.0003 | 268.92 | Erickson et al. 1996a,b |
| P. promelas | 87.08 | 22.00 | 8.12 | 0.30 | 23.33 | 7.02 | 12.87 | 1.95 | 18.73 | 15.95 | 59.05 | 0.0003 | 188.98 | Erickson et al. 1996a,b |
| P. promelas | 251.22 | 22.00 | 7.20 | 0.30 | 67.13 | 20.36 | 57.48 | 4.69 | 72.52 | 62.40 | 150.13 | 0.0003 | 662.56 | Erickson et al. 1996a,b |
| P. promelas | 252.22 | 22.00 | 7.58 | 0.30 | 67.39 | 20.44 | 57.48 | 4.69 | 70.60 | 62.04 | 164.15 | 0.0003 | 904.88 | Erickson et al. 1996a,b |
| P. promelas | 252.22 | 22.00 | 7.92 | 0.30 | 67.39 | 20.44 | 57.48 | 4.69 | 73.48 | 62.04 | 150.13 | 0.0003 | 995.68 | Erickson et al. 1996a,b |
| P. promelas | 251.22 | 22.00 | 8.28 | 0.30 | 67.13 | 20.36 | 57.48 | 4.69 | 73.48 | 62.04 | 143.13 | 0.0003 | 891.54 | Erickson et al. 1996a,b |
| P. promelas | 200.18 | 22.00 | 8.05 | 0.30 | 53.54 | 16.19 | 37.24 | 3.52 | 49.47 | 46.80 | 128.11 | 0.0003 | 757.62 | Erickson et al. 1996a,b |
| P. promelas | 140.12 | 22.00 | 7.95 | 0.30 | 37.44 | 11.35 | 22.99 | 2.35 | 28.82 | 25.17 | 99.09 | 0.0003 | 404.81 | Erickson et al. 1996a,b |
| P. promelas | 90.08 | 22.00 | 8.05 | 0.30 | 24.13 | 7.26 | 14.25 | 1.95 | 18.73 | 15.60 | 65.06 | 0.0003 | 262.13 | Erickson et al. 1996a,b |
| P. promelas | 19.02 | 22.00 | 7.53 | 0.30 | 5.08 | 1.54 | 3.45 | 0.78 | 0.96 | 4.96 | 19.02 | 0.0003 | 20.45 | Erickson et al. 1996a,b |
| P. promelas | 34.03 | 22.00 | 7.53 | 0.30 | 9.09 | 2.76 | 3.45 | 0.78 | 9.61 | 4.61 | 20.02 | 0.0003 | 23.16 | Erickson et al. 1996a,b |
| P. promelas | 51.05 | 22.00 | 7.54 | 0.30 | 13.64 | 4.14 | 3.45 | 0.78 | 16.81 | 4.61 | 21.02 | 0.0003 | 34.99 | Erickson et al. 1996a,b |
| P. promelas | 29.03 | 22.00 | 7.59 | 0.30 | 7.76 | 2.35 | 3.45 | 0.78 | 5.28 | 4.61 | 22.02 | 0.0003 | 27.94 | Erickson et al. 1996a,b |
| P. promelas | 30.03 | 22.00 | 7.61 | 0.30 | 8.02 | 2.43 | 1.38 | 0.78 | 4.32 | 2.48 | 23.02 | 0.0003 | 26.67 | Erickson et al. 1996a,b |
| P. promelas | 27.02 | 22.00 | 7.55 | 0.30 | 7.22 | 2.19 | 10.35 | 1.17 | 5.28 | 13.12 | 20.02 | 0.0003 | 20.32 | Erickson et al. 1996a,b |
| P. promelas | 27.02 | 22.00 | 7.53 | 0.30 | 7.22 | 2.19 | 20.69 | 1.56 | 10.57 | 26.59 | 20.02 | 0.0003 | 26.67 | Erickson et al. 1996a,b |
| P. promelas | 90.08 | 22.00 | 8.00 | 0.30 | 24.13 | 7.26 | 14.25 | 1.95 | 19.21 | 15.95 | 63.06 | 0.0003 | 182.88 | Erickson et al. 1996a,b |
| P. promelas | 60.05 | 22.00 | 8.11 | 0.30 | 16.05 | 4.87 | 11.96 | 1.56 | 3.84 | 17.37 | 58.05 | 0.0003 | 96.67 | Erickson et al. 1996a,b |
| P. promelas | 120.11 | 22.00 | 8.09 | 0.30 | 32.09 | 9.73 | 11.96 | 1.56 | 33.62 | 17.37 | 59.05 | 0.0003 | 182.88 | Erickson et al. 1996a,b |
| P. promelas | 180.16 | 22.00 | 8.09 | 0.30 | 48.14 | 14.60 | 11.96 | 1.56 | 62.44 | 17.02 | 58.05 | 0.0003 | 190.69 | Erickson et al. 1996a,b |
| P. promelas | 91.08 | 22.00 | 8.13 | 0.30 | 24.34 | 7.38 | 11.96 | 1.56 | 19.21 | 15.95 | 59.05 | 0.0003 | 127.06 | Erickson et al. 1996a,b |
| P. promelas | 90.08 | 22.00 | 8.16 | 0.30 | 24.07 | 7.30 | 2.30 | 6.26 | 15.85 | 6.03 | 60.05 | 0.0003 | 148.59 | Erickson et al. 1996a,b |
| P. promelas | 93.08 | 22.00 | 8.14 | 0.30 | 24.87 | 7.54 | 35.86 | 3.91 | 27.38 | 49.99 | 62.06 | 0.0003 | 223.52 | Erickson et al. 1996a,b |
| P. promelas | 92.08 | 22.00 | 8.15 | 0.30 | 24.60 | 7.46 | 71.73 | 7.43 | 41.31 | 102.81 | 61.05 | 0.0003 | 283.15 | Erickson et al. 1996a,b |
| P. promelas | 91.08 | 22.00 | 8.19 | 0.30 | 24.40 | 7.34 | 14.48 | 15.25 | 18.73 | 17.37 | 62.06 | 0.0003 | 150.24 | Erickson et al. 1996a,b |
| P. promelas | 144.13 | 22.00 | 8.38 | 0.30 | 38.51 | 11.68 | 34.49 | 3.13 | 12.49 | 42.19 | 138.12 | 0.0003 | 644.53 | Erickson et al. 1996a,b |
| P. promelas | 292.26 | 22.00 | 8.27 | 0.30 | 78.09 | 23.68 | 34.49 | 3.13 | 87.89 | 57.08 | 137.12 | 0.0003 | 697.55 | Erickson et al. 1996a,b |
| P. promelas | 440.39 | 22.00 | 8.23 | 0.30 | 117.67 | 35.69 | 34.49 | 3.13 | 175.31 | 41.13 | 133.12 | 0.0003 | 752.48 | Erickson et al. 1996a,b |
| P. promelas | 217.19 | 22.00 | 8.31 | 0.30 | 58.03 | 17.60 | 34.49 | 3.13 | 46.59 | 43.25 | 133.12 | 0.0003 | 653.42 | Erickson et al. 1996a,b |
| P. promelas | 218.19 | 22.00 | 8.31 | 0.30 | 58.30 | 17.68 | 6.90 | 1.56 | 38.90 | 9.57 | 140.12 | 0.0003 | 646.37 | Erickson et al. 1996a,b |
| P. promelas | 212.19 | 22.00 | 8.35 | 0.30 | 56.70 | 17.19 | 103.45 | 7.82 | 65.32 | 124.79 | 143.13 | 0.0003 | 939.80 | Erickson et al. 1996a,b |
| P. promelas | 92.08 | 22.00 | 8.13 | 0.30 | 24.67 | 7.42 | 14.25 | 1.95 | 19.21 | 16.66 | 63.06 | 0.0003 | 253.37 | Erickson et al. 1996a,b |
| P. promelas | 47.04 | 22.00 | 8.10 | 1.10 | 13.94 | 2.98 | 1.61 | 0.39 | 3.36 | 1.42 | 42.54 | 0.0003 | 27.94 | Erickson et al. 1996a,b |
| P. promelas | 243.22 | 22.00 | 8.01 | 1.10 | 92.73 | 2.88 | 47.13 | 0.39 | 3.36 | 143.23 | 43.04 | 0.0003 | 105.73 | Erickson et al. 1996a,b |
| P. promelas | 255.73 | 22.00 | 8.01 | 1.10 | 14.17 | 53.58 | 1.61 | 0.39 | 3.36 | 143.23 | 43.54 | 0.0003 | 40.06 | Erickson et al. 1996a,b |
| P. promelas | 47.04 | 22.00 | 8.10 | 1.10 | 13.94 | 2.98 | 47.59 | 0.39 | 3.36 | 72.32 | 43.54 | 0.0003 | 64.26 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 8.02 | 1.10 | 13.34 | 2.86 | 1.61 | 0.39 | 3.36 | 1.42 | 43.04 | 0.0003 | 49.02 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 8.65 | 1.10 | 13.34 | 2.86 | 1.61 | 0.39 | 3.36 | 1.42 | 47.04 | 0.0003 | 67.72 | Erickson et al. 1996a,b |
| P. promelas | 45.54 | 22.00 | 7.30 | 1.10 | 13.49 | 2.89 | 1.61 | 0.39 | 3.36 | 1.42 | 44.04 | 0.0003 | 18.67 | Erickson et al. 1996a,b |
| P. promelas | 49.04 | 22.00 | 6.63 | 1.10 | 14.53 | 3.11 | 1.61 | 0.39 | 3.36 | 1.42 | 49.04 | 0.0003 | 6.15 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.16 | 1.10 | 13.34 | 2.86 | 1.61 | 0.39 | 3.36 | 15.60 | 26.02 | 0.0003 | 20.45 | Erickson et al. 1996a,b |
| P. promelas | 43.04 | 22.00 | 7.93 | 1.10 | 12.75 | 2.73 | 1.61 | 0.39 | 3.36 | 1.42 | 41.04 | 0.0003 | 93.36 | Erickson et al. 1996a,b |
| P. promelas | 45.54 | 22.00 | 7.91 | 6.10 | 13.49 | 2.89 | 1.61 | 0.39 | 3.36 | 1.42 | 44.04 | 0.0003 | 245.36 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.94 | 1.10 | 13.34 | 2.86 | 1.61 | 0.39 | 3.36 | 1.42 | 43.04 | 0.0003 | 72.34 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.95 | 6.10 | 13.34 | 2.86 | 1.61 | 0.39 | 3.36 | 1.42 | 43.04 | 0.0003 | 229.81 | Erickson et al. 1996a,b |
| P. promelas | 45.54 | 22.00 | 7.94 | 3.60 | 13.49 | 2.89 | 1.61 | 0.39 | 3.36 | 1.42 | 44.04 | 0.0003 | 195.45 | Erickson et al. 1996a,b |
| P. promelas | 45.04 | 22.00 | 7.91 | 2.35 | 13.34 | 2.86 | 1.61 | 0.39 | 3.36 | 1.42 | 42.04 | 0.0003 | 109.35 | Erickson et al. 1996a,b |
| P. promelas | 44.04 | 22.00 | 7.87 | 1.10 | 13.05 | 2.79 | 1.61 | 0.39 | 3.36 | 1.42 | 42.04 | 0.0003 | 78.00 | Erickson et al. 1996a,b |
| P. promelas | 44.04 | 22.00 | 7.84 | 1.10 | 13.05 | 2.79 | 1.61 | 0.39 | 3.36 | 19.15 | 17.02 | 0.0003 | 45.52 | Erickson et al. 1996a,b |
| P. promelas | 22.52 | 22.00 | 6.01 | 0.30 | 6.02 | 1.82 | 3.45 | 0.39 | 3.36 | 4.25 | 15.01 | 0.0003 | 4.38 | Erickson et al. 1996a,b |
| P. promelas | 24.02 | 22.00 | 7.02 | 0.30 | 6.42 | 1.95 | 3.68 | 0.39 | 3.36 | 4.96 | 17.02 | 0.0003 | 12.43 | Erickson et al. 1996a,b |
| P. promelas | 23.02 | 22.00 | 8.00 | 0.30 | 6.15 | 1.87 | 4.14 | 0.78 | 3.36 | 4.96 | 17.52 | 0.0003 | 26.86 | Erickson et al. 1996a,b |
| P. promelas | 21.52 | 22.00 | 9.01 | 0.30 | 5.75 | 1.74 | 4.60 | 1.56 | 3.36 | 4.96 | 19.02 | 0.0003 | 51.33 | Erickson et al. 1996a,b |
| P. promelas | 48.00 | 25.00 | 8.03 | 2.64 | 14.11 | 3.11 | 1.35 | 0.57 | 3.54 | 1.25 | 44.00 | 0.0003 | 109.44 | Lind et al. 1978 |
| P. promelas | 45.00 | 25.00 | 8.04 | 2.64 | 13.23 | 2.92 | 1.27 | 0.57 | 3.33 | 1.17 | 44.00 | 0.0003 | 116.16 | Lind et al. 1978 |
| P. promelas | 46.00 | 25.00 | 7.98 | 2.64 | 13.52 | 2.98 | 1.30 | 0.57 | 3.40 | 1.20 | 41.00 | 0.0003 | 84.96 | Lind et al. 1978 |
| P. promelas | 30.00 | 25.00 | 6.82 | 10.47 | 7.14 | 2.96 | 1.63 | 0.50 | 6.13 | 1.25 | 21.00 | 0.0003 | 418.56 | Lind et al. 1978 |
| P. promelas | 37.00 | 25.00 | 7.28 | 11.34 | 8.80 | 3.66 | 2.00 | 0.50 | 7.55 | 1.54 | 21.00 | 0.0003 | 495.36 | Lind et al. 1978 |
| P. promelas | 87.00 | 25.00 | 7.11 | 31.40 | 20.70 | 8.44 | 16.07 | 1.86 | 22.35 | 18.63 | 20.00 | 0.0003 | 1522.56 | Lind et al. 1978 |
| P. promelas | 73.00 | 25.00 | 6.94 | 24.42 | 17.22 | 7.33 | 10.54 | 1.52 | 18.44 | 13.62 | 18.00 | 0.0003 | 1083.84 | Lind et al. 1978 |
| P. promelas | 84.00 | 25.00 | 7.07 | 14.52 | 20.46 | 8.01 | 6.00 | 1.40 | 34.50 | 10.95 | 12.00 | 0.0003 | 528.00 | Lind et al. 1978 |
| P. promelas | 66.00 | 25.00 | 6.97 | 32.90 | 16.08 | 6.29 | 4.71 | 1.40 | 27.11 | 8.60 | 12.00 | 0.0003 | 960.96 | Lind et al. 1978 |
| P. promelas | 166.00 | 5.00 | 8.05 | 0.50 | 27.40 | 23.74 | 51.42 | 4.11 | 159.36 | 3.73 | 132.50 | 0.0003 | 432.00 | Richards and Beitinger 1995 |
| P. promelas | 159.00 | 12.00 | 8.35 | 0.50 | 26.24 | 22.74 | 49.25 | 3.94 | 152.64 | 3.57 | 135.00 | 0.0003 | 285.12 | Richards and Beitinger 1995 |
| P. promelas | 168.00 | 22.00 | 8.30 | 0.50 | 27.73 | 24.02 | 52.03 | 4.16 | 161.28 | 3.77 | 142.50 | 0.0003 | 298.56 | Richards and Beitinger 1995 |
| P. promelas | 167.00 | 32.00 | 8.45 | 0.50 | 27.56 | 23.88 | 51.72 | 4.13 | 160.32 | 3.75 | 140.00 | 0.0003 | 492.48 | Richards and Beitinger 1995 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.73 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 395.50 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 4.97 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 499.50 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 9.56 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 1405.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.94 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 671.20 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 5.01 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 1514.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 10.58 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 2013.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.07 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 383.70 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 4.98 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 726.50 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 9.94 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 1681.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.96 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 137.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 3.94 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 259.50 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 9.53 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 597.70 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.42 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 283.60 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 5.68 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 596.30 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 8.93 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 1949.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.61 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 422.10 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 4.26 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 468.10 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 9.77 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 1564.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.46 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 363.60 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 5.14 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 494.80 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 10.16 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 1526.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.89 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 429.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 5.95 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 774.30 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 18.23 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 2034.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 2.45 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 402.40 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 5.47 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 613.30 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 10.94 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 1959.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 0.51 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 129.00 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 0.42 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 96.60 | Ryan et al. 2004 |
| P. promelas | 90.08 | 25.00 | 8.00 | 0.41 | 15.03 | 12.76 | 33.22 | 2.65 | 88.83 | 5.71 | 62.15 | 0.0003 | 106.10 | Ryan et al. 2004 |
| P. promelas | 290.00 | 25.00 | 6.27 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 14.40 | Schubauer-Berigan et al. 1993 |
| P. promelas | 290.00 | 25.00 | 7.14 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 42.24 | Schubauer-Berigan et al. 1993 |
| P. promelas | 290.00 | 25.00 | 8.60 | 0.50 | 47.86 | 41.47 | 89.82 | 7.18 | 278.40 | 6.51 | 235.00 | 0.0003 | 192.00 | Schubauer-Berigan et al. 1993 |
| P. promelas | 11.74 | 25.00 | 5.98 | 0.50 | 1.90 | 1.70 | 21.00 | 0.50 | 53.40 | 0.40 | 4.00 | 0.0003 | 2.90 | Sciera et al. 2004 |
| P. promelas | 20.68 | 25.00 | 6.00 | 0.50 | 3.50 | 2.90 | 21.10 | 0.60 | 61.50 | 0.40 | 4.00 | 0.0003 | 3.40 | Sciera et al. 2004 |
| P. promelas | 47.48 | 25.00 | 5.96 | 0.50 | 7.80 | 6.80 | 22.10 | 0.40 | 92.20 | 0.40 | 4.00 | 0.0003 | 3.60 | Sciera et al. 2004 |
| P. promelas | 12.41 | 25.00 | 7.32 | 0.50 | 2.00 | 1.80 | 26.40 | 0.50 | 48.50 | 0.40 | 28.00 | 0.0003 | 3.80 | Sciera et al. 2004 |
| P. promelas | 22.43 | 25.00 | 7.34 | 0.50 | 4.20 | 2.90 | 27.90 | 0.40 | 77.70 | 0.70 | 28.00 | 0.0003 | 16.40 | Sciera et al. 2004 |
| P. promelas | 46.33 | 25.00 | 7.33 | 0.50 | 8.00 | 6.40 | 29.40 | 0.60 | 46.30 | 0.60 | 28.00 | 0.0003 | 28.50 | Sciera et al. 2004 |
| P. promelas | 10.58 | 25.00 | 7.92 | 0.50 | 1.60 | 1.60 | 21.70 | 0.50 | 10.20 | 0.40 | 52.00 | 0.0003 | 10.50 | Sciera et al. 2004 |
| P. promelas | 21.09 | 25.00 | 7.94 | 0.50 | 3.50 | 3.00 | 22.00 | 0.60 | 18.30 | 0.40 | 54.00 | 0.0003 | 19.70 | Sciera et al. 2004 |
| P. promelas | 49.48 | 25.00 | 8.04 | 0.50 | 8.60 | 6.80 | 22.40 | 0.40 | 40.60 | 0.40 | 55.30 | 0.0003 | 31.30 | Sciera et al. 2004 |
| P. promelas | 10.92 | 25.00 | 5.90 | 5.20 | 1.90 | 1.50 | 42.60 | 0.40 | 63.70 | 7.80 | 4.00 | 0.0003 | 16.50 | Sciera et al. 2004 |
| P. promelas | 19.77 | 25.00 | 5.93 | 5.40 | 3.30 | 2.80 | 39.20 | 0.60 | 71.80 | 8.00 | 4.00 | 0.0003 | 16.60 | Sciera et al. 2004 |
| P. promelas | 44.01 | 25.00 | 5.90 | 6.00 | 7.40 | 6.20 | 40.80 | 0.70 | 94.20 | 8.80 | 4.00 | 0.0003 | 33.70 | Sciera et al. 2004 |
| P. promelas | 12.08 | 25.00 | 5.99 | 9.50 | 2.20 | 1.60 | 30.30 | 0.40 | 11.30 | 13.70 | 4.00 | 0.0003 | 45.90 | Sciera et al. 2004 |
| P. promelas | 21.84 | 25.00 | 5.94 | 11.10 | 3.80 | 3.00 | 32.30 | 0.30 | 19.40 | 16.10 | 4.00 | 0.0003 | 78.70 | Sciera et al. 2004 |
| P. promelas | 47.57 | 25.00 | 5.99 | 10.80 | 8.00 | 6.70 | 32.40 | 0.60 | 41.80 | 15.60 | 4.00 | 0.0003 | 112.40 | Sciera et al. 2004 |
| P. promelas | 11.26 | 25.00 | 7.85 | 5.10 | 2.20 | 1.40 | 30.10 | 0.50 | 4.30 | 7.60 | 44.70 | 0.0003 | 159.80 | Sciera et al. 2004 |
| P. promelas | 19.20 | 25.00 | 7.92 | 5.20 | 3.40 | 2.60 | 28.90 | 0.50 | 12.50 | 7.70 | 60.00 | 0.0003 | 256.50 | Sciera et al. 2004 |
| P. promelas | 26.94 | 25.00 | 7.92 | 5.20 | 6.50 | 2.60 | 27.70 | 0.40 | 34.80 | 7.80 | 60.00 | 0.0003 | 413.60 | Sciera et al. 2004 |
| P. promelas | 11.25 | 25.00 | 7.86 | 11.10 | 1.70 | 1.70 | 32.90 | 32.50 | 1.10 | 16.00 | 56.00 | 0.0003 | 202.50 | Sciera et al. 2004 |
| P. promelas | 17.45 | 25.00 | 7.85 | 11.20 | 2.70 | 2.60 | 32.70 | 26.10 | 1.10 | 16.10 | 56.00 | 0.0003 | 371.10 | Sciera et al. 2004 |
| P. promelas | 40.19 | 25.00 | 7.85 | 11.00 | 6.20 | 6.00 | 32.90 | 6.70 | 1.10 | 15.90 | 56.00 | 0.0003 | 427.30 | Sciera et al. 2004 |
| P. promelas | 11.33 | 25.00 | 6.05 | 4.10 | 1.90 | 1.60 | 48.40 | 0.40 | 99.60 | 4.20 | 4.00 | 0.0003 | 22.90 | Sciera et al. 2004 |
| P. promelas | 21.01 | 25.00 | 6.05 | 4.10 | 3.30 | 3.10 | 47.50 | 0.50 | 107.70 | 4.20 | 4.00 | 0.0003 | 39.50 | Sciera et al. 2004 |
| P. promelas | 44.35 | 25.00 | 5.95 | 4.50 | 7.70 | 6.10 | 49.00 | 1.50 | 130.40 | 4.50 | 4.00 | 0.0003 | 40.00 | Sciera et al. 2004 |
| P. promelas | 11.74 | 25.00 | 6.08 | 10.30 | 1.90 | 1.70 | 17.30 | 1.30 | 68.80 | 9.80 | 4.00 | 0.0003 | 64.10 | Sciera et al. 2004 |
| P. promelas | 22.25 | 25.00 | 6.11 | 9.20 | 3.80 | 3.10 | 17.10 | 0.60 | 76.20 | 8.80 | 4.00 | 0.0003 | 89.10 | Sciera et al. 2004 |
| P. promelas | 47.14 | 25.00 | 6.11 | 9.60 | 7.50 | 6.90 | 17.20 | 0.70 | 98.80 | 9.20 | 4.00 | 0.0003 | 106.70 | Sciera et al. 2004 |
| P. promelas | 11.99 | 25.00 | 7.07 | 5.30 | 2.00 | 1.70 | 26.60 | 0.50 | 45.00 | 5.00 | 20.00 | 0.0003 | 92.40 | Sciera et al. 2004 |
| P. promelas | 21.34 | 25.00 | 7.19 | 5.50 | 3.60 | 3.00 | 26.80 | 0.50 | 73.10 | 6.30 | 20.00 | 0.0003 | 134.50 | Sciera et al. 2004 |
| P. promelas | 45.99 | 25.00 | 7.27 | 5.50 | 7.70 | 6.50 | 26.10 | 0.60 | 40.70 | 10.30 | 20.00 | 0.0003 | 213.20 | Sciera et al. 2004 |
| P. promelas | 11.99 | 25.00 | 7.18 | 9.80 | 2.00 | 1.70 | 27.80 | 0.50 | 52.30 | 10.20 | 26.00 | 0.0003 | 260.90 | Sciera et al. 2004 |
| P. promelas | 20.43 | 25.00 | 7.15 | 9.60 | 3.40 | 2.90 | 29.50 | 0.50 | 74.30 | 11.30 | 20.00 | 0.0003 | 295.50 | Sciera et al. 2004 |
| P. promelas | 45.74 | 25.00 | 7.16 | 10.30 | 7.60 | 6.50 | 31.60 | 0.60 | 74.30 | 11.30 | 26.00 | 0.0003 | 391.00 | Sciera et al. 2004 |
| P. promelas | 9.76 | 25.00 | 7.93 | 4.60 | 1.60 | 1.40 | 51.70 | 0.90 | 13.70 | 4.60 | 44.00 | 0.0003 | 63.80 | Sciera et al. 2004 |
| P. promelas | 16.86 | 25.00 | 7.83 | 4.50 | 2.30 | 2.70 | 51.30 | 0.50 | 21.80 | 4.50 | 44.00 | 0.0003 | 103.00 | Sciera et al. 2004 |
| P. promelas | 39.45 | 25.00 | 7.87 | 4.50 | 6.40 | 5.70 | 50.10 | 0.80 | 44.20 | 4.50 | 44.00 | 0.0003 | 284.60 | Sciera et al. 2004 |
| P. promelas | 8.76 | 25.00 | 8.00 | 9.50 | 1.20 | 1.40 | 35.40 | 0.10 | 7.30 | 8.80 | 56.00 | 0.0003 | 140.50 | Sciera et al. 2004 |
| P. promelas | 14.86 | 25.00 | 7.97 | 9.20 | 1.50 | 2.70 | 31.80 | 0.10 | 7.50 | 8.40 | 61.30 | 0.0003 | 268.00 | Sciera et al. 2004 |
| P. promelas | 27.06 | 25.00 | 7.87 | 9.30 | 1.60 | 5.60 | 34.70 | 0.10 | 37.40 | 8.50 | 48.00 | 0.0003 | 208.40 | Sciera et al. 2004 |
| P. promelas | 43.90 | 25.00 | 7.40 | 2.00 | 12.90 | 2.85 | 1.24 | 0.57 | 3.24 | 1.14 | 42.40 | 0.0003 | 88.32 | Spehar and Fiandt 1986 |
| P. promelas | 19.00 | 22.00 | 7.06 | 0.60 | 4.90 | 1.64 | 3.70 | 0.78 | 9.60 | 5.80 | 11.17 | 0.0003 | 4.63 | Welsh et al. 1993 |
| P. promelas | 19.50 | 22.00 | 7.25 | 0.40 | 5.20 | 1.64 | 5.36 | 0.79 | 2.45 | 8.60 | 12.70 | 0.0003 | 7.87 | Welsh et al. 1993 |
| P. promelas | 16.50 | 22.00 | 6.36 | 3.30 | 4.10 | 1.54 | 2.82 | 0.76 | 9.40 | 4.70 | 8.46 | 0.0003 | 30.31 | Welsh et al. 1993 |
| P. promelas | 17.00 | 22.00 | 6.42 | 3.10 | 4.20 | 1.56 | 2.74 | 0.74 | 7.40 | 4.60 | 3.40 | 0.0003 | 20.22 | Welsh et al. 1993 |
| P. promelas | 19.00 | 22.00 | 6.38 | 4.30 | 5.00 | 1.62 | 7.04 | 0.72 | 10.20 | 12.20 | 7.83 | 0.0003 | 34.53 | Welsh et al. 1993 |
| P. promelas | 17.00 | 22.00 | 7.15 | 3.40 | 4.20 | 1.54 | 2.90 | 1.00 | 7.40 | 4.70 | 8.74 | 0.0003 | 57.44 | Welsh et al. 1993 |
| P. promelas | 17.00 | 22.00 | 7.16 | 0.80 | 4.50 | 1.46 | 2.68 | 0.78 | 10.90 | 3.80 | 9.30 | 0.0003 | 4.64 | Welsh et al. 1993 |
| P. promelas | 17.50 | 22.00 | 7.13 | 5.10 | 4.60 | 1.48 | 2.62 | 0.77 | 10.50 | 3.50 | 8.95 | 0.0003 | 67.47 | Welsh et al. 1993 |
| P. promelas | 18.50 | 22.00 | 7.06 | 10.50 | 5.00 | 1.54 | 2.64 | 0.80 | 10.70 | 3.50 | 8.29 | 0.0003 | 80.25 | Welsh et al. 1993 |
| P. promelas | 18.50 | 22.00 | 6.90 | 15.60 | 4.90 | 1.50 | 3.54 | 0.99 | 7.00 | 5.20 | 9.52 | 0.0003 | 174.72 | Welsh et al. 1993 |
| Poeciliopsis occidentalis occidentalis | 167.00 | 22.00 | 8.00 | 0.50 | 27.56 | 23.88 | 51.72 | 4.13 | 160.32 | 3.75 | 115.00 | 0.0003 | 153.60 | Dwyer et al. 1999 |
| Ptychocheilus lucius | 173.00 | 22.00 | 8.30 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 364.80 | Dwyer et al. 1995 |
| P. lucius | 173.00 | 22.00 | 7.25 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 460.80 | Dwyer et al. 1995 |
| Ptychocheilus oregonensis | 25.00 | 7.80 | 7.30 | 1.10 | 7.15 | 1.98 | 4.82 | 0.70 | 4.00 | 5.98 | 25.00 | 0.0003 | 22.08 | Andros and Garton 1980 |
| P. oregonensis | 54.00 | 11.50 | 7.30 | 1.10 | 15.09 | 3.64 | 6.88 | 0.70 | 12.16 | 9.69 | 43.00 | 0.0003 | 17.28 | Andros and Garton 1980 |
| Salvelinus confluentus | 214.00 | 7.64 | 7.94 | 0.27 | 49.40 | 24.10 | 10.30 | 1.75 | 18.90 | 5.28 | 198.00 | 0.0003 | 218.88 | Hansen et al. 2000 |
| S. confluentus | 220.00 | 7.74 | 7.92 | 0.36 | 51.20 | 25.50 | 8.36 | 2.10 | 24.00 | 4.64 | 197.00 | 0.0003 | 198.72 | Hansen et al. 2000 |
| S. confluentus | 105.00 | 7.77 | 7.82 | 0.10 | 23.10 | 11.80 | 3.54 | 3.22 | 17.10 | 2.91 | 94.10 | 0.0003 | 63.94 | Hansen et al. 2000 |
| S. confluentus | 98.20 | 8.49 | 7.89 | 0.05 | 22.30 | 11.20 | 3.58 | 0.90 | 11.50 | 2.85 | 87.90 | 0.0003 | 48.00 | Hansen et al. 2000 |
| S. confluentus | 104.00 | 16.30 | 7.83 | 0.28 | 22.40 | 11.40 | 3.76 | 2.72 | 12.40 | 3.01 | 97.60 | 0.0003 | 85.44 | Hansen et al. 2000 |
| Scaphirhynchus platorynchus | 167.00 | 22.00 | 7.60 | 0.50 | 27.56 | 23.88 | 51.72 | 4.13 | 160.32 | 3.75 | 115.00 | 0.0003 | 153.60 | Dwyer et al. 1999 |
| Scapholeberis sp. | 52.00 | 24.50 | 7.50 | 1.10 | 15.28 | 3.37 | 1.47 | 0.57 | 3.84 | 1.36 | 55.00 | 0.0003 | 17.28 | Carlson et al. 1986 |
| Scenedesmus obliquus | 81.1 | 23.8 | 8.03 | 0.9 | 21.54 | 6.54 | 8.24 | 2.36 | 23.12 | 14.02 | 85.61 | 0.0003 | 184.4 | Xu 2019 |
| S. obliquus | 114.3833 | 22.2 | 7.74 | 4.1 | 19.07 | 16.01 | 4.5 | 3.18 | 27.56 | 16.26 | 91.05 | 0.0003 | 356.1 | Xu 2019 |
| S. obliquus | 103.0167 | 22.4 | 7.79 | 2.3 | 28.74 | 7.48 | 4.67 | 5.46 | 25.31 | 15.09 | 62.35 | 0.0003 | 267.9 | Xu 2019 |
| S. obliquus | 62.95833 | 23.1 | 7.86 | 3.5 | 19.25 | 3.56 | 16.06 | 2.54 | 11.23 | 5.78 | 82.05 | 0.0003 | 302.5 | Xu 2019 |
| S. obliquus | 124.6917 | 24.2 | 8.1 | 1.8 | 20.06 | 17.89 | 9.45 | 8.72 | 42.07 | 20.18 | 75.06 | 0.0003 | 281.6 | Xu 2019 |
| Utterbackia imbecillis | 92.00 | 25.00 | 8.10 | 0.50 | 29.06 | 4.74 | 30.80 | 1.64 | 46.01 | 32.72 | 77.00 | 0.0003 | 72.96 | Keller 2000 |
| U. imbecillis | 86.00 | 25.00 | 8.20 | 0.50 | 27.17 | 4.43 | 28.79 | 1.53 | 43.01 | 30.58 | 78.00 | 0.0003 | 81.60 | Keller 2000 |
| U. imbecillis | 90.00 | 25.00 | 8.00 | 0.50 | 28.43 | 4.64 | 30.13 | 1.61 | 45.01 | 32.01 | 78.00 | 0.0003 | 39.36 | Keller 2000 |
| U. imbecillis | 90.00 | 24.00 | 8.20 | 0.50 | 14.85 | 12.87 | 13.94 | 1.11 | 43.20 | 1.01 | 99.00 | 0.0003 | 75.84 | Keller 2000 |
| U. imbecillis | 90.00 | 25.00 | 7.90 | 0.50 | 28.43 | 4.64 | 30.13 | 1.61 | 45.01 | 32.01 | 99.00 | 0.0003 | 69.12 | Keller 2000 |
| U. imbecillis | 86.00 | 25.00 | 7.90 | 0.50 | 14.19 | 12.30 | 13.32 | 1.06 | 41.28 | 0.97 | 59.00 | 0.0003 | 36.48 | Keller 2000 |
| U. imbecillis | 39.00 | 23.00 | 7.40 | 0.50 | 6.44 | 5.58 | 12.08 | 0.97 | 37.44 | 0.88 | 32.50 | 0.0003 | 82.56 | Keller and Zam 1991 |
| U. imbecillis | 90.00 | 23.00 | 7.60 | 0.50 | 13.97 | 12.12 | 26.25 | 2.10 | 81.37 | 1.90 | 65.00 | 0.0003 | 191.04 | Keller and Zam 1991 |
| Villosa iris | 100.75 | 20.00 | 8.30 | 0.40 | 26.00 | 8.70 | 8.90 | 1.00 | 18.00 | 10.00 | 90.00 | 0.0003 | 15.00 | Wang et al. 2011 |
| V. iris | 97.02 | 20.00 | 8.30 | 3.00 | 25.00 | 8.40 | 8.70 | 1.00 | 23.00 | 10.00 | 88.00 | 0.0003 | 32.00 | Wang et al. 2011 |
| V. iris | 99.93 | 20.00 | 8.30 | 5.80 | 26.00 | 8.50 | 9.10 | 1.00 | 19.00 | 11.00 | 87.00 | 0.0003 | 65.00 | Wang et al. 2011 |
| V. iris | 100.75 | 20.00 | 8.20 | 10.00 | 26.00 | 8.70 | 9.60 | 1.30 | 16.00 | 12.00 | 88.00 | 0.0003 | 72.00 | Wang et al. 2011 |
| Xyrauchen texanus | 173.00 | 22.00 | 8.15 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 211.20 | Dwyer et al. 1995 |
| X. texanus | 173.00 | 22.00 | 8.05 | 0.50 | 28.55 | 24.74 | 53.58 | 4.28 | 166.08 | 3.88 | 117.00 | 0.0003 | 326.40 | Dwyer et al. 1995 |

References list:

Andros, J.D. and Garton, R.R. 1980. Acute lethality of copper, cadmium, and zinc to northern squawfish. Trans. Am. Fish. Soc. 109, 235-238.

Arthur, J.W. and Leonard, E.N. 1970. Effects of copper on Gammarus pseudolimnaeus, Physa integra, and Campeloma decisum in soft water. Journal of the Fisheries Research Board of Canada. 27, 1277-1283.

Baird, D.J., Barber, I., Bradley, M., Soares, A.M.V.M. and Calow, P.1991. A comparative study of genotype sensitivity to acute toxic stress using clones of Daphnia magna Straus. Ecotoxicol. Environ. Saf. 21, 257-265.

Belanger, S.E. and Cherry, D.S.1990. Interacting effects of pH acclimation, pH, and heavy metals on acute and chronic toxicity to Ceriodaphnia dubia (Cladocera). J. Crustacean Biol. 10, (2), 225-235.

Belanger, S.E., Farris, J.L. and Cherry, D.S. 1989. Effects of diet, water hardness, and population source on acute and chronic copper toxicity to Ceriodaphnia dubia. Arch. Environ. Contam. Toxicol. 18, (4), 601-611.

Benoit, D. A. 1975. Chronic effects of copper on survival, growth, and reproduction of the bluegill (Lepomis macrochirus). Trans. Am. Fish. Soc. 104, (2), 353-358.

Besser, J.M., Dwyer, F.J., Ingersoll, C.G. and Wang, N. 2001. Early life-stage toxicity of copper to endangered and surrogate fish species; U.S. Environmental Protection Agency: Washington, D.C.

Besser, J.M., Wang, N., Dwyer, F.J., Mayer, J.F.L.and Ingersoll, C. G. 2005. Assessing contaminant sensitivity of endangered and threatened aquatic species: Part II. Chronic toxicity of copper and pentachlorophenol to two endangered species and two surrogate species. Arch. Environ. Contam. Toxicol. 48, 155-165.

Besser, J.M., Mebane, C.A., Mount, D.R., Ivey, C.D., Kunz, J.L., Greer, I.E., et al. 2007. Sensitivity of mottled sculpins (Cottus bairdi) and rainbow trout (Oncorhynchus mykiss) to acute and chronic toxicity of cadmium, copper, and zinc. Environ. Toxicol. Chem. 26, (8), 1657-1665.

Birge, W.J., Benson, W.H. aand Black, J.A. 1983. The induction of tolerance to heavy metals in natural and laboratory populations of fish; PB84-111756; National Technical Information Service, Springfield, VA.

Benson, W.H. and Birge, W.J. 1985. Heavy metal tolerance and metallothionein induction in fathead minnows (Pimephales promelas): Results from field and laboratory investigations. Environ. Toxicol. Chem. 4 (2), 209-218.

Blaylock, B.G., Frank, M.L. and McCarthy, J.F. 1985. Comparative toxicity of copper and acridine to fish, Daphnia and algae. Environ. Toxicol. Chem. 4 (1), 63-71.

Buckley, J.A. 1983. Complexation of copper in the effluent of a sewage treatment plant and an estimate of its influence on toxicity to coho salmon. Water Res. 17 (12), 1929-1934.

Cacela, D., Hudson, R., Lipton, J., Marr, J., Podrabsky, T. and Welsh, P. 1996. Preliminary toxicological evaluation U.S. v. iron mountain mines, Inc.; Prepared by Hagler Bailly Consulting, Inc., Boulder, CO.

Cairns, J., Jr., Thompson, K. W. and Hendricks, A. C. 1981. Effects of fluctuating, sublethal applications of heavy metal solutions upon the gill ventilation response of bluegills (Lepomis macrochirus). U.S. Environmental Protection Agency: Duluth, MN.

Carlson, A.R., Nelson, H. and Hammermeister, D. 1986. Evaluation of site-specific criteria for copper and zinc: An integration of metal addition toxicity, effluent and receiving water toxicity, and ecological survey data. EPA-600/3-85-005; EPA/600/3-86/026; U.S. Environmental Protection Agency, Duluth, MN, USA.

Chakoumakos, C., Russo, R.C. and Thurston, R. V. 1979. The toxicity of copper to cutthroat trout (Salmo clarki) under different conditions of alkalinity, pH, and hardness. Environ. Sci. Technol.13, 213-219.

Chapman, G.A. 1975. Toxicity of copper, cadmium and zinc to pacific northwest salmonids; U.S. Environmental Protection Agency, Corvallis, OR.

Chapman, G.A. 1978. Toxicities of cadmium, copper, and zinc to four juvenile stages of chinook salmon and steelhead. Trans. Am. Fish. Soc.107 (6), 841-847.

Chapman, G.A. Letter to Charles E. Stephan, 1982. U.S. EPA, Duluth, MN, December 6; U.S. Environmental Protection Agency: Corvallis, OR.

Chapman, G.A. and McCrady, J.K., 1977. Copper toxicity: A question of form. In recent advances in fish toxicology; Tubb, R. A., Ed. EPA-600/3-77-085; National Technical Information Service: Springfield, VA: p132.

Chapman, G.A. and Stevens, D.G. 1978. Acutely lethal levels of cadmium, copper and zinc to adult male coho salmon and steelhead. Trans. Am. Fish. Soc.107, 837-840.

Chapman, G.A., Ota, S. and Recht, F. 1980. Effects of water hardness on the toxicity of metals to Daphnia magna; U.S. Environmental Protection Agency: Corvallis, OR.

Cooper, N. L., Bidwell, J. R.and Kumar, A., 2009. Toxicity of copper, lead, and zinc mixtures to Ceriodaphnia dubia and Daphnia carinata. Ecotoxicol. Environ. Saf. 72 (5), 1523-1528.

Cusimano, R.F., Brakke, D.F. and Chapman, G.A. 1986. Effects of pH on the toxicities of cadmium, copper, and zinc to steelhead trout (Salmo gairdneri). Can. J. Fish. Aquat. Sci. 43, 1497-1503.

De Schamphelaere, K.A.C. and Janssen, C. R. 2004. Effects of dissolved organic carbon concentration and source, pH, and water hardness on chronic toxicity of copper to Daphnia magna. Environ. Toxicol. Chem. 23 (5), 1115-1122.

De Schamphelaere, K.A.C., Vasconcelos, F.M., Tack, F.M.G., Allen, H.E. and Janssen, C.R. 2004. Effect of dissolved organic matter source on acute copper toxicity to Daphnia magna. Environ. Toxicol. Chem. 23 (5), 1248-1255.

Chen, R., Wu, M., Wang, W.B., et al. 2017. Prediction of copper toxicity for Cyclops and the effect of water quality parameters using biotic ligand model．Environ. Chem. 36 (4):716-723. (in Chinese)

Diamond, J.M., Koplish, D.E., McMahon III, J. and Rost, R. 1997. Evaluation of the water-effect ratio procedure for metals in a riverine system. Environ. Toxicol. Chem. 16 (3), 509-520.

Dwyer, J.F., Sappington, L.C., Buckler, D.R. and Jones, S.B. 1995. Use of surrogate species in assessing contaminant risk to endangered and threatened fishes; U.S. Environmental Protection Agency: Washington, D.C.

Dwyer, F.J., Hardesty, D.K., Henke, C.E., Ingersoll, C.G., Whites, D.W., Mount, D.R., et al. 1999. Assessing contaminant sensitivity of endangered and threatened species: toxicant classes; U.S. Environmental Protection Agency: Washington, D.C.

Erickson, R.J., Benoit, D.A. and Mattson, V.R. 1996. A prototype toxicity factors model for site-specific copper water quality criteria; U.S. Environmental Protection Agency: Duluth, MN.

GLEC (Great Lakes Environmental Center). 2006. Development of a copper criteria adjustment pProcedure for michigan’s upper peninsula waters; Traverse City, MI and Columbus, OH,

Hansen, J.A., Lipton, J., Welsh, P.G. and Cacela, D. 2000. Relationship between exposure duration, tissue residues, growth, and mortality in rainbow trout (Oncorhynchus mykiss) sub-chronically exposed to copper; Stratus Consulting Inc.

Hartwell, S.I., Jin, J.H., Cherry, D.S.and Cairns, J.J. 1989. Toxicity versus avoidance response of golden shiner, Notemigonus crysoleucas, to five metals. J. Fish Biol. 35 (3), 447-456.

Ingersoll, C.G. and Winner, R.W. 1982. Effect on Daphnia pulex (De Geer) of daily pulse exposures to copper or cadmium. Environ. Toxicol. Chem. 1, 321-327.

Janssen, C.R., Persoone, G. and Snell, T.W. 1994. Cyst-based toxicity tests. VIII. short-chronic toxicity tests with the freshwater rotifer Brachionus calyciflorus. Aquat. Toxicol. 28, 243-258.

Jorge, M.B., Loro, V.L., Bianchini, A., Wood, C.M. and Gillis, P.L. 2013. Mortality, bioaccumulation and physiological responses in juvenile freshwater mussels (Lampsilis siliquoidea) chronically exposed to copper. Aquat. Toxicol. 126, 137-147.

Keller, A.E. 2000. Personal communication to USEPA: Water quality and toxicity data for unpublished Unionid mussel tests. Memo to R. Pepin and C. Roberts, USEPA Region 5, Chicago, IL.

Keller, A.E.and Zam, S.G. 1991. The acute toxicity of selected metals to the freshwater mussel, Anodonta imbecilis. Environ. Toxicol. Chem. 10, 539-546.

Kosalwat, P.and Knight, AW. 1987. Acute toxicity of aqueous and substrate-bound copper to the midge, Chironomus decorus. Arch. Environ. Contam. Toxicol. 16, (3), 275-282.

Lind, D., Alto, K.and Chatterton, S. 1978. Regional copper-nickel study; Minnesota Environmental Quality Board.

Long, K.E.L., Van Genderen, E.J. and Klaine, S.J. 2004. The effects of low hardness and pH on copper toxicity to Daphnia magna. Environ. Toxicol. Chem. 23 (1), 72-75.

Lydy, M.J. and Wissing, T.E. 1988. Effect of sublethal concentrations of copper on the critical thermal maxima (CTMax) of the fantail (Etheostoma flabellare) and johnny (E. nigrum) darters. Aquat. Toxicol. 12 (4), 311-321.

Markich, S.J., Batley, G.E., Stauber, J.L., Rogers, N.J., Apte, S.C., Hyne, R.V., et al. 2005. Hardness corrections for copper are inappropriate for protecting sensitive freshwater biota. Chemosphere. 60 (1), 1-8.

McKim, J.M., Eaton, J.G. and Holcombe, G.W.1978. Metal toxicity to embryos and larvae of eight species of freshwater fish-II: Copper. Bull. Environ. Contam. Toxicol. 19, 608-616.

Meador, J.P. 1991. The interaction of pH, dissolved organic carbon, and total copper in the determination of ionic copper and toxicity. Aquat. Toxicol. 19, 13-32.

Meyer, J.S., Ranville, J.F., Pontasch, M., Gorsuch, J.W. and Adams, W.J. 2015. Acute toxicity of binary and ternary mixtures of Cd, Cu, and Zn to Daphnia magna. Environ. Toxicol. Chem.34 (4), 799-808.

Mudge, J.E., Northstrom, T.E., Jeane, G.S., Davis, W. and Hickam, J.L. 1993. Effect of varying environmental conditions on the toxicity of copper to salmon. In Environmental Toxicology and Risk Assessment, Gorsuch, J.W., Dwyer, F.J., Ingersoll, C.G. and LaPoint, T.W., Eds., American Society for Testing and Materials: Philadelphia, PA, pp 19-33.

Naddy, R.B., Cohen, A.S. and Stubblefield, W.A.2015. The interactive toxicity of cadmium, copper, and zinc to Ceriodaphnia dubia and rainbow trout (Oncorhynchus mykiss). Environ. Toxicol. Chem. 34 (4), 809-815.

Nebeker, A.V., Savonen, C., Baker, R.J. and McCrady, J.K. 1984. Effects of copper, nickel and zinc on the life cycle of the caddisfly Clistorina magnifica (Limnephilidae). Environ. Toxicol. Chem. 3, 645-649.

Nebeker, A.V., Cairns, M.A., Onjukka, S.T. and Titus, R.H. 1986a Effect of age on sensitivity of Daphnia magna to cadmium, copper and cyanazine. Environ. Toxicol. Chem. 5, 527-530.

Nebeker, A.V., Stinchfield, A., Savonen, C., Chapman, G.A.1986b. Effects of copper, nickel and zinc on three species of Oregon freshwater snails. Environ. Toxicol. Chem. 5, 807-811.

Oris, J.T., Winner, R.W. and Moore, M.V. 1991. A four-day survival and reproduction toxicity test for Ceriodaphnia dubia. Environ. Toxicol. Chem. 10 (2), 217-224.

Richards, V.L. and Beitinger, T.L.1995. Reciprocal influences of temperature and copper on survival of fathead minnows, Pimephales promelas. Bull. Environ. Contam. Toxicol. 55 (2), 230-236.

Ryan, A. C. 2005. Influence of dissolved organic matter source and chemical characteristics on acute copper toxicity. Ph.D. Dissertation, Clemson University, Clemson, SC.

Ryan, A.C., Van Genderen, E.J., Tomasso, J.R., Klaine and S.J.2004. Influence of natural organic matter source on copper toxicity to larval fathead minnows (Pimephales promelas): implications for the biotic ligand model. Environ. Toxicol. Chem. 23 (6), 1567-1574.

Ryan, A.C., Tomasso, J.R. and Klaine, S.J.2009. Influence of pH, hardness, dissolved organic carbon concentration, and dissolved organic matter source on the acute toxicity of copper to Daphnia magna in soft waters: implications for the biotic ligand model. Environ. Toxicol. Chem. 28 (8), 1663-1670.

Schubauer-Berigan, M.K., Dierkes, J.R., Monson, P.D. and Ankley, G.T.1993. pH-dependent toxicity of Cd, Cu, Ni, Pb and Zn to Ceriodaphnia dubia, Pimephales promelas, Hyalella azteca, and Lumbriculus variegatus. Environ. Toxicol. Chem. 12, 1261-1266.

Sciera, K.L., Isely, J.J., Tomasso, J.R.and Klaine, S.J.2004. Influence of multiple water-quality characteristics on copper toxicity to fathead minnows (Pimephales promelas). Environ. Toxicol. Chem. 23 (12), 2900-2905.

Seim, W.K., Curtis, L.R., Glenn, S.W. and Chapman, G.A.1984. Growth and survival of developing steelhead trout (Salmo gairdneri) continuously or intermittently exposed to copper. Can. J. Fish. Aquat. Sci. 41, 433-438.

Servizi, J.A.and Martens, D.W. 1978. International pacific salmon fisheries commission. Progress Rep. 39; New Westminster, British Columbia.

Spehar, R.L.and Fiandt, J.T.1986. Acute and chronic effects of water quality criteria-based metal mixtures on three aquatic species. Environ. Toxicol. Chem. 5, 917-931.

Villavicencio, G., Urrestarazu, P., Carvajal, C., De Schamphelaere, K.A.C., Janssen, C. R.and Torres, J. C., et al. 2005. Biotic ligand model prediction of copper toxicity to daphnids in a range of natural waters in Chile. Environ. Toxicol. Chem. 24 (5), 1287-1299.

Villavicencio, G., Urrestarazu, P., Arbildua, J.and Rodriguez, P. 2011. Application of an acute biotic ligand model to predict chronic copper toxicity to Daphnia magna in natural waters of Chile and reconstituted synthetic waters. Environ. Toxicol. Chem. 30 (10), 2319-2325.

Wang, C.Y., Chen, H, Zheng, B.H., et al. Bioavailability of Cu to medaka in Xiangjiang with biotic ligand model. Asian Journal of Ecotoxicology, 2013a, 8 (6): 998-1004 (in Chinese)

Wang, C.Y., Chen, H., Wu, K.B., Zhen, B.H. and An, L.H. 2013b. Infuluence of dissolved organic carbon composition on prediction of copper toxicity to Medaka by biotic ligand model. Asian Journal of Ecotoxicology. 8 (4): 629-623 (in Chinese)

Wang, C.Y., Chen, H., Wu, K.B., An, L. and Zheng, B. 2011. Application of the biotic ligand model to predict copper acute toxicity to Medaka fish in typical Chinese rivers. Water Sci. Tech. 64, 1277-1283

Wang, N., Mebane, C.A., Kunz, J.L., Ingersoll, C.G., May, T.W., Arnold, W.R., et al. 2009. Evaluation of acute copper toxicity to juvenile freshwater mussels (fatmucket, Lampsilis siliquoidea) in natural and reconstituted waters. Environ. Toxicol. Chem. 28 (11), 2367-2377.

Wang, N., Mebane, C.A., Kunz, J.L., Ingersoll, C.G., Brumbaugh, W.G., Santore, R.C., et al. 2011. Influence of dissolved organic carbon on toxicity of copper to a unionid mussel (Villosa iris) and a cladoceran (Ceriodaphnia dubia) in acute and chronic water exposures. Environ. Toxicol. Chem. 30 (9), 2115-2125.

Wang, N., Ingersoll, C.G., Dorman, R.A., Brumbaugh, W.G., Mebane, C.A., Kunz, J.L. and Hardesty, D.K. 2014. Chronic sensitivity of white sturgeon (Acipenser transmontanus) and rainbow trout (Oncorhynchus mykiss) to cadmium, copper, lead, or zinc in laboratory water-only exposures. Environ. Toxicol. Chem. 33 (10), 2246-2258.

Wang, W.B.  2015.Application of biotic ligand model in predicting copper acute toxicity to Carp (Cyprinidae) and Chlorella (Chlorella vulgaris). Kunming: Kunming university of science and technology. (in Chinese)

Wang, W.B., Chen, S., Wu, M., Zhao, J. 2015a. Predicting Copper Toxicity to Hypophthalmichthys molitrix and Ctenopharyngodon idellus Based on Biotic Ligand Model. Environ Sci. 35:3947-3951.  (In Chinese)

Warnick, S.L. and Bell, H.L.1969. The acute toxicity of some heavy metals to different species of aquatic insects. Journal of the Water Pollution Control Federation. 41, 280-284.

Welsh, P.G. 1996. Influence of dissolved organic carbon on the speciation, bioavailability and toxicity of metals to aquatic biota in soft water lakes. Ph.D. Dissertation, University of Waterloo, Waterloo, Ontario.

Welsh, P.G., Skidmore, J.F., Spry, D.J., Dixon, D.G., Hodson, P.V., Hutchinson, N.J., et al. 1993. Effect of pH and dissolved organic carbon on the toxicity of copper to larval fathead minnow (Pimephales promelas) in natural lake waters of low alkalinity. Can. J. Fish. Aquat. Sci. 50 (7), 1356-1362.

Welsh, P.G., Lipton, J., Chapman, G.A. and Podrabsky, T.L.2000. Relative importance of calcium and magnesium in hardness-based modification of copper toxicity. Environ. Toxicol. Chem. 19 (6), 1624-1631.

Winner, R.W. 1985. Bioaccumulation and toxicity of copper as affected by interactions between humic acid and water hardness. Water Res. 19 (4), 449-455.

Xu, X. 2019. Biotic ligand model development predicting acute Cu toxicity to the aglae scenndesmus obliquus construction and validation. Hangzhou: Zhejiang University of Technology. (In Chinese)

Zhou, T.Y., Cao, Y., Qin, LM., Zhang, YH., Zeng, H.H. and Yan, Z.G. 2014. Application of biotic ligand model for the acute toxicity of copper to Daphnia magna in water of Liaohe River and Taihu Lake. Environ. Sci., 235 (5),1962-1967. (in Chinese)