Supplementary material

Methods

The specific steps of PCA

PCA is an efficient linear dimensionality reduction method that maps a high-dimensional data sample X∈Rd to a low-dimensional space AX∈Rk using the orthogonal matrix A∈Rk×d, i.e., reducing the data from k dimensions to d dimensions, which has the effect of simplifying the data dimensionality (Gewers et al., 2022).

In this study, the global level of urban flooding X was expressed as:

X=x1T⋮xnT #1

where xiT is the time varying level data transpose matrix of manholes or surface nodes numbered i, n is the number of manholes or surface nodes. Using PCA analysis, this can be downscaled to Y:

Y=y1T⋮ymT=AX #2

where yiT is the transpose matrix of values of the principal components numbered i over time, m denotes the number of principal components, A is the orthogonal matrix calculated by using PCA.

For each principal component PCj, the water level at each point in the global level of urban flooding xi can all find a load factor LDi,j corresponding to it:

PCj=LDi,j∙Nxi+cosntant #3

where Nxi is water level at the ith point after normalization, LDi,j is numerically equal to the correlation coefficient between principal component PCj and water level, The large the LDi,j, the stronger correlation between xi and PCj.

Through PCA analysis, the water level data containing m conduit and n urban surface nodes can be extracted from i principal components were used to explain a% of the total sample variance. By sorting the loading factors, we can obtain the water level of i nodes that best represent these i principal components. As the water level of each point of the drainage system is not completely isolated, but interacts with each other, especially the water level of neighboring point has a strong correlation due to the existence of hydraulic connection. By estimating the level of i nodes from PCA analysis, most of the level information of the drainage system can be obtained by machine learning modelling, thus simplifying the machine learning modelling process. In addition, similarly by PCA analysis, the j points that best represent the j principal components explaining b% of the total variance can be found as the monitoring sites

The specific calculation process of urban water level deduction

yi=jwi,jxi,j #4

where yi is the inferred water level of the ith point not include in the monitoring sites, xi,j denotes the jth independent variable affecting yi, wi,j are the weights obtained by XGBoost.

The specific calculation process of water level prediction of monitoring sites

yt+1i=Fyti,Xt+1,Xt,…,Xt-n#6

where yt+1i is the predicted value of water level at the future t+1th time step for node i, Xt+1 is the predicted value of water level driving variables at the t+1th time step, including rainfall intensity, cumulative rainfall, rainfall duration, etc. It can be obtained from the rainfall time range prediction made by the rainfall nowcasting prediction technology, and also includes the operation status of river water conservancy structures and the operation status of rainwater storage facilities, which can be preset according to the actual flood control scheduling work arrangement. F∙ is the mapping function built using the XGBoost. n is the historical flood level driving variable, determined by F∙ loss function l during the training process:

lyi,yi=yi-yi2#7

where yi is the measured values of flood level, yi is the prediction values of flood level. n takes the value that make l the smallest value:

n=argminlyi,yi#8

Fig. S1 Hydraulic model evaluation at three rainfall events: rainfall A (a-b), rainfall B (c-d), and rainfall C (e-f).

Table S1 PCA results of urban flood in study area

| Principal component | Eigenvalue |  |  | Eigenvalue after rotation |  |  | Point position corresponding to the maximum load |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | Explained variance | Cumulative explained variance | Total | Explained variance | Cumulative explained variance |  |
| 1 | 927.477 | 77.290 | 77.290 | 667.508 | 55.626 | 55.626 | M390 |
| 2 | 130.251 | 10.854 | 88.144 | 219.297 | 18.275 | 73.900 | M551 |
| 3 | 62.520 | 5.210 | 93.354 | 148.941 | 12.412 | 86.312 | M1155 |
| 4 | 22.954 | 1.913 | 95.267 | 90.591 | 7.549 | 93.861 | M411 |
| … | … | … | … | … | … | … | … |
| 14 | 1.061 | 0.088 | 99.260 | 1.231 | 0.103 | 99.260 | M1058 |
| … | … | … | … | … | … | … | … |
| 157 | 0.000 | 3.630E-05 | 99.999 | 0.001 | 6.486E-05 | 99.999 | M1122 |
| 158 | 0.000 | 3.509E-05 | 99.999 |  |  |  |  |
| … | … | … | … |  |  |  |  |
| 23465 | 0.000 | 0.000 | 100.000 |  |  |  |  |

Table S2 Results of ten simulations of the LGD model and hydraulic model for validation

|  | Metrics | T1 | T2 | T3 | T4 | T5 | T6 | T7 | T8 | T9 | T10 | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban surface | MAXE | 0.109 | 0.162 | 0.118 | 0.157 | 0.148 | 0.219 | 0.133 | 0.198 | 0.156 | 0.239 | 0.164 |
|  | MAE | 0.051 | 0.065 | 0.067 | 0.079 | 0.069 | 0.145 | 0.087 | 0.136 | 0.118 | 0.192 | 0.101 |
|  | RMSE | 0.074 | 0.081 | 0.097 | 0.104 | 0.098 | 0.236 | 0.128 | 0.218 | 0.156 | 0.252 | 0.144 |
|  | R2 | 0.952 | 0.947 | 0.932 | 0.927 | 0.929 | 0.863 | 0.915 | 0.877 | 0.894 | 0.852 | 0.909 |
| Conduit system | MAXE | 0.142 | 0.178 | 0.122 | 0.195 | 0.193 | 0.361 | 0.176 | 0.264 | 0.172 | 0.373 | 0.218 |
|  | MAE | 0.064 | 0.082 | 0.065 | 0.105 | 0.104 | 0.169 | 0.111 | 0.161 | 0.138 | 0.239 | 0.124 |
|  | RMSE | 0.088 | 0.103 | 0.089 | 0.131 | 0.126 | 0.254 | 0.151 | 0.230 | 0.166 | 0.286 | 0.162 |
|  | R2 | 0.945 | 0.923 | 0.940 | 0.915 | 0.919 | 0.848 | 0.893 | 0.880 | 0.888 | 0.836 | 0.899 |

Table S3 Total calculation time of hydraulic model, XGBoost and UFP-RD

| Model | GPU | Calculation time |
| --- | --- | --- |
| Hydraulic model | Yes | 318.52 |
|  | No | 350.24 |
| XGBoost | Yes | 0.26 |
|  | No | 0.28 |
| UFP-RD | Yes | 0.25 |
|  | No | 0.29 |

References

Gewers, F. L., Ferreira, G. R., Arruda, H. F. D., Silva, F. N., Comin, C. H., Amancio, D. R., & Costa, L. D. F. (2022). Principal Component Analysis: A Natural Approach to Data Exploration. ACM Computing Surveys, 54, 1–34.