# SUPPLEMENTARY MATERIAL

Article: An approach for comprehensive Life Cycle Assessment and effective eco design of IoT systems: data-driven modeling of reference flows

Journal: Discover Internet of Things

Authors: Ernesto Quisbert-Trujillo1* and Panagiota Morfouli2

1* G-SCOP, Univ. Grenoble Alpes - Grenoble INP, Grenoble, 38000, France.

2 IMEP-LAHC, Univ. Grenoble Alpes - Grenoble INP PHELMA, Grenoble, 38016, France.

Corresponding author(s). E-mail(s): ernesto.quisbert-trujillo@grenoble-inp.fr

Contributing authors: panagiota.morfouli@phelma.grenoble-inp.fr

## Determining the theoretical number of batteries for the flowmeter and its impact in two years

- Obtaining the maximal size of the LoRa packet

By considering a spreading factor of 7, a bandwidth of 125 kHz, a preamble of 8 symbols (Preamble), a maximal frame payload (Framepayload) of 242 Bytes of sensor data application (according to the unfavorable scenario for the case study, presented in section 3), and all the additional headers and footers of a typical LoRa frame (including the Headers and Footers in the physical layer (phyHF), the MAC layer (MACHF) and the Header of the application layer (FrameH)), the maximal size of a LoRa packet is 282 bytes, as showed below.

Max size Lora packet= Preamble+ PhyHF+ MACHF+ FrameH+FramePayload

Max LoRa data frame size=7 bytes+5 Bytes+5 Bytes+23 Bytes+242 Bytes=282 Bytes

- Obtaining tTx and ts (LoRa module)

By considering equation 2, the maximal size of a LoRa packet (282 bytes or 2256 bits), one transmission event every 3 minutes (or 180 seconds) and the maximal bitrate capacity of the LoRa transceiver (in bits per second) of the flowmeter in a bandwidth of 125 kHz and a spreading factor of 7, the value of tTx is 0,41 seconds (or 1,14  10-4 hours), and the value of ts is 179,59 seconds (or 4,98  10-2 hours) as it is showed below.

tTx=2256 bits5470 bps=0,41 s		;	ts=180 s-0,41 s=179,59 s

- Obtaining the battery depletion factors for the transmission and sleeping modes (BDTx, BDs)

By considering the nominal capacity of a 9V PP3-typed battery (1200 mAh), a performance rate of the battery of 95%, the current consumption of the LoRa transceiver in transmission mode (44,5 mA) and in sleeping mode (0,0016 mA), the value of BDTx and BDs is 1 battery/25,6 hours and 1 battery/712500 hours respectively, as it is showed below.

BDTx=1200 mAh44,5 mA×95%=25,6 h   or  "1 battery for 25,6 hours in continuous transmission state"

BDs=1200 mAh0,0016 mA×95%=712500 h   or  "1 battery for 712500 hours in continuous sleeping state"

- Obtaining the theoretical number of batteries for the flowmeter in two years (Bfm)

By considering equation 1, the values of tTx and ts obtained above (in hours), and a total LoRa transmission events (TTELoRa) of 350333 (in two years), the theoretical number of batteries used by the flowmeter in two years is 2 as it is showed below.

Bfm=350333  1 Battery25 hours∙1,14E˗4 hours+1 Battery712500 hours∙4,98E˗2 hours=1,62 batteries ~ 2 batteries

- Obtaining the environmental impact of the reference flow of the flowmeter

If producing 1 kg of 9V 3PP-type sized batteries generates 6,7576 Kg CO2-eq, the impact of producing one 6-cell-typed 9V battery (whose typical weights around 33,9 grams or 3,39 × 10-2 Kg) is 0,229 Kg CO2-eq. Consequently, the impact of the reference flow of the flowmeter (2 batteries) is 0,458 Kg CO2-eq.

## Determining the theoretical data volume, reference flows, and associated impact of the internet and cloud infrastructures

- Determining the packet size of a HTTP POST request

By assuming a maximal payload of 1410 Bytes (which is equivalent to 5 LoRa packets accumulated in 15 minutes), a maximal 802.11 header format of 30 bytes; and a maximal IP header format of 20 bytes and a maximal TCP header format of 20 bytes, the packet size of a HTTP POST request (for the regular operational mode) is 1480 bytes as it is showed below

Packet size of a HTTP POST request=1410 Bytes+30 Bytes+20 Bytes+20 Bytes=1480 Bytes

- Determining the packet traffic of the case study

By considering a connection-oriented communication (TCP/IP) between the gateway and the cloud server, and between the smartphone and the gateway, figure A1 shows the assumed packet traffic for the regular and consultation operational modes.

Fig. A1 TCP/IP packets (horizontal arrows) flowing in a single transmission between an edge device (flowmeter or smartphone) and the cloud server.

At the top of figure A1, the TCP/IP three-way handshake mechanism is showed (composed of 3 empty TCP/IP packets, each of 70 bytes including only respective flags in headers; and TCP, IP and 802.11 protocol overheads). In the middle, one observes the packets sent to the cloud server (a typical HTTP POST request, within the regular operation mode of the IoT system; or a HTTP GET request from a smartphone to the cloud server, within the consultation operational mode). At the bottom, the TCP teardown mechanism is showed (composed of 4 empty TCP/IP packets, each of 70 bytes including only respective flags in headers and TCP, IP and 802.11 protocol overheads). We assumed a minimal payload of one byte for the HTTP GET requests and respectively responses, with a maximal protocol overhead (70 bytes). Vertical arrows show the sequence for stablishing and closing connections.

- Determining the data volume circulating in the internet and cloud infrastructures

By taking into account the packet traffic showed and described in the part b above, a total number of transmission events of 70066 for the regular operational mode, a total number of transmission events of 17520 for the consultation operational mode, and the hypothetical size of every type of packets (see table A1 below), the total internet traffic of the case study amounts to 0,1434 GB in two years.

| Operational mode | Packet type | Packets | Size packet (bytes) | in 70066 transmissions (bytes) | in 17520 transmissions (bytes) | Total (GB) |
| --- | --- | --- | --- | --- | --- | --- |
| Regular | TCP (three-way handshake) | 3 | 70 | 14713860 |  | 0,1331 |
|  | HTTP post request | 1 | 1480 | 103697680 |  |  |
|  | TCP Ack | 1 | 70 | 4904620 |  |  |
|  | TCP (teardown) | 4 | 70 | 19618480 |  |  |
| Consultation | TCP (three-way handshake) | 3 | 70 |  | 3679200 | 0,0103 |
|  | HTTP GET request | 1 | 71 |  | 1243920 |  |
|  | HTTP reponse | 1 | 71 |  | 1243920 |  |
|  | TCP (teardown) | 4 | 70 |  | 4905600 |  |
|  |  |  |  |  |  | 0,1434 |

Table A1 Data volume circulating in the internet and cloud infrastructures

- Obtaining the reference flow and the associated impact of the data volume circulating in the internet and cloud resources

by considering the data volume in table A1, and the electricity intensity of 0,15 kWh/GB for the internet use and 0,14 kWh/GB for the clod server use, the reference flow of the internet networks and cloud server (in terms of energy) is 0,0215 kWh and 0,02 kWh respectively, as it is showed below.

Ref. flow internet=0,15kWhGB ×0,1434 GB=0,0215 kWh

Ref. flow Cloud server=0,14kWhGB ×0,1434 GB=0,02 kWh

In this way, the impact of the internet infrastructure is 0,017 Kg CO2-eq and the impact of the cloud server is 0,016 Kg CO2-eq (assuming that 1 kWh in a global electricity mix generates 0,7749 Kg CO2-eq).

## Determining the reference flow and impact of the smartphone during two years

By assuming that the user turns on the WiFi module during one minute to execute a online consultation hourly (which amount to 292 hours in two years), and by considering the power consumption of the WiFi module in transmission mode (1,62 × 10-3 kW), in receiving mode (1,375 × 10-3 kW), and a performance rate of the battery of 95%; the total energy consumption of the smartphone for sending HTTP GET request to the cloud server is 0,501 kWh and for receiving water consumptions stats is 0,423 kWh, as it is showed below.

1,62E˗3 kW ×292h95%=0,501 kWh		;	1,375E˗3 kW ×292h95%=0,423 kWh

In this way, the aggregated impact of the smartphone in two years is 0,045 Kg CO2-eq (assuming that 1 kWh in a French electricity mix generates 4,89 × 10-2 Kg CO2-eq).

## Packet analysis of sniffer A (local data traffic between the flowmeter and the gateway)

Figure A2 shows a sample of the local data traffic between the flowmeter and the gateway, captured by the sniffer A during 1 hour (from 21h06 to 22h06, in minute resolution).

Fig. A2 LoRa traffic between the flowmeter and the gateway (excluding UDP headers). Each point represents the aggregated data size of LoRa transmissions in one minute.

The table A2 shows the LoRa transmissions tracked for the sample of figure A2. The data traffic happens in the 868.1 frequency channel. The IP source 172.17.0.2 correspond to the Docker instance that runs our GNU-radio implementation and the IP destination 172.17.0.1 correspond to the IP addresses of the localhost that runs the Wireshark instance. For facilitating the visualization, a filter in the latter is applied, so that only UDP packets containing the data application of the case study are showed. The actual data application (the LoRa packet without the UDP headers) is show in the “Info” column (Len = X bytes). The total size of LoRa packets is obtained from the sum of the individual transmissions in minute resolution. For example, the first two packets happening on 21:06:18 and 21:06:18 are consider a complete LoRa packet whose size is 57 Bytes (28 Bytes + 29 Bytes). In this sense, the complete LoRa packets are highlighted in alternate tones.

| capture time | Source | Destination | Protocol | Length | Info |
| --- | --- | --- | --- | --- | --- |
| 21:06:18 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:06:18 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:09:14 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:09:14 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:12:12 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:12:12 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:15:17 | 172.17.0.2 | 172.17.0.1 | UDP | 72 | 34537 > 40868 Len=30 |
| 21:15:17 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:15:17 | 172.17.0.2 | 172.17.0.1 | UDP | 85 | 34537 > 40868 Len=43 |
| 21:15:17 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:18:15 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:18:15 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:21:19 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:24:21 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:24:21 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:30:19 | 172.17.0.2 | 172.17.0.1 | UDP | 72 | 34537 > 40868 Len=30 |
| 21:30:19 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:30:19 | 172.17.0.2 | 172.17.0.1 | UDP | 74 | 34537 > 40868 Len=32 |
| 21:30:19 | 172.17.0.2 | 172.17.0.1 | UDP | 85 | 34537 > 40868 Len=43 |
| 21:30:20 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:33:23 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:33:23 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:36:19 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:42:23 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:45:20 | 172.17.0.2 | 172.17.0.1 | UDP | 72 | 34537 > 40868 Len=30 |
| 21:45:20 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:45:21 | 172.17.0.2 | 172.17.0.1 | UDP | 85 | 34537 > 40868 Len=43 |
| 21:45:21 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:48:18 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:51:18 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:51:18 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:54:16 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 21:54:16 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 21:57:11 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 22:00:08 | 172.17.0.2 | 172.17.0.1 | UDP | 72 | 34537 > 40868 Len=30 |
| 22:00:08 | 172.17.0.2 | 172.17.0.1 | UDP | 78 | 34537 > 40868 Len=36 |
| 22:00:09 | 172.17.0.2 | 172.17.0.1 | UDP | 85 | 34537 > 40868 Len=43 |
| 22:03:05 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 22:03:05 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |
| 22:06:03 | 172.17.0.2 | 172.17.0.1 | UDP | 70 | 34537 > 40868 Len=28 |
| 22:06:03 | 172.17.0.2 | 172.17.0.1 | UDP | 71 | 34537 > 40868 Len=29 |

Table A2 LoRa transmissions tracked for the sample of figure A4

## Packet analysis of sniffer B (internet traffic between the gateway and the cloud server)

Figure A3 shows a sample of the WiFi traffic of the IoT system (for the regular and the consultation operational modes), occurred in approximately two minutes (from 21h58m03s to 21h59m51s, in second resolution).

Fig. A3 Data flow within the internet network and cloud server. Each point represent the aggregated data size of DNS, TCP and HTTP packets occurred in one second.

The table A3 shows the packets tracked for the sample of figure A3. The traffic type identified was DNS requests from the gateway (IP address 192.168.137.47) to the IAP device (IP address 192.168.137.1) and roundtrip TCP and HTTP packets between the gateway and the cloud server (www.mySolem.com). The green cells shows an instance of a regular transmission event for uploading the counts of the flowmeter in the cloud server (HTTP post message in bold text). It start by a DNS request to obtain the IP address of the cloud server (www.mySolem.com) and finish by a TCP teardown routine (the table shows red frames to highlight this behavior). The orange cells show the mechanisms that are activated in the gateway and the cloud server when the user consults his or her water consumption (the three orange alternate sections show the three high peaks seen in figure A3). It is assumed that the gateway send this information by the HTTP post messages (showed by the red bold text). Sometimes the IP source changes as the gateway reassign a new IP in the local network.

| capture time | Source | Destination | Protocol | Length | Info |
| --- | --- | --- | --- | --- | --- |
| 21:58:03 | mysolem.com | 192.168.137.47 | HTTP | 466 | HTTP/1.1 408 Request Timeout |
| 21:58:03 | 192.168.137.47 | mysolem.com | TCP | 54 | 33626 > 80 [ACK] Seq=147 Ack=413 Win=2508 Len=0 |
| 21:58:03 | 192.168.137.47 | mysolem.com | TCP | 54 | 33626 > 80 [FIN, ACK] Seq=147 Ack=413 Win=2508 Len=0 |
| 21:58:03 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 33626 [FIN, ACK] Seq=413 Ack=148 Win=30016 Len=0 |
| 21:58:03 | 192.168.137.47 | mysolem.com | TCP | 54 | 33626 > 80 [RST, ACK] Seq=148 Ack=414 Win=2920 Len=0 |
| 21:58:04 | 192.168.137.47 | 192.168.137.1 | DNS | 71 | Standard query 0xf6c7 A mysolem.com |
| 21:58:04 | 192.168.137.1 | 192.168.137.47 | DNS | 87 | Standard query response 0xf6c7 A mysolem.com A 51.159.17.150 |
| 21:58:04 | 192.168.137.47 | mysolem.com | TCP | 58 | 9167 > 80 [SYN] Seq=0 Win=2920 Len=0 MSS=1460 |
| 21:58:04 | mysolem.com | 192.168.137.47 | TCP | 58 | 80 > 9167 [SYN, ACK] Seq=0 Ack=1 Win=29200 Len=0 MSS=1460 |
| 21:58:04 | 192.168.137.47 | mysolem.com | TCP | 54 | 9167 > 80 [ACK] Seq=1 Ack=1 Win=2920 Len=0 |
| 21:58:04 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:58:04 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 9167 [ACK] Seq=1 Ack=147 Win=30016 Len=0 |
| 21:58:22 | mysolem.com | 192.168.137.47 | HTTP | 466 | HTTP/1.1 408 Request Timeout |
| 21:58:22 | 192.168.137.47 | mysolem.com | TCP | 54 | 9167 > 80 [ACK] Seq=147 Ack=413 Win=2508 Len=0 |
| 21:58:22 | 192.168.137.47 | mysolem.com | TCP | 54 | 9167 > 80 [FIN, ACK] Seq=147 Ack=413 Win=2508 Len=0 |
| 21:58:22 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 9167 [FIN, ACK] Seq=413 Ack=148 Win=30016 Len=0 |
| 21:58:22 | 192.168.137.47 | mysolem.com | TCP | 54 | 9167 > 80 [ACK] Seq=148 Ack=414 Win=2507 Len=0 |
| 21:58:23 | 192.168.137.47 | 192.168.137.1 | DNS | 71 | Standard query 0x2555 A mysolem.com |
| 21:58:23 | 192.168.137.1 | 192.168.137.47 | DNS | 87 | Standard query response 0x2555 A mysolem.com A 51.159.17.150 |
| 21:58:23 | 192.168.137.47 | mysolem.com | TCP | 58 | 12306 > 80 [SYN] Seq=0 Win=2920 Len=0 MSS=1460 |
| 21:58:24 | 192.168.137.47 | mysolem.com | TCP | 58 | [TCP Retransmission] 12306 > 80 [SYN] Seq=0 Win=2920 Len=0 MSS=1460 |
| 21:58:24 | mysolem.com | 192.168.137.47 | TCP | 58 | 80 > 12306 [SYN, ACK] Seq=0 Ack=1 Win=29200 Len=0 MSS=1460 |
| 21:58:24 | 192.168.137.47 | mysolem.com | TCP | 54 | 12306 > 80 [ACK] Seq=1 Ack=1 Win=2920 Len=0 |
| 21:58:24 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:58:24 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 12306 [ACK] Seq=1 Ack=147 Win=30016 Len=0 |
| 21:58:42 | mysolem.com | 192.168.137.47 | HTTP | 468 | HTTP/1.1 408 Request Timeout |
| 21:58:42 | 192.168.137.47 | mysolem.com | TCP | 54 | 12306 > 80 [FIN, ACK] Seq=147 Ack=415 Win=2506 Len=0 |
| 21:58:42 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 12306 [FIN, ACK] Seq=415 Ack=148 Win=30016 Len=0 |
| 21:58:42 | 192.168.137.47 | mysolem.com | TCP | 54 | 12306 > 80 [ACK] Seq=148 Ack=416 Win=2505 Len=0 |
| 21:58:43 | 192.168.137.47 | 192.168.137.1 | DNS | 71 | Standard query 0x21d5 A mysolem.com |
| 21:58:43 | 192.168.137.1 | 192.168.137.47 | DNS | 87 | Standard query response 0x21d5 A mysolem.com A 51.159.17.150 |
| 21:58:43 | 192.168.137.47 | mysolem.com | TCP | 58 | 9468 > 80 [SYN] Seq=0 Win=2920 Len=0 MSS=1460 |
| 21:58:43 | mysolem.com | 192.168.137.47 | TCP | 58 | 80 > 9468 [SYN, ACK] Seq=0 Ack=1 Win=29200 Len=0 MSS=1460 |
| 21:58:43 | 192.168.137.47 | mysolem.com | TCP | 54 | 9468 > 80 [ACK] Seq=1 Ack=1 Win=2920 Len=0 |
| 21:58:43 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:58:43 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 9468 [ACK] Seq=1 Ack=147 Win=30016 Len=0 |
| 21:59:01 | mysolem.com | 192.168.137.47 | HTTP | 466 | HTTP/1.1 408 Request Timeout |
| 21:59:01 | 192.168.137.47 | mysolem.com | TCP | 54 | 9468 > 80 [FIN, ACK] Seq=147 Ack=413 Win=2508 Len=0 |
| 21:59:01 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 9468 [FIN, ACK] Seq=413 Ack=148 Win=30016 Len=0 |
| 21:59:01 | 192.168.137.47 | mysolem.com | TCP | 54 | 9468 > 80 [ACK] Seq=148 Ack=414 Win=2507 Len=0 |
| 21:59:02 | 192.168.137.47 | 192.168.137.1 | DNS | 71 | Standard query 0xfa18 A mysolem.com |
| 21:59:02 | 192.168.137.1 | 192.168.137.47 | DNS | 87 | Standard query response 0xfa18 A mysolem.com A 51.159.17.150 |
| 21:59:02 | 192.168.137.47 | mysolem.com | TCP | 58 | 7695 > 80 [SYN] Seq=0 Win=2920 Len=0 MSS=1460 |
| 21:59:03 | 192.168.137.47 | mysolem.com | TCP | 58 | [TCP Retransmission] 7695 > 80 [SYN] Seq=0 Win=2920 Len=0 MSS=1460 |
| 21:59:03 | mysolem.com | 192.168.137.47 | TCP | 58 | 80 > 7695 [SYN, ACK] Seq=0 Ack=1 Win=29200 Len=0 MSS=1460 |
| 21:59:03 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=1 Ack=1 Win=2920 Len=0 |
| 21:59:03 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:03 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=1 Ack=147 Win=30016 Len=0 |
| 21:59:13 | mysolem.com | 192.168.137.47 | HTTP | 551 | HTTP/1.1 200 OK (application/json) |
| 21:59:13 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=147 Ack=498 Win=2423 Len=0 |
| 21:59:13 | 192.168.137.47 | mysolem.com | HTTP | 390 | POST /v2/response/0500000506F80001/B0ACD6FAE59147DFBDE8ECD2B34085BB HTTP/1.1 (application/json) |
| 21:59:13 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=498 Ack=483 Win=31088 Len=0 |
| 21:59:13 | mysolem.com | 192.168.137.47 | HTTP | 420 | HTTP/1.1 200 OK |
| 21:59:13 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:13 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=864 Ack=629 Win=32160 Len=0 |
| 21:59:16 | mysolem.com | 192.168.137.47 | HTTP | 570 | HTTP/1.1 200 OK (application/json) |
| 21:59:17 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=629 Ack=1380 Win=1541 Len=0 |
| 21:59:17 | 192.168.137.47 | mysolem.com | HTTP | 346 | POST /v2/response/0500000506F80001/CF5856A5567A4AA7B2E7DB0A9CAC8A60 HTTP/1.1 (application/json) |
| 21:59:17 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=1380 Ack=921 Win=33232 Len=0 |
| 21:59:17 | mysolem.com | 192.168.137.47 | HTTP | 420 | HTTP/1.1 200 OK |
| 21:59:17 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=921 Ack=1746 Win=2920 Len=0 |
| 21:59:17 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:17 | mysolem.com | 192.168.137.47 | HTTP | 572 | HTTP/1.1 200 OK (application/json) |
| 21:59:17 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=1067 Ack=2264 Win=2402 Len=0 |
| 21:59:17 | 192.168.137.47 | mysolem.com | HTTP | 787 | POST /v2/response/0500000506F80001/74F9D612DF944AE3A97A6139602EBA7E HTTP/1.1 (application/json) |
| 21:59:17 | mysolem.com | 192.168.137.47 | HTTP | 420 | HTTP/1.1 200 OK |
| 21:59:17 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=1800 Ack=2630 Win=2036 Len=0 |
| 21:59:17 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:17 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=2630 Ack=1946 Win=37383 Len=0 |
| 21:59:17 | mysolem.com | 192.168.137.47 | HTTP | 563 | HTTP/1.1 200 OK (application/json) |
| 21:59:17 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=1946 Ack=3139 Win=1527 Len=0 |
| 21:59:17 | 192.168.137.47 | mysolem.com | HTTP | 398 | POST /v2/response/0500000506F80001/50F7A8B8433F4EC6A4FF8ECE619233DA HTTP/1.1 (application/json) |
| 21:59:17 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=3139 Ack=2290 Win=38849 Len=0 |
| 21:59:17 | mysolem.com | 192.168.137.47 | HTTP | 420 | HTTP/1.1 200 OK |
| 21:59:17 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=2290 Ack=3505 Win=2920 Len=0 |
| 21:59:17 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:18 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=3505 Ack=2436 Win=40315 Len=0 |
| 21:59:30 | mysolem.com | 192.168.137.47 | HTTP | 570 | HTTP/1.1 200 OK (application/json) |
| 21:59:30 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=2436 Ack=4021 Win=2404 Len=0 |
| 21:59:30 | 192.168.137.47 | mysolem.com | HTTP | 346 | POST /v2/response/0500000506F80001/57218E3CBDC74FDEA213E444A1BDE558 HTTP/1.1 (application/json) |
| 21:59:30 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=4021 Ack=2728 Win=41781 Len=0 |
| 21:59:30 | mysolem.com | 192.168.137.47 | HTTP | 424 | HTTP/1.1 200 OK |
| 21:59:31 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:31 | mysolem.com | 192.168.137.47 | HTTP | 574 | HTTP/1.1 200 OK (application/json) |
| 21:59:31 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=2874 Ack=4911 Win=1514 Len=0 |
| 21:59:31 | 192.168.137.47 | mysolem.com | HTTP | 787 | POST /v2/response/0500000506F80001/0C572E37CA0148BA82BE9D4BD62BA48D HTTP/1.1 (application/json) |
| 21:59:31 | mysolem.com | 192.168.137.47 | HTTP | 420 | HTTP/1.1 200 OK |
| 21:59:31 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=3607 Ack=5277 Win=2920 Len=0 |
| 21:59:31 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:31 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=5277 Ack=3753 Win=46179 Len=0 |
| 21:59:31 | mysolem.com | 192.168.137.47 | HTTP | 565 | HTTP/1.1 200 OK (application/json) |
| 21:59:31 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=3753 Ack=5788 Win=2409 Len=0 |
| 21:59:31 | 192.168.137.47 | mysolem.com | HTTP | 398 | POST /v2/response/0500000506F80001/618B4FB6FFA845F7A815CB3F095168FC HTTP/1.1 (application/json) |
| 21:59:31 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=5788 Ack=4097 Win=47645 Len=0 |
| 21:59:31 | mysolem.com | 192.168.137.47 | HTTP | 422 | HTTP/1.1 200 OK |
| 21:59:31 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=4097 Ack=6156 Win=2041 Len=0 |
| 21:59:31 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:31 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [ACK] Seq=6156 Ack=4243 Win=49111 Len=0 |
| 21:59:49 | mysolem.com | 192.168.137.47 | HTTP | 464 | HTTP/1.1 408 Request Timeout |
| 21:59:50 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=4243 Ack=6566 Win=1631 Len=0 |
| 21:59:50 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [FIN, ACK] Seq=4243 Ack=6566 Win=1631 Len=0 |
| 21:59:50 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 7695 [FIN, ACK] Seq=6566 Ack=4244 Win=49111 Len=0 |
| 21:59:50 | 192.168.137.47 | mysolem.com | TCP | 54 | 7695 > 80 [ACK] Seq=4244 Ack=6567 Win=1630 Len=0 |
| 21:59:50 | 192.168.137.47 | 192.168.137.1 | DNS | 71 | Standard query 0x4a0e A mysolem.com |
| 21:59:50 | 192.168.137.1 | 192.168.137.47 | DNS | 87 | Standard query response 0x4a0e A mysolem.com A 51.159.17.150 |
| 21:59:50 | 192.168.137.47 | mysolem.com | TCP | 58 | 47749 > 80 [SYN] Seq=0 Win=2920 Len=0 MSS=1460 |
| 21:59:50 | mysolem.com | 192.168.137.47 | TCP | 58 | 80 > 47749 [SYN, ACK] Seq=0 Ack=1 Win=29200 Len=0 MSS=1460 |
| 21:59:50 | 192.168.137.47 | mysolem.com | TCP | 54 | 47749 > 80 [ACK] Seq=1 Ack=1 Win=2920 Len=0 |
| 21:59:50 | 192.168.137.47 | mysolem.com | HTTP | 200 | POST /v2/periodic/0500000506F80001/ HTTP/1.1 (application/json)Continuation |
| 21:59:50 | mysolem.com | 192.168.137.47 | TCP | 54 | 80 > 47749 [ACK] Seq=1 Ack=147 Win=30016 Len=0 |
| 22:00:08 | mysolem.com | 192.168.137.47 | HTTP | 468 | HTTP/1.1 408 Request Timeout |

Table A3 packets tracked for the sample of figure A3

## Determining the real number of batteries for the flowmeter and its impact in two years

- Obtaining tTx and ts (LoRa module)

By considering equation 2, the real size of a LoRa packet (57 bytes) in bits (456 bits), a LoRa transmission every 3 minutes (180 seconds), and the maximal bitrate capacity of the LoRa transceiver in a bandwidth of 125 kHz and a spreading factor of 7 (5470 bps), the value of tTx is 0,083 seconds (according to equation 1) and the value of ts is 179,92 seconds (as it is showed below).

tTx=456 bits5470 bps=0,083 s		;	ts=180 s-0,083 s=179,92 s

- Computing the number of the batteries for the flowmeter in two years (Bfm)

By considering equation 1, the real values of tTx and ts above (in hours), the same values for BDTx (1 battery for 25,6 hours in continuous transmission state) and for BDs (1 battery for712500 hours in sleeping state) of the flowmeter’s LoRa module (as obtained in the part c of the first section) and a total LoRa transmission events (TTELoRa) of 350333 transmission over two years, the real number batteries used by the flowmeter is 1 as is showed below.

Bfm=350333  1 Battery25 hours∙2,30E˗5 hours+1 Battery712500 hours∙4,99E˗2 hours=0,34 batteries ~ 1 battery

- Obtaining the environmental impact of the reference flow of the flowmeter

By considering that producing 1 kg of 9V 3PP-type sized batteries generates 6,7576 Kg CO2-eq, the impact of producing one 6-cell-typed 9V battery (whose typical weights around 33,9 grams or 3,39 × 10-2 Kg) is 0,229 Kg CO2-eq.

## Determining the real data volume flowing in the internet and cloud server from the packet traffic analysis

Table A4 calculates the real data volume flowing in the mutualized infrastructure, by taken into account the new traffic behavior (gray cells), the real packet size of TCP and HTTP packets (blue text); and the real number of transmission events (red text), all identified by the packet traffic analysis focused on the gateway and the cloud server interactions.

| Operational mode | Packet type | Packets | Size packet (bytes) | In 3503888 transmissions (bytes) | In 17520 transmissions (bytes) | Total (GB) |
| --- | --- | --- | --- | --- | --- | --- |
| Regular | DNS Request | 1 | 71 | 248776048 |  | 4,197 |
|  | DNS Response | 1 | 87 | 304838256 |  |  |
|  | TCP (three-way handshake) | 3 | 58 | 609676512 |  |  |
|  | HTTP post request | 1 | 200 | 700777600 |  |  |
|  | TCP Ack | 1 | 54 | 189209952 |  |  |
|  | HTTP Request timeout | 1 | 468 | 1639819584 |  |  |
|  | TCP (teardown) | 4 | 58 | 812902016 |  |  |
| Consultation | TCP (three-way handshake) | 3 | 58 |  | 3048480 | 0,108 |
|  | HTTP post request | 7 | 787 |  | 96517680 |  |
|  | HTTP OK | 1 | 572 |  | 10021440 |  |
|  | HTTP GET request | 1 | 71 |  | 1243920 |  |
|  | HTTP reponse | 1 | 71 |  | 1243920 |  |
|  | TCP (teardown) | 4 | 58 |  | 4064640 |  |
|  |  |  |  |  |  | 4,305 |

Table A4. Real data volume flowing in the mutualized infrastructure

## Determining the maximal lifetime span of one flowmeter’s battery under different conditions

By considering the real number of batteries required by the flowmeter in two years (0,34 batteries for 2 years, as it was obtained in the part b of section 6), the number of years that the flowmeter can operates with only one battery is 5,9 years (according to a rule of three), as it is showed below

lifetime span in years= 2 years0,34 battery×1 battery=5,9 years

The rightest column of Table A5 below shows the number of years that the flowmeter can operates with only one battery, according to different bitrates, under different operational contexts (bandwidths, distance ranges and Spreading factors).

| Range | SF | Bandwidth | bitrate in bits / s | In a transmission event |  | Bfm (in 2 years) | Lifetime 1 battery (in years) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | tTx (s) | ts (s) |  |  |
| 2 Km | SF7 | 250 kHz | 11000 | 0,041 | 179,959 | 0,182 | 11,0 |
| 2 Km | SF7 | 125 kHz | 5470 | 0,083 | 179,917 | 0,341 | 5,9 |
| 4 Km | SF8 | 125 kHz | 3125 | 0,146 | 179,854 | 0,579 | 3,5 |
| 6 Km | SF9 | 125 kHz | 1760 | 0,259 | 179,741 | 1,009 | 2,0 |
| 8 Km | SF10 | 125 kHz | 980 | 0,465 | 179,535 | 1,793 | 1,1 |
| 11 Km | SF11 | 125 kHz | 440 | 1,036 | 178,964 | 3,964 | 0,5 |
| 14 Km | SF12 | 125 kHz | 290 | 1,572 | 178,428 | 6,002 | 0,3 |

Table A5. Maximal lifetime span of one battery under different operational contexts

## Summary of the electronic components, functions and capacities considered to estimate theoretically the reference flow and impact of the case study

Table A6 summaries the electronic components, functions and capacities considered to estimated theoretically the reference flow and the impact of the case study (a simplified version is available in table 2 of the main article).

| Device | Electronic components | Functions | Capacity |  | Reference flow | Impact (Kg CO2-eq) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Feature | Value |  |  |
| Pulse emitter | Capacitors & inductors | Generating pulses | Sampling rate | 10 Pulses/sec | 1 Li-ion battery | 0,038 |
|  | Li-ion Battery | Power supply | Nominal capacity | 1650 mAh |  |  |
| Flowmeter | LoRa module | Counting pulses | Current consumption sleep mode | 0,0016 mA | 2 Li-ion 9V batteries | 0,458 |
|  |  | Sending count | Bitrate (SF7, 125 kHz) | 5470 bps |  |  |
|  |  |  | Current consumption (Tx mode) | 44,5 mA |  |  |
|  | Li-ion 9V Battery | Power supply | Nominal capacity | 1200 mAh |  |  |
| Gateway | LoRa module | Receiving counts | Power consumption | 0,006 kW | 105,12 kWh | 5,150 |
|  | WiFi module | Preparing packet |  |  |  |  |
|  |  | Resending counts |  |  |  |  |
| Smartphone | WiFi module | Hourly consultation | Power consumption (sending) | 1629 mW | 0,501 kWh | 0,045 |
|  |  |  | Power consumption (Receiving) | 1375 mW | 0,423 kWh |  |
| Internet networks |  |  | Electricity consumption | 0,15 kWh/GB | 0,0215 kWh | 0,017 |
| Cloud server |  |  | Electricity consumption | 0,14 kWh/GB | 0,020 kWh | 0,016 |
|  |  |  |  |  | Total | 5,723 |

Table A6 Summary of the electronic components, functions and capacities considered to estimated theoretically the reference flow and the impact of the case study.

# References

| [1] | M. Sylvain, LoRa – LoRaWAN and Internet of Things, 2021. |
| --- | --- |
| [2] | M. Gast, 802.11 wireless networks: The definitive guide, China: Southeast University Press, 2006. |
| [3] | C. Sanders, Practical packet analysis: Using Wireshark to solve real-world network problems, No Starch Press, 2017. |
| [4] | J. Malmodin, D. Lundén, Å. Moberg, G. Andersson and M. Nilsson, "Life Cycle Assessment of ICT: Carbon Footprint and Operational Electricity Use from the Operator, National, and Subscriber Perspective in Sweden," Journal of Industrial Ecology, vol. 18, no. 6, pp. 829-845, 2014. |
| [5] | L. Krug, M. Shackleton and F. Saffre, "Understanding the environmental costs of fixed line networking," in e-Energy '14: The Fifth International Conference on Future Energy Systems, 2014. |
| [6] | A. Andrae and T. Edler, "On Global Electricity Usage of Communication Technology: Trends to 2030," Challenges, vol. 6, no. 1, pp. 117-157, 2015. |

Based on a typical LoRa frame format as that one illustrated by Montagny, S. [1]

According to the Global Warming impact for the process “battery cell production, Li-ion, RoW”, provided by the CML-IA 2001 (a100) LCIA methodology.

Based on a typical 802.11 header format as that one illustrated by Gast, M. [2]

Based on a typical IP and TCP header formats as those ones illustrated by Sanders, C. [3]

The median between the proposed energy intensity ratios found in Malmodin, J. et al. [4] and Krug, L. et al. [5].

Proposed by Andrae, A. S., & Edler, T. [6], who collected and analyzed several works oriented to estimate the energy needs of data centers.

According to the Global Warming impact for the process “market group for electricity, medium voltage, GLO”, provided by the CML-IA (a100) LCIA methodology.

According to the Global Warming impact for the process “market group for electricity, medium voltage, FR”, provided by the CML-IA (a100) LCIA methodology.