A1. HEART generic categories [48], [49].

| Generic task | Proposed nominal human unreliability (5th-95th percentile bounds) |
| --- | --- |
| (A) Totally unfamiliar, performed at speed with no real idea of likely consequences | 041 (0.19-085) |
| (B) Shift or restore system to a new or original state on a single attempt without supervision or procedures | 0.17 (0.02-1.0a) |
| (C) Complex task requiring high level of comprehension & skill | 0.17 (0.05-0.6) |
| (D) Fairly simple task performed rapidly or given scant attention | 0.06 (0.02-0.19) |
| (E) Routine, highly practised, rapid task involving relatively low level of skill | 0.02 (0.005-0.09) |
| (F) Restore or shift a system to original or new state following procedures, with some checking | 0.001 (0.00002-0.04) |
| (G) Completely familiar, well-designed, highly practised, routine task occurring several times per hour, performed to highest possible standards by highly motivated, highly trained & experienced person, totally aware of implications of failure, with time to correct potential error, but without the benefit of significant job aids. | 0.002 (0.0002-0.01) |
| (H) Respond correctly to system command even when there is an augmented or automated supervisory system providing accurate interpretation of system stage. | 0.00004 (0.000006-0.00009) |

A2. HEART Error-Producing Conditions [46], [49].

| Error-producing conditions | Maximum predicted amount by which error probability changes from best to worst-case conditions |
| --- | --- |
| 1. Unfamiliarity with a situation which is potentially important but which only occurs infrequently or which is novel | 17 |
| 2. A shortage of time for error detection and correction | 11 |
| 3. A low signal-noise ratio (when really bad) | 10 |
| 4. A means of suppressing or over-riding information or features which is too easily accessible | 9 |
| 5. No means of conveying spatial and functional information to operators in a form which they can readily assimilate - also known as Spatial and Functional incompatibility | 8 |
| 6. A mismatch between an operator's model of the world and that of a designer- also known as a Model Mismatch | 8 |
| 7. No obvious means of reversing an unintended action - also known as Irreversibility | 8 |
| 8. A channel capacity overload, particularly one caused by simultaneous presentation of non-redundant information - a/so known as channel overload | 6 |
| 9. A need to unlearn a technique and apply one which requires an opposing philosophy- also known as technique unlearning | 6 |
| 10. The need to transfer specific knowledge from task to task without loss- a/so known as knowledge transfer | 5.5 |
| 11. Ambiguity in the required performance standards | 5 |
| 12. A mismatch between perceived and real risk | 4 |
| 13. Poor, ambiguous or ill-matched system feedback | 4 |
| 14. No clear, direct, and timely confirmation of an intended action from the portion of the system over which control is to be exerted | 3 |
| 15. Operator inexperience (e.g., a newly-qualified tradesman but not an "expert") | 3 |
| 16. An impoverished quality of information conveyed by procedures and person/person interaction | 3 |
| 17. Little or no independent checking or testing of output | 3 |
| 18. A conflict between immediate and long-term objectives | 2.5 |
| 19. No diversity of information input for veracity checks | 2.5 |
| 20. A mismatch between the educational achievement level of an individual and the requirements of the task | 2 |
| 21. An incentive to use other, more dangerous procedures | 2 |
| 22. Little opportunity to exercise mind and body outside the immediate confines of a job | 1.8 |
| 23. Unreliable instrumentation (enough that it is noticed) | 1.6 |
| 24. A need for absolute judgements that are beyond the capabilities or experience of an operator | 1.6 |
| 25. Unclear allocation of function and responsibility | 1.6 |
| 26. No obvious way to keep track of progress during an activity | 1.4 |
| 27. A danger that finite physical abilities will be exceeded | 1.4 |
| 28. Little or no intrinsic meaning in task | 1.4 |
| 29. High level of emotional stress | 2 |
| 30. Evidence of Ill-health among operatives, especially fever | 1.2 |
| 31. Low workforce morale | 1.2 |
| 32. Inconsistency of meaning of displays and procedures | 3 |
| 33. A poor or hostile environment (below 75% of health or life-threatening severity | 2 |
| 34. Prolonged inactivity or repetitious cycling of low mental workload tasks (x 1.1 for first half-hour, x 1.05 for each hour thereafter) | 1.1 |
| 35. Disruption of normal work-sleep cycles - 1.2 per 24 hours of sleep lost | 1.2 |
| 36. Task pacing caused by the intervention of others | 1.06 |
| 37. Additional team members over and above those necessary to perform the task normally and satisfactorily - 1.2 per additional person | 1.2 |
| 38. Age of personnel performing recall, recognition, and detection tasks - 1.16 for every ten years for ages 25 to 85 years (Mentally competent) | 1.16 |
| 39. Distraction /Task Interruption | 4 |
| 40. Time-of-Day- 2.4 from diurnal high arousal to diurnal/low arousal | 2.4 |