Seasonal distribution, source identification, health and eco-toxicological risk assessment of PAHs in water and sediment from the Danube River in Hungary

Ruqayah Ali Grmasha a,b,*, Csilla Stenger-Kovács a,c, Osamah J. Al-sareji d,b, Raed A. Al-Juboori e,f, Mónika Meiczinger d, Manolia Andredaki g, Ibijoke A. Idowu g, Khalid S. Hashim g

aUniversity of Pannonia, Faculty of Engineering, Center for Natural Science, Research Group of Limnology, H-8200 Veszprém city, Egyetem u. 10, Hungary

bEnvironmental Research and Studies Center, University of Babylon, Al-Hillah city, Iraq

cELKH-PE Limnoecology Research Group, H-8200 Veszprém city, Egyetem utca 10. Hungary

dSustainability Solutions Research Lab, Faculty of Engineering, University of Pannonia, Egyetem str. 10, Veszprém city, 8200, Hungary

eNYUAD Water Research Center, New York University-Abu Dhabi Campus, Abu Dhabi, PO Box 129188, Abu Dhabi city, United Arab Emirates

f Water and Environmental Engineering Research Group, Department of Built Environment, Aalto University, PO Box 15200, Aalto, FI-00076, Espoo, Finland

g  School of Civil Engineering and Built Environment, Liverpool John Moores University, Liverpool city, UK

*Corresponding authors: ruqayah.grmasha@unswalumni.com

| Table S1: The 16 PAHs with their abbreviations and specifications |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Abb. | PAH | Molecular weight | CAS# | Ring | MW | TEF* |
| Nap | Naphthalene | LMW | 91-20-3 | 2 | 128.17 | 0.001 |
| Acy | Ascenaphthylene | LMW | 208-96-8 | 3 | 152.19 | 0.001 |
| Ace | Acenaphthene | LMW | 83-32-9 | 3 | 154.21 | 0.001 |
| Fl | Fluorene | LMW | 86-73-7 | 3 | 166.22 | 0.001 |
| Phe | Phenanthrene | LMW | 85-01-8 | 3 | 178.23 | 0.001 |
| Ant | Anthracene | LMW | 120-12-7 | 3 | 178.23 | 0.01 |
| Flu | Fluoranthene | HMW | 206-44-0 | 4 | 202.26 | 0.001 |
| Pyr | Pyrene | HMW | 129-00-0 | 4 | 202.26 | 0.001 |
| BaA | Benz(a)anthracene | HMW | 56-55-3 | 4 | 228.29 | 0.1 |
| Chr | Chrysene | HMW | 218-01-9 | 4 | 228.29 | 0.01 |
| BbF | Benzo(b)f luoranthene | HMW | 205-99-2 | 5 | 253.32 | 0.1 |
| BkF | Benzo(k)f luoranthene | HMW | 207-08-9 | 5 | 253.32 | 0.1 |
| BaP | Benzo(a)pyrene | HMW | 50-32-8 | 5 | 253.32 | 1 |
| DBA | Dibenz(a,h)anthracene | HMW | 215-58-7 | 6 | 278.35 | 1 |
| BghiP | Benzo (g,h,i) perylene | HMW | 191-24-2 | 6 | 276.34 | 0.01 |
| IND | Indeno(1,2,3-cd)pyrene | HMW | 193-39-5 | 6 | 276.34 | 0.1 |
| * (Nisbet and LaGoy, 1992). |  |  |  |  |  |  |

| Table S2 Sampling coordinates |  |  |
| --- | --- | --- |
| Sites | Latitude (N) | Longitude (E) |
| S1 | 47.788016 | 19.114344 |
| S2 | ## 47.680858 | 19.125760 |
| S3 | 47.523438 | 19.049630 |
| S4 | 47.248683 | 18.910842 |
| S5 | ## 46.912084 | 18.964844 |
| S6 | ## 46.616233 | 18.858203 |

| Table S3. Sources diagnostic ratios of PAHs in water Explanation of PAHs diagnostic ratios |  |  |  |  |
| --- | --- | --- | --- | --- |
| Ratio | Range | Source | Calculated Ratios | References |
| LMW/HMW | >1 | Petrogenic inputs (liquid fuel discharges) | Winter = 2.26 > 1 Spring = 1.70 > 1 Summer = 0.48 < 1 autumn= 1.60 >1 | (Baran et al., 2021; Yang et al., 2020) |
|  | <1 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| Flu/(Flu+Pyr) | <0.4 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.42 ( 0.4-0.5) Spring = 0.49 ( 0.4-0.5) Summer = 0.36 < 0.4 Autumn = 0.45 ( 0.4-0.5) | (Adeniji et al., 2019; Baran et al., 2021; Montuori et al., 2016) |
|  | 0.4-0.5 | Pyrolytic (burning of liquid fossil fuels and crude oil. vehicles) |  |  |
|  | >0.5 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| IND/(IND+BghiP) | <0.2 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.48 ( 0.2-0.5) Spring = 0.58 > 0.5 Summer = 0.35 (0.2-0.5) Autumn = 0.41 ( 0.2-0.5) | (Adeniji et al., 2019; Baran et al., 2021; Montuori et al., 2016) |
|  | 0.2-0.5 | Pyrolytic (burning of liquid fossil fuels and crude oil. vehicles) |  |  |
|  | >0.5 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| BaA/(BaA+Chr) | <0.2 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.37 > 0.35 Spring = 0.45 > 0.35 Summer = 0.44 > 0.35 Autumn = 0.48 > 0.35 | (Adeniji et al., 2019; Baran et al., 2021; Howard et al., 2021; Montuori et al., 2016) |
|  | 0.2-0.35 | Mixed sources (petrogenic / pyrogenic) |  |  |
|  | >0.35 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| BaP/(BaP+Chr) | <0.4 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.35 < 0.4 Spring = 0.49 > 0.4 Summer = 0.62 > 0.4 Autumn = 0.55 > 0.4 | (Yunker et al., 2002) |
|  | >0.4 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| BaP/BghiP | >0.6 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.95 > 0.6 Spring = 0.78 > 0.6 Summer = 0.68 > 0.6 Autumn = 1.19 >0.6 | (Barhoumi et al., 2019) |
|  | <0.6 (non-traffic emission) | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |

| Table S4 Sources diagnostic ratios of PAHs in sediments Explanation of PAHs diagnostic ratios |  |  |  |  |
| --- | --- | --- | --- | --- |
| Ratio | Range | Source | Calculated Ratios | References |
| LMW/HMW | >1 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.34 < 1 Spring = 0.43 < 1 Summer = 0.39 < 1 autumn= 0.39 < 1 | (Baran et al., 2021; Yang et al., 2020) |
|  | <1 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| Flu/(Flu+Pyr) | <0.4 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.32 < 0.4 Spring = 0.37 < 0.4 Summer =0.43 (0.4-0.5) Autumn = 0.54 > 0.5 | (Adeniji et al., 2019; Baran et al., 2021; Montuori et al., 2016) |
|  | 0.4-0.5 | Pyrolytic (burning of liquid fossil fuels and crude oil. vehicles) |  |  |
|  | > 0.5 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| IND/(IND+BghiP) | <0.2 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.53 > 0.5 Spring = 0.37 (0.2-0.5) Summer = 0.51 > 0.5 Autumn = 0.57 > 0.5 | (Adeniji et al., 2019; Baran et al., 2021; Montuori et al., 2016) |
|  | 0.2-0.5 | Pyrolytic (burning of liquid fossil fuels and crude oil. vehicles) |  |  |
|  | >0.5 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| BaA/(BaA+Chr) | <0.2 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.39 > 0.35 Spring = 0.51 > 0.35 Summer=0.28 (0.2-0.35) Autumn = 0.55 > 0.35 | (Adeniji et al., 2019; Baran et al., 2021; Howard et al., 2021; Montuori et al., 2016) |
|  | 0.2-0.35 | Mixed sources (petrogenic / pyrogenic) |  |  |
|  | >0.35 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| BaP/(BaP+Chr) | <0.4 | Petrogenic inputs (liquid fuel discharges) | Winter = 0.39 < 0.4 Spring = 0.68 > 0.4 Summer = 0.52 > 0.4 Autumn = 0.70 > 0.4 | (Yunker et al., 2002) |
|  | >0.4 | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |
| BaP/BghiP | > 0.6 | Petrogenic inputs (liquid fuel discharges) | Winter = 1.70 > 0.6 Spring = 1.1 > 0.6 Summer = 2.45 > 0.6 Autumn = 5.10 > 0.6 | (Barhoumi et al., 2019) |
|  | < 0.6 (non-traffic emission) | Pyrogenic - Combustion of solid fuel - natural sources such as biomass (grass and wood) and coal |  |  |

| Table S5: Concentration range of 16 individual PAHs (ng/g dw) and toxicity guidelines |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PAHs | ERL-ERM | Range (ng/g dw) |  | <ERL | ≥ERL and <ERM | ≥ERM |
|  |  | Min Max |  |  |  |  |
| Nap | 160–2100 | 14.01 | 19.23 | ✓ |  |  |
| Acy | 16–500 | 11.67 | 18.45 | Rest Seasons | Winter, Spring |  |
| Ace | 44–640 | 11.20 | 21.99 | ✓ |  |  |
| FI | 19–540 | 11.89 | 23.30 | Rest Seasons | Spring autumn |  |
| Phe | 240–1500 | 14.58 | 23.21 | ✓ |  |  |
| Ant | 600–5100 | 13.21 | 24.85 | ✓ |  |  |
| Flu | 85.3–1100 | 8.99 | 21.44 | ✓ |  |  |
| Pyr | 665–2500 | 11.40 | 36.00 | ✓ |  |  |
| BaA | 261–1600 | 10.17 | 32.88 | ✓ |  |  |
| Chr | 384–2800 | 16.16 | 50.82 | ✓ |  |  |
| BbF | 320–1880 | 22.93 | 42.62 | ✓ |  |  |
| BkF | 280–1620 | 26.31 | 36.85 | ✓ |  |  |
| BaP | 430–1600 | 28.79 | 52.47 | ✓ |  |  |
| DBA | 63.4–260 | 14.97 | 42.42 | ✓ |  |  |
| BghiP | 430–1600 | 13.33 | 33.81 | ✓ |  |  |
| IND | 240- | 17.00 | 29.38 | ✓ |  |  |

| Table S6: Parameters descriptions used for ILCR model |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Description | Unit | Adults | Children |  | References |
| ABS | Dermal-Absorption-Factor | unitless | 0.13 | 0.13 | (US EPA, 2011) |  |
| AF | Dermal-Adherence-Factor | mg/cm2 | 0.07 | 0.2 | (US EPA, 2011) |  |
| AT | Average-Time (70years_365 days/year) | Days | 25,550 | 25,550 | (Soltani et al., 2015) |  |
| BW | Body-Weight | Kg | 70 | 15 | (US EPA, 2014) |  |
| ED | Exposure-Duration | Years | 20 | 6 | (US EPA, 2014) |  |
| EF | Exposure-Frequency | days/year | 350 | 350 | (US EPA, 2014) |  |
| IR ingestion | Ingestion-Rate | mg/day | 100 | 200 | (US EPA, 2011) |  |
| IR inhalation | Inhalation-Rate | m3/day | 20 | 10 | (Soltani et al., 2015) |  |
| PEF | Particular-Emission-Factor | m3/kg | 1.36×109 | 1.36×109 | (US EPA, 2014) |  |
| SA | Dermal-Surface-Area-Exposure | cm2 | 5700 | 2800 | (US EPA, 2014) |  |

References

Adeniji, A.O., Okoh, O.O. and Okoh, A.I., 2019. Levels of polycyclic aromatic hydrocarbons in the water and sediment of Buffalo River Estuary, South Africa and their health risk assessment. Archives of environmental contamination and toxicology, 76, pp.657-669.

Baran, A., Klimkowicz-Pawlas, A., Ukalska-Jaruga, A., Mierzwa-Hersztek, M., Gondek, K., Szara-Bąk, M., Tarnawski, M. and Spałek, I., 2021. Distribution of polycyclic aromatic hydrocarbons (PAHs) in the bottom sediments of a dam reservoir, their interaction with organic matter and risk to benthic fauna. Journal of Soils and Sediments, 21(6), pp.2418-2431.

Barhoumi, B., Beldean-Galea, M.S., Al-Rawabdeh, A.M., Roba, C., Martonos, I.M., Bălc, R., Kahlaoui, M., Touil, S., Tedetti, M., Driss, M.R. and Baciu, C., 2019. Occurrence, distribution and ecological risk of trace metals and organic pollutants in surface sediments from a Southeastern European river (Someşu Mic River, Romania). Science of the Total Environment, 660, pp.660-676.

Montuori, P., Aurino, S., Garzonio, F., Sarnacchiaro, P., Nardone, A. and Triassi, M., 2016. Distribution, sources and ecological risk assessment of polycyclic aromatic hydrocarbons in water and sediments from Tiber River and estuary, Italy. Science of the Total Environment, 566, pp.1254-1267.

Nisbet, I.C. and Lagoy, P.K., 1992. Toxic equivalency factors (TEFs) for polycyclic aromatic hydrocarbons (PAHs). Regulatory toxicology and pharmacology, 16(3), pp.290-300.

Soltani, N., Keshavarzi, B., Moore, F., Tavakol, T., Lahijanzadeh, A.R., Jaafarzadeh, N. and Kermani, M., 2015. Ecological and human health hazards of heavy metals and polycyclic aromatic hydrocarbons (PAHs) in road dust of Isfahan metropolis, Iran. Science of the total environment, 505, pp.712-723.

U.S. Environmental Protection Agency, 2014. Human health evaluation manual, supplemental guidance: update of standard default exposure factors. OSWER Directive 9200 1–120.

US EPA, 2011. Exposure Factors Handbook. 2011 ed., Final Report. Environmental Protection Agency, Washington, DC EPA/600/R-09/052F.

Yang, Y., Wang, H., Chang, Y., Yan, G., Chu, Z., Zhao, Z., Li, L., Li, Z. and Wu, T., 2020. Distributions, compositions, and ecological risk assessment of polycyclic aromatic hydrocarbons and phthalic acid esters in surface sediment of Songhua river, China. Marine Pollution Bulletin, 152, p.110923.

Yunker, M.B., Macdonald, R.W., Vingarzan, R., Mitchell, R.H., Goyette, D. and Sylvestre, S., 2002. PAHs in the Fraser River basin: a critical appraisal of PAH ratios as indicators of PAH source and composition. Organic geochemistry, 33(4), pp.489-515.