Supplementary Materials

Table of contents

S1. Supplementary Notes on “Proteomics studies of samples”	2

S2. Supplementary Notes on “Dyestuffs analysis of samples”	3

S1. Supplementary Notes on “Proteomics studies of samples”

The peptides were separated via Thermo Fisher Scientific EASY-nLC 1200, with a C18 pre-column (5μm, 100μm*20mm, Dr. Maish GmbH) and a C18 analytical column (3μm, 75μm*150m, Dr. Maish GmbH). Buffer A (0.1% FA) was at 100%, while buffer B (from 2%~100% in 60 min) was a mixture of 0.1% FA, acetonitrile (80%), and water. A Thermo Fisher Scientific Q-Exactive HFX mass spectrometer was used for data collection, and the detection mode was positive ion with a precursor ion scan range of 300~1,800 m/z. The resolution of primary mass spectrometry and secondary mass spectrometry was 60,000 and 15,000, respectively.

The data were compared with the standard database by MaxQuant 1.6.1.0. Uniprot Protein Database with species Bombyx mori and Antheraea pernyi was used, with 25,970 protein sequences. The database searching parameters were set as follows: a maximum of 2 missed cleavages were allowed, precursor ions of 350-8000 Da were searched with the 20 ppm tolerance, and the fragment ion mass tolerance was 0.02 Da. Cysteine carbamidomethyl was used as a static modification, while N-terminal acetylation and methionine oxidation were used as dynamic modifications. To ensure reliable results, a peptide spectrum match (PSM) false discovery rate (FDR) of 0.01 and a protein FDR of 0.01 were applied.

S2. Supplementary Notes on “Dyestuffs analysis of samples”

Nexera X2 LC-30AD Chromatography (Shimadzu, Japan) was used, and the column used was ACQUITY UPLC HSS T3 (1.8 µm, 2.1×100 mm, Waters). Buffer A (0.1% FA) was at 100%, and buffer B was acetonitrile (0%~100% for the first 12min, 100%~0% in the last 3 min). A Thermo Fisher Scientific Q-Exactive Plus mass spectrometer was used for data collection, and detection was performed in positive and negative ion modes, respectively. The precursor ion scan range was 70~1,050m/z, and the resolution of the primary mass spectrum and the secondary mass spectrum were 70,000 and 17,500, respectively.