Supporting Information (SI) of

Personal exposure to source-specific particulate polycyclic aromatic hydrocarbons (PAHs) and systemic inflammation: the results from a cross-sectional study among the urban elderly in China

Jia Xu1#, Nan Zhang1#, Yucong Fu2,7,8, Yujuan Zhang3,1, Penghui Li4, Jinbao Han5, Shuang Gao6, Xinhua Wang1*, Chunmei Geng1, Wen Yang1, Liwen Zhang2,7,8, Bin Han1*, Zhipeng Bai1

1. State Key Laboratory of Environmental Criteria and Risk Assessment, Chinese Research Academy of Environmental Sciences, Beijing 100012, China

2. Department of Occupational and Environmental Health, School of Public Health, Tianjin Medical University, Tianjin 300070, China

3. Department of Family Planning, The Second Hospital of Tianjin Medical University, Tianjin 300211, China

4. School of Environmental Science and Safety Engineering, Tianjin University of Technology, Tianjin 300384, China

5. School of Quality and Technical Supervision, Hebei University, Baoding 071002, China

6. School of Geographic and Environmental Sciences, Tianjin Normal University, Tianjin 300387, China

7. Tianjin Key Laboratory of Environment, Nutrition, and Public Health, Tianjin Medical University, Tianjin, 300070, China

8. Center for International Collaborative Research on Environment, Nutrition and Public Health, Tianjin, 300070, China

# These two authors contributed equally to the paper.

Text S1: Introduction of the Positive Matrix Factorization (PMF) model

PMF model was developed by Paatero [1, 2], and has been a widely applied model with unknown sources profiles, especially after being released by the U.S. EPA. In this model, a data matrix X, in which i is the samples and j is the chemical species measured, can be viewed as a speciated data set, and the concept of this model is shown in Equation (1):

Xij=k=1pgikfkj+eij  (1)

where: p is the number of factors; f is the chemical profile of each source, g is the mass contribution by each factor to each single sample; fjk is the source profile, eijis the residual for each species or sample.

PMF solves equation (1) by minimizing the sum of the square of residuals weighted inversely with error estimates of the data points, Q, which is defined as

Q=i=1nj=1mxij-k=1pgikfkjuij2                      (2)

where: uij is the uncertainty of each species in the model, and it can be calculated using following equation:

Uncertainty=(Error Fraction ×concentrion)2+(MDL)2          (3)

Where: MDL here is the method detection limit of each constituent.

Text S2: Source apportionment results interpretations

EPA PMF5.0 was run in random seed model with different number of factors. The corresponding Q/Qexp was obtained and the decrease from five-factor to six-factor was the largest. Therefore, six sources were determined for exposure of senior citizens PM2.5-bound PAHs in winter and the source profile was depicted in Fig.1.

Factor 1 was dominated by DahA (63.8%) and moderately loaded by BaA (30.6%) and Chr (26.6%). DahA was used as the indicatory PAH for biomass burning [3], and BaA and Chr were good tracers for biomass burning [4, 5]. Biomass was burned in an open field as agricultural waste or in stoves as fuel [6], which was not forbidden in Tianjin in 2011. Moreover, regional transport from nearby provinces, such as Hebei and Shandong, was considered [7]. Zhang et al. [6] reported that a 20% increase in the contribution of biomass burning in the Beijing-Tianjin area was from nearby provinces. Therefore, factor 1 was identified as biomass burning, contributing 14.7% to PAHs mass concentration.

Factor 2 depicted the major loading of Phe (77.5%) and Fl (43.6%). Yao et al. [8] concluded that 3–4 ring PAHs were related to cooking emission. An abundance of Phe was found in commercial and domestic kitchen exhausts. Furthermore, it indicated liquid petroleum gas burning and cooking oil fumes together with Fl [9], as the relatively low temperature during cooking mainly produced lighter PAHs. Zhang et al. [10] established an emission profile of domestic cooking in China, suggesting that Phe and Fl could be recognized as source signatures of cooking. Moreover, the ratio of IND/(IND+BghiP) in this factor (0.7) was close to the reported value of 0.5 [10], displaying the impact of cooking emission. According to the time-activity survey, 71.3% of the elderly spent an average of 1.5 h cooking daily. Thus, Factor 2 was identified as cooking, which contributed 9.9% to PAHs mass concentration.

Factor 3 was enriched with Pyr (58.3%), Chr (38.1%), and Fl (36.5%). Pyr was regarded as a source tracer of coal combustion [4, 11]. Several studies revealed that Pyr, Chr, and Fl were dominant PAH species in coal combustion [4, 7, 12-14]. In 2011, 23.66 million tons of coal were consumed, accounting for over 50% of fossil fuel usage in Tianjin. Coal emissions increased ambient PAHs concentration in winter from heating sources [15-18]. Hence, factor 3 could represent coal combustion, which contributed 27.1% to PAHs mass concentration.

Factor 4 had to predominate BkF (61.4%) and moderate Flu (38.9%), and BaP (37.4%). BkF showed significant dominance in diesel emission profile and was often used as diesel markers [4, 9, 11, 19, 20]. High fraction of Flu and BaP in diesel emission profile was reported [11, 14, 20]. The ratio of BbF/BkF in the factor (1.5) was higher than 0.5, which could be used to indicate diesel emission [13]. Two arterial roads surrounded the selected communities, with a bus stop nearby. The senior citizens preferred buses for commuting, and diesel was the main fuel for buses in Tianjin in 2011. Hence, factor 4 was classified as diesel emission, which contributed 18.7% of PAHs mass concentration.

Factor 5 was mainly loaded with BaP (27.0%), BaA (21.6%), and Fl (19.9%). H Lu and L Zhu [21] found that over 80% of residential Bap came from tobacco smoking. BaA was characterized as a source marker of environmental tobacco smoking [22, 23]. MR Mannino and S Orecchio [24] pointed out that the level of Fl was higher in smoking-permitting families than that in smoking-forbidding families. The ratio of BaA/BaA+Chr in the factor was 0.6, equal to that reported by Zhang et al. [23], which could be used to identify the ETS. Among the participants, 14.9% of the elderly were smokers, while 28.7% were exposed to ETS. Shang et al. [25] found that ETS sources could affect people with no cigarette exposure, suggesting that cigarette smoke could be present in ambient PM2.5. Therefore, factor 5 was regarded as ETS, which contributed 9.2% of PAHs mass concentration.

Factor 6 was highly loaded with Ace (49.4%), Flu (49.0%), IND (39.4%), and BghiP (39.1%). The LWM-PAHs, such as Ace, were linked with unburned petroleum and gasoline emissions [11]. High loading of Flu indicated the emission of gasoline-powered vehicles [11, 26]. In addition, IND and BghiP were typical PAHs in gasoline vehicle exhaust [9, 14, 27]. The ratio of Pyr/BaP in the factor was 0.6 [28], suggesting the presence of gasoling emission. Tianjin was experiencing a 20.6% car ownership growth rate in 2011; thus, factor 6 was considered as gasoline emission, which contributed 20.4% of PAHs mass concentration.

Text S3 The discussions on the cause that led to the differences of personal exposure from traffic emission

Living in the same community within a small area, the participants are supposed to exposure to the similar environment. However, it should be noted that the time spent at outdoor environment varied greatly across participant. Our previous publication [29] applied eighteen variables that this study collected for artiﬁcial neural network (ANN) simulation, and found that principal components that represented time spent outdoors, time spent in traffic, outdoor concentration could influence the final prediction on personal exposure PM2.5 mass concentrations. Specific to the PM2.5 bound PAHs, the concentrations of PAH individual of each participant are supposed to be affected by the time spent outdoors. Therefore, the contributions of external sources (traffic, coal combustion, biomass burning, etc.) may vary largely due to their time spent outdoors.

In addition, the air change rate mainly depends on the weather (temperature difference and windspeed), building-related factors (tightness of the building envelope), and occupancy-associated factors (window opening, use of flued combustion, operation of heating and cooling systems) [30]. These factors were different among the homes of the participants. Therefore, the influence from external sources varied with the participants.

Moreover, due to the lack of personal exposure monitor (PEM), we could not ask all the participants to undergo the sample collection at the same day. Therefore, they were equipped with the PEM at different days. Therefore, we did not assume all elderly are exposed to the same environment due to the variations on the outdoor PM2.5 concentration, compositions, and source origins.

Table S1 Basic characteristics and clinical parameters of participants (N=82)

| Variable | Median (5th, 95th percentiles) or percent |
| --- | --- |
| Hypertension | 44.6% |
| Hyperlipidemia | 33.0% |
| Diabetes | 17.0% |
| SBP (mmHg) | 138.5 (120,160) |
| DBP (mmHg) | 86 (70,117) |
| LDL (mmol/L) | 3.25 (2.17,4.42) |
| HDL (mmol/L) | 1.28 (0.89,1.75) |
| Triglycerides (mmol/L) | 1.41 (0.66,3.02) |
| Total cholesterol (mmol/L) | 5.43 (4.09,7.04) |
| Fast glucose (mmol/L) | 5.43 (4.72,8.60) |

Note: SBP, systolic blood pressure; DBP, diastolic blood pressure; LDL, low-density lipoprotein; HDL, high-density lipoprotein.

Table S* Estimated changes of cytokines with IQR increase of *** exposure stratiﬁed by smoking status.

| Sources | Il1β |  |  |  |  | IL6 |  |  |  |  | TNFα |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Smoker |  | Non-smoker |  |  | Smoker |  | Non-smoker |  |  |  | Smoker |  | Non-smoker |  |
| Cooking Fumes | 2.8 (-29.0,48.8) |  | 12.6 (-13.4,46.4) |  |  | 13.1 (-19.6,59.1) |  | 2.1 (-13.0,19.7) |  |  |  | 17.3 (-10.2,53.2) |  | 3.3 (-18.0,30.0) |  |
| Diesel Vehicle Emission | -1.5 (-39.1,59.4) |  | 8.8 (-7.6,28.2) |  |  | -5.6 (-33.3,33.7) |  | 10.5 (-1.1,23.6) |  |  |  | 9.2 (-24.5,57.9) |  | 14.9 (-1.1,33.6) |  |
| Coal Combustion | -6.1 (-22.7,14.0) |  | 6.2 (-11.1,26.8) |  |  | -1.0 (-18.8,20.7) |  | 5.5 (-6.2,18.7) |  |  |  | 10.3 (-6.8,30.7) |  | 10.0 (-6.7,29.7) |  |
| Environmental Tobacco Smoking | -1.8 (-27.0,32.1) |  | 1.2 (-10.2,14.1) |  |  | -5.7 (-29.6,26.3) |  | 5.5 (-3.9,15.9) |  |  |  | -13.9 (-33.0,10.7) |  | 8.2 (-2.4,19.8) |  |
| Gasoline Vehicle Emission | 5.4 (-22.3,43.0) |  | -1.4 (-17.6,18.0) |  |  | 1.2 (-25.5,37.4) |  | -2.3 (-15.9,13.4) |  |  |  | -3.1 (-25.4,25.8) |  | -5.7 (-19.8,10.9) |  |
| Biomass Burning | 12.8 (-13.4,47.0) |  | 22.7 (3.1,45.9) |  |  | 8.1 (-13.0,34.2) |  | 19.5 (3.3,38.1) |  |  |  | 3.4 (-19.6,33.0) |  | 1.2 (-11.2,15.4) |  |

Figure S1 Location of the research community

Figure S1 Location of the research community (a. Tianjin city; b. the district where the study community locates; c. the study community)

(Map sources: a. http://wegslttisnl.com/tianjin-tours/maps.html; b. OpenStreetMap; c. OpenStreetMap)

Figure S2 Source profile obtained from PMF model (the bars represent the percentage of the constituent of the specific source in the total observed mass of the constituent)

Fig.1. Source profile of personal PM2.5-bound PAHs exposure.

Figure S2 Source profile obtained from PMF model (the bars represent the percentage of the constituent of the specific source in the total observed mass of the constituent)

Note: (F1: Cooking Fumes, F2: Diesel Vehicle Emission, F3: Coal Combustion, F4: Environmental Tobacco Smoking, F5: Gasoline Vehicle Emission, F6: Biomass Burning)

Figure S3 Comparisons between measured and modeled PAH individuals

Figure S3 Comparisons between measured and modeled PAH individuals

Figure S4 Associations between source-specific PAHs and cytokines, stratiﬁed by smoking status

Figure S4 Associations between source-specific PAHs and cytokines, stratiﬁed by smoking status (F1: Cooking Fumes, F2: Diesel Vehicle Emission [DV], F3: Coal Combustion, F4: Environmental Tobacco Smoking, F5: Gasoline Vehicle Emission, F6: Biomass Burning).

Note: IL, interleukin; TNFα, tumor necrosis alpha; IQR, interquartile range

Figure S5 The estimates with 95%CI of inflammatory cytokines with per IQR increase of source-specific PM2.5 concentrations after before and after controlling for OC, disease conditions, PM2.5 mass concentration, residual PM2.5 mass, concentration, or residual total PAHs mass concentration

Figure S3 The estimates with 95%CI of inflammatory cytokines with per IQR increase of source-specific PM2.5 concentrations after before and after controlling for OC, disease conditions, PM2.5 mass concentration, residual PM2.5 mass, concentration, or residual total PAHs mass concentration

(F1: Cooking Fumes, F2: Diesel Vehicle Emission, F3: Coal Combustion, F4: Environmental Tobacco Smoking, F5: Gasoline Vehicle Emission, F6: Biomass Burning).

Note: CI, confidence interval; IL, interleukin; TNFα, tumor necrosis alpha; IQR, interquartile range; * 0.05 < P value < 0.1; ** P value <0.05.

Note: ** P value < 0.05; * P value < 0.1; CI, confidence interval; IL, interleukin;

Reference

1.	Paatero P, Tapper U: Positive matrix factorization: A non-negative factor model with optimal utilization of error estimates of data values. Environmetrics 1994, 5(2):111-126.

2.	Paatero P: Least squares formulation of robust non-negative factor analysis. Chemometrics and Intelligent Laboratory Systems 1997, 37(1):23-35.

3.	Wang Q, Jiang N, Yin S, Li X, Yu F, Guo Y, Zhang R: Carbonaceous species in PM2.5 and PM10 in urban area of Zhengzhou in China: Seasonal variations and source apportionment. Atmospheric Research 2017, 191:1-11.

4.	Wang F, Lin T, Li Y, Ji T, Ma C, Guo Z: Sources of polycyclic aromatic hydrocarbons in PM2.5 over the East China Sea, a downwind domain of East Asian continental outflow. Atmospheric Environment 2014, 92:484-492.

5.	Yu Q, Gao B, Li G, Zhang Y, He Q, Deng W, Huang Z, Ding X, Hu Q, Huang Z et al: Attributing risk burden of PM2.5-bound polycyclic aromatic hydrocarbons to major emission sources: Case study in Guangzhou, south China. Atmospheric Environment 2016, 142:313-323.

6.	Zhang Y, Lin Y, Cai J, Liu Y, Hong L, Qin M, Zhao Y, Ma J, Wang X, Zhu T et al: Atmospheric PAHs in North China: Spatial distribution and sources. Science of The Total Environment 2016, 565:994-1000.

7.	Chao S, Liu J, Chen Y, Cao H, Zhang A: Implications of seasonal control of PM2.5-bound PAHs: An integrated approach for source apportionment, source region identification and health risk assessment. Environmental Pollution 2019, 247:685-695.

8.	Yao Z, Li J, Wu B, Hao X, Yin Y, Jiang X: Characteristics of PAHs from deep-frying and frying cooking fumes. Environmental Science and Pollution Research 2015, 22(20):16110-16120.

9.	Ray D, Chatterjee A, Majumdar D, Ghosh SK, Raha S: Polycyclic aromatic hydrocarbons over a tropical urban and a high altitude Himalayan Station in India: Temporal variation and source apportionment. Atmospheric Research 2017, 197:331-341.

10.	Zhang N, Han B, He F, Xu J, Zhao R, Zhang Y, Bai Z: Chemical characteristic of PM2.5 emission and inhalational carcinogenic risk of domestic Chinese cooking. Environmental Pollution 2017, 227:24-30.

11.	Chen Y-C, Chiang H-C, Hsu C-Y, Yang T-T, Lin T-Y, Chen M-J, Chen N-T, Wu Y-S: Ambient PM2.5-bound polycyclic aromatic hydrocarbons (PAHs) in Changhua County, central Taiwan: Seasonal variation, source apportionment and cancer risk assessment. Environmental Pollution 2016, 218:372-382.

12.	Lu W, Yang L, Chen J, Wang X, Li H, Zhu Y, Wen L, Xu C, Zhang J, Zhu T et al: Identification of concentrations and sources of PM2.5-bound PAHs in North China during haze episodes in 2013. Air Quality, Atmosphere & Health 2016, 9(7):823-833.

13.	Callén MS, Iturmendi A, López JM: Source apportionment of atmospheric PM2.5-bound polycyclic aromatic hydrocarbons by a PMF receptor model. Assessment of potential risk for human health. Environmental Pollution 2014, 195:167-177.

14.	Zhang Y, Zheng H, Zhang L, Zhang Z, Xing X, Qi S: Fine particle-bound polycyclic aromatic hydrocarbons (PAHs) at an urban site of Wuhan, central China: Characteristics, potential sources and cancer risks apportionment. Environmental Pollution 2019b, 246:319-327.

15.	Han B, You Y, Liu Y, Xu J, Zhou J, Zhang J, Niu C, Zhang N, He F, Ding X et al: Inhalation cancer risk estimation of source-specific personal exposure for particulate matter–bound polycyclic aromatic hydrocarbons based on positive matrix factorization. Environmental Science and Pollution Research 2019, 26(10):10230-10239.

16.	Han J, Zhang N, Niu C, Han B, Bai Z: Personal Exposure of Children to Particle-Associated Polycyclic Aromatic Hydrocarbons in Tianjin, China. Polycyclic Aromatic Compounds 2014, 34(4):320-342.

17.	Wang C, Meng Z, Yao P, Zhang L, Wang Z, Lv Y, Tian Y, Feng Y: Sources-specific carcinogenicity and mutagenicity of PM2.5-bound PAHs in Beijing, China: Variations of contributions under diverse anthropogenic activities. Ecotoxicology and Environmental Safety 2019, 183:109552.

18.	Wang S, Ji Y, Zhao J, Lin Y, Lin Z: Source apportionment and toxicity assessment of PM2.5-bound PAHs in a typical iron-steel industry city in northeast China by PMF-ILCR. Science of The Total Environment 2020b, 713:136428.

19.	Cao X, Hao X, Shen X, Jiang X, Wu B, Yao Z: Emission characteristics of polycyclic aromatic hydrocarbons and nitro-polycyclic aromatic hydrocarbons from diesel trucks based on on-road measurements. Atmospheric Environment 2017, 148:190-196.

20.	Liu J, Man R, Ma S, Li J, Wu Q, Peng J: Atmospheric levels and health risk of polycyclic aromatic hydrocarbons (PAHs) bound to PM2.5 in Guangzhou, China. Marine Pollution Bulletin 2015, 100(1):134-143.

21.	Lu H, Zhu L: Pollution patterns of polycyclic aromatic hydrocarbons in tobacco smoke. Journal of Hazardous Materials 2007, 139(2):193-198.

22.	Saraga DE, Maggos TE, Sfetsos A, Tolis EI, Andronopoulos S, Bartzis JG, Vasilakos C: PAHs sources contribution to the air quality of an office environment: experimental results and receptor model (PMF) application. Air Quality, Atmosphere & Health 2010, 3(4):225-234.

23.	Zhang N, Han B, Zhao R, Zhao X, Xu J, Zhang Y, Bai Z: Source profile and excess cancer risk evaluation of environmental tobacco smoking under real conditions, China. Atmospheric Pollution Research 2019a, 10(6):1994-1999.

24.	Mannino MR, Orecchio S: Polycyclic aromatic hydrocarbons (PAHs) in indoor dust matter of Palermo (Italy) area: Extraction, GC–MS analysis, distribution and sources. Atmospheric Environment 2008, 42(8):1801-1817.

25.	Shang J, Khuzestani RB, Tian J, Schauer JJ, Hua J, Zhang Y, Cai T, Fang D, An J, Zhang Y: Chemical characterization and source apportionment of PM2.5 personal exposure of two cohorts living in urban and suburban Beijing. Environmental Pollution 2019, 246:225-236.

26.	Murillo JH, Villalobos MC, Rojas Marín JF, Guerrero VHB, Solórzano Arias D: Polycyclic aromatic hydrocarbons in PM2.5 and PM10 atmospheric particles in the Metropolitan Area of Costa Rica: Sources, temporal and spatial variations. Atmospheric Pollution Research 2017, 8(2):320-327.

27.	Ho KF, Ho SSH, Lee SC, Cheng Y, Chow JC, Watson JG, Louie PKK, Tian L: Emissions of gas- and particle-phase polycyclic aromatic hydrocarbons (PAHs) in the Shing Mun Tunnel, Hong Kong. Atmospheric Environment 2009, 43(40):6343-6351.

28.	Ravindra K, Sokhi R, Van Grieken R: Atmospheric polycyclic aromatic hydrocarbons: Source attribution, emission factors and regulation. Atmospheric Environment 2008, 42(13):2895-2921.

29.	Gao S, Zhao H, Bai Z, Han B, Xu J, Zhao R, Zhang N, Chen L, Lei X, Shi W et al: Combined use of principal component analysis and artificial neural network approach to improve estimates of PM2.5 personal exposure: A case study on older adults. Science of The Total Environment 2020, 726:138533.

30.	Nazaroff WW: Residential air-change rates: A critical review. Indoor Air 2021, 31(2):282-313.