Microorganisms and its maintenance

In the present study, Sphingobacterium thalpophilum DP9 – Gram negative bacteria (GenBank accession number: MG–000135) isolated in our laboratory from automobile workshop soil (Anand, Gujarat, India N 22°54’ and E 72°95’) was used.  The bacterial culture was maintain in Luria Bertani (LB) agar slants at 4 °C and was sub-cultured at every three month after confirming purity of culture by Gram’s staining.

Inoculum preparation, Medium composition and culture conditions

Inoculum was prepared by adding a well isolated colony from L.B agar plate in to 100 ml of LB medium maintained at 30 °C with 150 rpm shaking condition overnight. Next day, required amount of inoculum (after O.D.600 reaches to 1.00) was added to production medium.

Bushnell–Haas medium (BHM) added with carbon and nitrogen source was used as production medium. Media were sterilized by autoclaving 121 °C for 15 min. The screening of factors viz. carbon source, nitrogen source, temperature, pH, inoculum size and aeration were carried out primarily by One–Factor–At–Time (OFAT) method. Afterwards, Placket- Burman design made by Microsoft Office-2007 with Excel add-on , was employed to screen significant factor.

Carbon source used were rice straw, corn straw, wheat straw, maize bran, wheat bran, potato peel powder, cotton seed cake, sugarcane husk, ground nut husk, coconut oil cake, and sugarcane molasses.

Nitrogen sources used were urea, peptone, ammonium sulfate, ammonium nitrate, ammonium chloride, sodium nitrate, yeast extract and malt extract.

Screening of agricultural wastes or byproducts by One–Factor–at A –Time [OFAT] method

OFAT method was used to screen optimum carbon source (at 1% concentration), nitrogen source (0.1% concentration), inoculum size (0.5 % to 4 % when 1.00 O.D.600), pH (5 to 11), temperature (25 °C to 45 °C) and aeration ( by varying the headspace volume in 250 ml conical flask by changing the medium volume in the flask by 10 % to 50 % v/v of flask i.e. 250 ml as suggested by Abdel-mawgoud et al. 2008)). Carbon sources and nitrogen sources were screened by separately added in 100 ml of production medium (i.e. BHM). Screening of aeration was carried out by preparing production media with different volume viz. 25 ml, 50 ml, 75 ml, 100 ml, 125 ml and 150 ml in 250 ml Erlenmeyer flask. Later on, pH, inoculum size and temperature were screened sequentially. All media were kept under 150 rpm shaking condition and response was checked for 10 days by emulsification index (E24% test) as suggested by Shahaliyan et al. (2015).

Plackett–Burman Design

Plackett–Burman design was used to find out important medium component (carbon or nitrogen source) and production parameter (temperature and pH). Plackett-Burman design matrix also assumes that there is no interaction between variables considered in present study in the range. It consents to explore up to ‘N–1’ variables with N experiments without inter-component interactions. In present study, four variables were selected viz. potential carbon source, potential nitrogen source, pH and temperature. Each variable was checked at two level (–1, +1 or low level and high level) as given in Table-1. Plackett-Burman design matrix is a full factorial design and main effects of design is simply calculated as per the difference between average value made at the high level values (+) of the factor and average of measurement at the low level values (–). The critical factors are identified through this experimental matrix and then Central Composite Design (CCD) was used to obtain quadratic model.

Table.1 The various media components including in Plackett-Burman experiments and their corresponding higher, medium and lower concentration

Table.2   Plackett–Burman Design for biosurfactant production from S. thalpophilum DP9.

The linear approach was considered to be sufficient for screening

Y=β0+βifi  (i=1…k)

Where Y is the response in terms of protein assayed as per the method suggested by Lowery et al. (1951), βi are regression coefficients, fi is the level of the independent variable. The experiments were conducted three times and results were noted in mean ± standard deviation (Mnif et al. 2012; Anvari et al. 2015; Ekpenyong et al. 2017).

Central Composite Design (CCD)

Following the screening of significant media components by Plackett-Burman design, relationship among the quantitative factors and the response (biosurfactant production) was carried out by central composite design (CCD) under response surface methodology (RSM). Best response was evaluated by permutation of the factor levels (El-Gherab et al. 2019). Significant factor obtained after analysis by Plackett-Burman design, was used to create CCD matrix and statistical analysis. Most optimum level, impact and interaction of factors i.e. carbon source (X1), nitrogen source (X2) and temperature (X3) at three coded level (–1, 0 and +1) as shown in the table-3 by full factorial (23) CCD using Design s (X2) and pH (X3) were optimized by full factorial (23) central composite design (CCD) using Design Expert software® (Stat-Ease Inc. Minneapolis, MN, USA, version–6.0.8). 23 full factorial design was generated in to 20 experimental runs. The experiments were carried out in 250 ml Erlenmeyer flasks containing 100ml of medium. Each production medium was inoculated with 1% overnight grown bacterial cells (O.D.600 = 1.00) and incubated for four days. Interaction experimentations were designed as per CCD model based on three factors. According to CCD design, second order polynomial regression equation was

Y=β0+β1X1 + β2X2 + β3X3 + β11X12+ β22  X22    + β33 X32 + β12X1X2 + β13X1X3 + β23X2X3.

Where, Y– Predicted response (Emulsification index; indirectly biosurfactant production); β0– Intercept; X1– carbon source concentration (g/l); X2– Nitrogen source (%w/v); X3 – Temperature; β1, β2 and β3 – linear co-efficient; β12, β23 and β13 – interaction coefficient; X12, X22, X32, X1X2, X2X3and  X1X3 – interaction between the variables as significant terms.

Experimental Validation of statistical model

From the response surface design the optimum experimental conditions were tested and validated three times. Results were recorded as in terms of mean ± standard deviation.

Extraction and purification of biosurfactants

Cell free supernatant was collected from each flask prepared as per the optimized and validated process for biosurfactant production by centrifugation (6000 rpm for 20 min) and pH was set 2.00 using 6.0 N HCl. Broth was preserved at 4 ºC till visible precipitates observed. Precipitates were collected by centrifugation and kept for air drying. Biosurfactant was collected as dry powder by scraping.

Partial characterization

Qualitative analysis by thin layer chromatography

Biosurfactant produced from optimized medium was qualitatively analyzed through thin layer chromatography for presence of carbohydrate, protein and lipid. Solvent systems used were ethyl acetate: acetic acid: methanol: water (12:3:3:2); n-butanol: acetic acid: water (4:1:1) and chloroform: methanol: water (65: 25: 4) for carbohydrate, protein and lipid, respectively. Carbohydrate was detected by treating the TLC with α-naphthol followed by sulfuric acid. Protein was detected by spraying 0.3% ninhydrine solution and for lipid content iodine vapor was used.

Spectrophotometric qualitative test

Biosurfactant powder was dissolve in distilled water to get 10 mg/ml of concentration. Carbohydrate, protein and lipid contents were estimated by phenol-sulfuric acid test, Folin-Lawry method and GPO/POD method (according to diagnostic triglyceride kit procured from Sigma Dignostic Pvt. Ltd. Vadodara, India), respectively.

Electrophoretic studies

Biosurfactant was further analyzed through sodium dodecyl sulfate polyacrylamide gel electrophoresis (SDS-PAGE) followed by coomassie brilliant blue stain, silver stain. Native-PAGE followed by Periodic acid-Schiff (PAS) stain as per the standard protocol.

FTIR [Fourier- Tranform Infrared] spectroscopic study

Presence of chemical groups in biosurfactant was evaluated by Bruker FTIR (Alpha-T) spectrophotometer (Bruker Optics Inc., USA and M/s Lab India Analytical Instruments Pvt. Ltd.). Briefly, the pellet was made with 1 mg dry biosurfactant grinded in 100 mg (final volume) of IR grade KBr under mechanical hydraulic. KBr pellet without biosurfactant serve as control or blank. Both, KBr and biosurfactant were dried in hot air oven at 65 °C for complete removal of moisture content overnight. Chromatogram generated by FTIR spectrophotometer was compared with standard table and presence of possible chemical groups was recorded.