The perspective of maximizing biosurfactant yield can be fulfill by utilizing agricultural waste or byproducts which not only help to cut down the capital cost but also helps in waste management. In present study, we have screen eleven agricultural wastes or byproducts for their efficacy as media supplement. The right choice of agricultural waste is very important because the biosurfactant yield, its type and purity is solely depends on the media constituents (Helmy and Kardena, 2011). Agricultural waste or byproducts provides adequate balance of carbohydrate and lipid to the bacteria along with other nutrients like protein, mineral salts etc. for their growth and also for production of secondary metabolites like biosurfactant (Mercade and Manresa, 1994). Therefore, by using a single agricultural waste or byproduct, the production of biosurfactant can be made economic. In present study, we have screened eight different nitrogen sources for their impact on biosurfactant. They are either organic compound or inorganic salt.

In order to maximize production two methods were adopted. First OFAT to screen effective carbon source, nitrogen source, pH, temperature, % inoculum and aeration, which is followed by statistical approach i.e. Plackett-Burman Design using Microsoft Office-2007 Excel add-on and CCD matrix using Expert Design software for optimum concentration of media constitutes and cultural conditions (Nurfarahin et al. 2019).

OFAT analysis

One factor at a time method for optimization is traditional technique used to screen effective agricultural waste or byproduct.  The major advantage of this method is its simplicity (because it does not require any statistical analysis) and results can be narrated by simple graphs (Hema et al. 2019). In present study, among the all agricultural wastes or byproducts listed above highest emulsification index was observed in potato peel powder containing medium (E24% = 43.12 ± 3.41%), which was followed by deoiled coconut oil cake (E24% = 31.00 ± 8.27%) and wheat bran (E24% = 31.47 ± 3.41%) (Fig.1(a)). Bacillus subtilis DDU20161 was reported to produce 253.79 gm/L of biosurfactant when cultured in media containing potato peel and pulp during 40 h of incubation time under stirred tank bioreactor (Pande et al. 2020); while potato peel significantly effect on emulsification of biosurfactant from Bacillus subtilis SNW3 (Naeem 2018). The emulsification index of biosurfactant produced by B. subtilis SNW3 strain was 15. 48% when bacteria were grown in media supplemented with potato peel (Naeem 2018).  In case of B. licheniformis J1 potato peel was act as biostimulator to increase the petroleum oil degradation potential (Jyoti and Rajesh 2018). According to Javed et al. (2019) potato peel is an economy booster for developing countries because it is inexpensive waste of food processing industry and can support many of the industries viz. backing industry, biogas production industry, lactic acid production industry, enzyme production industry, bio-fertilizer industry, bio-fuels industry, bio-sorbent industry, pharmaceutical industry and biosurfactant production industry.

Similarly, highest emulsification index 46.00 ± 3.09 % was observed in media containing urea, followed by yeast extract (E24% = 38.00 ± 0.10%) and ammonium sulfate (35.33 ± 0.90%) (Fig.1(b)). Serratia nematodiphila produced 20% more glycolipid biosurfactant when it grown under media containing 0.1% w/v of urea (Panjiar et al. 2020). Bacillus cereus produced biosurfactant effectively reduced surface tension of water when it was cultured in presence of urea (Durval et al. 2018). Whereas, urea was less preferred by B. subtilis RSL2 for the production of biosurfactant (Sharma and Pandey 2020) while, for Arthrobacter paraffines ATCC 19558 urea was preferable organic source for biosurfactant production (Ezebuiro et al. 2019).

Dissolve oxygen influences of biosurfactant production (Kronemberger et al. 2007). Effect of aeration on biosurfactant was assayed by filling different volume of media in 250 ml flasks and it was observed that  40 % of aeration (100 ml media in 250 ml flask) was most suitable (E24% = 47.41 ± 1.62%), which followed by 30 % (E24% = 38.00 ± 2.30%) and 20% (E24% = 27.825 ± 1.73%) (Fig.2(a)). Bacillus mojavensis A21 produced highest surfactin when cultured in 10% flask (25 ml media in 250 ml of flask) while the production in decreased when culture was kept in 70% aeration ( 75 ml media in 250 ml flask) (Hmidet et al. 2017). Bacillus subtilis BS5 produced maximum surfactin 10% media containing flask where as production is decline sharply when media volume was increased from 10% (Abdel-mawgoud et al. 2008). Optimum pH and temperature for maximum biosurfactant production was 7 (E24% = 41.74 ± 1.15%) and 30 °C (E24% = 44.00 ± 2.36%) (Fig.2(b) and (c)). Bacterial inoculum (O.D.600=1.00) size of maximum biosurfactant production observed was 1% (E24% = 26.39 ± 1.60%) (Fig.2(d)).

Therefore, potato peel powder and urea were used as carbon and nitrogen sources respectively, under 40% of aeration for further studies. Bacterial inoculum used was 1% with O.D.600=1.00, pH 7 and incubation temperature was kept 30 °C.

Optimization by Plackett-Burman Design

Once the optimum media components and production parameter was screened by traditional OFAT method, Plackett-Burma design was used for find out most influencing factor on biosurfactant production among the carbon source, nitrogen source, temperature and pH. The design generated by Microsoft Office-2007 with Excel add-on given in Table-2. The Plackett-Burman design for eleven experiments with four variables (i.e. carbon source, nitrogen souse, temperature and pH) shown in Table-2. The protein concentration (indirectly biosurfactant production) was considered as response. The results achieved from the experimental run was noted as in Table-2 and it showed variation from 38.4 μg/ml (run no: 12) to 265.9 μg/ml  (run no: 5). Data was subjected to statistical analysis using Microsoft Office-7 Excel add-on, to calculate main effect, standard error F-value, p-value and 90% clearance. Analysis of the data showed in Table-3. Components showed more than 90% clearance were considered as most influencing factor on biosurfactant production.

From the obtained data it was clear that carbon source, temperature and nitrogen source showed clearance value above 90% than i.e. 97.93%, 92.35% and 92.25%, respectively. Therefore, most influencing factor was carbon source (in present study potato peel powder) followed by temperature and nitrogen source (in present study urea).

After finding the critical factors i.e. potato peel powder, temperature and nitrogen source, response surface methodology was used to study interactions between factors and to find out exact concentration or values of the variables for biosurfactant production through CCD.

Optimization of media component and production parameter by Central composite Design (CCD)

Selected variables which gave significantly influence on biosurfactant production were optimized. Experiment was carried out using CCD which consisted of three levels i.e. low level, high level and central point for each variable. The design and obtained values are described in Table-4. Highest response (E24% = 50%) was obtained in run-4 (5 gm% potato peel powder, urea 1.5 gm% and temperature 45 °C) and lowest response (E24% = 52%). ANOVA analysis was carried out to scrutinize the variability produced by a factor. Regression analysis showed urea (X2) and temperature (X4) were significant while potato peel powder (X1) was insignificant (p < 0.05) variables. Linear positive co-efficient values of all three variables were non-significant. The interaction between X2 – X4 (urea and temperature; p=0.0248) was significant while X1 – X2 (potato peel powder and urea) as well as X1 – X4 (potato peel powder and temperature) were insignificant. The correlation co-efficient R2 indicates accuracy of model and it was 76.94% and adjusted R2 value was 56.19%. Second-order polynomial equation was used to study effect of factors on E24% (indirectly biosurfactant production). The equation for present study was as follow:

Y=78.1175+6.2663( X1)-52.3805X2-2.5806X4-1.4975X12+7.7356X22+0.0246X42=1.5X1×X2+0.0583X1×X4+1.175(X2×X4)

Here, Y is response i.e. E24% (indirectly biosurfactant production); X1 carbon source (potato peel powder), X2 nitrogen source (urea) and X4 temperature.

To evaluate the optimum value of each factor for highest E24% (or biosurfactant production), 3D response surface plots (Figure-3) were created in Design Expert software by plotting the response function of two factors while keeping another at central point. The significant interaction between X2 and X4 was obtained from response surface plot.

It was clear from the 3D plots that, interaction between X1-X2 and X1-X4 the center point of carbon source is 3.2 gm%. Beyond this concentration the predicted value of E24% was increased but, technically, the values of E24% never go beyond 100%. Therefore, values of variables were selected in such a way that response resided below 100%. Similarly, as the values of nitrogen source goes near to 4 mg% and temperature near to 28 °C, the response or value or E24% was predicted approximately to be 100%. Hence, for validation purpose, the E24% (response) was predicted 100% under optimized condition and verified by experimentation carried out in triplicates. Under un-optimized conditions, the E24% was 43.62 ± 2.55%; which was increased after optimization to 86.11 ± 3.47%. The achieved results proposed considerable accuracy in developed model and model validation under the prescribed conditions.

The data are comparable with the results reported in case of B. aryabhattai strain ZDY2 by (Yaraguppi et al. 2020). Author documented that, B. aryabhattai ZDY2 produced 2.51 fold higher biosurfactant under optimized media when supplied with 4.0% cruide oil, 0.7% yeast extract and 3.0 % NaNO3. Bacillus subtilis SPB1 produced 1.65 fold higher biosurfactant with optimized media supplemented with glucose, urea and K2HPO4 at 15 g/L, 6 g/L and 1g/L of concentration respectively (Mnif et al. 2012). Optimum conditions for biosurfactant production by B. subtilis N3-1P was 7% v/v brewery waste, 6.22 mg/L NH4NO3 containing media with pH 6.41 which was maintained at 27 °C (Moshtagh et al. 2019).

Partial characterization of biosurfactant

Biosurfactant produced by S. thalpophilum DP9 from optimized media was extracted simply by acid precipitation. Partial purified biosurfactant was analyzed for presence of carbohydrate, protein and lipid qualitatively by TLC and quantitatively by spectrophotometric methods. Thin layer chromatography results indicated presence of carbohydrate (Rf = 0.71) and protein (Rf = 0.31). Lipid was absent in the biosurfactant. Spectroscopic qualitative analysis revealed that biosurfactant contain 439.58 ± 0.0129  μg/ml of carbohydrate and 507.4 ± 0.0064 μg/ml of protein, while lipid was absent. On the basis of qualitative and quantitative tests, it was assumed that the biosurfactant was glycoprotein in nature.

Glycoprotein nature of biosurfactant was confirmed by PAS staining Native-PAGE gel. Moreover, the size of the protein part present in biosurfactant was evaluated by SDS-PAGE electrophoresis followed by Coomassie Brilliant Blue (CBB) staining and Silver staining. Approximate size of protein part carried by biosurfactant was found between 42 kDa to 51 kDa. We assumed that it was near to 48 kDa of size. The presence of single band in all three staining i.e. PAS staining, CBB staining and silver staining at almost same place indicated that the biosurfactant obtained was relatively pure. Further purity can be confirmed by High Pressure Liquid Chromatography (HPLC). Results are contradictory to previous reports. Burgos-Díaz et al. (2011) have reported that the Sphingobacterium stain 6.2S produces mixture of biosurfactant which belongs to lipopeptide, phospholipud or glycolipid. Similarly, Noparat et al. (2014) have documented that S. spiritivorum AS43 produced lipopeptide biosurfactant which having 50.2% lipid and 38.5% or protein in its structure. To our best knowledge, this is the first report of glycoprotein biosurfactant production from Sphingobacterium genus.

FTIR analysis

Molecular composition of partially purified biosurfactant was carried out by Fourier transform infrared spectroscopy. Weak stretching peak at 3292.61 cm–1 indicated presence of stretching vibrations from –NH of peptide (Noparat et al. 2014). Strong peaks at 3009.51 cm–1 and 2927.05 cm–1 indicated –CH stretching vibrations from –CH2 and –CH3 group of aliphatic chain (Burgos-Díaz et al. 2011; Noparat et al. 2014; Yaraguppi et al. 2020). Stretching at 1746.68 cm–1 indicated presence of ester or carbonyl –C=O of –COOH (Yaraguppi et al. 2020) and 1463.71 cm–1 described –C–O–H planner bending of carboxylic acid or carbohydrate protons (Panjiar et al. 2020) or –C–O stretch of ester (Burgos-Díaz et al. 2011). 1652.17 cm–1 peak indicated presence of –CO–N bond stretching (Noparat et al. 2014) or –N–H stretching of –NH3+ from peptide group (Burgos-Díaz et al. 2011) which gave confirmation of presence of peptide bond (Hema et al. 2019). Peak at 1541.01 cm–1 designated to –NH bond at deformation mode combined with –CN stretching or aliphatic and peptide like moiety (Noparat et al. 2014). Peaks at 1378.85 cm–1, 1239.47 cm–1 and 1164.67 cm–1 were observed due to ester –CO bond stretching, –CH stretching and lactones –C–O bond stretching or pyranose –C–O–C– symmetric stretching, respectively (Hema et al. 2019;  Yaraguppi et al. 2020;  Panjiar et al. 2020). Peak at 1099.02 cm–1 indicated the stretch of –C–N group of aliphatic amines or –C–O–H stretch of alcohol or phenol or –C–O of ester, ether, alcohol or carboxylic acids. 77=22.32 cm–1 peak occurred due to vibrations of primary or secondary amines or due to methylene scissoring vibrations of protein moiety (Yaraguppi et al. 2020). Hence, from the FTIR chromatogram (peaks at 1652.17 cm–1, 1541.01 cm–1 , 1463.71 cm–1 and 722.32 cm–1 ) the presence of peptide bond, peptide moiety, carbohydrate protons and primary as well as secondary amines were confirmed which again indicated that the biosurfactant is belongs to glycoprotein class.