Supporting Information

Iron-Lanthanum supported on graphite sheets for As(III) removal from aqueous solution: Kinetics, thermodynamic and ecotoxicity assessment

Hong-yu Sun1,Yu-tan Chu1, Yan Li1, Jia-hao Shi1, Qiong-li BAO1*, Yi-zong HUANG2*

1 Innovation Team of Toxicology and Pollution Remediation of Heavy metal, Agro-Environmental Protection Institute, Ministry of Agriculture and Rural Affairs, Tianjin 300191, China

2 School of Energy and Environmental Sciences, Yunnan Normal University, Kunming 650500, China

* Corresponding author.

E-mail address: qionglibao1982@163.com. yizonghuang@126.com.

Langmuir and Freundlich isotherm

The Langmuir and Freundlich isotherm models were used to describe the arsenic adsorption data.

The Langmuir isotherm model assumes that the adsorbent surface only comprises a surface monolayer and adsorption occurs homogeneously. The Langmuir isotherm is expressed as follows:

Qe=QmKLCe1+KLCe

where Ce (mg/L) is the equilibrium concentration of arsenate and arsenate after adsorption; Qe (mg/g) is the adsorption capacity at equilibrium; Qm is the calculated maximum adsorption capacity (mg/g); and KL (L/mg) is the Langmuir constant.

The Freundlich isotherm model describes the multilayer adsorption of an adsorbate on a heterogeneous adsorbent surface. The Freundlich isotherm is represented by the following equation:

Qe=KF Cen

where KF is the Freundlich adsorption constant; and n denotes the interaction of adsorbate and adsorbent.

Kinetic models

In order to further understand the adsorption behavior of FL-graphite sheet composite samples, the adsorption kinetics of the adsorbent was studied. In the adsorption process, the determination of adsorption kinetics is a necessary condition to understand the adsorption mechanism of adsorbent. In this paper, pseudo-first-order and pseudo-second-order dynamics are used.

The pseudo-first-order equation can be expressed as:

Qt =Qm(1-e-K1t)

The pseudo-second-order equation can be expressed as:

Qt=Qm2 K2t1+QmK2t

where Qt (mg/g) is the amount of As(III) adsorbed at time t (min), Qm (mg/g) is the amount of adsorption at equilibrium, and K1 (1/min) and K2 (g/mg·min) are the kinetic rate constants for the pseudo-first-order and the pseudo-second-order models, respectively.

Fig. S1. Morphological and structural characterization of graphite sheets (a) SEM image, (b) XRD image, (c) AFM image and (d) XPS image.

Fig. S2. Mapping images of FL-graphite sheet.

Table S1. Langmuir and Freundlich isotherm parameters calculated for As(III) adsorption.

| Sample | T(k) | Qe(mg/g) | Langmuir model |  |  | Feeundlich model |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | KL(L/mg) | Qm(mg/g) | R2 | n | KF(L/mg) | R2 |
| Graphite sheet | 293 | 13.53 | 0.18 | 12.48 | 0.967 | 0.60 | 5.91 | 0.869 |
|  | 303 | 18.34 | 0.49 | 18.70 | 0.973 | 0.65 | 2.23 | 0.935 |
|  | 313 | 21.91 | 0.49 | 19.52 | 0.978 | 0.57 | 6.11 | 0.835 |
| La-graphite sheet | 293 | 19.14 | 1.10 | 10.78 | 0.674 | 0.65 | 4.99 | 0.987 |
|  | 303 | 22.58 | 0.57 | 17.05 | 0.815 | 0.59 | 4.57 | 0.973 |
|  | 313 | 26.06 | 0.40 | 23.50 | 0.856 | 0.60 | 3.72 | 0.948 |
| Fe-graphite sheet | 293 | 34.08 | 3.50 | 30.01 | 0.964 | 0.52 | 11.78 | 0.810 |
|  | 303 | 37.22 | 3.16 | 32.03 | 0.950 | 0.62 | 12.12 | 0.772 |
|  | 313 | 40.26 | 3.04 | 38.96 | 0.946 | 0.62 | 13.11 | 0.800 |
| FL-graphite sheet | 293 | 42.64 | 3.47 | 38.29 | 0.841 | 0.53 | 15.83 | 0.967 |
|  | 303 | 45.98 | 3.15 | 41.35 | 0.824 | 0.66 | 15.22 | 0.984 |
|  | 313 | 51.69 | 2.88 | 45.55 | 0.745 | 0.89 | 17.08 | 0.962 |

Table S2. Kinetics parameters calculated for As(III) adsorption onto graphite-based materials

| Sample | Qmexp (mg/g) | Pseudo-first-order model |  |  | Pseudo-second-order model |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | K1 (1/min) | Qmcal (mg/g) | R2 | K2 （g/mg·min） | Qmcal (mg/g) | R2 |
| Graphite sheet | 16.95 | 0.43 | 14.80 | 0.936 | 1.67 | 10.69 | 0.388 |
| La-Graphite sheet | 16.90 | 1.12 | 14.71 | 0.948 | -2.06 | 13.03 | 0.374 |
| Fe-Graphite sheet | 26.45 | 1.33 | 24.91 | 0.959 | -3.56 | 23.31 | 0.628 |
| FL-Graphite sheet | 29.85 | 2.37 | 29.63 | 0.987 | 0.28 | 30.06 | 0.999 |

Fig. S3. Characterization of the adsorbents after adsorption of arsenite: mapping image of arsenite+FL-graphite sheet.

Fig. S4. Effects of FL-graphite sheet on (a) germination potential and (b) germination rate of rice seeds.