APPENDIX

Secondary Product Use Category Descriptions

New residential construction- All wood products used in the construction of housing built in year 0 for single family housing, multi-family housing, and manufactured housing. Does not include furniture but does include permanent furnishings. Sub-categories include floors, walls, roofs, foundations, decks, exterior doors, interior doors, garage doors, windows, moulding, shelving, cabinets, and countertops. Does not include outdoor fence posts. (63–65)

Residential upkeep - All wood products used in maintenance, repair, additions, alterations, and replacements done on single family housing, multi-family housing, and manufactured housing. (64)

New building construction - All wood products used in the construction of lodging, office, commercial, healthcare, educational, religious, public safety, amusement and recreation, and manufactured buildings built in year 0. (64)

Other new structures - All wood products used in the construction of transportation, communication, dams, power, highway and street, sewage and waste disposal, water supply, and conservation and development projects built in year 0. Does not include utility poles or marine pilings. (64)

Other construction (non-residential upkeep, etc.) - All wood products used in maintenance, repair, additions, alterations, and replacements done to non-residential buildings and structures, and any other wood products used in construction and not included elsewhere (64).

Pallets - Includes wooden shipping pallets.

Other shipping - Includes wood boxes, crates, hampers, baskets, dunnage, blocking, bracing, and all other wood products used in shipping and handling, excluding pallets. (64)

Poles - All wooden utility poles.

Posts and pilings - Marine pilings and fence posts. Consumption data for this category, as separate from “poles,” “other new structures,” or “miscellaneous,” were not available. This category was created separately from “poles” to account for the shorter life span of marine pilings vs. utility poles. It was created separately from “other new structures” to account for recycling of treated wood from utility poles and the shorter lifespans of these downcycled products.

Bedding - Wood chips, shavings, or sawdust used by animals, including livestock, poultry, horses, small pets, etc.

Landscaping - Includes bark and chip mulch and any other wood product or byproduct used in landscaping.

Corrugated Boxes - Includes linerboard kraft, which typically makes up the outer layer of corrugated boxes, and corrugating medium, which typically is used for the fluting or middle layer (66).

Sanitary Products - Includes commercial and retail bathroom tissue, facial tissue, and other tissue. Excludes food and kitchen related tissues such as napkins and paper towels. Also includes products manufactured with wood fluff, such as diapers, incontinence pads, and menstrual products (66).

Packaging Cartonboard - Includes virgin cartonboard products used for packaging a variety of grocery store and other retail items. This category is intended for cartonboard that is kept relatively clean and may therefore be recycled. It includes cartonboard products such as soda, beer, and other beverage carriers; boxes containing dry and frozen food, auto parts, toys, etc.; cigarette cartons; milk and juice cartons; etc. (66).

Disposable Food Related Products - Includes of variety of paper products used to package, carry, eat, or clean food and other contaminating products. This category is intended for items that are not typically recycled. It includes products such as paper bags; pet food and cement bags; fast food wrappers; microwavable popcorn bags; butter and meat wrappers; paper napkins and towels; paper plates, cups, and bowls; etc. (66).

Miscellaneous Paper Products - Includes cigarette filters, cellophane, rayon, newsprint, bristols, and any other paper and pulp products not included in previous categories (66).

Furniture - Includes household, commercial, and institutional furniture. (64)

Other manufacturing - Includes all manufactured items that are not furniture, such as musical instruments, sports equipment, toys, brooms and mops, caskets, etc. (64)

Miscellaneous - Wood products that do not fit into other categories, including craft and hobby projects, advertising structures, excelsior, mining supports, wood flour, non-structural wood used in non-residential construction, etc. (64)

Table A1. Parameters Used in LobWISE

| PARAMETER |  |  |  | SOURCE |
| --- | --- | --- | --- | --- |
| Specific gravity of wood | 0.47 |  |  | (60) |
| Sawmill Efficiency |  |  |  |  |
| 8-11 in DBH | 0.35 |  |  | (40,59,60,67) professional judgment |
| 12-18 in DBH | 0.4 |  |  |  |
| 19+ in DBH | 0.5 |  |  |  |
| General End Use Category by Product |  |  |  |  |
| Lumber |  |  |  |  |
| New Residential | 0.317 |  |  | (64) |
| Residential Upkeep | 0.305 |  |  | (64) |
| New Non-residential | 0.067 |  |  | (64) |
| Shipping | 0.169 |  |  | (64) |
| Manufacturing | 0.08 |  |  | (64) |
| Other | 0.054 |  |  | (64) |
| Softwood Plywood |  |  |  |  |
| New Residential | 0.485 |  |  | (64) |
| Residential Upkeep | 0.232 |  |  | (64) |
| New Non-residential | 0.058 |  |  | (64) |
| Shipping | 0.059 |  |  | (64) |
| Manufacturing | 0.09 |  |  | (64) |
| Other | 0.077 |  |  | (64) |
| OSB |  |  |  |  |
| New Residential | 0.485 |  |  | (64) |
| Residential Upkeep | 0.232 |  |  | (64) |
| New Non-residential | 0.058 |  |  | (64) |
| Shipping | 0.059 |  |  | (64) |
| Manufacturing | 0.09 |  |  | (64) |
| Other | 0.077 |  |  | (64) |
| Non-structural Panels |  |  |  |  |
| New Residential | 0.014 |  |  | (64) |
| Residential Upkeep | 0.216 |  |  | (64) |
| New Non-residential | 0.058 |  |  | (64) |
| Shipping | 0.03 |  |  | (64) |
| Manufacturing | 0.37 |  |  | (64) |
| Other | 0.314 |  |  | (64) |
| Engineered Wood |  |  |  |  |
| New Residential | 0.8 |  |  | Professional judgment |
| Residential Upkeep | 0.05 |  |  | Professional judgment |
| New Non-residential | 0.15 |  |  | Professional judgment |
| Shipping | 0 |  |  | Professional judgment |
| Manufacturing | 0 |  |  | Professional judgment |
| Other | 0 |  |  | Professional judgment |
| Poles |  |  |  |  |
| Poles | 0.35 |  |  | (68) professional judgment |
| Posts and pilings | 0.65 |  |  | (68) professional judgment |
| New Residential | 0 |  |  | Professional judgment |
| Residential Upkeep | 0 |  |  | Professional judgment |
| New Non-residential | 0 |  |  | Professional judgment |
| Shipping | 0 |  |  | Professional judgment |
| Manufacturing | 0 |  |  | Professional judgment |
| Other | 0 |  |  | Professional judgment |
| Paper and Paper Products |  |  |  |  |
| From Roundwood |  |  |  |  |
| Corrugated Boxes | 0.556 |  |  | (66) |
| Sanitary Products | 0.227 |  |  | (66) |
| Packaging Cartonboard | 0.120 |  |  | (66) |
| Disposable Food Related Products | 0.061 |  |  | (66) |
| Miscellaneous Paper Products | 0.036 |  |  | (66) |
| From Coarse Residue |  |  |  |  |
| Corrugated Boxes | 0.608 |  |  | (66) |
| Sanitary Products | 0.196 |  |  | (66) |
| Packaging Cartonboard | 0.099 |  |  | (66) |
| Disposable Food Related Products | 0.068 |  |  | (66) |
| Miscellaneous Paper Products | 0.029 |  |  | (66) |
| From Fine Residue |  |  |  |  |
| Corrugated Boxes | 0 |  |  | (66) |
| Sanitary Products | 0.498 |  |  | (66) |
| Packaging Cartonboard | 0 |  |  | (66) |
| Disposable Food Related Products | 0.502 |  |  | (66) |
| Miscellaneous Paper Products | 0 |  |  | (66) |
| Specific End Use Category by Product |  |  |  |  |
| Lumber, New Residential |  |  |  |  |
| Manufactured | 0.042 |  |  | (64) |
| Multi-family homes | 0.125 |  |  | (64) |
| Single family homes | 0.833 |  |  | (64) |
| Lumber, Residential Upkeep |  |  |  |  |
| Manufactured | 0.042 |  |  |  |
| Multi-family homes | 0.125 |  |  |  |
| Single family homes | 0.833 |  |  |  |
| Lumber, New Non-residential |  |  |  |  |
| Buildings | 0.627 |  |  | (64) |
| Other | 0.373 |  |  | (64) |
| Lumber, Shipping |  |  |  |  |
| Pallets | 0.92 |  |  | (64) |
| Other | 0.08 |  |  | (64) |
| Lumber, Manufacturing |  |  |  |  |
| Furniture | 0.284 |  |  | (64) |
| Other | 0.716 |  |  | (64) |
| Softwood Plywood, New Residential |  |  |  |  |
| Manufactured | 0.037 |  |  | (64) |
| Multi-family homes | 0.11 |  |  | (64) |
| Single family homes | 0.853 |  |  | (64) |
| Softwood Plywood, Residential Upkeep |  |  |  |  |
| Manufactured | 0.037 |  |  |  |
| Multi-family homes | 0.11 |  |  |  |
| Single family homes | 0.853 |  |  |  |
| Softwood Plywood, New Non-residential |  |  |  |  |
| Buildings | 0.882 |  |  | (64) |
| Other | 0.118 |  |  | (64) |
| Softwood Plywood, Shipping |  |  |  |  |
| Pallets | 0 |  |  |  |
| Other | 1 |  |  | Professional judgment |
| Softwood Plywood, Manufacturing |  |  |  |  |
| Furniture | 0.225 |  |  | (64) |
| Other | 0.775 |  |  | (64) |
| OSB, New Residential |  |  |  |  |
| Manufactured | 0.037 |  |  | (64) |
| Multi-family homes | 0.11 |  |  | (64) |
| Single family homes | 0.853 |  |  | (64) |
| OSB, Residential Upkeep |  |  |  |  |
| Manufactured | 0.037 |  |  |  |
| Multi-family homes | 0.11 |  |  |  |
| Single family homes | 0.853 |  |  |  |
| OSB, New Non-residential |  |  |  |  |
| Buildings | 0.882 |  |  | (64) |
| Other | 0.118 |  |  | (64) |
| OSB, Shipping |  |  |  |  |
| Pallets | 0 |  |  |  |
| Other | 1 |  |  |  |
| OSB, Manufacturing |  |  |  |  |
| Furniture | 0.225 |  |  | (64) |
| Other | 0.775 |  |  | (64) |
| Non-structural Panels, New Residential |  |  |  |  |
| Manufactured | 0.823 |  |  | (64) |
| Multi-family homes | 0.002 |  |  | (64) |
| Single family homes | 0.175 |  |  | (64) |
| Non-structural Panels, Residential Upkeep |  |  |  |  |
| Manufactured | 0.823 |  |  |  |
| Multi-family homes | 0.002 |  |  |  |
| Single family homes | 0.175 |  |  |  |
| Non-structural Panels, New Non-residential |  |  |  |  |
| Buildings | 0.006 |  |  | (64) |
| Other | 0.994 |  |  | (64) |
| Non-structural Panels, Shipping |  |  |  |  |
| Pallets | 0 |  |  |  |
| Other | 1 |  |  |  |
| Non-structural Panels, Manufacturing |  |  |  |  |
| Furniture | 0.438 |  |  | (64) |
| Other | 0.562 |  |  | (64) |
| Engineered Wood, New Residential |  |  |  |  |
| Manufactured | 0.05 |  |  | Professional judgment |
| Multi-family homes | 0.202 |  |  | (65) |
| Single family homes | 0.748 |  |  | (65) |
| Engineered Wood, Residential Upkeep |  |  |  |  |
| Manufactured | 0.05 |  |  |  |
| Multi-family homes | 0.202 |  |  |  |
| Single family homes | 0.748 |  |  |  |
| Engineered Wood, New Non-residential |  |  |  |  |
| Buildings | 1 |  |  | Professional judgment |
| Other | 0 |  |  | Professional judgment |
| Engineered Wood, Shipping |  |  |  |  |
| Pallets | 0 |  |  |  |
| Other | 1 |  |  |  |
| Engineered Wood, Manufacturing |  |  |  |  |
| Furniture | 0.5 |  |  | Professional judgment |
| Other | 0.5 |  |  | Professional judgment |
| Poles and Pilings, New Residential |  |  |  |  |
| Manufactured | 0 |  |  | Professional judgment |
| Multi-family homes | 0 |  |  | Professional judgment |
| Single family homes | 1 |  |  | Professional judgment |
| Poles and Pilings, Residential Upkeep |  |  |  |  |
| Manufactured | 0 |  |  | Professional judgment |
| Multi-family homes | 0 |  |  | Professional judgment |
| Single family homes | 1 |  |  | Professional judgment |
| Poles and Pilings, New Non-residential |  |  |  |  |
| Buildings | 1 |  |  | Professional judgment |
| Other | 0 |  |  | Professional judgment |
| Poles and Pilings, Shipping |  |  |  |  |
| Pallets | 0 |  |  |  |
| Other | 1 |  |  |  |
| Poles and Pilings, Manufacturing |  |  |  |  |
| Furniture | 1 |  |  | Professional judgment |
| Other | 0 |  |  | Professional judgment |
| Product Weibull Coefficients (How long will the product stay in use?) | modal estimate | scale factor (k) | shape factor (λ) |  |
| Single Family Home (whole house) | 125 | 150 | 2.63 | (41) |
| Floors - lumber | 112 | 135 | 2.63 | (41,69) professional judgment |
| Walls - lumber | 112 | 135 | 2.63 | (41,69) professional judgment |
| Roofs - lumber | 125 | 150 | 2.63 | (41,69) |
| Foundations - lumber | 125 | 150 | 2.63 | (41,69) |
| Decks - lumber | 13 | 16 | 2.63 | (68) |
| Exterior doors - lumber | 112 | 135 | 2.63 | (41,69) professional judgment |
| Interior doors - lumber | 112 | 135 | 2.63 | (41,69) professional judgment |
| Garage doors - lumber | 25 | 30 | 2.63 | (70) |
| Windows - lumber | 30 | 36 | 2.63 | (41,69) |
| Moulding - lumber | 112 | 135 | 2.63 | (41,69) professional judgment |
| Shelving - lumber | 100 | 120 | 2.63 | (41,69) professional judgment |
| Cabinets - lumber | 50 | 60 | 2.63 | (41,69) |
| Countertops - lumber | 100 | 120 | 2.63 | (41,69) professional judgment |
| Floors - engineered wood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Walls - engineered wood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Roofs - engineered wood | 125 | 150 | 2.63 | (41,69) |
| Foundations - engineered wood | 125 | 150 | 2.63 | (41,69) |
| Decks - engineered wood | 13 | 16 | 2.63 | (68) |
| Exterior doors - engineered wood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Interior doors - engineered wood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Garage doors - engineered wood | 25 | 30 | 2.63 | (70) |
| Windows - engineered wood | 30 | 36 | 2.63 | (41,69) |
| Moulding - engineered wood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Shelving - engineered wood | 100 | 120 | 2.63 | (41,69) professional judgment |
| Cabinets - engineered wood | 50 | 60 | 2.63 | (41,69) |
| Countertops - engineered wood | 100 | 120 | 2.63 | (41,69) professional judgment |
| Floors - plywood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Walls - plywood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Roofs - plywood | 125 | 150 | 2.63 | (41,69) |
| Foundations - plywood | 125 | 150 | 2.63 | (41,69) |
| Decks - plywood | 13 | 16 | 2.63 | (68) |
| Exterior doors - plywood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Interior doors - plywood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Garage doors - plywood | 25 | 30 | 2.63 | (70) |
| Windows - plywood | 30 | 36 | 2.63 | (41,69) |
| Moulding - plywood | 112 | 135 | 2.63 | (41,69) professional judgment |
| Shelving - plywood | 100 | 120 | 2.63 | (41,69) professional judgment |
| Cabinets - plywood | 40 | 48 | 2.63 | (41,69) professional judgment |
| Countertops - plywood | 100 | 120 | 2.63 | (41,69) professional judgment |
| Floors - OSB | 112 | 135 | 2.63 | (41,69) professional judgment |
| Walls - OSB | 112 | 135 | 2.63 | (41,69) professional judgment |
| Roofs - OSB | 125 | 150 | 2.63 | (41,69) |
| Foundations - OSB | 125 | 150 | 2.63 | (41,69) |
| Decks - OSB | 13 | 16 | 2.63 | (68) |
| Exterior doors - OSB | 112 | 135 | 2.63 | (41,69) professional judgment |
| Interior doors - OSB | 112 | 135 | 2.63 | (41,69) professional judgment |
| Garage doors - OSB | 25 | 30 | 2.63 | (70) |
| Windows - OSB | 30 | 36 | 2.63 | (41,69) |
| Moulding - OSB | 112 | 135 | 2.63 | (41,69) professional judgment |
| Shelving - OSB | 100 | 120 | 2.63 | (41,69) professional judgment |
| Cabinets - OSB | 40 | 48 | 2.63 | (41,69) professional judgment |
| Countertops - OSB | 100 | 120 | 2.63 | (41,69) professional judgment |
| Floors - Non-structural panels | 112 | 135 | 2.63 | (41,69) professional judgment |
| Walls - Non-structural panels | 112 | 135 | 2.63 | (41,69) professional judgment |
| Roofs - Non-structural panels | 125 | 150 | 2.63 | (41,69) |
| Foundations - Non-structural panels | 125 | 150 | 2.63 | (41,69) |
| Decks - Non-structural panels | 13 | 16 | 2.63 | (68) |
| Exterior doors - Non-structural panels | 112 | 135 | 2.63 | (41,69) professional judgment |
| Interior doors - Non-structural panels | 112 | 135 | 2.63 | (41,69) professional judgment |
| Garage doors - Non-structural panels | 25 | 30 | 2.63 | (70) |
| Windows - Non-structural panels | 30 | 36 | 2.63 | (41,69) |
| Moulding - Non-structural panels | 112 | 135 | 2.63 | (41,69) professional judgment |
| Shelving - Non-structural panels | 100 | 120 | 2.63 | (41,69) professional judgment |
| Cabinets - Non-structural panels | 40 | 48 | 2.63 | (41,69) professional judgment |
| Countertops - Non-structural panels | 100 | 120 | 2.63 | (41,69) professional judgment |
| Floors - Poles and pilings | 125 | 150 | 2.63 | (41,69) professional judgment |
| Walls - Poles and pilings | 125 | 150 | 2.63 | (41,69) |
| Roofs - Poles and pilings | 125 | 150 | 2.63 | (41,69) |
| Foundations - Poles and pilings | 125 | 150 | 2.63 | (41,69) |
| Decks - Poles and pilings | 13 | 16 | 2.63 | (68) |
| Exterior doors - Poles and pilings | 125 | 150 | 2.63 | (41,69) |
| Interior doors - Poles and pilings | 125 | 150 | 2.63 | (41,69) |
| Garage doors - Poles and pilings | 25 | 30 | 2.63 | (70) |
| Windows - Poles and pilings | 30 | 36 | 2.63 | (41,69) |
| Moulding - Poles and pilings | 125 | 150 | 2.63 | (41,69) |
| Shelving - Poles and pilings | 100 | 120 | 2.63 | (41,69) professional judgment |
| Cabinets - Poles and pilings | 50 | 60 | 2.63 | (41,69) |
| Countertops - Poles and pilings | 100 | 120 | 2.63 | (41,69) professional judgment |
| Multi-Family Home (whole house) | 110 | 132 | 2.63 | (41) professional judgment |
| Floors - lumber | 99 | 119 | 2.63 | (41,69) professional judgment |
| Walls - lumber | 99 | 119 | 2.63 | (41,69) professional judgment |
| Roofs - lumber | 110 | 132 | 2.63 | (41,69) |
| Foundations - lumber | 110 | 132 | 2.63 | (41,69) |
| Decks - lumber | 13 | 16 | 2.63 | (68) |
| Exterior doors - lumber | 88 | 106 | 2.63 | (41,69) professional judgment |
| Interior doors - lumber | 88 | 106 | 2.63 | (41,69) professional judgment |
| Garage doors - lumber | 25 | 30 | 2.63 | (70) |
| Windows - lumber | 30 | 36 | 2.63 | (41,69) |
| Moulding - lumber | 88 | 106 | 2.63 | (41,69) professional judgment |
| Shelving - lumber | 77 | 92 | 2.63 | (41,69) professional judgment |
| Cabinets - lumber | 40 | 48 | 2.63 | (41,69) professional judgment |
| Countertops - lumber | 77 | 92 | 2.63 | (41,69) professional judgment |
| Floors - engineered wood | 99 | 119 | 2.63 | (41,69) professional judgment |
| Walls - engineered wood | 99 | 119 | 2.63 | (41,69) professional judgment |
| Roofs - engineered wood | 110 | 132 | 2.63 | (41,69) |
| Foundations - engineered wood | 110 | 132 | 2.63 | (41,69) |
| Decks - engineered wood | 13 | 16 | 2.63 | (68) |
| Exterior doors - engineered wood | 88 | 106 | 2.63 | (41,69) professional judgment |
| Interior doors - engineered wood | 88 | 106 | 2.63 | (41,69) professional judgment |
| Garage doors - engineered wood | 25 | 30 | 2.63 | (70) |
| Windows - engineered wood | 30 | 36 | 2.63 | (41,69) |
| Moulding - engineered wood | 88 | 106 | 2.63 | (41,69) professional judgment |
| Shelving - engineered wood | 77 | 92 | 2.63 | (41,69) professional judgment |
| Cabinets - engineered wood | 40 | 48 | 2.63 | (41,69) professional judgment |
| Countertops - engineered wood | 77 | 92 | 2.63 | (41,69) professional judgment |
| Floors - plywood | 99 | 119 | 2.63 | (41,69) professional judgment |
| Walls - plywood | 99 | 119 | 2.63 | (41,69) professional judgment |
| Roofs - plywood | 110 | 132 | 2.63 | (41,69) |
| Foundations - plywood | 110 | 132 | 2.63 | (41,69) |
| Decks - plywood | 13 | 16 | 2.63 | (68) |
| Exterior doors - plywood | 88 | 106 | 2.63 | (41,69) professional judgment |
| Interior doors - plywood | 88 | 106 | 2.63 | (41,69) professional judgment |
| Garage doors - plywood | 25 | 30 | 2.63 | (70) |
| Windows - plywood | 30 | 36 | 2.63 | (41,69) |
| Moulding - plywood | 88 | 106 | 2.63 | (41,69) professional judgment |
| Shelving - plywood | 77 | 92 | 2.63 | (41,69) professional judgment |
| Cabinets - plywood | 30 | 36 | 2.63 | (41,69) professional judgment |
| Countertops - plywood | 77 | 92 | 2.63 | (41,69) professional judgment |
| Floors - OSB | 99 | 119 | 2.63 | (41,69) professional judgment |
| Walls - OSB | 99 | 119 | 2.63 | (41,69) professional judgment |
| Roofs - OSB | 110 | 132 | 2.63 | (41,69) |
| Foundations - OSB | 110 | 132 | 2.63 | (41,69) |
| Decks - OSB | 13 | 16 | 2.63 | (68) |
| Exterior doors - OSB | 88 | 106 | 2.63 | (41,69) professional judgment |
| Interior doors - OSB | 88 | 106 | 2.63 | (41,69) professional judgment |
| Garage doors - OSB | 25 | 30 | 2.63 | (70) |
| Windows - OSB | 30 | 36 | 2.63 | (41,69) |
| Moulding - OSB | 88 | 106 | 2.63 | (41,69) professional judgment |
| Shelving - OSB | 77 | 92 | 2.63 | (41,69) professional judgment |
| Cabinets - OSB | 30 | 36 | 2.63 | (41,69) professional judgment |
| Countertops - OSB | 77 | 92 | 2.63 | (41,69) professional judgment |
| Floors - Non-structural panels | 99 | 119 | 2.63 | (41,69) professional judgment |
| Walls - Non-structural panels | 99 | 119 | 2.63 | (41,69) professional judgment |
| Roofs - Non-structural panels | 110 | 132 | 2.63 | (41,69) |
| Foundations - Non-structural panels | 110 | 132 | 2.63 | (41,69) |
| Decks - Non-structural panels | 13 | 16 | 2.63 | (68) |
| Exterior doors - Non-structural panels | 88 | 106 | 2.63 | (41,69) professional judgment |
| Interior doors - Non-structural panels | 88 | 106 | 2.63 | (41,69) professional judgment |
| Garage doors - Non-structural panels | 25 | 30 | 2.63 | (70) |
| Windows - Non-structural panels | 30 | 36 | 2.63 | (41,69) |
| Moulding - Non-structural panels | 88 | 106 | 2.63 | (41,69) professional judgment |
| Shelving - Non-structural panels | 77 | 92 | 2.63 | (41,69) professional judgment |
| Cabinets - Non-structural panels | 30 | 36 | 2.63 | (41,69) professional judgment |
| Countertops - Non-structural panels | 77 | 92 | 2.63 | (41,69) professional judgment |
| Floors - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Walls - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Roofs - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Foundations - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Decks - Poles and pilings | 13 | 16 | 2.63 | (68) |
| Exterior doors - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Interior doors - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Garage doors - Poles and pilings | 25 | 30 | 2.63 | (70) |
| Windows - Poles and pilings | 30 | 36 | 2.63 | (41,69) |
| Moulding - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Shelving - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Cabinets - Poles and pilings | 40 | 48 | 2.63 | (41,69) professional judgment |
| Countertops - Poles and pilings | 110 | 132 | 2.63 | (41,69) |
| Other Products |  |  |  |  |
| Manufactured home | 70 | 84 | 2.63 | (41) professional judgment |
| Single family home upkeep | 37 | 45 | 2.63 | (41,71) |
| Multi-family home upkeep | 33 | 40 | 2.63 | (41,71) |
| Manufactured upkeep | 21 | 25 | 2.63 | (41,71) |
| Building | 70 | 84 | 2.63 | (41,42,72,73) |
| Other new structure | 60 | 72 | 2.63 | (41,74–76) |
| Other (non-residential upkeep, etc.) | 21 | 25 | 2.63 | (41,42,71–73) |
| Pallets | 1 | 1 | 2.63 | (41,77) |
| Other shipping | 2 | 2 | 2.63 | Professional judgment |
| Poles | 60 | 72 | 2.63 | (41,78) |
| Posts/pilings | 30 | 36 | 2.63 | (41,70,79) professional judgment |
| Bedding | 1 | 1 | 2.63 | Professional judgment |
| Landscaping | 2 | 2 | 2.63 | (41,80) |
| Corrugated boxes | 1 | 1 | 2.63 | (57) professional judgment |
| Sanitary products | 1 | 1 | 2.63 | (57) professional judgment |
| Packaging cartonboard | 1 | 1 | 2.63 | (57) professional judgment |
| Disposable food related products | 1 | 1 | 2.63 | (57) professional judgment (81) |
| Miscellaneous paper products | 1 | 1 | 2.63 | (57) professional judgment |
| Furniture | 13 | 16 | 2.63 | (41,81) |
| Other manufacturing | 7 | 8 | 2.63 | (41,82–85) Professional judgment |
| Miscellaneous | 4 | 5 | 2.63 | Professional judgment |
| Percent of "Decayed" Product Going to Landfill | MSW Landfill | C&D Landfill |  |  |
| Single family homes | 0 | 0.722 |  | (57) |
| Multi-family homes | 0 | 0.722 |  | (57) |
| Manufactured home | 0 | 0.722 |  | (57) |
| Single family home upkeep | 0 | 0.722 |  | (57) |
| Multi-family home upkeep | 0 | 0.722 |  | (57) |
| Manufactured upkeep | 0 | 0.722 |  | (57) |
| Building | 0 | 0.722 |  | (57) |
| Other new structure | 0 | 0.722 |  | (57) |
| Other (non-residential upkeep, etc.) | 0 | 0.722 |  | (57) |
| Pallets | 0.007 | 0.003 |  | (86,87) |
| Other shipping | 0.632 | 0 |  | (88) |
| Poles | 0 | 0.32 |  | (78) |
| Posts/pilings | 0 | 0.7 |  | (79) |
| Bedding | 0.5 | 0 |  | Professional judgment |
| Landscaping | 0 | 0 |  | Professional judgment |
| Corrugated boxes | 0.028 | 0 |  | (44) |
| Sanitary products | 0.656 | 0 |  | (44,66,89) |
| Packaging cartonboard | 0.640 | 0 |  | (44) |
| Disposable food related products | 0.819 | 0 |  | (44) |
| Miscellaneous paper products | 0.578 | 0 |  | (44) |
| Furniture | 0.801 | 0 |  | (44) |
| Other manufacturing | 0.801 | 0 |  | (44); Professional judgment |
| Miscellaneous | 0.801 | 0 |  | (44); Professional judgment |
| Percent of Transitioned Product Recycled as Landscaping |  |  |  |  |
| Single family homes | 0.061 |  |  | (57) |
| Multi-family homes | 0.061 |  |  | (57) |
| Manufactured home | 0.061 |  |  | (57) |
| Single family home upkeep | 0.061 |  |  | (57) |
| Multi-family home upkeep | 0.061 |  |  | (57) |
| Manufactured upkeep | 0.061 |  |  | (57) |
| Building | 0.061 |  |  | (57) |
| Other new structure | 0.061 |  |  | (57) |
| Other (non-residential upkeep, etc.) | 0.061 |  |  | (57) |
| Pallets | 0.109 |  |  | (87) |
| Other shipping | 0.138 |  |  | (87,88) Professional judgment |
| Poles | 0.061 |  |  |  |
| Posts/pilings | -- |  |  |  |
| Bedding | -- |  |  |  |
| Landscaping | -- |  |  |  |
| Packaging paper products | 0 |  |  |  |
| Household/sanitary | 0 |  |  |  |
| Printing/writing | 0 |  |  |  |
| Newsprint | 0 |  |  |  |
| Other paper products | 0 |  |  |  |
| Furniture | -- |  |  |  |
| Other manufacturing | -- |  |  |  |
| Miscellaneous | 0 |  |  |  |
| Percent of Transitioned Product Recycled as Bedding |  |  |  |  |
| Single family homes | 0 |  |  |  |
| Multi-family homes | 0 |  |  |  |
| Manufactured home | 0 |  |  |  |
| Single family home upkeep | 0 |  |  |  |
| Multi-family home upkeep | 0 |  |  |  |
| Manufactured upkeep | 0 |  |  |  |
| Building | 0 |  |  |  |
| Other new structure | 0 |  |  |  |
| Other (non-residential upkeep, etc.) | 0 |  |  |  |
| Pallets | 0.012 |  |  | (87) |
| Other shipping | 0 |  |  |  |
| Poles | 0 |  |  |  |
| Posts/pilings | -- |  |  |  |
| Bedding | -- |  |  |  |
| Landscaping | -- |  |  |  |
| Packaging paper products | 0 |  |  |  |
| Household/sanitary | 0 |  |  |  |
| Printing/writing | 0 |  |  |  |
| Newsprint | 0 |  |  |  |
| Other paper products | 0 |  |  |  |
| Furniture | -- |  |  |  |
| Other manufacturing | -- |  |  |  |
| Miscellaneous | 0 |  |  |  |
| Percent of Transitioned Product Recycled as Posts and Pilings |  |  |  |  |
| Single family homes | 0 |  |  |  |
| Multi-family homes | 0 |  |  |  |
| Manufactured home | 0 |  |  |  |
| Single family home upkeep | 0 |  |  |  |
| Multi-family home upkeep | 0 |  |  |  |
| Manufactured upkeep | 0 |  |  |  |
| Building | 0 |  |  |  |
| Other new structure | 0 |  |  |  |
| Other (non-residential upkeep, etc.) | 0 |  |  |  |
| Pallets | 0 |  |  |  |
| Other shipping | 0 |  |  |  |
| Poles | 0.409 |  |  | (57,79) |
| Posts/pilings | -- |  |  |  |
| Bedding | -- |  |  |  |
| Landscaping | -- |  |  |  |
| Packaging paper products | 0 |  |  |  |
| Household/sanitary | 0 |  |  |  |
| Printing/writing | 0 |  |  |  |
| Newsprint | 0 |  |  |  |
| Other paper products | 0 |  |  |  |
| Furniture | -- |  |  |  |
| Other manufacturing | -- |  |  |  |
| Miscellaneous | 0 |  |  |  |
| Percent of Transitioned Product Recycled as Structural Panels |  |  |  |  |
| Single family homes | 0 |  |  |  |
| Multi-family homes | 0 |  |  |  |
| Manufactured home | 0 |  |  |  |
| Single family home upkeep | 0 |  |  |  |
| Multi-family home upkeep | 0 |  |  |  |
| Manufactured upkeep | 0 |  |  |  |
| Building | 0 |  |  |  |
| Other new structure | 0 |  |  |  |
| Other (non-residential upkeep, etc.) | 0 |  |  |  |
| Pallets | 0 |  |  |  |
| Other shipping | 0 |  |  |  |
| Poles | 0 |  |  |  |
| Posts/pilings | -- |  |  |  |
| Bedding | -- |  |  |  |
| Landscaping | -- |  |  |  |
| Packaging paper products | 0 |  |  |  |
| Household/sanitary | 0 |  |  |  |
| Printing/writing | 0 |  |  |  |
| Newsprint | 0 |  |  |  |
| Other paper products | 0 |  |  |  |
| Furniture | -- |  |  |  |
| Other manufacturing | -- |  |  |  |
| Miscellaneous | 0 |  |  |  |
| Percent of Transitioned Product Recycled as Non-Structural Panels |  |  |  |  |
| Single family homes | 0.029 |  |  | (57) |
| Multi-family homes | 0.029 |  |  | (57) |
| Manufactured home | 0.029 |  |  | (57) |
| Single family home upkeep | 0.029 |  |  | (57) |
| Multi-family home upkeep | 0.029 |  |  | (57) |
| Manufactured upkeep | 0.029 |  |  | (57) |
| Building | 0.029 |  |  | (57) |
| Other new structure | 0.029 |  |  | (57) |
| Other (non-residential upkeep, etc.) | 0.029 |  |  | (57) |
| Pallets | 0 |  |  |  |
| Other shipping | 0 |  |  |  |
| Poles | 0 |  |  |  |
| Posts/pilings | -- |  |  |  |
| Bedding | -- |  |  |  |
| Landscaping | -- |  |  |  |
| Packaging paper products | 0 |  |  |  |
| Household/sanitary | 0 |  |  |  |
| Printing/writing | 0 |  |  |  |
| Newsprint | 0 |  |  |  |
| Other paper products | 0 |  |  |  |
| Furniture | -- |  |  |  |
| Other manufacturing | -- |  |  |  |
| Miscellaneous | 0 |  |  |  |
| Emissions from Fossil Fuels |  |  |  |  |
| (metric tonnes of CO2e emitted per tonne of CO2e stored in final product) | Extraction | Trans-portation | Production |  |
| Lumber | 0.0186 | 0.0168 | 0.0882 | (60) |
| Plywood | 0.0715 | 0.0096 | 0.1906 | (90) |
| OSB | 0.0687 | 0.0060 | 0.1309 | (91) |
| Non-structural panels | 0.2007 | 0.0057 | 0.2360 | (92,93) |
| Engineered wood |  |  | 0.3902 | (94–99) |
| Poles | 0.0186 | 0.0168 | 0.0882 |  |
| Paper and paper products | 0.0533 | 0.0484 | 0.2609 | (53) |
| Pellets |  |  | 0.0867 | (52) |
| Decay in Landfills | MSW | C&D |  |  |
| Portion of paper that is decayable | 0.9 | 0.9 |  | (100) |
| Portion of wood that is decayable | 0.72 | 0.72 |  | (100) |
| Half-life of decayable paper | 15 | 15 |  | (36) |
| Half-life of decayable wood | 30 | 30 |  | (36) |
| Methane Emissions from Landfills | MSW | C&D |  |  |
| F (portion of landfill gas that is CH4 by volume) | 0.5 | 0.5 |  | (43) |
| R (portion of generated CH4 that is recovered) | 0.6019 | 0 |  | (15) |
| OX (portion of non-recovered CH4 that is oxidized) | 0.2221 | 0.1 |  | (15) |
| Methane to CO2e conversion factor | 30 | 30 |  | (53) |
| Construction and manufacturing waste |  |  |  |  |
| Portion transitioned from housing pool at time of construction | 0.056 |  |  | (16,64,68) |
| Portion transitioned from other pools at time of manufacture (excludes poles, landscaping, and bedding) | 0.08 |  |  | (30) |
| Engineered wood (glu-lam, I-joists, LVL, PSL, LSL, OSL, OSB rim boards, SW plywood rim boards) |  |  |  |  |
| Portion of lumber that is made into engineered wood | 0.03 |  |  | (64) professional judgment |
| Portion of plywood that is made into engineered wood | 0.06 |  |  | (64) professional judgment |
| Portion of OSB that is made into engineered wood | 0.1 |  |  | (64) professional judgment |
| Percent of "Fiber Product" that Goes Into Composite vs. Paper Products |  |  |  |  |
| Bark --> paper | 0.181 |  |  | (66) |
| Bark --> composites | 0.819 |  |  | (66) |
| Fine residue --> paper | 0.181 |  |  | (66) |
| Fine residue --> composites | 0.819 |  |  | (66) |
| Coarse residue --> paper | 0.954 |  |  | (66) |
| Coarse residue --> composites | 0.046 |  |  | (66) |
| Residue and Product Production |  |  |  |  |
| (Of the C that goes into the mill, what portion of C comes out as product vs. each residue type) |  |  |  |  |
| Sawtimber |  |  |  |  |
| Base residue portions (with total residue equaling 100%) |  |  |  |  |
| Bark | 0.14 |  |  | (47) |
| Coarse residue | 0.48 |  |  | (47) |
| Fine residue | 0.38 |  |  | (47) |
| Calculated residue portions (based on mill efficiency) |  |  |  |  |
| 8-11 in DBH |  |  |  |  |
| Bark | 0.09 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| Coarse residue | 0.31 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| Fine residue | 0.25 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| 12-18 in DBH |  |  |  |  |
| Bark | 0.08 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| Coarse residue | 0.29 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| Fine residue | 0.23 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| 19+ in DBH |  |  |  |  |
| Bark | 0.07 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| Coarse residue | 0.24 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| Fine residue | 0.19 |  |  | Calculated from “sawtimber - base residue portions” and “sawmill efficiency” coefficients |
| Pulpwood |  |  |  |  |
| Pulp | 0.89 |  |  | (47) |
| Bark | 0.11 |  |  | (47) |
| Veneer |  |  |  |  |
| Plywood | 0.41 |  |  | (47) |
| Bark | 0.09 |  |  | (47) |
| Coarse residue | 0.3 |  |  | (47) |
| Fine residue | 0.23 |  |  | (47) |
| Poles |  |  |  |  |
| Poles | 0.81 |  |  | (47) |
| Bark | 0.13 |  |  | (47) |
| Coarse residue | 0.06 |  |  | (47) |
| Bioenergy |  |  |  |  |
| Fuel product | 0.9 |  |  | (47) |
| Bark | 0.1 |  |  | (47) |
| Composites |  |  |  |  |
| Composite products | 0.9 |  |  | (47) |
| Bark | 0.1 |  |  | (47) |
| Residue Use |  |  |  |  |
| Pulpwood - Bark |  |  |  |  |
| Landscaping | 0.03 |  |  | (47) |
| Burned | 0.97 |  |  | (47) |
| Sawtimber - Bark |  |  |  |  |
| Landscaping | 0.48 |  |  | (47) |
| Miscellaneous | 0.02 |  |  | (47) |
| Fiber product | 0.01 |  |  | (47) |
| Pellets | 0.02 |  |  | (47) |
| Otherwise burned | 0.47 |  |  | (47) |
| Sawtimber - Coarse Residue |  |  |  |  |
| Landscaping | 0.01 |  |  | (47) |
| Miscellaneous | 0.095 |  |  | (47) |
| Fiber product | 0.79 |  |  | (47) |
| Animal bedding | 0.005 |  |  | (47) |
| Pellets | 0.02 |  |  | (47) |
| Otherwise burned | 0.08 |  |  | (47) |
| Sawtimber - Fine Residue |  |  |  |  |
| Landscaping | 0.01 |  |  | (47) |
| Miscellaneous | 0.03 |  |  | (47) |
| Fiber product | 0.11 |  |  | (47) |
| Animal bedding | 0.08 |  |  | (47) |
| Pellets | 0.19 |  |  | (47) |
| Otherwise burned | 0.58 |  |  | (47) |
| Composites - Bark |  |  |  |  |
| Landscaping | 0.1 |  |  | (47) |
| Fiber product | 0.06 |  |  | (47) |
| Burned | 0.84 |  |  | (47) |
| Veneer - Bark |  |  |  |  |
| Landscaping | 0.08 |  |  | (47) |
| Burned | 0.92 |  |  | (47) |
| Veneer - Coarse Residue |  |  |  |  |
| Fiber product | 0.76 |  |  | (47) |
| Burned | 0.24 |  |  | (47) |
| Veneer - Fine Residue |  |  |  |  |
| Landscaping | 0.02 |  |  | (47) |
| Fiber product | 0.07 |  |  | (47) |
| Burned | 0.91 |  |  | (47) |
| Poles - Bark |  |  |  |  |
| Landscaping | 0.65 |  |  | (47) |
| Fiber product | 0.01 |  |  | (47) |
| Burned | 0.34 |  |  | (47) |
| Poles - Coarse Residue |  |  |  |  |
| Landscaping | 0.15 |  |  | (47) |
| Miscellaneous | 0.08 |  |  | (47) |
| Fiber product | 0.5 |  |  | (47) |
| Burned | 0.27 |  |  | (47) |
| Bioenergy - Bark |  |  |  |  |
| Landscaping | 0.04 |  |  | (47) |
| Burned | 0.96 |  |  | (47) |
| Recycling (as same product) |  |  |  |  |
| Pallets |  |  |  |  |
| Portion of pallets recycled as pallets | 0.7 |  |  | (87) |
| Number of times pallets are recycled | 3 |  |  | (87,101) |
| Paper and pulp products |  |  |  |  |
| Portion of corrugated boxes recycled as corrugated boxes | 0.914 |  |  | (58) |
| Number of times corrugated boxes are recycled | 5 |  |  | (102) |
| Portion of sanitary products recycled as sanitary products | 0 |  |  | (44) |
| Number of times sanitary products are recycled | 5 |  |  | (102) |
| Portion of packaging cartonboard recycled as packaging cartonboard | 0.208 |  |  | (44) |
| Number of packaging cartonboard is recycled | 5 |  |  | (102) |
| Portion of disposable food related products recycled as disposable food related products | 0 |  |  | (44) |
| Number of times disposable food related products are recycled | 5 |  |  | (102) |
| Portion of miscellaneous paper products recycled as miscellaneous paper products | 0.081 |  |  | (44,66) |
| Number of times miscellaneous paper is recycled | 5 |  |  | (102) |
| Where in the house does the wood go? |  |  |  |  |
| Step 1 - wood is divided into either structural parts of the house or non-structural parts of the house |  |  |  |  |
| Single Family | Structural Parts | Non- structural Parts |  |  |
| Lumber | 0.926 | 0.074 |  | (63) |
| Engineered wood | 1 | 0 |  | Professional judgment |
| Structural panels | 0.992 | 0.008 |  | (63) |
| Non-structural panels | 0.57 | 0.43 |  | (63) |
| Poles and pilings | 1 | 0 |  | Professional judgment |
| Multi-Family |  |  |  |  |
| Lumber | 0.924 | 0.076 |  | (63) |
| Engineered wood | 1 | 0 |  | Professional judgment |
| Structural panels | 0.982 | 0.018 |  | (63) |
| Non-structural panels | 0.441 | 0.559 |  | (63) |
| Poles and pilings | 1 | 0 |  | Professional judgment |
| Step 2 - structural/non-structural subsets are divided into specific applications |  |  |  |  |
| Structural Parts | Single Family | Multi- Family |  |  |
| Softwood lumber |  |  |  |  |
| Floors | 0.109 | 0.188 |  | (65) |
| Walls | 0.443 | 0.553 |  | (65) |
| Roofs | 0.351 | 0.254 |  | (65) |
| Foundations | 0.008 | 0.002 |  | (65) |
| Decks | 0.088 | 0.003 |  | (65) |
| Engineered wood |  |  |  |  |
| Floors | 0.862 | 0.906 |  | (65) |
| Walls | 0.089 | 0.059 |  | (65) |
| Roofs | 0.049 | 0.035 |  | (65) |
| Foundations | 0.000 | 0.000 |  | (65) |
| Decks | 0.000 | 0.000 |  | (65) |
| Softwood plywood |  |  |  |  |
| Floors | 0.339 | 0.453 |  | (65) |
| Walls | 0.203 | 0.118 |  | (65) |
| Roofs | 0.455 | 0.429 |  | (65) |
| Foundations | 0.003 | 0.000 |  | (65) |
| Decks | 0.000 | 0.000 |  | (65) |
| OSB |  |  |  |  |
| Floors | 0.236 | 0.427 |  | (65) |
| Walls | 0.340 | 0.153 |  | (65) |
| Roofs | 0.424 | 0.420 |  | (65) |
| Foundations | 0.000 | 0.000 |  | (65) |
| Decks | 0.000 | 0.000 |  | (65) |
| Non-structural panels |  |  |  |  |
| Floors | 0.918 | 0.563 |  | (65) |
| Walls | 0.082 | 0.438 |  | (65) |
| Roofs | 0.000 | 0.000 |  | (65) |
| Foundations | 0.000 | 0.000 |  | (65) |
| Decks | 0.000 | 0.000 |  | (65) |
| Poles and Pilings |  |  |  |  |
| Floors | 0.100 | 0.100 |  |  |
| Walls | 0.600 | 0.600 |  |  |
| Roofs | 0.100 | 0.100 |  |  |
| Foundations | 0.100 | 0.100 |  |  |
| Decks | 0.100 | 0.100 |  |  |
| Non-structural Parts | Single Family | Multi- Family |  |  |
| Lumber |  |  |  |  |
| Exterior doors | 0.059 | 0.045 |  | (63) |
| Interior doors | 0.397 | 0.338 |  | (63) |
| Garage doors | 0.046 | 0.030 |  | (63) |
| Windows | 0.100 | 0.030 |  | (63) |
| Moulding | 0.231 | 0.361 |  | (63) |
| Shelving | 0.037 | 0.023 |  | (63) |
| Cabinets | 0.130 | 0.173 |  | (63) |
| Countertops | 0.000 | 0.000 |  | (63) |
| Engineered Wood |  |  |  |  |
| Exterior doors | 0.000 | 0.000 |  | (63) |
| Interior doors | 0.000 | 0.000 |  | (63) |
| Garage doors | 0.000 | 0.000 |  | (63) |
| Windows | 0.000 | 0.000 |  | (63) |
| Moulding | 0.000 | 0.000 |  | (63) |
| Shelving | 0.000 | 0.000 |  | (63) |
| Cabinets | 0.000 | 0.000 |  | (63) |
| Countertops | 0.000 | 0.000 |  | (63) |
| Structural Panels |  |  |  |  |
| Exterior doors | 0.034 | 0.042 |  | (63) |
| Interior doors | 0.171 | 0.208 |  | (63) |
| Garage doors | 0.078 | 0.042 |  | (63) |
| Windows | 0.000 | 0.000 |  | (63) |
| Moulding | 0.000 | 0.000 |  | (63) |
| Shelving | 0.410 | 0.458 |  | (63) |
| Cabinets | 0.000 | 0.000 |  | (63) |
| Countertops | 0.307 | 0.250 |  | (63) |
| Non-structural Panels |  |  |  |  |
| Exterior doors | 0.096 | 0.030 |  | (63) |
| Interior doors | 0.212 | 0.077 |  | (63) |
| Garage doors | 0.072 | 0.012 |  | (63) |
| Windows | 0.000 | 0.000 |  | (63) |
| Moulding | 0.000 | 0.000 |  | (63) |
| Shelving | 0.108 | 0.098 |  | (63) |
| Cabinets | 0.256 | 0.692 |  | (63) |
| Countertops | 0.256 | 0.090 |  | (63) |
| Poles and Pilings |  |  |  |  |
| Exterior doors | 0.000 | 0.000 |  | Professional judgment |
| Interior doors | 0.000 | 0.000 |  | Professional judgment |
| Garage doors | 0.000 | 0.000 |  | Professional judgment |
| Windows | 0.300 | 0.300 |  | Professional judgment |
| Moulding | 0.700 | 0.700 |  | Professional judgment |
| Shelving | 0.000 | 0.000 |  | Professional judgment |
| Cabinets | 0.000 | 0.000 |  | Professional judgment |
| Countertops | 0.000 | 0.000 |  | Professional judgment |
| Step 3 - wood is divided throughout the whole house |  |  |  | (step 1 * step 2) |
| Structural Parts | Single Family | Multi- Family |  |  |
| Softwood lumber |  |  |  |  |
| Floors | 0.100 | 0.174 |  | Calculated from step 1 and 2 |
| Walls | 0.410 | 0.511 |  | Calculated from step 1 and 2 |
| Roofs | 0.325 | 0.235 |  | Calculated from step 1 and 2 |
| Foundations | 0.008 | 0.002 |  | Calculated from step 1 and 2 |
| Decks | 0.082 | 0.002 |  | Calculated from step 1 and 2 |
| Engineered wood |  |  |  |  |
| Floors | 0.862 | 0.906 |  | Calculated from step 1 and 2 |
| Walls | 0.089 | 0.059 |  | Calculated from step 1 and 2 |
| Roofs | 0.049 | 0.035 |  | Calculated from step 1 and 2 |
| Foundations | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Decks | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Softwood plywood |  |  |  |  |
| Floors | 0.336 | 0.445 |  | Calculated from step 1 and 2 |
| Walls | 0.201 | 0.116 |  | Calculated from step 1 and 2 |
| Roofs | 0.451 | 0.422 |  | Calculated from step 1 and 2 |
| Foundations | 0.003 | 0.000 |  | Calculated from step 1 and 2 |
| Decks | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| OSB |  |  |  |  |
| Floors | 0.234 | 0.419 |  | Calculated from step 1 and 2 |
| Walls | 0.337 | 0.150 |  | Calculated from step 1 and 2 |
| Roofs | 0.421 | 0.412 |  | Calculated from step 1 and 2 |
| Foundations | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Decks | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Non-structural panels |  |  |  |  |
| Floors | 0.523 | 0.248 |  | Calculated from step 1 and 2 |
| Walls | 0.047 | 0.193 |  | Calculated from step 1 and 2 |
| Roofs | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Foundations | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Decks | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Poles and Pilings |  |  |  |  |
| Floors | 0.057 | 0.044 |  | Calculated from step 1 and 2 |
| Walls | 0.342 | 0.265 |  | Calculated from step 1 and 2 |
| Roofs | 0.057 | 0.044 |  | Calculated from step 1 and 2 |
| Foundations | 0.057 | 0.044 |  | Calculated from step 1 and 2 |
| Decks | 0.057 | 0.044 |  | Calculated from step 1 and 2 |
| Non-structural Parts |  |  |  |  |
| Lumber |  |  |  |  |
| Exterior doors | 0.004 | 0.003 |  | Calculated from step 1 and 2 |
| Interior doors | 0.029 | 0.026 |  | Calculated from step 1 and 2 |
| Garage doors | 0.003 | 0.002 |  | Calculated from step 1 and 2 |
| Windows | 0.007 | 0.002 |  | Calculated from step 1 and 2 |
| Moulding | 0.017 | 0.027 |  | Calculated from step 1 and 2 |
| Shelving | 0.003 | 0.002 |  | Calculated from step 1 and 2 |
| Cabinets | 0.010 | 0.013 |  | Calculated from step 1 and 2 |
| Countertops | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Engineered Wood |  |  |  |  |
| Exterior doors | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Interior doors | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Garage doors | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Windows | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Moulding | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Shelving | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Cabinets | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Countertops | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Structural Panels |  |  |  |  |
| Exterior doors | 0.000 | 0.001 |  | Calculated from step 1 and 2 |
| Interior doors | 0.001 | 0.004 |  | Calculated from step 1 and 2 |
| Garage doors | 0.001 | 0.001 |  | Calculated from step 1 and 2 |
| Windows | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Moulding | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Shelving | 0.003 | 0.008 |  | Calculated from step 1 and 2 |
| Cabinets | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Countertops | 0.002 | 0.005 |  | Calculated from step 1 and 2 |
| Non-structural Panels |  |  |  |  |
| Exterior doors | 0.041 | 0.017 |  | Calculated from step 1 and 2 |
| Interior doors | 0.091 | 0.043 |  | Calculated from step 1 and 2 |
| Garage doors | 0.031 | 0.007 |  | Calculated from step 1 and 2 |
| Windows | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Moulding | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Shelving | 0.046 | 0.055 |  | Calculated from step 1 and 2 |
| Cabinets | 0.110 | 0.387 |  | Calculated from step 1 and 2 |
| Countertops | 0.110 | 0.050 |  | Calculated from step 1 and 2 |
| Poles and Pilings |  |  |  |  |
| Exterior doors | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Interior doors | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Garage doors | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Windows | 0.129 | 0.168 |  | Calculated from step 1 and 2 |
| Moulding | 0.301 | 0.391 |  | Calculated from step 1 and 2 |
| Shelving | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Cabinets | 0.000 | 0.000 |  | Calculated from step 1 and 2 |
| Countertops | 0.000 | 0.000 |  | Calculated from step 1 and 2 |

Table A2. Results from Sensitivity Analysis

File name: Additional file 1

File type: Excel Worksheet (.xlsx)

Title: Results from Sensitivity Analysis

Description: Contains sums of discounted (3%) total storage, bio emissions, fossil fuel emissions, total emissions, net storage, and net emissions over 120 years for all sensitivity tests; parameters for all tests; percent change in carbon value as a result of the test; difference () in the carbon value as a result of the test; parameters ranked by sensitivity.