Supplementary information

Systematic study of the variability in the gasoline sampling process at service stations and its contribution to the measurement variance

Jerson González-Hernándeza,b and Paola Fuentes-Schweizera,b

a Centro de Investigación en Electroquímica y Energía Química (CELEQ), Universidad de Costa Rica, San José, 11501-2060, Costa Rica.

b Escuela de Química, Universidad de Costa Rica, San José, 11501-2060, Costa Rica.

Contacts: jerson.gonzalez@ucr.ac.cr and paola.fuentes@ucr.ac.cr

Table S1 Results of the analysis of variance applied to the 2-level factorial design matrix for three response variables with a significance level of α = 0.05.

| Factor | p-value for vapor pressure | p-value for initial boiling point | p-value for final boiling point |
| --- | --- | --- | --- |
| A | 0.8686 | 0.0579 | 0.9485 |
| B | 0.0103* | 0.9286 | 0.5957 |
| C | 0.277 | 0.4848 | 0.9143 |
| D | 0.0073* | 0.1806 | 0.2532 |
| A:B | 0.1137 | 0.9642 | 0.6849 |
| A:C | 0.1137 | 0.2300 | 0.7161 |
| A:D | 0.6238 | 0.1505 | 0.2391 |
| B:C | 0.6238 | 0.2592 | 0.9828 |
| B:D | 0.4239 | 0.7216 | 0.4639 |
| C:D | 0.8686 | 0.5386 | 0.8465 |

*Statistical significance. A = Sample transportation time, B = Material of the sample container, C = Temperature of the sample during transportation and D = Storage time of the sample in the lab.

Table S2 Linear regression equations describing the relationship between the response varables and the terms in the model.

| Response factor | Regression equation |
| --- | --- |
| Vapor pressure (VP, kPa) | VP=63.63+0.01∙A+0.14∙B-0.04∙C- 0.16∙D+0.07∙A∙B-0.07∙A∙C+0.02∙A∙D+0.02∙B∙C+0.03∙B∙D-0.01∙C∙D |
| Initial boiling point (IBP, ºC) | IBP=33.59-0.65∙A-0.03∙B-0.20∙C+0.41∙D+0.01∙A∙B+0.36∙A∙C+0.45∙A∙D-0.34∙B∙C+0.10∙B∙D+ 0.18∙C∙D |
| Final boiling point (FBP, ºC) | FBP=192.67+0.02∙A-0.16∙B+0.03∙C-0.36∙D+0.12∙A∙B+0.11∙A∙C+0.37∙A∙D+0.01∙B∙C+0.22∙B∙D+0.06∙C∙D |

Fig. S1 Box-plot comparing the initial boiling point as a response variable for (a) sample transportation time (b) sample container material (c) sample temperature and (d) storage time in the laboratory. The asterisk represents the mean value of the data.

Fig. S2 Box-plot comparing the final boiling point as a response variable for (a) sample transportation time (b) sample container material (c) sample temperature and (d) storage time in the laboratory. The asterisk represents the mean value of the data.

Fig. S3 Main effects plot for vapor pressure (kPa) as a response variable. For variable B 1 represents the plastic container and 2 corresponds to the glass container. For D variable 1 is 2 h and 2 represents 72 h.