Research Highlights

- Samples from Binary Mg-xZn (x=1, 3, 5) alloys were produced using the spark plasma sintering process at different temperatures, times, and pressures.

- Modeling the SPS process to obtain the optimal input parameters was carried out using the Taguchi method.

- Microstructural studies using optical/scanning electron microscopes showed a single-phase structure of α-Mg in Mg-1Zn. At the same time, the Mg-3Zn and Mg-5Zn alloys had intermetallic phase of MgZn in a matrix of α-Mg phase.

- By adding 1-5 wt. % Zn reinforcement, the grain size initially dropped (up to 3 wt. %) and then ascended by merging the grains to form larger ones due to the SPS pressure and enhanced diffusion kinetics.

- Increasing Zn content from 1 to 3 wt. % reduced the hardness and increased the grain size and density of samples. The enhanced hardness of Mg-5Zn compared to Mg-3Zn is mainly due to the smaller grain size.