# Title: Towards the use of a smartphone imaging-based tool for point-of-care detection of asymptomatic low-density malaria parasitemia

Ashlee J. Colbert1, Katrina Co2, Giselle Lima-Cooper2, Dong Hoon Lee3, Katherine N. Clayton4, Steven T. Wereley3, Chandy C. John2, Jacqueline C. Linnes1*, and Tamara L. Kinzer-Ursem2*

1Weldon School of Biomedical Engineering, Purdue University, West Lafayette, IN, 47907, USA,

E-mail: jlinnes@purdue.edu ; tursem@purdue.edu

2Indiana University School of Medicine, Indiana University, Indianapolis, IN, 46202, USA

3School of Mechanical Engineering, Purdue University, West Lafayette, IN, 47907, USA

4OmniVis Inc., Indianapolis, IN, 46201, USA

# Keywords: malaria, particle-diffusometry, nucleic-acid based tests, smartphone-detection, LAMP-assay

Figure S1. Real-time fluorescence and PD-LAMP measurements for each individual repeat. (A) Real-time fluorescence curves for DNA amplification of varying concentrations for repeat 1. (B) PD measurements performed on smartphone for same samples in repeat 1 with significant difference from control for 3x104, 3x103, 3x102, and 3x100 (**p<0.01). (C) PD measurements performed on microscope for all samples in repeat 2 with significant differences from control for 3x104(****p<0.0001), 3x103 and 3x102 (***p<0.001). (D) Real-time fluorescence curves for DNA amplification of varying concentrations for repeat 2. (E) PD measurements performed on smartphone for same samples in repeat 2 with significant difference from control for 3x101(*p<0.05). (F) PD measurements performed on microscope for all samples in repeat 2 with significant differences from control for 3x104, 3x103 (***p<0.001), 3x102, 3x101, and 3x100  (**p<0.01). (G) Real-time fluorescence curves for DNA amplification of varying concentrations for repeat 3. (H) PD measurements performed on smartphone for same samples in repeat 3 with significant difference from control for 3x104 (***p<0.001), 3x103, 3x102, and 3x101 **(p<0.01). (I) PD measurements performed on microscope for all samples in repeat 3 with significant differences from control for 3x104, 3x103, 3x102, 3x101 (****p<0.0001), and 3x100  (**p<0.01). (J) Real-time fluorescence curves for DNA amplification of varying concentrations for repeat 4. (K) PD measurements performed on smartphone for same samples in repeat 4 with significant difference from control for 3x104, 3x103, 3x102 (***p<0.001), and 3x101 *(p<0.05). (L) PD measurements performed on microscope for all samples in repeat 4 with significant differences from control for 3x104(**p<0.01), 3x103(****p<0.0001), 3x101 and 3x100  (*p<0.05).

Figure S2. Positive (+) (3x104) and negative (-) samples (molecular biology water) at blood concentrations of 0-10% (v/v) show a significant difference from controls at each concentration (**p<0.01, ****p<0.0001) using a t-test (A). The 2% agarose gel confirms that there was no amplification in 15% blood (no banding pattern) and no negatives amplified (N=3) (B).

Figure S3. Specificity of malaria PD-LAMP in 0, 5 and 10% blood (v/v). (A) PD diffusion coefficients at each concentration were significantly different from chikungunya (CHIKV) and dengue virus Type 3 (DENV3) controls (***p<0.0001). (B) 2% agarose gel confirmation of amplification for only positive reactions identified by characteristic LAMP DNA banding. All DNA concentrations used were 3x104 DNA copies/µL. Malaria DNA used here was P. falciparum.

Figure S4. Specificity of malaria PD-LAMP for 18s rRNA in 10% blood. (A) PD Diffusivity coefficients at each concentration significantly different from each control (***p<0.0001). (B) Representative agarose gel to confirm presence of banding only in malaria samples. (N=4)

Figure S5. PD-LAMP performed on patient malaria samples. (A) 2% agarose gel confirmation of amplification in all patient samples using 18s rRNA primers. (B) Diffusion coefficients of patient samples with varying parasite densities after a 45-minute reaction (18s rRNA). PD analysis shows all patient samples were significantly different from the NTC (****p<0.0001). (C) A representative 2% agarose gel after a 90-minute LAMP reaction (28s rRNA) indicated amplification in all samples except patient ID 4 and faint amplification in patient ID 3. (D) Diffusion coefficients of patient samples with varying parasite densities after a 90-minute reaction (28s rRNA).  PD analysis showed a statistical significance from NTC for patient IDs 1, 5 (****p<0.0001), 2 (***p<0.001), and 6 (**p<0.01). N=3.

Figure S6. Dot Plot of patient samples from blinded study. Diffusion coefficients of patient samples with varying parasite densities using the 28s primer set. PD analysis showed a statistical significance from NTC for patient IDs 1, 5 (****p<0.0001), 2 (***p<0.001), and 6 (**p<0.01). N=3.(B) Diffusion coefficients averaged form each run (N=3) of patient samples using the18s primer set. PD analysis shows all patient samples were significantly different from the NTC (****p<0.0001).

Table S1. Nucleotide Sequences for LAMP Primers Targeting 28srRNA (Bio = biotin)

| Primer | Sequences (5’-3’) |
| --- | --- |
| B3 | AACACAAACCCCACGATT |
| F3 | GAGATCCCATAAAAGGTGTTG |
| BIP | CGCTAAAGCGAATTACCGATACCCTACTCATAAAAAGATCTGAGGTA |
| FIP | GGTGAGTTGTTACACACTCCTAAGGTTCATAATGACAGTAGGACG |
| Loop B | AGGCCATAAAAAGGTAGG |
| Loop F | Bio-ATTTCGACTTCCATGACC |

Table S2. Nucleotide Sequences for LAMP Primers Targeting 18srRNA (Bio = biotin)

| Primer | Sequences (5’-3’) |
| --- | --- |
| B3 | AGTCGGCATAGTTTATGGT |
| F3 | GCTTAGTTACGATTAATAGGAGTA |
| BIP | GCGAAAGCATTTGCCTAATCTATTTAAGATTACGACGGTATCTGATC |
| FIP | AGTAGTCCGTCTCCAGAAAATCTTACTTTGGGGGCATTCGTATT |
| Loop B | GTTAAGGGAGTGAAGACG |
| Loop F | Bio-TCACCTCTGACATCTG |

Table S3. LAMP Master Mix Used for Amplification of Malaria DNA

| Reagent | Concentration |
| --- | --- |
| Sample | 2.5 µL |
| Isothermal amplification buffer II (10X) | 1X |
| Betaine (5M) | 400 mM |
| dNTPs (100 mM) | 2 mM |
| F3 primer (100 µM) | 0.2 µM |
| B3 primer (100 µM) | 0.2 µM |
| FIP primer (100 µM) | 2 µM |
| BIP primer (100 µM) | 2 µM |
| LF primer (100 µM) | 2 µM |
| LB primer (100 µM) | 2 µM |
| EvaGreen (20X) | 0.2X |
| ROX reference Dye (1/10 =50X) | 1X |
| Bst Polymerase 3.0 (8U/ul) | 8 U |
| Nuclease-free H2O | Fill to 25 µL |

Table S4. Oligonucleotide Sequences and qPCR Conditions for varATS Assays

| Oligonucleotide sequences | varATS |
| --- | --- |
| Primer-For (5’-3’) | CCC ATA CAC AAC CAA YTG GA |
| Primer-Rev (5’-3’) | TTC GCA CAT ATC TCT ATG TCT ATC |
| qPCR reaction conditions | Concentration |
| PowerUp™ SYBR™ Green Master Mix | 1x |
| Primer, 5 µM (each For & Rev) | Concentration 0.37µM |
| DNA | 2 µL |
| Total volume 20 µL | Total volume 20 µL |
| qPCR cycling conditions | Time/Temperature |
| Pre-incubation | 2 min – 50°C |
| Initial denaturation | 2 min – 95°C |
| Denaturation | 15 sec – 95 °C |
| Annealing | 15 sec – 56°C |
| Elongation | 1 min – 72°C |
| Number of cycles | 40 |
| Melt Curve | 95-60°C, 1.6°C decrement, 60-95°C, 0.05°C increment |

Table S5. Asymptomatic Individual Sample Groups and Amplification Results

| Study ID | Group | Microscopy (parasites/µL) | nPCR (dWB) | qPCR (parasite/ µL, WB) | PD-LAMP Significance (N=4) |
| --- | --- | --- | --- | --- | --- |
| 7 | aCC | 0 | 0 | 0 | ns |
| 8 | CC | 0 | 0 | 1 | * |
| 9 | CC | 0 | 0 | 0 | ns |
| 10 | CC | 0 | 1 | 0 | ns |
| 11 | CC | 0 | 1 | 0 | ns |
| 12 | CC | 0 | 1 | 1 | ns |
| 13 | CC | 0 | 1 | 56 | ** |
| 14 | CC | 0 | 1 | 35 | * |
| 15 | CC | 84 | 1 | 12,749 | *** |
| 16 | CC | 612 | 1 | 8.741 | *** |
| 17 | CC | 43,376 | 1 | 100,669 | ** |
| 18 | bCM | 134,902 | not done | 377,406 | * |
| 19 | cSMA | 12,458 | not done | 136,926 | *** |

aCommunity Control; b Cerebral Malaria; cSevere Malaria Anemia; dWhole Blood

nPCR: 0 = negative for P. falciparum, 1 = positive for P. falciparum