Table 1

Parameters of CLT wall specimens

| Specimen type | Total thickness /mm | Layer number | CLT thickness /mm | Half-lap joint | Stud types and construction method | Deadening felt | Gypsum board | Damping coating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W1 | 105 | 3 | 105 | — | — | — | — | — |
| W2 | 175 | 5 | 175 |  |  |  |  |  |
| W3 | 105 | 3 | 105 | Yes |  |  |  |  |
| W4 | 117 |  |  |  |  |  | Yes |  |
| W5 | 155 |  |  |  | Light wood frame filled with glass wool |  |  |  |
| W6 | 193 |  |  |  | Staggered wood stud frame filled with glass wool |  |  |  |
| W7 | 160 |  |  |  | Light wood frame filled with glass wool |  |  | Yes |
| W8 | 175 |  |  |  |  | Yes |  |  |
| W9 | 200 |  |  |  | Light steel frame filled with glass wool |  |  |  |

Table 2

Airborne impact sound insulation classes as specified in GB/T 50121

| Class | Range |
| --- | --- |
| 1 level | 20 dB ≤ Rw + Cj < 25 dB |
| 2 level | 25 dB ≤ Rw + Cj < 30 dB |
| 3 level | 30 dB ≤ Rw + Cj < 35 dB |
| 4 level | 35 dB ≤ Rw + Cj < 40 dB |
| 5 level | 40 dB ≤ Rw + Cj < 45 dB |
| 6 level | 45 dB ≤ Rw + Cj < 50 dB |
| 7 level | 50 dB ≤ Rw + Cj < 55 dB |
| 8 level | 55 dB ≤ Rw + Cj < 60 dB |
| 9 level | Rw + Cj ≥ 60 dB |

Note: Internal separation components, Cj=C; enclosure components, Cj=Ctr.

Table 3

Weighted sound reduction index (Rw) evaluation

| Specimen | Test Rw 100-5000 Hz (C; Ctr) (dB) | Sound insulation class |  | Theoretical Rw (dB) | Error (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | Internal separation component | Enclosure component |  |  |
| W1 | 33 (-1; -3) | 3 (32 dB) | 3 (30 dB) | 32 | 1 |
| W2 | 37 (-1; -4) | 4 (36 dB) | 3 (33 dB) | 35 | 2 |
| W3 | 33 (-1; -3) | 3 (32 dB) | 3 (30 dB) | 32 | 1 |
| W4 | 37 (-1; -4) | 4 (36 dB) | 3 (33 dB) | 35 | 2 |
| W5 | 40 (-2;- 8) | 4 (38 dB) | 3 (32 dB) | 40 | 0 |
| W6 | 42 (-2; -6) | 5 (40 dB) | 4 (36 dB) | 45 | -3 |
| W7 | 42 (-2; -6) | 5 (40 dB) | 4 (36 dB) | 44 | -2 |
| W8 | 43 (-1; -4) | 5 (42 dB) | 4 (39 dB) | 44 | -1 |
| W9 | 47 (-2; -6) | 6 (45 dB) | 5 (41 dB) | 46 | 1 |