Supporting Information

A signal-on electrochemical aptasensor for methylamphetamine rapid detection based on competition with complementary DNA

Wenzhuo Changa,b, Zhixiang Zhenga*,Yongjun Mab

a Key Laboratory of Evidence Science Techniques Research and Application of Gansu Province, Gansu University of Political Science and Law, Lanzhou 730070, China

b College of Chemistry and Chemical Engineering, Northwest Normal University, Lanzhou 730070, China

* Corresponding author

Email: dqzhenghao@163.com;

1. Electrochemically cleaning gold electrode

The gold electrode was electrochemically cleaned in 0.5 M H2SO4 aqueous solution (3 mL) by an oxidation step (2 V for 5 s) and then a reduction step (-0.35 V for 10 s), before cyclic voltammetry (CV) in the range of -0.35 V ~ 1.55 V (20 scans at a scan rate of 4 V/s, followed by 10 scans at a scan rate of 0.1 V/s). After that, a second electrochemical cleaning was performed in a solution of 0.1 M sulfuric acid containing 0.01 M potassium chloride. In details, a set of CV scans were performed in four different potential ranges: (a) potential range from 0.2 V ~ 0.75 V; (b) potential range from 0.2 V ~ 1 V; (c) potential range from 0.2V ~ 1.25 V; (d) potential range from 0.2 ~ 1.5 V. 10 segments for each potential range, at a scan rate of 0.1 V/s. Finally, the gold electrode was rinsed with ultrapure water and blow-dried by nitrogen, and it was ready for modification.