Additional file 1

Study design

BSL, baseline; MRI, magnetic resonance imaging; R, randomisation; s.c., subcutaneous

Additional file 2

MRI sequence parameters

|  | Parameter Ranges | T1 (Sagittal and Transversal) | STIR (Sagittal and Transversal) |
| --- | --- | --- | --- |
| Mandatory settings | Sequence type | TSE/FSE | STIRa |
|  | Dimension | 2D | 2D |
|  | Repetition time - TR (ms) | 400-600 | 2800-3500 |
|  | Echo time - TE (ms) | Maximum 25 | 60-100 |
|  | Inversion time - TI (ms) | N/A | 140-170 |
|  | Slice thickness (mm) | 3 | 3-4 |
|  | Distance factor/gap (mm) | Maximum 10% | Maximum 10% |
|  | In-plane resolution better than (mm x mm) | 0.6 x 0.6 | 0.7 x 0.7 |
|  | Specific settings | - Interpolation off - Do not use parallel acquisition - Adapt the number of slices (approximately 20) so as to cover the complete bone structure by the slice stack |  |

aTurbo Inversion Recovery, Fast Inversion Recovery, Turbo Inversion Recovery Magnitude or equivalent sequence may be used.

FSE, fast spin-echo; MRI, magnetic resonance imaging; N/A, not applicable; STIR, short inversion time inversion-recovery; TSE, turbo spin-echo