# Appendix

Table A1

| Economic indicator | Calculation | Description | Ref. |
| --- | --- | --- | --- |
| Total Life Cycle Cost (TLCC) | TLCC= INVcst+ n=1tO&Mcst1+αn | The economic period of the project is considered to be 20 y and the nominal discount rate α is 10%. Here, O&Mcst is represented as operation & maintenance costs. | [76] |
| Tax paid on profit (Tpaid) | Tpaid=Projectprofit×Tr | The profit gained from the project is represented as Projectprofit and marginal tax rate, Tr as 15%. | [76], [77] |
| Revenue (Projectrev) | Projectrev=ProjectEP×rfit | The feed-in tariff (rfit) of biogas-based electricity generation for Bangladesh is considered as 0.106 $/kWh as agreed for PV. | [76], [78] |
| Profit (Projectprofit) | Projectprofit=Projectrev-O&Mcst | The revenue earned from the project is represented as Projectprofit | [76] |
| Net Cash Flow (Ln) | Ln=Projectrev-O&Mcst-INVcst-Tpaid | The tax paid on the profit of the project is represented as Tpaid. | [76] |
| Investment Payback Period (IPBP) | IPBP=TLCC $Projectprofit $t | The investment payback period of the project is represented as IPBP. | [76] |
| Internal Rate of Return (IRR) | IRR=0=n=0tLn1+IRRn | The Internal Rate of Return of the project is represented as IRR, the holding period of the project (20 yr) by t and each period represents by n. | [76] |
| Carbon-di-oxide (CO2) reduction potential | ΔMCO2=Mdiesel-Msolar-pyro oil =(2.7 × Qdiesel) – (Epyrolytic oil) | The reduction of CO2 emission is presented by ΔMCO2 is the difference between emission from diesel operated pump and pyrolytic oil plant. Qdiesel and Epyrolytic oil are the equivalent diesel quantity and emission from pyrolytic plant. | [79] |