List of Tables

Table 1 Specifications of the Mowon Reservoir

Table 2 Comparison of a stair and trees reproduced from different photography angles

Table 3 Results of the preliminary test for the selection of the optimal aerial photography condition

Table 4 The relative coordinate accuracy of the point cloud model of the target site

Table 5 Status of the weather stations in the Mowon Reservoir watershed

Table 6 Hydrograph data of each watershed for the target area

Table 7 Setup of the two-dimensional hydraulic modelling for the bridge

Table 8 The computer resources used for flood wave analysis in this study

Table 9 Comparison of the peak runoff arrival times for each bridge

Table 10 Comparison of the water depth, peak runoff, and flow velocity at Bridge 1

Table 11 Comparison of the estimated flood wave propagation area

Table 1 Specifications of the Mowon Reservoir

| Basin | Embankment |  |  |  | Spillway |  |
| --- | --- | --- | --- | --- | --- | --- |
| Watershed area (ha) | Length (m) | Height (m) | Upstream slope (V:H) | Downstream slope (V:H) | Length (m) | Height (m) |
| 79 | 127.0 | 5.6 | 1.0:3.5 | 1.0:1.7 | 14.0 | 2.0 |

Table 2 Comparison of a stair and trees reproduced from different photography angles

| Case | Photography angles |  |  |  |
| --- | --- | --- | --- | --- |
|  | 45° | 55° | 65° | 75° |
| Staircase |  |  |  |  |
| Tree B |  |  |  |  |
| Tree C |  |  |  |  |
| Tree D |  |  |  |  |

Table 3 Results of the preliminary test for the selection of the optimal aerial photography condition

| Case | Condition | Images | Battery | Reproducibility* |
| --- | --- | --- | --- | --- |
| Overlap (%, based on 45˚ photography angle) | front 70%, side 63% | 11 | 8% | staircase ×, trunks × |
|  | front 80%, side 72% | 18 | 11% | staircase ▽, trunks △ |
|  | front 90%, side 81% | 41 | 18% | staircase ○, trunks ○ |
| Photography angle (˚, based on front 80%, side 72% overlap) | 45 | 22 | 12% | staircase ▽, trunks △ |
|  | 55 | 22 | 12% | staircase ○, trunks ○ |
|  | 65 | 22 | 12% | staircase ○, trunks ▽ |
|  | 75 | 22 | 11% | staircase ○, trunks ▽ |
| * Reproducibility rate: ○ = upper 90%, △ = 70 ~ 90%, ▽ = 50 ~ 70%, × = under 50% |  |  |  |  |

Table 4 The relative coordinate accuracy of the point cloud model of the target site

| Scale (m) | X (%) | Y (%) | Z (%) |
| --- | --- | --- | --- |
| 1.0 | 98.42 | 99.59 | 99.81 |
| 2.0 | 100.00 | 100.00 | 100.00 |
| 3.0 | 100.00 | 100.00 | 100.00 |

Table 5 Status of the weather stations in the Mowon Reservoir watershed

| Name | Observatory code | Latitude |  | Longitude | Elevation (EL.m) |  | Observer type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Jangdong | 40074082 | 34°46’43” | 127°00’34” |  |  | 200 | Telemeter |
| Jangheung | 51011260 | 34°41’19” | 126°55’10” |  |  | 44 | Telemeter |

Table 6 Hydrograph data of each watershed for the target area

| Data | Watershed A | Watershed B | Watershed C | Watershed D |
| --- | --- | --- | --- | --- |
| Watershed area (m2) | 790,000 | 924,000 | 188,000 | 115,000 |
| Flow path (km) | 1.570 | 2.158 | 1.110 | 0.861 |
| Curve number (CNⅢ) | 86 | 86 | 88 | 92 |
| Time of concentration (TC, h) | 0.138 | 0.183 | 0.108 | 0.105 |
| Storage coefficient (K, h) | 0.162 | 0.213 | 0.125 | 0.147 |
| Peak runoff (m³/s) | 12.0 | 13.9 | 2.9 | 1.8 |

Table 7 Setup of the two-dimensional hydraulic modelling for the bridge

| Modelling for the bridge flow | Beams upper flow |  |  | Piers |
| --- | --- | --- | --- | --- |
|  | 2D numerical analysis (DWE or SWE) |  |  | Culvert option |
|  | Shape |  |  | Material |
|  | Rectangular |  |  | Concrete |
|  | Entrance loss coefficient | Exit loss coefficient | Manning’s n |  |
|  | 0.5 | 1.0 | 0.011 |  |
| The flow under the bridge calculation method (Brunner, 2016) | Inlet control headwater | Unsubmerged | Eihi=Hchi+KQbabhi0.5M-0.5sb |  |
|  |  | Submerged | Eihi=cQbabhi0.52+Y-0.5sb |  |
|  | Outlet control headwater | Z3+Y3+a3V322g=Z2+Y2+a2V222g+HL |  |  |
| where Ei is headwater energy depth above the invert of the bridge inlet (m); hi is interior height of the bridge barrel (m); Qb is the discharge through the bridge (cm); Hc is the specific head at critical depth (m); ab is the full cross-sectional area of the bridge barrel (m2); sb is the bridge barrel slope (m/m); K, M, c, Y are the equation constants, which vary depending on the bridge shape and entrance conditions; Z3 is the upstream invert elevation of the bridge; Y3 is depth of water above the upstream bridge inlet; V3 is the average velocity upstream of the bridge; a3 is the velocity weighting coefficient upstream of the bridge; Z2 is the downstream invert elevation of the bridge; Y2 is the depth of water above the downstream bridge inlet; V2 is the average velocity downstream of the bridge; a2 is the velocity weighting coefficient downstream of the bridge; and HL is the total energy loss through the bridge. |  |  |  |  |

Table 8 The computer resources used for flood wave analysis in this study

| Resources | Name |
| --- | --- |
| CPU | Intel® Core™ i7-8700 @ 3.20 GHz |
| RAM | SAMSUNG DDR4 PC-19200 32GB |
| Graphic | NVIDIA GeForce GTX 1060 3GB |
| Storage | Western Digital WD Black 3D M.2 2280 |
| Operating system | Windows 10 pro x64 |

Table 9 Comparison of the peak runoff arrival times for each bridge

| Case | Bridge 1 (s) | Bridge 2 (s) | Bridge 3 (s) |
| --- | --- | --- | --- |
| DWE | 49 | 138 | 168 |
| SWE | 52 | 181 | 227 |

Table 10 Comparison of the water depth, peak runoff, and flow velocity at Bridge 1

| Case | Water depth (m) | Peak flow (beams upper flow) (m3/s) | Velocity (m/s) |
| --- | --- | --- | --- |
| DWE | 2.70 | 42.34 (0.00) | 2.36 |
| SWE | 2.96 | 35.90 (1.34) | 4.29 |

Table 11 Comparison of the estimated flood wave propagation area

| Case | Actual flood wave propagation area (A, m²) | Simulation results (B, m²) | Difference of A and B (B–A, m²) | Overlapping area (A∩B, m²) | Estimation accuracy ((A∩B/A)×100, %) | Total analysis time (hour:min) |
| --- | --- | --- | --- | --- | --- | --- |
| DWE | 19,422.006 | 18,135.371 | –1,286.634 | 9,487.657 | 48.9 | 52:06 |
| SWE |  | 25,501.029 | 6,079.024 | 13,074.712 | 67.3 | 63:21 |
| 3D method |  | 28,387.809 | 8,965.804 | 16,383.704 | 84.4 | 174:32 |