# Supplementary Material

Study of the Preparation and Quality of Gastrodia elata with Different Drying Processes

Nannan Wanga,b, Xindi Jina,c, Yongxiang Lia,b, Tong Zhaoa,b, Yangyang Lua,b, Jiale Changa,d, Bin Liua,b, Zhenpeng Gaoa,b, Xubo Zhaoa,b,*

Author address

aCollege of Food Science and Engineering, Northwest A&F University, No.22 Xinong Road, Yangling, Shaanxi 712100, P. R. China

bLaboratory of Quality & Safety Risk Assessment for Agro-products (Yangling), Ministry of Agriculture and Rural Affairs, Yangling, Shaanxi 712100, P. R. China

cShaanxi Province Grain and Oil Science Research Institute, Xi’an, Shaanxi 710000, P. R. China

dYangling Digital Agricultural Technology Co., Ltd., Xi’an, Shaanxi 710000, P. R. China

*Corresponding author

### E-mail: zhxubo@nwafu.edu.cn     Tel: +86 13636717902

Materials and methods

### Raw material

The ripe and undamaged fruits of G. elata were collected from Ningshan County, the main production area of Shaanxi Province. These fruits were placed in low-temperature boxes and brought back to the laboratory, where they were kept in a controlled atmosphere at 0-4℃.

Gastrodin, p-hydroxybenzyl alcohol, vanillin, vanillyl alcohol, parishin A, parishin B, parishin C, and parishin E standard products were purchased from Shanghai Yuanye Biotechnology Co., Ltd. ACE enzyme and T-AOC test kits were purchased from Shanghai Solaybao Biotechnology Co., Ltd.

### Sample processing and drying procedures

The fresh G. elata samples of uniform size were washed, the scaly and rough skin was removed, cut into approximately 2 mm thickness and divided into equal portions for further use. One portion of G. elata was placed on a steamer and steamed until the center turned white, which was the steaming method for preparing thin slices of the G. elata. Unprocessed and steamed G. elata were dried by the four methods to meet the requirement of 2020 Chinese Pharmacopoeia that the moisture content is less than 12%. (i) For NAD, the samples were laid out evenly in an area with adequate ventilation and allowed to dry naturally. (ii) For HAD, the samples were placed evenly on the shelf of a blast drying oven (DHG-9000A, Jiangsu Tianling Instrument Co., Ltd, China), with the temperature set at 60℃. (iii) For CFD, the samples were pre-cooled at -80℃ for 12 h before being evenly placed on a shelf in a freeze drying oven (LGJ-25C, The Beijing Fourth Ring Instrument Factory, China) with a cold trap temperature of 90℃. (iv) For VMD, the samples were placed in the chamber of a vacuum microwave dryer (VYS-4ZH1/2V, Shanghai Wanmuchun Biological Engineering Co., Ltd, China), and the drying process was divided into two stages. The first stage had a power of 1000 W applied for 10 min, while the second stage had a power of 600 W applied for 5 min. In addition, the temperature was set at 60±5℃.

### Determination of moisture content

The standard mode was set and the temperature was adjusted to 105℃ using an infrared moisture analyzer (MA160-1CN, Beijing Sedolis Scientific Instrument Co., Ltd, China), and each sample was measured three times in parallel.

### Measurement of hardness

The hardness of G. elata samples was determined using a physical property tester (TA.XT PLUS/50, STABLEMICVO, UK), and the probe was selected as P5. The diameter was 6 mm. The descent and test speeds were both 1 mm/s and the recovery speed was 10 mm/s. The trigger start value was set to 5 g.

### Microstructure characterization

After pre-treatment and drying, the micromorphology of G. elata was observed by scanning electron microscope (Nano SEM-450, FEI, USA). The conductive film was applied to the sample seat, followed by the even sprinkling of a small amount of G. elata powder. The non-adhered powder was blown away with an ear ball to ensure successful observation.

### Determination of eight functional components

The G. elata samples were prepped according to the guidelines of the Chinese Pharmacopoeia with additional refinements. Briefly, 2 g of crushed dried G. elata tablets were precisely combined with 50 mL of dilute ethanol in a conical flask. After weighing, the mixture was subjected to ultrasonic treatment for 30 min at a power of 120 W and a frequency of 40 kHz. After cooling, the samples were reweighted and the loss of weight was made up with dilute ethanol. Then, 10 mL of the filtrate was filtered and accurately measured. The solution was then concentrated until it was almost dry and odorless, and the residue was dissolved in a mixture of acetonitrile/water (3:97, v:v). Finally, the solution was diluted to 25 mL, mixed thoroughly, filtered, and the filtrate collected.

The contents of gastrodin, p-hydroxybenzyl alcohol, vanillyl alcohol, vanillin, parishin A, parishin B, parishin C, and parishin E were determined by high-performance liquid chromatography (HPLC). HPLC analysis was performed on a Shimadzu LC-20AD instrument (Japan). An Agilent C18 column (250 mm × 4.6 mm, 4.5 μm) was selected for sample separation. The injection volume was 10 μL and the flow rate was 1 mL/min. The mobile phase consisted of acetonitrile (A) and 0.05% phosphoric acid solution (B) with a gradient elution as follows: 0-10 min, 10% B, 10-30 min, 30% B. The detection wavelength and column temperature were set at 220 nm and 35℃, respectively.

### Determination of Gastrodia polysaccharide

The determination of G. elata polysaccharide was performed according to the method of DB22 / T 1685-2012. 2.00 g of G. elata was extracted with distilled water by ultrasound for 30 min. The mixture was then extracted in a boiling water bath for 4 h, cooled to room temperature and made up to 100 mL with distilled water. 5 mL of the extract was mixed with 15 mL of ethanol solution. The mixture was centrifuged at 10000 ×g for 10 min and the supernatant was discarded. Then 5 mL of ethanol solution was added, mixed and centrifuged, repeated twice, and the supernatant was discarded. The residue was dissolved with 100 mL of water. Finally, 2 mL of this solution and 1 mL of phenol solution (5%, w/v) were added and shaken well. Then 5 mL of sulfuric acid was quickly added and shaken for 5 min. The mixture was allowed to stand for 10 min before being heated in a boiling water bath for 20 min. After removal from the water bath, the solution was cooled to room temperature before measuring the absorbance at 486 nm. The calibration curve of the polysaccharide (y = 3.9567x + 0.39) ranged from 100 to 800 μg/g (R2 = 0.9936).

### Determination of total phenolic and total flavonoid content

G. elata samples were treated according to the method established by M. Rojas-Bravo et al. [1] with minor modifications. 3.00 g of ground G. elata powder was ultrasonically extracted in 20 mL of 80% ethanol for 30 min. After standing for 2 h, the extract was centrifuged at 10000 ×g for 20 min and the process was repeated twice to obtain the extraction solution.

Total phenolic content was determined using the Folin–Ciocalteu colorimetric method [2]. Briefly, 1 mL of the extraction solution was mixed with 5 mL of deionized water, 0.5 mL of Folin-Ciocalteu reagent, and 1 mL of Na2CO3 solution (7%, w/v). After reaction for 30 min in the dark at room temperature, the absorbance was measured at 765 nm using an enzyme calibration (HH35000922, Tecan Austria GmbH, Austria). The calibration curve of gallic acid (y = 0.9007x + 0.0593) ranged from 50 to 300 μg/mL (R2 = 0.9937), and the results were expressed as milligrams of gallic acid equivalents (GAE) per gram of dry weight (mg GAE/g DW).

Total flavonoid content (TFC) was quantified by the aluminium trichloride colorimetric method [3]. Briefly, 1 mL of extraction solution was added to 5 mL of deionized water and 0.3 mL of NaNO2 solution (5%, w/v). The mixture was vortexed and 0.3 mL of AlCl3 solution (10%, w/v) was added. After reaction for 5 min at room temperature, 2 mL of NaOH (0.5 mol/L) was added. After sufficient reaction, the absorbance was measured at 510 nm. The calibration curve of rutin (y = 0.5034x + 0.0426) ranged from 100 to 300 μg/mL (R2 = 0.9971). The results were expressed as mg rutin equivalents (RE)/g dry weight (mg RE/g DW).

### Antioxidant activity

The sample pre-treatment was the same as total phenolic.

Total antioxidant capacity

It was determined using the Total Antioxidant Capacity (T-AOC) test kit.

DPPH free radical (DPPH•) scavenging assay

1 mL of G. elata extract dilutions and 0.5 mL of DPPH solution were mixed well and allowed to react for 30 min in the dark. The absorbance was then determined at 517 nm. Samples were assayed three times in parallel.

ABTS cationic radical (ABTS +•) scavenging assay

The working ABTS solution was diluted with distilled water at room temperature until the absorbance at 734 nm was 0.7±0.02. The 0.1 mL of G. elata extract dilutions and 1.9 mL of diluted ABTS solution were mixed. The mixture was allowed to react for 10 min in the dark before measuring the absorbance at 734 nm. Samples were assayed three times in parallel.

### Antihypertensive activity

ACE enzyme inhibition rate was determined according to the method of Wang et al. [4]. 50 μL of G. elata extract solution and 30 μL of hippuryl-histidine-leucine (HHL) were mixed. After 5 min of water bath at 37℃, 10 μL of ACE enzyme solution was added. The mixture was then placed in a water bath at 37°C for another 30 min, and 100 μL of hydrochloric acid (1 mol/L) was added to stop the reaction. The reaction was filtered through a 0.45 μm filter membrane, and the filtrate was reserved for further use.

Separation was performed on an Agilent C18 column (250 mm × 4.6 mm, 4.5 μm) with a stable flow rate of 1 mL/min at 30℃. The mobile phase consisted of methanol (A) and 0.1% trifluoroacetic acid solution (B) with isocratic elution. The injection volume was 10 μL, and the detection wavelength was 228 nm.

### Statistical analysis

Data analysis and mapping were performed using SPSS 18.0 and Origin 2018, and linear dimensionless analysis of total antioxidant capacity (T-AOC) results was performed using the threshold method, with the threshold set at 1 μmol/mL.

Abbreviations

G. elata: Gastrodia elata

NGB-NAD: unprocessed-natural air drying G. elata

NGB-HAD: unprocessed-hot air drying G. elata

NGB-CFD: unprocessed-vacuum freeze drying G. elata

NGB-VMD: unprocessed-vacuum microwave drying G. elata

PGB-NAD: steam processed-natural air drying G. elata

PGB-HAD: steam processed-hot air drying G. elata

PGB-CFD: steam processed-vacuum freeze drying G. elata

### PGB-VMD: steam processed-vacuum microwave drying G. elata

### References

1. Rojas-Bravo M, Rojas-Zenteno EG, et al. (2019) A potential application of mango (Mangifera indica L. cv Manila) peel powder to increase the total phenolic compounds and antioxidant capacity of edible films and coatings. Food and Bioprocess Technology 12(9):1584-1592. http://doi.org/10.1007/s11947-019-02317-8.

2. Papillo VA, Locatelli M, Travaglia F, Bordiga M, Garino C, Coïsson JD, Arlorio M (2019) Cocoa hulls polyphenols stabilized by microencapsulation as functional ingredient for bakery applications. Food Research International, 115:511-518. http://doi.org/10.1016/j.foodres.2018.10.004.

3. Xu L, Wang P, Ali B, Yang N, Chen Y, Wu F, Xu X (2017) Changes of the phenolic compounds and antioxidant activities in germinated adlay seeds. Journal of the Science of Food and Agriculture 97(12):4227-4234. http://doi.org/10.1002/jsfa.8298.

4. Wang K, Cong J, Gu S, Wang X (2021) Effect of processed foods on ACE activity. Chinese Journal of Food Science, 21(05), 90-95. http://doi.org/10.16429/j.1009-7848.2021.05.011.