Table Captions

Table 1: The main functionalities required in RF Photonic systems with a brief description of each one, their main applications and where the systems implementing them are hosted. The left column also includes a brief list of references corresponding to ASPICs reported for each of them.

Table 1

| Functionality and some reported ASPICs | Short description | Hosting | Application |
| --- | --- | --- | --- |
| Tunable RF- Filtering [5], [11], [33]-[35], [52], [53] | Channel selection in the RF domain & Noise reduction | Base stations, Central offices, planes, satellites, drones, edge data centers | Mobile & satellite comms, IoT, Sensing, Radar, Imaging |
| Tunable Optical Filtering [52] | Superchannel RF selection in the optical domain & Noise reduction |  |  |
| Tunable RF Delay line [11], [49], [50] | Selective RF channel delay and time-delayed signal combination | Radio over fiber links, beamformers | Phased array antenas, Radar, Sensing, Lidar |
| Tunable RF Phase Shifter [11], [49] | Selective RF channel dephasing and phase-delayed signal combination |  |  |
| Frequency measurement [21] | Identification of the frequency of non-desired interfering signals | Radar and antenna infrastructures sensors, satellites, base stations | Radar, navigation systems, self-interference cancellation |
| Optoelectronic Oscillators (OEOs) [11], [36], [37] | Ultrapure tunable RF tone generation based on combined optical/RF energy storagein a mixed optoelectronic cavity. | Clock reference systems, Spectrum analyzers, RF coherent receivers | Avionics and Satellite comms, Navigation systems. Time and frequency measurement |
| Arbitrary Waveform Generation (AWG) [11], [51] | Generation of RF wavepackets with arbitrary amplitude shapes and central frequencies of operation. | Central stations, Radar, satellites, planes, drones, infrastructures, IoT terminals | Radar, Mobile communications, microwave imaging, instrumentation |
| Optical generation of MM-wave CW signals [11], [5] | Baseband and Intermediate frequency conversion to/from the radio spectrum | Central stations, radar antennas, base stations | Mobile, wearable & satellite comms, Radar, Imaging |
| Beamforming [5], [11], [49], [50] | Broadband angular steering of a radiation pattern from/to an antenna array | Base stations, Radar infrastructures | Phased array antennas, 5-6G mobile & satellite comms, Sensing, Radar, Imaging |
| Radio over Fiber Transmission [5] | Broadband communication transceivers between central stations and base stations | Edge Data centers, Base stations, Central stations | Mobile and satellite communications |
| Tunable Beam splitting [1],[2] | Signal distribution among different physical outputs | Central stations, Edge Data Centers, base stations, satelltes, planes | Mobile &satellite communications, |
| Tunable RF interconnection and switching [1], [2] | RF Channel and superchannel connection between selected input/output ports |  |  |