Additional File 4

Transoral instillation

In brief, anaesthetized mice were suspended by the cranial incisors on a thin thread from a plastic slanted intubation stand and fixed in position with a piece of tape. The tongue was gently extracted from the mouth using blunt forceps and held between two fingers to visualize the base of the tongue and the pharynx. Particle suspension or vehicle was placed in the posterior pharynx with a micropipette, and nares were gently pinched with blunt forceps. Respiration was monitored to ensure that the suspension was fully aspirated before the tongue and nares were released. Mice were allowed to recover until fully awake in a separate cage. General health of the mice was assessed by monitoring weight weekly and visual check-ups based on fur condition, posture, and behavior.

In vivo micro-CT scanning

The following parameters were used: 50 kVp X-ray source voltage and 350 µA current, a composite X-ray filter of 1 mm aluminum, 150 ms exposure time per projection, acquiring and averaging three projections per rotation step with 0.9° increments over a total angle of 220°, and 10 cm field of view covering the whole body producing reconstructed data sets with 50 µm isotropic voxel size. Each scan took approximately 3 minutes and is associated with a measured radiation dose of 60-80 mGy. Software provided by the manufacturer (Tsort, Nrecon, DataViewer, and CTan) was used to retrospectively gate, reconstruct, visualize, and process µCT data as described by Vande Velde et al. (1). For Hounsfield unit (HU) calibration, a phantom was scanned consisting of an air-filled 1.5 ml microcentrifuge tube inside a water-filled 50 ml tube. Based on full stack histograms of a volume of interest (VOI) containing only water or air, the mean grayscale index of water [130] was set at 0 HU and grayscale index of air [9] was set at -1000 HU. Quantification of the lung volumes and respective densities was carried out for a VOI covering the lung, manually delineated on the coronal micro-CT images, avoiding the heart and main blood vessels. The threshold used to distinguish aerated from non-aerated lung tissue volume was manually set at -190 HU and kept constant for all scans.

Cytokine and chemokine detection limits

V-PLEX Proinflammatory Panel 1 Mouse Kit

| Analyte | LLOD | LLOQ | ULOQ | Dynamic Range | Unit |
| --- | --- | --- | --- | --- | --- |
| IFN-γ | 0.04 | 0.39 | 570 | 0.04 - 570 | pg/mL |
| IL-1β | 0.11 | 0.72 | 1,030 | 0.11 - 1,030 | pg/mL |
| IL-2 | 0.22 | 1.03 | 1,570 | 0.22 - 1,570 | pg/mL |
| IL-4 | 0.11 | 0.818 | 1,060 | 0.11 - 1,060 | pg/mL |
| IL-5 | 0.06 | 0.302 | 590 | 0.06 - 590 | pg/mL |
| IL-6 | 0.61 | 7.61 | 3,140 | 0.61 - 3,140 | pg/mL |
| IL-10 | 0.94 | 7.26 | 2,030 | 0.94 - 2,030 | pg/mL |
| IL-12p70 | 9.95 | 179 | 20,600 | 9.95 - 20,600 | pg/mL |
| KC/GRO | 0.24 | 3.29 | 1,230 | 0.24 - 1,230 | pg/mL |
| TNF-α | 0.13 | 0.98 | 403 | 0.13 - 403 | pg/mL |

V-PLEX Cytokine Panel 1 Mouse Kit

| Analyte | LLOD | LLOQ | ULOQ | Dynamic Range | Unit |
| --- | --- | --- | --- | --- | --- |
| IL-9 | 3.84 | 21.9 | 2,600 | 3.84 - 2,600 | pg/mL |
| IL-15 | 16 | 43.2 | 26,000 | 16.0 - 26,000 | pg/mL |
| IL-17A/F | 0.231 | 1.39 | 1,620 | 0.231 - 1,620 | pg/mL |
| IL-27p28/IL-30 | 1.39 | 5.91 | 6,500 | 1.39 - 6,500 | pg/mL |
| IL-33 | 0.364 | 1.85 | 1,950 | 0.364 - 1,950 | pg/mL |
| IP-10 | 0.328 | 2.15 | 650 | 0.328 - 650 | pg/mL |
| MCP-1 | 0.672 | 4.42 | 325 | 0.672 - 325 | pg/mL |
| MIP-1α | 0.081 | 0.38 | 390 | 0.081 - 390 | pg/mL |
| MIP-2 | 0.053 | 0.58 | 423 | 0.053 - 423 | pg/mL |

1.	Vande Velde G, Poelmans J, De Langhe E, Hillen A, Vanoirbeek J, Himmelreich U, et al. Longitudinal micro-CT provides biomarkers of lung disease that can be used to assess the effect of therapy in preclinical mouse models, and reveal compensatory changes in lung volume. Dis Model Mech. 2016;9(1):91-8.