APPENDIX-1

Calculation of initial cracking Load of Asymmetrical Bare Frame

The cracking moment capacity of the column section  Mcr=fcr x Z

fcr	=  3.81 N/mm2  	Z=bd26

fcr  -  flexural cracking

Z  - Section Modulus

Z=75x10526=137812.5 mm2

Therefore, the cracking moment of resistance of section is;

Mcr=3.81 x 137812.5 x 10-6

Mcr  = 0.53 kN-m

From the analysis results from SAP 2000 [137] computer program.

Maximum B.M @ A (ground floor column) = 0.045 kN-m for 16 kN Vertical load.

Based on the column capacity the cracking load is Pcr = 160.045 x 0.53 = Pcr = 188kN or 18.8Tons

A4.3.2 	Calculation of ultimate Load of bare frame

The plastic section of a rectangular column section of the doubly reinforced beam;

Taking a moment about the ultimate tension force section;

Mp =25×fck×b × xu×d-0.25d+0.87×fy×Asc×d-d'

Mp	=	plastic moment of the column

fck	=	compressive strength of concrete (30 N/mm2)

b	=	width of the column (= 75 mm)

xu	=	depth of neutral axis (= 0.5 d)

d	=	effective depth of column section (= 105 mm)

fy	=	yield strength of steel (= 415 N/mm2)

Asc	=	area of compression steel  =2×π×624

d'	=	clear cover (24 mm)

Mp=9.07 kNm

The ultimate load capacity of Bare frame;

By choosing initial hinge formation from SAP 2000 [138] computer program.

Pu=15θMphθ

Pu	=	Ultimate load capacity of a column

Mp	=	Plastic moment capacity of a column

h	=	height of storey (= 0.5 m)

Pu= 272.1 kN or 27.2 Tons