Supporting Information

Efficient degradation of bisphenol S by ultraviolet/persulfate oxidation in fresh and saline waters: Effects of operating conditions and reaction mechanism

Guoqiang Liua,b, Kai Wub, Cunliang Zhangd, Yuefei Jib, Junhe Lub, Jing Chenb,c,⁎, Deyang Konga,⁎

a Nanjing Institute of Environmental Sciences, Ministry of Ecology and Environment of China, Nanjing, 210042, China

b Department of Environmental Science and Engineering, Nanjing Agricultural University, Nanjing, 210095, China

c State Key Laboratory of Pollution Control and Resources Reuse, Nanjing 210023, China

d Shandong Provincial Eco-environment Monitoring Center, Jinan 250033, China

# *Corresponding author: E-mail: chenjnj2008@njau.edu.cn

Telephone: +86-25-84395164; Fax: +86-25-84395210

Text 1 Analytic methods

Solid extraction (SPE), CNWBOND LC-C18 SPE cartridges (2.CA0955.0001, 1 g, 6 mL) (CNW, ANPEL Laboratory Technologies (Shanghai), Int., China), were used to concentrate sample solutions for the authentication of the reaction products according to the previous study (Chen et al., 2016, 2019).

LC-MS analysis was performed by using AB Sciex HPLC system, equipped with a high-resolution hybrid quadrupole-time-of-flight mass spectrometer (X500R QTOF, AB Sciex, USA). An electrospray ionizer (ESI) in negative mode was acted as an ion source. The analytes were separated on an Agilent ZORBAX Eclipse Plus C18 column (100 mm × 2.1 mm, 2.6 μm). The binary solvent system used was composed of 0.3% formic acid in water (A) and methanol (B) at a flow rate of 0.20 mL/min. The gradient program was 10% B for 4 min, followed by 100% B in 11.5 min and kept for 9.5 min, then returned to 10% B in 0.5 min and equilibrated for 14.5 min. The injection volume was 10 μL. Mass spectra were set as m/z 80 - 1100 in full-scan negative electrospray ion (ESI) ionization mode. Ion source parameters were set as follows: ionspray voltage floating, -4500 V; declustering potential, -80 V; collision energy, -10 V; temperature, 550 ℃; ion source gas 1, -55 psi; ion source gas 2, -55 psi; curtain gas, -35 psi. Nitrogen gas were used overall process. SCIEX OS program (AB Sciex) was used as the data processing software. For the purposes of identifying unknown products, the mass accuracy in this study was set at <5 ppm.

Fig. S1.  The ultraviolet spectra of BPS in different pH conditions.

Fig. S2.  The extracted ion chromatograms (XIC) of P12 (A) and P15 (B) at different reaction time, (B) the experimental m/z value at 265.01 of P12 (C) and P15 (D).

Supplementary References

Chen, J., Qi, Y., Pan, X., Wu, N., Zuo, J., Li, C., Qu, R., Wang, Z., Chen, Z., 2019. Mechanistic insights into the reactivity of Ferrate(VI) with phenolic compounds and the formation of coupling products. Water Res. 158, 338-349.

Chen, J., Qu, R., Pan, X., Wang, Z., 2016. Oxidative degradation of triclosan by potassium permanganate: kinetics, degradation products, reaction mechanism, and toxicity evaluation. Water Res. 103, 215-223.