5. APPENDIX-A

| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 | #--------------------------------------------------------------------------------Description #This program is about determination of the stress of trapizodal crane hook under the present study's boundary conditions. #Units are millimeter (mm) and netwon (N). #As both profile area 2827 mm square and parallet sides distant 60mm are constant, the sum of both parallel sides must be (2827*2)/60 = 94.23mm almost. So, neither any parallel side can be larger than 94.23mm. #--------------------------------------------------------------------------------Abbreviations #A = Area of trapezoidal profile #h = Height of trapizodal parallel sides #F = Applied vertical force #Ri = Inside radius of curved beam #Ro = Outside radius of curved beam #Si = Inside parallel side length #So = Outside parallel side length #Rc = Centroid_radius #Rn = Neutral_radius #Ci = Distrance between neutral axis to the inside surface #Co = Distrance between neutral axis to the outside surface #e = Distrance between centroid axis to neutral axis #Dh = Horizontal distance between the vertical line along which force is applied and the centroidal axis #M = Moment #inside_stress = Classical equation for calculating stress at the inside concave surface. #outside_stress = Classical equation for calculating stress at the outside convex surface. #FoS = Factor of safety #--------------------------------------------------------------------------------Standard Imports import math #--------------------------------------------------------------------------------Set Boundary Conditions A = 2827 #millimeters h = 60 #millimeters Ri = 50 #millimeters F = 50000 #Netwon #--------------------------------------------------------------------------------Calculations So = 90 #millimeters #initiate the value print("{:<20} {:<25} {:<25} {:<20} {:<8} {:<17} {:<17}".format("Iteration number", "Inside parallel side", "Outside parallel side", "Parallel distance", "Area", "Maximum stress", "Factor of safety")) for i in range(1,10): Si = (float(A)*2/float(h)) - float(So) #millimeters Ro = float(Ri)+float(h) #millimeters Rc = float(Ri)+(float(h)*(float(Si)+2*float(So)))/(3*(float(Si)+float(So))) #millimeters Rn = float(A)/(((float(Si)*float(Ro)- float(So)*float(Ri))/float(h))*math.log(float(Ro)/float(Ri),2.7182818284590452353602874713527) - (float(Si)-float(So))) #millimeters Ci = float(Rn) - float(Ri) #millimeters Co = float(Ro) - float(Rn) #millimeters e = float(Rc) - float(Rn) #millimeters Dh = float(Rc) #millimeters M = float(F)*float(Dh) #newton-millimeters inside_stress = (float(M)*float(Ci))/(float(A)*float(e)*float(Ri)) + (float(F)/float(A)) #newton-millimeters square outside_stress = -(float(M)*float(Co))/(float(A)*float(e)*float(Ro)) + (float(F)/float(A)) #newton-millimeters square if abs(float(inside_stress)) >= abs(float(outside_stress)): FoS = 650000000/(1000000*float(inside_stress)) #multiple by 1000000 in order to make the same unit "pascal" print("{:<20} {:<25} {:<25} {:<20} {:<8} {:<17} {:<17}".format(i, round(Si,2), round(So), h, A, round(1000000*inside_stress), round(FoS,3))) #inside_stress multipled by 1000000 in order to make the unit "pascal" So = float(So) - 10 #millimeters else: FoS = 650000000/(1000000*float(outside_stress)) #multiple by 1000000 in order to make the same unit "pascal" print("{:<20} {:<25} {:<25} {:<20} {:<8} {:<17} {:<17}".format(i, round(Si,2), round(So), h, A, round(1000000*outside_stress), round(FoS,3))) #outside_stress multipled by 1000000 in order to make the unit "pascal" So = float(So) - 10 #millimeters |
| --- | --- |