Title: Fluorine contamination, mobility, and risks in soils at a phosphate-gypsum waste landfill: A new analytical method and comparison with previous methods

Authors: Seok-Young Oh1*, Hyeongseok Kim1, Hye-On Yoon2

1Department of Civil and Environmental Engineering, University of Ulsan, Ulsan 44610, South Korea

2Korea Basic Science Institute Seoul Center, Seoul 02841, South Korea

Supplementary Material

Number of pages: 10 (including a cover page)

Number of descriptions: 4

Number of figures: 1

Number of tables: 3

Description S1. The analytical procedure for incineration/distillation-UV/vis spectrophotometry.

The soil samples were pretreated using two consecutive processes—incineration and distillation (Willard and Winter, 1933). During incineration, the sample was pulverized, sieved using a 0.075 mm standard sieve (200 mesh), and dried at 105 ℃; subsequently, 1 g of the soil sample and 5 g of calcium oxide (CaO) were filled into a nickel melting pot and mixed thoroughly. The sample was incinerated for 5 h in an electric furnace at 500 ℃, heated to 800 ℃ for an additional 2 h, and left to cool. After incineration, the sample was washed with 25 mL of deionized water and 50 mL of perchloric acid (70%) and transferred into a three-necked distillation glass flask with 10 drops of perchloric acid (17%). When the color of the sample turned milky white, an additional 10 drops of perchloric acid (17%) were added along with 8–10 boiling chips. When the sample contained a large amount of chloride, a large amount of Ag+ ions was added to remove the chlorides. Subsequently, approximately 600 mL of deionized water was added to the distillation flask, which was heated with a distillation apparatus. The temperature in the three-necked flask was maintained at 135 ± 2℃, and the distillation rate was maintained at 5–6 mL/min. The effluent from the distillation was received in a pre-prepared 500 mL volumetric flask into which one drop of nitrophenol indicator and one drop of sodium hydroxide solution were added. When the effluent volume reached 480 mL, the distillation procedure was terminated, and the interior of the cooling tube was washed with a small amount of deionized water with which the receiving flask was filled to the mark. In the next step, 50 mL of the pretreated sample was mixed with a 10 mL zirconium oxychloride-SPADNS mixture in a 100 mL volumetric flask; the absorbance at 570 nm was measured using a UV/vis spectrophotometer (DR-2800, Hach, Loveland, CO, USA). The fluorine concentration in the solution was calculated from the calibration curve prepared using fluorine standard solutions. In addition, the fluorine concentration in the soil (mg/kg) was calculated as follows (Na et al., 2010):

The concentration of fluorine in soil (mg/kg) =  Cs-CbWd ×f×V,

Cs: Concentration of fluorine in solution (mg/L);

Cb: Concentration of fluorine in the blank sample (mg/L);

f: Dilution factor (when the calibration curve is out of range);

V: Final volume of solution (in this case: 0.5 L);

Wd: Dry weight of soil sample (in this case: 0.001 kg).

Description S2. The analytical procedure for the alkaline fusion-selective ion electrode method.

The fluorine concentration was determined following the procedure reported by McQuaker and Gurney (1977). Briefly, after the soil, rock, and gypsum waste samples had been pulverized such that they could pass through the 200-mesh sieve (< 75 µm), 0.5 g of the sample was mixed with 16 M NaOH in a nickel melting pot. After drying at 150 °C for 1 h, the mixture was heated from 300 to 600 °C for 0.5 h and then cooled in the desiccator. Subsequently, 10 mL of distilled water was added to the sample, which was then slightly heated to accelerate the dissolution of the alkali cake. After the pH was adjusted to a value between 8 and 9 by adding 7–8 mL concentrated hydrochloric acid, the mixture was filtered using a glass fiber-type B filter, and the filtrate was diluted to 100 mL with deionized water in a 100 mL volumetric flask. In the next step, 25 mL of the prepared solution was mixed with 25 mL TISAB Ⅲ solution at pH 5.2, and the fluorine concentration of the mixture (mg/L) was measured using an Orion benchtop pH meter (Thermo Scientific, Pittsburgh, PA, USA) equipped with a fluorine electrode. The fluorine concentration (mg/kg) was determined based on the solid (g)-to-solution (mL) ratio.

Description S3. The analytical procedure for aqua regia extraction-UV/vis spectrophotometry.

To avoid the difficulties of the pretreatment process required for incineration/distillation or alkaline fusion, the combination of aqua regia extraction and the determination of the fluorine concentration in water using UV/vis spectrophotometry was employed (Kwon et al., 2015). A soil sample sieved through a 200 mesh sieve was mixed with 10 mL aqua regia, and the mixture was heated at 70 ℃ for 2 h, filtered using the glass fiber-type B filter, and diluted to 100 mL with deionized water. The fluorine concentration in the solution was analyzed with the lanthanum alizarin complexone method (Wada et al., 1985). After 30 mL of the solution had been mixed with 20 mL lanthanum alizarin complexone solution for 1 h, the absorbance at 620 nm was measured using a UV/vis spectrophotometer. The fluorine concentration in the soil was calculated as follows (Na et al., 2010):

Fluorine concentration in soil (mg/kg) = (y-b)a×I×V:

y: Absorbance of sample;

b: Y-Intercept of the calibration curve (absorbance);

a: Slope of calibration curve;

I: Sample dilution factor;

V: Final volume of solution (0.1 L).

Description S4. Calculation of the hazard quotient (HQ) for fluorine in soil.

1. HQ by ingestion of crops and soil

HQ=ADDRfD0

ADD (mg/kg-day)=Cs×(BCF×CRp+CRs×CF1)×ABSGI×EF×EDBW×AT

Cs= Exposure concentration of soil (mg/kg)

BCF = Bioconcentration factor (mg/kg)crops/(mg/kg)soil (dimensionless)

(crop, fruit, and root vegetables: 0.006, leafy vegetables: 0.06)

CRp = Consumption rate of produce (kg/day) (industrial area: 0)

CRs = Consumption rate of soil (mg/kg) (adult: 50; child: 118)

CF1 = Conversion factor 1 (kg/mg) (10-6)

ABSGI = Inhalation absorption Factor (1)

EF = Exposure frequency (days/year) (residential and agricultural area: 350, industrial area: 100)

ED = Exposure duration (year) (adult: 25; child: 6)

BW = Body weight (kg) (adult: 62.8; child: 16.8)

AT = Average time (days) (28,689)

RfD0 = Reference dose of fluorine (0.04 mg/kgday)

2. HQ by groundwater containing soil-derived contaminants

HQ=ADDRfD0

ADD (mg/kg-day)=Cw×CRw×EF×EDBW×AT

CL= Cs(kd+(θw+θaH)ρb)= 800(150+0.161.6) =5.3297

Cw mgL=CLDAF=5.329720=0.2664

CRw = Consumption rate of groundwater (L/day) (adult: 2; child: 1)

EF = Exposure frequency (days/year) (residential and agricultural area: 350; industrial area: 100)

ED = Exposure duration (year) (adult: 25; child: 6)

BW = Body weight (kg) (adult: 62.8; child: 16.8)

AT = Average time (days) (28,689)

RfD0 = Reference dose of fluorine (0.04 mg/kgday)

CL = Concentration in liquid (mg/L)

Cs = Concentration in soil (mg/kg)

Cw = Exposure concentration of groundwater (mg/L)

Kd = Distribution coefficient (150)

w = moisture porosity (0.16)

α = Air porosity (0, assuming full saturation of water)

H = Henry’s constant

b = Soil bulk density (1.6)

DAF = Dilution attenuation factor (20)

3. HQ by inhalation of outdoor fugitive dust

HQ=ADDRfC

ADD (mg/m3)=Cs×ABSinf ×(TSP×frs×Fr)×EF×ED×CF1AT

ABSinh = Inhalation absorption factor (dimensionless) (1)

TSP = Concentration of suspended particles in air (mg/m3) (measurement or 0.07)

frs= Fraction of soil in suspended particles in air (dimensionless) (0.5)

Fr = Fugitive dust residual rate in respiratory (dimensionless) (0.75)

CF1= Conversion factor 1 (kg/mg) (10-6)

EF = Exposure frequency (days/year) (residential and agricultural area: 350; industrial area: 100)

ED = Exposure duration (year) (adult: 25; child: 6)

AT = Average time (days) (28,689)

RfC = Reference concentration of fluorine (0.013 mg/m3)

Fig. S1. The relationship between the F concentrations obtained through alkaline fusion and those obtained through aqua regia extraction. The total number of samples was 69 (BH and GH). The correlation coefficient between the two data was 0.955 (P < 0.01).

Fig. S2. The relationships between the total F concentrations obtained using the incineration/distillation procedure at three different institutes. The total number of samples was 43 (GBH).

Table S1. The results of the total F concentrations (mg/kg) in soils sampled in 2019 (Samples prefixed with GBH; 43 samples).

| Sample # | Total (alkaline) | Total (aqua regia) | TCLP | KWLP | Total (XRF) | Total (Institute A) | Total (Institute B) | Total (Institute C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 452 | 212 | 223 | 107 | 610 | 0 | 1052 | 196 |
| 2 | 357 | 453 | 119 | 40 | 510 | 1383 | 714 | 219 |
| 3 | 359 | 222 | 187 | 82 | 580 | 1105 | 808 | 184 |
| 4 | 705 | 351 | 359 | 140 | 1160 | 1601 | 10997 | 118 |
| 5 | 535 | 830 | 177 | 57 | 720 | 1334 | 939 | 262 |
| 6 | 394 | 196 | 234 | 95 | 580 | 1539 | 822 | 260 |
| 7 | 547 | 355 | 181 | 58 | 740 | 0 | 1128 | 279 |
| 8 | 195 | 121 | 96 | 44 | 260 | 0 | 492 | 272 |
| 9 | 225 | 425 | 130 | 50 | 320 | 0 | 627 | 253 |
| 10 | 316 | 132 | 167 | 69 | 480 | 1292 | 527 | 570 |
| 11 | 546 | 721 | 158 | 48 | 740 | 1546 | 3633 | 181 |
| 12 | 236 | 456 | 199 | 53 | 410 | 446 | 538 | 183 |
| 13 | 212 | 376 | 66 | 50 | 290 | 995 | 483 | 125 |
| 14 | 636 | 567 | 391 | 101 | 1140 | 3784 | 1408 | 3053 |
| 15 | 436 | 543 | 137 | 57 | 730 | 0 | 727 | 973 |
| 16 | 293 | 172 | 174 | 82 | 430 | 0 | 717 | 441 |
| 17 | 486 | 301 | 140 | 53 | 760 | 0 | 854 | 536 |
| 18 | 936 | 531 | 261 | 72 | 1210 | 1412 | 2393 | 281 |
| 19 | 563 | 320 | 244 | 104 | 1120 | 2172 | 971 | 324 |
| 20 | 686 | 786 | 237 | 91 | 1160 | 1340 | 1738 | 274 |
| 21 | 403 | 197 | 124 | 35 | 750 | 3521 | 733 | 1826 |
| 22 | 722 | 454 | 228 | 67 | 660 | 829 | 2121 | 483 |
| 23 | 301 | 189 | 101 | 25 | 550 | 0 | 529 | 283 |
| 24 | 574 | 275 | 209 | 58 | 1030 | 1406 | 990 | 280 |
| 25 | 902 | 423 | 253 | 85 | 1680 | 1503 | 2836 | 217 |
| 26 | 410 | 186 | 254 | 96 | 710 | 1055 | 1172 | 378 |
| 27 | 1484 | 623 | 361 | 113 | 2410 | 1426 | 3421 | 331 |
| 28 | 645 | 980 | 332 | 119 | 950 | 866 | 3221 | 308 |
| 29 | 354 | 172 | 144 | 55 | 580 | 0 | 824 | 222 |
| 30 | 400 | 208 | 128 | 57 | 650 | 0 | 756 | 163 |
| 31 | 179 | 81 | 100 | 41 | 470 | 0 | 474 | 200 |
| 32 | 529 | 275 | 170 | 53 | 990 | 0 | 8541 | 5094 |
| 33 | 576 | 657 | 163 | 62 | 1000 | 0 | 11441 | 899 |
| 34 | 336 | 137 | 132 | 55 | 620 | 1103 | 6771 | 154 |
| 35 | 1612 | 725 | 402 | 97 | 2310 | 1681 | 13591 | 320 |
| 36 | 273 | 234 | 146 | 43 | 680 | 1640 | 1301 | 19618 |
| 37 | 368 | 172 | 170 | 59 | 730 | 1714 | 672 | 829 |
| 38 | 330 | 185 | 132 | 53 | 615 | 0 | 3985 | 365 |
| 39 | 266 | 130 | 160 | 58 | 490 | 0 | 13660 | 257 |
| 40 | 467 | 317 | 210 | 78 | 675 | 0 | 5360 | 229 |
| 41 | 368 | 250 | 183 | 65 | 990 | 0 | 7400 | 201 |
| 42 | 312 | 168 | 179 | 56 | 520 | 192 | 6070 | 2126 |
| 43 | 390 | 206 | 166 | 81 | 640 | 1576 | 825 | 900 |

Table S2. The results of the total F concentrations (mg/kg) in soils and gypsum wastes sampled in 2020 (Samples prefixed with BH; 44 samples).

| Sample # | Total | TCLP | KWLP | Total (XRF) |
| --- | --- | --- | --- | --- |
| 1 | 3237.4 | 740 | 380 | 4380 |
| 2 | 556.1 | 170 | 26 | 750 |
| 3 | 597.1 | 69 | 22 | 750 |
| 4 | 1840.6 | 1000 | 320 | 2570 |
| 5 | 486.6 | 110 | 112 | 650 |
| 6 | 3536.6 | 630 | 150 | 4820 |
| 7 | 627.8 | 97 | 42 | 790 |
| 8 | 595.5 | 180 | 98 | 750 |
| 9 | 3426.1 | 620 | 136 | 4070 |
| 10 | 332.7 | 66 | 90 | 300 |
| 11 | 375.6 | 110 | 46 | 500 |
| 12 | 4205.9 | 530 | 540 | 5820 |
| 13 | 4495.7 | 490 | 118 | 5740 |
| 14 | 546.3 | 80 | 48 | 650 |
| 15 | 478.5 | 92 | 66 | 530 |
| 16 | 568.1 | 97 | 74 | 850 |
| 17 | 1912.3 | 530 | 138 | 2810 |
| 18 | 510 | 95 | 100 | 680 |
| 19 | 386.9 | 78 | 64 | 340 |
| 20 | 442 | 63 | 56 | 680 |
| 21 | 2054.2 | 510 | 280 | 4510 |
| 22 | 378.5 | 100 | 96 | 610 |
| 23 | 460.7 | 100 | 132 | 580 |
| 24 | 632.7 | 92 | 140 | 710 |
| 25 | 2000 | 550 | 126 | 2640 |
| 26 | 455.6 | 360 | 44 | 620 |
| 27 | 429 | 180 | 47 | 550 |
| 28 | 610 | 230 | 50 | 860 |
| 29 | 1787.3 | 1500 | 620 | 2460 |
| 30 | 752.4 | 140 | 66 | 950 |
| 31 | 776 | 110 | 88 | 660 |
| 32 | 626.7 | 78 | 50 | 490 |
| 33 | 340 | 91 | 52 | 340 |
| 34 | 450 | 110 | 56 | 450 |
| 35 | 230 | 69 | 68 | 250 |
| 36 | 250 | 69 | 44 | 210 |
| 37 | 120 | 59 | 100 | 110 |
| 38 | 100 | 47 | 52 | 80 |
| 39 | 100 | 29 | 56 | 70 |
| 40 | 50 | 51 | 62 | 0 |
| 41 | 100 | 63 | 66 | 80 |
| 42 | 50 | 46 | 34 | 20 |
| 43 | 60 | 33 | 50 | 40 |
| 44 | 50 | 100 | 56 | 0 |

Table S3. The results of the total F concentrations (mg/kg) in soils sampled in 2021 (Samples prefixed with GH; 25 samples)

| Sample # | Total (alkaline) | Total (aqua regia) | Total (XRF) |
| --- | --- | --- | --- |
| 1 | 375 | 350 | 570 |
| 2 | 396 | 316 | 560 |
| 3 | 380 | 325 | 710 |
| 4 | 619 | 454 | 1130 |
| 5 | 360 | 259 | 590 |
| 6 | 465 | 414 | 620 |
| 7 | 365 | 277 | 780 |
| 8 | 173 | 149 | 340 |
| 9 | 346 | 458 | 550 |
| 10 | 318 | 433 | 560 |
| 11 | 237 | 305 | 380 |
| 12 | 559 | 512 | 870 |
| 13 | 379 | 343 | 500 |
| 14 | 433 | 371 | 860 |
| 15 | 308 | 236 | 420 |
| 16 | 513 | 434 | 1010 |
| 17 | 468 | 351 | 720 |
| 18 | 375 | 326 | 510 |
| 19 | 561 | 512 | 770 |
| 20 | 376 | 311 | 860 |
| 21 | 589 | 480 | 910 |
| 22 | 476 | 241 | 770 |
| 23 | 800 | 501 | 980 |
| 24 | 524 | 447 | 690 |
| 25 | 340 | 190 | 420 |