Supplementary Information of

Dispersive Fourier transform based dual-comb ranging

Bing Chang1,#, Teng Tan1,2,#, Junting Du1,#, Zeping Wang1, Zihan Liu1, Xinyue He1, Chun Wang1,3, Handing Xia3, Zhaohui Wu3, Jindong Wang4, Kenneth K. Y. Wong5, Tao Zhu4, Lingjiang Kong2, Bowen Li1,*, Yunjiang Rao1,6,*, Baicheng Yao1,*

1 Key Laboratory of Optical Fiber Sensing and Communications (Education Ministry of China), University of Electronic Science and Technology of China, Chengdu 611731, China.

2 School of Information and Communication Engineering, University of Electronic Science and Technology of China, Chengdu 611731, China.

3 Research Center of Laser Fusion, China Academic of Engineering Physics, Mianyang 621900, China.

4 Key Laboratory of Optoelectronic Technology & Systems (Education Ministry of China), Chongqing University, Chongqing 400044, China.

5 Department of Electrical and Electronic Engineering, University of Hong Kong, Pokfulam Road, Hong Kong SAR 990777, China.

6 Research Centre for Optical Fiber Sensing, Zhejiang Laboratory; Hangzhou, 310000, China.

These authors contributed equally: Bing Chang, Teng Tan, Junting Du

Corresponding authors: yaobaicheng@uestc.edu.cn; bowen.li@uestc.edu.cn; yjrao@uestc.edu.cn

This Supplementary Information consists of the following sections:

S1. Theoretical analysis

S2. Characterization of the fiber dual-comb source

S3. Extended measurements

Supplementary Figures S1-S9

Supplementary Table S1

Supplementary References S1-S9

Supplementary note S1: Theoretical analysis

S1.1 Dual-comb based time-of-flight ranging

Dual-comb ranging based on the time-of-flight (TOF) method mainly uses the vernier effect of the two pulse trains with a minor repetition rate difference [1], one comb is used as the ruler (local comb), and the other comb is divided into two parts: a probe comb and a reference comb, they are together called signal comb [2,3]. Fig. S1 shows the schematic diagram of dual-comb vernier effect in time domain. Assuming that the repetition rates of signal comb and local comb are fr1 and fr2 respectively, the corresponding pulse periods are T1=1/ fr1 and T2=1/ fr2, and thus the repetition rate difference is Δfr = |fr1 - fr2|. Here, we mainly discuss the case that fr1 is smaller than fr2 (T1 > T2) for instance.

In one complete update period (Tupdate=1/Δfr) of a dual-comb interferometry, determined by the repetition rate difference between the two combs, the local comb will encounter the reference comb and the probe comb in turn. At this point, one can always find the case where two adjacent local pulses are sandwiched between two adjacent signal pulses, as shown in the dotted box at the bottom of Fig. S1, which produce four types of pulse-to-pulse time intervals (τ1~τ4), corresponding to the interval between pulses i and iii (τ1), pulses ii and iv (τ2), pulses v and vii (τ3), and pulses vi and viii (τ4). Obviously, they meet the following relationship:

(1)

When we regard the time of pulse i as the benchmark (ti = 0), the time of pulse ii~ viii can be written as follows:

tii = T1, tiii = τ1, tiv = τ1+T2, tv = ttof + (N-1) T1, tvi = ttof + N×T1, tvii = tiii + N×T2 = τ1 + N×T2, tviii = τ1 + (N+1) × T2.

Here ttof is the time of flight. Since tvii – tv = τ3, we can obtain the pulse-to-pulse delay ttof between the probe comb and the reference comb by combining the above time expressions:

(2)

Wherein, the integer N can be obtained by:

(3)

Then the distance can be calculated by LD=c·ttof/2n, n denotes the refractive index of the light transmitting medium. For instance, in air, n = 1.

It is worth mentioning that although we only use τ1 and τ3 when calculating ttof, τ2 and τ4 are equally important, because it can help us judge the relative position of the signal comb pulse and the local comb pulse, based on Eq. (1).

Fig. S1. Schematic diagram of dual-comb pulse interference. Here the dark blue pulses show the reference comb, light blue pulses show the probe comb, they have the same period T1. The red pulses show the local comb, which has a period T2. Dual-comb interferences only appear when two pulses overlap.

S1.2 Dispersive Fourier Transformation

According to Eq. (2), the detection accuracy of the time-of-flight of the signal comb pulse depends on the temporal demodulation accuracy of τ1 and τ3, which determines the detection accuracy the dual-comb ranging. Commonly, limited by the bandwidth of a photodetector (up to tens of gigahertz), it is impossible to distinguish two pulses whose time interval is in the order of sub picoseconds. On the other hand, the dispersive Fourier transform (DFT) provides a unique tool for visualizing and analyzing ultrafast time-domain signals [4], which can effectively satisfy the precise measurement requirement of pulse intervals in ranging applications.

Starting from the classical electromagnetic theory, we define E1 to represent the output electric field of a soliton pulse, whose spectrum can be written as:

(4)

Herein  illustrates the Fourier transform, and θ(ω) denote the magnitude and phase of the soliton spectrum, respectively. The chirp of the pulse can be written as, which equals zero when there is no chirp. Now consider a dispersive element with a length L. Its transfer function of linear group-delay dispersion (GDD) is in the following form [5]:

(5)

Herein ϕ(ω) is the phase profile and it can be Taylor-expanded as:

(6)

Then the group delay can be written as:

(7)

Here βm is the mth-order coefficient of the expansion, where the group velocity is 1/β1 while the group velocity dispersion (GVD) is β2. Assuming without loss of generality, that means β0 = 0, the electric field of a transformation-limited pulse (without chirp) passing through a large GVD element could be deduced as:

(8)

Herein,  represents the retarded phase profile,

(9)

corresponding to a retarded group delay

(10)

Here  represents the time variable retarded by dispersion element.

Similar to Fraunhofer diffraction, the spectrum of a pulse can be mapped to the time domain when sufficient dispersion satisfies the stationary phase approximation (SPA) [6,7]. In the case of linear dispersion, the classic time-stretched dispersion Fourier transform (TS-DFT) can be obtained. So, according to SPA condition, under a large GVD condition, the term  in Eq. (8) would oscillate rapidly and cause the integral to vanish, except in a small range around the stationary frequency ωs.

(11)

Here,  denotes the initial chirp of the incident pulse, which can be ignored for near-transform-limited pulses. When high-order dispersion is not considered, Eq. (11) can be further simplified as:

(12)

Accordingly, in the case that there is only linear dispersion (i.e. group delay is a linear function of optical frequency), the time delay of the frequency component ω in the pulse is T. Therefore, the spectral components in the output pulse are separated, and thus there exists a linear map between frequency and time. When two pulses with a small time delay are simultaneously transmitting in a dispersive element, the two pulses will be co-stretched and overlap each other. Since their frequency components are spread out in time, spectral interference occurs in their overlapping parts. Considering the central moment of the pulse1 is t, and the central moment of the pulse2 is t-τ, the instantaneous frequency of the pulse1 and the pulse2 are  and , respectively. Here τ is the relative delay of the two pulses. The frequency difference fi at each moment is [8]:

(13)

As a result, we can obtain the relative delay of the two pulses through the spectral analysis of the interference fringes. When considering higher-order dispersions such as the third order dispersion β3, the time delay of the frequency component ω in the pulse would become more complicated:

(14)

Then the instantaneous frequency of the pulse1 and the pulse2 are:

(15)

In this case, we can also write the interferometric fringe frequency as:

(16)

The beating frequency of the above equation can be divided into a beat frequency caused by linear dispersion and another beat frequency caused by the third-order dispersion.

In our experiment, two dispersion-compensating fiber modules (Corning PureForm DCM-D-080-04) were used as the dispersive element with a dispersion parameter of D = 87 ps/nm·km (The total length is 30.39 km) and a dispersion slope of S = 0.025 ps/nm2·km. Correspondingly, β2 = 110.94 ps2/km and β3 = 0.142 ps3/km can be obtained after simple calculation. It can be seen that the frequency chirp caused by third-order dispersion is much smaller than that of linear dispersion so that it can be ignored.

S1.3 Parametric Simulation and Discussion

Based on Eq. (13), for a pulse delay τ, the frequency of interference fringes depends on the total dispersion of the dispersive element. Therefore, optimization of the total dispersion amount is an important issue in practical experiment. The total dispersion |β2|L influences the effect of the time-stretched DFT by limiting the range of pulse delay which we can measure, and changing the fringe frequency directly. Here, we discuss the relationship between the amount of the total dispersion |β2|L and main parameters of the optical frequency comb (∆λ, fr1, fr2, Δfrep) in simulations.

On one hand, |β2|L cannot be too large. According to the DFT, the broadened pulse can have an enlarged temporal width δ,

(17)

Referring the cross-relationship among the stretched pulse width δ, the spectral 3dB bandwidth ∆λ, and the dispersion |β2|L of the dispersive element (Fig. S2a), in order to avoid pulse aliasing, the stretched pulse width should not exceed the pulse repetition period (the dashed yellow line in Fig. S2a), that is:

(18)

Correspondingly, for an optical frequency comb with spectral width of ∆λ, the amount of dispersion has a maximum value:

(19)

Fig. S2b shows the interference fringe frequency fi, versus with the pulse delay τ and the total dispersion |β2|L. According to Eq. (13), fi is proportional to τ. Meanwhile, |β2|L affects its slope (chirp rate). Besides, in the DFT-enhanced dual-comb ranging system, Eq. (2) requires that the maximum detectable pulse delay should be larger than the maximum possible value of τi (i=1,2,3,4), we write:

(20)

On the other hand, the bandwidth limitation of the detector Bpd should be considered. In Fig. S2b, we plot the case when we cannot detect the fringes with frequencies larger than the photodetector’s bandwidth (the dashed yellow line). Due to the above double limitation (the maximum detectable pulse delay and Bpd), we estimate a minimum value of the total dispersion:

(21)

Furthermore, when Eq. (19) and Eq. (21) are satisfied, a larger total dispersion and a wider pulse broadening, would bring more fringes and higher signal-to-noise ratio (SNR) in the spectrum after Fast Fourier transform (FFT). In practice, to obtain a large total dispersion commonly relies on the increment of L (the length of dispersive element), which is usually accompanied by considerable losses. In this work, taking into account both the number of fringes and the loss, we chose the amount of dispersion |β2|L = 1697.382 ps2 (the white dotted line). In addition, according to Eq. (2), in the DFT enhanced dual-comb ranging system, the distance measurement result can be expressed as:

(22)

Since N is an integer, the measurement of distance can be divided into discrete intervals (corresponding to different N), and the measurement range of each interval is determined by τ1-τ3. Here, we divide the value range of τ1-τ3 into three intervals, as shown in Fig. S2c. One is τmax < ΔTr/2, corresponding to a value range of τ1-τ3 is (-ΔTr/2, ΔTr/2) (R1). There are two problems in this case, on the one hand, we cannot obtain correct τ1 and τ3, because we cannot obtain the complete τ1, τ2, τ3, τ4 through Eq. (1). On the other hand, the distance measurement is discontinuous, which means there is a measurement dead zone (purple part in the Fig. S2c). The second case is ΔTr/2 ≤ τmax < ΔTr, corresponding to a value range of τ1-τ3 is (-ΔTr, ΔTr) (R2). In this case, although the distance can be covered continuously, it still does not satisfy the Eq. (1), so that the correct τ1 and τ3 cannot be obtained in each frame. The third case is τmax > ΔTr (R3), in this case, the measurable distance is continuous and we can obtain τ1 and τ3 with certainty for each frame. In our system, we meet the requirements of R3 by selecting a larger GVD.

Fig. S2. Parametric Simulation and Discussion. a. Simulated parameter space, showing the relationship between δ, ∆λ, and |β2|L. b. Simulated parameter space, showing the relationship between fi, τmax, and |β2|L. c. The relationship between the ranging distance and τmax.

Supplementary Note S2: Characterization of the fiber dual-comb source

S2.1 Experimental setup

The dual-comb fiber mode-locked laser used in the experiment is realized via a hybrid passive mode-locking mechanism, by using the NPR effect and intracavity semiconductor saturable absorber mirror (SESAM). Each fiber laser comb is composed by a 980 nm pump laser diode, a section of 2 m highly-nonlinear erbium doped fiber (HNL-EDF), a SESAM, a section of 6 m single mode fiber (SMF), a squeeze polarization controller (PC), a polarization dependent-optical integrated component (PD-OIC) and a piezoelectric ceramic transducer (PZT), as shown in Fig. S3a. The HNL-EDF simultaneously provides rare-earth gain for the laser excitation and high nonlinear gain for nonlinear interactions (e.g. SPM, XPM and SRS). The PD-OIC offers comprehensive functions including 980/1550nm wavelength-division-multiplexing, 80/20 coupling, and polarization-selective-isolation, which can also help to suppress the backward Brillouin scattering. By adjusting the polarization of the light in the cavity, we generate a soliton mode-locked pulse sequence with average output power of 0.5 mW when the pump power was 10 mW. As shown in Fig. S3b, each comb is packaged in a box with a size of 20×15×10 cm3 and a weight of <1 kg. After vibration isolation treatment, each fiber laser comb can plug and play stably, with long term stability > 48 hours and intensity uncertainty < 1%.

For fully locking the repetition rates of each fiber comb respectively, we control the cavity length Lf of the fiber loop by using a PZT. Fig. S3a also shows the servo feedback control setup for the repetition rate locking. First, the difference between the comb repetition frequency (detected by the photodetector) and an ultra-stable frequency signal (from a standard radio frequency oscillator) is mixed through a frequency mixer (Mini-circuits ZLW-6+), generating a down-converted reference. A low-pass filter (Mini-circuits BLP-5+, DC to 1.9 MHz) is then used to filter out the low-frequency differential signal, which works as the error signal after amplified. Finally, a high-speed servo controller (Vescent D2-125-IP-230) is used to generate a feedback control signal according to the input error signal to drive the voltage controller on the PZT (Core morrow E53.A1K, Input 0~10V, output 0~150V). Due to the electrostriction effect of PZT (maximum diameter expansion 0.38 μm at 150V), a cavity length stretches of 10 μm can be achieved under full load driving, which corresponds to a repetition frequency locking range of 30 Hz. The controlling accuracy of the feedback loop is 0.003 μm.

Fig. S3. Fiber comb generation. a. Experimental setup. b. Packaged prototype of our mode locked fiber laser comb.

S2.2 Dual-comb characterization

We characterize the output pulses of the signal laser comb and the local laser comb separately. Fig. S4a demonstrates the measured optical spectra of the two combs. Their full width at half maximum (FWHM) is about 6.84 nm (0.855 THz) for the local comb and 6.09 nm (0.761 THz) for the signal comb, respectively. The Kelly sideband is 25dB lower than the main peak, which means that the energy of the dispersion wave is low enough compared to the soliton envelope. Using the second harmonic generation (SHG)-based autocorrelation technique, we measure frequency-resolved-optical-gating (FROG) maps of the two soliton pulses (Fig. S4b). The retrieved pulse duration of the local and the signal comb is 368 fs and 416 fs, corresponding to time-bandwidth products of 0.315 (local comb) and 0.316 (signal comb), respectively, approaching the Fourier transform limit. In Fig. S4c, we show the line-to-line beat notes of the local comb and the signal comb. It verifies that repetition rate of the local comb is 24.55 MHz, while repetition rate of the signal comb is 24.465 MHz . For the first self-beating frequency of each comb, the SNR is higher than 70 dB.

Fig. S4. Frequency and time characterization. a. Measured optical spectra of the local comb and the signal comb. b. Measured FROG maps of the local comb and the signal comb. c. Measured self-beat notes of the local comb and the signal comb.

In electronics, we test the noises of our comb sources. First, we measure the relative intensity noise (RIN) of our comb source in the top panel of Fig. S5a. Typically, the RIN of each laser comb is -98 dBc/Hz at 1 Hz offset, -121 dBc/Hz at 1 kHz offset, and < -150 dBc/Hz at 1 MHz offset. In addition, we measure the single sideband phase noise (SSB-PN) by using a Phase Noise Analyzer (Rohde & Schwarz FSWP26) in the bottom panel of Fig. S5a. Typically, the SSB-PN of either the signal comb or the local comb (carrier ≈ 24.55 MHz) is down to -75 dB/Hz at 1 Hz offset, -126 dBc/Hz at 1 kHz offset, while -134 dB/Hz at 1 MHz offset. In Fig. S5b, we show the measured Allan deviation. When the averaging time is 2.5 ms, the Alan deviation of the two combs reaches the minimum value, telling a number 1.26×10-10. In Fig. S5c, we show the measured spectrum of the dual-comb beating (first beating line). First we confirm that the dual comb beats show a difference frequency 85 kHz, it demonstrates a signal-to-noise ratio > 60 dB. There are no excess heterogeneous frequency components in the low-frequency section. Moreover, we also characterize the long-term frequency stability of the two combs in free-running and locked states, respectively. Fig. S5d plots the frequency drift of the two combs within 60 s in the free-running (blue) and locked (red) states, respectively. Frequency drift reduced from 6 Hz to 0.25 mHz after locking. In this figure, we also show the stability of the radio frequency reference. Due to the locking operation, performance of the comb sources is mainly limited by the microwave generator.

Fig. S5. Stability characterization. a. Measured RIN and SSB-PN of our stabilized fiber laser comb. b. Allan deviation, it suggests a minimum number at the averaging time of 2.5 ms. c. Measured beat note of the dual comb heterodyne (signal and local). d. Measured frequency drifting of the local comb and the signal comb. Here the grey curves show the unlock (free running) case.

Supplementary Note S3: Extended measurements

S3.1 DFT characterization

We use the experimental setup (shown in Fig. S6a) to characterize the real-time interferometry based on the DFT. One of the dual combs (frep=24.465MHz) is divided into two paths through a fiber coupler (50:50), one path passes through a motorized delay stage, and the other path passes through a polarization controller (PC) to maximize modulation depth of the interference fringes. The two paths are then coupled together. The large group velocity dispersion (GVD) is provided by two dispersion-compensating fiber modules (high dispersive fiber, or HDF, Corning PureForm DCM-D-080-04) with a total group delay dispersion (GDD) of 2644 ps/nm. Here, for consistency, the dispersive element is same as in the ranging experiment. Due to the high insertion-loss of the HDF, a low noise erbium-doped fiber amplifier (EDFA) is used to boost the optical power. The interference signal is detected in a photodetector (Newport, Model 1414, 25 GHz bandwidth) and digitized in a high-speed oscilloscope (Tektronix, DPO72004B, 20 GHz bandwidth, 80 GHz sampling rate). Fig. S6b shows the temporal waveforms of the output pulses of the mode-locked laser before and after dispersion stretching. The initial pulse width is 368 fs, while the stretched pulse width is 18 ns. By adjusting the fiber delay line, clear interference fringes are observed in the oscilloscope. Fig. S6c shows the interferogram when the pulse delay are 8 ps, 16 ps and 80 ps respectively. After Fourier transform, the interference signal demonstrates corresponding fringe frequencies 0.3726GHz, 0.7453GHz and 3.7264GHz. We then characterize the accuracy of the DFT. Fig. S6d shows the measured frequencies of the interference fringes, scale with the pulse delays. The result suggests high linearity, the correlation coefficient R of its linear fit is approximately equal to 0.99999. and the slope matches the dispersion medium, and Fig. S6e shows the difference between the retrieval pulse separation measured by the DFT and the pre-set value. The small difference may be caused by the random fluctuation of the fiber link or the noise of Digital acquisition system.

Fig. S6. Characterization of DFT method. a. Experimental setup for characterizing the DFT. b. The measured pulse before and after dispersion stretching. c. Interference fringes for different time delays. d. The relationship between the time delay and the frequency of fringes. e. The absolute error between DFT retrieval time delay and pre-set time delay.

S3.2 Signal processing method

Now we provide more details about our signal processing method in ranging experiment. The interferograms are detected by an AC-coupled 25-GHz InGaAs photodetector (Model 1414, Newport) and digitized with a high-speed 20-GHz oscilloscope (DPO72004B, Tektronix), commonly we can use the sampling rate at 50 GS·s−1. Fig. S7 shows the detailed calculating procedure for solving the distance automatically. First, we cut the interferogram into K segments according to the period of the local comb, and number them Pk (k=1~K). Then we use fast Fourier transform (FFT) on each segment of the waveform to obtain its frequency spectrum. Next, we search for the frequency fk with the highest signal-to-noise ratio (SNR) in the retrieved spectrum and calculate τ by using the Eq. (13). According to the theoretical analysis in S1.1, the sum of a pair of adjacent two τk can always be found at the place where the probe comb and the local comb, and the reference comb and the local comb interfere with each other to satisfy the Eq. (1), corresponding to τ1, τ2 and τ3, τ4. Therefore, we can easily find τm,1 & τm+1,2 and τn,3 & τn+1,4 by querying τk in the two adjacent segments, where m and n are the number of segments they are located at. Then N can be obtained by N = n-m. Then we calculate the flight time ttof by Eq. (2), and finally we can calculate the distance LD according to LD =c·ttof/2.

Fig. S7. Detailed calculating procedure of distance solution. The data process includes 8 steps.

S3.3 Comparison with traditional dual-comb T	OF ranging

Now we compare the DFT enhanced dual-comb ranging and traditional TOF ranging. The comparison is finished by using the same signal and local combs. We note that in traditional TOF method without DFT, for meeting the Nyquist sampling theorem, the repetition rate difference Δfrep and spectral bandwidth Δνcomb of the two soliton pulse need to satisfy the following relations[2]:

(23)

The dual comb parameters used in the experiment are as follows: fr1=24.465MHz, fr2=24.55MHz, Δνcomb ≈ 0.855 THz, Δfrep ≈ 85 kHz. Obviously, the above dual comb parameters do not satisfy Eq. (23), resulting that no stable interference pattern will be generated, and only an intensifier peak due to too long relaxation time of the photodetector can be obtained. In this case, distance demodulation based on interference patterns is not applicable, and enhanced peak envelope fitting demodulation limits resolution. To achieve traditional dual-comb ranging under the existing repetition rate parameter, one solution is to reduce Δfrep to less than 351 Hz, and another is to compress Δνcomb to less than 2.24 GHz. The former scheme greatly reduces the acquisition rate and accuracy while the latter scheme needs an optical filter with a bandwidth of < 0.028 nm, which will greatly reduce the SNR because the filter bandwidth is much smaller than the original spectral bandwidth. Fig. S8 shows a schematic diagram of the ranging signal under three parameter profiles. Fig. S8a and Fig. S8b demonstrate that the traditional dual-comb TOF ranging mechanism requires a trade-off between detection resolution and detection rate. In our approach, this tradeoff is greatly weakened, as shown in Fig. S8c. Under the same light source parameters in Fig. S8b, the DFT-enhanced dual-comb ranging system can produce resolved interference fringes stably at a high measurement frame rate of 85 kHz.

Fig. S8. Comparison of measurement results between the two methods. a. Traditional TOF method with Δfrep = 300 Hz. b. Traditional TOF method with Δfrep = 85 kHz. c. Our DFT enhanced dual-comb ranging with Δfrep = 85 kHz.

S3.4 Ranging distance extension

Here we provide more discussions about the distance extension based on dual-comb vernier scheme. Since the repetition rate of the signal comb (fr1) used in the experiment is 24.465 MHz, the range ambiguity R=c/2fr1 is ~6.13 m in free space. Thanks to the Vernier effect [9], a new range ambiguity R'=c/2nΔfrep of ~1.765 km can be achieved by simply exchanging the roles of signal and local combs. We measure the distance without exchanging and with exchanging the dual combs respectively, with a measurement step of 1 m. As shown in Fig. S9a, when the to-measure distance is larger than 8m, the measured result will alias if we don’t exchange the dual combs. However, in the case that we exchange the dual combs, we can obtain the correct distance. In the experiment, we use a fixed fiber collimator (LBTEK TAD-11) to collimate & launch the signal light to the free space with abeam diameter 2.0 mm. A movable target mirror is placed on an electrically controllable slide guides to precisely tune the distance (Fig. S9b).

Fig. S9 Extended distance by vernier effect. a. Comparison of the result measured with/without exchanging. b. Photos of ranging devices.

S3.5 Ranging error analysis

According to Eq. (22), the ranging distance is proportional to the time of flight of the pulse. Therefore, the uncertainty of ranging can be obtained from the uncertainty of TOF. According to Eq. (2), the TOF can be written as ttof = NT2-(N-1)T1+τ1-τ3. Since N is an integer, the uncertainty of pulse time of flight depends mainly on the pulse period jitter of the light source and the fringe demodulation error of the system. In our method, the uncertainty of the measured ttof comes from two aspects: 1) the instability of T1 and T2; 2) the instability of the fiber system. Generally, the instability of the dual comb source plays the major role. We assume the instability of T1 is θ1, while the instability of T2 is θ2. Here could be a positive number or a negative number. Such an error will also influence τ1 and τ3, which indicates two pulse displacements. Therefore, the measured ttof could be written in ttof = NT2+θ2-(N-1)T1+θ1+τ1-τ3+(θ1+θ2). We obtain the maximum measured temporal error is 2θ1+2θ2, the maximum measured distance error is c(2θ1+2θ2). For single shot measurement, frequency instabilities of both the two laser combs are smaller than 0.5 Hz (see Fig. S5) in each sampling period (11.76 μs). Referring the repetition rates of the two combs are 24.465 MHz and 24.55 MHz, θ1 < 0.835 fs, while θ2 < 0.83 fs. As a result, the comb instability limited error in ranging experiment is smaller than 250 nm. On the other hand, we discuss the averaging effect. After averaging, frequency uncertainty of the comb repetition is on 10-10 level (< 5 mHz), in this case either θ1 or θ1 is smaller than 5×10-18 s. As a result, the dual comb uncertainty limited measurement error is < 1.5 nm. Table S1 compares the dual comb limited precision in principle and the real measured precision in our measurement. Our ranging system doesn’t reach the theoretical limitation, since there are random fluctuations in the fiber system.

Table S1. Performance comparison: ranging accuracy

|  | Single shot | After averaging |
| --- | --- | --- |
| Limited by the dual comb instability in principle | 250 nm | 1.5 nm |
| Experimentally measured | 262 nm | 2.8 nm |

Supplementary reference

[1]	Suh M-G, Vahala K J. Soliton microcomb range measurement[J]. Science, 2018, 359(6378): 884–887.

[2]	Zhu Z, Wu G. Dual-Comb Ranging[J]. Engineering, 2018, 4(6): 772–778.

[3]	Trocha P, Karpov M, Ganin D, et al. Ultrafast optical ranging using microresonator soliton frequency combs[J]. Science, 2018, 359(6378): 887–891.

[4]	Zhou Y, Chan J C K, Jalali B. A Unified Framework for Photonic Time‐Stretch Systems[J]. Laser & Photonics Reviews, 2022, 16(8): 2100524.

[5]	Mahjoubfar A, Churkin D V, Barland S, et al. Time stretch and its applications[J]. Nature Photonics, 2017, 11(6): 341–351.

[6]	Klauder J R. Path integrals and stationary-phase approximations[J]. Physical Review D, 1979, 19(8): 2349–2356.

[7]	Solli D R, Chou J, Jalali B. Amplified wavelength–time transformation for real-time spectroscopy[J]. Nature Photonics, 2008, 2(1): 48–51.

[8]	Herink G, Kurtz F, Jalali B, et al. Real-time spectral interferometry probes the internal dynamics of femtosecond soliton molecules[J]. Science, 2017, 356(6333): 50–54.

[9]	Coddington I, Swann W C, Nenadovic L, et al. Rapid and precise absolute distance measurements at long range[J]. Nature Photonics, 2009, 3(6): 351–356.