SUPPLEMENTARY MATERIAL

Supplementary Data 1:

Several settings of the FLIR One® Camera had to be selected before taking a thermal image:

- Iron” color palette of the FLIR One® camera had to be selected,

- The range of colors of the camera can be locked so as not to adapt automatically to the room and Temperature. The locking must be done manually at each time the camera is started.

- The calibration of the FLIR One® camera had to be set to “Matte mode” and without automatic calibration

We took thermal images of patients with the FLIR One® camera in standardized conditions:

- Thermal images of patients were taken in a dedicated room with ambient temperature as stable as possible (between 20 and 22 °C),

- the patient had to place his hands and forearms on a wooden support (material having the best compromise between the phenomena of conduction, remanence and reflection),

- the patient was not allowed to touch or be near any source of heat before taking the image

- nor to touch the wooden support except when taking the thermal image in order to avoid the heating of the support and therefore the contamination of the image.

- The patient was not allowed to smoke 30 minutes before taking the image and had to remove all jewelry from his wrists and hands. A resting period of 15 minutes had to be observed by the patient before taking the image.

- Patient’s ear temperature was systematically taken to avoid any fever.

- The photo had to be taken at a distance of 40 cm to improve thermal image quality.

- To facilitate recognition of the region of interests (ROI), forearms had to be placed paralleled and hands with fingers well apart without contact between them. The image had to include forearms up to 15 cm above the wrists.

Supplementary Data 2: Automatic detection of ROI using the GipsVision® software

Supplementary Data 3: Evaluation at the Patient Level

Clinical evaluation scores were then calculated (DAS28, CDAI and SDAI) to evaluate the global activity of RA. Patients were categorized by clinical DAS28 status (<2.6; [2.6-5.1]; > 5.1).

Mean ΔTp was respectively -0.06°C±0.50 in the remission DAS28 group (n=16), -0.44°C±0.65 in the medium DAS28 group (n=28) and +0.14°C±0.43 in the high DAS28 group (n=12). Overall, ΔTp differed across clinical DAS28 categories (<2.6; [2.6-5.1]; > 5.1) (p-ANOVA=0.01, Boxplot Data 4).

Mean ΔTp was not significantly different between the patients with CRP<5 mg/L (ΔTp= -0.18°C ±0.59, n=30) and those with CRP≥5 mg/L (ΔTp= -0.21°C±0.64, n=23) (p=0.83).

Boxplot Data 4: Comparison at the Patient Level between ΔT-patient (ΔTp) and clinical DAS28 categories

Legend:

- DAS28 <2.6 group (n=16): mean ΔT-p = -0.06°C ± 0.50

- DAS28 [2.6-5.1] group (n=28): mean ΔT-p = -0.44°C ± 0.65

- DAS28 >5.1 group (n=16): mean ΔT-p = +0.14°C ± 0.43

ΔTp differed across clinical DAS28 categories (<2.6; [2.6-5.1]; > 5.1), p-ANOVA = 0.01

ΔT-patient (ΔTp)

DAS 28 CRP categories