# Techno-economical evaluation of three methods for stripping gas recovery in gas dehydration by unit of Khark petrochemical complex

# Arman Mohammadi Abdehvanda, Mahdi Haghia,b,*, Ebrahim Hoseinirada

# a Research and Development department, Khark Petrochemical Company, Khark Island, Bushehr, Iran

# b Department of Chemical and Petroleum Engineering, Sharif University of Technology, Tehran, Iran

# * Corresponding author: haghi_1321@yahoo.com

Fig. S1 Sketch of options proposed for the stripping gas recovery

Table S1 Specification of equipment required for each options

| Equipment | Specification | Unit | Value |
| --- | --- | --- | --- |
| Option 1 |  |  |  |
| Compressor | Power | kW | 74.58 |
| Intercooler | Duty | kW | 175.3 |
| Cooling Water | Flowrate | m3/h | 15.2 |
| First Suction Drum | Diameter / Height | m | 0.5 /2.3 |
| Second Suction Drum | Diameter / Height | m | 0.35 /2.2 |
| Fuel Stop Tank | Diameter / Height | m | 0.5 /1.5 |
| Option 2 |  |  |  |
| Compressor | Power | kW | 22.4 |
| After Cooler | Duty | kW | 146.2 |
| Cooing Water | Volume Flowrate | m3/h | 12.6 |
| Propane Chiller | Duty | kW | 41.1 |
| Propane Refrigerant | Mass Flowrate | ton/h | 0.38 |
| Compressor Suction Drum | Diameter / Height | m | 0.5 / 2.3 |
| Compressor Discharge Drum | Diameter / Height | m | 0.4 / 2.2 |
| New Glycol Regenerator | Diameter / Height | m | 0.3 / 5.0 |
| New Reboiler | Duty | kW | 568 |
| Option 3 |  |  |  |
| New Reboiler | Duty | kw | 625 |
| Propane Chiller | Duty | kW | 196.7 |
| Propane Refrigerant | Mass Flowrate | ton/h | 1.83 |
| Water Separator | Diameter / Height | m | 0.35 / 2.2 |

Table S2 Cost estimation coefficient for CAPEX

| Row | Description | Cost Estimation Coefficient (F) |
| --- | --- | --- |
| 1 | Piping | 30% |
| 2 | Electrical | 10% |
| 3 | Instrument | 10% |
| 4 | Insulation | 7% |
| 5 | Construction and Installation | 30% |
| 6 | Contingencies | 10% |

Table S3 Utility prices used for the calculation of the operating cost

| Utility | Price ( Cop,i) | Unit |
| --- | --- | --- |
| Electricity | 0.04 | $/ kWh |
| Steam | 8.33 | $/ ton |
| Cooling Water | 0.034 | $/ ton |
| Propane for chilling | 2.94 | $ /ton |

Table S4 Capital cost estimation for different options

| CAPEX Item (1000 $) | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- |
| Compressor | 632.2 | 0.0 | 0.0 |
| Heat Exchanger | 39.3 | 64.6 | 38.8 |
| Drum | 4.4 | 4.3 | 2.5 |
| Tower | - | 8.1 | - |
| Total Equipment Cost | 675.9 | 77.0 | 41.3 |
| Piping | 202.8 | 23.1 | 12.4 |
| Electrical | 67.6 | 7.7 | 4.1 |
| Instrument | 67.6 | 7.7 | 4.1 |
| Insulation | 47.3 | 5.4 | 2.9 |
| Construction and Installation | 202.8 | 23.1 | 12.4 |
| Contingencies | 67.6 | 7.7 | 4.1 |
| Total CAPEX | 1331.6 | 151.8 | 81.4 |

Table S5 Utility consumption of for different options

| Utility Consumption | Unit | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| Power | kW | 74.58 | 22.4 | 0 |
| Steam | ton/h | 0.1 | 0.1 | 0.3 |
| Cooling Water | ton/h | 15.2 | 12.6 | 0 |
| Propane Chilling | ton/h | 0 | 0.4 | 1.83 |

Table S6 Operating cost estimation for different options

| Operating Cost | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- |
| Power | 26250 | 7884 | 0 |
| Steam | 5428 | 5428 | 17732 |
| Cooling Water | 4103 | 0 | 0 |
| Propane Chilling | 0 | 8849 | 42497 |
| Total Utility Price | 35780 | 22161 | 60229 |
| Maintenance | 20278 | 2311 | 1239 |
| Contingencies | 5605.9 | 2447.2 | 6146.7 |
| Total Operating Cost | 61664 | 26919 | 67614 |